IEEE P802.11
Wireless LANs

Usage Models

Date:	May 30, 2004

Abstract

This document defines usage models for 802.11 TGs, intended to be used to define the functional requirements for ESS Mesh and to specify well-defined simulation scenarios.

# Revision History

| ### Revision | ### Comments | ### Date | ### Author |
| --- | --- | --- | --- |
| R0 | Initial draft based on TGn Usage Models document 11-03-0802r16 | May 21, 2004 | SC |
| R1 | Edits and additions from ad-hoc group comments | May 30, 2004 | SC |

# Introduction

To support the definition of the 802.11 ESS Mesh WLAN standard within the IEEE (to be published eventually as the 802.11s amendment), this document attempts to define usage models based on various market-based use-cases. The usage models are intended to support the definitions of network simulations that will allow 802.11 TGs to evaluate the performance of various proposals in terms of, for example, network throughput, delay, packet loss and other metrics. It is anticipated that the outputs of this document will aid in the subsequent development of the evaluation and selection criteria used by TGs.

Note - These usage models that the usage model committee develops here are subject to the following constraints :

C1:	They are relevant to the expected uses of the technology

C2:	They pose a specific problem that can be addressed with 802.11 ESS mesh technology

C3:	They are capable of being turned into an unambiguous simulation scenario

# Process going forward

The following is a suggested process for developing usage model descriptions for TGs that will be useful for driving specific functional requirements, simulation scenarios, and evaluation criteria for 802.11s proposals.

- A usage model description template will be created to allow volunteers to describe different usage models in a consistent, comparable fashion.

- Volunteers will fill in the template with descriptions of different usage models of interest for TGs, including sample deployment topologies for each usage model.

- Usage model descriptions will be revised and prioritized based on group feedback.

- Detailed simulation scenarios will be defined for high priority usage models.  These simulation scenarios will include a specification of a representative topology, one or more channel model, mapping of application endpoints to devices in the topology, and other simulation parameters.

# Definitions

This section defines some of the terms used in this document.

Application – a source or sink of wireless data that relates to a particular type of user activity.

Examples: Streaming video. VOIP.

Environment – The type of place a WLAN system is deployed in.  Initial examples:  home, large office.

### Use case – A use case is a description of how an end user uses a system that exercises that system’s deployment of WLAN.  A use case includes an application with details regarding the user activity and both sides of the end-to-end link.

Usage Model – A specification of one or more applications and environments from which a simulation scenario can be created once the traffic patterns of the applications are known.  Usage models are created to "cover" use cases.

### Simulation Scenario – A simulation scenario is a description of a usage model that supports simulation.  A simulation scenario includes details needed for simulation.  Types of details to be included are descriptions that link the usage model to the simulation scenario: environment linked to a channel model, position of the Mesh APs and Mesh Points, uplink and downlink traffic, etc.  A simulation scenario is created from a Usage Model by characterising the traffic profile of the applications and possibly merging multiple applications together to reduce simulation time.

# Mappings between Application, Use case, Usage Model and Simulation Scenario

Understanding and defining the application, use case, usage model and simulation scenario are all necessary to create comparative results from 802.11 TGs proposals.

Each use case involves the use of one or more applications.  It represents a single type of use of a system using the technology.

Each application reflects a source or sink of data.   They will eventually be characterised in terms of a traffic profile that allows a simulation of the application to be created.

Each usage model contains a representative mixture of applications and channel models designed to adequately cover the important use cases.  There is a many to many mapping between use cases and usage models (i.e., the same use case may contribute to multiple usage models and the same usage model may include applications from multiple use cases).

There will be a one-to-one mapping between usage models and simulation scenarios.   The usage model is a marketing-oriented description of a "reasonable mixture" covering the important use cases.  The simulation scenario fills in any technical details necessary to fully define the simulation inputs not present in the usage model.

# - Environment to Channel Model Mapping

- Environment Definitions

# Applications

Table 3 lists the applications that are referred to from the usage models, together with relevant traffic parameters.

The parameters are defined as follows:

- MSDU size:	Packet size at the top of the MAC

- Maximum E2E PLR:	Maximum end-to-end packet loss rate at the top of the MAC.  This is defined by the loss rate that can be tolerated by the application.

- Maximum E2E Delay: 	Maximum end-to-end transport delay at the top of the MAC – i.e. between matching MA-UNITDATA.request and .indication.

- Protocol:	Indicates the network-layer protocol running between the data source and the MAC.  It takes one of two values: TCP or UDP.  These are intended to represent a generic acknowledged and a generic unacknowledged network-layer protocol.

Note on the meaning of "Offered load" and "Protocol".  Applications identified as being carried by UDP are assumed to generate MSDUs at a fixed rate, as identified in the "Offered load" column.  Inability to carry the traffic generated by a UDP application, due to insufficient throughput  capability, results in lost MSDUs, which is reported in simulation results as a packet loss rate, or an outage, associated with the application.  The comparison criteria include a measure of whether this packet loss rate exceeds the maximum specified for the application in this table.

Traffic carried by TCP is assumed to be served on a best-effort basis, and applications using TCP are assumed to generate MSDUs at rates up to the

value given in the "Offered load" column.  Being an acknowledged protocol with a constrained window size, TCP responds to congestion in the BSS by reducing application throughput without losing MSDUs.  This effect is reflected in simulation results by reporting achieved throughput for applications using TCP. Note: Acknowledgement traffic is generated by TCP sinks, this is not explicitly specified in the simulation scenarios, but it is included in the count of non-QoS flows and measurement of goodput.

The delay and PLR requirements for each application are specified in 4. Simulations may map applications and flows to specific QoS classes as necessary to satisfy these requirements. Proposers shall clearly state how applications and flows are mapped to specific QoS classes in their simulations.

Table 3 - Application Definitions

| Number | Application | Offered Load (Mbps) | Protocol | MSDU Size (B) | Maximum E2E PLR | Maximum E2E Delay (ms) | Source [ref] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DV Audio/video | 28.8 | UDP | 1024, Is this true? All other video/audio (SDTV, HDTV) transfers has this parameter set to 1500. | 10^-7 (corresponds to a rate of 0.5 loss/hour) | 200 | SD Specifications of Consumer-Use Digital VCRs Max PLR: 15-03-276r0 |
| 2 | VoD control channel | 0.06 | UDP | 64 | 10^-2 | 100 | Guess |
| 3 | SDTV | 4-5 | UDP | 1500 | 5*10^-7 | 200 | 1 |
| 4 | HDTV (Video/Audio) | 19.2-24 | UDP | 1500 | 10^-7 | 200 | 1 |
| 5 | DVD | 9.8 peak | UDP | 1500 | 10^-7 | 200 | 1 |
| 6 | Video Conf | 0.128 - 2 | UDP | 512 | 10^-2 | 100 | 1 |
| 7 | Internet Streaming video/audio | 0.1 – 4 | UDP | 512 | 10^-2 | 200 | 1 |
| 8 | Internet Streaming audio | 0.064~0.256 | UDP | 418 | 10^-4 | 200 | Group guess |
| 9 | VoIP | 0.02 – 0.15 | UDP | 100 | 5% | 30 | ITU-T G.114 300ms round-trip delay |
| 10 | Reserved |  |  |  |  |  |  |
| 11 | Reserved |  |  |  |  |  |  |
| 12 | MP3 Audio Other formats are taking over (AAC/MPEG-4, OggVorbis, etc) | 0.064 – 0.32 | UDP | 418 | 10^-4 | 200 | 1 |
| 12.5 | Reserved |  |  |  |  |  |  |
| 13 | Content download (photo camera) | 11 | TCP | 1500 | n/a |  | Corresponds to USB and flash speed |
| 14 | Internet File transfer (email, web, chat) | 1 | TCP | 300 | n/a |  |  |
| 15 | Local File transfer, printing | 30 | TCP | 1500 | n/a |  | Aps guess |
| 16 | Interactive Gaming [Controller to Console x 1] | 0.5 | UDP | 50 | 10^-4 | 16 | 2 |
| 17 | Interactive Gaming [Console to Display] | 100+ | UDP | 1500 | 10^-2 | 10 | 2 video image: 320x240x15 @ 60Hz |
| 18 | Interactive Gaming [Console to Internet Access] *NOTE : Depends on Game Type | 1 | UDP | 512 | 10^-4 | 50ms | Group consensus |
| 19 | Netmeeting application/desktop sharing | 0.5 | TCP | 512 | n/a |  | Group guess |
| 20 | Reserved |  |  |  |  |  |  |
| 21 | Reserved |  |  |  |  |  |  |
| 22 | Reserved |  |  |  |  |  |  |
| 23 | Video phone | 0.5 | UDP | 512 | 10^-2 | 100 | Aps guess |
| 24 | Remote user interface (X11, Terminal Server Client) (remote display/keyboard/mouse) | 0.5-1.5 (peak) | UDP | 700 | n/a | 100 | 11-03-0696r0 |

# Use Cases

contains a definition of the use cases used in this document.

Table 4 - Use Case Definitions

| Number | Use case | Application | Typical Environment (not exclusive) | Both Application Endpoints within the WLAN Mesh? |
| --- | --- | --- | --- | --- |
| 1 | One personal phone everywhere – home, office. Each person has a phone that works everywhere, home, office – same number. An extension of the cell phone into the office building. This includes cordless phone over VoIP. | VOIP integrated with other wireless WAN technologies | Residential, Enterprise – large and small, conference room |  |
| 2 | Multiplayer Internet gaming anywhere within the home / Internet Café. | Interactive gaming (console to internet), internet gaming (controller to console) | Residential/small enterprise (internet cafes) |  |
| 3 | Multiple TVs running throughout the home getting their content from a single remotely located AV-server/AP/set top box. Local control of the content (changing channels, etc). | HDTV, SDTV, VoD control channel | Residential |  |
| 4 | Link the home digital camera/video to the TV/display for display of pictures and movies taken. | DV Audio/Video | Residential |  |
| 5 | Watch a movie of your choice, when you want it, it your hotel room. | Internet streaming audio/video, SDTV, HDTV | Hotspot |  |
| 6 | Watch a clear replay of an event from your seat in a sporting arena. | Internet Streaming Video | Outdoor |  |
| 7 | Remotely located security cameras transmitting video signal to a monitoring location. | SDTV | Outside/Inside Residential, Small office building (not covered) |  |
| 8 | Music real time on multiple receivers throughout the home from a remotely located AV-server/AP/set top box receiver. | PCM Audio, MP3 Audio | Residential |  |
| 9 | Net meeting in a conference/class room to share someone’s display. 30 participants/students | Netmeeting application/desktop sharing | Conference room/class room |  |
| 10 | Reconfigurable / temporary office space, Ethernet cable replacement (similar throughput to wired cable). Back up files, email, web surfing, printing, etc. | Local File Transfer, printing | Enterprise – sea of cubes, home, hotel |  |
| 11 | Download video, music and other data files to a device in an automobile in the home garage or driveway. Broadband file transfer – at HT rates. | Internet File Transfer | Residential / Outdoor |  |
| 12 | Backup/transfer files between PCs located throughout the home, printing. Access point router. | Local File Transfer | Residential |  |
| 13 | Synchronize your local device with the server – email, calendar, etc. Hot spot/airport/airplane | Internet File Transfer | Large open area – hot spot, airport, train station, bus terminal. Airplane, Train |  |
| 14 | Download digital pictures and home movies to a PC/AV-server | Content download | Residential |  |
| 15 | Exchange files between PCs or between CE devices | Local File Transfer | Residential |  |
| 16 | Update inventory from the warehouse and the retail floor. | Local File Transfer | Industrial |  |
| 18 | Access of networked software from the classroom. 30 participants, simultaneously signing on. | Local File Transfer | Conference room/class room |  |
| 19 | Update/view medical records from patient rooms. | Local File Transfer | Industrial. |  |
| 20 | Obtain real time interactive player and game stats from your seat at a sporting event. | Internet File Transfer | Outdoor |  |
| 20.5 | View broadcast SDTV video/audio at a sporting event | SDTV | Outdoor / Arena |  |
| 20.6 | View Video on demand at a sporting event / concert-hall /hotspot | SDTV | Outdoor / Arena |  |
| 21 | Interactive multi-person gaming – ad hoc. | Interactive gaming | Home, train station |  |
| 22 | Point-point link for wireless backhaul | Backhaul traffic | Outdoor. |  |
| 23 | Point-multipoint link for wireless backhaul | Backhaul traffic. | Arena? Hotspot? Could be both outdoor and indoor |  |
| 24 | Point-multipoint link for Fixed Wireless Access | FWA traffic | Outdoor |  |
| 25 | reserved |  |  |  |
| 26 | reserved |  |  |  |
| 27 | reserved |  |  |  |
| 28 | Real-time streaming of ultrasound video and real-time viewing of x-ray/MRI/CT images as well as medical diagnostics signal streams / patient monitoring data | SDTV, local file transfer | Hospital (Industrial) similar to Large Enterprise (?) |  |
| 29 | Online distance learning/broadcasting locally | Internet File Transfer, Internet Streaming Audio/Video | Residential, small/large enterprises |  |
| 30 | Video conferencing with headset | Internet Streaming video/audio + headset interference | Small Enterprise |  |
| 31 | Enterprise high stress. Surfing the web, e-mail, printing, file transfers within the intranet. | Internet File Transfer. Printing. Local File Transfer | Small/Large Enterprise |  |
| 32 | Portable /Internet AV Devices. MP3 or other player playing music directly from an internet through a residential gateway. | Internet Streaming Audio | Residential |  |
| 33 | AV Communication Video Phone: Peer to peer AV communication. Video Conferencing: AV conference between multiple devices | Internet Streaming Video/Audio (multicast/broadcast) | Residential, Small/large Enterprise |  |
| 34 | reserved |  |  |  |
| 36 | Enterprise conference room – 20 to 30 users | Local file transfer, internet file transfer, printing | Enterprise |  |
| 37 | Lightweight terminal wirelessly connected to a remote computer | Remote user interface | Residential, Industrial, Enterprise |  |

# Usage Models

Table 5 defines the usage models defined by this document.

The purpose of these models is to merge representative use cases to create a small number of credible worst-case mixtures of applications.  Usage models also include deployment characteristics.  The usage models have to be realistic (in terms that they are covered by the use cases listed above), different from each other and cover some subset of the use cases that are identified to be priorities and capable of implementation in proposed 802.11s technology.

Table 5 - Usage Model Description Table

| Usage Model # | Usage Model Name | Deployment Characteristics |  |  |  |  |  |  |  |  | Traffic Characteristics |  |  | Unique Security Requirements/ Characteristics | Unique Mesh AP/ Mesh Point Device Characteristics (e.g. power, antenna, etc) | Management and Configuration of WLAN Mesh (Self-configuring or Managed?) | Motivations for WLAN Mesh Deployment | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | # Mesh APs/ Points | Mesh Physical Topology (Physical Area Size/Shape, Include Sample Topology Map) | Mesh Deployment Environment | Mesh AP/ Point Mobility | Mesh AP / Point Join / Exit Frequency | # Mesh APs/ Points with Portals to Other LANs | # STAs | STA distri-bution | STA mobility | Use Cases (Including Applications) | Mesh APs/ Points may be Application End-points? | Both application End-points commonly within the WLAN Mesh? |  |  |  |  |  |
| 1 | Small Office Example | Small (10) | 80 m^2 Rectangle (1:3) | Indoor | Low | Low | Small (2-3) | Small (25) | (Out of scope?) |  | 10, 31, 33, 1, 29 | Yes (SoftAP PCs) | Yes | Central or Distributed Key management | Omni-antenna | Self- configuring | Low cost, convenient deployment |  |
| 2 | Hotel Hotzone Example | Large (32) | 250 m^2 Circle | Indoor | Low | Low | Medium (6-8) | Large (150) |  |  | 1, 5, 13 | No (Dedicated MeshAPs) | No, primarily Internet access | Central Key management | Omni-antenna | Managed configuration | Low cost, convenient deployment, reliable coverage |  |

# Detailed Simulation Scenarios

This section is a placeholder for more detailed descriptions of usage models, including enough details to specify simulation scenarios.  These simulation scenarios will include a specification of a representative topology, one or more channel model, mapping of application endpoints to devices in the topology, and other simulation parameters.

Table 6 - Environment to Channel Model Mapping

| Model | Environment |
| --- | --- |
| A | Flat fading (no multipath) |
| B | Residential |
| C | Residential / Small Office |
| D | Typical Office |
| E | Large Office |
| F | Large Space (indoors / outdoors) |

Channel model specifications TBD.

Note, this corresponds to a loss of a 1024B MSDU per hour.   The TS PDU PLR is higher than this.  It is not known what is the effect to the decoder of giving it burst packet losses.

Note, a PLR of 10^-7 will not be measurable in our simulation technologies.