IEEE P802.11
Wireless LANs

Usage Models

Date:	November 17, 2004

Authors/Contributors:

| Name | Company | Address | Phone | Email |
| --- | --- | --- | --- | --- |
| W. Steven Conner | Intel Corporation | 2111 NE 25th Ave. Hillsboro, OR 97124 U.S.A. | +503-264-8036 | w.steven.conner@intel.com |
| Jonathan Agre | Fujitsu Laboratories of America | College Park, MD, U.S.A. | +301-486-0978 | jagre@fla.fujitsu.com |
| Hidenori Aoki | NTT DoCoMo Inc | DoCoMo R&D Center, 3-5 Hikarino-oka, Yokosuka-shi, Kanagawa, 239-8536 Japan | +81-46-840-6526 | aokihid@nttdocomo.co.jp |
| Malik Audeh | Tropos Networks | 555 Del Rey Ave., Sunnyvale, CA 94085 U.S.A. | +408-331-6814 | audeh@tropos.com |
| Michael Bahr | Siemens AG | CT IC 2, Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-49926 | bahr@siemens.com |
| Manoj Bhatnagar | Centillium |  |  | manoj@centillium.com |
| Narasimha Chari | Tropos Networks | 555 Del Rey Ave., Sunnyvale, CA 94085 U.S.A. | +408-331-6814 | chari@tropos.com |
| Kevin Dick | Nortel Networks | P.O. Box 3511 Station C, Ottawa ON K1Y 4H7 Canada | +613-763-4366 | kdick@nortelnetworks.com |
| Donald E. Eastlake III | Motorola Laboratories | 111 Locke Dr., Marlboro, MA 01752, U.S.A. | +508-786-7554 | Donald.eastlake@motorola.com |
| Jörg Habetha | Philips Research Laboratories | Weisshausstr. 2, D-52066 Aachen, Germany | +49-241-6003-560 | joerg.habetha@philips.com |
| Jim Hauser | Naval Research Laboratory | 4555 Overlook Ave., S.W., Washington, DC 20375 | +202-767-2771 | e-Mail: hauser@itd.nrl.navy.mil |
| Guido R. Hiertz | Aachen University | Kopernikusstr. 16 52064 Aachen Germany | +49-241-80-25-82-9 | hiertz@ieee.org |
| Yeonkwon Jeong | Information Communications University | 103-6 Munji-dong, Yuseong-gu, Daejeon, 305-714, KOREA | +82-42-866-6942 | ykwjeong@icu.ac.kr |
| Tyan-Shu Jou | Janusys Networks, Inc. | 502 Lowell Ave., Palo Alto, CA 94301, U.S.A. | +919-0656-2945 | tsjou@janusysnetworks.com |
| Ted Kuo | Janusys Networks, Inc. | 502 Lowell Ave., Palo Alto, CA 94301, U.S.A. | +650-387-0589 | ted@janusysnetworks.com |
| Art Martin | Intel Corporation | 2111 NE 25th Ave. Hillsboro, OR 97124 U.S.A. | +503-264-9261 | art.martin@intel.com |
| Yoichi Matsumoto | NTT DoCoMo Inc | DoCoMo R&D Center, 3-5 Hikarino-oka, Yokosuka-shi, Kanagawa, 239-8536 Japan | +81-46-840-3894 | matsumotoyou@nttdocomo.co.jp |
| Koji Omae | NTT DoCoMo Inc | DoCoMo R&D Center, 3-5 Hikarino-oka, Yokosuka-shi, Kanagawa, 239-8536 Japan | +81-46-840-3890 | omae@mlab.yrp.nttdocomo.co.jp |
| Stephen G. Rayment | BelAir Networks | 603 March Road, Kanata, ON K2K 2M5, Canada | +613-254-7070 x112 | srayment@belairnetworks.com |
| Kazuyuki Sakoda | Sony Corporation | Oval Court Ohsaki MW 2-17-1 Higashigotanda Shinagawa-ku Tokyo 141-0022 Japan | +81-3-6409-3201 | sako@wcs.sony.co.jp |
| D.J. Shyy | MITRE |  |  | djshyy@mitre.org |
| Tricci So | Nortel Networks | 3500 Carling Ave., Ottawa ON K2H 8E9, Canada | +613-763-9639 | tso@nortelnetworks.com |
| Jin Kue Wong | Nortel Networks | P.O. Box 3511 Station C, Ottawa, ON K1Y 4H7, Canada | +613-763-2515 | jkwong@nortelnetworks.com |
| James Woodyatt | Apple Computer |  |  | jhw@apple.com |
| Juan Carlos Zuniga | InterDigital | 1000 Sherbrooke W. 10th Floor, Montreal, QC, Canada | +514-904-6251 | JuanCarlos.Zuniga@InterDigital.com |
| Other contributors to be added (with permission) |  |  |  |  |

Abstract

This document defines usage models for 802.11 TGs, intended to be used to define the functional requirements for 802.11s Mesh Networking and to specify well-defined simulation scenarios.

Revision History

| ### Revision | ### Comments | ### Date | ### Author |
| --- | --- | --- | --- |
| R0 | Initial draft based on TGn Usage Models document 11-03-0802r16 | May 21, 2004 | SC |
| R1 | Edits and additions from ad-hoc group comments | May 30, 2004 | SC + contributions from adhoc group |
| R2 | Added additional notes on purpose of applications and use cases and more detailed instructions for filling out usage model template. | June 4, 2004 | SC |
| R3 | Edits based on feedback and new integrated contributions - reorganized usage model section with two tables, one for high level descriptions and topologies and the other for collecting more detailed characteristics - integrated all usage model examples, modifications, and comments received to-date - started abbreviations and acronyms table for completeness - added clarifying description text to applications and usage model sections - renumbered applications, removing skipped numbers - added additional example applications and use cases | June 17, 2004 | SC + contributions from adhoc group |
| R4 | Integrated new usage model contributions - Public Safety/First Responder Community Network - Residential Access Community Network - Parade or Temporary Event | June 18, 2004 | SC + contributions from adhoc group |
| R5 | Integrated additional usage model description text and sample topologies | June 25, 2004 | SC + contributions from adhoc group |
| R6 | Integrated additional usage model examples and list of contributing authors. | July 9, 2004 | SC + contributions from adhoc group |
| R7 | Updates from discussion in face-to-face ad-hoc group meeting on Monday morning, July 12, in Portland, OR. | July 12, 2004 | SC, and ad-hoc group |
| R8 | Added Straw Poll Results from July TGs meeting. | August 1, 2004 | SC |
| R9 | Integrated consolidated usage model descriptions from small teams for each of four major usage model categories: Residential, Office, Campus/Community/Public Access, Public Safety | August 30, 2004 | SC, and ad-hoc group |
| R10 | Updates to integrate comments from August 31 ad-hoc group teleconference call. | September 5, 2004 | SC, and ad-hoc group |
| R11 | Integrated stand-alone military usage model category description as instructed by TGs on Nov. 16th in San Antonio (see document 11-04/1393r0) | November 17, 2004 | SC |

Table of Contents

1.	Introduction	5

2.	Suggested process going forward	5

3.	TGs Straw Poll Prioritization Results	6

4.	Definitions	6

5.	Abbreviations and Acronyms	7

6.	Mappings between Application, Use case, Usage Model and Simulation Scenario	7

7.	High-Level Traffic Characteristics and Requirements	8

8.	Usage Models	8

9.	Detailed Simulation Scenarios	15

10.	Appendix	16

# Introduction

To support the definition of the 802.11 ESS Mesh WLAN standard within the IEEE (to be published eventually as the 802.11s amendment), this document attempts to define usage models based on various market-based use-cases. The usage models are intended to support the definitions of network simulations that will allow 802.11 TGs to evaluate qualitative requirements such as deployment characteristics and quantitative requirements such as the performance of various proposals in terms of, for example, network throughput, delay, packet loss and other metrics. It is anticipated that the outputs of this document will aid in the subsequent development of the evaluation and selection criteria used by TGs.

Note - These usage models that the usage model committee develops here are subject to the following constraints :

C1:	They are relevant to the expected uses of the technology

C2:	They pose a specific problem that can be addressed with 802.11 ESS mesh technology

C3:	They are capable of being turned into an unambiguous simulation scenario

# Suggested process going forward

The following is a suggested process for developing usage model descriptions for TGs that will provide a useful foundation for defining specific functional requirements, simulation scenarios, and evaluation criteria for 802.11s proposals.

- A usage model description template will be created to allow volunteers to describe different usage models in a consistent, comparable fashion.

- Volunteers will fill in the template with descriptions of different usage models of interest for TGs, including sample deployment topologies for each usage model.  These usage models will be organized in a small number of usage model categories, allowing similar usage models to be grouped together.

- For each category, a small sub-team of volunteers interested in the category will be formed to:

  - Refine usage model descriptions within each category

  - Merge similar usage models within each category

  - Develop initial requirements based on usage each usage model category (Qualitative and Quantitative Requirements)

- Usage models and initial requirements will then be prioritized, for example using one or more of the following criteria:

  - Scope of TGs from the PAR

  - Identified major outliers

  - Technology limitations

  - Voting

- Detailed functional requirements, representative simulation scenarios, and evaluation criteria for 802.11s will then be generated for high priority usage models.

This process is summarized in Figure 1.

Figure 1 – Possible Process for Using Usage Model Descriptions as Foundation for Detailed Requirements and Evaluation Criteria.

# TGs Straw Poll Prioritization Results

During the July 2004 TGs meeting, the following straw polls were taken (see 11-04/800r3):

  - Is documenting Usage Cases important?

    - Result: 48-0

  - Is each category important?

    - Residential – 36

    - Office – 43

    - Campus/Community/Public Access – 42

    - Public Safety – 34

    - Car to Car – 7

Based on these results, the following usage model categories will be the primary focus of this document: Residential, Office, Campus/Community/Pubic Access, and Public Safety.

# Definitions

This section defines some of the terms used in this document.

Application – a source or sink of wireless data that relates to a particular type of user activity.

Examples: Streaming video. VOIP.

Environment – The type of place a WLAN system is deployed in.  Initial examples:  home, large office.

### Use case – A use case is a description of how an end user uses a system that exercises that system’s deployment of WLAN.  A use case includes an application with details regarding the user activity and both sides of the end-to-end link.

Usage Model – A specification of one or more applications and environments from which a simulation scenario can be created once the traffic patterns of the applications are known.  Usage models are created to "cover" use cases.

### Simulation Scenario – A simulation scenario is a description of a usage model that supports simulation.  A simulation scenario includes details needed for simulation.  Types of details to be included are descriptions that link the usage model to the simulation scenario: environment linked to a channel model, position of the Mesh APs and Mesh Points, uplink and downlink traffic, etc.  A simulation scenario is created from a Usage Model by characterising the traffic profile of the applications and possibly merging multiple applications together to reduce simulation time.

# Abbreviations and Acronyms

DV		Digital Video

HDTV		High Definition TV

MSDU		MAC Service Data Unit

PLR		Packet Loss Rate

SDTV		Standard Definition TV

SMB		Small/Medium Business

TCP		Transmission Control Protocol

UDP		User Datagram Protocol

VoD		Video on Demand

VoIP		Voice Over Internet Protocol

# Mappings between Application, Use case, Usage Model and Simulation Scenario

Understanding and defining the application, use case, usage model and simulation scenario are all necessary to create comparative results from 802.11 TGs proposals.

Each use case involves the use of one or more applications.  It represents a single type of use of a system using the technology.

Each application reflects a source or sink of data.   They will eventually be characterised in terms of a traffic profile that allows a simulation of the application to be created.

Each usage model contains a representative mixture of applications and channel models designed to adequately cover the important use cases.  There is a many to many mapping between use cases and usage models (i.e., the same use case may contribute to multiple usage models and the same usage model may include applications from multiple use cases).

The usage model is a marketing-oriented description of a "reasonable mixture" covering the important use cases.  The simulation scenario fills in any technical details necessary to fully define the simulation inputs not present in the usage model.

# High-Level Traffic Characteristics and Requirements

TODO: Summarize Table 4 and Table 5 here.  The applications and use cases listed in these tables have been moved to the appendix of this document, since they provide  more detailed information than necessary for initial usage model descriptions for 802.11s.  This section will include a higher-level description of traffic descriptions and requirements to be referenced in the usage model descriptions.

# Usage Models

Table 1 includes a brief description and example topology for the usage models defined by this document.  Table 2 summarizes important characteristics for each usage models defined by this document.  Both tables should include identical row headers, with one row for each defined usage model.

The purpose of these models is to merge representative use cases to create a small number of credible worst-case mixtures of applications.  Usage models also include deployment characteristics.  The usage models have to be realistic (in terms that they are covered by the use cases listed above), different from each other and cover some subset of the use cases that are identified to be priorities and capable of implementation in proposed 802.11s technology.

The high-level usage model characteristics captured in these tables are intended to capture different expectations, assumptions, and characteristics of mesh network deployments for different scenarios.  The tables are intended to be a guideline to allow different volunteers to create comparable usage model descriptions.  However, it is not meant to limit the inclusion of relevant characteristics that are not explicitly listed in a column header.  If you have additional characteristics for a usage model that are not explicitly listed in an existing column in the table, please feel free to note them under a related column or in the comments column.

Note that the primary focus for usage model topology descriptions and deployment characteristics is on Mesh Points and Mesh APs, based on the scope of 802.11s.

Table 1 - Usage Model Descriptions and Sample Topologies Table

| Usage Model # | Usage Model Category | Description of Usage Model | Sample Topology |
| --- | --- | --- | --- |
| 1 | Residential | In the digital home usage model, the primary purposes for the mesh network are to create low-cost, easily deployable, high performance wireless coverage throughout the home. The mesh network should help to eliminate RF dead-spots and areas of low-quality wireless coverage throughout the home. High-bandwidth applications such as video distribution are likely to be used within a home network, thus high bandwidth performance will be very important for residential mesh networks. The most demanding usage of bandwidth in the mesh network is expected to come from device-to-device communication within the home, e.g. multi-media content distribution between different devices in the home. Mesh Points and Mesh APs may be implemented in dedicated AP devices, PCs, and high-bandwidth CE devices with line-power supply such as TVs, media center devices, and game consoles. STAs may be a combination of computing devices such as PCs, laptops, and PDAs, CE devices such as digital cameras, MP3 players, DVD players, and home automation devices such as control panels. In the short-term (3-5 years), the home network is expected to consist of a small number of Mesh APs/Mesh Points that are primarily dedicated devices or PCs. In the longer-term (5+ years), a larger number of CE devices are expected to become Mesh APs/Mesh Points, increasing the size of the mesh network over time. Some devices (e.g. battery powered CE devices) may be capable of operating as Mesh Points but require more conservative use of power than AC-powered Mesh Points. These low-power devices may optionally require Mesh Points to choose not to forward packets for other nodes in the network or to support a doze mode with lower duty cycle to conserve energy. A mesh network should be self-configuring to allow easy installation by non-technical consumers and ongoing operation without system administration. Mesh Points and Mesh APs may need to be configured as bridges to other LANs within the home, including, but not limited to, legacy ethernet LANs, 802.15 WPANs, and legacy 802.11 WLANs. As mesh deployments become more popular in the future, the coexistence of multiple mesh networks deployed in neighboring homes of dense residential complexes (such as apartments and neighborhoods) will become an important factor for network performance. Residential home networks must be able to coexist with other mesh networks and BSS networks deployed in nearby houses. This may require dynamic, self-configuring adaptation of RF settings such as channel and TX power for effective radio resource sharing. This also means that residential network deployments will often have multiple overlapping security domains, requiring security protocols to protect communication from malicious users that may overhear data transmissions. | Red points denote Mesh Points, and blue points denote STA. Two of the Mesh Points are Mesh APs. |
| 2 | Office | In the office usage model, the primary motivation for using mesh network technology is to create low-cost, easily deployable wireless networks that provide reliable coverage and performance. Wireless mesh networks are particularly useful in areas where Ethernet cabling does not exist or is cost prohibitive to install. With wireless mesh networks, offices can reduce capital costs associated with cable installation and reduce time required for deployment. They may also benefit from an increase in employee productivity through expanded connectivity to key data network resources. Examples offices where mesh networking technology may be deployed include small and medium businesses (SMB), large enterprise buildings, manufacturing plants, government buildings, and health care facilities. Mesh APs and Mesh Points will mostly be dedicated infrastructure devices, but PCs and other computing devices may also participate as Mesh Points and Mesh APs in the network. STAs may be a combination of PCs, laptops, PDAs, printers, mobile and desktop phones and other devices commonly found in an office environment. It is expected that many office networks will have higher device density and bandwidth requirements as compared to the campus and public safety scenarios. The office scenario will have particularly stringent security requirements in comparison to other usage model categories such as residential. It is important for the security of wireless mesh networks to equal the security of existing office WLAN networks. Many SMBs do not have a dedicated IT department to manage the network infrastructure, thus Mesh Points and Mesh APs should support an unmanaged mode in which the network can be self-configuring/self-managing. Most large enterprise offices have an IT department to manage the network infrastructure, thus the Mesh Points and Mesh APs should also support a mode where they can be centrally manageable. While enterprise networks may be deployed across both indoor and peripheral outdoor areas, the primary focus of this usage model category is on indoor deployments. Outdoor deployments (e.g. across a large enterprise campus) will be considered separately in the campus/community usage model category. |  |
| 3 | Campus/ Comm-unity/ Public Access Network | Mesh networking technology provides numerous and unique capabilities that can facilitate the deployment of large campus, community, and public access wireless networks. Some of the common requirements for this usage category include: - Seamless connectivity over large geographic areas. For example, university campus, community center / downtown areas, parks, and residential neighborhoods. This usage model differs from the other categories in that the number of Mesh APs that are required to deploy services is very high. Since these Mesh AP are typically located outdoors, and in locations with no wired infrastructure, this usage model requires a mechanism to wirelessly interconnect multiple Mesh Networks into a larger network. - Rapidly provide connectivity to locations where wired infrastructure is not available or is cost prohibitive. - Provide a lower cost / higher bandwidth alternative to traditional internet access methods (dial up, cable, DSL, fiber). Enable higher reliability internet access service by providing fault tolerant infrastructure, and redundant (to traditional wired access methods) access links. - Scalability, automatic/simplified reconfiguration, & reliability to minimize deployment and operating costs in harsh outdoor environments. Lower susceptibility to vandalism. - Enable advanced applications/services through ubiquitous access & reliable connectivity. For example, a common application that would need to be supported is a mobile WiFi phone than can roam throughout the 802.11s network. - Enable location based services to be deployed in an 802.11s network. Location information is needed for public safety services. Other applications include location specific programming / advertising such as email / instant messages to all within a certain area. For example 802.11s could enable restaurants to broadcast lunch specials information to all nearby WiFi stations. The above list captures requirements that may be unique to the campus usage model. Real-world deployments may include elements from both office usage model and campus usage model. During the standards development process, when simulations are developed, a mixed office & campus usage model scenario should also be considered. |  |
| 4 | Public Safety | Public safety mesh networks provide wireless network access to emergency and municipal safety personnel such as fire, police, and emergency workers responding to an incident scene. The network may be used for video surveillance, tracking emergency workers with bio-sensors, voice and data communication between emergency workers, uploading images, downloading hazmat information, tracking air status, etc. Public safety networks may be deployed over a wide range of scales, with respect to both the physical dimensions of the network and the number of Mesh Points/Mesh APs. Public safety mesh network deployments may consist of a combination of semi-permanent infrastructure installation (e.g. radios installed on poll tops) as well as mobile mesh points and mesh APs deployed at a scene during an emergency. While many mesh points in a public safety network may be mobile during the operation of the network, many back haul links are expected to be from fire trucks or other vehicles that are less mobile, more secure, and have better power. Communications for public safety networks are mostly outdoors, but may include communicating with first responders inside buildings (potentially deep inside with contact only by multi-hop relaying). The number of forwarding nodes may naturally exceed 32, which may require some ability for automatic partitioning into clusters, each of which uses 802.11s. Node mobility, dynamic variations in radio propagation, equipment/power failures, etc. make network self-configuration and self-management essential in these scenarios. | Note: need to update with topology that could be used in a simulation scenario. |
| 5 | Military | Military usage of mesh networks can be classified into two categories. The first category, non-combat usage, is adequately represented by the usage cases previously described in this document. The second category, combat operational usage, is distinguished by node mobility, a heavy reliance on fully automated network management and, for disadvantaged nodes, e.g., dismounted troops, sensitivity to energy conservation. Combat operations may occur both indoors and outdoors. The accompanying graphic illustrates an outdoor scenario with combat units clearing areas in an urban neighbourhood. This scenario can easily be extended to include indoor operations as troops enter buildings to clear them of enemy combatants. A key element of this scenario is the requirement for client STAs to temporarily switch roles to become mesh APs in order to relay traffic for troops that are at the forward point of the operation and, consequently, out of range of a mesh AP. When the former client STA is no longer needed as a critical relay, it may revert to its more energy conservative client STA role. The AP can be installed on a vehicle, inside a ship or on the backpack of dismounted military personnel. Power conservation is important for the latter AP deployment scenario. Situational awareness (SA) and voice communications are primary applications of interest to the military. SA traffic may include short, periodic packet transmissions to report troop positions and conditions to a combat operations center. SA traffic could also be real-time video feeds from individual troops or automated surveillance devices, e.g., UAVs. Moreover, the combat operations center may broadcast a common tactical picture back to the troops engaged in operations. Typically, military applications rely heavily on broadcast/multicast in addition to unicast traffic delivery. |  |

Table 2 - Usage Model Characteristics Table

| Usage Model # | Usage Model Category | Deployment Characteristics |  |  |  |  |  |  |  |  | Traffic Characteristics |  |  | Unique Security Requirements/ Characteristics | Unique Mesh AP/ Mesh Point Device Characteristics (e.g. power, antenna, etc) | Management and Configuration of WLAN Mesh (Self-configuring or Managed?) | Motivations for WLAN Mesh Deployment | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Total # Mesh APs/ Points | Mesh Physical Topology (Physical Area Size/Shape, Include Sample Topology Map) | Mesh Deployment Environment | Mesh AP/ Point Mobility | Mesh AP / Point Join / Exit Frequency | # Mesh APs/ Points with Portals to Other LANs | # STAs | STA distri-bution | STA mobility | Use Cases (Including Applications) | Mesh APs/ Points may be Application End-points? | Both application End-points commonly within the WLAN Mesh? |  |  |  |  |  |
| 1 | Residen-tial | Relatively Small (8) | 100 m2 to 400 m2 Note: the diameter (number of hops from edge-to-edge of the mesh network using one of the Basic Rates) of a residential mesh network is expected to typically be 2-3 hops. However, application performance demands may require longer path lengths to be used for data communication, e.g. to enable the use of high data rates. | Indoor | Low | Low | Small (2-3) Note: Mesh Portals are expected to connect a home mesh network to a broadband Internet connection (e.g. DSL or cable modem) as well as to connect a mesh network to several other networks (e.g. 802.3) in different rooms into a single household network. | Small (6) | (Out of scope) |  | 1, 2, 3, 4, 7, 8, 10, 11, 12, 14, 15, 21, 29, 32, 33 | Yes (Set-top box, VCR, TV, Desktop PC) | Yes | 1. Simple, Distributed Key Management (cannot assume presence of a radius server) (it should be possible for non-technical home owners to securely install Mesh APs/Mesh Points into a home network with minimal complexity. Many devices in a home mesh network will have minimal human interface capability for configuring security parameters.) 2. Prevent malicious aggressor from peep or destroying network 3. Different security domains may exist in same co-channel (e.g. neighboring homes) | 1.Omni-antenna 2. Some mesh points have restricted transmission power 3. Some mesh points need to conserve power 4. Some mesh points do not forward packets due to need to doze to save power | Self-configuring | 1. low-cost, non-invasive, convenient deployment (no need to install wires through walls) 3. seamless connectivity within the home 4. give always connected experience to users | Quantitative Measurement Parameters: -overall E-to-E throughput/jitter in an isolated mesh deployment -network maintenance signaling complexity -fairness among co-channel interfering WLAN Meshes -stability of the network -mesh point duty cycle (in the case of power saving mesh points). Qualitative Evaluation Parameters: -Support for mesh point power saving -ability to quickly recover from sleep mode -robustness against aggressor. |
| 2 | Office | Large (32 per 11s mesh) Note: A large enterprise may have a total of 50-100 Mesh AP/Point devices deployed across multiple interconnected Layer2 802.11s meshes. | 1000m2 -8000m2 | Mostly indoor, although may extend to outdoor courtyards, etc. | Mostly fixed, rare mobility. | Low | Medium (5-10 per 11s mesh) | 50-300 (per 11s mesh) |  |  | , 9, 10, 11, 13, 16, 18, 29, 30, 31, 33, 36, 37 | Mostly dedicated Mesh APs/Points, but some client devices (e.g. SoftAP PCs) could also be Mesh APs/Points (most likely in small/medium office) | Yes, although in large enterprise most traffic is expected to be between clients and servers on the intranet. Examples of peer-to-peer traffic within the wireless mesh network include printing, file sharing, connecting to a projector, etc. | A large enterprise will typically require centralized key management, but small offices may require distributed key management (similar to residential). | Most indoor mesh deployments are expected to use omni-directional antennas. Some large enterprise campuses may use directional-antennas for outdoor deployment between buildings (similar to campus network). | Mosted enterprise networks will require managed configuration by an IT department. However, some small/medium offices without a dedicated IT department will require the network to be self-configuring. | Low cost wireless network infrastructure, convenient and non-invasive deployment (reduce wire installation), reliable coverage for productivity and accessibility. | Focus on indoor deployment for simulation scenario, since campus usage model should cover outdoor deployment? |
| 3 | Campus/ Comm-unity/ Public Access Network | Large (32-100) Note: A large community network may have a total of 50-100 Mesh AP/Point devices deployed across multiple interconnected Layer2 802.11s meshes. | 1000 m2 –several square kilometers | Mostly outdoor, although public access model extend to indoor environment. | Mostly fixed, sometimes nomadic mobility. | Low | Medium (2-8 per 11s mesh) The mesh must have >= 2 portals to support scalability to larger mesh hierarchies. Perferably all the APs at the perimiter of the mesh would be mesh portals. | Large (20-1000 per 11s mesh) |  |  | 1, 2, 6, 7, 8, 9, 10, 11, 13, 14, 15, 18, 19, 20, 21, 23, 24, 25, 31 | Typically no, but some client devices could also be Mesh Points (e.g. at a temporary event deployment) | Typically no, although some Mesh Point capable clients may be the destination of the draffic. Note: In additional to traffic that is terminating in the mesh, there may be traffic that is passing through from one portal to another portal. Mechanisms to set priorities and QOS parameters for pass through traffic are required. | Mostly centralized key management. Some of the Mesh Points may participate in multiple security domains. Mesh portals must be able to participate in multiple security domains. For example, in the case of broadband service delivery over mesh networks, the mesh owned and operated by a residence would intersect with the mesh owned and operated by a service provider. Mesh portal devices must participate in both the local mesh and external mesh security domains. | For fixed Mesh Point/AP, directional antenna will be utilized, while some other Mesh Point/AP will utilize omni-directional antenna. | Mostly managed configuration. But some part of the mesh will be operated by self-configuration. | Accessibility, low cost infrastructure, border coverage. Wireless Internet, lower cost infrastructure, scalability. Low cost, reliable coverage. Reduced deployment time and cost, reliable coverage. Low cost, rapid deployment, self-configuring. Point of sales, advertisement, amusement customer attraction. | Focus on outdoor deployment for simulation scenario, since office usage model should cover indoor deployment? |
| 4 | Public Safety | Large (32 per 11s mesh) Note: A large public safety network may have a total of 50-100s of Mesh AP/Point devices deployed across multiple interconnected Layer2 802.11s meshes. | 250m2 – several square kilometers | Mostly outdoor, although may extend to emergency workers inside buildings. | Mix of fixed radios (e.g. on poll tops) and mobile nodes deployed in emergency vehicles and carried by emergency workers. | High | Large (5-20 per 11s mesh) | 30-250 |  |  | 1, 7, 8, 9, 11, 13, 14, 15, 37, 41, 42 | Yes, communication terminals carried by vehicles and personnel could also Mesh APs/Points | Yes | It should be possible to use either centralized or distributed key management, depending on the sitution. In a small incident scene, may have hierarchically organized user security, while a large incident scene may require the coexistence of several organizations with disjoint management. | Most mobile nodes are expected to use omni-directional antennas, but some dedicated backhaul nodes (e.g. on lamp posts) may use directional antennas for increased range and reliability. | Most public safety networks must be self-configuring to allow for rapid deployment in an emergency situation. | Zero infrastructure, low-cost and rapid deployement, improved range and reliability, improved battery life. | It should be possible for a mesh portal to connect the mesh network to the Internet over a cellular connection. |
| 5 | Military | Large (32 per 11s mesh) Note: A large military network may have a total of 50-100s of Mesh AP/Point devices deployed across multiple interconnected Layer2 802.11s meshes. | 250m2 – several square kilometers | Mostly outdoor, although may extend to military personnel inside buildings, tunnels, ships, etc. | May have a few stationary radios (e.g. at a combat operations center or temporarily placed on building tops, utility poles, etc.) and mobile nodes deployed in combat vehicles, ships or carried by dismounted troops. | High (e.g. pedestrian speeds, or slow vehicle speeds) | Large (5-20 per 11s mesh) | 30-250 |  |  | 1, 7, 8, 9, 11, 13, 14, 15, 37, 41, 42 | Yes, communication terminals carried by military vehicles and personnel could also be Mesh APs/Points. | Yes | It should be possible to use either centralized or distributed key management, depending on the situation. Operations involving a single command structure may have hierarchically organized user security, while large, joint combat operations may require the coexistence of several command structures with disjoint management. | Most mobile nodes are expected to use omni-directional antennas, but some advantaged nodes may use directional antennas for increased range, covertness, and reliability. | Most military networks must be self-configuring to allow for rapid deployment in a combat situation. | Zero infrastructure, low-cost and rapid deployment, improved range and reliability, improved battery life. | It should be possible for a mesh portal to connect the mesh network to a large network over long-range, non-802.11 military RF links or networks. |

# Detailed Simulation Scenarios

This section is a placeholder for more detailed descriptions of usage models, including enough details to specify simulation scenarios.  These simulation scenarios will include a specification of a representative topology, one or more channel model, mapping of application endpoints to devices in the topology, and other simulation parameters.

Table 3 - Environment to Channel Model Mapping

| Model | Environment |
| --- | --- |
| A | Flat fading (no multipath) |
| B | Residential |
| C | Residential / Small Office |
| D | Typical Office |
| E | Large Office |
| F | Large Space (indoors / outdoors) |

Channel model specifications TBD.

# Appendix

## Applications

Table 4 lists the applications that are referred to from the usage models, together with relevant traffic parameters.  The primary purpose of this table is to allow concrete application expectations/ requirements to be included in usage model descriptions.  Packet loss rate and maximum delay values listed in this table represent the end-to-end loss and delay budgets required for the various applications.   When defining evaluation criteria and detailed simulation scenarios for particular usage models, this table will also be useful for evaluating whether proposed routing protocols and/or MAC enhancements for 802.11s will be able to support application requirements under different configurations (e.g. topology, PHY/MAC choice, etc).

The parameters are defined as follows:

- MSDU size:	Packet size at the top of the MAC

- Maximum E2E PLR:	Maximum end-to-end packet loss rate at the top of the MAC.  This is defined by the loss rate that can be tolerated by the application.

- Maximum E2E Delay: 	Maximum end-to-end transport delay at the top of the MAC – i.e. between matching MA-UNITDATA.request and .indication.

- Protocol:	Indicates the network-layer protocol running between the data source and the MAC.  It takes one of two values: TCP or UDP.  These are intended to represent a generic acknowledged and a generic unacknowledged network-layer protocol.

Note on E2E characteristics: 802 .11s scenarios will often require data to be transmitted over multiple radio hops. Thus, when considering application performance characteristics we must consider the overall performance across multiple hops (not just across one link). This was the rationale for listing end-to-end characteristics.

Note on the meaning of "Offered load" and "Protocol".  Applications identified as being carried by UDP are assumed to generate MSDUs at a fixed rate, as identified in the "Offered load" column.  Inability to carry the traffic generated by a UDP application, due to insufficient throughput  capability, results in lost MSDUs, which is reported in simulation results as a packet loss rate, or an outage, associated with the application.  The comparison criteria may include a measure of whether this packet loss rate exceeds the maximum specified for the application in this table.

Traffic carried by TCP is assumed to be served on a best-effort basis, and applications using TCP are assumed to generate MSDUs at rates up to the

value given in the "Offered load" column.  Being an acknowledged protocol with a constrained window size, TCP responds to congestion in the BSS by reducing application throughput without losing MSDUs.  This effect may be reflected in simulation results by reporting achieved throughput for applications using TCP. Note: Acknowledgement traffic is generated by TCP sinks, this is not explicitly specified in the simulation scenarios, but it is included in the count of non-QoS flows and measurement of goodput.

The delay and PLR requirements for each application are specified in Table 4. Simulations may map applications and flows to specific QoS classes as necessary to satisfy these requirements. Proposers shall clearly state how applications and flows are mapped to specific QoS classes in their simulations.

Table 4 - Application Definitions

| Number | Application | Offered Load (Mbps) | Protocol | MSDU Size (B) | Maximum E2E PLR | Maximum E2E Delay (ms) | Source [ref] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DV Audio/video | 28.8 | UDP | 1024, Is this true? All other video/audio (SDTV, HDTV) transfers has this parameter set to 1500. | 10^-7 (corresponds to a rate of 0.5 loss/hour) | 200 | SD Specifications of Consumer-Use Digital VCRs Max PLR: 15-03-276r0 |
| 2 | VoD control channel | 0.06 | UDP | 64 | 10^-2 | 100 | 11n Guess |
| 3 | SDTV | 4-5 | UDP | 1500 | 5*10^-7 | 200 | 1 |
| 4 | HDTV (Video/Audio) | 19.2-24 | UDP | 1500 | 10^-7 | 200 | 1 |
| 5 | DVD | 9.8 peak | UDP | 1500 | 10^-7 | 200 | 1 |
| 6 | Video Conf | 0.128 - 2 | UDP | 512 | 10^-2 | 100 | 1 |
| 7 | Internet Streaming video/audio | 0.1 – 4 | UDP | 512 | 10^-2 | 200 | 1 |
| 8 | Internet Streaming audio | 0.064~0.256 | UDP | 418 | 10^-4 | 200 | 11n Group guess |
| 9 | VoIP | 0.02 – 0.15 | UDP | 100 | 5% | 300 | ITU-T G.114 300ms round-trip delay |
| 10 | MP3 Audio Other formats are taking over (AAC/MPEG-4, OggVorbis, etc) | 0.064 – 0.32 | UDP | 418 | 10^-4 | 200 | 1 |
| 11 | Content download (photo camera) | 11 (typical, but highly variable) | TCP | 1500 | n/a |  | Corresponds to USB and flash speed |
| 12 | Internet File transfer (email, web, chat) | 1 (typical, but highly variable) | TCP | 300 | n/a |  |  |
| 13 | Local File transfer, printing | 30 (typical, but highly variable) | TCP | 1500 | n/a |  |  |
| 14 | Interactive Gaming [Controller to Console x 1] | 0.5 | UDP | 50 | 10^-4 | 16 | 2 |
| 15 | Interactive Gaming [Console to Display] | 100+ | UDP | 1500 | 10^-2 | 10 | 2 video image: 320x240x15 @ 60Hz |
| 16 | Interactive Gaming [Console to Internet Access] *NOTE : Depends on Game Type | 1 | UDP | 512 | 10^-4 | 50ms | 11n Group consensus |
| 17 | Netmeeting application/desktop sharing | 0.5 | TCP | 512 | n/a |  | 11n Group guess |
| 18 | Video phone | 0.5 | UDP | 512 | 10^-2 | 400 | IMT-2000 |
| 19 | Remote user interface (X11, Terminal Server Client) (remote display/keyboard/mouse) | 0.5-1.5 (peak) | UDP | 700 | n/a | 100 | 11-03-0696r0 |
| 20 | BioSensor Data | .1 - .5 | UDP | ? | ? |  | JRA guess based on .5s update |
| 21 | Provide transparent L2 transport services among buildings of a campus | 1.0+ | L2, L3 protocols | Up to 1500 | n/a | n/a |  |

## Use Cases

Table 5 contains a definition of the use cases used in this document.  Use cases provide context about how various applications are used in usage models.  The same use case can potentially be used in multiple usage models, so use cases are referenced from this table rather than being listed in detail within usage model descriptions.

Table 5 - Use Case Definitions

| Number | Use case | Application | Typical Environment (not exclusive) | Both Application Endpoints within the WLAN Mesh? |
| --- | --- | --- | --- | --- |
| 1 | One personal phone everywhere – home, office. Each person has a phone that works everywhere, home, office – same number. An extension of the cell phone into the office building. This includes cordless phone over VoIP. | VOIP integrated with other wireless WAN technologies | Residential, Enterprise – large and small, conference room | Usually no |
| 2 | Multiplayer Internet gaming anywhere within the home / Internet Café. | Interactive gaming (console to internet), internet gaming (controller to console) | Residential/small enterprise (internet cafes) | Usually no |
| 3 | Multiple TVs running throughout the home getting their content from a single remotely located AV-server/AP/set top box. Local control of the content (changing channels, etc). | HDTV, SDTV, VoD control channel | Residential | Yes |
| 4 | Link the home digital camera/video to the TV/display for display of pictures and movies taken. | DV Audio/Video | Residential | Yes |
| 5 | Watch a movie of your choice, when you want it, it your hotel room. | Internet streaming audio/video, SDTV, HDTV | Hotspot | Usually no |
| 6 | Watch a clear replay of an event from your seat in a sporting arena. | Internet Streaming Video | Outdoor | Usually no |
| 7 | Remotely located security cameras transmitting video signal to a monitoring location. | SDTV | Outside/Inside Residential, Small office building (not covered) | Usually no |
| 8 | Music real time on multiple receivers throughout the home from a remotely located AV-server/AP/set top box receiver. | PCM Audio, MP3 Audio | Residential | Yes |
| 9 | Net meeting in a conference/class room to share someone’s display. 30 participants/students | Netmeeting application/desktop sharing | Conference room/class room | Sometimes |
| 10 | Reconfigurable / temporary office space, Ethernet cable replacement (similar throughput to wired cable). Back up files, email, web surfing, printing, etc. | Local File Transfer, printing | Enterprise – sea of cubes, home, hotel | Sometimes |
| 11 | Download video, music and other data files to a device in an automobile in the home garage or driveway. Broadband file transfer – at HT rates. | Internet File Transfer | Residential / Outdoor | Sometimes |
| 12 | Backup/transfer files between PCs located throughout the home, printing. Access point router. | Local File Transfer | Residential | Yes |
| 13 | Synchronize your local device with the server – email, calendar, etc. Hot spot/airport/airplane | Internet File Transfer | Large open area – hot spot, airport, train station, bus terminal. Airplane, Train | Usually no |
| 14 | Download digital pictures and home movies to a PC/AV-server | Content download | Residential | Yes |
| 15 | Exchange files between PCs or between CE devices | Local File Transfer | Residential | Yes |
| 16 | Update inventory from the warehouse and the retail floor. | Local File Transfer | Industrial | Usually no |
| 18 | Access of networked software from the classroom. 30 participants, simultaneously signing on. | Local File Transfer | Conference room/class room | Sometimes |
| 19 | Update/view medical records from patient rooms. | Local File Transfer | Industrial. | Sometimes |
| 20 | Obtain real time interactive player and game stats from your seat at a sporting event. | Internet File Transfer | Outdoor | Usually no |
| 20.5 | View broadcast SDTV video/audio at a sporting event | SDTV | Outdoor / Arena | No |
| 20.6 | View Video on demand at a sporting event / concert-hall /hotspot | SDTV | Outdoor / Arena | No |
| 21 | Interactive multi-person gaming – ad hoc. | Interactive gaming | Home, train station | Yes |
| 28 | Real-time streaming of ultrasound video and real-time viewing of x-ray/MRI/CT images as well as medical diagnostics signal streams / patient monitoring data | SDTV, local file transfer | Hospital (Industrial) similar to Large Enterprise (?) | Sometimes |
| 29 | Online distance learning/broadcasting locally | Internet File Transfer, Internet Streaming Audio/Video | Residential, small/large enterprises | No |
| 30 | Video conferencing with headset | Internet Streaming video/audio + headset interference | Small Enterprise | Usually no |
| 31 | Enterprise high stress. Surfing the web, e-mail, printing, file transfers within the intranet. | Internet File Transfer. Printing. Local File Transfer | Small/Large Enterprise | Sometimes |
| 32 | Portable /Internet AV Devices. MP3 or other player playing music directly from an internet through a residential gateway. | Internet Streaming Audio | Residential | No |
| 33 | AV Communication Video Phone: Peer to peer AV communication. Video Conferencing: AV conference between multiple devices | Internet Streaming Video/Audio (multicast/broadcast) | Residential, Small/large Enterprise | Usually no |
| 36 | Enterprise conference room – 20 to 30 users | Local file transfer, internet file transfer, printing | Enterprise | Sometimes |
| 37 | Lightweight terminal wirelessly connected to a remote computer | Remote user interface | Residential, Industrial, Enterprise | Sometimes |
| 38 | Enterprise VPN | Secure Tunneling through the mesh | Residential, Industrial, Enterprise | Usually no |
| 39 | Firefighters | Internet File Transfer Audio/Video (multi-,broad-cast) Bio-sense data Gateway (Portal) to cellular or sat nets | Indoor/Outdoor | Yes |
| 40 | A company, hospital, or university campus composed of a few nearby buildings interconnected by a set of mesh points at the top of each building and light posts in-between buildings to form an inter-building backbone | L2 transport service | Large enterprise | Yes |
| 41 | Command and Control Voice | VOIP? with group broadcast and pre-emption by privileged stations | Incident scenes | Usually |
| 42 | Emergency signal | Fire/burglar alarms, “man down” button on station, etc. (High priority, very low bandwidth, may wish to by-pass some security) | All | Sometimes |
| 43 | Home environment, CE Several Consumer Electronics (CE) devices spread across an apartment; worst case: several apartments in a skyscraper/tall building, neighbors to the left, right, below, above and behind; | Streaming of video and audio files; Video games, streaming between TV and games console; download of picture and data from portable to home devices, VoIP all over the apartment, Internet access; | Indoor, house with several floors and brick walls or apartment in apartment building | Yes |
| 44 | Public Access Public mesh network in a city for urban annotation and data-centric applications; high density of meshed APs (distance ~50 meters) in certain areas; several APs are connected through one gateway | Information services (traffic, urban), VoIP, some streaming applications; Eventually SoHo applications (broadband access) | City street, Airport and train station, supermarket area, mall and shopping centre area | No |
| 45 | Car2Car A multitude of highly mobile cars on a highway or in a city; varying density of the nodes from a few meters distance (in a traffic jam) to one kilometer; mesh networking among the cars; 250km/h | Wireless Local Danger Warning for among cars; Navigation support; Traffic jam information; Cacheable data transportation | Roadside, Highway, City | No for entertainment and information applications; Yes for danger and emergency application |

## Raw Usage Model Descriptions (Collected before July TGs meeting)

Table 6 includes a brief description and example topology for the usage models defined by this document.  Table 7 summarizes important characteristics for each usage models defined by this document.  Both tables should include identical row headers, with one row for each defined usage model.

The purpose of these models is to merge representative use cases to create a small number of credible worst-case mixtures of applications.  Usage models also include deployment characteristics.  The usage models have to be realistic (in terms that they are covered by the use cases listed above), different from each other and cover some subset of the use cases that are identified to be priorities and capable of implementation in proposed 802.11s technology.

The high-level usage model characteristics captured in these tables are intended to capture different expectations, assumptions, and characteristics of mesh network deployments for different scenarios.  The tables are intended to be a guideline to allow different volunteers to create comparable usage model descriptions.  However, it is not meant to limit the inclusion of relevant characteristics that are not explicitly listed in a column header.  If you have additional characteristics for a usage model that are not explicitly listed in an existing column in the table, please feel free to note them under a related column or in the comments column.

Note that the primary focus for usage model topology descriptions and deployment characteristics is on Mesh Points and Mesh APs, based on the scope of 802.11s.

Table 6 - Usage Model Descriptions and Sample Topologies Table  (Raw usage model descriptions collected before July TGs meeting)

| Usage Model # | Usage Model Category | Usage Model Name | Description of Usage Model | Sample Topology |
| --- | --- | --- | --- | --- |
| 1 | Residen-tial | Digital Home #1 | In the digital home usage model, the primary purposes for the mesh network are to create low-cost, easily deployable, high performance wireless coverage throughout the home. The mesh network should help to eliminate RF dead-spots. The most demanding usage of bandwidth in the mesh network is expected to come from device-to-device communication, e.g. multi-media content distribution between different devices in the home. Mesh Points and Mesh APs may be a combination of dedicated AP devices, PCs, and high-bandwidth CE devices with line-power supply such as TVs, media center devices, and game consoles. STAs may be a combination of computing devices such as PCs, laptops, and PDAs, CE devices such as digital cameras, MP3 players, DVD players, and home automation devices such as control panels. In the short-term (3-5 years), the home network is expected to consist of a small number of Mesh APs/Mesh Points that are primarily dedicated devices or PCs. In the longer-term (5+ years), a larger number of CE devices are expected to become Mesh APs/Mesh Points, increasing the size of the mesh network over time. A mesh network should be self-configuring to allow easy installation by non-technical consumers. |  |
| 2 |  | Digital home #2 (full usage of BW) | Digital home network will be configured by non-engineers. The network will be deployed by end users, and the location of the devices highly depends on the layout of the home. Typically, there is no well-educated system administrator or professional in home. Naturally, self-configuring network is preferable (possibly “zero-configuration from the user’s point of view”). Some of the STAs or Mesh Points may only have restricted transmission power due to power conservation, etc. Some of the BSSs configured by MeshAP may utilize same frequency channel as WLAN mesh. Co-channel interference from other BSS or other WLAN mesh is treated as out of scope of this deployment model. Mesh Portal or MeshAP which configure BSS at the different frequency channel may exist, but those are treated as out of scope of this usage model, as well. The motivation for bringing this usage model is: To measure overall E-to-E throughput/jitter performance at the isolated WLAN Mesh and BSS deployment. | Isolated Residential Red points denote Mesh Points, and blue points denote STA. Two of the Mesh Points are Mesh APs. |
| 3 |  | Digital home (network maintenance with power saving) | Same deployment assumption is made as above. Some of the mesh point is not forwarding a packet, and are in the doze mode for power saving, while maintaining network configuration. The motivation for bringing this usage model is: Many of the Mesh Points are installed in CE devices such as TV, VCR, STB, Audio Set, etc. Mesh Points with those devices serve WLAN Mesh services regardless of their own application activity. Late at night, most of those applications are in stand-by mode, and the power consumption caused by Mesh Point functionality will be dominant. To measure possible power saving functionality, as well as network stability, and network maintenance signaling complexity, using this model. Network stability must be somewhat restricted by power saving, but it shall be maintained. |  |
| 4 |  | Digital home (co-channel utilization with neighbours) | Digital home network may be configured by neighbors also, if TGs successfully creates a large market . Although frequency reuse must be performed as for the closest neighborhoods by means of DFS, etc, some of the neighboring signal could be reachable to the another WLAN Mesh network(another security domain) due to lack of frequency channel. Appropriate radio resource sharing among the neighbors should be treated as one of the functionality of WLAN Mesh services. The motivation for bringing this usage model is: To measure network stability in case of co-channel utilization by different households (network owner) from radio resource management point of view. | Co-channel Residential Red points denote Mesh Points. 2 Mesh Points located in the hall is aggressor. |
| 5 |  | Digital home (mis-user) | Similar deployment assumption is made as above. Digital home network could be affected by malicious aggressor, since the radio wave could be reachable to the network from outside of the home. WLAN Mesh service shall be robust against mis-users. The motivation for bringing this usage model is: To measure robustness against aggressor and stability of the network. |  |
| 6 | Office | Small/ Medium Office | In the small/medium office usage model, the primary purposes for the mesh network are to create low-cost, easily deployable networks with reliable coverage throughout an office building. Mesh APs will mostly be dedicated infrastructure devices, but some PCs may also participate as Mesh APs in the network. STAs may be a combination of PCs, laptops, PDAs, printers, telephones and other devices commonly found in an office environment. Since many small/medium offices do not have large IT departments to manage the networking infrastructure, the mesh network should be self-configuring to allow for easy deployment and management. |  |
| 7 |  | Large Enterprise #1 | In the large enterprise usage model, the primary purposes for the mesh network are to create a low-cost, easily deployable wireless network that provides reliable coverage and performance and is manageable by an IT department. Mesh APs will mostly be dedicated infrastructure devices. STAs may be a combination of PCs, laptops, PDAs, printers, mobile and desktop phones and other devices commonly found in an office environment. Since most large enterprises have an IT department to manage the network infrastructure, the mesh network should be centrally manageable. |  |
| 8 |  | Large Enterprise #2 | Wireless mesh networks can cover broad open areas both indoor and outdoor. Wireless mesh networks are particularly useful in areas where ethernet cabling does not exist or is cost prohibitive to install. With wireless mesh networks, enterprises can reduce capital costs associated with cable installation and reduce time required for deployment. Many enterprises will benefit from an increase in employee productivity through expanded connectivity to key data network resources. Examples are big business / campus, manufacturing plants, government buildings and health care / hospitals. |  |
| 9 | Campus/ Comm-unity/ Public Access Network | University Campus | Wireless mesh networks can provide cost effective campus wide coverage for faculty and students. Schools may be able to attract students and faculty members with innovative wireless data services. The university avoids cabling and trenching which are costly and disruptive. Students and faculty can enjoy the convenience of data connectivity over the entire campus. |  |
| 10 |  | Community Area Network (residential Internet Access) | Community–based wireless internet access built on wireless mesh networks may be used as an alternative to dial-up, DSL, or cable modem service. Wireless mesh networks offer rapid, scaleable, and flexible cost-effective deployment with lower recurring backhaul operational costs. Local programming / advertising can be targeted to interest groups within the neighborhood. Internet service can be provided inside and outside the home, and extended to coffee shops, corner stores, recreational areas and local schools. Community area mesh networks can be used to offer data services to municipalities or public safety initiatives. |  |
| 11 |  | Residential Access Community Network #2 | tbd | tbd |
| 12 |  | Inter-building backbone | tbd | Transparent L2 Transport Service Network |
| 13 |  | Parade or Temporary Event | tbd | tbd |
| 14 |  | Public Access: Urban streets | Groups of people will use VoIP and data-centric services in the HotSpot scenario. At pedestrian speed there will be low mobility and no topology changes of the mesh backbone. If end user devices were capable of mesh technology they could be used as relaying nodes, however, because of battery and privacy constraints, it is assumed that they will not be part of the mesh. The main purpose for the Mesh network in this scenario is ease of deployment and reduction of installation cost. There are high bandwidth demands and requirements on medium to small delay. |  |
| 15 | Public Safety | First Responder | Firefighter company with several trucks responds to fire scene and sets up network. Network tracks firemen with bio-sensors, firemen communicate, upload images, download hazmat information, track air status, equipment location. |  |
| 16 |  | Small Incident Scene | Small incident scene relates to fire/police/emergency personnel responding to an incident scene. Everything is mobile but probably back haul links are from fire trucks or other vehicles that are less mobile, more secure, and have better power. Communications are mostly outdoors but may include communicating with first responders inside (potentially deep inside with contact only by relaying) buildings. Certainly the number of forwarding nodes around may naturally exceed 32 which may require some ability for automatic partitioning into clusters each of which uses 802.11s. | Tbd |
| 17 |  | Large Incident Scene | Large incident scene relates to fire/police/emergency personnel responding to an incident scene. Everything is mobile but probably back haul links are from fire trucks or other vehicles that are less mobile, more secure, and have better power. Communications are mostly outdoors but may include communicating with first responders inside (potentially deep inside with contact only by relaying) buildings. Certainly the number of forwarding nodes around may naturally exceed 32 which may require some ability for automatic partitioning into clusters each of which uses 802.11s. | Tbd |
| 18 |  | Public Service Grid | Public Service Grid is the case where large areas are blanketed with pole top radios most of which are co-located with remote surveillance cameras or the like and not all of which are wired to a LAN. Variations in radio propagation, equipment/power failures, etc., make static configuration of the link topology of these radios undesirable. Furthermore, mobile units such as patrolmen and police/emergency vehicles should be able to access services via the nearest pole top radio. | Tbd |
| 19 |  | Public Safety/ First Responder Community Network |  | tbd |
| 20 | Car2Car | Danger Warning | Car to car (Car2Car) communication enables active safety measures. Danger warnings can be transported on an ad hoc network of cars using 802.11 mesh technology, i.e. informing succeeding cars of accidents or other blockings of the road ahead. The scenario is characterized by highly mobile stations. The Physical Layer required to support such high speeds will be standardized by 802.11p. However, the mesh networking aspects have to be covered by 802.11s. The Mesh network is subject to frequent topology changes and constant neighbor discovery. All routes and connections have to be set up in a very short time. On the other hand only a small amount of data has to be delivered, however, the delay has to be as small as possible. |  |
| 21 |  | Data/Telematics applications | Car-2-car communication enables data applications in cars in urban or rural scenarios where no full WLAN coverage is available. Typical applications are telematics applications for traffic or parking management. In traffic jam scenarios gaming, chatting or other applications are also possible. The scenario is characterized by medium to fast mobility of the participating stations. The network might be a hybrid of infrastructure-based and ad hoc components. When having a connection to the network each station will download at the highest possible data rate to fill up the internal cache again. The scenario is therefore characterized by high bandwidth demands but less strict delay demands. |  |

Table 7 – Usage Model Characteristics Table (Raw usage model characteristics collected before July TGs meeting)

| Usage Model # | Usage Model Category | Usage Model Name | Deployment Characteristics |  |  |  |  |  |  |  |  | Traffic Characteristics |  |  | Unique Security Requirements/ Characteristics | Unique Mesh AP/ Mesh Point Device Characteristics (e.g. power, antenna, etc) | Management and Configuration of WLAN Mesh (Self-configuring or Managed?) | Motivations for WLAN Mesh Deployment | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total # Mesh APs/ Points | Mesh Physical Topology (Physical Area Size/Shape, Include Sample Topology Map) | Mesh Deployment Environment | Mesh AP/ Point Mobility | Mesh AP / Point Join / Exit Frequency | # Mesh APs/ Points with Portals to Other LANs | # STAs | STA distri-bution | STA mobility | Use Cases (Including Applications) | Mesh APs/ Points may be Application End-points? | Both application End-points commonly within the WLAN Mesh? |  |  |  |  |  |
| 1 | Residen-tial | Digital Home #1 | 8 | 10m x 15m | Indoor | Low | Low | 1 | 20 | (Out of scope) |  | 1, 2, 3, 4, 7, 8, 10, 11, 12, 14, 15, 21, 29, 32, 33 | Yes (SoftAP PCs and CE Devices) | Yes | Central or Distributed Key management | Omni-antenna | Self- configuring | Low cost, convenient deployment |  |
| 2 |  | Digital home #2 (full usage of BW) | Relatively small (6) | 100m^2 Rectangle (1:2) [ref to sample topology figure R1] | Indoor | Low | Low | Small (1) | Small (6) |  |  |  | Yes. VCR, TV, desktop PC | Yes. | Distributed key management | Omni-antenna Some of the mesh point and STA has restricted transmission power. | Self-configuring | Seamless connectivity within home, give always connected experience to users. | Measurement parameter for this scenario is : Overall E-to-E throughput/jitter at the isolated Mesh and BSS deployment. |
| 3 |  | Digital home (network maintenance with power saving) | Relatively small (6) | 100m^2 Rectangle (1:2) [ref to sample topology figure R1] | Indoor | Low | Low | Small (1) | Small (6) |  |  |  | Yes. VCR, TV, desktop PC | Yes. | Distributed key management | Omni-antenna Most of the mesh points are conscious about power conservation. Some of the mesh point is not forwarding a packet, and are in the doze mode for power saving whle maintaining network configuration. | Self-configuring | Power saving functionality validation for mesh points within mesh network. | Measurement parameter for this scenario is ; Power saving validation. Network maintenance signalling complexity. Quick recovery from doze mode (mesh point). |
| 4 |  | Digital home (co-channel utilization with neighbours) | Medium (20) | Approx. 900m^2 (30m x 30m) [ref to sample topology figure R2] | Indoor | Low | Low | Small (1) | Small (3) |  |  |  | Yes. VCR, TV, desktop PC | Yes. | Distributed key management. Different security domain exists in co-channel. | Omni-antenna | Self-configuring | Robustness against co-channel utilization by the different households from radio resource management point of view. | Measurement parameter for this scenario is ; Fairness among different mesh network owners, and stability of the network. |
| 5 |  | Digital home (mis-user) | Medium (20) | Approx. 900m^2 (30m x 30m) [ref to sample topology figure R2] | Indoor | Low | Low | Small (1) | Small (3) |  |  |  | Yes. VCR, TV, desktop PC | Yes. | Malicious aggressor is trying to peep or destroy mesh network. | Omni-antenna | Self-configuring | Robustness against aggressor provides safety operation of wireless network. | Measurement parameter for this scenario is ; Robustness against aggressor, and stability of the network. |
| 6 | Office | Small/ Medium Office | 16 | 20m x 40m | Indoor | Low | Low | 2 | 80 |  |  | 1, 9, 10, 18, 29, 30, 31, 33, 36 | Yes (SoftAP PCs) | Yes | Central or Distributed Key management | Omni-antenna | Self- configuring | Low cost, convenient deployment, reliable coverage |  |
| 7 |  | Large Enterprise #1 | 32 | 6000m^2 | Indoor | Low | Low | 7 | 300 |  |  | 10, 31, 33, 1, 29, 9, 36, 18, 30 | No (Dedicated Mesh APs) | Yes (Voice, Printing, Projector, etc.) | Central Key Management | Omni-antenna | Managed Configuration | Low cost, convenient deployment, reliable coverage |  |
| 8 |  | Large Enterprise #2 | Large (50-100) | 8000 m^2 (per AP) | Indoor | Fix | N/A | Medium (5-10) | Large (500-2000) |  |  | 1, 9, 10, 11,13,16,18, 29,30, 31, 36,37 | No | No | Central Key management | Directional-antenna | Managed configuration | Productivity, accessibility and lower cost infrastructure |  |
| 9 | Campus/ Comm-unity/ Public Access Network | University Campus | Large (30-50) | 50 k m^2 – 100 km^2 (per AP) | Outdoor | Fix | N/A | Medium (4-5) | Large (300-1000) |  |  | 2,9,10,11,13, 18,20,21,31 | No | No | Central Key management | Directional antenna | Managed configuration | Accessibility, lower cost infrastructure, border coverage |  |
| 10 |  | Community Area Network (residential Internet Access) | Large (100) | 100 k m^2 (per AP) | Outdoor | Fix | N/A | Medium (10) | Large (2000 – 3000) |  |  | 1,2,11,14,21 | No | No | Central Key management | Directional antenna | Managed configuration | Wireless Internet, lower cost infrastructure, scalability |  |
| 11 |  | Residential Access Community Network #2 | Large (32) | 5 km^2 | Outdoor | Low | Low | Medium (4-5) | Large (1000) |  |  | ??? | No | No | Central Key Management | Omni/directional antennas, multiple radios/antennas | Managed configuration | Low cost, reliable coverage |  |
| 12 |  | Inter-building backbone | Large | 5km^2 [ref to topology figure 1] | Outdoor | Fixed | Low (restricted access only) | Medium (8-10) * Not sure what this field is for? Is it meant for the ratio of # of mesh points to a portal? | N/A |  |  | 6-9, 14, 15, 19, 23, 24, 25 | Yes (Access switches could be Mesh Portals) | Yes | Distributed key management | Omni, Sector, Smart-antenna | Managed configuration | Reduced deployment time and cost, reliable coverage |  |
| 13 |  | Parade or Temporary Event | Medium (16) | 2 km^2 | Outdoor | Medium | Medium | Small (1-2) | Small (25) |  |  | ??? | Yes | Yes | Distributed Key Management | Omni antennas | Self-configuring | Low cost, rapid deployment, self-configuring |  |
| 14 |  | Public Access: Urban access | Middle to large (32) | 50m*10m up to 1000m * 100m | Indoor and outdoor | None | Moderate | Medium – high (32) | Tens to hundred |  |  | 21 | No | No | Central key management | Omni directional | Self-configuring | Point of sales, advertisement, amusement customer attraction | Shopping center (Mall), outlet store etc. |
| 15 | Public Safety | First Responder | Large (32) | 250 m^2 Circle [ref to sample topology figure] | Indoor/Outdoor | Medium | Medium | Medium (6-8) | Medium (50) |  |  | Apps: 1,7,8,9,14,15,19; Use: 7,8,9,10,13,15,28,30,31,32,33,37 | Yes | Yes | Central or Distributed Key management | Omni or directional | Self- configuring | No Infrasturcture,Low cost,fast depoloy | Wireless portal to Internet (cellular) |
| 16 |  | Small Incident Scene | Medium (20) | 1000 m^2 | Mostly Outdoor | High | High | Small (2-3) | Small (30) |  |  | 11, 37, 41, 42 | Yes | Yes | Central Key management, users hierarchically organized |  | Self- configuring | Mesh improves range, reachability, battery life |  |
| 17 |  | Large Incident Scene | Very Large (100) | 1 km^2 | Mostly Outdoor | High | High | Large (10-20) | Large (250) |  |  | 11, 37, 41, 42 | Yes | Yes | Like DE-2 but may have several organizations with disjoint management |  | Self- configuring | Mesh improves range, reachability, battery life |  |
| 18 |  | Public Service Grid | Very Large (100s) | 100 km^2 | Outdoor | Low | Low | Very Large (50-100) | Large (250) |  |  | 7, 11, 13, 37, 41, 42 | Yes | No, most paths back to persons/services on fixed LAN |  |  | Self- configuring | Low cost, convenient deployment, reliable coverage |  |
| 19 |  | Public Safety/ First Responder Community Network | Large (32) | 5 km^2 | Outdoor | Low | Low | Small (1-2) | Low (25) |  |  | ??? | No | No | Central Key Management | Omni/ directional antennas, multiple radios/ antennas | Managed configuration | Low cost, reliable coverage |  |
| 20 | Car2Car | Danger Warning | Depending on number of cars | From urban (10m*100m) to highway (1000m*30m) dimesnion | Outdoor | Extremely high | High | 0 | High (up to some hundreds in traffic jam situation) |  |  | 20 | Yes | Yes | Distributed key management | Omni directional | Self-configuring | Driver safety | Low data to transmit, however very challenging due to high speed mobility |
| 21 |  | Data/Telematics applications | Depending of number of cars |  | Outdoor | Moderate to low | Low | 1-2 covering many cars | High (up to some hundreds in traffic jam situation) |  |  | 20 | Yes | Yes | Central key management possible | Omni directional | Self-configuring | Data access in car |  |

Note, this corresponds to a loss of a 1024B MSDU per hour.   The TS PDU PLR is higher than this.  It is not known what is the effect to the decoder of giving it burst packet losses.

Note, a PLR of 10^-7 will not be measurable in our simulation technologies.