IEEE P802.11
Wireless LANs

Usage Models

Date:	December 14, 2004

Authors/Contributors:

| Name | Company | Address | Phone | Email |
| --- | --- | --- | --- | --- |
| W. Steven Conner | Intel Corporation | 2111 NE 25th Ave. Hillsboro, OR 97124 U.S.A. | +503-264-8036 | w.steven.conner@intel.com |
| Jonathan Agre | Fujitsu Laboratories of America | College Park, MD, U.S.A. | +301-486-0978 | jagre@fla.fujitsu.com |
| Hidenori Aoki | NTT DoCoMo Inc | DoCoMo R&D Center, 3-5 Hikarino-oka, Yokosuka-shi, Kanagawa, 239-8536 Japan | +81-46-840-6526 | aokihid@nttdocomo.co.jp |
| Malik Audeh | Tropos Networks | 555 Del Rey Ave., Sunnyvale, CA 94085 U.S.A. | +408-331-6814 | audeh@tropos.com |
| Michael Bahr | Siemens AG | CT IC 2, Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-49926 | bahr@siemens.com |
| Manoj Bhatnagar | Centillium |  |  | manoj@centillium.com |
| Narasimha Chari | Tropos Networks | 555 Del Rey Ave., Sunnyvale, CA 94085 U.S.A. | +408-331-6814 | chari@tropos.com |
| Kevin Dick | Nortel Networks | P.O. Box 3511 Station C, Ottawa ON K1Y 4H7 Canada | +613-763-4366 | kdick@nortelnetworks.com |
| Donald E. Eastlake III | Motorola Laboratories | 111 Locke Dr., Marlboro, MA 01752, U.S.A. | +508-786-7554 | Donald.eastlake@motorola.com |
| Jörg Habetha | Philips Research Laboratories | Weisshausstr. 2, D-52066 Aachen, Germany | +49-241-6003-560 | joerg.habetha@philips.com |
| Jim Hauser | Naval Research Laboratory | 4555 Overlook Ave., S.W., Washington, DC 20375 | +202-767-2771 | hauser@itd.nrl.navy.mil |
| Guido R. Hiertz | Aachen University | Kopernikusstr. 16 52064 Aachen Germany | +49-241-80-25-82-9 | hiertz@ieee.org |
| Yeonkwon Jeong | Information Communications University | 103-6 Munji-dong, Yuseong-gu, Daejeon, 305-714, KOREA | +82-42-866-6942 | ykwjeong@icu.ac.kr |
| Tyan-Shu Jou | Janusys Networks, Inc. | 502 Lowell Ave., Palo Alto, CA 94301, U.S.A. | +919-0656-2945 | tsjou@janusysnetworks.com |
| Ted Kuo | Janusys Networks, Inc. | 502 Lowell Ave., Palo Alto, CA 94301, U.S.A. | +650-387-0589 | ted@janusysnetworks.com |
| Art Martin | Intel Corporation | 2111 NE 25th Ave. Hillsboro, OR 97124 U.S.A. | +503-264-9261 | art.martin@intel.com |
| Yoichi Matsumoto | NTT DoCoMo Inc | DoCoMo R&D Center, 3-5 Hikarino-oka, Yokosuka-shi, Kanagawa, 239-8536 Japan | +81-46-840-3894 | matsumotoyou@nttdocomo.co.jp |
| Koji Omae | NTT DoCoMo Inc | DoCoMo R&D Center, 3-5 Hikarino-oka, Yokosuka-shi, Kanagawa, 239-8536 Japan | +81-46-840-3890 | omae@mlab.yrp.nttdocomo.co.jp |
| Stephen G. Rayment | BelAir Networks | 603 March Road, Kanata, ON K2K 2M5, Canada | +613-254-7070 x112 | srayment@belairnetworks.com |
| Kazuyuki Sakoda | Sony Corporation | Oval Court Ohsaki MW 2-17-1 Higashigotanda Shinagawa-ku Tokyo 141-0022 Japan | +81-3-6409-3201 | sako@wcs.sony.co.jp |
| D.J. Shyy | MITRE | 7515 Colshire Drive, McLean, VA 22102, USA | +703-883-6515 | djshyy@mitre.org |
| Tricci So | Nortel Networks | 3500 Carling Ave., Ottawa ON K2H 8E9, Canada | +613-763-9639 | tso@nortelnetworks.com |
| Jin Kue Wong | Nortel Networks | P.O. Box 3511 Station C, Ottawa, ON K1Y 4H7, Canada | +613-763-2515 | jkwong@nortelnetworks.com |
| James Woodyatt | Apple Computer |  |  | jhw@apple.com |
| Juan Carlos Zuniga | InterDigital | 1000 Sherbrooke W. 10th Floor, Montreal, QC, Canada | +514-904-6251 | JuanCarlos.Zuniga@InterDigital.com |

Abstract

This document defines usage models for 802.11 TGs, intended to be used to define the functional requirements for 802.11s Mesh Networking and to specify well-defined simulation scenarios.

Revision History

| ### Revision | ### Comments | ### Date | ### Author |
| --- | --- | --- | --- |
| R0 | Initial draft based on TGn Usage Models document 11-03-0802r16 | May 21, 2004 | SC |
| R1 | Edits and additions from ad-hoc group comments | May 30, 2004 | SC + contributions from adhoc group |
| R2 | Added additional notes on purpose of applications and use cases and more detailed instructions for filling out usage model template. | June 4, 2004 | SC |
| R3 | Edits based on feedback and new integrated contributions - reorganized usage model section with two tables, one for high level descriptions and topologies and the other for collecting more detailed characteristics - integrated all usage model examples, modifications, and comments received to-date - started abbreviations and acronyms table for completeness - added clarifying description text to applications and usage model sections - renumbered applications, removing skipped numbers - added additional example applications and use cases | June 17, 2004 | SC + contributions from adhoc group |
| R4 | Integrated new usage model contributions - Public Safety/First Responder Community Network - Residential Access Community Network - Parade or Temporary Event | June 18, 2004 | SC + contributions from adhoc group |
| R5 | Integrated additional usage model description text and sample topologies | June 25, 2004 | SC + contributions from adhoc group |
| R6 | Integrated additional usage model examples and list of contributing authors. | July 9, 2004 | SC + contributions from adhoc group |
| R7 | Updates from discussion in face-to-face ad-hoc group meeting on Monday morning, July 12, in Portland, OR. | July 12, 2004 | SC, and ad-hoc group |
| R8 | Added Straw Poll Results from July TGs meeting. | August 1, 2004 | SC |
| R9 | Integrated consolidated usage model descriptions from small teams for each of four major usage model categories: Residential, Office, Campus/Community/Public Access, Public Safety | August 30, 2004 | SC, and ad-hoc group |
| R10 | Updates to integrate comments from August 31 ad-hoc group teleconference call. | September 5, 2004 | SC, and ad-hoc group |
| R11 | Integrated stand-alone military usage model category description as instructed by TGs on Nov. 16th in San Antonio (see document 11-04/1393r0) | November 17, 2004 | SC |
| R12 | Incorporated proposed updates from informal ad-hoc discussions | December 7, 2004 | SC |
| R13 | Updated contributing author list. Added strawpoll results for adding military usage case. Minor editorial updates to improve document format. | December 14, 2004 | SC |

Table of Contents

1.	Introduction	5

2.	TGs Straw Poll Prioritization Results	5

3.	Definitions	5

4.	Abbreviations and Acronyms	6

5.	Mappings between Application, Use case, Usage Model and Simulation Scenario	6

6.	Usage Models	7

# Introduction

To support the definition of the 802.11 ESS Mesh WLAN standard within the IEEE (to be published eventually as the 802.11s amendment), this document attempts to define usage models based on various market-based use-cases. The usage models are intended to support the definitions of network simulations that will allow 802.11 TGs to evaluate qualitative requirements such as deployment characteristics and quantitative requirements such as the performance of various proposals in terms of, for example, network throughput, delay, packet loss and other metrics. It is anticipated that the outputs of this document will aid in the subsequent development of the evaluation and selection criteria used by TGs.

Note - These usage models that the usage model committee develops here are subject to the following constraints :

C1:	They are relevant to the expected uses of the technology

C2:	They pose a specific problem that can be addressed with 802.11 ESS mesh technology

C3:	They are capable of being turned into an unambiguous simulation scenario

# TGs Straw Poll Prioritization Results

During the July 2004 TGs meeting, the following straw polls were taken (see 11-04/800r3):

  - Is documenting Usage Cases important?

    - Result: 48-0

  - Is each category important?

    - Residential – 36

    - Office – 43

    - Campus/Community/Public Access – 42

    - Public Safety – 34

    - Car to Car – 7

During the November 2004 TGs meeting, the following straw poll was taken (see 11-04/1464r0):

      - Should military be a separate usage case?

        - Don’t include – 0

        - Include as separate usage case – 23

        - Include merged with public safety usage case – 20

Decision taken to add military as a separate case in the use case document

Based on these results, the following usage model categories are the primary focus of this document: Residential, Office, Campus/Community/Pubic Access, Public Safety, and Military.

# Definitions

This section defines some of the terms used in this document.

Application – a source or sink of wireless data that relates to a particular type of user activity.

Examples: Streaming video. VOIP.

Environment – The type of place a WLAN system is deployed in.  Initial examples:  home, large office.

### Use case – A use case is a description of how an end user uses a system that exercises that system’s deployment of WLAN.  A use case includes an application with details regarding the user activity and both sides of the end-to-end link.

Usage Model – A specification of one or more applications and environments from which a simulation scenario can be created once the traffic patterns of the applications are known.  Usage models are created to "cover" use cases.

### Simulation Scenario – A simulation scenario is a description of a usage model that supports simulation.  A simulation scenario includes details needed for simulation.  Types of details to be included are descriptions that link the usage model to the simulation scenario: environment linked to a channel model, position of the Mesh APs and Mesh Points, uplink and downlink traffic, etc.  A simulation scenario is created from a Usage Model by characterising the traffic profile of the applications and possibly merging multiple applications together to reduce simulation time.

# Abbreviations and Acronyms

DV		Digital Video

HDTV		High Definition TV

MSDU		MAC Service Data Unit

PLR		Packet Loss Rate

SDTV		Standard Definition TV

SMB		Small/Medium Business

TCP		Transmission Control Protocol

UDP		User Datagram Protocol

VoD		Video on Demand

VoIP		Voice Over Internet Protocol

# Mappings between Application, Use case, Usage Model and Simulation Scenario

Understanding and defining the application, use case, usage model and simulation scenario are all necessary to create comparative results from 802.11 TGs proposals.

Each use case involves the use of one or more applications.  It represents a single type of use of a system using the technology.

Each application reflects a source or sink of data.   They will eventually be characterised in terms of a traffic profile that allows a simulation of the application to be created.

Each usage model contains a representative mixture of applications and channel models designed to adequately cover the important use cases.  There is a many to many mapping between use cases and usage models (i.e., the same use case may contribute to multiple usage models and the same usage model may include applications from multiple use cases).

The usage model is a marketing-oriented description of a "reasonable mixture" covering the important use cases.  The simulation scenario fills in any technical details necessary to fully define the simulation inputs not present in the usage model.

# Usage Models

Table 1 includes a brief description and example topology for the usage models defined by this document.  Table 2 summarizes important characteristics for each usage models defined by this document.  Both tables should include identical row headers, with one row for each defined usage model.

The purpose of these models is to merge representative use cases to create a small number of credible worst-case mixtures of applications.  Usage models also include deployment characteristics.  The usage models have to be realistic (in terms that they are covered by the use cases listed above), different from each other and cover some subset of the use cases that are identified to be priorities and capable of implementation in proposed 802.11s technology.

The high-level usage model characteristics captured in these tables are intended to capture different expectations, assumptions, and characteristics of mesh network deployments for different scenarios.  The tables are intended to be a guideline to allow different volunteers to create comparable usage model descriptions.  However, it is not meant to limit the inclusion of relevant characteristics that are not explicitly listed in a column header.  If you have additional characteristics for a usage model that are not explicitly listed in an existing column in the table, please feel free to note them under a related column or in the comments column.

Note that the primary focus for usage model topology descriptions and deployment characteristics is on Mesh Points and Mesh APs, based on the scope of 802.11s.

Table 1 - Usage Model Descriptions and Sample Topologies Table

| Usage Model # | Usage Model Category | Description of Usage Model | Sample Topology |
| --- | --- | --- | --- |
| 1 | Residential | In the digital home usage model, the primary purposes for the mesh network are to create low-cost, easily deployable, high performance wireless coverage throughout the home. The mesh network should help to eliminate RF dead-spots and areas of low-quality wireless coverage throughout the home. High-bandwidth applications such as video distribution are likely to be used within a home network, thus high bandwidth performance will be very important for residential mesh networks. The most demanding usage of bandwidth in the mesh network is expected to come from device-to-device communication within the home, e.g. multi-media content distribution between different devices in the home. Mesh Points and Mesh APs may be implemented in dedicated AP devices, PCs, and high-bandwidth CE devices with line-power supply such as TVs, media center devices, and game consoles. STAs may be a combination of computing devices such as PCs, laptops, and PDAs, CE devices such as digital cameras, MP3 players, DVD players, and home automation devices such as control panels. In the short-term (3-5 years), the home network is expected to consist of a small number of Mesh APs/Mesh Points that are primarily dedicated devices or PCs. In the longer-term (5+ years), a larger number of CE devices are expected to become Mesh APs/Mesh Points, increasing the size of the mesh network over time. Some devices (e.g. battery powered CE devices) may be capable of operating as Mesh Points but require more conservative use of power than AC-powered Mesh Points. These low-power devices may optionally require Mesh Points to choose not to forward packets for other nodes in the network or to support a doze mode with lower duty cycle to conserve energy. A mesh network should be self-configuring to allow easy installation by non-technical consumers and ongoing operation without system administration. Mesh Points and Mesh APs may need to be configured as bridges to other LANs within the home, including, but not limited to, legacy ethernet LANs, 802.15 WPANs, and legacy 802.11 WLANs. As mesh deployments become more popular in the future, the coexistence of multiple mesh networks deployed in neighboring homes of dense residential complexes (such as apartments and neighborhoods) will become an important factor for network performance. Residential home networks must be able to coexist with other mesh networks and BSS networks deployed in nearby houses. This may require dynamic, self-configuring adaptation of RF settings such as channel and TX power for effective radio resource sharing. This also means that residential network deployments will often have multiple overlapping security domains, requiring security protocols to protect communication from malicious users that may overhear data transmissions. | Red points denote Mesh Points, and blue points denote STA. Two of the Mesh Points are Mesh APs. |
| 2 | Office | In the office usage model, the primary motivation for using mesh network technology is to create low-cost, easily deployable wireless networks that provide reliable coverage and performance. Wireless mesh networks are particularly useful in areas where Ethernet cabling does not exist or is cost prohibitive to install. With wireless mesh networks, offices can reduce capital costs associated with cable installation and reduce time required for deployment. They may also benefit from an increase in employee productivity through expanded connectivity to key data network resources. Examples offices where mesh networking technology may be deployed include small and medium businesses (SMB), large enterprise buildings, manufacturing plants, government buildings, and health care facilities. Mesh APs and Mesh Points will mostly be dedicated infrastructure devices, but PCs and other computing devices may also participate as Mesh Points and Mesh APs in the network. STAs may be a combination of PCs, laptops, PDAs, printers, mobile and desktop phones and other devices commonly found in an office environment. It is expected that many office networks will have higher device density and bandwidth requirements as compared to the campus and public safety scenarios. The office scenario will have particularly stringent security requirements in comparison to other usage model categories such as residential. It is important for the security of wireless mesh networks to equal the security of existing office WLAN networks. Many SMBs do not have a dedicated IT department to manage the network infrastructure, thus Mesh Points and Mesh APs should support an unmanaged mode in which the network can be self-configuring/self-managing. Most large enterprise offices have an IT department to manage the network infrastructure, thus the Mesh Points and Mesh APs should also support a mode where they can be centrally manageable. While enterprise networks may be deployed across both indoor and peripheral outdoor areas, the primary focus of this usage model category is on indoor deployments. Outdoor deployments (e.g. across a large enterprise campus) will be considered separately in the campus/community usage model category. |  |
| 3 | Campus/ Comm-unity/ Public Access Network | Mesh networking technology provides numerous and unique capabilities that can facilitate the deployment of large campus, community, and public access wireless networks. Some of the common requirements for this usage category include: - Seamless connectivity over large geographic areas. For example, university campus, community center / downtown areas, parks, and residential neighborhoods. This usage model differs from the other categories in that the number of Mesh APs that are required to deploy services is very high. Since these Mesh AP are typically located outdoors, and in locations with no wired infrastructure, this usage model requires a mechanism to wirelessly interconnect multiple Mesh Networks into a larger network. - Rapidly provide connectivity to locations where wired infrastructure is not available or is cost prohibitive. - Provide a lower cost / higher bandwidth alternative to traditional internet access methods (dial up, cable, DSL, fiber). Enable higher reliability internet access service by providing fault tolerant infrastructure, and redundant (to traditional wired access methods) access links. - Scalability, automatic/simplified reconfiguration, & reliability to minimize deployment and operating costs in harsh outdoor environments. Lower susceptibility to vandalism. - Enable advanced applications/services through ubiquitous access & reliable connectivity. For example, a common application that would need to be supported is a mobile WiFi phone than can roam throughout the 802.11s network. - Enable location based services to be deployed in an 802.11s network. Location information is needed for public safety services. Other applications include location specific programming / advertising such as email / instant messages to all within a certain area. For example 802.11s could enable restaurants to broadcast lunch specials information to all nearby WiFi stations. The above list captures requirements that may be unique to the campus usage model. Real-world deployments may include elements from both office usage model and campus usage model. During the standards development process, when simulations are developed, a mixed office & campus usage model scenario should also be considered. |  |
| 4 | Public Safety | Public safety mesh networks provide wireless network access to emergency and municipal safety personnel such as fire, police, and emergency workers responding to an incident scene. The network may be used for video surveillance, tracking emergency workers with bio-sensors, voice and data communication between emergency workers, uploading images, downloading hazmat information, tracking air status, etc. Public safety networks may be deployed over a wide range of scales, with respect to both the physical dimensions of the network and the number of Mesh Points/Mesh APs. Public safety mesh network deployments may consist of a combination of semi-permanent infrastructure installation (e.g. radios installed on poll tops) as well as mobile mesh points and mesh APs deployed at a scene during an emergency. While many mesh points in a public safety network may be mobile during the operation of the network, many back haul links are expected to be from fire trucks or other vehicles that are less mobile, more secure, and have better power. Communications for public safety networks are mostly outdoors, but may include communicating with first responders inside buildings (potentially deep inside with contact only by multi-hop relaying). The number of forwarding nodes may naturally exceed 32, which may require some ability for automatic partitioning into clusters, each of which uses 802.11s. Node mobility, dynamic variations in radio propagation, equipment/power failures, etc. make network self-configuration and self-management essential in these scenarios. | Note: need to update with topology that could be used in a simulation scenario. |
| 5 | Military | Military usage of mesh networks can be classified into two categories. The first category, non-combat usage, is adequately represented by the usage cases previously described in this document. The second category, combat operational usage, is distinguished by node mobility, a heavy reliance on fully automated network management and, for disadvantaged nodes, e.g., dismounted troops, sensitivity to energy conservation. Combat operations may occur both indoors and outdoors. The accompanying graphic illustrates an outdoor scenario with combat units clearing areas in an urban neighbourhood. This scenario can easily be extended to include indoor operations as troops enter buildings to clear them of enemy combatants. A key element of this scenario is the requirement for client STAs to temporarily switch roles to become mesh APs in order to relay traffic for troops that are at the forward point of the operation and, consequently, out of range of a mesh AP. When the former client STA is no longer needed as a critical relay, it may revert to its more energy conservative client STA role. The AP can be installed on a vehicle, inside a ship or on the backpack of dismounted military personnel. Power conservation is important for the latter AP deployment scenario. Situational awareness (SA) and voice communications are primary applications of interest to the military. SA traffic may include short, periodic packet transmissions to report troop positions and conditions to a combat operations center. SA traffic could also be real-time video feeds from individual troops or automated surveillance devices, e.g., UAVs. Moreover, the combat operations center may broadcast a common tactical picture back to the troops engaged in operations. Typically, military applications rely heavily on broadcast/multicast in addition to unicast traffic delivery. |  |

Table 2 - Usage Model Characteristics Table

| Usage Model # | Usage Model Category | Deployment Characteristics |  |  |  |  |  |  |  |  | Traffic Characteristics |  |  | Unique Security Requirements/ Characteristics | Unique Mesh AP/ Mesh Point Device Characteristics (e.g. power, antenna, etc) | Management and Configuration of WLAN Mesh (Self-configuring or Managed?) | Motivations for WLAN Mesh Deployment | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Total # Mesh APs/ Points | Mesh Physical Topology (Physical Area Size/Shape, Include Sample Topology Map) | Mesh Deployment Environment | Mesh AP/ Point Mobility | Mesh AP / Point Join / Exit Frequency | # Mesh APs/ Points with Portals to Other LANs | # STAs | STA distri-bution | STA mobility | Use Cases (Including Applications) | Mesh APs/ Points may be Application End-points? | Both application End-points commonly within the WLAN Mesh? |  |  |  |  |  |
| 1 | Residen-tial | Relatively Small (8) | 100 m2 to 400 m2 Note: the diameter (number of hops from edge-to-edge of the mesh network using one of the Basic Rates) of a residential mesh network is expected to typically be 2-3 hops. However, application performance demands may require longer path lengths to be used for data communication, e.g. to enable the use of high data rates. | Indoor | Low | Low | Small (2-3) Note: Mesh Portals are expected to connect a home mesh network to a broadband Internet connection (e.g. DSL or cable modem) as well as to connect a mesh network to several other networks (e.g. 802.3) in different rooms into a single household network. | Small (6) | (Out of scope) |  | 1, 2, 3, 4, 7, 8, 10, 11, 12, 14, 15, 21, 29, 32, 33 | Yes (Set-top box, VCR, TV, Desktop PC) | Yes | 1. Simple, Distributed Key Management (cannot assume presence of a radius server) (it should be possible for non-technical home owners to securely install Mesh APs/Mesh Points into a home network with minimal complexity. Many devices in a home mesh network will have minimal human interface capability for configuring security parameters.) 2. Prevent malicious aggressor from peep or destroying network 3. Different security domains may exist in same co-channel (e.g. neighboring homes) | 1.Omni-antenna 2. Some mesh points have restricted transmission power 3. Some mesh points need to conserve power 4. Some mesh points do not forward packets due to need to doze to save power | Self-configuring | 1. low-cost, non-invasive, convenient deployment (no need to install wires through walls) 3. seamless connectivity within the home 4. give always connected experience to users | Quantitative Measurement Parameters: -overall E-to-E throughput/jitter in an isolated mesh deployment -network maintenance signaling complexity -fairness among co-channel interfering WLAN Meshes -stability of the network -mesh point duty cycle (in the case of power saving mesh points). Qualitative Evaluation Parameters: -Support for mesh point power saving -ability to quickly recover from sleep mode -robustness against aggressor. |
| 2 | Office | Large (32 per 11s mesh) Note: A large enterprise may have a total of 50-100 Mesh AP/Point devices deployed across multiple interconnected Layer2 802.11s meshes. | 1000m2 -8000m2 | Mostly indoor, although may extend to outdoor courtyards, etc. | Mostly fixed, rare mobility. | Low | Medium (5-10 per 11s mesh) | 50-300 (per 11s mesh) |  |  | , 9, 10, 11, 13, 16, 18, 29, 30, 31, 33, 36, 37 | Mostly dedicated Mesh APs/Points, but some client devices (e.g. SoftAP PCs) could also be Mesh APs/Points (most likely in small/medium office) | Yes, although in large enterprise most traffic is expected to be between clients and servers on the intranet. Examples of peer-to-peer traffic within the wireless mesh network include printing, file sharing, connecting to a projector, etc. | A large enterprise will typically require centralized key management, but small offices may require distributed key management (similar to residential). | Most indoor mesh deployments are expected to use omni-directional antennas. Some large enterprise campuses may use directional-antennas for outdoor deployment between buildings (similar to campus network). | Mosted enterprise networks will require managed configuration by an IT department. However, some small/medium offices without a dedicated IT department will require the network to be self-configuring. | Low cost wireless network infrastructure, convenient and non-invasive deployment (reduce wire installation), reliable coverage for productivity and accessibility. | Focus on indoor deployment for simulation scenario, since campus usage model should cover outdoor deployment? |
| 3 | Campus/ Comm-unity/ Public Access Network | Large (32-100) Note: A large community network may have a total of 50-100 Mesh AP/Point devices deployed across multiple interconnected Layer2 802.11s meshes. | 1000 m2 –several square kilometers | Mostly outdoor, although public access model extend to indoor environment. | Mostly fixed, sometimes nomadic mobility. | Low | Medium (2-8 per 11s mesh) The mesh must have >= 2 portals to support scalability to larger mesh hierarchies. Perferably all the APs at the perimiter of the mesh would be mesh portals. | Large (20-1000 per 11s mesh) |  |  | 1, 2, 6, 7, 8, 9, 10, 11, 13, 14, 15, 18, 19, 20, 21, 23, 24, 25, 31 | Typically no, but some client devices could also be Mesh Points (e.g. at a temporary event deployment) | Typically no, although some Mesh Point capable clients may be the destination of the draffic. Note: In additional to traffic that is terminating in the mesh, there may be traffic that is passing through from one portal to another portal. Mechanisms to set priorities and QOS parameters for pass through traffic are required. | Mostly centralized key management. Some of the Mesh Points may participate in multiple security domains. Mesh portals must be able to participate in multiple security domains. For example, in the case of broadband service delivery over mesh networks, the mesh owned and operated by a residence would intersect with the mesh owned and operated by a service provider. Mesh portal devices must participate in both the local mesh and external mesh security domains. | For fixed Mesh Point/AP, directional antenna will be utilized, while some other Mesh Point/AP will utilize omni-directional antenna. | Mostly managed configuration. But some part of the mesh will be operated by self-configuration. | Accessibility, low cost infrastructure, border coverage. Wireless Internet, lower cost infrastructure, scalability. Low cost, reliable coverage. Reduced deployment time and cost, reliable coverage. Low cost, rapid deployment, self-configuring. Point of sales, advertisement, amusement customer attraction. | Focus on outdoor deployment for simulation scenario, since office usage model should cover indoor deployment? |
| 4 | Public Safety | Large (32 per 11s mesh) Note: A large public safety network may have a total of 50-100s of Mesh AP/Point devices deployed across multiple interconnected Layer2 802.11s meshes. | 250m2 – several square kilometers | Mostly outdoor, although may extend to emergency workers inside buildings. | Mix of fixed radios (e.g. on poll tops) and mobile nodes deployed in emergency vehicles and carried by emergency workers. | High | Large (5-20 per 11s mesh) | 30-250 |  |  | 1, 7, 8, 9, 11, 13, 14, 15, 37, 41, 42 | Yes, communication terminals carried by vehicles and personnel could also Mesh APs/Points | Yes | It should be possible to use either centralized or distributed key management, depending on the sitution. In a small incident scene, may have hierarchically organized user security, while a large incident scene may require the coexistence of several organizations with disjoint management. | Most mobile nodes are expected to use omni-directional antennas, but some dedicated backhaul nodes (e.g. on lamp posts) may use directional antennas for increased range and reliability. | Most public safety networks must be self-configuring to allow for rapid deployment in an emergency situation. | Zero infrastructure, low-cost and rapid deployement, improved range and reliability, improved battery life. | It should be possible for a mesh portal to connect the mesh network to the Internet over a cellular connection. |
| 5 | Military | Large (32 per 11s mesh) Note: A large military network may have a total of 50-100s of Mesh AP/Point devices deployed across multiple interconnected Layer2 802.11s meshes. | 250m2 – several square kilometers | Mostly outdoor, although may extend to military personnel inside buildings, tunnels, ships, etc. | May have a few stationary radios (e.g. at a combat operations center or temporarily placed on building tops, utility poles, etc.) and mobile nodes deployed in combat vehicles, ships or carried by dismounted troops. | High (e.g. pedestrian speeds, or slow vehicle speeds) | Large (5-20 per 11s mesh) | 30-250 |  |  | 1, 7, 8, 9, 11, 13, 14, 15, 37, 41, 42 | Yes, communication terminals carried by military vehicles and personnel could also be Mesh APs/Points. | Yes | It should be possible to use either centralized or distributed key management, depending on the situation. Operations involving a single command structure may have hierarchically organized user security, while large, joint combat operations may require the coexistence of several command structures with disjoint management. | Most mobile nodes are expected to use omni-directional antennas, but some advantaged nodes may use directional antennas for increased range, covertness, and reliability. | Most military networks must be self-configuring to allow for rapid deployment in a combat situation. | Zero infrastructure, low-cost and rapid deployment, improved range and reliability, improved battery life. | It should be possible for a mesh portal to connect the mesh network to a large network over long-range, non-802.11 military RF links or networks. |