IEEE P802.11
Wireless LANs

Proposed 802.11 TGs Scope

Date:	25 July 2004

Authors:

| Name | Company | Email | Contact Info |
| --- | --- | --- | --- |
| Tricci So | Nortel Networks | tso@nortelnetworks.com | 3500 Carling Ave., Ottawa, ON, K2H 8E9, Canada |

Abstract

This document recommends the scope as one of the activities to be undertaken by 802.11 TGs ESS Mesh Network Architecture Framework Ad Hoc team.

Table of Contents

1	Document Version History	4

2	Working Notes and Issues	4

3	Introduction	4

4	TGs Process	4

5	Key IEEE Standard Requirements Extracted from Five Criteria	5

5.1	Compatibility	5

5.2	Distinct Identity	5

5.3	Technical Feasibility	5

5.4	Economic Feasibility	6

6	802.11 TGs PAR Summary	6

6.1	Scope of Proposed Project	6

6.2	Purpose of Proposed Project	6

6.3	Additional Explanatory Notes	6

7	Key Highlevel Requirements Derived from PAR and Five Criteria	6

8	Areas of System Functions Derived from PAR and Five Criteria	9

9	Proposed Scope Based on System Functional Areas for ESS Mesh Standard Development	9

9.1	Mesh Components	11

9.1.1	PHYs/MACs	11

9.1.2	Mesh Points Functions and Supported Services	12

9.1.3	STA or Mesh STA	13

9.2	System Functions	13

9.2.1	System and Interworking Considerations	13

9.2.2	System Performance Considerations	14

9.2.3	Mesh Topologies, Routing and Forwarding	15

10	Standards Implications	17

10.1	Task Group Co-ordination	17

10.1.1	Existing Standards	18

11	Use Cases	18

12	Conclusion	18

List of Tables

Table 1: Key Highlevel Technical Requirements from PAR and Five Criteria	7

Table 2: Key Highlevel Marketing Requirements from PAR and Five Criteria	8

Table 3: Mandatory System Functional Areas According to TGs PAR and Five Criteria	9

# Document Version History

11-04-0xxx-00-000s-scope (mm/dd/yy)

- First draft document derived from IEEE 802.11 ESS Mesh PAR (11-04/054) and Five Criteria (11-04/056).

- Edited on 7/29/04 by J. Agre

- Edited on 8/1/04 by Tricci So to incorporate J. Agre’s inputs

# Working Notes and Issues

[Editor Note: To Be Updated.]

# Introduction

This document defines the scope of the efforts undertaken by TGs.   It indicates which aspects of standards develop are or are not considered by the TGs effort as follows:

- In-scope – aspects that will be considered, addressed, or affected by TGs

- Out-of-scope – aspects that will not be considered or addressed by TGs

- Unaffected – aspects that may co-exist with TGs, but will be unaffected, or essentially preserved, by TGs

- To-be-determined – aspects which are, as yet, TBD (to-be-determined) with respect to TGs

This document does not define the requirements for TGs, it only indicates which aspects of standards developed will, or will not, be addressed in the TGs requirements document and subsequent TGs documents.

This document is a living document that presents a “best estimate” proposal for the scope of TGs, but which will also evolve and be updated as the TGs effort progresses.

# TGs Process

The generic Task Group process of getting to Letter Ballot in IEEE are as follows (11-04/800r3):

- Adoption of PAR and 5 Criteria

- Technical Presentations and Discussions

- Specify Any Additional Requirements and Comparison Criteria

- Call For Proposals

- Select/Combine from Submitted Complete/Partial Proposals to Produce a Draft

- Refine Draft

- Letter Ballot

Currently, the following process has been used based on the discussion in TGs in July04 Meeting (11-04/800r3):

- Ad Hoc Subgroups

  - Have no special status unless TGs votes on them or their output

  - Any group of 802.11 members can get together and make submissions

- Within the Policies and Procedures (11-04/510r0), TGs can do what TGs want internally:

  - “Adopted” internal TGs motions/documents can be amended by majority with notice (i.e. if change is on agenda) or 2/3 vote without notice.

  - TGs can issue a call for proposals

    - With no requirements other than the PAR & 5 Criteria

    - With some guidelines

    - With detailed requirements

  - We can have “Functional Requirements Document” that is as general or specific as TGs like.

The intent of this document is to propose the following process to develop the draft specification to submit for the Letter Ballot.

- Clarify the scope and  the set of system functions that are based on the PAR and the five criteria of the 802.11 ESS Mesh

- Agree on the interpretation of the PAR and five criteria, and define the list of functional requirements that are within the scope of development

- Identify a mesh-end to mesh-end network architecture which is based on the usage models and network deployment scenarios to define the functional requirements that are within the scope

- Define the Timeline and Milestones and Deliverables

- Define the set of Selection Criteria

- Define the set of Test Plans

Just as important as it is for the TGs to agree on the scope, it is necessary for TGs to clarify and to agree on the set of system functions that are considered as Out-of-scope, Unaffected and To-be-determined so that the members of the TGs can focus to develop proposals that will be evaluated based on the same set of criteria.

# Key IEEE Standard Requirements Extracted from Five Criteria

## Compatibility

IEEE 802 defines a family of standards.  All standards shall be in conformance with the IEEE 802.1 Architecture, Management and Interworking documents as follows: 802. Overview and Architecture, 802.1D, 802.1Q and parts of 802.1f.  If any variances in conformance emerge, they shall be thoroughly disclosed and reviewed with 802.  Each standard in the IEEE 802 family of standards shall include a definition of managed objects, which are compatible with system management standards.

ESS Mesh specifies one possible Wireless Distribution System  (WDS) that behaves in every respect as an IEEE 802.11 Infrastructure Mode network.  As such, it is entirely compatible with the 802.11 architecture and, by inference, compatible with the IEEE 802 architecture, including IEEE 802.1D, IEEE 802.1Q and IEEE 802.1F.

[Editor Note:  The definition of infrastructure according to IEEE 802.11 specification is as follows,

The” infrastructure” includes the distribution system medium (DSM), access point (AP), and portal entities. It is also the logical location of distribution and integration service functions of an extended service set (ESS). An infrastructure contains one or more APs and zero or more portals in addition to the distribution system (DS).]

## Distinct Identity

…Instantaneous radio-awareness is currently out-of-scope for IETF Mobile Ad Hoc Networks (MANET), limiting its ability to make efficient use of the wireless channel, unlike ESS Mesh which allows tight integration between the MAC and the multi-hop WDS.

…These design goals (i.e. of IEEE 802.11f) differ significantly from those of ESS Mesh which primarily concerns itself with topology discovery, delivery, and mobility management for a wireless DS implementation.

…IEEE 802.11 ESS Mesh is the only wireless distribution system (WDS) protocol defined in 802.11.

## Technical Feasibility

…Address mapping information enables roaming, i.e. the ability of the DS to determine the correct output point or points for user traffic, while global topology information is used to compute unicast paths.

## Economic Feasibility

…Considering the cost of IEEE 802.11 APs in the market place, it is a reasonable cost to add a wireless DS, especially in comparison to the cost of a wired DS.

…The proposed amendment has no known impact on installation costs.

# 802.11 TGs PAR Summary

## Scope of Proposed Project

To develop an IEEE 802.11 Extended Service Set (ESS) Mesh* with an IEEE 802.11 Wireless Distribution System (WDS) using the IEEE 802.11 MAC/PHY layers that supports both broadcast/multicast and unicast delivery over self-configuring multi-hop topologies.

## Purpose of Proposed Project

The IEEE 802.11-1999 (2003 edition) standard provides a four-address frame format for exchanging data packets between APs for the purpose of creating a Wireless Distribution System (WDS), but does not define how to configure or use a WDS.  The purpose of the project is to provide a protocol for auto-configuring paths between APs over self-configuring multi-hop topologies in a WDS to support both broadcast/multicast and unicast traffic in an ESS Mesh using the four-address frame format or extensions.

## Additional Explanatory Notes

Scope of the Project: An IEEE 802.11 Extended Sevice Set (ESS) Mesh* is a collection of APs interconnected with wireless links that enable automatic topology learning and dynamic path configuration.

The proposed amendment shall be an extension to the IEEE 802.11 MAC.  The amendment will define an architecture and protocol for providing an IEEE 802.11 ESS Mesh using the IEEE 802.11 MAC to create an IEEE 802.11 Wireless Distribution System that supports both broadcast/multicast and unicast delivery at the MAC layer using radio-aware metrics over self-configuring multi-hop topologies.  An ESS Mesh is functionally equivalent to a wired ESS, with respect to the STAs relationship with the BSS and ESS.

The amendment shall enable interoperable formation and operation of an ESS Mesh, but shall be extensible to allow for alternative path selection metrics and/or protocols based on application requirements.  A target configuration is up to 32 devices participating as AP forwarders in the ESS Mesh.  However, larger configurations may also be contemplated by the standard.   It is intended that the architecture defined by the amendment shall allow an ESS Mesh to interface with higher layers and to connect with other networks using higher layer protocols.

The amendment shall utilize IEEE 802.11i security mechanisms, or an extension thereof, for the purpose of securing an ESS Mesh in which all of the APs are controlled by a single logical administrative entity for security.  The amendment shall allow the use of one or more IEEE 802.11 radios on each AP in the ESS Mesh.

# Key Highlevel Requirements Derived from PAR and Five Criteria

The following table explains the key highlevel requirements, in terms of technical and marketing considerations, that are derived from the IEEE 802.11 TGs PAR and Five Criteria.  The information of this table is used as the basis to derive the scope to be used as the reference to develop the draft 802.11 amendments for ESS Mesh networking specification by TGs.

| Key Highlevel Technical Requirements |  | PAR | 5 Criteria | Reference Details |
| --- | --- | --- | --- | --- |
| T.1 | 802.11 Distribution System Service (DSS) Integration - An ESS Mesh is functionally equivalent to a wired ESS, with respect to the STAs relationship with the BSS and ESS. |  |  | - Refer to 6.2 Compatibility section in 11-04/056 (Five Criteria) - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR) |
| T.2 | Radio-awareness metrics to support Self-configuring and Auto topology discovery for Multi-hop and Alternative-path routing based on application requirements |  |  | - Refer to 6.3.a Distinct Identity section in 11-04/056 (Five Criteria) - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR) |
| T.3 | Support broadcast/multicast and unicast delivery at the MAC layer to the end user via multi-hop forwarding |  |  | - Refer to 12. Scope of Proposed Project, 13. Purpose of the Proposed Project and 18. Additional Explanatory Notes sections in 11-04/054 (PAR) |
| T.4 | Multi-hop mobility management over WDS |  |  | - Refer to 6.3.a Distinct Identity section in 11-04/056 (Five Criteria) |
| T.5 | Addressing based on the use of the four-address frame format or an extension |  |  | - Refer to 6.3.a Distinct Identity section in 11-04/056 (Five Criteria) - Refer to 13. Purpose of Proposed Project section in 11-04/054 (PAR) |
| T.6 | Interfacing with high layer protocols and interworking with other non-802.11 network using high layer protocols |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR). |
| T.7 | Security based on the use of existing 802.11i standard or an extension thereof, for the purpose of securing an ESS Mesh in which all of the APs are controlled by a single logical administrative entity for security. |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR). |
| T.8 | Support the use of one or more IEEE 802.11 radios on each AP in the ESS Mesh. |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR). |
| T.9 | ESS Mesh does not require radio modifications |  |  | - Refer to 6.5.a Economic Feasibility section in 11-04/056 (Five Criteria) |

Table 1: Key Highlevel Technical Requirements from PAR and Five Criteria

| Key Highlevel Marketing Requirements |  | PAR | 5 Criteria | Reference Details |
| --- | --- | --- | --- | --- |
| M.1 | Support an integrated subnet routing/mobility layer with the WLAN architecture |  |  | - Refer to 6.1.a Broad sets of applicability section in 11-04/056 (Five Criteria) |
| M.2 | Using ESS Mesh, coverage within a house, a hospital, a hotel, and airport, a neighbourhood, a campus, etc., may be extended wirelessly and without manual configuration other than setting the SSID and/or AP introduction. |  |  | - Refer to 6.1.a Broad sets of applicability section in 11-04/056 (Five Criteria) |
| M.3 | ESS Mesh improves the versatility and functionality of conventional WLAN APs by extending services wirelessly as an alternative or an addition to wired connections |  |  | - Refer to 6.1.c Broad sets of applicability section in 11-04/056 (Five Criteria) |
| M.4 | Considering the cost of IEEE 802.11 APs in the market place, it is a reasonable cost to add a wireless DS, especially in comparison to the cost of a wired DS. |  |  | - Refer to 6.5.b Economic Feasibility section in 11-04/056 (Five Criteria) |
| M.5 | The proposed amendment has no known impact on installation costs |  |  | - Refer to 6.5.c Economic Feasibility section in 11-04/056 (Five Criteria) |

Table 2: Key Highlevel Marketing Requirements from PAR and Five Criteria

# Areas of System Functions Derived from PAR and Five Criteria

Based on the highlevel requirements that were extracted from previous section, this following table summarizes the list of system functions that IEEE 802.11s ESS Mesh standard MUST be addressed.

| System Functional Areas |  | High Level Requirements References (PAR and Five Criteria) |
| --- | --- | --- |
| A.1 | WDS specific services and WDS and DSS Services Integration | T.1 and M.4 |
| A.2 | RF Resource Control and Management | T.2, T.8 , T.9 and M2 |
| A.3 | Mesh Multipath Support for Multi-hop Routing | T.2 and M.1 |
| A.4 | MAC Layer Multi-path Support for Multi-hop Forwarding | T.3 and M.1 |
| A.5 | Multi-hop Mobility Management over WDS (NOTE: Need to distinguish the functions to support “roaming” vs. “handoff” for mobility management.) | T.4 and M.1 |
| A.6 | MAC Layer WDS Addressing | T.5 and M.1 |
| A.7 | ESS Mesh and High Layer Protocols Service and Network Interworking | T.6 and M3 |
| A.8 | Mesh Points Single Administrative Domain Based Security | T.7 |
| A.9 | Mesh Network Operational, Administration, Control and Measurement (i.e. including accounting and billing) | M.2 |

Table 3: Mandatory System Functional Areas According to TGs PAR and Five Criteria

[Editor Note: Based on what criteria that we should develop the optional System Functional Areas, e.g. QoS support? ]

# Proposed Scope Based on System Functional Areas for ESS Mesh Standard Development

The following sections are to clarify the scope of the ESS Mesh standard development based on the system functional areas that were derived from the previous section.   The architectural and system components are organized in term of their roles and definitions as described in the “Terms and Definitions” (see 11-04/0730r3 and 11-04/0822r1 for details) and their applicability to the scope and five criteria.   Each of these components is examined with respect to its relative impact to the system functional areas that were discussed earlier.  The results are captured in a form of table under the categories of

- In-scope

- Out-of-scope

- Un-affected

- To-be-determined

## Mesh Components

### PHYs/MACs

The following captures the scope of various PHYs/MACs extensions that may be addressed by TGs are as follows:

| Component Applicability |  | Clarifications | System Functional Areas Mapping |
| --- | --- | --- | --- |
| In-scope |  |  |  |
|  | 802.11 / 802.11e MAC | - To enable MPs communications with each other for more than one neighbor in infrastructure mode [Editor Note: Need to coordinate the MAC extension effort with 802.11n.] | - Refer to system functional areas A.1, A.4, A.5 and A.6 |
| Unaffected |  |  |  |
|  | 802.11 a, b, g PHY | - Change to the existing 802.11 PHY is out of the scope. [Editor Note: Need to consider the co-existence with other IEEE standards for the same unlicensed spectrum. ] | - Refer to key highlevel technical requirement T.9 |
| To-be-determined |  |  |  |
|  |  | - Passive/Active Scanning mechanism |  |
| Out-of-scope |  |  |  |
|  | Other PHY and MAC | - 802.16 - 802.15 - 802.3 - Cellular |  |

### Mesh Points Functions and Supported Services

Mesh Point (MP) is any IEEE 802.11 entity that contains an IEEE 802.11 – conformant Medium Access Control (MAC) and Physical Layer (PHY) interface to Wireless Medium (WM), is within a WLAN Mesh, and supports WLAN Mesh Services.   The functions of the MP that are addressed by TGs are as follows.

| Mesh Points Functions and Supported Services |  | Clarifications | System Functional Areas Mapping |
| --- | --- | --- | --- |
| In-scope |  |  |  |
|  | Mesh Neighborhood Formation | For a WLAN Mesh that is identified by a Mesh ID within the same ESS (i.e. same SSID), - Support MP Neighbors Auto-discovery - MP-to-MP authentication and secured information exchange and frame transport - Establish peer-to-peer MP adjacency - Exchange mesh topology information - Coordinate the Tx/Rx time slot over the RF channel between the peers | - A.1, A.4, A.5 and A.6 |
|  | MP Services for WLAN Mesh | - Support unicast , multicast and broadcast data delivery to the associated STAs - Support location service for STA - Support WDS and DSS service integration - MAP-to-STA, MP-to-MP and MP-to-AS context establishment and exchange to support control and management services for WLAN Mesh - Continue to support of security context (TGi) - Continue to maintain MAC QoS context (TGe) - Continue to maintain the management context to support roaming and mobility - Propose a new context (e.g. security, QoS, network configuration, management and operational parameters, routing metrics etc.) between MP-to-MP with or without off-broad server support | - A.1 |
|  | Accounting and Billing | - Continue to support 802.11 network accounting and billing | - A.9 |
|  | RF Resource Control & Manageemnt | To support Auto-configuration - Calibration - Auto adjust transmit power levels - Auto channel selection/assignment to minimize interference and to maximize coverage and throughput - Optimization - Optimize radio resource allocation to adapt to RF environment changes - Self healing due to MP failure or severe RF interference - Load Balancing across multiple MP neighbors and reject STA associations attempt due to local traffic overloading condition - Link quality monitoring on bandwidth rates, frame error rates, frame fragmentation rates, frame receive rate and retry rate etc. | - A.2 |
|  | Control and Management Messaging and Format | - Mesh Management Message - Mesh Unicast Acknowledgement - MAC layer WDS Addressing frame format or extensions | - A2, A3, A5, A.6, A.8 and A.9 |
| Unaffected |  |  |  |
|  | Legacy 802.11 AP | - No new function introduces to the legacy 802.11 AP | - T.1 and T.9 |
| To-be-determined |  |  | - T.8 |
|  | Reference MP‘s logical configuration | - Single vs. multiple radios would have significant impact to the performance and functional requirements for a MP. |  |
|  | Power Saving |  |  |
| Out-of-scope |  |  |  |

### STA or Mesh STA

There should be no change required for the legacy STA as TGs PAR explicitely states that “An ESS Mesh is functionally equivalent to a wired ESS, with respect to the STAs relationship with the BSS and ESS.”.

[Editor Note: Do we need to consider the Mesh Aware STA?  Also, how about the consideration of a device that is functioning as a MP and a STA? ]

## System Functions

### System and Interworking Considerations

The system and  interworking  considerations that will be addressed by TGs are as follows:

| System and Interworking Considerations |  | Clarifications | System Functional Areas Mapping |
| --- | --- | --- | --- |
| In-scope |  |  |  |
|  | Backward Compatibility | - To interwork with the existing legacy wired Distribution System (DS) which supports the legacy APs and STAs - Co-existence consideration – RF coverage, DSS, roaming and mobility | - A.1 |
|  | Interfaces to other 802, layer-3 networks | - To recommend a reference inteworking architecture to allow the WLAN Mesh to interface with other non-802.11 802 and layer-3 networks. - To define the Mesh Portal requirements that can interconnect the non-802.11 network with the WLAN Mesh network | - A.7 |
| Unaffected |  |  |  |
| To-be-determined |  |  |  |
| Out-of-scope |  |  |  |

### System Performance Considerations

To support the given usage models and to enable the realistic deployment scenario and configuration,  system performance of WLAN Mesh for a network as a whole is an important aspect that will be  addressed by TGs .

| System Performance Considerations |  | Clarifications | System Functional Areas Mapping |
| --- | --- | --- | --- |
| In-scope |  |  |  |
|  | WLAN Mesh Performance Management | - Depending on the usage scenarios, the MP physical and WLAN Mesh system configuration, the aggregated WLAN Mesh Wireless Distribution System throughput over the wireless transit link(s) should be capable of sustaining the aggregated 802.11 mobile user access throughput for the given the WLAN Mesh. - RF interference mitigation (e.g. Hidden node, Exposed Node prevention) through intelligence algorithm of dynamic channel selection, antenna selection and tx power control etc. - Mesh link quality (e.g. effective bandwidth) and latency over multi-hop topology - AP initiated load balancing across the WLAN Mesh - STAs mobility performance over WLAN Mesh - Management frame prioritization - MP-MP and MP-STA security performance - WLAN Mesh topology convergence time - Routing protocol overhead - WLAN Mesh control and management overhead - Set of system metrics, path metrics and link metrics to be monitored and collected for WLAN Mesh | - A.9 |
|  | WLAN Mesh network reference topology | - To provide one or more reference WLAN mesh network configurations to be used for system performance analysis and evaluation criteria - To define the set of possible DS integration configurations for the WLAN Mesh and the existing wired DS. | - A.9 |
| Unaffected |  |  |  |
|  | Wired DS and BSS |  |  |
| To-be-determined |  |  |  |
|  | Mesh System Constraints | - Identify the System design constraints |  |
|  | Relationship with other non-mesh 802.11 networks | - Site reports, Management frames, etc. |  |
| Out-of-scope |  |  |  |
|  | Heterogeneous 802.11 network | - Mesh configuration with the mix of legacy and Mesh Points - Mix of wired and wireless DS |  |

### Mesh Topologies, Routing and Forwarding

Mesh Topology aspects that are to be addressed by TGs are as follows:

| Mesh Topology, Routing and Forwarding |  | Clarifications | System Functional Areas Mapping |
| --- | --- | --- | --- |
| In-scope |  |  |  |
|  | WLAN Mesh Discovery | - Secured WLAN Mesh discovery and selection by a MP with the assumption that the WLAN Mesh is operating under a single administrative domain. - Clarification of Mesh IDs and BSSIDs, SSIDs relationships | - A.1 |
|  | WLAN Mesh Topology Formation, Convergence, and Change | - Mesh membership – i.e. MP’s authenticity, MP’s operational status (e.g. wireless link condition) - Mesh topology condition – i.e. connected Mesh, disconnected Mesh - Unicast vs. Multicast topology relative to the WLAN Mesh and relative to the STAs associated with the Mesh APs - Identifying and avoiding pathological failures scenarios - Optimized multipath topology formation (i.e. maximum disjoint paths, minimum overlapping paths) - Mesh topology considerations and limitations for the minimum or greater than 32 MPs and 256 STAs | - A.3, A.4 |
|  | Routing and Forwarding | - Routing vs. Forwarding – i.e. routing decision that is used to form the network topology should be separated from the frame forwarding decision.that is based on the convergence view of the network topology and the nodal performance. - Unicast vs. multicast forwarding over multi-hop - Multipath support for multi-hop forwarding - MAC layer WDS Framing and extensions - Mesh-end to mesh-end flow and congestion control | - A.3, A.4 |
| Unaffected |  |  |  |
| To-be-determined |  |  |  |
|  | WLAN Mesh and Spanning Tree interworking over the DS | - Spanning Tree interworking with wired DS |  |
|  | WLAN Mesh Spanning Tree extensions | - Spanning Tree extensions to be supported over the WDS |  |
|  | VLAN Mapping | - VLAN over the WDS |  |
| Out-of-scope |  |  |  |

# Standards Implications

## Task Group Co-ordination

Task Group co-ordination efforts addressed by TGs are as follows:

- In-scope

  - 802.11r

  - 802.11i

    - Key distribution

  - 802.11e

    - Direct links

  - 802.21

  - TGk

  - Future 802.11 PHYs/MACs/etc should be defined to be compatible with TGs

- Out-of-scope

- Unaffected

- To-be-determined

  - WEIN

  - WNM

  - 802.15.5 MESH

  - WPP

### Existing Standards

Existing standards implications addressed by TGs are as follows:

- In-scope

  - Recommended practices and/or modifications to existing base standards wrt Mesh Services

  - Identification of existing standards that are preserved/unaffected by TGs

  - Indentification of existing standards that TGs interoperates with

- Out-of-scope

- Unaffected

  - <This is where we list standards that are preserved or unaffected by TGs>

  - <This is where we list existing standards that TGs will interoperate with >

- To-be-determined

  - 802.11f?

# Use Cases

Use case aspects addressed by TGs are as follows:

- In-scope

  - Use case definitions

    - Office, Home, Public Safety, Community

  - Applications and application requirements

  - Traffic patterns

- Out-of-scope

  - Automobile (high mobility) use cases

  - Low pwoere sensor networks

- Unaffected

- To-be-determined

# Conclusion