IEEE P802.11
Wireless LANs

Proposed 802.11 TGs Scope

Date:	             24 August 2004

Authors:

| Name | Company | Email | Contact Info |
| --- | --- | --- | --- |
| Tricci So | Nortel Networks | tso@nortelnetworks.com | 3500 Carling Ave., Ottawa, ON, K2H 8E9, Canada |
| Jonathan Agre | Fujitsu | jagre@fla.fujitsu.com |  |
| Ambatipudi Sastry | Packethop | sastry@packethop.com |  |
| Kazuyuki Sakoda | Sony | sako@wcs.sony.co.jp |  |
| Malik Audeh | Tropos Networks | malik@audeh@tropos.com |  |
| Narasimha Chari | Tropos Networks | chari@tropos.com |  |
| Vann Hastry | Mesh Networks | vhastry@meshnetworks.com |  |
| Sebnem Z. Ozer | Mesh Networks | sozer@meshnetworks.com |  |
| Juan Carlos Zuniga | Interdigital | Juancarlos.zuniga@interdigital.com |  |
| Stephan Rayment | BelAir Networks | srayment@belairnetworks.com |  |
| Lily Yang | Intel | Lily.l.yang@intel.com |  |
| W. Steven Conner | Intel | w.steven.conner@intel.com |  |
| Benoit Fremont | France Telecom | Benoit.Fremont@francetelecom.com |  |
| Veronique Bayle | France Telecom | Vero.bayle@francetelecom.com |  |
| Avinash Joshi | Mesh Networks | ajoshi@meshnetworks.com |  |

Abstract

This document proposes the scope as one of the activities to be undertaken by 802.11 TGs ESS Mesh Network Architecture Framework Ad Hoc team.

Table of Contents

1	Document Version History	3

2	Working Notes and Issues	3

3	Introduction	3

4	TGs Process	3

5	802.11 TGs PAR Summary	4

5.1	Scope of Proposed Project	4

5.2	Purpose of Proposed Project	4

5.3	Additional Explanatory Notes	4

6	Key Highlevel Requirements Derived from PAR and Others	5

7	Areas of System Functions Derived from PAR	6

8	Proposed Scope Based on System Functional Areas for ESS Mesh Standard Development	7

8.1	<Functional Area>	8

9	Conclusion	8

APPENDIX- A	9

A. 1 Key IEEE Standard Requirements Extracted from Five Criteria	9

A.1. 1 Compatibility	9

A.1. 2 Distinct Identity	9

A.1. 3 Technical Feasibility	9

A.1. 4 Economic Feasibility	9

A.1. 5 Marketing Requirments Considerations Based on Five Criteria	9

List of Figures

Figure 1:  Scope Considerations by 802.11 TGs	6

Figure 2: Highlevel view on the System Functions Interactions	7

List of Tables

Table 1: Key Highlevel Technical Requirements from PAR and cross reference to Five Criteria	6

Table 2: Additional Highlevel Technical Requirements NOT Directly from the PAR	6

Table 3: Mandatory System Functional Areas According to TGs PAR	7

Table 4: Key Highlevel Marketing Requirements Considerations from Five Criteria	10

# Document Version History

11-04-0970-00-000s-scope (08/24/04)

- First draft document generated by the TGs Network Architecture Framework Ad Hoc group to derive the highlevel requirements from IEEE 802.11 ESS Mesh PAR (11-04/054) and the key functional areas.

# Working Notes and Issues

Editor Note: Next step of this document is to clarify the scope in each of these functional areas.

# Introduction

This document defines the scope of the efforts undertaken by TGs.   It indicates which aspects of standards develop are or are not considered by the TGs effort as follows:

- In-scope – aspects that will be considered, addressed, or affected by TGs

- Out-of-scope – aspects that will not be considered or addressed by TGs

- Unaffected – aspects that may co-exist with TGs, but will be unaffected, or essentially preserved, by TGs

- To-be-determined – aspects which are, as yet, TBD (to-be-determined) with respect to TGs

This document does not define the requirements for TGs, it only indicates which aspects of standards developed will, or will not, be addressed in the TGs requirements document and subsequent TGs documents.

This document is a living document that presents a “best estimate” proposal for the scope of TGs, but which will also evolve and be updated as the TGs effort progresses.

# Process Considerations To Clarify the Scope of TGs

The generic Task Group process of getting to Letter Ballot in IEEE are as follows (11-04/800r3):

- Adoption of PAR and 5 Criteria

- Technical Presentations and Discussions

- Specify Any Additional Requirements and Comparison Criteria

- Call For Proposals

- Select/Combine from Submitted Complete/Partial Proposals to Produce a Draft

- Refine Draft

- Letter Ballot

Currently, the following process has been used based on the discussion in TGs in July04 Meeting (11-04/800r3):

- Ad Hoc Subgroups

  - Have no special status unless TGs votes on them or their output

  - Any group of 802.11 members can get together and make submissions

- Within the Policies and Procedures (11-04/510r0), TGs can do what TGs want internally:

  - “Adopted” internal TGs motions/documents can be amended by majority with notice (i.e. if change is on agenda) or 2/3 vote without notice.

  - TGs can issue a call for proposals

    - With no requirements other than the PAR

    - With some guidelines

    - With detailed requirements

  - We can have “Functional Requirements Document” that is as general or specific as TGs like.

The intent of this document is to propose the following process to develop the draft specification to submit for the Letter Ballot.

The most important objectives of this document are to:

- Clarify the scope and  the set of system functions that are based on the PAR of the 802.11 ESS Mesh

- Agree on the interpretation of the PAR, and define the list of functional requirements that are within the scope of development

Once the common understanding on the scope is established among the TGs members, the network architecture ad-hoc group should then work together with TGs members to:

- Identify a mesh-end to mesh-end network architecture which is based on the usage models and network deployment scenarios to define the functional requirements that are within the scope

- Define the set of Selection and Comparison Criteria on proposals

- Define the set of PICs Areas to verify the feasibility of final recommendations

The results of these activities should produce the following set of documents from this ad-hoc team:

- High-level TGs Network Architecture Framework and  Reference Model and detailed system requirements

- Comparision and Selection Criteria on TGs proposals

- PICs Areas

Just as important as it is for the TGs to agree on the scope, it is necessary for TGs to clarify and to agree on the set of system functions that are considered as Out-of-scope, Unaffected and To-be-determined so that the members of the TGs can focus to develop proposals that will be evaluated based on the same set of criteria.

# 802.11 TGs PAR Summary

## Scope of Proposed Project

To develop an IEEE 802.11 Extended Service Set (ESS) Mesh* with an IEEE 802.11 Wireless Distribution System (WDS) using the IEEE 802.11 MAC/PHY layers that supports both broadcast/multicast and unicast delivery over self-configuring multi-hop topologies.

## Purpose of Proposed Project

The IEEE 802.11-1999 (2003 edition) standard provides a four-address frame format for exchanging data packets between APs for the purpose of creating a Wireless Distribution System (WDS), but does not define how to configure or use a WDS.  The purpose of the project is to provide a protocol for auto-configuring paths between APs over self-configuring multi-hop topologies in a WDS to support both broadcast/multicast and unicast traffic in an ESS Mesh using the four-address frame format or extensions.

## Additional Explanatory Notes

Scope of the Project: An IEEE 802.11 Extended Sevice Set (ESS) Mesh* is a collection of APs interconnected with wireless links that enable automatic topology learning and dynamic path configuration.

The proposed amendment shall be an extension to the IEEE 802.11 MAC.  The amendment will define an architecture and protocol for providing an IEEE 802.11 ESS Mesh using the IEEE 802.11 MAC to create an IEEE 802.11 Wireless Distribution System that supports both broadcast/multicast and unicast delivery at the MAC layer using radio-aware metrics over self-configuring multi-hop topologies.  An ESS Mesh is functionally equivalent to a wired ESS, with respect to the STAs relationship with the BSS and ESS.

The amendment shall enable interoperable formation and operation of an ESS Mesh, but shall be extensible to allow for alternative path selection metrics and/or protocols based on application requirements.  A target configuration is up to 32 devices participating as AP forwarders in the ESS Mesh.  However, larger configurations may also be contemplated by the standard.   It is intended that the architecture defined by the amendment shall allow an ESS Mesh to interface with higher layers and to connect with other networks using higher layer protocols.

The amendment shall utilize IEEE 802.11i security mechanisms, or an extension thereof, for the purpose of securing an ESS Mesh in which all of the APs are controlled by a single logical administrative entity for security.  The amendment shall allow the use of one or more IEEE 802.11 radios on each AP in the ESS Mesh.

# Key Highlevel Requirements Derived from PAR and Others

The following table explains the key highlevel requirements of the TGs scope, in terms of technical and other considerations, that are derived from the IEEE 802.11 TGs PAR and also cross reference to the Five Criteria as well as to the 802.11 standards.  The information of these tables are used as the basis to derive the key highlevel technical requirements of the scope that is used to develop the draft 802.11 amendments for ESS Mesh networking specification by TGs.   The figure below is to capture the considerations of the scope that needs to be addressed by the TGs.

| Key Highlevel Technical Requirements |  | PAR | 5 Criteria | Reference Details |
| --- | --- | --- | --- | --- |
| T.1 | Compatibility with 802.11 Distribution System Service (DSS) - An ESS Mesh is functionally equivalent to a wired ESS, with respect to the STAs relationship with the BSS and ESS. |  |  | - Refer to 6.2 Compatibility section in 11-04/056 (Five Criteria) - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR) |
| T.2 | Enable automatic topology learning and dynamic path configuration to support self-configuring multihop topology. - The amendment shall be extensible to allow for alternative path selection metrics and/or protocols based on application requirements |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR) |
| T.3 | Radio-awareness metrics to support self-configuring topology discovery and multi-hop routing. |  |  | - Refer to 6.3.a Distinct Identity section in 11-04/056 (Five Criteria) - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR) |
| T.4 | Support broadcast/multicast and unicast delivery at the MAC layer to the end user via multi-hop forwarding |  |  | - Refer to 12. Scope of Proposed Project, 13. Purpose of the Proposed Project and 18. Additional Explanatory Notes sections in 11-04/054 (PAR) |
| T.5 | Addressing based on the use of the four-address frame format or an extension |  |  | - Refer to 6.3.a Distinct Identity section in 11-04/056 (Five Criteria) - Refer to 13. Purpose of Proposed Project section in 11-04/054 (PAR) |
| T.6 | Interfacing with high layer protocols and interworking with other non-802.11 network using high layer protocols |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR). |
| T.7 | Security based on the use of existing 802.11i standard or an extension thereof, for the purpose of securing an ESS Mesh in which all of the APs are controlled by a single logical administrative entity for security. |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR). |
| T.8 | Support the use of one or more IEEE 802.11 radios on each AP in the ESS Mesh. |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR). |
| T.9 | ESS Mesh is an extension to the IEEE 802.11 MAC and does not require PHY changes |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR). |

Table 1: Key Highlevel Technical Requirements from PAR and cross reference to Five Criteria

| Key Highlevel Requirements Outside of PAR |  | Justification |
| --- | --- | --- |
| O.1 | Mesh Network STA Mobility Support | - Ensure the mobility service integration is compatible to the existing 802.11 architecture. |
| O.2 | Support both the managed and unmanged network management models | - Consider the network management requirements to address the non-home networking products. |

Table 2: Additional Highlevel Technical Requirements NOT Directly from the PAR

Figure 1:  Scope Considerations by 802.11 TGs

# Areas of System Functions Derived from PAR

Based on the highlevel requirements that were extracted from previous section, this following table summarizes the list of system functions that IEEE 802.11s ESS Mesh standard MUST address.

| System Functional Areas |  | References to High Level Requirements based on Section 6 |
| --- | --- | --- |
| A.1 | Compatibility to 802.11 Distribution System Services (DSS) | T.1. T.9 |
| A.2 | Mesh RF Resource Control and Management | T.3, T.8 |
| A.3 | Mesh Routing & Forwarding | T.2. T4. T.9 |
| A.4 | Mesh Security | T.7 |
| A.5 | Mesh Measurement | T.9 |
| A.6 | Mesh Interworking | T.6 |
| A.7 | Mesh Auto-discovery and topology learning | T.2, T.9 |
| A.8 | Mesh Medium Access Coordination and Flow Control | T.2, T.3, T.8, T.9 |

Table 3: Mandatory System Functional Areas According to TGs PAR

Editor Note: Based on what criteria that we should develop the optional System Functional Areas, e.g. QoS support?

The following figure captures the highlevel view of these system functions and their possible dependency and interactions.

Figure 2: Highlevel view on the System Functions Interactions

# Proposed Scope Based on System Functional Areas for ESS Mesh Standard Development

The following sections are to clarify the scope of the ESS Mesh standard development based on the system functional areas that were derived from the previous section.   The architectural and system components are organized in term of their roles and definitions as described in the “Terms and Definitions” (see 11-04/0730r3 and 11-04/0822r1 for details) and their applicability to the scope and five criteria.   Each of these components is examined with respect to its relative impact to the system functional areas that were discussed earlier.  The results are captured in a form of table under the categories of

- In-scope

- Out-of-scope

- Un-affected

- To-be-determined

## <Functional Area>

The following captures the scope of 802.11 WDS service integration that may be addressed by TGs:

| Component Applicability |  | Clarifications | System Functional Areas Mapping |
| --- | --- | --- | --- |
| In-scope |  |  |  |
|  | <Functional Area> |  |  |
| Unaffected |  |  |  |
| To-be-determined |  |  |  |
| Out-of-scope |  |  |  |

# Conclusion

APPENDIX-

A. 1 Key IEEE Standard Requirements Extracted from Five Criteria

# A.1. 1 Compatibility

IEEE 802 defines a family of standards.  All standards shall be in conformance with the IEEE 802.1 Architecture, Management and Interworking documents as follows: 802. Overview and Architecture, 802.1D, 802.1Q and parts of 802.1f.  If any variances in conformance emerge, they shall be thoroughly disclosed and reviewed with 802.  Each standard in the IEEE 802 family of standards shall include a definition of managed objects, which are compatible with system management standards.

ESS Mesh specifies one possible Wireless Distribution System  (WDS) that behaves in every respect as an IEEE 802.11 Infrastructure Mode network.  As such, it is entirely compatible with the 802.11 architecture and, by inference, compatible with the IEEE 802 architecture, including IEEE 802.1D, IEEE 802.1Q and IEEE 802.1F.

[Editor Note:  The definition of infrastructure according to IEEE 802.11 specification is as follows,

The” infrastructure” includes the distribution system medium (DSM), access point (AP), and portal entities. It is also the logical location of distribution and integration service functions of an extended service set (ESS). An infrastructure contains one or more APs and zero or more portals in addition to the distribution system (DS).]

# A.1. 2 Distinct Identity

…Instantaneous radio-awareness is currently out-of-scope for IETF Mobile Ad Hoc Networks (MANET), limiting its ability to make efficient use of the wireless channel, unlike ESS Mesh which allows tight integration between the MAC and the multi-hop WDS.

…These design goals (i.e. of IEEE 802.11f) differ significantly from those of ESS Mesh which primarily concerns itself with topology discovery, delivery, and mobility management for a wireless DS implementation.

…IEEE 802.11 ESS Mesh is the only wireless distribution system (WDS) protocol defined in 802.11.

# A.1. 3 Technical Feasibility

…Address mapping information enables roaming, i.e. the ability of the DS to determine the correct output point or points for user traffic, while global topology information is used to compute unicast paths.

# A.1. 4 Economic Feasibility

…Considering the cost of IEEE 802.11 APs in the market place, it is a reasonable cost to add a wireless DS, especially in comparison to the cost of a wired DS.

…The proposed amendment has no known impact on installation costs.

# A.1. 5 Marketing Requirments Considerations Based on Five Criteria

| Key Highlevel Marketing Requirements |  | Reference Details |
| --- | --- | --- |
| M.1 | Support an integrated subnet routing/mobility layer with the WLAN architecture | - Refer to 6.1.a Broad sets of applicability section in 11-04/056 (Five Criteria) |
| M.2 | Using ESS Mesh, coverage within a house, a hospital, a hotel, and airport, a neighbourhood, a campus, etc., may be extended wirelessly and without manual configuration other than setting the SSID and/or AP introduction. | - Refer to 6.1.a Broad sets of applicability section in 11-04/056 (Five Criteria) |
| M.3 | ESS Mesh improves the versatility and functionality of conventional WLAN APs by extending services wirelessly as an alternative or an addition to wired connections | - Refer to 6.1.c Broad sets of applicability section in 11-04/056 (Five Criteria) |
| M.4 | Considering the cost of IEEE 802.11 APs in the market place, it is a reasonable cost to add a wireless DS, especially in comparison to the cost of a wired DS. | - Refer to 6.5.b Economic Feasibility section in 11-04/056 (Five Criteria) |
| M.5 | The proposed amendment has no known impact on installation costs | - Refer to 6.5.c Economic Feasibility section in 11-04/056 (Five Criteria) |

Table 4: Key Highlevel Marketing Requirements Considerations from Five Criteria