IEEE P802.11
Wireless LANs

Suggestions and Comments for 802.11s Functional Requirements and Scope Document

Date:	October 11, 2004

Author:	W. Steven Conner
	Intel Corporation
	2111 NE 25th Ave, Hillsboro, OR 97124, USA
	Phone: +1 503 264 8036
	e-Mail: w.steven.conner@intel.com

Abstract

This document provides suggestions and comments for evolving document 11-04/0970r4 into the Functional Requirements and Scope document for 802.11s.  The remainder of this document is based on document 11-04/0970r4 and includes redline edits for proposed discussion during the October 13, 2004 Task Group s teleconference.

IEEE P802.11
Wireless LANs

Draft 802.11 TGs Functional Requirements and Scope

Date:	October 11, 2004

Authors:

| Name | Company | Email / Phone | Contact Info |
| --- | --- | --- | --- |
| Tricci So | Nortel Networks | tso@nortelnetworks.com tel: +613-763-9639 | 3500 Carling Ave., Ottawa, ON, K2H 8E9, Canada |
| Jonathan Agre | Fujitsu | jagre@fla.fujitsu.com tel: +301-486-0978 | 8400 Baltimore Ave., Suite 302, College Park, MD 20740, USA |
| Ambatipudi Sastry | Packethop | sastry@packethop.com tel: +650-292-5003 | 1301 Shoreway Rd Belmont, CA94002 |
| Kazuyuki Sakoda | Sony | sako@wcs.sony.co.jp tel: +81-3-6409-3201 | Oval Court Ohsaki MW 2-17-1 Higashigotanda Shinagawa-ku Tokyo 141-0022 Japan |
| Malik Audeh | Tropos Networks | malik@audeh@tropos.com tel: +408-331-6814 | 555 Del Rey Ave., Sunnyvale, CA 94085 |
| Narasimha Chari | Tropos Networks | chari@tropos.com tel: +408-331-6814 | 555 Del Rey Ave., Sunnyvale, CA 94085 |
| Vann Hastry | Mesh Networks | vhastry@meshnetworks.com tel: +407-659-5371 | 485 N. Keller Rd. Suite 250, Maitland, FL 32771 |
| Sebnem Z. Ozer | Mesh Networks | sozer@meshnetworks.com tel: +407-659-5391 | 485 N. Keller Rd. Suite 250, Maitland, FL 32771 |
| Juan Carlos Zuniga | Interdigital | Juancarlos.zuniga@interdigital.com tel: +514-904-6251 | 1000 Sherbrooke W., 10th Fl., Montreal QC, H3A 3G4, Canada |
| Stephan Rayment | BelAir Networks | srayment@belairnetworks.com tel: +613-254-7070 | 603 March Road, Kanata, Ontario Canada, K2K 2M5 |
| Lily Yang | Intel | Lily.l.yang@intel.com tel: +503-264-8813 | 2111 NE 25th Ave. Hillsboro, OR 97124 U.S.A. |
| W. Steven Conner | Intel | w.steven.conner@intel.com tel: +503-264-8036 | 2111 NE 25th Ave. Hillsboro, OR 97124 U.S.A. |
| Avinash Joshi | Mesh Networks | ajoshi@meshnetworks.com tel: | 485 N. Keller Rd. Suite 250, Maitland, FL 32771 |
| Guido R. Hiertz | Aachen University | hiertz@ieee.org tel: +49-241-80-25-82-9 | Kopernikusstr. 16 52064 Aachen Germany |

Abstract

This document specifies the minimum functional requirements and scope for ESS Mesh Networking mechanisms to be defined by 802.11 TGs.

Table of Contents

Document Version History	4

1	Introduction	4

1.1	Document Purpose	4

2	Categories for Organization of Functional Requirements and Scope	4

3	Minimum Functional Requirements	4

4	Scope Clarification Organized by Functional Categories for TGs	6

4.1	Mesh Topology Learning, Routing, and Forwarding (TOPO_RT_FWD)	6

4.2	Mesh Security (SECURITY)	7

4.3	Mesh Measurement (MEAS)	8

4.4	Mesh Discovery and Association (DISC_ASSOC)	8

4.5	Mesh Medium Access Coordination (MAC)	9

Compatibility to	10

4.6	802.11 Distributoin System Services (DSS_CMP)	10

4.7	Mesh Interworking (INTRWRK)	10

4.8	Mesh RF Resource Control and Management (RFCTL_MGMT)	11

5	References	11

APPENDIX- A	13

A.1 Key Highlevel Requirements Derived from PAR and Others	13

A.2. 1 Compatibility	14

A.2. 2 Distinct Identity	15

A.2. 3 Technical Feasibility	15

A.2. 4 Economic Feasibility	15

A.2. 5 Marketing Requirments Considerations Based on Five Criteria	15

List of Figures

Figure 1:  Scope Considerations by 802.11 TGs	14

List of Tables

Table 1: Functional Categories for TGs	4

Table 2: Minimum Functional Requirements for TGs	6

Table 3: Key Highlevel Technical Requirements from PAR and cross reference to Five Criteria	13

Table 4: Additional Highlevel Technical Requirements NOT Directly from the PAR	14

Table 5: Key Highlevel Marketing Requirements Considerations from Five Criteria	15

# Document Version History

11-04-0970-00-000s-scope (08/24/04)

- First draft document generated by the TGs Network Architecture Framework Ad Hoc group to derive the highlevel requirements from IEEE 802.11 ESS Mesh PAR (11-04/054) and the key functional areas.

11-04-0970-04-000s-scope (09/25/04)

- Updated draft based on Sept04 TGs Berlin Interim Meeting ad-hoc group face-to-face discussion

# Introduction

Document Purpose

This document specifies the minimum functional requirements that must be addressed by the first draft specification of IEEE 802.11 Task Group s.

It also defines the scope of the efforts undertaken by TGs.  It indicates which aspects of standards developed will, or will not, be addressed by TGs.

This document is a working document that presents a “best estimate” summary for the minimum functional requirements and scope of TGs, but which will also evolve and be updated as the TGs effort progresses.

# Categories for Organization of Functional Requirements and Scope

The following table includes a list of system functional categories that are relevant for IEEE 802.11s ESS Mesh Networking, and will be covered by the functional requirements and scope descriptions in this document.

| Number | Functional Category Label | Functional Category Description |
| --- | --- | --- |
| FC1 | TOPO_RT_FWD | Mesh Topology Learning, Routing, and Forwarding |
| FC2 | SECURITY | Mesh Security |
| FC3 | MEAS | Mesh Measurement |
| FC4 | DISC_ASSOC | Mesh Discovery and Association |
| FC5 | MAC | Mesh Medium Access Coordination |
| FC6 | DSS_CMP | Compatibility to 802.11 Distribution System Services (DSS) |
| FC7 | INTRWRK | Mesh Interworking |
| FC8 | RFCTL_MGMT | Mesh RF Resource Control and Management |

Table 1: Functional Categories for TGs

Table

Table

Table

Minimum Functional Requirements

The following table defines the minimum functional requirements that must be addressed by the first draft specification of IEEE 802.11 Task Group s.  A proposal submitted for consideration to 802.11 TGs and declared to be complete is required to meet the functional requirements defined in this table.  A partial proposal may meet some of the requirements stated here.

| Number | Category | Name | Requirement | Notes (informative) |
| --- | --- | --- | --- | --- |
| FR1 | TOPO_RT_FWD | Mesh Topology Discovery | The proposal shall enable automatic topology learning, including the status and quality of mesh links between Mesh APs and Mesh Points. |  |
| FR2 | TOPO_RT_FWD | Mesh Routing Protocol | At least one layer-2 protocol and algorithm shall be defined for dynamic auto-configuration of layer-2 data delivery paths between Mesh APs and Mesh Points over 802.11 wireless links (including across multiple wireless hops). |  |
| FR3 | TOPO_RT_FWD | Mesh Broadcast Data Delivery | The proposal shall enable layer-2 broadcast/multicast data delivery across the ESS Mesh. |  |
| FR4 | TOPO_RT_FWD | Mesh Unicast Data Delivery | The proposal shall enable layer-2 unicast data delivery across the ESS Mesh. |  |
| FR5 | TOPO_RT_FWD | Extensible Mesh Routing Architecture | The proposal shall define an architecture and protocol to allow for alternative path selection metrics and/or protocols based on application requirements. |  |
| FR6 | TOPO_RT_FWD | Support for Single and Multiple Radios | The proposal shall allow the use of one or more IEEE 802.11 radios on Mesh APs and Mesh Points. |  |
| FR7 | SECURITY | Mesh Security | The proposal shall utilize IEEE 802.11i security mechanisms, or an extension thereof, for the purpose of securing an ESS Mesh in which all of the Mesh APs and Mesh Points are controlled by a single logical administrative entity for security. |  |
| FR8 | MEAS | Radio-Aware Routing Metrics | At least one radio-aware routing metric shall be defined for use by the mesh routing protocol(s). |  |
| FR9 | DISC_ASSOC | Discovery and Association with an ESS Mesh | The proposal shall allow Mesh APs and Mesh Points to discover and associate with an ESS Mesh network. |  |
| FR10 | MAC | Amendment to MAC with no PHY changes required | The proposal shall not require modification to the 802.11 physical layer. |  |
| FR11 | DSS_CMP | Backwards compatibility with legacy BSS and STA | The proposal shall not require modification to the STA’s relationship with the BSS and ESS. |  |
| FR12 | INTRWRK | Compatibility with higher-layer protocols | An IEEE 802.11 ESS Mesh Network shall appear to higher layers as a current style IEEE 802 LAN. | An ESS Mesh network should be able to connect with other IEEE 802 LANs using IEEE 802.1D and should be compatible with IP. |
| FR13 | PAR | Compliance to PAR | The proposal shall comply with all mandatory requirements of the TGs PAR. |  |

Table 5: Minimum Functional Requirements for TGs

# Scope Clarification Organized by Functional Categories for TGs

This document defines the scope of the efforts undertaken by TGs.   It indicates which aspects of standards develop are or are not considered by the TGs effort as follows:

- In-scope – aspects that will be considered, addressed, or affected by TGs

- Out-of-scope – aspects that will not be considered or addressed by TGs

- Unaffected – aspects that may co-exist with TGs, but will be unaffected, or essentially preserved, by TGs

- To-be-determined – aspects which are, as yet, TBD (to-be-determined) with respect to TGs

The focus of this section is to clarify the areas of functionality that will or will not be considered by TGs.  This is intended to provide additional informative guidelines for proposals.  This section does not attempt to further prioritize the “in-scope” functions.  Labeling of individual features as mandatory or optional will be done as a later step.

## Mesh Topology Learning, Routing, and Forwarding (TOPO_RT_FWD)

The following captures the scope of ESS Mesh multi-hop routing and frame forwarding support to be considered by TGs:

[Editor Note: More info is expected to be provided by the Routing ad-hoc team.]

| Functionality Applicability |  |  |  | Notes (Informative) |  |
| --- | --- | --- | --- | --- | --- |
|  | In-scope |  |  |  |  |
|  | TOPO_RT_FWD_SCP1 |  | - Mesh topology discovery, including MP and MAP neighbor discovery within an ESS Mesh |  |  |
|  |  |  | - Layer2 mesh routing protocols and algorithms |  |  |
|  |  |  | - Layer2 mesh broadcast/multicast and unicast data delivery |  |  |
|  |  |  | - Architecture to support alternative routing protocols and metrics |  |  |
|  |  |  | - Mesh routing with single-radio devices |  |  |
|  |  |  | - Mesh routing with multiple-radio devices |  |  |
|  |  | - Routing topology synchronization |  |  |  |
|  |  | - Use of radio-aware route selection metrics |  |  |  |
|  |  | - QoS-based route selection |  |  |  |
|  |  |  | - Proactive routing |  |  |
|  |  |  | - On-demand routing |  |  |
|  |  |  | - Hybrid routing |  |  |
|  |  |  | - MP and MAP neighbor discovery within an ESS Mesh via passive scanning |  |  |
|  |  |  | - MP and MAP neighbor discovery within an ESS Mesh via active scanning |  |  |
|  |  |  | - Peer-to-peer mesh link adjacency |  |  |
|  | Unaffected |  |  |  |  |
|  | To-be-determined |  |  |  |  |
|  | Out-of-scope |  |  |  |  |
|  |  | - Layer-3 mesh routing protocols |  |  |  |

## Mesh Security (SECURITY)

The following captures the scope of system security functions to be considered by the TGs:

[Editor Note: More info is expected to be provided by the Security ad-hoc team.]

| Functionality Applicability |  |  |  | Notes (Informative) |  |
| --- | --- | --- | --- | --- | --- |
|  | In-scope |  |  |  |  |
|  | SECURITY_SCP1 |  | - Secure association of Mesh Points and Mesh APs to an ESS Mesh |  |  |
|  |  |  | - Securing an ESS Mesh in which all of the MPs and MAPs are controlled by a single logical administrative entity for security. |  |  |
|  |  |  | - Secure data message exchange between MPs and MAPs over mesh links |  |  |
|  |  |  | - Secure management message exchange between MPs and MAPs over mesh links |  |  |
|  |  |  | - Secure topology and routing information exchange between MPs and MAPs over mesh links |  |  |
|  |  |  | - Centralized key distribution |  |  |
|  |  |  | - Distributed key distribution |  |  |
|  |  |  | - Hybrid key distribution |  | - Mix of centralized and distributed key management |
|  |  |  | - Static key support |  |  |
|  |  |  | - Dynamic key support |  |  |
|  |  |  | - Extension of IEEE 802.11i security mechanisms for mesh |  |  |
|  | Unaffected |  |  |  |  |
|  | To-be-determined |  |  |  |  |
|  | Out-of-scope |  |  |  |  |
|  |  |  | - Securing an ESS Mesh in which all of the MPs and MAPs are not controlled by a single logical administrative entity for security |  |  |
|  |  | - DOS attack |  |  |  |
|  |  | - Rogue MP detection |  |  |  |

## Mesh Measurement (MEAS)

The following captures the scope of mesh measurement functions to be considered by the TGs:

| Functionality Applicability |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| MEAS_SCP1 | - Specification of radio-aware metrics for use by mesh routing protocols |  |
|  | - Specification of radio-aware metrics for use by mesh medium access coordination |  |
|  | - Mesh link quality measurements |  |
|  | - Mesh path quality measurements |  |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |
|  | - Legacy STA measurements (e.g. 802.11k AP-STA measurements) |  |

## Mesh Discovery and Association (DISC_ASSOC)

The following captures the scope of MP and MAP discovery of ESS Mesh networks and subsequent association with an ESS Mesh network to be considered by TGs:

| Functionality Applicability |  |  |  | Notes (Informative) |  |
| --- | --- | --- | --- | --- | --- |
|  | In-scope |  |  |  |  |
|  | DISC_ASSOC_SCP1 |  | - Protocols to allow MPs and MAPs to discover ESS Mesh networks |  |  |
|  |  |  | - Protocols to allow MPs and MAPs to associate with ESS Mesh networks |  |  |
|  |  |  | - Protocols to allow MPs and MAPs to associate with other MPs and MAPs within an ESS Mesh |  |  |
|  | Unaffected |  |  |  |  |
|  | To-be-determined |  |  |  |  |
|  | Out-of-scope |  |  |  |  |

## Mesh Medium Access Coordination (MAC)

The following captures the scope of mesh medium access coordination support to be considered by TGs:

[Editor Note: More info is expected to be provided by the MAC Coordination ad-hoc team.]

| Functionality Applicability |  |  |  | Notes (Informative) |  |
| --- | --- | --- | --- | --- | --- |
|  | In-scope |  |  |  |  |
| MAC_SCP1 |  | - Mitigate performance degradation caused by hidden nodes |  |  |  |
|  |  |  | - Mitigate performance degradation caused by exposed nodes |  |  |
|  |  |  | - Mitigate performance degradation caused by multi-hop message forwarding |  |  |
|  |  |  | - Flow control over multi-hop paths to avoid performance degradation caused by local congestion. |  |  |
|  |  |  | - Scheduling across multiple nodes to achieve higher throughput and lower delay in the multi-hop network. |  |  |
|  |  |  | - Traffic prioritization within an ESS Mesh |  |  |
|  |  |  | - Enhancements to make the MAC work well across a range of different network size, usage model, etc. |  |  |
|  |  |  | - Peer-to-peer mesh link communication coordination |  |  |
|  | Unaffected |  |  |  |  |
|  | To-be-determined |  |  |  |  |
|  |  | - Determine if a particular flow can be admitted into the network based on the availability of the bandwidth resource and existing usages. |  |  |  |
|  | Out-of-scope |  |  |  |  |
|  |  | - Modifications to 802.11 physical layer |  |  |  |
|  |  |  | - MAC extension to support the use of dynamic directional antenna. |  | - Likely requires high complexity, e.g. TDMA scheduling to know when a neighbor’s antenna is pointed at a node. |

Compatibility to

802.11 Distributoin System Services (DSS_CMP)

The following captures the scope of 802.11s service integration to be considered by TGs:

| Functionality Applicability |  |  |  | Notes (Informative) |  |
| --- | --- | --- | --- | --- | --- |
|  | In-scope |  |  |  |  |
| DSS_CMP_SCP1 |  | - Mesh Point DS Services (DSS) Integration |  | - Ensure an ESS implemented with an ESS Mesh can continue to support distribution service, integration service, association service, re-association service, disassociation service in infrastructure mode to enable unicast, multicast and broadcast data delivery for the associated STAs |  |
|  | Unaffected |  |  |  |  |
|  | To-be-determined |  |  |  |  |
|  | Out-of-scope |  |  |  |  |
|  |  |  | - 802.11 BSS functionality |  |  |
|  |  | - 802.11e DLP |  | - DLP is defined within a BSS |  |

## Mesh Interworking (INTRWRK)

The following captures the scope of interworking functions to be considered by TGs:

| Functionality Applicability |  |  |  | Notes (Informative) |  |
| --- | --- | --- | --- | --- | --- |
|  | In-scope |  |  |  |  |
| INTRWRK_SCP1 |  | - Compatibility with higher layer protocols |  |  |  |
|  |  |  | - Interfacing an ESS Mesh with other IEEE 802 LANs |  |  |
|  |  |  | - Support for interfacing an ESS Mesh with other IEEE 802 LANs using 802.1D |  | - E.g. something along the lines of 802.17 Annex F |
|  | Unaffected |  |  |  |  |
|  | To-be-determined |  |  |  |  |
|  | Out-of-scope |  |  |  |  |

## Mesh RF Resource Control and Management (RFCTL_MGMT)

The following captures the scope of RF resource control and management to to be considred by TGs:

| Functionality Applicability |  |  |  | Notes (Informative) |  |
| --- | --- | --- | --- | --- | --- |
|  | In-scope |  |  |  |  |
| RFCTL_MGMT_SCP1 |  | - Interfaces to support 802.11h DFS compliancy |  | - Must enable compliance to the existing 802.11h DFS requirements |  |
|  |  | - Interfaces and parameter exchange to enable RF auto configuration support |  | Support the following parameters exchange over the mesh link between MPs - parameters exchange to support Tx power control - parameters exchange to support channel and antenna selection to minimize interference and to maximize coverage and throughput |  |
|  | Unaffected |  |  |  |  |
|  | To-be-determined |  |  |  |  |
|  |  | - Algorithm to determine the channel selection |  | - At a minimum, should TGs include an informative section on how to achieve coalescence of a mesh to a common channel to enable interoperability? |  |
|  |  |  | - Algorithm to control the Tx power level |  |  |
|  | Out-of-scope |  |  |  |  |
|  |  | - Algorithms to steer directional antennas. |  |  |  |
|  |  | - Alogorithm to control RF rate adaptation |  | - Rate adaptation algorithm is implementation dependent |  |
|  |  |  | - IP address assignment |  |  |

# References

[1] IEEE 802 11-04/54r2, PAR for IEEE 802.11s ESS Mesh

[2] IEEE 802 11-04/56r1, Five Criteria for IEEE 802.11s ESS Mesh

[3] IEEE 802 11-04/662r10, TGs Usage Models

[4] IEEE 802.11-04/969r2, Draft Terms and Definitions for 802.11s

[5] IEEE 802.11-04/970r4, Proposed 802.11 TGs Scope

- APPENDIX-

# A.1 Key Highlevel Requirements Derived from PAR and Others

The following table explains the key highlevel requirements of the TGs scope, in terms of technical and other considerations, that are derived from the IEEE 802.11 TGs PAR and also cross reference to the Five Criteria as well as to the 802.11 standards.  The information of these tables are used as the basis to derive the key highlevel technical requirements of the scope that is used to develop the draft 802.11 amendments for ESS Mesh networking specification by TGs.   The figure below is to capture the considerations of the scope that needs to be addressed by the TGs.

| Key Highlevel Technical Requirements |  | PAR | 5 Criteria | Reference Details |
| --- | --- | --- | --- | --- |
| T.1 | Compatibility with 802.11 Distribution System Service (DSS) - An ESS Mesh is functionally equivalent to a wired ESS, with respect to the STAs relationship with the BSS and ESS. |  |  | - Refer to 6.2 Compatibility section in 11-04/056 (Five Criteria) - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR) |
| T.2 | Enable automatic topology learning and dynamic path configuration to support self-configuring multihop topology. - The amendment shall be extensible to allow for alternative path selection metrics and/or protocols based on application requirements |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR) |
| T.3 | Radio-awareness metrics to support self-configuring topology discovery and multi-hop routing. |  |  | - Refer to 6.3.a Distinct Identity section in 11-04/056 (Five Criteria) - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR) |
| T.4 | Support broadcast/multicast and unicast delivery at the MAC layer to the end user via multi-hop forwarding |  |  | - Refer to 12. Scope of Proposed Project, 13. Purpose of the Proposed Project and 18. Additional Explanatory Notes sections in 11-04/054 (PAR) |
| T.5 | Addressing based on the use of the four-address frame format or an extension |  |  | - Refer to 6.3.a Distinct Identity section in 11-04/056 (Five Criteria) - Refer to 13. Purpose of Proposed Project section in 11-04/054 (PAR) |
| T.6 | Interfacing with high layer protocols and interworking with other non-802.11 network using high layer protocols |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR). |
| T.7 | Security based on the use of existing 802.11i standard or an extension thereof, for the purpose of securing an ESS Mesh in which all of the APs are controlled by a single logical administrative entity for security. |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR). |
| T.8 | Support the use of one or more IEEE 802.11 radios on each AP in the ESS Mesh. |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR). |
| T.9 | ESS Mesh is an extension to the IEEE 802.11 MAC and does not require PHY changes |  |  | - Refer to 18. Additional Explanatory Notes section in 11-04/054 (PAR). |

Table 6: Key Highlevel Technical Requirements from PAR and cross reference to Five Criteria

| Key Highlevel Requirements Outside of PAR |  | Justification |
| --- | --- | --- |
| O.1 | Mesh Network STA Mobility Support | - Ensure the mobility service integration is compatible to the existing 802.11 architecture. |
| O.2 | Support both the managed and unmanged network management models | - Consider the network management requirements to address the non-home networking products. |

Table 7: Additional Highlevel Technical Requirements NOT Directly from the PAR

Figure 3:  Scope Considerations by 802.11 TGs

A. 2 Key IEEE Standard Requirements Extracted from Five Criteria

# A.2. 1 Compatibility

IEEE 802 defines a family of standards.  All standards shall be in conformance with the IEEE 802.1 Architecture, Management and Interworking documents as follows: 802. Overview and Architecture, 802.1D, 802.1Q and parts of 802.1f.  If any variances in conformance emerge, they shall be thoroughly disclosed and reviewed with 802.  Each standard in the IEEE 802 family of standards shall include a definition of managed objects, which are compatible with system management standards.

ESS Mesh specifies one possible Wireless Distribution System  (WDS) that behaves in every respect as an IEEE 802.11 Infrastructure Mode network.  As such, it is entirely compatible with the 802.11 architecture and, by inference, compatible with the IEEE 802 architecture, including IEEE 802.1D, IEEE 802.1Q and IEEE 802.1F.

[Editor Note:  The definition of infrastructure according to IEEE 802.11 specification is as follows,

The” infrastructure” includes the distribution system medium (DSM), access point (AP), and portal entities. It is also the logical location of distribution and integration service functions of an extended service set (ESS). An infrastructure contains one or more APs and zero or more portals in addition to the distribution system (DS).]

# A.2. 2 Distinct Identity

…Instantaneous radio-awareness is currently out-of-scope for IETF Mobile Ad Hoc Networks (MANET), limiting its ability to make efficient use of the wireless channel, unlike ESS Mesh which allows tight integration between the MAC and the multi-hop WDS.

…These design goals (i.e. of IEEE 802.11f) differ significantly from those of ESS Mesh which primarily concerns itself with topology discovery, delivery, and mobility management for a wireless DS implementation.

…IEEE 802.11 ESS Mesh is the only wireless distribution system (WDS) protocol defined in 802.11.

# A.2. 3 Technical Feasibility

…Address mapping information enables roaming, i.e. the ability of the DS to determine the correct output point or points for user traffic, while global topology information is used to compute unicast paths.

# A.2. 4 Economic Feasibility

…Considering the cost of IEEE 802.11 APs in the market place, it is a reasonable cost to add a wireless DS, especially in comparison to the cost of a wired DS.

…The proposed amendment has no known impact on installation costs.

# A.2. 5 Marketing Requirments Considerations Based on Five Criteria

| Key Highlevel Marketing Requirements |  | Reference Details |
| --- | --- | --- |
| M.1 | Support an integrated subnet routing/mobility layer with the WLAN architecture | - Refer to 6.1.a Broad sets of applicability section in 11-04/056 (Five Criteria) |
| M.2 | Using ESS Mesh, coverage within a house, a hospital, a hotel, and airport, a neighbourhood, a campus, etc., may be extended wirelessly and without manual configuration other than setting the SSID and/or AP introduction. | - Refer to 6.1.a Broad sets of applicability section in 11-04/056 (Five Criteria) |
| M.3 | ESS Mesh improves the versatility and functionality of conventional WLAN APs by extending services wirelessly as an alternative or an addition to wired connections | - Refer to 6.1.c Broad sets of applicability section in 11-04/056 (Five Criteria) |
| M.4 | Considering the cost of IEEE 802.11 APs in the market place, it is a reasonable cost to add a wireless DS, especially in comparison to the cost of a wired DS. | - Refer to 6.5.b Economic Feasibility section in 11-04/056 (Five Criteria) |
| M.5 | The proposed amendment has no known impact on installation costs | - Refer to 6.5.c Economic Feasibility section in 11-04/056 (Five Criteria) |

Table 8: Key Highlevel Marketing Requirements Considerations from Five Criteria