IEEE P802.11
Wireless LANs

Draft 802.11 TGs Functional Requirements and Scope

Date:	November 15, 2004

Author:	W. Steven Conner
	Intel Corporation
	2111 NE 25th Ave, Hillsboro, OR 97124, USA
	Phone: +1 503 264 8036
	e-Mail: w.steven.conner@intel.com

Abstract

This document specifies the minimum functional requirements and scope for ESS Mesh Networking mechanisms to be defined by 802.11 TGs.  This document is based as a starting point on document 11-04/0970r4.

Contributors

(This will grow to reflect those providing contributions and review comments of this document)

| Name | Company | Email / Phone | Contact Info |
| --- | --- | --- | --- |
| W. Steven Conner | Intel Corp. | w.steven.conner@intel.com tel: +503-264-8036 | 2111 NE 25th Ave. Hillsboro, OR 97124 U.S.A. |
| Jonathan Agre | Fujitsu | jagre@fla.fujitsu.com tel: +301-486-0978 | 8400 Baltimore Ave., Suite 302, College Park, MD 20740, USA |
| Malik Audeh | Tropos Networks | malik@audeh@tropos.com tel: +408-331-6814 | 555 Del Rey Ave., Sunnyvale, CA 94085 |
| Narasimha Chari | Tropos Networks | chari@tropos.com tel: +408-331-6814 | 555 Del Rey Ave., Sunnyvale, CA 94085 |
| Vann Hasty | Mesh Networks | vhastry@meshnetworks.com tel: +407-659-5371 | 485 N. Keller Rd. Suite 250, Maitland, FL 32771 |
| Guido R. Hiertz | Aachen University | hiertz@ieee.org tel: +49-241-80-25-82-9 | Kopernikusstr. 16 52064 Aachen Germany |
| Avinash Joshi | Mesh Networks | ajoshi@meshnetworks.com tel: | 485 N. Keller Rd. Suite 250, Maitland, FL 32771 |
| Sebnem Z. Ozer | Mesh Networks | sozer@meshnetworks.com tel: +407-659-5391 | 485 N. Keller Rd. Suite 250, Maitland, FL 32771 |
| Stephan Rayment | BelAir Networks | srayment@belairnetworks.com tel: +613-254-7070 | 603 March Road, Kanata, Ontario Canada, K2K 2M5 |
| Kazuyuki Sakoda | Sony | sako@wcs.sony.co.jp tel: +81-3-6409-3201 | Oval Court Ohsaki MW 2-17-1 Higashigotanda Shinagawa-ku Tokyo 141-0022 Japan |
| Ambatipudi Sastry | Packethop | sastry@packethop.com tel: +650-292-5003 | 1301 Shoreway Rd Belmont, CA94002 |
| Tricci So | Nortel Networks | tso@nortelnetworks.com tel: +613-763-9639 | 3500 Carling Ave., Ottawa, ON, K2H 8E9, Canada |
| Lily Yang | Intel Corp. | Lily.l.yang@intel.com tel: +503-264-8813 | 2111 NE 25th Ave. Hillsboro, OR 97124 U.S.A. |
| Juan Carlos Zuniga | Interdigital | Juancarlos.zuniga@interdigital.com tel: +514-904-6251 | 1000 Sherbrooke W., 10th Fl., Montreal QC, H3A 3G4, Canada |

Document Version History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | - First version based on document 11-04/0970r4 | 12 October 2004 |
| R1 | - Reorganization of document | 13 October 2004 |
| R2 | - Accepted all changes in document revision history | 13 October 2004 |
| R3 | - Integrated changes from comments received during Oct. 13 TGs teleconference | 26 October 2004 |
| R4 | - Integrated changes from comments received during Nov. 11 TGs teleconference | 11 November 2004 |
| R5 | - Editorial change to add labels to scope items in Section 4. | 15 November 2004 |

Table of Contents

1	Introduction	3

1.1	Document Purpose	3

2	Categories for Organization of Functional Requirements and Scope (Informative)	3

3	Minimum Functional Requirements	3

4	Scope Clarification Organized by Functional Categories for TGs	5

4.1	Mesh Topology Learning, Routing, and Forwarding (TOPO_RT_FWD)	6

4.2	Mesh Security (SECURITY)	7

4.3	Mesh Measurement (MEAS)	7

4.4	Mesh Discovery and Association (DISC_ASSOC)	8

4.5	Mesh Medium Access Coordination (MMAC)	9

4.6	Compatibility to 802.11 Services (SERV_CMP)	10

4.7	Mesh Interworking (INTRWRK)	10

4.8	Mesh Configuration and Management (CFG_MGMT)	11

5	References	12

# Introduction

## Document Purpose

This document specifies the minimum functional requirements that must be addressed by the first draft specification of IEEE 802.11 Task Group s.  It also defines the scope of the efforts undertaken by TGs.  This document is intended to supplement the 802.11s PAR [1] and 5 Criteria [2] documents, which provide the overriding requirements for Task Group s.

The focus of the scope description is to clarify the areas of functionality that will or will not be considered by TGs.  This is intended to provide additional informative guidelines for proposals.  This document does not attempt to further prioritize the “in-scope” functions.  Labeling of individual features as mandatory or optional will be done as a later step.

This document is a working document that presents a “best estimate” summary for the minimum functional requirements and scope of TGs, but which will also evolve and be updated as the TGs effort progresses.

# Categories for Organization of Functional Requirements and Scope (Informative)

The following table includes a list of system functional categories that are relevant for IEEE 802.11s ESS Mesh Networking, and will be covered by the functional requirements and scope descriptions in this document.   These categories are only informative and intended to help with the organizational structure of the functional requirements and scope items in Section 3 and Section 4.

| Functional Category Label | Functional Category Description |
| --- | --- |
| TOPO_RT_FWD | Mesh Topology Learning, Routing, and Forwarding |
| SECURITY | Mesh Security |
| MEAS | Mesh Measurement |
| DISC_ASSOC | Mesh Discovery and Association |
| MMAC | Mesh Medium Access Coordination |
| SERV_CMP | Compatibility to 802.11 Services |
| INTRWRK | Mesh Interworking |
| CFG_MGMT | Mesh Configuration and Management |

Table 1: Functional Categories for TGs

# Minimum Functional Requirements

The following table defines the minimum functional requirements that must be addressed by the first draft specification of IEEE 802.11 Task Group s.  A proposal submitted for consideration to 802.11 TGs and declared to be complete is required to meet the functional requirements defined in this table.  A partial proposal may meet some of the requirements stated here.

A separate comparison criteria document [6] includes a template with an entry for each functional requirement in this table, allowing those submitting proposals to check off which FRs are covered by a proposal and provide references to relevant sections of the proposal where each FR is covered.

Note that the following functional requirement descriptions refer primarily to Mesh Points, which are a class of 802.11 entities including Mesh APs, as defined in [4].

| Number | Functional Category (see Table 1) | Name | Requirement | From PAR | Notes (informative) |
| --- | --- | --- | --- | --- | --- |
| FR1 | TOPO_RT_FWD | Mesh Topology Discovery | The proposal shall enable automatic topology learning, including the status and quality of mesh links between Mesh Points. | X | This functional requirement is focused on the peer-to-peer discovery of Mesh Points witin an ESS Mesh and establishment of link-adjacency. |
| FR2 | TOPO_RT_FWD | Mesh Routing Protocol | At least one MAC address-based protocol and algorithm shall be defined for dynamic auto-configuration of layer-2 data delivery paths between Mesh Points over 802.11 wireless links (including across multiple wireless hops). | X | Routing protocol(s) should support unicast, broadcast, and multicast (may be provided by a single protocol or multiple protocols). |
| FR3 | TOPO_RT_FWD | Extensible Mesh Routing Architecture | The proposal shall define a protocol architecture to allow for alternative path selection metrics and/or routing protocols based on application requirements. | X |  |
| FR4 | TOPO_RT_FWD | Mesh Broadcast Data Delivery | The proposal shall enable layer-2 broadcast/multicast data delivery across the ESS Mesh. | X |  |
| FR5 | TOPO_RT_FWD | Mesh Unicast Data Delivery | The proposal shall enable layer-2 unicast data delivery across the ESS Mesh. | X |  |
| FR6 | TOPO_RT_FWD | Support for Single and Multiple Radios | The proposal shall support the use of one or more IEEE 802.11 physical-layer transcievers on Mesh Points. | X | A proposal should not preclude the use of a single radio. ESS Mesh should work with single radio devices but should allow for improved performance with multi-radio devices. |
| FR7 | SECURITY | Mesh Security | The proposal shall utilize IEEE 802.11i security mechanisms, or an extension thereof, for the purpose of securing an ESS Mesh in which all of the Mesh Points and Mesh Portals are controlled by a single logical administrative entity for security. | X | The single logical administrative entity is with respect to the authentication of Mesh Points. Mesh Points within a secure 11s mesh have an assumed relationship, e.g. all may belong to a home owner (residential) or all may be under the control of an IT department (office). |
| FR8 | MEAS | Radio-Aware Routing Metrics | At least one radio-aware routing metric shall be defined for use by the mesh routing protocol(s). | X | Radio-aware routing metrics may include all aspects of MAC/PHY statistics such as data rate encoding, PER, latency, noise, and other statistics specified by TGk. The term “radio-aware routing metric” is not intended to limit the type of information that could be included in a metric. |
| FR9 | SERV_CMP | Backwards compatibility with legacy BSS and STA | The proposal shall not require modification to the STA’s relationship with the BSS and ESS. | X |  |
| FR10 | DISC_ASSOC | Discovery and Association with an ESS Mesh | The proposal shall support the discovery of and association with an ESS Mesh network by Mesh Points. |  |  |
| FR11 | MMAC | Amendment to MAC with no PHY changes required | The proposal shall not require modification to the 802.11 physical layer. |  |  |
| FR12 | INTRWRK | Compatibility with higher-layer protocols | An IEEE 802.11 ESS Mesh Network shall appear to higher layers as a current style IEEE 802 LAN. |  | An ESS Mesh network should be able to connect with other IEEE 802 LANs using IEEE 802.1D and should be compatible with IP. (See 802.11 Clause 5.1.1.4). This requirement relates to the overall 802.11s system (collection of Mesh Points), not necessarily to individual Mesh Points. |

Table 2: Minimum Functional Requirements for TGs

# Scope Clarification Organized by Functional Categories for TGs

This document defines the scope of the efforts undertaken by TGs.   It indicates which aspects of standards develop are or are not considered by the TGs effort as follows:

- In-scope – aspects that will be considered, addressed, or affected by TGs

- Out-of-scope – aspects that will not be considered or addressed by TGs

- Unaffected – aspects that may co-exist with TGs, but will be unaffected, or essentially preserved, by TGs

- To-be-determined – aspects which are, as yet, TBD (to-be-determined) with respect to TGs

The focus of this section is to clarify the areas of functionality that will or will not be considered by TGs.  This is intended to provide additional informative guidelines for proposals.  This section does not attempt to further prioritize the “in-scope” functions.  Labeling of individual features as mandatory or optional will be done as a later step.

## Mesh Topology Learning, Routing, and Forwarding (TOPO_RT_FWD)

The following captures the scope of ESS Mesh multi-hop routing and frame forwarding support to be considered by TGs:

[Editor Note: More info is expected to be provided by the Routing ad-hoc team.]

| Functionality Applicability |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| TOPO_RT_FWD_SCP1 | - Mesh topology discovery, including MP and MAP neighbor discovery within an ESS Mesh |  |
| TOPO_RT_FWD_SCP2 | - MAC address-based mesh routing protocols and algorithms |  |
| TOPO_RT_FWD_SCP3 | - Layer2 mesh broadcast/multicast and unicast data delivery |  |
| TOPO_RT_FWD_SCP4 | - Architecture to support alternative routing protocols and metrics |  |
| TOPO_RT_FWD_SCP5 | - Mesh routing with single-radio devices |  |
| TOPO_RT_FWD_SCP6 | - Mesh routing with multiple-radio devices |  |
| TOPO_RT_FWD_SCP7 | - Routing topology synchronization |  |
| TOPO_RT_FWD_SCP8 | - Use of radio-aware route selection metrics |  |
| TOPO_RT_FWD_SCP9 | - QoS-based route selection |  |
| TOPO_RT_FWD_SCP10 | - Proactive routing |  |
| TOPO_RT_FWD_SCP11 | - On-demand routing |  |
| TOPO_RT_FWD_SCP12 | - Hybrid routing |  |
| TOPO_RT_FWD_SCP13 | - MP and MAP neighbor discovery within an ESS Mesh via passive scanning |  |
| TOPO_RT_FWD_SCP14 | - MP and MAP neighbor discovery within an ESS Mesh via active scanning |  |
| TOPO_RT_FWD_SCP15 | - Peer-to-peer mesh link adjacency |  |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |
|  | - Layer-3 mesh routing protocols |  |

## Mesh Security (SECURITY)

The following captures the scope of system security functions to be considered by the TGs:

[Editor Note: More info is expected to be provided by the Security ad-hoc team.]

| Functionality Applicability |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| SECURITY_SCP1 | - Secure association of Mesh Points and Mesh APs to an ESS Mesh |  |
| SECURITY_SCP2 | - Securing an ESS Mesh in which all of the MPs and MAPs are controlled by a single logical administrative entity for security. |  |
| SECURITY_SCP3 | - Secure data message exchange between MPs and MAPs over mesh links |  |
| SECURITY_SCP4 | - Secure management message exchange between MPs and MAPs over mesh links |  |
| SECURITY_SCP5 | - Secure topology and routing information exchange between MPs and MAPs over mesh links |  |
| SECURITY_SCP6 | - Centralized key distribution |  |
| SECURITY_SCP7 | - Distributed key distribution |  |
| SECURITY_SCP8 | - Hybrid key distribution | - Mix of centralized and distributed key management |
| SECURITY_SCP9 | - Static key support |  |
| SECURITY_SCP10 | - Dynamic key support |  |
| SECURITY_SCP11 | - Extension of IEEE 802.11i security mechanisms for mesh |  |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |
|  | - Securing an ESS Mesh in which all of the MPs and MAPs are not controlled by a single logical administrative entity for security |  |
|  | - DOS attack |  |
|  | - Rogue MP detection |  |

## Mesh Measurement (MEAS)

The following captures the scope of mesh measurement functions to be considered by the TGs:

| Functionality Applicability |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| MEAS_SCP1 | - Specification of radio-aware metrics for use by mesh routing protocols |  |
| MEAS_SCP2 | - Specification of radio-aware metrics for use by mesh medium access coordination |  |
| MEAS_SCP3 | - Mesh link quality measurements |  |
| MEAS_SCP4 | - Mesh path quality measurements |  |
| Unaffected |  |  |
| To-be-determined |  |  |
| MEAS_SCP5 | - Measurements to support the use of directional antennas in a mesh network |  |
| MEAS_SCP6 | - Mesh related measurements to aid STAs in making roaming decisions, e.g., WDS capacity currently available for traffic forwarding. |  |
| Out-of-scope |  |  |
|  | - Legacy STA measurements (e.g. 802.11k AP-STA measurements) |  |

## Mesh Discovery and Association (DISC_ASSOC)

The following captures the scope of MP and MAP discovery of ESS Mesh networks and subsequent association with an ESS Mesh network to be considered by TGs:

| Functionality Applicability |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| DISC_ASSOC_SCP1 | - Protocols to allow MPs and MAPs to discover ESS Mesh networks |  |
| DISC_ASSOC_SCP2 | - Protocols to allow MPs and MAPs to associate with ESS Mesh networks |  |
| DISC_ASSOC_SCP3 | - Protocols to allow MPs and MAPs to associate with other MPs and MAPs within an ESS Mesh |  |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |

## Mesh Medium Access Coordination (MMAC)

The following captures the scope of mesh medium access coordination support to be considered by TGs:

[Editor Note: More info is expected to be provided by the MAC Coordination ad-hoc team.]

| Functionality Applicability |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| MMAC_SCP1 | - Mitigate performance degradation caused by hidden nodes |  |
| MMAC_SCP2 | - Mitigate performance degradation caused by exposed nodes |  |
| MMAC_SCP3 | - Mitigate performance degradation caused by multi-hop message forwarding |  |
| MMAC_SCP4 | - Flow control over multi-hop paths to avoid performance degradation caused by local congestion. |  |
| MMAC_SCP5 | - Scheduling across multiple nodes to achieve higher throughput and lower delay in the multi-hop network. |  |
| MMAC_SCP6 | - Traffic prioritization within an ESS Mesh |  |
| MMAC_SCP7 | - Enhancements to make the MAC work well across a range of different network size, usage model, etc. |  |
| MMAC_SCP8 | - Peer-to-peer mesh link communication coordination |  |
| MMAC_SCP9 | - Extensions to 802.11e MAC to support QoS in an ESS Mesh |  |
| Unaffected |  |  |
| To-be-determined |  |  |
| MMAC_SCP10 | - Determine if a particular flow can be admitted into the network based on the availability of the bandwidth resource and existing usages. |  |
| Out-of-scope |  |  |
|  | - Modifications to 802.11 physical layer |  |
|  | - MAC extension to support the use of dynamic directional antenna. | - Likely requires high complexity, e.g. TDMA scheduling to know when a neighbor’s antenna is pointed at a node. |

## Compatibility to 802.11 Services (SERV_CMP)

The following captures the scope of 802.11s service integration to be considered by TGs:

| Functionality Applicability |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| SERV_CMP_SCP1 | - Mesh Point DS Services (DSS) Integration | - Ensure an ESS implemented with an ESS Mesh can continue to support distribution service, integration service, association service, re-association service, disassociation service in infrastructure mode to enable unicast, multicast and broadcast data delivery for the associated STAs |
| SERV_CMP_SCP2 | - ESS Mesh compatibility with STA mobility/roaming | - E.g., delivery of traffic to correct Mesh AP to reach a STA. |
| SERV_CMP_SCP3 | - Techniques to allow ESS Mesh to meet 802.11r performance requirements |  |
| Unaffected |  |  |
| To-be-determined |  |  |
| SERV_CMP_SCP4 | - Dissemination of STA/AP mapping information in the ESS Mesh |  |
| Out-of-scope |  |  |
|  | - 802.11 BSS functionality |  |
|  | - 802.11e DLP | - DLP is defined within a BSS |
|  | - STA roaming solutions (when to roam, where to roam, how to roam) | - Covered by TGr? |

## Mesh Interworking (INTRWRK)

The following captures the scope of interworking functions to be considered by TGs:

| Functionality Applicability |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| INTRWRK_SCP1 | - Compatibility with higher layer protocols |  |
| INTRWRK_SCP2 | - Interfacing an ESS Mesh with other IEEE 802 LANs |  |
| INTRWRK_SCP3 | - Support for interfacing an ESS Mesh with other IEEE 802 LANs using 802.1D | - E.g. something along the lines of 802.17 Annex F |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |

## Mesh Configuration and Management (CFG_MGMT)

The following captures the scope of configuration and management to to be considred by TGs:

| Functionality Applicability |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| CFG_MGMT_SCP1 | - Protocol extensions to support self-configuring formation of an ESS Mesh network |  |
| CFG_MGMT_SCP2 | - Interfaces to support 802.11h DFS compliancy | - Must enable compliance to the existing 802.11h DFS requirements |
| CFG_MGMT_SCP3 | - Interfaces and parameter exchange to enable RF auto configuration support | Support the following parameters exchange over the mesh link between MPs - parameters exchange to support Tx power control - parameters exchange to support channel and antenna selection to minimize interference and to maximize coverage and throughput |
| CFG_MGMT_SCP4 | - Support for managed network management model |  |
| CFG_MGMT_SCP5 | - Support for unmanaged network management model |  |
| Unaffected |  |  |
| To-be-determined |  |  |
| CFG_MGMT_SCP6 | - Algorithm to determine the channel selection | - At a minimum, should TGs include an informative section on how to achieve coalescence of a mesh to a common channel to enable interoperability? |
| CFG_MGMT_SCP7 | - Algorithm to control the Tx power level |  |
| CFG_MGMT_SCP8 | - QoS policy and management algorithms |  |
| Out-of-scope |  |  |
|  | - Algorithms to steer directional antennas. |  |
|  | - Alogorithm to control RF rate adaptation | - Rate adaptation algorithm is implementation dependent |
|  | - IP address assignment |  |
|  | - Protocols for remote management of an ESS Mesh Network |  |
|  | - 802.11F |  |

# References

[1] IEEE 802 11-04/54r2, PAR for IEEE 802.11s ESS Mesh

[2] IEEE 802 11-04/56r1, Five Criteria for IEEE 802.11s ESS Mesh

[3] IEEE 802 11-04/662r10, TGs Usage Models

[4] IEEE 802.11-04/969r2, Draft Terms and Definitions for 802.11s

[5] IEEE 802.11-04/970r4, Proposed 802.11 TGs Scope

[6] IEEE 802.11-04/1175r2, Draft IEEE 802.11 TGs Comparison Categories and Criteria