IEEE P802.11
Wireless LANs

| IEEE 802.11 TGs Functional Requirements and Scope |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-01-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| W. Steven Conner | Intel Corp. | M/S JF3-206 2111 NE 25th Ave, Hillsboro OR 97124 | +1-503-264-8036 | w.steven.conner@intel.com |

Abstract

This document specifies the minimum functional requirements and scope for WLAN Mesh mechanisms to be defined by 802.11 Task Group “s”.  This document is based as a starting point on document 11-04/0970r4.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contributors

| Name | Company | Email / Phone | Contact Info |
| --- | --- | --- | --- |
| W. Steven Conner | Intel Corp. | w.steven.conner@intel.com tel: +503-264-8036 | 2111 NE 25th Ave. Hillsboro, OR 97124 U.S.A. |
| Jonathan Agre | Fujitsu | jagre@fla.fujitsu.com tel: +301-486-0978 | 8400 Baltimore Ave., Suite 302, College Park, MD 20740, USA |
| Malik Audeh | Tropos Networks | malik@audeh@tropos.com tel: +408-331-6814 | 555 Del Rey Ave., Sunnyvale, CA 94085 |
| Narasimha Chari | Tropos Networks | chari@tropos.com tel: +408-331-6814 | 555 Del Rey Ave., Sunnyvale, CA 94085 |
| Vann Hasty | Mesh Networks | vhastry@meshnetworks.com tel: +407-659-5371 | 485 N. Keller Rd. Suite 250, Maitland, FL 32771 |
| Guido R. Hiertz | Aachen University | hiertz@ieee.org tel: +49-241-80-25-82-9 | Kopernikusstr. 16 52064 Aachen Germany |
| Avinash Joshi | Mesh Networks | ajoshi@meshnetworks.com tel: | 485 N. Keller Rd. Suite 250, Maitland, FL 32771 |
| Sebnem Z. Ozer | Mesh Networks | sozer@meshnetworks.com tel: +407-659-5391 | 485 N. Keller Rd. Suite 250, Maitland, FL 32771 |
| Stephen Rayment | BelAir Networks | srayment@belairnetworks.com tel: +613-254-7070 | 603 March Road, Kanata, Ontario Canada, K2K 2M5 |
| Kazuyuki Sakoda | Sony | sako@wcs.sony.co.jp tel: +81-3-6409-3201 | Oval Court Ohsaki MW 2-17-1 Higashigotanda Shinagawa-ku Tokyo 141-0022 Japan |
| Ambatipudi Sastry | Packethop | sastry@packethop.com tel: +650-292-5003 | 1301 Shoreway Rd Belmont, CA94002 |
| Tricci So | Nortel Networks | tso@nortelnetworks.com tel: +613-763-9639 | 3500 Carling Ave., Ottawa, ON, K2H 8E9, Canada |
| Lily Yang | Intel Corp. | Lily.l.yang@intel.com tel: +503-264-8813 | 2111 NE 25th Ave. Hillsboro, OR 97124 U.S.A. |
| Juan Carlos Zuniga | Interdigital | Juancarlos.zuniga@interdigital.com tel: +514-904-6251 | 1000 Sherbrooke W., 10th Fl., Montreal QC, H3A 3G4, Canada |

Document Version History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | - First version based on document 11-04/0970r4 | 12 October 2004 |
| R1 | - Reorganization of document | 13 October 2004 |
| R2 | - Accepted all changes in document revision history | 13 October 2004 |
| R3 | - Integrated changes from comments received during Oct. 13 TGs teleconference | 26 October 2004 |
| R4 | - Integrated changes from comments received during Nov. 11 TGs teleconference | 11 November 2004 |
| R5 | - Editorial change to add labels to scope items in Section 4. | 15 November 2004 |
| R6 | - Incorporated updates based on comments and discussion in Tuesday evening TGs Ad-Hoc session in San Antonio, Nov. 16. | 16 November 2004 |
| R7 | - Incorporated updates based on comments and discussion in Wednesday afternoon TGs meeting in San Antonio, Nov. 17 | 17 November 2004 |
| R8 | - Suggested document clean-up in preparation for potential consideration as reference document for TGs call for proposals. All scope items listed as TBD were tentatively reclassified as “in-scope” since the TG has not stated that these items are out of scope for consideration. | 17 November 2004 |
| R9 | - Incorporated updates based on comments and discussion in Thursday morning TGs Ad-Hoc session in San Antonio, Nov. 18. | 18 November 2004 |
| R10 | - Incorporated proposed updates from informal ad-hoc discussions | 7 December 2004 |
| R11 | - Incorporated updates based on comments received during Dec. 15 TGs teleconference. Minor editorial changes to clean up document formatting. | 4 January 2005 |
| R12 | - Updated with new document template. Integrated additional MMAC scope items from document 11-04/0968r10. | 16 January 2005 |
| R13 | - Incorporated updates based on comments received during Jan. 18 afternoon TGs session. Replaced all instances of “ESS Mesh” with “WLAN Mesh”. Incorporated in-scope item TOPO_RT_FWD_SCP18, based on contribution 11-05/1586r1. | 18 January 2005 |

Table of Contents

1	Introduction	4

1.1	Document Purpose	4

2	Categories for Organization of Functional Requirements and Scope (Informative)	4

3	Minimum Functional Requirements	4

4	Scope Clarification Organized by Functional Categories for TGs	7

4.1	Mesh Topology Learning, Routing, and Forwarding (TOPO_RT_FWD)	7

4.2	Mesh Security (SECURITY)	8

4.3	Mesh Measurement (MEAS)	9

4.4	Mesh Discovery and Association (DISC_ASSOC)	10

4.5	Mesh Medium Access Coordination (MMAC)	10

4.6	Compatibility to 802.11 Services (SERV_CMP)	12

4.7	Mesh Interworking (INTRWRK)	12

4.8	Mesh Configuration and Management (CFG_MGMT)	13

5	References	14

# Introduction

## Document Purpose

This document specifies the minimum functional requirements that must be addressed by the first draft specification of IEEE 802.11 Task Group s.  It also defines the scope of the efforts undertaken by TGs.  This document is intended to supplement the 802.11s PAR [1] and 5 Criteria [2] documents, which provide the overriding requirements for Task Group s.

The focus of the scope description is to clarify the areas of functionality that will or will not be considered by TGs.  This is intended to provide additional informative guidelines for proposals.  This document does not attempt to further prioritize the “in-scope” functions.  Labeling of individual features as mandatory or optional will be done as a later step.

This document is a working document that presents a “best estimate” summary for the minimum functional requirements and scope of TGs, but which will also evolve and be updated as the TGs effort progresses.

# Categories for Organization of Functional Requirements and Scope (Informative)

The following table includes a list of system functional categories that are relevant for IEEE 802.11s WLAN Mesh Networking, and will be covered by the functional requirements and scope descriptions in this document.   These categories are only informative and intended to help with the organizational structure of the functional requirements and scope items in Section 3 and Section 4.

| Functional Category Label | Functional Category Description |
| --- | --- |
| TOPO_RT_FWD | Mesh Topology Learning, Routing, and Forwarding |
| SECURITY | Mesh Security |
| MEAS | Mesh Measurement |
| DISC_ASSOC | Mesh Discovery and Association |
| MMAC | Mesh Medium Access Coordination |
| SERV_CMP | Compatibility to 802.11 Services |
| INTRWRK | Interworking |
| CFG_MGMT | Mesh Configuration and Management |

Table 1: Functional Categories for TGs

# Minimum Functional Requirements

The following table defines the minimum functional requirements that must be addressed by the first draft specification of IEEE 802.11 Task Group s.  A proposal submitted for consideration to 802.11 TGs and declared to be complete is required to meet the functional requirements defined in this table.  A partial proposal may meet some of the requirements stated here.

A separate comparison criteria document [6] includes a template with an entry for each functional requirement in this table, allowing those submitting proposals to check off which FRs are covered by a proposal and provide references to relevant sections of the proposal where each FR is covered.

Note that the following functional requirement descriptions refer primarily to Mesh Points, which are a class of 802.11 entities including Mesh APs, as defined in [4].

| Number | Functional Category (see Table 1) | Name | Requirement | From PAR | Notes (informative) |
| --- | --- | --- | --- | --- | --- |
| FR1 | TOPO_RT_FWD | Mesh Topology Discovery | The proposal shall enable automatic topology learning, including the status and quality of mesh links between Mesh Points. | X | This functional requirement is focused on the peer-to-peer discovery of Mesh Points within a WLAN Mesh. This may involve partial or full topology discovery, as required by mesh services. |
| FR2 | TOPO_RT_FWD | Mesh Routing Protocol | At least one MAC address-based protocol and algorithm shall be defined for dynamic auto-configuration of MAC-layer data delivery paths between Mesh APs and Mesh Points over 802.11 wireless links (including across multiple wireless hops). | X | Routing protocol(s) should support unicast, broadcast, and multicast (may be provided by a single protocol or multiple protocols). The term dynamic encompasses mobility of mesh points, RF propagation variations, etc. |
| FR3 | TOPO_RT_FWD | Extensible Mesh Routing Architecture | The proposal shall define a protocol architecture to allow for alternative path selection metrics and/or routing protocols based on application requirements. | X | The intention of this requirement is to allow different networks to use different protocols, not to require a method by which a single network could use more than one protocol simultaneously. |
| FR4 | TOPO_RT_FWD | Mesh Broadcast Data Delivery | The proposal shall enable MAC-layer broadcast/multicast data delivery across the WLAN Mesh. | X |  |
| FR5 | TOPO_RT_FWD | Mesh Unicast Data Delivery | The proposal shall enable MAC-layer unicast data delivery across the WLAN Mesh. | X |  |
| FR6 | TOPO_RT_FWD | Support for Single and Multiple Radios | The proposal shall support the use of one or more IEEE 802.11 physical-layer transcievers on Mesh Points. | X | A proposal should not preclude the use of a single radio. WLAN Mesh should work with single radio devices but should allow for improved performance with multi-radio devices. |
| FR7 | TOPO_RT_FWD | Mesh Network Size | The proposal shall support 32 Mesh Points in the WLAN Mesh. | X | A target configuration is up to 32 devices participating as AP forwarders in the WLAN Mesh. However, larger configurations may also be contemplated by the standard. |
| FR8 | SECURITY | Mesh Security | The proposal shall utilize IEEE 802.11i security mechanisms, or an extension thereof, for the purpose of securing a WLAN Mesh in which all of the Mesh Points and Mesh Portals are controlled by a single logical administrative entity for security. | X | It is assumed that the single logical administrative entity enables the authentication of Mesh Points within a WLAN Mesh. Mesh Points within a secure 11s mesh have an assumed relationship, e.g. all may belong to a home owner (residential) or all may be under the control of an IT department (office). |
| FR9 | MEAS | Radio-Aware Routing Metrics | At least one radio-aware routing metric shall be defined for use by the mesh routing protocol(s). | X | Radio-aware routing metrics may include all aspects of MAC/PHY statistics such as data rate encoding, PER, latency, noise, and other statistics specified by TGk. The term “radio-aware routing metric” is not intended to limit the type of information that could be included in a metric. |
| FR10 | SERV_CMP | Backwards compatibility with legacy BSS and STA | The proposal shall not require modification to the STA’s relationship with the BSS and ESS. | X |  |
| FR11 | SERV_CMP | Use of WDS 4-Addr Frame or Extension | The proposal shall support data delivery in a WLAN Mesh network using the four-address frame format or an extension. | X |  |
| FR12 | DISC_ASSOC | Discovery and Association with a WLAN Mesh | The proposal shall support the discovery of and association with a WLAN Mesh network by Mesh Points. |  |  |
| FR13 | MMAC | Amendment to MAC with no PHY changes required | The proposal shall not require modification to the 802.11 physical layer. |  |  |
| FR14 | INTRWRK | Compatibility with higher-layer protocols | An IEEE 802.11 WLAN Mesh Network shall appear to higher layers as a current style IEEE 802 LAN segment. |  | A WLAN Mesh network should be able to connect with other IEEE 802 LANs using IEEE 802.1D and should be compatible with IP. (See 802.11 Clause 5.1.1.4). This requirement relates to the overall 802.11s system (collection of Mesh Points), not necessarily to individual Mesh Points. |

Table 2: Minimum Functional Requirements for TGs

# Scope Clarification Organized by Functional Categories for TGs

This document defines the scope of the efforts undertaken by TGs.   It indicates which aspects of standards develop are or are not considered by the TGs effort as follows:

- In-scope – aspects that will be considered, addressed, or affected by TGs

- Out-of-scope – aspects that will not be considered or addressed by TGs

- Unaffected – aspects that may co-exist with TGs, but will be unaffected, or essentially preserved, by TGs

- To-be-determined – aspects which are, as yet, TBD (to-be-determined) with respect to TGs

The focus of this section is to clarify the areas of functionality that will or will not be considered by TGs.  This is intended to provide additional informative guidelines for proposals.  This section does not attempt to further prioritize the “in-scope” functions as mandatory or optional.  The in-scope functions listed below are NOT mandatory for inclusion in a proposal and are NOT functional requirements.

## Mesh Topology Learning, Routing, and Forwarding (TOPO_RT_FWD)

The following captures the scope of WLAN Mesh multi-hop routing and frame forwarding support to be considered by TGs.

The in-scope functions listed below are NOT mandatory for inclusion in a proposal and are NOT functional requirements.

| Function |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| TOPO_RT_FWD_SCP1 | - Mesh topology discovery, including Mesh Point neighbor discovery within a WLAN Mesh |  |
| TOPO_RT_FWD_SCP2 | - MAC address-based mesh routing protocols and algorithms |  |
| TOPO_RT_FWD_SCP3 | - MAC-layer mesh broadcast/multicast and unicast data delivery |  |
| TOPO_RT_FWD_SCP4 | - Architecture to support alternative routing protocols and metrics |  |
| TOPO_RT_FWD_SCP5 | - Mesh routing with single-radio devices |  |
| TOPO_RT_FWD_SCP6 | - Mesh routing with multiple-radio devices |  |
| TOPO_RT_FWD_SCP7 | - Use of radio-aware route selection metrics |  |
| TOPO_RT_FWD_SCP8 | - QoS-based route selection |  |
| TOPO_RT_FWD_SCP9 | - Proactive routing |  |
| TOPO_RT_FWD_SCP10 | - On-demand routing |  |
| TOPO_RT_FWD_SCP11 | - Hybrid routing |  |
| TOPO_RT_FWD_SCP12 | - Mesh Point neighbor discovery within a WLAN Mesh via passive scanning |  |
| TOPO_RT_FWD_SCP13 | - Mesh Point neighbor discovery within a WLAN Mesh via active scanning |  |
| TOPO_RT_FWD_SCP14 | - Mesh routing in the presence of low-power (e.g. battery-powered) Mesh Points | - Some usage scenarios may require some mesh point devices to be battery-powered. Energy-aware routing metrics may be necessary in this case. |
| TOPO_RT_FWD_SCP15 | - Support for WLAN Mesh network configurations with more than 32 Mesh Points. |  |
| TOPO_RT_FWD_SCP16 | - Ability to recognize changes in the topology within a bounded time |  |
| TOPO_RT_FWD_SCP17 | - Ability to reconfigure the routing scheme within a bounded time in response to detected changes |  |
| TOPO_RT_FWD_SCP18 | - SAP interfaces and management primitives to provide support to enable higher-layer (e.g., Layer 2.5 and Layer 3) routing protocols | - See 11-05/1586r1 for more information. |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |
|  | - Specification of mesh routing protocols implemented at Layer-3 |  |

## Mesh Security (SECURITY)

The following captures the scope of system security functions to be considered by the TGs.

The in-scope functions listed below are NOT mandatory for inclusion in a proposal and are NOT functional requirements.

| Function |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| SECURITY_SCP1 | - Secure association of Mesh Points to a WLAN Mesh |  |
| SECURITY_SCP2 | - Securing a WLAN Mesh in which all of the Mesh Points are controlled by a single logical administrative entity for security. |  |
| SECURITY_SCP3 | - Secure data message exchange between Mesh Points over mesh links |  |
| SECURITY_SCP4 | - Secure management message exchange between Mesh Points over mesh links |  |
| SECURITY_SCP5 | - Secure topology and routing information exchange between Mesh Points over mesh links |  |
| SECURITY_SCP6 | - Centralized authentication and key management |  |
| SECURITY_SCP7 | - Distributed authentication and key management |  |
| SECURITY_SCP8 | - Hybrid authentication and key management | - Including a mix of centralized and distributed authentication and key management |
| SECURITY_SCP9 | - Static key support | - E.g., pre-shared keys |
| SECURITY_SCP10 | - Dynamic key support | - E.g., IEEE 802.1X |
| SECURITY_SCP11 | - Extension of IEEE 802.11i security mechanisms for mesh |  |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |
|  | - Securing a WLAN Mesh in which all of the Mesh Points are not controlled by a single logical administrative entity for security |  |
|  | - DOS attack |  |
|  | - Rogue MP detection |  |

## Mesh Measurement (MEAS)

The following captures the scope of mesh measurement functions to be considered by the TGs.

The in-scope functions listed below are NOT mandatory for inclusion in a proposal and are NOT functional requirements.

| Function |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| MEAS_SCP1 | - Specification of radio-aware metrics for use by mesh routing protocols | - The radio-aware metrics may encompass, for example, characteristics such as the dynamic variation in quality of Mesh Links. |
| MEAS_SCP2 | - Specification of radio-aware metrics for use by mesh medium access coordination | - The radio-aware metrics may encompass, for example, characteristics such as the dynamic variation in quality of Mesh Links. |
| MEAS_SCP3 | - Mesh link quality measurements | - E.g., delay, jitter, throughput, etc. |
| MEAS_SCP4 | - Mesh path quality measurements | - E.g., delay, jitter, throughput, etc. |
| MEAS_SCP5 | - Measurements to support the use of various types of antennas in a mesh network | - E.g., omni-directional, directional, steerable, etc. |
| MEAS_SCP6 | - Mesh related measurements to aid STAs in making roaming decisions, e.g., WDS capacity currently available for traffic forwarding. |  |
| MEAS_SCP7 | - Measurements to support channel selection within the WDS |  |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |
|  | - Specification of BSS measurements (e.g. 802.11k AP-STA measurements) | - TGs may use relevant measurements defined in TGk or other groups that are relevant for a WLAN Mesh, but the group does not intend to modify BSS-specific measurements |

## Mesh Discovery and Association (DISC_ASSOC)

The following captures the scope of MP and MAP discovery of WLAN Mesh networks and subsequent association with a WLAN Mesh network to be considered by TGs.

The in-scope functions listed below are NOT mandatory for inclusion in a proposal and are NOT functional requirements.

| Function |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| DISC_ASSOC_SCP1 | - Protocols to allow Mesh Points to discover WLAN Mesh networks |  |
| DISC_ASSOC_SCP2 | - Protocols to allow Mesh Points to associate and disassociate with a WLAN Mesh network |  |
| DISC_ASSOC_SCP3 | - Protocols to allow Mesh Points to associate and disassociate with other Mesh Points within a WLAN Mesh |  |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |

## Mesh Medium Access Coordination (MMAC)

The following captures the scope of mesh medium access coordination support to be considered by TGs.

The in-scope functions listed below are NOT mandatory for inclusion in a proposal and are NOT functional requirements.

| Function |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| MMAC_SCP1 | - Mitigate performance degradation caused by hidden nodes |  |
| MMAC_SCP2 | - Mitigate performance degradation caused by exposed nodes |  |
| MMAC_SCP3 | - Flow control over multi-hop paths to avoid performance degradation and/or meet QoS goals. | - This may be necessary, for example, to fairly share throughput between mesh points that are near a destination and upstream mesh points that are routing through mesh points near a destination. Flows may be defined, for example, by 802.11 addresses, QoS control fields, etc. |
| MMAC_SCP4 | - Coordinating channel access across multiple nodes to avoid performance degradation and/or meet QoS goals in the multi-hop network. |  |
| MMAC_SCP5 | - Traffic prioritization within a WLAN Mesh |  |
| MMAC_SCP6 | - Enhancements to make the MAC work well across a range of different network sizes, usage models, etc. |  |
| MMAC_SCP7 | - Mesh link communication coordination | - E.g. coordination between two mesh neighbors to allow them to communicate with each other and/or avoid interfering with each other. This could include, for example, the use of different channels or time schedules to avoid interference between communicating Mesh Point pairs. |
| MMAC_SCP8 | - Support for admission control to determine if a particular flow can be admitted into the mesh network based on the availability of resources and existing usages. |  |
| MMAC_SCP9 | - Mangement of multiple classes of traffic, for example when both BSS traffic and mesh forwarding traffic are present in one device (e.g. in a Mesh AP). | See 11-04/0968r10 section 4.8 for a discussion of this topic. |
| MMAC_SCP10 | - Rate control enhancements for efficient multicasting and broadcasting in a mesh network. | See 11-04/0968r10 section 4.12 for a discussion of this topic. |
| MMAC_SCP11 | - Improving spatial reuse in a mesh network. | See 11-04/0968r10 section 4.1 for a discussion of this topic. |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |
|  | - Modifications to 802.11 physical layer |  |
|  | - MAC extension to support the use of dynamic directional antenna. | - Likely requires high complexity, e.g. TDMA scheduling to know when a neighbor’s antenna is pointed at a node. |

## Compatibility to 802.11 Services (SERV_CMP)

The following captures the scope of 802.11s service integration to be considered by TGs.

The in-scope functions listed below are NOT mandatory for inclusion in a proposal and are NOT functional requirements.

| Function |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| SERV_CMP_SCP1 | - Mesh Point DS Services (DSS) Integration | - Ensure an ESS implemented with a WLAN Mesh can continue to support distribution service, integration service, association service, re-association service, disassociation service in infrastructure mode to enable unicast, multicast and broadcast data delivery for the associated STAs |
| SERV_CMP_SCP2 | - WLAN Mesh compatibility with STA mobility/roaming | - E.g., delivery of traffic to correct Mesh AP to reach a STA. |
| SERV_CMP_SCP3 | - Techniques to allow WLAN Mesh to meet 802.11r system requirements | - E.g., performance, security, etc. |
| SERV_CMP_SCP4 | - Dissemination of STA-to- destination MeshAP or STA-to-destination Mesh Portal routing information in the WLAN Mesh. | - E.g., to support proxy ARP |
| SERV_CMP_SCP5 | - Securing all data frames in the WDS |  |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |
|  | - Changes to 802.11 BSS STA functionality |  |
|  | - 802.11e DLP | - DLP is defined within a BSS |
|  | - STA roaming solutions (when to roam, where to roam, how to roam) | - Covered by TGr? |

## Interworking (INTRWRK)

The following captures the scope of interworking functions to be considered by TGs.

The in-scope functions listed below are NOT mandatory for inclusion in a proposal and are NOT functional requirements.

| Function |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| INTRWRK_SCP1 | - Allow a WLAN Mesh to interface with higher layer protocols |  |
| INTRWRK_SCP2 | - Interfacing a WLAN Mesh with other IEEE 802 LANs | - Including, but not limited to, interfacing with 802.3, 802.11, 802.16, and other 802.11s WLAN Mesh LANs |
| INTRWRK_SCP3 | - Support for interfacing a WLAN Mesh with other IEEE 802 LANs using 802.1D | - E.g. something along the lines of 802.17 Annex F |
| INTRWRK_SCP4 | - Support for efficient utilization of multiple Mesh Portals in a single WLAN Mesh. | - E.g. for load balancing, redundancy, etc. |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |

## Mesh Configuration and Management (CFG_MGMT)

The following captures the scope of configuration and management to to be considred by TGs.

The in-scope functions listed below are NOT mandatory for inclusion in a proposal and are NOT functional requirements.

| Function |  | Notes (Informative) |
| --- | --- | --- |
| In-scope |  |  |
| CFG_MGMT_SCP1 | - Protocol extensions to support self-configuring formation of a WLAN Mesh network |  |
| CFG_MGMT_SCP2 | - Interfaces to support 802.11h DFS compliancy | - Must enable compliance to the existing 802.11h DFS requirements |
| CFG_MGMT_SCP3 | - Interfaces and parameter exchange to enable RF auto configuration support | Support the following parameters exchange over the mesh link between MPs - parameters exchange to support Tx power control - parameters exchange to support channel and antenna selection to minimize interference and to maximize coverage and throughput |
| CFG_MGMT_SCP4 | - Support for managed network management model |  |
| CFG_MGMT_SCP5 | - Support for unmanaged network management model | - i.e., zero-configuration |
| CFG_MGMT_SCP6 | - Interfaces and parameter exchange to exchange information about the capabilities of Mesh Point devices | - For example, information about the power capability of a device may need to be exchanged (battery powered vs. line powered) |
| CFG_MGMT_SCP7 | - Mesh network channel selection | - E.g., TGs may consider including a recommendation on how to achieve consensus on a channel or channels in a mesh network to enable interoperability |
| CFG_MGMT_SCP8 | - Mesh network Tx power control | - E.g., TGs may consider including a recommendation on how to tune transmit power, or other RF parameters, on Mesh Points to improve performance. |
| CFG_MGMT_SCP9 | - QoS policy and management | - E.g., TGs may consider recommendations or perhaps specify limitations or boundaries of QoS policies |
| CFG_MGMT_SCP10 | - Support for time synchronization of Mesh Points if required by mesh services | - If methods are proposed that require time syncronization, the method for time synchronization should be specified |
| Unaffected |  |  |
| To-be-determined |  |  |
| Out-of-scope |  |  |
|  | - Algorithms to steer directional antennas. |  |
|  | - IP address assignment |  |
|  | - Protocols for remote management of a WLAN Mesh Network | - Support for remote management protocols may be considered, but TGs does not intend to define a protocol to replace existing management protocols such as SNMP |
|  | - 802.11F |  |

# References

[1] IEEE 802 11-04/54r2, PAR for IEEE 802.11s ESS Mesh

[2] IEEE 802 11-04/56r1, Five Criteria for IEEE 802.11s ESS Mesh

[3] IEEE 802 11-04/662r11, TGs Usage Models

[4] IEEE 802.11-04/1477r3, Terms and Definitions for 802.11s

[5] IEEE 802.11-04/970r4, Proposed 802.11 TGs Scope

[6] IEEE 802.11-04/1175r10 TGs Comparison Categories and Informative Checklists

[7] IEEE 802.11-04/968r10, Issues for Mesh Media Access Coordination Component in 11s