IEEE P802.11
Wireless LANs

Draft IEEE 802.11 TGs Comparison Categories and Criteria

Date:	November 17, 2004

Author:	W. Steven Conner
	Intel Corporation
	2111 NE 25th Ave, Hillsboro, OR 97124, USA
	Phone: +1 503 264 8036
	e-Mail: w.steven.conner@intel.com

Abstract

This document specifies comparison criteria that must be addressed by any proposal in response to the IEEE 802.11 TGs call for proposals.  This document is intended to provide categories for comparison of proposals and a recommendation for data to include with proposals.

Document Version History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | - First strawman document version | 13 October 2004 |
| R1 | - Added additional informative text and initial category teamplates | 27 October 2004 |
| R2 | - Incorporated updates based on comments during Oct. 27 TGs teleconference | 28 October 2004 |
| R3 | - Updated to align with the latest version of the functional requirements and scope document 11-04/1174r4. | 15 November 2004 |
| R4 | - Incorporated updates based on comments discussion in Tuesday evening TGs Ad-Hoc session in San Antonio, Nov. 16. | 16 November 2004 |
| R5 | - Updated to align with the latest version of the functional requirements and scope document 11-04/1174r7. | 17 November 2004 |

Table of Contents

1	Introduction	2

1.1	Purpose of document	2

1.2	Relationship to Functional Requirements and Scope	3

2	Definitions	3

3	Additional Supporting Material	3

4	Coverage of Minimum Functional Requirements	3

5	Coverage of In-Scope Functionalities	4

6	Applicability to Usage Scenarios	8

7	Quantitative Comparison Criteria	9

8	References	10

# Introduction

## Purpose of document

This document provides categories for comparison of proposals and a recommendation for data to include with proposals that are submitted to 802.11 TGs.

This document is intended to provide a recommended structure for convenient comparison of features and characteristics of different proposals.  It provides recommended guidelines to developers of proposals on the types of characteristics that TGs members will evaluate when selecting one or more winning proposals.

This document does not have the goal of specifying an exhaustive collection of detailed simulation scenarios and measurements for comparison.  It is the responsibility of those submitting proposals to TGs to convince the task group of the technical merits of their proposal.  Proposers are welcome to provide additional quantitative and/or qualitative data to that specified in this document.

Section 3 through Section 7 of this document provide templates to be filled out by those submitting proposals to 802.11 TGs.  The templates in sections 3, 4, 5, and 6 are check lists of what is included in a proposal along with references on where to find relevant materials for each category.  Section 7 defines quantitative metrics for comparing and evaluating proposals and provides a template for listing references for relevant quantitative descriptions and data.

In the case of partial proposals, only the relevant template sections must be filled in.

## Relationship to Functional Requirements and Scope

The main purpose of the comparison criteria is to define categories and metrics to enable comparison of TGs proposals, while the functional requirements and scope document [5] specifies the minimum functional requirements that must be addressed by IEEE 802.11s and clarifies the scope of efforts undertaken by TGs.

# Definitions

| Term | Definition |
| --- | --- |

# Additional Supporting Material

This section contains requirements for additional documentation that should be submitted with a proposal.  This is a template that should be filled in and included with a proposal submission.

| Number | Name | Definition | Coverage (Yes/No) |  | Notes | References |
| --- | --- | --- | --- | --- | --- | --- |
| AD1 | Reference submissions | A list of IEEE submissions related to the proposal, both documents and presentations. |  |  |  |  |
| AD2 | Simulation and/or experimental methodology | Any proposal submission that includes simulation results must include a description of the simulation methodology used for mesh simulations. The simulation methodology documentation should provide enough information to, in principle, reproduce the simulation (e.g., including node positions, traffic and propagation model (including PHY assumptions), packet sizes, etc.). |  |  |  |  |

# Coverage of Minimum Functional Requirements

This section contains a template for disclosure of coverage of minimum functional requirements with a proposal.  See [6] for detailed definitions of functional requirements.  This template should be filled in and included with a proposal submission.

[Editor Note: This template should be synchronized with Section 3 in 11-04/1174 when the CFP is issued.]

| Number | Category | Name | Coverage (Yes/No) | Notes |  | References |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | TOPO_RT_FWD | Mesh Topology Discovery |  |  |  |  |
| FR2 | TOPO_RT_FWD | Mesh Routing Protocol |  |  |  |  |
| FR3 | TOPO_RT_FWD | Extensible Mesh Routing Architecture |  |  |  |  |
| FR4 | TOPO_RT_FWD | Mesh Broadcast Data Delivery |  |  |  |  |
| FR5 | TOPO_RT_FWD | Mesh Unicast Data Delivery |  |  |  |  |
| FR6 | TOPO_RT_FWD | Support for Single and Multiple Radios |  |  |  |  |
| FR7 | TOPO_RT_FWD | Mesh Network Size |  |  |  |  |
| FR8 | SECURITY | Mesh Security |  |  |  |  |
| FR9 | MEAS | Radio-Aware Routing Metrics |  |  |  |  |
| FR10 | SERV_CMP | Backwards compatibility with legacy BSS and STA |  |  |  |  |
| FR11 | SERV_CMP | Use of WDS 4-Addr Frame or Extension |  |  |  |  |
| FR12 | DISC_ASSOC | Discovery and Association with an ESS Mesh |  |  |  |  |
| FR13 | MMAC | Amendment to MAC with no PHY changes required |  |  |  |  |

# Coverage of In-Scope Functionalities

This section contains a template for disclosure of coverage of in-scope functionalities with a proposal.  See [6] for detailed description of in-scope functionalities considered by TGs.  This template should be filled in and included with a proposal submission.

This purpose of this template is primarily to aid the reader in identifying which in-scope features are covered by a proposal, and which sections of the proposal are relevant to each feature.

[Editor Note: This template should be synchronized with Section 4 in 11-04/1174 when the CFP is issued.]

| Number | Name | Coverage Yes/No | Notes | References |  |
| --- | --- | --- | --- | --- | --- |
| Mesh Topology Learning, Routing, and Forwarding (TOPO_RT_FWD) |  |  |  |  |  |
| TOPO_RT_FWD_SCP1 | - Mesh topology discovery, including MP and MAP neighbor discovery within an ESS Mesh |  |  |  |  |
| TOPO_RT_FWD_SCP2 | - MAC address-based mesh routing protocols and algorithms |  |  |  |  |
| TOPO_RT_FWD_SCP3 | - Layer2 mesh broadcast/multicast and unicast data delivery |  |  |  |  |
| TOPO_RT_FWD_SCP4 | - Architecture to support alternative routing protocols and metrics |  |  |  |  |
| TOPO_RT_FWD_SCP5 | - Mesh routing with single-radio devices |  |  |  |  |
| TOPO_RT_FWD_SCP6 | - Mesh routing with multiple-radio devices |  |  |  |  |
| TOPO_RT_FWD_SCP7 | - Use of radio-aware route selection metrics |  |  |  |  |
| TOPO_RT_FWD_SCP8 | - QoS-based route selection |  |  |  |  |
| TOPO_RT_FWD_SCP9 | - Proactive routing |  |  |  |  |
| TOPO_RT_FWD_SCP10 | - On-demand routing |  |  |  |  |
| TOPO_RT_FWD_SCP11 | - Hybrid routing |  |  |  |  |
| TOPO_RT_FWD_SCP12 | - MP and MAP neighbor discovery within an ESS Mesh via passive scanning |  |  |  |  |
| TOPO_RT_FWD_SCP13 | - MP and MAP neighbor discovery within an ESS Mesh via active scanning |  |  |  |  |
| TOPO_RT_FWD_SCP14 | - Mesh routing in the presence of low-power (e.g. battery-powered) Mesh Points |  |  |  |  |
| TOPO_RT_FWD_SCP15 | - Support for ESS Mesh network configurations with more than 32 Mesh Points. |  |  |  |  |
| TOPO_RT_FWD_SCP16 | - Ability to recognize changes in the topology within a bounded time |  |  |  |  |
| TOPO_RT_FWD_SCP17 | - Ability to reconfigure the routing scheme within a bounded time in response to detected changes |  |  |  |  |
| Mesh Security (SECURITY) |  |  |  |  |  |
| SECURITY_SCP1 | - Secure association of Mesh Points and Mesh APs to an ESS Mesh |  |  |  |  |
| SECURITY_SCP2 | - Securing an ESS Mesh in which all of the MPs and MAPs are controlled by a single logical administrative entity for security. |  |  |  |  |
| SECURITY_SCP3 | - Secure data message exchange between MPs and MAPs over mesh links |  |  |  |  |
| SECURITY_SCP4 | - Secure management message exchange between MPs and MAPs over mesh links |  |  |  |  |
| SECURITY_SCP5 | - Secure topology and routing information exchange between MPs and MAPs over mesh links |  |  |  |  |
| SECURITY_SCP6 | - Centralized key distribution |  |  |  |  |
| SECURITY_SCP7 | - Distributed key distribution |  |  |  |  |
| SECURITY_SCP8 | - Hybrid key distribution |  |  |  |  |
| SECURITY_SCP9 | - Static key support |  |  |  |  |
| SECURITY_SCP10 | - Dynamic key support |  |  |  |  |
| SECURITY_SCP11 | - Extension of IEEE 802.11i security mechanisms for mesh |  |  |  |  |
| Mesh Measurement (MEAS) |  |  |  |  |  |
| MEAS_SCP1 | - Specification of radio-aware metrics for use by mesh routing protocols |  |  |  |  |
| MEAS_SCP2 | - Specification of radio-aware metrics for use by mesh medium access coordination |  |  |  |  |
| MEAS_SCP3 | - Mesh link quality measurements |  |  |  |  |
| MEAS_SCP4 | - Mesh path quality measurements |  |  |  |  |
| MEAS_SCP5 | - Measurements to support the use of directional antennas in a mesh network |  |  |  |  |
| MEAS_SCP6 | - Mesh related measurements to aid STAs in making roaming decisions, e.g., WDS capacity currently available for traffic forwarding. |  |  |  |  |
| Mesh Discovery and Association (DISC_ASSOC) |  |  |  |  |  |
| DISC_ASSOC_SCP1 | - Protocols to allow MPs and MAPs to discover ESS Mesh networks |  |  |  |  |
| DISC_ASSOC_SCP2 | - Protocols to allow MPs and MAPs to associate with ESS Mesh networks |  |  |  |  |
| DISC_ASSOC_SCP3 | - Protocols to allow MPs and MAPs to associate with other MPs and MAPs within an ESS Mesh |  |  |  |  |
| Mesh Medium Access Coordination (MMAC) |  |  |  |  |  |
| MMAC_SCP1 | - Mitigate performance degradation caused by hidden nodes |  |  |  |  |
| MMAC_SCP2 | - Mitigate performance degradation caused by exposed nodes |  |  |  |  |
| MMAC_SCP3 | - Mitigate performance degradation caused by multi-hop message forwarding |  |  |  |  |
| MMAC_SCP4 | - Flow control over multi-hop paths to avoid performance degradation caused by local congestion. |  |  |  |  |
| MMAC_SCP5 | - Scheduling across multiple nodes to achieve higher throughput and lower delay in the multi-hop network. |  |  |  |  |
| MMAC_SCP6 | - Traffic prioritization within an ESS Mesh |  |  |  |  |
| MMAC_SCP7 | - Enhancements to make the MAC work well across a range of different network size, usage model, etc. |  |  |  |  |
| MMAC_SCP8 | - Peer-to-peer mesh link communication coordination |  |  |  |  |
| MMAC_SCP9 | - Extensions to 802.11e MAC to support QoS in an ESS Mesh |  |  |  |  |
| MMAC_SCP10 | - Statistics to determine if a particular flow can be admitted into the mesh network based on the availability of the bandwidth resource and existing usages. |  |  |  |  |
| Compatibility to 802.11 Services (SERV_CMP) |  |  |  |  |  |
| SERV_CMP_SCP1 | - Mesh Point DS Services (DSS) Integration |  |  |  |  |
| SERV_CMP_SCP2 | - ESS Mesh compatibility with STA mobility/roaming |  |  |  |  |
| SERV_CMP_SCP3 | - Techniques to allow ESS Mesh to meet 802.11r performance requirements |  |  |  |  |
| SERV_CMP_SCP4 | - Dissemination of STA/MeshAP mapping information in the ESS Mesh |  |  |  |  |
| Mesh Interworking (INTRWRK) |  |  |  |  |  |
| INTRWRK_SCP1 | - Compatibility with higher layer protocols |  |  |  |  |
| INTRWRK_SCP2 | - Interfacing an ESS Mesh with other IEEE 802 LANs |  |  |  |  |
| INTRWRK_SCP3 | - Support for interfacing an ESS Mesh with other IEEE 802 LANs using 802.1D |  |  |  |  |
| INTRWRK_SCP4 | - Support for efficient utilization of multiple Mesh Portals in a single ESS Mesh. |  |  |  |  |
| Mesh Configuration and Management (CFG_MGMT) |  |  |  |  |  |
| CFG_MGMT_SCP1 | - Protocol extensions to support self-configuring formation of an ESS Mesh network |  |  |  |  |
| CFG_MGMT_SCP2 | - Interfaces to support 802.11h DFS compliancy |  |  |  |  |
| CFG_MGMT_SCP3 | - Interfaces and parameter exchange to enable RF auto configuration support |  |  |  |  |
| CFG_MGMT_SCP4 | - Support for managed network management model |  |  |  |  |
| CFG_MGMT_SCP5 | - Support for unmanaged network management model |  |  |  |  |
| CFT_MGMT_SCP6 | - Interfaces and parameter exchange to exchange information about the capabilities of Mesh Point devices |  |  |  |  |
| CFG_MGMT_SCP7 | - Mesh network channel selection |  |  |  |  |
| CFG_MGMT_SCP8 | - Mesh network Tx power control |  |  |  |  |
| CFG_MGMT_SCP9 | - QoS policy and management |  |  |  |  |

# Applicability to Usage Scenarios

This section contains a template for reporting usage scenarios for which the proposer believes the solution is relevant.  Proposals should provide evidence of meeting the soft requirements from the Usage Models document [3].  This template should be filled in and included with a proposal submission.

It is not mandatory to prove the applicability of a proposal to any of the following usage scenarios.  If simulation results are included with a proposal, simulation scenarios relating to the following usage models are preferred.  If simulation results are include with the proposal, a detailed simulation methodology must also be included (See AD2: Simulation and/or experimental methodology in Section 3).

[Editor Note: Should this document reference example simulation scenarios for each usage model category?]

| Number | Name | Definition | Data Included? Yes/No |  | Notes |  | References |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UM1 | Residential | Include a description of how the proposal is applicable to residential usage scenarios, as described in [3]. |  |  |  |  |  |
| UM2 | Office | Include a description of how the proposal is applicable to office usage scenarios, as described in [3]. |  |  |  |  |  |
| UM3 | Campus/Community/Public Access Networks | Include a description of how the proposal is applicable to campus/community/public access usage scenarios, as described in [3]. |  |  |  |  |  |
| UM4 | Public Safety/ Military | Include a description of how the proposal is applicable to public safety/military usage scenarios, as described in [3]. |  |  |  |  |  |

# Quantitative Comparison Criteria

This section contains a template for reporting quantitative results relating to a proposal.  This list provides a list of quantitative metrics that are considered useful for comparing proposals to TGs.   It is expected that the comparison process for proposals will be iterative, and more detailed comparison methods may be added later as necessary.

This template should be filled in and included with a proposal submission.  Proposers are welcome to provide additional quantitative data to that specified in this section.

If simulation or experimental results are included with the proposal, a detailed simulation methodology must also be included (See AD2: Simulation and/or experimental methodology in Section 3).  In addition to simulations, a proposal may also include analytical results with clearly documented scenarios and assumptions.

[Editor Note: The metrics listed in this template should be updated with non-ambiguous definitions]

| Number | Name | Definition | Data Included? Yes/No | Notes | References |
| --- | --- | --- | --- | --- | --- |
| QC1 | Routing complexity | Memory, computation, communication complexity as a function of number of Mesh Points, number of STAs, number of active Mesh Paths, diameter and degree of the topology graph |  |  |  |
| QC2 | Routing convergence and recovery | Time to discover a route, detect a link or node failure or performance metric change, repair a route due to failure or metric change as a function of number of Mesh Points, number of STAs, number of active Mesh Paths, diameter and degree of the topology graph, rate of change of the topology |  |  |  |
| QC3 | Data delivery | End-to-end throughput, latency, reliability for traffic flows across the mesh as a function of the number of Mesh Points, number of active Mesh Paths, number of simultaneous traffic flows, diameter and degree of the topology graph, rate of change of the topology |  |  |  |

# References

[1] IEEE 802 11-04/54r2, PAR for IEEE 802.11s ESS Mesh

[2] IEEE 802 11-04/56r1, Five Criteria for IEEE 802.11s ESS Mesh

[3] IEEE 802 11-04/662r10, TGs Usage Models

[4] IEEE 802.11-04/969r2, Draft Terms and Definitions for 802.11s

[5] IEEE 802.11-04/1174r7, Draft 802.11 TGs Functional Requirements and Scope