IEEE P802.11
Wireless LANs

| Proposal for a Dynamic Backbone Mesh |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-06-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dennis J. Baker |  | 100 Brickells Glade Edenton, NC 27932 | 252-482-0747 | d.baker@mchsi.com |
| James P. Hauser | Naval Research Laboratory | Code 5521 Washington, DC 20375 | 202-767-2771 | hauser@itd.nrl.navy.mil |

Abstract

The 802.11 specification describes the concept of an Extended Service Set (ESS), which consists of multiple Basic Service Sets (BSSs) connected by a (largely) unspecified Distribution System (DS). This paper proposes an 802.11 amendment to specify a wireless DS mesh that uses normal 802.11 PHY layers. The resultant wireless mesh can serve as all of the DS or it may be only part of a larger DS. The proposed approach allows the WLAN mesh to be implemented using only a single 802.11 channel, although multiple channels can also be supported. Mobility effects are handled completely within the mesh, and, to externally attached networks, the WLAN mesh resembles a static, broadcast ethernet LAN segment.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

| Number | Category | Name | Coverage (Complete /Partial/ None) | Notes | References |
| --- | --- | --- | --- | --- | --- |
| FR1 | TOPO_RT_FWD | Mesh Topology Discovery | C |  |  |
| FR2 | TOPO_RT_FWD | Mesh Routing Protocol | C |  |  |
| FR3 | TOPO_RT_FWD | Extensible Mesh Routing Architecture | C |  |  |
| FR4 | TOPO_RT_FWD | Mesh Broadcast Data Delivery | C |  |  |
| FR5 | TOPO_RT_FWD | Mesh Unicast Data Delivery | C |  |  |
| FR6 | TOPO_RT_FWD | Support for Single and Multiple Radios | C |  |  |
| FR7 | TOPO_RT_FWD | Mesh Network Size | C |  |  |
| FR8 | SECURITY | Mesh Security | N |  |  |
| FR9 | MEAS | Radio-Aware Routing Metrics | P |  |  |
| FR10 | SERV_CMP | Backwards compatibility with legacy BSS and STA | C |  |  |
| FR11 | SERV_CMP | Use of WDS 4-Addr Frame or Extension | C |  |  |
| FR12 | DISC_ASSOC | Discovery and Association with a WLAN Mesh | P |  |  |
| FR13 | MMAC | Amendment to MAC with no PHY changes required | C |  |  |
| FR14 | INTRWRK | Compatibility with higher-layer protocols | C |  |  |

# 1 Overview

1.1 Mesh design goals

- Minimal impact on existing STA and AP implementations

- Extensible design (e.g., multiple channels, directional antennas, QoS, …)

- Support for wide range of use-cases (e.g., static and dynamic topologies)

- Emulate static, broadcast Ethernet to external world

- Provide techniques to avoid mutual interference caused by mesh management traffic

1.2 Assumptions

This (partial) proposal makes the following assumptions:

- Wireless mesh frames are distinguishable from other 4-address WDS frames by a SNAP header with a unique ethernet protocol type (ETH_P_80211_MESH).

- A mesh information element exists that can be inserted into beacons for the purpose of advertising mesh capabilities. At a minimum, this information element will contain a “mesh identifier” (MID) field.

- All mesh points periodically transmit beacons to advertise their existence and capabilities.

Although this proposal does not address security, we envision that mesh point startup will proceed somewhat along the following lines:

- At startup, the mesh point will scan for beacons identifying other mesh points within the mesh it desires to join.

- If no other mesh points are found, it will start a new mesh.

- Otherwise, it will select one of the neighboring mesh points and begin a mutual authentication process with the selected mesh point.

- Through the “entry” mesh point it will perform mutual authentication with other mesh points within the mesh.

- It will begin sending mesh traffic and decoding mesh traffic sent from other mesh points with which it is mutually authenticated.

# 2 Definitions

authenticated mesh point - A mesh point that has been authenticated as a valid participant in the WLAN mesh. The authentication is with repect to a common policy determined by a single administrative entity.

backbone connection node (BCN): In the Dynamic Backbone Algorithm, this refers to the backbone node chosen by a non-backbone node as its primary connection to the backbone.

connected mesh - The status of the WLAN mesh in which all mesh points that are participating members of a WLAN mesh are reachable.

disconnected mesh - The status of the WLAN mesh in which a subset of mesh points that are participating members within the WLAN mesh are not reachable. It is also called a partitioned mesh.

mesh AP  - Any mesh point that is also an access point.

mesh association - The service used to establish the mesh point membership within a WLAN mesh. Mesh association is independent from STA association to an AP.

mesh broadcast - Frame forwarding mechanism for transporting MSDUs to all mesh points within a WLAN mesh.

mesh coordination function - A logical function used to coordinate use of mesh resources by mesh points.

mesh identifier - A unique identifier for a WLAN mesh.

mesh link - A bidirectional IEEE 802.11 link between two mesh points.

mesh link metric - A criterion used to characterize the performance/quality/eligibility of a mesh link as a member of a mesh path. A mesh link metric may be used in a computation of a path metric.

mesh management frame - Frame defined for managing and operating the mesh. The frame is sent between mesh points, e.g. for path determination, neighbor discovery, topology discovery, etc. This definition of message is intended to be generic and does not specify the form of conveyor.

mesh member - An associated mesh point.

mesh member set - The set of associated mesh points within a WLAN mesh.

mesh multicast - Frame forwarding mechanism for transporting MSDUs to a group of mesh points within a WLAN mesh.

mesh neighbor - Any mesh point that is directly connected to another mesh point with a mesh link.

mesh path - A concatenated set of connected mesh links from a source mesh point to a destination mesh point.

mesh path selection - The process of selecting mesh paths.

mesh point - Any IEEE 802.11 entity that contains an IEEE 802.11–conformant Medium Access Control (MAC) and Physical Layer (PHY) interface to the Wireless Medium (WM), that is within a WLAN mesh, and that supports WLAN mesh services.

mesh portal  - A point at which MSDUs exit and enter a WLAN mesh to and from other parts of a DS or to and from a non-802.11 network. A mesh portal can be collocated with an IEEE 802.11 portal.

mesh service area - The conceptual area within which members of a WLAN mesh may communicate.

mesh topology - A graph consisting of the full set of mesh points and mesh links in a WLAN mesh.

mesh unicast - Frame forwarding mechanism for transporting MSDUs to an individual mesh point within a WLAN mesh.

path metric - Criterion used for mesh path selection.

wlan mesh – A WLAN mesh (previously known as ESS mesh) is an IEEE 802.11-based WDS which is part of a DS, consisting of a set of two or more mesh points interconnected via IEEE 802.11 links and communicating via the WLAN mesh services. A WLAN mesh may support zero or more entry points (mesh portals), automatic topology learning and dynamic path selection (including across multiple hops).

wlan mesh services – The set of services provided by the WLAN mesh that support the control, management, and operation of the WLAN mesh, including the transport of MSDUs between mesh points within the WLAN mesh. WLAN mesh services supplement DSS (Distribution System Services).

3 Abbreviations, and acronyms

AP		access point

ARP		address resolution protocol

BCN		backbone connection node

BSS		basic service set

DBA		dynamic backbone algorithm

DBM		dynamic backbone mesh

DMPID		MPID of destination of a mesh frame

DS		distribution system

DSM		distribution system medium

DSS		distribution system services

ESS		extended service set

IEEE		Institute of Electrical and Electronic Engineers

IP		internet protocol

LAN		local area network

LLC		logical link control

LQI		link quality indicator

LSA		link state advertisement

LSR		link state report

MAC		media access control

MCF		mesh coordination function

MID		mesh identifier

MAP		mesh access point

MP		mesh point

MPID		mesh point identifier

MPIDn		MPID = n

MSDU		MAC service data unit

MTSF		mesh timing synchronization function

PHY		physical (layer)

RMPID		MPID of designated receiver of mesh frame

SMPID		MPID of source of a mesh frame

SNAP		Sub-network access protocol

STA		station

TBD		to be determined

TMPID		MPID of transmitter of a mesh frame

TSF		timing synchronization function

TU		time unit

WDS		wireless distribution system

WLAN		wireless local area network

WM		wireless media

# 4 Proposed Dynamic Backbone Mesh (DBM) architecture

Hidden from external view but essential to understanding the operation of the proposed mesh is the concept of a dynamically maintained mesh backbone architecture. Figure s1 is an example of the proposed architecture. The main features of DBM are as follows:

- A single backbone structure is shared by the entire mesh.

- Periodically a new backbone structure is “installed” (which may be identical to the old one).

- Mesh points assume one of two roles: backbone node or non-backbone node.

- Every non-backbone node is bidirectionally connected to a backbone node (its Backbone Connection Node (BCN)) by a BCN link.

- The architecture defines a set of mesh backbone links that connect the set of backbone nodes (if possible).

- The primary role of backbone nodes is to relay broadcast and multicast traffic.

- All mesh links (i.e., backbone links, BCN links, and ordinary links) can be used to pass traffic.

Figure s1 - Proposed Dynamic Backbone Mesh (DBM) architecture.

# 5 Mesh message formats

## 5.1 General mesh frame format

The proposed format for mesh frames is defined in Figure s2, where the mesh frame is shown with its MAC encapsulation and assumed SNAP header. The fields that follow the SNAP header and precede the mesh body are collectively referred to as the mesh header. Mesh frames are carried in the body of 4-address 802.11 WDS data frames. Conceptually, the mesh header is an extension of the 4-address 802.11 WDS header.

| Octets: 30 | 8 | 2 | 1 | 1 | 1 | 1 | 1 | 2 | 0 to 2295 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4-addr MAC (data) | SNAP | mesh control | MID | RMPID | TMPID | DMPID | SMPID | MSEQ | mesh body | FCS |

Figure s2 – Mesh frame format with its MAC encapsulation

## 5.2 Mesh frame fields

### 5.2.1 Mesh frame control field

The subfields within the mesh control field of the mesh header are shown in Figure s3.

| Mesh Protocol Version B0 | Mesh Message Type | Mesh Message Subtype | Source is MP | Destination is MP | Precedence | Reserved B15 |
| --- | --- | --- | --- | --- | --- | --- |
| Bits: 2 | 2 | 4 | 1 | 1 | 3 | 3 |

Figure s3 – Mesh frame control field

#### 5.2.1.1 Mesh protocol version field

The Mesh Protocol Version field is 2 bits in length and is invariant in size and placement across all revisions. The present value of the protocol version is 0. All other values are reserved. A mesh point that receives a frame with a higher revision level than it supports will discard the frame without indication to the sending MP.

#### 5.2.1.2 Mesh message type and subtype fields

The mesh message type field is 2 bits in length, and the subtype field is 4 bits in length.  The mesh message type and subtype fields together identify the function of the mesh message. There are presently two defined mesh message types: mesh management and mesh data. Each of the mesh message types has several defined mesh subtypes. Table s1 defines the valid combinations of mesh message type and mesh message subtype.

Table s1 - Valid mesh message type and subtype combinations 
(numeric values in this table are shown in binary)

| mesh msg type value b3 b2 | mesh msg type description | mesh msg subtype value b7 b6 b5 b4 | mesh msg subtype description |
| --- | --- | --- | --- |
| 00 | management | 0000 | reserved |
| 00 | management | 0001 | DBA frame 1 announcement |
| 00 | management | 0010 | DBA frame 2 announcement |
| 00 | management | 0011 | DBA frame 3 announcement |
| 00 | management | 0100 | DBA frame 4 announcement |
| 00 | management | 0101 | Asynchronous protocol message |
| 00 | management | 0110-1111 | reserved |
| 01 | reserved | 0000-1111 | reserved |
| 10 | data | 0000 | data message |
| 11 | reserved | 0000-1111 | reserved |

#### 5.2.1.3 Source is MP

This flag is 1 if the SADDR of the prepended 802.11 4-address frame header corresponds to a mesh member and 0 otherwise.

#### 5.2.1.4 Destination is MP

This flag is 1 if the DADDR of the prepended 802.11 4-address frame header corresponds to a mesh member and 0 otherwise.

#### 5.2.1.5 Precedence field

This field is used to indicate the transmission precedence of the attached MSDU. The highest precedence level is 7 and the lowest is 0.

#### 5.2.1.6 Reserved

This field is reserved for future use. It should be set to 000 (binary).

### 5.2.2 Mesh identifier

The mesh identifier (MID) is a 1-byte field that uniquely identifies to which mesh within the ESS that this frame belongs. This is the same identifier as that which appears in the mesh information element of mesh point beacons.

### 5.2.3 Mesh point identifier fields

Mesh point identifiers (MPIDs) are unsigned, 1-octet mesh point addresses. MPIDs in the range 0 to 127 are reserved for unicast address, while MPIDs in the range 128 to 255 represent multicast addresses. Table s2 lists the present allocation of mesh point identifiers. When the mesh point identifier represents a unicast address, it can be thought of as a short alias for the MAC address. A unicast MPID may be assigned manually or automatically.

Table s2 - Valid MPID values 
(numeric values in this table are shown in binary)

| Mesh Point Identifier b7 b6 b5 b4 b3 b2 b1 b0 | Type | Multicast group name | Multicast group MAC address (hex) |
| --- | --- | --- | --- |
| 00000000-00011111 | unicast | not applicable | not applicable |
| 00100000-01111110 | unicast, reserved | not applicable | not applicable |
| 01111111 | NO_MPID | not applicable | not applicable |
| 100000000 | multicast | local DS announcement | to be assigned |
| 10000001-10011110 | multicast, reserved | unassigned, reserved | unassigned, reserved |
| 10011111 | multicast | mesh broadcast | to be assigned |
| 10100000-11111110 | multicast, reserved | unassigned, reserved | unassigned, reserved |
| 11111111 | multicast | subnet broadcast | ff:ff:ff:ff:ff:ff |

The mesh header contains four mesh point identifiers: DMPID, SMPID, RMPID, and TMPID, which are described as follows.

#### 5.2.3.1 Destination mesh point identifier (DMPID) field

DMPID is the mesh point identifier of the destination of the mesh frame. If the destination MAC address (DA) is a STA, the DMPID refers to the mesh access point with which the STA is associated. If the MAC DA is a node external to the mesh and its associated STAs, the DMPID refers to the mesh point where the portal to the destination resides.

#### 5.2.3.2 Source mesh point identifier (SMPID) field

SMPID is the mesh point identifier of the originating source of the mesh frame. If the source MAC address (SA) is a STA, the SMPID refers to the mesh access point with which the STA is associated. If the MAC SA is a node external to the mesh and its associated STAs, the SMPID refers to the mesh point containing the portal from which the source MSDU was obtained.

#### 5.2.3.3 Receiver mesh point identifier (RMPID) field

RMPID is the mesh point identifier of the next immediate recipient of a unicast destination DMPID. If DMPID represents a multicast address, the appropriate setting for RMPID is TBD.

#### 5.2.3.4 Transmitter mesh point identifier (TMPID) field

TMPID is the mesh point identifier of the transmitter of the mesh frame.

### 5.2.4 Mesh frame sequence (MSEQ) field

MSEQ, in conjunction with SMPID, serves as an end-to-end identifier of a (possibly) relayed mesh frame.

### 5.2.5 Mesh frame body field

The mesh frame body field is a variable length field that contains information specific to individual mesh message types and subtypes.

## 5.3 Format of individual mesh messages

This section specifies the formats of the mesh messages listed in table s1.

### 5.3.1 Mesh management messages

There are several subtypes of mesh management messages. Most of them have certain required fields. However, after these required fields, arbitrary mesh management information elements can be appended. However, when adding these optional elements, care must be taken that all elements are intended for the same unicast or multicast destination.

#### 5.3.1.1 DBA frame 1 announcement

DBA frames 1 through 4 are “synchronous” frames in the sense that they should be transmitted just after the start of the transmission slot to which they have been assigned. For example, MPID = 2 should transmit its DBA frame transmissions as soon as possible after the start of each slot 2. See Figure s12.

The Dynamic Backbone Algorithm (DBA) defines four message formats that are used to exchange the information required to form and maintain the mesh’s dynamic backbone. This information is exchanged in four consecutive “DBA frames” that are repeated every DBA epoch, as described in section 6.1. DBA messages are sent as “announcements”, that is, these messages are never relayed. Section 6.2 describes the operation of the DBA and explains how the values of the various fields of DBA messages are determined. The period of time from the start of DBA frame 1 until the start of the following DBA frame 1, is called a “DBA epoch”. Figure s4 shows the format of the DBA frame 1 announcements.

| Octets: 4 | 8 |
| --- | --- |
| Probe ack B0 B31 | MTSF B0 B63 |

Figure s4 – DBA frame 1 announcement

##### 5.3.1.1.1 DBA frame 1 probe ack field

This field is used to acknowledge the reception of previous DBA frame 1 transmissions from the current DBA epoch. A “1” in bit position Bn indicates that the transmitting MP has successfully received the DBA frame 1 transmission from MPIDn, whereas a “0” indicates failure to receive this transmission. The transmitting MP places a “0” in its own bit position.

##### 5.3.1.1.2 DBA frame 1 MTSF field

This field is used to indicate the current value of the mesh point’s MTSF.

#### 5.3.1.2 DBA frame 2 announcement

Figure s5 shows the format of all DBA frame 2 announcements.

| Octets: 4 | 1 |
| --- | --- |
| Bidirectional links B0 B31 | own clusterhead |

Figure s5 – DBA frame 2 announcement

##### 5.3.1.2.1 DBA frame 2 bidirectional links field

This 32-bit field is used to indicate MPs that the transmitting MP considers to be directly linked to it via bidirectional links. A “1” in bit Bn indicates that the link to MPIDn is considered to be a bidirectional link, whereas a “0” indicates that the link to MPIDn is not considered to be a bidirectional link. The transmitting MP places a “0” in its own bit position. These “links” are used for the purpose of routing broadcast/multicast traffic.

##### 5.3.1.2.2 DBA frame 2 message own clusterhead field

The DBA frame 2 own clusterhead field contains the MPID of the MP that has been designated by the DBA as the transmitting MP’s own clusterhead.

#### 5.3.1.3 DBA frame 3 announcement

Figure s6 shows the format of all DBA frame 3 announcements.

| Bits: 2 | … | 2 | 8 |
| --- | --- | --- | --- |
| Type of 2-way link with MPID0 | … | Type of 2-way link with MPID31 | Node type |

Figure s6 – DBA frame 3 announcement

##### 5.3.1.3.1 DBA frame 3 link type fields

These are 2-bit fields that represent the type of bidirectional links the transmitting MP has with other MPs. The encoding for link type fields is as follows: 0 = no link, 1 = link and 2 = backbone link. The transmitting MP places a “0” in its own link type field.

##### 5.3.1.3.2 DBA frame 3 node type field

The DBA frame 3 node type field indicates the type of node that is reporting, i.e., the current role that the node has in the network restructuring. Valid node types are the following:  1 = non-backbone node, 2 = clusterhead, and 3 = gateway. All nodes having node types set to clusterhead or gateway are considered to be backbone nodes.

#### 5.3.1.4 DBA frame 4 announcement

Figure s7 shows the format of all DBA frame 4 announcements.

| Bits: 2 | … | 2 | Octets: 8 | 1 | 2 |  | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of 2-way link with MPID0 | … | Type of 2-way link with MPID31 | Node type | P, reserved, n B0, B1 - B4, B5 B6 B7 | Link 1 ID | … | Link n ID |

Figure s7 – DBA frame 4 announcement

##### 5.3.1.4.1 DBA frame 4 link type fields

These are 2-bit fields that represent the type of bidirectional links the transmitting MP has with other MPs. The encoding for link type fields is as follows: 0 = no link, 1 = link and 2 = backbone link. This encoding is extended from frame 3 with the additional link type 3 = backbone connection link. The transmitting MP places a “0” in its own link type field.

##### 5.3.1.4.2 DBA frame 4 node type field

The DBA frame 4 node type field indicates the type of node that is reporting, i.e., the final role that the node has in the network restructuring. Valid node types are the following:  1 = non-backbone node, 2 = clusterhead, and 3 = gateway. All nodes having node types set to clusterhead or gateway are considered to be backbone nodes.

##### 5.3.1.4.3 DBA frame 4 P field

Bit 0 of the first octect after the node type field of DBA frame 4 is a flag indicating whether the MP is leaving the backbone (P = 1) or not leaving (P = 0).

##### 5.3.1.4.4 DBA frame 4 reserved field

Bits B1 through B4 of the first octect after the node type field of DBA frame 4 are reserved bits and are set to zeroes (binary).

##### 5.3.1.4.5 DBA frame 4 n field

Each MP that is leaving the backbone announces which of its backbone neighbors have to build links to each other in order to keep the backbone connected. Field n (bits 5, 6, and 7 of the first octect after the node type field of DBA frame 4) holds the number of such links that are being reported. Some of these backbone links may already exist.

##### 5.3.1.4.6 DBA frame 4 link id fields

Each of the DBA frame 4 link id fields is defined by giving the MPID’s of the nodes at the ends of the link, as shown in Figure s8.

| Octets: 1 | 1 |
| --- | --- |
| MPIDxj | MPIDyj |

Figure s8 – DS Link j is identified by the MPIDs x and y at the link endpoints

#### 5.3.1.5 Asynchronous protocol message

This message is used to send mesh management elements asynchronously.

### 5.3.2 Mesh management elements

Mesh management elements are defined to have a common general format consisting of a 1-octet mesh element ID (MEID) field, a 1-octet length field, and a fixed or variable-length element-specific information field. Each mesh management element is assigned a unique MEID as defined herein. The Length field specifies the number of octets in the Information field. See Figure s9.

| Octets: 1 | 1 | length |
| --- | --- | --- |
| MEID | Length | Information |

Figure s9 – Mesh element format

The present list of mesh element IDs is shown in Table s3.

Table s3 – Mesh element IDs

| Information Element | Element ID |
| --- | --- |
| Mesh ARP query | 0 |
| Mesh ARP reply | 1 |
| Link state advertisement | 2 |

#### 5.3.2.1 Mesh ARP query

Figure s10 shows the format of a mesh ARP query element. This element is used to find the target mesh point identifier corresponding to the given target MAC address. It is generally sent as a mesh broadcast within an asynchronous protocol message.

| Octets: 1 | 1 | 1 | 6 | 6 |
| --- | --- | --- | --- | --- |
| MEID = 0 | Length = 13 | Source MPID | Source MAC Address | Target MAC Address |

Figure s10 – Mesh ARP query element

##### 5.3.2.1.1 Mesh ARP query source MPID field

This field contains the mesh point identifier of the mesh point that originated the query.

##### 5.3.2.1.2 Mesh ARP query source MAC address

This field contains the MAC address of the mesh point that originated the query.

##### 5.3.2.1.3 Mesh ARP query target MAC address field

This field contains the MAC address of the node whose mesh point identifier is being sought. For example, in the case where the target MAC address belongs to a STA that is associated with one of the mesh’s APs, this query is to find the MPID of the AP to which the STA is presently associated. As another example, in the case where the target is reachable via a portal, the query is to find the MPID of the AP where the exit portal is located.

#### 5.3.2.2 Mesh ARP reply

Figure s11 shows the format of a mesh ARP reply element.

| Octets: 1 | 1 | 1 | 6 | 6 | 1 |
| --- | --- | --- | --- | --- | --- |
| MEID = 1 | Length = 14 | Source MPID | Source MAC Address | Target MAC Address | Target MPID |

Figure s11 – Mesh ARP reply element

##### 5.3.2.2.1 Mesh ARP query source MPID field

This field contains the mesh point identifier of the mesh point that originated the query.

##### 5.3.2.2.2 Mesh ARP query source MAC address

This field contains the MAC address of the mesh point that originated the query.

##### 5.3.2.2.3 Mesh ARP query target MAC address field

This field contains the MAC address of the node whose mesh point identifier is being sought.

##### 5.3.2.2.4 Mesh ARP reply target MPID field

This field contains the MPID of the mesh point via which the target MAC address can be reached.

#### 5.3.2.3 Link state advertisement

Figure s12 shows the format of a link state advertisement (LSA) element. This element typically combines the link state reports (LSRs) of several MPs. The default is to send this within a synchronous DBA announcement message. Optionally, it could be sent asynchronously within an asynchronous protocol message with DMPID set to local DS announcement.

| Octets: 1 | 1 | 1 | 1 | 1 | 2 | Depends on LSA format used |  | 1 | 2 | varies |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MEID = 2 | Length | number of reports n | RTAID | OMPID1 | LSEQ1 | LQI1 | … | (OMPID)n | LSEQn | LQIn |

Figure s12 – Link state advertisement element

##### 5.3.2.3.1 LSA number of reports field

This field indicates how many reports follow the RTAID field in this element. Each link state report (LSR) consists of three fields: the originating MPID (OMPID), the link report sequence number (LSEQ), and the link quality indicator (LQI).

##### 5.3.2.3.2 LSA routing algorithm identifier (RTAID) field

This field identifies the unicast routing algorithm being used within the mesh. Associated with each unicast routing algorithm is a format description of the corresponding LQI field that it uses.

##### 5.3.2.3.3 LSR OMPID field

The originating MPID is the mesh point to which a report applies. The report gives the status of all incoming links at OMPID.

##### 5.3.2.3.4 LSR sequence (LSEQ) field

The LSEQ field is used to determine the relative age of two link state reports from the same originating MP. Everytime the status of one or more incoming links changes at OMPID, the value of LSEQ is incremented.

##### 5.3.2.3.5 LSR link quality indicator (LQI) field

The format and values for the LQI fields depends upon the link metrics being used, e.g., hop count, link data rate, etc. The format for the LQI field is indicated by the value in the routing algorithm identifier field.

### 5.3.3 Mesh data frames

#### 5.3.3.1 Mesh data messages of subtype data

The frame format for a mesh data message of subtype data is identical to that shown in Figure s2. The mesh body holds the “user data” (beginning with the standard SNAP header) to be transmitted.

# 6 Mesh algorithms and protocols

## 6.1 Mesh synchronization

All mesh points within a single mesh shall be synchronized to a common “mesh” clock using the mesh timing synchronization function (MTSF) mechanism defined herein.

### 6.1.1 Mesh-wide DBA timing and synchronous counters

The Dynamic Backbone Algorithm (DBA) relies on the mesh clock to properly order DBA transmissions and to drive the synchronous counters shown in Figure s13.

Figure s13 – Dynamic Backbone Algorithm (DBA) timing and synchronous counters

### 6.1.2 Mesh clock synchronization

A Mesh Timing Synchronization Function (MTSF) keeps the mesh timers for all mesh points within the same mesh synchronized. The MTSF is implemented via a distributed algorithm that is performed by all of the mesh points within the mesh. Conceptually, the MTSF is a modulus 264 counter whose value at any instant can be represented as the local TSF value plus a positive, mesh_clock_offset (also represented by a modulus 264 counter).

Initially, each mesh point sets mesh_clock_offset = 0. Thus, prior to receiving any mesh traffic, MTSF = TSF at the local mesh point. Periodically, with period TE (see Figure s12), each mesh point announces the current value of its MTSF in its DBA frame 1 message. Upon receiving a DBA frame 1 message, the receiving mesh point compares its current value of MTSF with the value in the DBA frame 1 message. If the value of MTSF in the DBA frame 1 message is greater than the local MTSF, the receiving mesh point must increment its mesh_clock_offset so that the local MTSF matches the received MTSF.

## 6.2 Backbone maintenance (Dynamic Backbone Algorithm, DBA)

### 6.2.1 Overview of DBA

Periodically, the WLAN mesh is presented with a new backbone structure, an example of which is shown in Figure s1. A synchronous, distributed algorithm, the Dynamic Backbone Algorithm (DBA), is responsible for maintaining this backbone. The DBA takes as input i (the MPID of the host MP) and n (the maximum number of access points allowed in the WLAN mesh). DBA is driven by synchronous events, indicating time to transmit a DBA message and indicating when a new DBA frame begins, and by asynchronous events, indicating reception of DBA messages from other mesh points.

The DBA requires four frames of communications to determine a new backbone; these communications take place during the first four frames of each epoch. Each access point uses its MPID to determine in which slot to transmit its DBA control frames, that is, MPID = 0 transmits in slot 0, MPID = 1 transmits in slot 1, etc. These transmissions are triggered by FRAME_<frame number>_TRANSMIT events. The transmissions should complete with sufficient time to allow the next access point to process this frame before sending its own DBA control frame.

The algorithm proceeds in four phases, which approximately coincide with the four frames required for backbone formation. These phases are the following: 1) neighbor discovery, 2) cluster forming, 3) cluster linking, 4) backbone pruning and connecting to the backbone.

Neighbor discovery at an MP is accomplished before that MP transmits in frame 2. This is accomplished by explicitly acknowledging previously heard frame 1 transmissions when an MP sends its own frame 1 transmission. By examining acknowledgements received in frame 1, each MP can determine which higher-numbered MPs it is currently bidirectionally connected to. Bidirectional connectivity to lower-numbered MPs is determined when these MPs report their bidirectional neighbors as part of their frame 2 transmissions.

All DBA messages are sent as mesh frames and have the 9-byte mesh header prepended to them. The subfields of the mesh frame control field are set as follows (binary): version = 00, type = 00 (management), source is MP =1, destination is MP = 1, precedence = 111 (highest), and reserved = 000. The subtype subfield varies depending on the DBA frame being transmitted and has the values shown in Table s1. Mesh point identifier fields in the mesh header are set as follows: RMPID = Local DS announcement, TMPID = host’s MPID, DMPID = Local DS announcement, and SMPID = host’s MPID.

The MSEQ field is set to all 0’s and is ignored on reception for Local DS announcement messages.

Just prior to its frame 2 transmission, each MP makes a determination whether it should be a “clusterhead”. Clusterhead status has significance only within the execution of the DBA; it is a temporary “bookkeeping” state that is used in the process of determining whether an MP should become part of the dynamic backbone. The rule is that an MP becomes a clusterhead unless it is already bidirectionally linked to one. Each MP selects as its “own clusterhead” the lowest numbered clusterhead to which it is bidirectionally connected. If an MP is a clusterhead, then it is also its own clusterhead. An MP includes the MPID of its own clusterhead in its frame 2 transmission.

In the context of the DBA, a cluster is defined as a set of nodes consisting of a clusterhead and all its bidirectional neighbors. Note that this implies that, depending on the topology, a non-clusterhead node may be a member of several clusters simultaneously. Therefore, a non-clusterhead node is not restricted to being a member only of the cluster of its own clusterhead. Just as the designation of a node as a clusterhead is strictly a DBA bookkeeping technique, the DBA concept of  “cluster membership” is also a temporary designation, which has no significance outside the operation of the DBA.

At the start of frame 3, the WLAN mesh has been organized into clusters. Now begins the process of actually forming the backbone. This process is accomplished by linking the clusters together. Each MP that is not a clusterhead determines at this time whether it should become a “gateway” to link “adjacent” clusters. Here also, gateway status is only a temporary, bookkeeping state; at the end of frame 4 all clusterheads and gateway nodes can just be considered as backbone nodes. Adjacent clusters are of two types: overlapping or non-overlapping. Adjacent, overlapping clusters have one or more MPs that are bidirectionally connected to each clusterhead. Adjacent, non-overlapping clusters have no nodes that are bidirectionally linked to both clusterheads. However, in the latter case, there exists at least one pair of MPs that are bidirectionally linked to each other and which in turn are bidirectionally linked to different clusterheads.

The following section provides pseudocode for the DBA. This code defines two procedures linkup1 and linkup2 that determine whether an MP should become a gateway to link adjacent, overlapping or non-overlapping clusters, respectively. Briefly, these two procedures operate as follows.

In linkup1, every MP that is not a clusterhead is a candidate gateway. The candidate examines the DBA database for all combinations of pairs of clusterheads to which it is directly connected via bidirectional links. The corresponding clusters are overlapping. The lowest numbered MP in the intersection of the two clusters makes the determination to become a gateway for that pair. All mesh points in the intersection are “aware of each other” since they can be at most two hops away from each other, and every MP possesses the connectivity information for every one of its neighbor mesh points. Links between the new gateway and the clusterheads that it is linking are marked as backbone links. It should be noted that in both linkup1 and linkup2, a single MP can act as gateway for linking more than one pair of clusters.

Procedure linkup2 handles the case of adjacent, non-overlapping clusters. As in linkup1, only non-clusterhead nodes are candidates to form this linkage. The candidate examines the DBA database for all known pairs of clusterheads consisting of its own clusterhead and clusterhead k that is 2-hops away. To avoid redundant linkage, the node attempts to ascertain the need for the creation of a linkage by checking, for each such pair of clusterheads, whether a linkage may already have been created by linkup1. If such a prior linkage has not occurred, it then ranks the potential gateway-to-gateway links that join a node from cluster k to a node belonging to its own clusterheads cluster. The ranking is done by adding the MPIDs of the candidate gateway pair and awarding the highest ranking to the link with the lowest sum. In case of ties, the link with the lowest numbered node is ranked the highest. If the host node is an endpoint on the highest ranking link, then it becomes a gateway for linking clusterheads h1 and h2 and both the link between the new gateway and its own clusterhead and the highest ranked gateway-to-gateway link are marked as backbone links. In some cases only one of the two potential gateway nodes in a pair may decide to become a gateway while the other may find that it is not needed if another link of higher rank is available and known to it. Such asymmetric situations are detected and corrected during frame 3.

Just prior to the beginning of frame 3 transmissions, gateway nodes have determined which of their links are to be promoted to backbone type. In frame 3 all nodes learn of the backbone links within two hops of itself. The frame 3 transmission also has the purpose of resolving any inconsistencies when non-clusterhead nodes assign a different link type to the link between them as a result of linkup2. Here the rule is that the resultant link type is determined by the lowest numbered endpoint node. Thus, when such a link-type inconsistency is detected as a result of receiving a frame 3 transmission, the receiving node will adjust its own database to be consistent with the information received. This may involve changing the types of one (or possibly two) links as well as possibly changing whether the host node is a backbone node or not.

The purpose of frame 4 is to allow pruning of the backbone and to allow non-backbone nodes to select a Backbone Connection Node (BCN). A non-backbone node selects as its BCN the highest numbered clusterhead that it has heard in frame 4 and to which it is bidirectionally linked. If there is no such node, it will select as BCN the highest numbered backbone node to which it is bidirectionally connected. The link between a non-backbone node and its BCN is designated to be of type backbone-connection-link.

A backbone node may revert to non-backbone status if it determines that all of the following conditions are met: 1) the node is currently a backbone node, 2) no non-backbone node has chosen in as its Backbone Connection Node (BCN), 3) it has already heard the frame 4 transmissions from all other neighboring backbone nodes that have already transmitted in frame 4, 4) it has at least one backbone neighbor, 5) every higher-numbered, non-backbone neighbor of the node is linked to at least one other backbone node, and 6) becoming a non-backbone node will not cause bifurcation of the backbone. As a byproduct of checking for possible bifurcation of the backbone, the host node develops a tree for linking its backbone neighbors. If the node determines that it can leave the backbone, the backbone links that make up this tree are announced as part of the node’s frame 4 transmission along with the indication that the node is leaving the backbone. If the node does leave the backbone, it also selects its BCN at this time. A backbone node is considered its own BCN.

Just before the end of frame 4, the new backbone structure has been completely determined. At this time the “new” structure is “installed”, and from that time until the next backbone is installed, this structure is used to route broadcast and multicast traffic.

### 6.2.2 Detailed operation of DBA

DBA(i,n) // i is local MPID and n is max MPs allowed

{ 	Reset();

switch(event) {

case BEGIN_FRAME_1:

Reset();

case FRAME_1_TRANSMIT:

pkt.Probe_ack ⇐ for (∀k ∈ N) nodesHeard[k];

Transmit pkt;

case FRAME_1_RECEIVE:

// rcv pkt from Node with mpid = j (ji)

nodesHeard[j] = TRUE;

if (i<j) { // Process j’s response to i’s probe

if (pkt.Probe_ack[i] 1) {

linkType[i,j] = linkType[j,i] = LINK;

bidirectionalNeighbors[j] = TRUE;

}

}

case FRAME_2_TRANSMIT:

if (ownHeadNO_MPID) {

nodeType[i] = CLUSTERHEAD;

isBackboneNode[i] = TRUE; ownHead = i;

}

else { // ownHead has already been set

nodeType[i] = NON_BACKBONE_NODE;

isBackboneNode[i] = FALSE;

}

pkt.own_clusterhead ⇐ ownHead;

pkt.Bidirectional_links ⇐ for (∀k∈N)

bidirectionalNeighbors[k];

Transmit pkt;

case FRAME_2_RECEIVE pkt from Node j (ji):

if (j<i) {

if (pkt.Bidirectional_links[i] 1) {

bidirectionalNeighbors[j] = TRUE;

}

else break;	// exit case

}

elseif (bidirectionalNeighbors[j]TRUE)

break;

for (∀k ∈ N) {

if (pkt.Bidirectional_links [k] 1)

linkType[k,j] = linkType[j,k] = LINK;

else

linkType[k,j] = linkType[j,k] = NO_LINK;

}

if (pkt.own_clusterhead  j) { // sender is a CH

if (linkType[i,j]LINK)

isBackboneNode[j] = TRUE;

if (ownHeadNO_MPID)

ownHead = j;

}

if (pkt.own_clusterhead i) {

if (linkType[i, pkt.own_clusterhead]LINK) {

headsOneHopAway[pkt.own_clusterhead] =

TRUE;

numHeadsOneHopAway++;

}

else {

headsTwoHopsAway[pkt.own_clusterhead] =

TRUE;

numHeadsTwoHopsAway++;

}

}

case BEGIN_FRAME_3:

Linkup1(); Linkup2();

case FRAME_3_TRANSMIT:

pkt.Node_type ⇐ nodeType[i];

pkt.Link_type ⇐ for (∀k ∈ N) linkType[i,k];

Transmit pkt;

case FRAME_3_RECEIVE pkt from Node j (ji):

if (bidirectionalNeighbors[j]  FALSE) break;

nodeType[j] ⇐ pkt.Node_type;

for (∀k ∈ N, kj) {

if (pkt.Link_type [k]BB_LINK) {

isBackboneNode[j] = isBackboneNode[k] =

TRUE;

if (ki and

nodeType[i]NON_BACKBONE_NODE)						nodeType[i] = BACKBONE_NODE;

linkType[k,j] = linkType[j,k] = BB_LINK;

}

if (pkt.Link_Type[k] BB_LINK and ki

and nodeType[j]CLUSTERHEAD

and linkType[i,j]BB_LINK) {

linkType[i,j] = linkType[j,i] = LINK;

needToBeBbn = FALSE;

for (∀m ∈ N, mi, m ownHead) {

if (linkType[i,m]  BB_LINK) {

needToBeBbn = TRUE;

break;

}

}

if (needToBeBbn  FLASE) {

nodeType[i] = NON_BACKBONE_NODE;

isBackboneNode[i] = FALSE;

linkType[i,ownHead] = LINK;

linkType[ownHead,i] = LINK;

}

}

}

case FRAME_4_TRANSMIT:

numLinks = 0;

isLeavingBackbone = CanLeaveBB();

pkt.P ⇐ isLeavingBackbone;

if (isLeavingBackbone) {

isBackboneNode[i] = FALSE;

nodeType[i] = NON_BACKBONE_NODE;

for (∀k ∈ N) {

if (linkType[i,k]LINK)

linkType[i,k] = linkType[k,i] = LINK;

}

// Build BB links connecting all BB neighbors

for (∀k ∈ N) {

if (cnxTree[k]>1) {

m = cnxTree[k] - 2;

linkType[m,k] = linkType[k,m] =

BB_LINK;

}

}

// Put new BB link IDs into packet

for (∀k ∈ N) {

m = cnxTree[k] – 2;

if (m0) {

nodePair[numLinks,0] = k;

nodePair[numLinks,1] = m;

pkt.MPIDxj ⇐ nodePair[numLinks,0];

pkt.MPIDyj⇐ nodePair[numLinks,1];

numLinks++;

}

}

}

pkt.n ⇐ numLinks;

ChooseBCN();

bestBCN = backboneConnectionNode[i];

linkType[bestBCN,i] = linkType[i,bestBCN] =

BCN_LINK;

pkt.Link_type ⇐ for (∀k ∈ N)  linkType[i,k];

pkt.Node_type ⇐ nodeType[i];

Transmit pkt;

case FRAME_4_RECEIVE pkt from Node j (ji):

nodesHeardFrame4[j] = TRUE;

if (bidirectionalNeighbors[j]  FALSE) break;

if (pkt.P) { // Is node j leaving backbone?

if (linkType [i,j] > LINK)

linkType[i,j] = linkType[j,i] = LINK;

isBackboneNode[j] = FALSE;

if (!isBackboneNode[i]) break;  // exit case

for (∀k ∈ L) { // L is set 1, ... pkt.n

linkType[pkt.MPIDxk,pkt.MPIDyk] =

BB_LINK;

linkType[pkt.MPIDyk,pkt.MPIDxk] =

BB_LINK;

}

}

for (∀k ∈ N, kj) {

switch (pkt.Link_type [k]) {

case BB_LINK:

linkType[j,k] = linkType[k,j] = BB_LINK;

isBackboneNode[j]=

isBackboneNode[k]=TRUE;

backboneConnectionNode[k]=k;

backboneConnectionNode[j]=j;

case BCN_LINK:

linkType[j,k] = linkType[k,j] = BCN_LINK;

if (pkt.Node_type

NON_BACKBONE_NODE) {

backboneConnectionNode[k]=k;

backboneConnectionNode[j]=k;

} else {

backboneConnectionNode[k]=j;

backboneConnectionNode[j]=j;

}

case LINK:

linkType[j,k] = linkType[k,j] = LINK;

case NO_LINK:

linkType[j,k] = linkType[k,j] = NO_LINK;

} // End of FRAME_5_RECEIVE switch

}

} // End of event switch

} // End of DBA

Reset()

{	ownHead = NO_MPID;

isLeavingBackbone = FALSE;

numLinks = 0;

numHeadsOneHopAway = 0;

numHeadsTwoHopsAway = 0;

for (∀j ∈ N) {

nodeType[j] = NON_BACKBONE_NODE;

// NON_BACKBONE_NODE=1,

// CLUSTERHEAD=2,

// GATEWAY=3 (BACKBONE_NODE  2)

bidirectionalNeighbors[j] = FALSE;

headsOneHopAway[j] = FALSE;

headsTwoHopsAway[j] = FALSE;

nodesHeard[j] = FALSE;

nodesHeardFrame4[j] = FALSE;

isBackboneNode[j] = FALSE;

backboneConnectionNode[j] = NO_MPID;

for (∀k ∈ N)  linkType[j,k] = NO_LINK;

// NO_LINK=0,LINK=1,BB_LINK=2,BCN_LINK=3

}

} // End of Reset

Linkup1()

{

if (nodeType[i]CLUSTERHEAD) return;

if (numHeadsOneHopAway >1) {

for (∀k ∈ H1) { //H1 is set of heads 1-hop away

for (∀m ∈ H1, mk) {

for (∀p ∈ N) {

if (linkType[p,k]LINK and

linkType[p,m]LINK)

if (p ≤ i) break;

}

if (p  i) {

isBackboneNode[i] = TRUE;

nodeType[i] = GATEWAY;

linkType[i,k] = linkType[k,i] = BB_LINK;

linkType[i,m] = linkType[m,i] = BB_LINK;

}

}

}

}

} // End of Linkup1

Linkup2()

{

if (nodeType[i]CLUSTERHEAD) return;

for (∀k ∈ H2) { // H2 is set of heads 2-hops away

for (∀m ∈ N) {

if (mownHead or mk) continue;

if (linkType[m,k]LINK) {

for (∀p ∈ H1) {

if (linkType[m,p]LINK) break;

}

if (p < numHeadsOneHopAway) break;

}

}

if (m < n) continue;

hig1 = hig2 = n;

for (∀g1 ∈ N) {

if (nodeType[g1]CLUSTERHEAD) continue;

for (∀g2 ∈ N) {

if (nodeType[g2]CLUSTERHEAD) continue;

if (g1g2) continue;

if (linkType[ownHead,g1]LINK and

linkType[g1,g2]LINK and

linkType[g2,k]LINK) {

if (g1+g2 < hig1+hig2 or

(g1+g2  hig1+hig2 and

min(g1,g2) < min(hig1,hig2))) {

hig1 = g1;

hig2 = g2;

}

}

}

}

if (i  hig1) {

nodeType[i] = GATEWAY;

isBackboneNode[i] = TRUE;

linkType[ownHead,i]  = linkType[i,ownHead]  =

BB_LINK;

linkType[i,hig2]  = linkType[hig2,i]  =

BB_LINK;

}

if (i  hig2) {

nodeType[i] = GATEWAY;

isBackboneNode[i] = TRUE;

linkType[hig1,i]  = linkType[i,hig1]  =

BB_LINK;

linkType[i,k]  = linkType[k,i]  = BB_LINK;

}

}

} // End of Linkup2

CanLeaveBB()

{

if (!isBackboneNode[i]) return (FALSE);

for (∀k ∈ N, k<i) { // don’t leave if already a BCN

if (backboneConnectionNode[k]  i)

return (FALSE);

}

for (∀k ∈ N, k<i) { // don’t leave if missed BB xmit

if (isBackboneNode[k] and

nodesHeardFrame4[k]FALSE)

return (FALSE);

}

// Check that it has a backbone neighbor

firstBbn = NO_MPID;

numBBnbrs = 0;

for (∀k ∈ N, ki) {

if (isBackboneNode[k] and linkType[i,k]LINK) {

if (firstBbnNO_MPID) {

firstBbn = k;	// lowest # backbone node

}

numBBnbrs++;

}

}

if (firstBbnNO_MPID) return (FALSE);

// Check to see if needed as BCN

for (∀k ∈ N, ki) {

if (!isBackboneNode[k] and linkType[i,k]LINK) {

loopCount = 0;

for (∀m ∈ N, mi, mk) {

if (isBackboneNode[m] and

linkType[m,k]LINK and

linkType[i,m]LINK) break;

loopCount++;

}

if (loopCountn) return (FALSE);

}

}

// Would leaving split the backbone?

for (∀k ∈ N) cnxTree[k] = 0;

cnxTree[firstBbn] = 1;

treeSize = 0;

BuildCnxTree(firstBbn);

If (treeSize < (numBBnbrs – 1)) return (FALSE);

return (TRUE);

} // End of CanLeaveBB

BuildCnxTree(m)

{

for (∀k ∈ N) {

if (linkType[i,k]LINK and linkType[m,k]LINK

and isBackboneNode[k] and cnxTree[k]0) {

cnxTree[k] = m+2;

treeSize++;

BuildCnxTree (k);

}

}

} // End of BuildCnxTree

ChooseBCN()

{

if (isBackboneNode[i]) {

backboneConnectionNode[i] = i;

return;

}

bestBCN = NO_MPID;

// BCN is highest num BB already heard in frame 4

for (∀k ∈ N, k<i) {

if (isBackboneNode[k] and

nodesHeardFrame4[k]TRUE and

linkType[i,k]LINK and

linkType[k,i]LINK) {

bestBCN = k;

}

}

// BCN is highest num BB neighbor

if (bestBCN  NO_MPID) {

for (∀k ∈ N, ki) {

if (isBackboneNode[k] and

linkType[i,k]LINK and

linkType[k,i]LINK) {

bestBCN = k;

}

}

}

if (bestBCN  NO_MPID) return;

backboneConnectionNode[i] = bestBCN;

} // End of ChooseBCN

## 6.3 Unicast traffic routing

### 6.3.1 Link state dissemination protocol (LSDP)

A suite of unicast routing algorithms is supported. The different routing algorithms may use different link metrics and different techniques for determining paths. However, they share the same default link state dissemination protocol (LSDP) to send this information to all mesh points.

The LSDP is responsible for ensuring that each mesh point has a complete database of link state reports (LSRs) for every MP. In addition to the OMPID, LSEQ, and LQI fields of the LSR, each LSR is also associated with an expiration time (LSRtexp) and a state. The LSR state is one of the values {UNINITIALIZED, SAVE, SEND, SENT, STALE}. If no LSR information has been received for a given OMPID, state is UNINITIALIZED. In addition, each MP maintains a refresh timer (LSRtref) to control the retransmission of its own LSR when that information is unchanged for long periods of time.

The LSDP is driven by the origination of new LSRs. The LSDP uses a synchronous technique for the generation of new LSRs. At the start of its DBA frame 1 transmission slot, a MP updates its own LSR based on recent link measurements. The new LQI is compared with that in the old LSR. If they are different, LSEQ is incremented by one; the state is set to SEND; (LSRtexp), and (Ltref) are computed based on current time plus time-to-live (LSRTTL  = 50 TE) and time-to-refresh (LSRTTR = 10TE), respectively.

If the new LQI is unchanged from the old one, LSRtexp is still updated as above. Nothing further needs to be done except for the case where Ltref has been reached or exceeded. In that case, Ltref is recomputed as above, LSEQ is incremented by one, and state is set to SEND.

Relaying of LSRs is handled as follows. When an LSR is received its LSEQ field is examined to see if this is a more recent LSR than is currently in the database. If the database holds more up-to-date information, the received LSR can be ignored. If the LSR in the database has the same LSEQ as that in the received LSR, only LSRtexp needs to be updated as above. If LSEQ indicates that the received LSR is more recent, then the new LSR replaces the old one and LSRtexp is updated as above. If the receiving MP is a DBA backbone node, the state is set to SEND, otherwise it is set to SAVE.

At the start of its own DBA frame n transmission (n = 1, 2, 3, 4), an MP examines it LSR database to see if any reports are marked SEND. If there are reports to send, these reports are assembled into a LSA element and added to the DBA frame n message. The state of those LSRs that were transmitted is marked SENT. If any LSRs have exceeded there expiration time, their state is changed to STALE and they should not be used in any route computations.

### 6.3.2 Algorithm 0: min-hop routing

#### 6.3.2.1 Link state measurement

At the start of its own DBA frame 1 transmission slot MP i examines database of the number of DBA messages received from MP j over the previous DBA epoch duration (TE). If that number exceeds 0, LQIij = 1, else LQIij = 0.

#### 6.3.2.2 Link state advertisement format

Figure s14 shows the link state report format for a simple min-hop routing scheme. Here, the link state report uses only a single bit to indicate whether a unidirectional link is “up” or “down”.  A “1” in Bit: n indicates that a link from MPID=n to OMPID exists, whereas a “0” indicates the lack of a link.  The originating MP places a “0” in its own bit position.

| Bit: 0 | … | Bit: 31 |
| --- | --- | --- |
| LQI (MPID0 to OMPID) |  | LQI (MPID31 to OMPID) |

Figure s14 – Link state report format for min-hop routing algorithm

#### 6.3.2.3 Route computation

At the end of DBA frame 4, a matrix of undirected links is derived from the LSR database. Dijkstra’s Shortest Path Algorithm is applied to this matrix to generate next-hop routing information for all destinations based on min-hops.

#### 6.3.3 Additional unicast routing algorithms

To be supplied in a future draft.

### 6.4 Broadcast/multicast traffic routing

### 6.4.1 All 1’s (ff:ff:ff:ff:ff:ff) broadcast traffic routing

When an “all 1’s” broadcast message is received (address ff:ff:ff:ff:ff:ff), either from the upper protocol layer or from the physical layer, it is relayed, if required, according to the following rules. Every MAP forwards the broadcast to its BSS members, portals, and LLC. In addition, if the message is entering the mesh, it is marked for transmission by the MP. If the message was received as a mesh data frame at a DBA backbone MP, and if was not previously transmitted by this node, it is marked for transmission.

### 6.4.2 IP multicast traffic routing

IP multicast is handled within the mesh just like an all 1’s broadcast. However, unlike the all 1’s broadcast, multicast filtering (if multicast group membership information is available) can be applied when the message is sent to STAs, portals, and LLC.

### 6.4.3 Mesh multicast group routing

Table s2 identifies several mesh multicast groups that are defined as follows.

#### 6.4.3.1 local DS announcement

This multicast MPID is used to indicate that the 802.11 frame is to be transmitted without regard to any specific destination node. This frame is never relayed.

#### 6.4.3.2 mesh broadcast

This multicast MPID is used to indicate that the frame is to be delivered to all MPs in the WLAN mesh.

## 6.5 Message handling

When a mesh message either originates at or is received by a mesh point and marked for retransmission, it is saved along with its arrival time tmarr. These messages have an associated message time-to-live value (Tmttl) (TBD). Messages that have exceeded their time-to-live must be deleted.

Each node maintains a small database that is used to determine if a received mesh message is a duplicate of one already received. This database contains tuples (SMPID, MSEQ, tmarr) values for recently received messages from each MP. A newly received mesh message from source SMPID and with sequence number MSEQ is compared with those found in this database. If a match is found, the message is a duplicate.

For each new entry in the received sequence number database the arrival time (tmarr) is noted. Entries into this database have a specified time-to-live (Tsttl)(TBD). Entries in the database whose time-to-live has been exceeded must be removed.

The mesh header field MSEQ is not to be used to detect duplicates for mesh messages of type Local DS Announcement nor should they generate an entry into the received sequence number database. These messages are never relayed and therefore the sequence number found in the MAC header is sufficient for determining duplicates.

It may be necessary to add jitter to relayed broadcast messages to avoid channel interference. The mechanism for providing this is TBD.

When source traffic from STAs or portals is inserted into mesh messages, the MAC source address (SA) is set to the original source not to the mesh point MAC address. Likewise, the destination MAC address (DA) in the MAC header is set to the original destination, which may not be the same as the MAC address of the destination MP.

## 6.6 Mesh address management

This proposal introduces the concept of a single-octet, mesh point identifier (MPID). This section describes procedures that deal with MPIDs.

### 6.6.1 Assignment of mesh point identifiers

MPIDs can be assigned manually or dynamically. It is also permissible in a mesh to assign some MPIDs manually and some dynamically.

#### 6.6.1.1 Manual assignment of mesh point identifier

In this case, the user sets the MPID as a configuration parameter.

#### 6.6.1.2 Dynamic assignment of mesh point identifier

To be supplied in future draft.

### 6.6.2 Learning correspondences between MAC addresses and mesh point identifiers

There are presently four ways of learning the correspondences between MAC addresses and MPIDs: 1) at time of own MPID assignment, 2) when a STA associates with an MP, 3) by inspection of the headers of mesh messages, 4) by explicitly querying for this correspondence by sending mesh arp queries.

#### 6.6.2.1 Address learning at time of assignment of MPID

A mesh point learns its own MPID as soon as it is manually assigned (at configuration) or dynamically learned.

#### 6.6.2.2 Address learning at time of STA association

For purposes of routing data frames to STAs, the MAC address of a STA is associated with the MPID of the MAP with which it is currently associated.

#### 6.6.2.3 Address learning by inspection of headers

Any mesh point that receives a mesh frame can learn some of the MAC address / MPID correspondences from that frame since they are explicitly given.

#### 6.6.2.4 Address learning by direct query

A “reactive” mesh ARP query management element is defined for directly obtaining a target MPID for a given target MAC address.

## 6.7 Sequence Numbers

Both mesh headers and link state reports contain sequence number fields both for determining duplicates or determing relative age. We use the assumption that at any instant of time less than half the available sequence numbers will be in use. With this assumption, it follows that sequence number SEQ2 is more recent than SEQ1 provided

SEQ2 > SEQ1 and SEQ2 – SEQ1 <= MAX_SEQ/2 or

SEQ1 > SEQ2 and SEQ1 – SEQ2 > MAX_SEQ/2

# References:

- IEEE Standard 802.11-1999