IEEE P802.11
Wireless LANs

| 802.11 TGs Simple Efficient Extensible Mesh (SEE-Mesh) Proposal Checklists |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-06-15 |  |  |  |  |
| Author(s) (alphabetical order): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hidenori Aoki | NTT DoCoMo, Inc. | DoCoMo R&D Center, 3-5 Hikarino-oka, Yokosuka-shi, Kanagawa, 239-8536 Japan | +81-46-840-6526 | aokihid@nttdocomo.co.jp |
| Narasimha Chari | Tropos Networks | 555 Del Rey Ave, Sunnyvale, CA 94085 | +1-408-331-6814 | chari@tropos.com |
| Liwen Chu | STMicroelectronics Inc. | 1060 East Brokaw Road, Mail Station 212, San Jose, CA 95131 | +1-408-467-8436 | liwen.chu@st.com |
| W. Steven Conner | Intel Corp. | 2111 NE 25th Ave, M/S JF3-206, Hillsboro, OR 97124 | +1-503-264-8036 | w.steven.conner@intel.com |
| Stefano M. Faccin | Nokia | 3421 Dartmoor Dr, Dallas TX 75229 | +1-972-894-4994 | stefano.faccin@nokia.com |
| David Gurevich | Packethop | 1301 Shoreway Road, Belmont, California, 94002 | +1-650-292-5007 | dgurevich@packethop.com |
| Vann Hasty | Motorola Inc. | 485 N. Keller Rd., Suite 250, Maitland, FL 32751 | +1-407-659-5371 | Vann.Hasty@motorola.com |
| Jorjeta Jetcheva | Firetide, Inc. | 16795 Lark Ave., Los Gatos, CA 95032 | +1-408-355-7215 | jjetcheva@firetide.com |
| Shantanu Kangude | Texas Instruments | 12500 TI Blvd., Dallas, TX 75243 | +1-214-480-1810 | skangude@ti.com |
| Shin Saito | Sony | 6-7-35 Kitashinawaga Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-3175 | shin_saito@sm.sony.co.jp |
| Rakesh Taori | Samsung | SAIT, Mt. 14-1, Nongseo-Ri Kiheung-Eup, Youngin, Korea, 449-712 | +82 31 280 9635 | rakesh.taori@samsung.com |

Abstract

This document provides checklists and references for supporting materials to supplement the 802.11 TGs Simple Efficient Extensible Mesh (SEE-Mesh) Proposal (document 11-05/0562).  The tables and checklists in this document were copied from the TGs Comparison Categories and Informative Checklists template document 11-04/1175r6, which is referenced in the Call for Proposals.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Document Version History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | - Checklists for proposal submission on 6/15/2005. | 15 June 2005 |

Table of Contents

1	Introduction	2

2	Additional Supporting Material	2

3	Coverage of Minimum Functional Requirements	3

4	Coverage of In-Scope Functions (Informative)	5

7	References	12

# Introduction

This document provides checklists and references for supporting materials to supplement the 802.11 TGs Simple Efficient Extensible Mesh (SEE-Mesh) Proposal (document 11-05/0562).  The tables and checklists in this document were copied from the TGs Comparison Categories and Informative Checklists template document 11-04/1175r6, which is referenced in the Call for Proposals.

# Additional Supporting Material

This section contains requirements for additional documentation that must be submitted with a proposal.  This is a template that must be filled in and included with a proposal submission.

| Number | Name | Definition | Coverage (Yes/No) |  | Notes | References |
| --- | --- | --- | --- | --- | --- | --- |
| AD1 | Reference submissions | A list of IEEE 802 submissions related to the proposal, both documents and presentations. |  | Yes |  | --IEEE 802 11-05/562r0, 802.11 TGs Simple Efficient Extensible Mesh (SEE-Mesh) Proposal --IEEE 802.11-05/563r0, 802.11 TGs Simple Efficient Extensible Mesh (SEE-Mesh) Proposal Checklists -- IEEE 802.11-05/567, 802.11 TGs Simple Efficient Extensible Mesh (SEE-Mesh) Proposal Overview Presentation -- IEEE 802.11-05/568r0, Simulation Results for SEEMesh Congestion Control Protocol |
| AD2 | Simulation and/or experimental methodology | Any proposal submission that includes simulation results must include a description of the simulation methodology used for mesh simulations. The simulation methodology documentation should provide enough information to, in principle, reproduce the simulation (e.g., including node positions, traffic and propagation model (including PHY assumptions), packet sizes, etc.). |  | Yes |  | IEEE 802.11-05/568r0, Simulation Results for SEEMesh Congestion Control Protocol |

# Coverage of Minimum Functional Requirements

This section contains a template for disclosure of coverage of minimum functional requirements with a proposal.  See [6] for detailed definitions of functional requirements.  This template must be filled in and included with a proposal submission.

| Number | Category | Name | Coverage (Complete /Partial/ None) | Notes | References |
| --- | --- | --- | --- | --- | --- |
| FR1 | TOPO_RT_FWD | Mesh Topology Discovery | Complete | The proposal specifies automatic topology learning, including the status and quality of mesh links between Mesh Points. | [1] Section 6.3 |
| FR2 | TOPO_RT_FWD | Mesh Routing Protocol | Complete | The proposal specifies two MAC address-based protocols and algorithms (one default and one optional) for dynamic auto-configuration of MAC-layer data delivery paths between Mesh APs and Mesh Points over 802.11 wireless links (including across multiple wireless hops). | [1] Section 6.4.3 |
| FR3 | TOPO_RT_FWD | Extensible Mesh Routing Architecture | Complete | The proposal specifies a protocol architecture to allow for alternative path selection metrics and/or routing protocols based on application requirements (two protocols are specified as examples to demonstrate this extensibility) | [1] Sections 6.3.2, 6.4 |
| FR4 | TOPO_RT_FWD | Mesh Broadcast Data Delivery | Complete | The proposal specifies MAC-layer broadcast/multicast data delivery across the WLAN Mesh. | [1] Sections 6.4.4.4, 6.4.4.5 |
| FR5 | TOPO_RT_FWD | Mesh Unicast Data Delivery | Complete | The proposal specifies MAC-layer unicast data delivery across the WLAN Mesh. | [1] Sections 6.4.4.2, 6.4.4.3 |
| FR6 | TOPO_RT_FWD | Support for Single and Multiple Radios | Complete | The proposal supports the use of one or more IEEE 802.11 physical-layer transcievers on Mesh Points. | [1] Sections 4.2.3, 6.2 |
| FR7 | TOPO_RT_FWD | Mesh Network Size | Complete | The protocols specified in this proposal support 32 or more Mesh Points in the WLAN Mesh. | [1] Section 6.4.3 |
| FR8 | SECURITY | Mesh Security | Complete | The proposal utilizes IEEE 802.11i security mechanisms for the purpose of securing a WLAN Mesh in which all of the Mesh Points and Mesh Portals are controlled by a single logical administrative entity for security. | [1] Section 6.5 |
| FR9 | MEAS | Radio-Aware Routing Metrics | Complete | The proposal specifies a default radio-aware routing metric for use by the mesh routing protocol(s) and includes a framework to allow other routing metrics to be implemented in the standard framework. | [1] Section 6.4.2 |
| FR10 | SERV_CMP | Backwards compatibility with legacy BSS and STA | Complete | The proposal does not require modification to the STA’s relationship with the BSS and ESS. It specifies mechanisms to enable seamless integration of legacy STAs into a WLAN Mesh deployment. | [1] Sections 4.2.2, 6.4.4.3 |
| FR11 | SERV_CMP | Use of WDS 4-Addr Frame or Extension | Complete | The proposal support data delivery in a WLAN Mesh network using an extension of the four-address frame format . | [1] Section 5.1 |
| FR12 | DISC_ASSOC | Discovery and Association with a WLAN Mesh | Complete | The proposal supports the discovery of and association with a WLAN Mesh network by Mesh Points. | [1] Section 6.3. |
| FR13 | MMAC | Amendment to MAC with no PHY changes required | Complete | The proposal specifies amendments to the 802.11 MAC for mesh networking, but it does not require modification to the 802.11 physical layer. | [1] Sections 4, 5, 6 |
| FR14 | INTRWRK | Compatibility with higher-layer protocols | Complete | The proposal specifies mechanisms to enable an IEEE 802.11 WLAN Mesh Network to appear to higher layers as a current style IEEE 802 LAN segment. | [1] Sections 4.2.4, 6.8, 6.11 |

# Coverage of In-Scope Functions (Informative)

This section contains a template for informative disclosure of coverage of in-scope functions with a proposal.  See [6] for detailed description of in-scope functions considered by TGs.  This template may be filled in and included with a proposal submission.

This purpose of this template is primarily to aid the reader in identifying which in-scope features are covered by a proposal, and which sections of the proposal are relevant to each feature.   The in-scope functions listed below are NOT mandatory for inclusion in a proposal and are NOT functional requirements.  Proposers are welcome to provide additional functions that are in-scope.

| Number | Name | Coverage Yes / No / N/A | Notes | References |  |
| --- | --- | --- | --- | --- | --- |
| Mesh Topology Learning, Routing, and Forwarding (TOPO_RT_FWD) |  |  |  |  |  |
| TOPO_RT_FWD_SCP1 | - Mesh topology discovery, including Mesh Point neighbor discovery within a WLAN Mesh | Yes | The proposal specifies Mesh Point neighbour discovery mechanism and automatic topology learning, including the status and quality of mesh links between Mesh Points. | [1] Sections 6.3, 6.4.3.2.12, and 6.4.3.1 |  |
| TOPO_RT_FWD_SCP2 | - MAC address-based mesh routing protocols and algorithms | Yes | The proposal specifies MAC address-based mesh routing protocols, including both reactive and proactive algorithms. | [1] Section 6.4 |  |
| TOPO_RT_FWD_SCP3 | - MAC-layer mesh broadcast/multicast and unicast data delivery | Yes | The proposal specifies MAC-layer mesh broadcast/multicast and unicast data delivery. | [1] Section 6.4.4 |  |
| TOPO_RT_FWD_SCP4 | - Architecture to support alternative routing protocols and metrics | Yes | The proposal specifies an architectural framework that supports different routing protocols. It also specifies a mechanism to learn about alternative routing protocols and metrics supported by neighbor Mesh Points. | [1] Sections 6.4, 6.12, and 6.3.2 |  |
| TOPO_RT_FWD_SCP5 | - Mesh routing with single-radio devices | Yes | The proposal supports mesh routing with single or multiple radios. | [1] Sections 6.2 |  |
| TOPO_RT_FWD_SCP6 | - Mesh routing with multiple-radio devices | Yes | The proposal supports mesh routing with single or multiple radios. | [1] Section 6.2, 6.4.3.1.5, and 6.4.3.2.6 |  |
| TOPO_RT_FWD_SCP7 | - Use of radio-aware route selection metrics | Yes | The proposal specifies an extensible framework for the use of arbitrary route selection metrics (via metric IDs and IEs) and specifies a default radio aware route selection metric based on airtime for interoperability. | [1] Sections 5.3.3.1.2, 6.4.2, and 6.4.3 |  |
| TOPO_RT_FWD_SCP8 | - QoS-based route selection |  | The proposal specifies a framework for the use of arbitrary route selection metrics, including those relating to QoS. | [1] Section 6.4 |  |
| TOPO_RT_FWD_SCP9 | - Proactive routing | Yes | The proposal specifies the optional Radio-aware OLSR routing protocol. | [1] Section 6.4.3.2 |  |
| TOPO_RT_FWD_SCP10 | - On-demand routing | Yes | The proposal specifies the default Radio-metric AODV routing protocol. | [1] Section 6.4.3.1 |  |
| TOPO_RT_FWD_SCP11 | - Hybrid routing |  |  |  |  |
| TOPO_RT_FWD_SCP12 | - Mesh Point neighbor discovery within a WLAN Mesh via passive scanning | Yes | The proposal describes neighbor discovery through beacons. | [1] Section 6.3 |  |
| TOPO_RT_FWD_SCP13 | - Mesh Point neighbor discovery within a WLAN Mesh via active scanning | Yes | The proposal describes neighbor discovery using Mesh Probe Requests. | [1] Section 6.3 |  |
| TOPO_RT_FWD_SCP14 | - Mesh routing in the presence of low-power (e.g. battery-powered) Mesh Points | Yes | The optional RA-OLSR protocol supports this feature through 'N_willingness' of the node in MPR selection: For example, we can set 'N_willingness' of the low-power MPs to 'WILL_NEVER' in order to exclude them in MPR selection. | [1] Section 6.4.3.2.10 |  |
| TOPO_RT_FWD_SCP15 | - Support for WLAN Mesh network configurations with more than 32 Mesh Points. | Yes | Both default and optional routing algorithms in the proposal support cofigurations with more than 32 Mesh Points. | [1] Section 6.4.3 |  |
| TOPO_RT_FWD_SCP16 | - Ability to recognize changes in the topology within a bounded time |  |  |  |  |
| TOPO_RT_FWD_SCP17 | - Ability to reconfigure the routing scheme within a bounded time in response to detected changes |  |  |  |  |
| TOPO_RT_FWD_SCP18 | - SAP interfaces and management primitives to provide support to enable higher-layer (e.g., Layer 2.5 and Layer 3) routing protocols | Yes | The proposal specifies SAP interfaces and management objects which support higher-layer implementation of path selection and forwarding protocols. | [1] Section 6.12 |  |
| TOPO_RT_FWD_SCP_Other |  |  |  |  |  |
| Mesh Security (SECURITY) |  |  |  |  |  |
| SECURITY_SCP1 | - Secure association of Mesh Points to a WLAN Mesh | Yes | The proposal specifies 802.11i mechanisms for secure association. | [1] Section 6.5 |  |
| SECURITY_SCP2 | - Securing a WLAN Mesh in which all of the Mesh Points are controlled by a single logical administrative entity for security. | Yes | The 802.11i procedures specified in the proposal require that all Mesh Points be controlled by a single logical administrative entity for security. | [1] Section 6.5 |  |
| SECURITY_SCP3 | - Secure data message exchange between Mesh Points over mesh links | Yes | 802.11i procedures will be used to secure data message exchanges over mesh links. | [1] Section 6.5 |  |
| SECURITY_SCP4 | - Secure management message exchange between Mesh Points over mesh links | Yes | Management message exchange will be secured using 802.11i and, possibly, 802.11w procedures. | [1] Section 6.5.3 |  |
| SECURITY_SCP5 | - Secure topology and routing information exchange between Mesh Points over mesh links | Yes | 802.11i procedures will be used to secure topology and routing information exchange. | [1] Section 6.5 |  |
| SECURITY_SCP6 | - Centralized authentication and key management | Yes | 802.1X procedures will be used for centralized AKM | [1] Section 6.5.1.2 |  |
| SECURITY_SCP7 | - Distributed authentication and key management | Yes | 802.1X procedures will be used for distributed AKM | [1] Section 6.5.1.3 |  |
| SECURITY_SCP8 | - Hybrid authentication and key management |  |  |  |  |
| SECURITY_SCP9 | - Static key support | Yes | The 802.1X procedures specified in the proposal support static key based security. | [1] Sections 6.5.1.2, 6.5.1.3 |  |
| SECURITY_SCP10 | - Dynamic key support | Yes | The 802.1X procedures specified in the proposal support dynamic key based security. | [1] Sections 6.5.1.2, 6.5.1.3 |  |
| SECURITY_SCP11 | - Extension of IEEE 802.11i security mechanisms for mesh | Yes | The proposal provides an extensible framework for security, based on 802.11i procedures. | [1] Section 6.5 |  |
| SECURITY_SCP_Other |  |  |  |  |  |
| Mesh Measurement (MEAS) |  |  |  |  |  |
| MEAS_SCP1 | - Specification of radio-aware metrics for use by mesh routing protocols | Yes | The proposal specifies a default radio-aware routing metric for use by the mesh routing protocol(s) and includes a framework to allow other routing metrics to be implemented in the standard framework, specifically it defines a mechanism to inform neighbors of link-dependent radio-aware parameters | [1] Sections 6.4.2, 6.3.6.2.1, and 5.3.3.4 |  |
| MEAS_SCP2 | - Specification of radio-aware metrics for use by mesh medium access coordination | Yes | The congenstion control gives a neighbour channel load information | [1] Section 6.7 |  |
| MEAS_SCP3 | - Mesh link quality measurements | Yes | The proposal speficies that MPs exchange and maintain mesh link quality with neighbors | [1] Section 6.3.6 |  |
| MEAS_SCP4 | - Mesh path quality measurements | Yes | The proposal specifies an extensible framework for the use of arbitrary route selection metrics (via metric IDs and IEs) and specifies a default radio aware route selection metric based on airtime for interoperability. | [1] Sections 5.3.3.1.2, 6.4.2, and 6.4.3 |  |
| MEAS_SCP5 | - Measurements to support the use of various types of antennas in a mesh network |  |  |  |  |
| MEAS_SCP6 | - Mesh related measurements to aid STAs in making roaming decisions, e.g., WDS capacity currently available for traffic forwarding. |  |  |  |  |
| MEAS_SCP_Other |  |  |  |  |  |
| Mesh Discovery and Association (DISC_ASSOC) |  |  |  |  |  |
| DISC_ASSOC_SCP1 | - Protocols to allow Mesh Points to discover WLAN Mesh networks | Yes | The proposal specifies both passive and active mechanisms to discover WLAN Mesh networks and identifies specific Information Elements to perform discovery. | [1] Section 6.3 |  |
| DISC_ASSOC_SCP2 | - Protocols to allow Mesh Points to associate and disassociate with a WLAN Mesh network | Yes | The proposal specifies mechanisms for a Mesh Point to both discover a WLAN Mesh network and associate with its members | [1] Section 6.3 |  |
| DISC_ASSOC_SCP3 | - Protocols to allow Mesh Points to associate and disassociate with other Mesh Points within a WLAN Mesh | Yes | The proposal defines a Peer Link Maintenance mechanism, which is akin to association/disassociation. | [1] Section 6.3.6.1 |  |
| DISC_ASSOC_SCP_Other |  |  |  |  |  |
| Mesh Medium Access Coordination (MMAC) |  |  |  |  |  |
| MMAC_SCP1 | - Mitigate performance degradation caused by hidden nodes | Yes | The proposal provides informative text and guidelines on NAV setting mechanisms to reduce the effect of hidden node problem. | [1] Section 6.6.1 |  |
| MMAC_SCP2 | - Mitigate performance degradation caused by exposed nodes | Yes | The proposal provides informative text and guidelines on NAV setting mechanisms to reduce the effect of exposed node problem. | [1] Section 6.6.1 |  |
| MMAC_SCP3 | - Flow control over multi-hop paths to avoid performance degradation and/or meet QoS goals. | Yes | The proposal specifies a congestion/flow control mechanism which avoids performance degradation due to multi-hopping and also provides hooks to implement mechanisms to meet the required QoS goals of the flows. | [1] Section 6.7 |  |
| MMAC_SCP4 | - Coordinating channel access across multiple nodes to avoid performance degradation and/or meet QoS goals in the multi-hop network. |  |  |  |  |
| MMAC_SCP5 | - Traffic prioritization within a WLAN Mesh | Yes | The proposal exploits 11e based mechanisms to provide traffic prioritization. | [1] Section 6.6 |  |
| MMAC_SCP6 | - Enhancements to make the MAC work well across a range of different network sizes, usage models, etc. |  |  |  |  |
| MMAC_SCP7 | - Mesh link communication coordination |  |  |  |  |
| MMAC_SCP8 | - Support for admission control to determine if a particular flow can be admitted into the mesh network based on the availability of resources and existing usages. |  |  |  |  |
| MMAC_SCP9 | - Mangement of multiple classes of traffic, for example when both BSS traffic and mesh forwarding traffic are present in one device (e.g. in a Mesh AP). | Yes | The proposal provides informative text on interaction of forwarding and BSS traffic in a Mesh Point. | 6.6.2 |  |
| MMAC_SCP10 | - Rate control enhancements for efficient multicasting and broadcasting in a mesh network. |  |  |  |  |
| MMAC_SCP11 | - Improving spatial reuse in a mesh network. |  |  |  |  |
| MMAC_SCP_Other |  |  |  |  |  |
| Compatibility to 802.11 Services (SERV_CMP) |  |  |  |  |  |
| SERV_CMP_SCP1 | - Mesh Point DS Services (DSS) Integration | Yes (implicitly) | A mesh implements WDS but does not change any of the AP functions including DSS support. |  |  |
| SERV_CMP_SCP2 | - WLAN Mesh compatibility with STA mobility/roaming | Yes (implicitly) | No change to AP is proposed and hence current support for mobility/roaming remains the same. |  |  |
| SERV_CMP_SCP3 | - Techniques to allow WLAN Mesh to meet 802.11r system requirements |  |  |  |  |
| SERV_CMP_SCP4 | - Dissemination of STA-to- destination MeshAP or STA-to-destination Mesh Portal routing information in the WLAN Mesh. | Yes | MAPs generate and manage Radio Metric AODV messages on behalf of the legacy STAs that are associated with them. Optinally, MAPs generate and manage association discovery messages with Radio Aware OLSR messages. | [1] Sections 6.4.3.1.3, 6.4.3.2.14 |  |
| SERV_CMP_Other |  |  |  |  |  |
| Mesh Interworking (INTRWRK) |  |  |  |  |  |
| INTRWRK_SCP1 | - Allow a WLAN Mesh to interface with higher layer protocols | Yes | The proposal specifies how to make the WLAN Mesh seem like any other 802 LAN segment. | [1] Section 6.8 |  |
| INTRWRK_SCP2 | - Interfacing a WLAN Mesh with other IEEE 802 LANs | Yes | The proposal enables bridging a layer-2 WLAN Mesh to other 802 LANs in a manner compatible with 802.1D. | [1] Section 6.8.2 |  |
| INTRWRK_SCP3 | - Support for interfacing a WLAN Mesh with other IEEE 802 LANs using 802.1D | Yes | The proposal enables bridging a layer-2 WLAN Mesh to other 802 LANs in a manner compatible with 802.1D. | [1] Section 6.8.2 |  |
| INTRWRK_SCP4 | - Support for efficient utilization of multiple Mesh Portals in a single WLAN Mesh. |  |  |  |  |
| INTRWRK_SCP_Other |  |  |  |  |  |
| Mesh Configuration and Management (CFG_MGMT) |  |  |  |  |  |
| CFG_MGMT_SCP1 | - Protocol extensions to support self-configuring formation of a WLAN Mesh network | Yes | The proposal specifies a topolgy discovery and formation mechanism. | [1] Section 6.3 |  |
| CFG_MGMT_SCP2 | - Interfaces to support 802.11h DFS compliancy | Yes | The proposal supports DFS in a WLAN mesh. | [1] Section 6.9 |  |
| CFG_MGMT_SCP3 | - Interfaces and parameter exchange to enable RF auto configuration support |  |  |  |  |
| CFG_MGMT_SCP4 | - Support for managed network management model |  |  |  |  |
| CFG_MGMT_SCP5 | - Support for unmanaged network management model |  |  |  |  |
| CFG_MGMT_SCP6 | - Interfaces and parameter exchange to exchange information about the capabilities of Mesh Point devices | Yes | The proposal specifies that MPs can exchange information regarding their capabilities by use of beacon or probe frame. Moreover, it defines an information element to exchange routing capabilities of Mesh Points. | [1] Sections 6.3.4, 5.3.3.2 |  |
| CFG_MGMT_SCP7 | - Mesh network channel selection | Yes | The proposal specifies a channel selection protocol. | [1] Sections 4.2.3, 6.3.5 |  |
| CFG_MGMT_SCP8 | - Mesh network Tx power control | Yes | The proposal gives a framework to resuse 11h TPC mechanism. | [1] Section 5.3.4.10 |  |
| CFG_MGMT_SCP9 | - QoS policy and management |  |  |  |  |
| CFG_MGMT_SCP10 | - Support for time synchronization of Mesh Points if required by mesh services |  |  |  |  |
| CFG_MGMT_SCP_Other |  |  |  |  |  |

# References

[1] IEEE 802 11-05/562r0, 802.11 TGs Simple Efficient Extensible Mesh (SEE-Mesh) Proposal

[2] IEEE 802 11-04/54r2, PAR for IEEE 802.11s ESS Mesh

[3] IEEE 802 11-04/56r1, Five Criteria for IEEE 802.11s ESS Mesh

[4] IEEE 802 11-04/662r13, TGs Usage Models

[5] IEEE 802.11-04/1477r1, Terms and Definitions for 802.11s

[6] IEEE 802.11-04/1174r13, 802.11 TGs Functional Requirements and Scope