IEEE P802.11
Wireless LANs

| 802.11 TGs MAC Enhancement Proposal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date: 2005-09-12 |  |  |  |  |  |  |  |  |  |
| Author(s): |  |  |  |  |  |  |  |  |  |
| Name | Company |  | Address |  | Phone |  | email |  |  |
| Ming Sheu |  | Accton |  | 1362 Borregas Avenue, Sunnyvale CA, 94089, USA |  | +1 (408) 747-0994 |  | ming_sheu@accton.com |  |
| Ted Kuo |  | Accton |  | 1362 Borregas Avenue, Sunnyvale CA, 94089, USA |  | +1 (408) 747-0994 |  | ted_kuo@accton.com |  |
| Tyan-Shu Jou |  | Accton |  | 1362 Borregas Avenue, Sunnyvale CA, 94089, USA |  | +1 (408) 747-0994 |  | ts_jou@accton.com |  |
| Jua-Ru Li |  | Extreme Networks |  | 3585 Monroe Street, Santa Clara, CA, 95051, USA |  | +1 (408) 579 3374 |  | jili@extremenetworks.com |  |
| Catherine Livet |  | InterDigital |  | 1000 Sherbrooke W. 10th Floor, Montreal, QC, CANADA |  | +1 (514) 904 6252 |  | catherine.livet@interdigital.com |  |
| John Tomici |  | InterDigital |  | Two Huntington Quadrangle, 3rd Floor, Melville, NY 11747, USA |  | +1 (631) 622 4079 |  | john.tomici@interdigital.com |  |
| Juan Carlos Zuniga |  | InterDigital |  | 1000 Sherbrooke W. 10th Floor, Montreal, QC, CANADA |  | +1 (514) 904 6251 |  | juancarlos.zuniga@interdigital.com |  |
| Marian Rudolf |  | InterDigital |  | 1000 Sherbrooke W. 10th Floor, Montreal, QC, CANADA |  | +1 (514) 904 6258 |  | marian.rudolf@interdigital.com |  |
| Vincent Roy |  | InterDigital |  | 1000 Sherbrooke W. 10th Floor, Montreal, QC, CANADA |  | +1 (514) 904 6263 |  | vincent.roy@interdigital.com |  |

| D. J. Shyy | MITRE | 7515 Colshire Drive, McLean, VA 22102, USA | +1 (703) 983-6515 | djshyy@mitre.org |
| --- | --- | --- | --- | --- |
| Nehru Bhandaru | NextHop | NextHop Technologies, Inc., 1911 Landings Drive, Mtn View, CA 94043 USA | +1 (650) 429 4800 | bhandaru@nexthop.com |
| Susan Hares | NextHop | 825 Victors Way, Suite 100, Ann Harbor, MI, USA | +1 (734) 222 1610 | shares@nexthop.com |
| Osama Aboul-Magd | Nortel | 3500 Carling Ave., Ottawa ON K2H 8E9, CANADA | +1 (613) 763 5827 | osama@nortel.com |
| Sheng Sun | Nortel | 3500 Carling Ave., Ottawa ON K2H 8E9, CANADA | +1 (613) 763 4460 | shengs@nortel.com |
| Tricci So | Nortel | 3500 Carling Ave., Ottawa ON K2H 8E9, CANADA | +1 (613) 763 9639 | tso@nortel.com |
| Don Fedyk | Nortel | 600 Technology Park Drive, Billerica, MA 01821 | +1 (978) 288 3041 | dwfedyk@nortel.com |
| Guido R. Hiertz | Philips & ComNets, RWTH Aachen University | ComNets, RWTH Aachen University, Kopernikusstr. 16, 52064 Aachen, GERMANY | +49 (241) 8025 829 | hiertz@ieee.org |
| Hans-Jürgen Reumerman | Philips | Weißhausstr. 2, 52066 Aachen, GERMANY | +49 (241) 6003-629 | hans-j.reumerman@philips.com |
| Hang Liu | Thomson | 2 Independence Way, Suite 300, Princeton, NJ, 08540, USA | +1 (609) 987 7335 | hang.liu@thomson.net |
| Jun Li | Thomson | 2 Independence Way, Suite 300, Princeton, NJ, 08540, USA | +1 (609) 987 7317 | jun.li@thomson.net |

| Saurabh Mathur | Thomson | 2 Independence Way, Suite 300, Princeton, NJ, 08540, USA | +1 (609) 987 7320 | saurabh.mathur@thomson.net |
| --- | --- | --- | --- | --- |
| Dennis J. Baker | Naval Research Laboratory | 100 Brickells Glade, Edenton, NC 27932 | +1 (252) 482 0747 | d.baker@mchsi.com |
| James P. Hauser | Naval Research Laboratory | Code 5521, Washington, DC 20375 | +1 (202) 767 2771 | hauser@itd.nrl.navy.mil |

Abstract

This document is a draft specification of the 802.11 TGs Wi-Mesh Alliance Proposal.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Document Version History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Proposal submission on 6/15/2005. | 15 June 2005 |
| R1 | Proposal updated | 17 July 2005 |
| R2 | Proposal submission on 9/12/2005 | 12 September 2005 |

- Table of Contents

1	Executive Summary	11

2	References	11

3	Definitions	12

4	Abbreviations and Acronyoums	14

5	Introduction	17

5.1	General Description	17

5.1.1	Usage Models	18

5.1.2	WLAN Mesh Networking Requirements	18

5.1.3	Operation Principles	19

5.1.4	Operation Modes and Radio Configuration Considerations	19

5.1.5	Topology Management	20

5.1.6	Routing Design Considerations	20

5.1.7	Interworking with other Networks and Higher Layer Protcols	21

5.2	WLAN Mesh Architecture	22

5.2.1	WLAN Mesh Reference Model	22

5.2.2	WLAN Mesh Links Architcture	23

6	Frame Formats	25

6.1	General Frame Format	25

6.1.1	Mesh Control Field	25

6.1.2	Address Fields Usage in Frame Header	26

6.2	Mesh Management Frame	27

6.2.1	Specific Management Frame Formats	27

6.3	Control Frames	32

6.4	Data Frames	32

6.4.1	Mdata Frame Format	33

6.4.2	Mdata+Tunnel Frame Format	34

6.4.3	Quality of Service Support	34

6.5	Information Elements	34

6.5.1	Mesh ID element	35

6.5.2	MP Capability element	35

6.5.3	Mesh IE Indicator element	36

6.5.4	Multicast RSN (mRSN) element	36

6.5.5	Authentication element	36

6.5.6	QoS Parameter Set element	37

6.5.7	Resource Configuration element	38

6.5.8	Power Saving Configuration element	39

6.5.9	RF Profile element	39

6.5.10	Mobility Profile element	39

6.5.11	Mesh Measurement Capability element	39

6.5.12	Mesh Channel Switch Announcement element	39

6.5.13	MP Measurement Request element	40

6.5.14	MP Measurement Report element	40

6.5.15	MP TPC Request element	41

6.5.16	MP TPC Report element	41

6.5.17	MP Neighbor Request element	42

6.5.18	MP Neighbor Report element	42

6.5.19	MP Channel Report element	42

6.5.20	MP RCPI Report element	43

6.5.21	Routing element	43

6.6	Link State Informational Elements [optional]	53

6.6.1	Action header	54

6.6.2	Routing IE contents IE in the Action Frames Link state Hello packet	54

6.6.3	Link State Database Request packet IE fields	55

6.6.4	Link State Announcement IEs in Action Frame	58

6.7	Action Frame for Hybrid On-Demand Pro-Active Routing Protocols	63

6.7.1	RREQ Element	63

6.7.2	RREP Element	64

6.7.3	RANN Element	64

6.7.4	RERR Element	65

6.8	Applicability of Mesh-specific IEs to Frame Type	66

6.9	Frame Aggregation	68

7	MAC Sublayer Functional Description	70

7.1	Mesh Discovery and Association	70

7.1.1	Mesh Discovery	70

7.1.2	Mesh Association	72

7.2	Mesh Medium Access Control and Coordination Function	75

7.2.1	Mesh Transmission Opportunity	75

7.2.2	Distributed Reservation Control Access (DRCA)	76

7.2.3	Multi-Radio Support	89

7.2.4	MCF Operation	90

7.3	RF Resource Management	101

7.3.1	Transmit Power Control	101

7.3.2	Mesh Link Maintenance (informative)	103

7.3.3	Mesh Measurements	104

8	Mesh Management	105

8.1	Routing Architecture	105

8.1.1	Overview	105

8.1.2	The Need for Adopting Different Routing Algorithms for Different Networks	106

8.1.3	Highlights of the Routing Protocols	107

8.1.4	Summary	109

8.2	Forwarding Table Generation	109

8.2.1	Static Setup of FDB	109

8.2.2	Dynamic Setup of FDB	109

8.3	Hello Discovery protocol	110

8.3.1	Neighbor Discovery- "hello neighbour" Processing	111

8.3.2	Ack Implication with Neighbor IE Field	118

8.4	Hybrid Link State Routing Protocol	119

8.4.1	Link State Database Update process	119

8.4.2	SPF Algorithm	124

8.5	Hybrid On-Demand and Pro-Active protocol	127

8.5.1	On-demand Unicast Path Estrablishment	127

8.5.2	Proactive Path Establishment	129

8.5.3	Route Maintenance	131

8.5.4	Support of Quality of Service via Contraint-Based Routing	132

8.5.5	Support for Non-Forwarding MP	133

8.5.6	Support for IEEE 802.11 Stations	133

8.5.7	Interaction between ARP and Route Establishment	134

8.5.8	Multicast Routing	134

8.6	Mesh Interworking	137

8.6.1	Background	137

8.6.2	WLAN Mesh Interworking Frame Translation	140

8.7	Consistent Routing Topology via Hello LSA update	142

8.8	Traffic Engineering Routing	142

8.9	Power Saving	143

8.9.1	Discovery and Association	145

8.9.2	Resource Allocation and Medium Access	145

8.9.3	Routing	145

8.10	Multicast/Broadcast Reduction Experimental Results	145

9	Mesh Security	146

9.1	Security Architecture	147

9.2	Support of Hop-by-Hop and Multi-Hop Encryption	149

9.2.1	Use of Pair-Wise Keys (PMK, PTK)	149

9.2.2	Use of Group Key (GTK)	150

9.2.3	Use of Neighbor Keys (NTK)	151

9.2.4	Multicast Group Specific keys (MMK/MTK)	152

9.3	Authentication of Pre-Association Management Frames prevents Forgery	153

9.3.1	RSNA Esablishment	154

9.4	Security for Different Types of Data Paths	157

9.4.1	Mesh Management Frames Security	157

9.4.2	Mesh Control Frames	157

9.4.3	Mesh Data Frames	157

9.5	Mesh Association and Keys	159

9.5.1	RSNA keys Exchange PTK, GTK	159

9.5.2	Keys exchanged for PTK, GTK, and an NTK per peer.	160

9.5.3	Keys exchanged with PTK, GTK, NTK per Mesh point, and MMK/MTK (pre-configured MAC addresses)	160

9.5.4	MTK Set-up Group	161

9.6	Key Update and Re-Key	162

9.6.1	Re-keying based on a timer or change in MP membership	162

9.6.2	Re-keying times during failures	163

9.6.3	Change in multicast membership	164

9.6.4	SME event	164

9.7	Optional MD5 Authentication Methodology	164

10	Annex A (informative)	167

10.1	IEEE 802.11 Contention Free Period	167

10.2	Beacon generation and application in 802.11	168

10.3	Concurrent transmission on the same frequency channel	168

10.4	Learning Mesh Points	169

10.5	DEVID addressing	170

10.6	Beacon dissemination	171

10.7	Alternative Beacon Timing Structure	171

10.8	Measuring the Learning Performance	172

10.8.1	The World Model	173

10.8.2	Learning the Network’s Participants	176

10.8.3	Learning the Signal Strength	176

- List of Figures

Figure 5-1: Usage Models	18

Figure 5-2: Mesh Networks Interworking	22

Figure 5-3: WLAN Mesh Reference Model	23

Figure 5-4: Mesh Hierarchical ID	24

Figure 6-1: General Frame Format	25

Figure 6-2. Mesh Control field format	25

Figure 6-3. Example of multi-hop transmission of Mesh Management frame	27

Figure 6-4: Mdata Frame Format	33

Figure 6-5: Mdata + Tunnel Frame Format	34

Figure 6-6: The general Structure of a packet train, constituting of a header and several wagons	68

Figure 6-7: The structure of the train header	69

Figure 6-8: The structure of a data frame	70

Figure 7-1: Frame Format for Mesh Beacon/Mesh Probe Response	71

Figure 7-2: Mesh Discovery Example- Nominal (Passive) Scenario	73

Figure 7-3: Mesh Discovery Example- Nominal (Active) Scenario	73

Figure 7-4: Mesh Discovery Example- Expedited (Passive) Scenario	74

Figure 7-5: Mesh Discovery Example- Expedited (Active) Scenario	74

Figure 7-6: Mesh Discovery Example- Expedited (Passive) Scenario Using Mesh Report	75

Figure 7-7: Distributed Reservation Control Access (DRCA)	77

Figure 7-8: Structure of a MTXOP Ownership	80

Figure 7-9: DRCA Message negotiation	83

Figure 7-10: The layered structure incorporating legacy 802.11	90

Figure 7-11: The detailed structure of a beacon period with three zones	91

Figure 7-12: The standards 802.11 beacon with a CFP Parameter set and a BPOIE	92

Figure 7-13: Alternation of BSS-traffic and Mesh-traffic	95

Figure 7-14: Clock Drift Re-Synchronization	98

Figure 7-15: Mesh Link Maintenance Example	103

Figure 7-16: Multi-hop Measurements Example	105

Figure 8-1: Hello message	111

Figure 8-2: Beacon sending Information about Wireless Stations not in the Mesh	112

Figure 8-3: Example of Simple Flooding and Sending of LSA	123

Figure 8-4: Link State Calculation	124

Figure 8-5: Joining a Multicast Group	125

Figure 8-6: Leaving a Multicast Group	126

Figure 8-7: Broken Tree Repair	126

Figure 8-8: On Demand Unicast Route Establishment	129

Figure 8-9: Proactive Routing	131

Figure 8-10: the method whereby a MP establishes the routes on-demand and proactively, and processes the routing control messages.	132

Figure 8-11: Mesh AP with Multiple Assocaited Stations	133

Figure 8-12: A node joins the multicast group	135

Figure 8-13: A node leaves Multicast group	136

Figure 8-14: Repair of broken tree link	137

Figure 8-15: IEEE Std. 802.1D Transparnet Bridging Architecture	139

Figure 8-16: IEEE Std. 802.1Q VLAN Bridging Architecture	140

Figure 8-17: WLAN Mesh Interworking Architecture	141

Figure 18 Hello Protocol	142

Figure 19 Traffic Classification	143

Figure 20 Example of Power Saving	144

Figure 8-21: Multicast/Broadcast reduction Results	146

Figure 9-1: Secuirty Architecture	149

Figure 9-2: Secure Architecture with Tunnels	150

Figure 10-1: As the basic configuration for simple Mesh WLAN Access Points, a single radio is sufficient to provide the AP services to the stations and to be able to participate in the Mesh WLAN.	167

Figure 10-2: A simple scenario where spatial reuse is possible	168

Figure 10-3: An optimal alignment of the transmissions during time for scenario in Figure 10-2.	169

Figure 10-4: The general structure if an interference-aware Mesh Points	170

Figure 10-5: While MP1 sends a beacon, the beacon period access protocol has to avoid interference by MP4	171

Figure 10-6: The four steps of a BP contraction	172

Figure 10-7: RSS measurement while (a) Tx transmits or (b) Rx receives.	173

Figure 10-8: The signal strength graph for scenario with stations Tx Rx and 1 to 3	174

Figure 10-9: The signal strength graph for the scenario given in Figure 10-2	175

Figure 10-10: Mesh Point "2" wants to learn the signal strength of route (1)	177

Figure 10-11: Structure of the CFP with Mesh Traffic Period and Beacon Period	178

- List of Tables

Table 6-1. To/From Intermediate Node bit combinations for mesh	26

Table 7-1: The Four Possible Beacon Slot States, as indicated in BP Bitmap	93

Table 10-1: Interference CoI and Receiver CoI if a simultaneous transmission from Transmitter to Receiver would happen	175

Table 10-2: The minimum signal strength for the successful reception, depending on the PHY - mode	177

# Executive Summary

This document presents a full proposal for a scalable, adaptive and secure 802.11s mesh standard. The proposal offers the flexibility required to satisfy all residential, office, campus, public safety and military usage models. The proposal focuses on multiple dimensions: the MAC sublayer, the routing, the security and high layer interworking

The proposal supports both single-radio and multi-radio platforms. Moreover, the proposed Medium Coordination Function offers three modes of operation which allow for simple and robust implementations as well as for sophisticated solutions offering optimal performance and spectrum efficiency. The proposal capitalizes on existing 802.11 amendments by extending QoS prioritization schemes, measurement mechanisms and spectrum management solutions of 802.11 e, k and h to the reality of mesh systems. The proposal also includes features such as: adaptive physical carrier sensing for enhanced spectrum spatial reuse, channel access coordination and RF resource management solutions

It provides an extended mesh discovery solution with dynamic auto-configuration features and integrated BSS and WLAN 802.11e QoS traffic handling. Security is addressed and extensions and enhancements are provided over the current 802.11i.

# References

- http://www.wi-mesh.org

- http://www.accton.com

- http://www.interdigital.com

- http://www.mitre.org

- http://www.nexthop.com

- http://www.nortel.com

- http://www.philips.com

- http://www.thomson.net

- http://www.comnets.rwth-aachen.de

- IEEE Std. 802.11, 1999

- IEEE Standard for Information Technology – Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless Medium Access Control (MAC) and Physical Layer (PHY) specifications: Amendment: Medium Access Control (MAC) Quality of Service (QoS) Enhancements, IEEE Draft Amendment IEEE P802.11e/D13.0, Jan. 2005.IEEE P802.11e/D13.0: Amendment: Medium Access Control (MAC) Quality of Service Enhancements

- 11-04/54r2 “PAR for IEEE 802.11 ESS Mesh”

- 11-04/56r1 “Five Criteria for IEEE 802.11 ESS Mesh”

- 11-04/1430r12 „Call for Proposals for IEEE 802.11s“

- 11-04/1174r13 „Functional Requirements and Scope for IEEE 802.11s“

- 11-04/1175r10 „Comparison Categories and Informative Checklists for IEEE 802.11s“

- 11-04/662r16 „Usage Models for IEEE 802.11s“

- 11-04/1477r4 „Terms and Definitions for 802.11s“

- 11-04/1075r0 „Mesh networks: The benefits of interference awareness“, Sep. 2004

- 11-04/0709r2 „Multi hop connections using 802.11“, Jul. 2004

- 15-04/0312r0 „Applications and usage scenarios for Mesh-WPAN“, Jul. 2004

- 11-04/0530r0 „Mesh networks for home entertainment“, May 2004

- 11-04/0558r2 „Is the 802.11 MAC sufficient for wireless high speed mesh LANs?“, May 2004

- “Throughput and Delay Performance of IEEE 802.11e Wireless LAN with Block Acknowledgments“, Hiertz, Guido and Stibor, Lothar and Habetha, Jörg and Weiss, Erik and Mangold, Stefan, Proceedings of 11th European Wireless Conference 2005, Apr. 2005

- „Reservation-based Spectrum Load Smoothing as Cognitive Medium Access for Spectrum Sharing Wireless Networks“, Berlemann, Lars and Hiertz, Guido and Walke, Bernhard, Proceedings of European Wireless Conference 2005, Apr. 2005

- „IEEE 802.11e/802.11k wireless LAN: spectrum awareness for distributed resource sharing“, Mangold, Stefan and Zhong, Zhun and Hiertz, Guido and Walke, Bernhard, Wireless Communications and Mobile Computing 2004, Dec. 2004

- „A cut-through switching technology for IEEE 802.11“, Hiertz, Guido and Habetha, Jörg and Weiss, Erik and Mangold, Stefan, Proceedings of the IEEE 6th Circuits and Systems Symposium on Emerging Technologies: Frontiers of Mobile and Wireless Communication, May 2004

- „Improving Ad Hoc Routing for Future Wireless Multihop Networks“, Weiss, Erik and Frewel, Markus and Hiertz, Guido and Xu, Bagnan, Proceedings of the fifth European Wireless Conference: Mobile and Wireless Systems beyond 3G, EW04, Feb. 2004

- „Analysis of IEEE 802.11 for QoS Support in Wireless LANs“, Mangold, Stefan and Choi, S. and Hiertz, Guido and Klein, Ole and Walke, Bernhard, IEEE Wireless Communications, Dec. 2003

- „IEEE 802.11e Wireless LAN - Resource Sharing with Contention Based Medium Access“, Mangold, Stefan and Hiertz, Guido and Walke, Bernhard, Proceedings of the PIMRC 2003, Sep. 2003

- „A Decentralized Reservation Scheme for IEEE 802.11 Ad Hoc Networks“, Hiertz, Guido and Habetha, Jörg and May, Peter and Weiss, Erik and Bagul, Rajesh and Mangold, Stefan“, The 14th IEEE 2003 International Symposium on Personal, Indoor and Mobile Radio Communications, Sep. 2003

- „A new MAC Protocol for a wireless multi-hop broadband system beyond IEEE 802.11“, Hiertz, Guido and Habetha, Jörg, Wireless World Research Forum, 9th Meeting in Zurich, Switzerland, Jul. 2003

- “Fisheye state routing: a routing scheme for ad hoc wireless networks”, Pei, G and Macker, J, Proceedings of the IEEE International Conference on Communications (ICC), 2000

- “Optimized Link State Routing Protocol”, Laouiti, A and Viennot, V and Clausen, V., Internet Draft, draft-ietf-manet-olsr-07.txt, 2005

# Definitions

Insert the following new definitions at the appropriate locations in clause 3 of IEEE 802.11-1999, renumbering as necessary

- WLAN Mesh – A WLAN Mesh (previously known as ESS Mesh) is an IEEE 802.11-based WDS which is part of a DS, consisting of a set of two or more Mesh Points interconnected via IEEE 802.11 links and communicating via the WLAN Mesh Services. A WLAN Mesh may support zero or more entry points (Mesh Portals), automatic topology learning and dynamic path selection (including across multiple hops).

- WLAN Mesh Services – The set of services provided by the WLAN Mesh that support the control, management, and operation of the WLAN Mesh, including the transport of MSDUs between Mesh Points within the WLAN Mesh. WLAN Mesh Services supplement DSS (Distribution System Services).

- Mesh Point - Any IEEE 802.11 entity that contains an IEEE 802.11–conformant Medium Access Control (MAC) and Physical Layer (PHY) interface to the Wireless Medium (WM), that is within a WLAN Mesh, and that supports WLAN Mesh Services.

- Mesh AP - Any Mesh Point that is also an Access Point.

- Mesh Portal - A point at which MSDUs exit and enter a WLAN Mesh to and from other parts of a DS or to and from a non-802.11 network. A Mesh Portal can be collocated with an IEEE 802.11 portal.

- Mesh Link - A bidirectional IEEE 802.11 link between two Mesh Points.

- Mesh Path - A concatenated set of connected Mesh Links from a source Mesh Point to a destination Mesh Point.

- Mesh Path Selection – The process of selecting Mesh Paths

- Path Metric – Criterion used for Mesh Path Selection.

- Mesh Topology – A graph consisting of the full set of Mesh Points and Mesh Links in a WLAN Mesh.

- Mesh Neighbour - Any Mesh Point that is directly connected to another Mesh Point with a Mesh Link.

- Mesh Unicast - Frame forwarding mechanism for transporting MSDUs to an individual Mesh Point within a WLAN Mesh.

- Mesh Multicast - Frame forwarding mechanism for transporting MSDUs to a group of Mesh Points within a WLAN Mesh.

- Mesh Broadcast - Frame forwarding mechanism for transporting MSDUs to all Mesh Points within a WLAN Mesh.

- Mesh Management Message – Message defined for managing and operating the mesh. The message is sent between Mesh Points, e.g. for path determination, neighbor discovery, topology discovery, etc. This definition of message is intended to be generic and does not specify the form of conveyor.

- Mesh Control Message - Message defined for controlling access to the WM (Wireless Media) used for communication among Mesh Points

- Shared Coordination Channel (SCC) – It is a logical channel used for exchange of control and management frames by all MPs in a particular mesh domain, e.g. for negotiating channel access for pending mesh data frames. The use of the SCC option may be enabled during MP start-up or via subsequent discovery/association procedures

- Mesh Unicast Acknowledgment – An acknowledgement sent back from destination Mesh Point to source Mesh Point of a mesh path for a unicast message.

- Authenticated Mesh Point – A Mesh Point that has been authenticated as a valid participant in the WLAN Mesh. The authentication is with respect to a common policy determined by a single administrative entity.

- Mesh Identifier (Mesh ID) – A unique identifier for a WLAN Mesh.

- Mesh Discovery - A method to discover a WLAN Mesh network’s characteristics.

- Mesh neighbourhood – A set of Mesh neighbours of a mesh point.

- Mesh Performance Metric – A criterion used to evaluate the performance of the Mesh.

- Mesh Link Metric – A criterion used to characterize the performance/quality/eligibility of a mesh link as a member of a mesh path. A mesh link metric may be used in a computation of a path metric.

- Connected Mesh – The status of the WLAN Mesh in which all Mesh Points that are participating members of a WLAN Mesh are reachable.

- Disconnected Mesh – The status of the WLAN Mesh in which a subset of Mesh Points that are participating members within the WLAN Mesh are not reachable. It is also called a partitioned Mesh

- Mesh Association - The service used to establish the Mesh Point membership within a WLAN Mesh. Mesh association is independent from STA association to an AP.

- Mesh Member – An associated Mesh Point.

- Mesh Member Set – The set of associated Mesh Points within a WLAN Mesh.

- Mesh Service Area - The conceptual area within which members of a WLAN Mesh may communicate.

- Mesh Coordination Function (MCF) - A logical function used to coordinate access of the Mesh Points (MPs) on the WM. The MCF is used for communication among MPs.

- Mesh Transmission Opportunity (MTXOP) – Interval of time during when 2 MPs exchange information over a Mesh Link on one or more specified channels

- Hop-by-Hop Encryption – Encryption that is unique per link

  - Link-by-link encryption – encryption that is unique per link

  - Global Multicast MAC encryption – encryption on Multicast frames (control or data) may be sent, depending on routing, to all mesh points

  - Local Multicast – multicast sent from a node to its neighbors

  - Virtual link – link that is created from a tunnel

  - Direct link – physical radio link to a neighbor

  - Group Leadermac – MAC group mesh point that takes the lead in security (rekeying and AS interaction)

  - Backup Group Leadermac – Back-up MP for a Group Leader to encrypted Multicast MAC (for rekeying and AS interactions)

- MP-Security Designated MP – designated node to handle re-keying request for GTK

- MP-Security Backup Designated MP – back-up for the MP-Secruity Designated MP in case of failure

- Tunnel WEP Flag - A flag (pre-configured or in mesh control frame header) that indicates tunnel encryption type)

# Abbreviations and Acronyoums

- Insert the following new acronyoums at appropriate locations in Clause 4 of IEEE 802.11 1999:

| ACK |  | Acknowledgment |
| --- | --- | --- |
| AKM |  | Authentication and Key Management |
| AODV |  | Ad-hoc On-demand Distance Vector protocol |
| AS |  | Authentication Server |
| ATP |  | Access Point Traffic Period |
| BP |  | Beacon Period |
| BPAP |  | Beacon Period Access Protocol |
| BPOIE |  | Beacon Period Occupancy Information Element |
| BTP |  | BSS Traffic Period |
| BTS |  | Beacon Transmission Slot |
| DA |  | Destination Address |
| DP |  | Drop Precedence |
| DRCA |  | Distributed Reservation Channel Access |
| DS |  | Distribution System |
| FCS |  | Frame Check Sequence |
| FDB |  | Forwarding Table |
| GRPH |  | Group Hello message |
| GTK |  | Group Transient Key |
| HCF |  | Hybrid Coordination Function |
| IE |  | Information Element |
| IOM |  | Internal Occupancy Map |
| LLC |  | Logical-Link Control |
| LSA |  | Link State Advertisment message |
| MACT |  | Multicast Activation message |
| MANET |  | Mobile Ad-hoc NETworks working group |
| MAP |  | Mesh Access Point |
| MCDS |  | Minimum Connected Dominating Set |
| MCF |  | Mesh Coordination Function |
| MCFP |  | Mesh Contention Free Period |
| MHC |  | Mesh Hybrid Coordinator |
| MIC |  | Message Integrity Check |
| ML |  | Mesh Link |
| MLID |  | Mesh Link Identifier |
| MMK |  | Multicast MAC Master Key |
| MOSPF |  | Multicast OSPF protocol |
| MP |  | Mesh Point |
| MPID |  | Mesh Point Identifier |
| MPDU |  | MAC Protocol Data Unit |
| MPR |  | Multi-Point Relay node |
| M-RSN ie field |  | information field that passes information about hints for 802.1X for MAC addresses |
| MSDU |  | MAC Service Data Unit |
| MTK |  | Multicast Transient Key |
| MTKmac |  | Multicast transient key for a multicast neighbor |
| MTP |  | Mesh Traffic Period |
| NS-MPR |  | Non Source-specific MPR |
| NTK |  | Generic form of NTKneighbor |
| NTKneighbor |  | Network Transient Key (NTK) for Multicast key per neighbor. |
| PCCA |  | Physical Clear Channel Assessment |
| PMK |  | Pairwise Master key |
| PSK |  | Pre-Shared Key |
| PSKSA |  | Pre-Shared key Security Association |
| PTK |  | Pair-wise Transient Key |
| QoS |  | Quality of Service |
| RA |  | Receiver Address |
| RANN |  | Route Announcement message |
| RREP |  | Route Reply message |
| RREQ |  | Route Request message |
| RSN |  | Robust Security Network |
| RSNA |  | Robust Security network Association |
| SA |  | Source Address |
| SA |  | Security Assocation |
| Sec BDMP |  | MP Security Backup Designated MP |
| Sec DMP |  | MP Security Designated MP |
| S-MPR |  | Source-specific MPR |
| SNR |  | Signal to Noise Ratio |
| SPF |  | Shortest Path First algorithm |
| TA |  | Transmitter Address |
| TCI |  | Tag Control Information |
| TE |  | Traffic Engineering |
| TPC |  | Transmit Power Control |
| TPID |  | Tag Protocol Identifier |
| TTL |  | Time To Live |
| VCCA |  | Virtual Clear Channel Assessment |
| VOIP |  | Voice Over IP |

# Introduction

## General Description

A WLAN Mesh is an ad-hoc collection of nodes, where node location and connectivity may change over time. A WLAN Mesh is characterized by its multi-hop wireless connected topology, and its use of frame switching and forwarding techniques. Once participating nodes and the wireless links that connect them are operational, the network looks like an interconnected mesh, hence the name.

A forefather of "mesh networks" is the class of Mobile Ad-hoc Networks (Manet) that grew primarily out of Dept. of Defense (DoD) research. The Manet routing working group of the Internet Engineering Task force (IETF) has been designing the routing algorithms associated with these systems.

The demand for a variety of wireless applications that could benefit from a mesh network is growing exponentially. Network application examples include CBRN (Chemical, Biological, Radiological and Nuclear) monitoring, mobile communication networks, vertically integrated solutions for point-of-sale (POS) and campus connectivity, consumer driven applications such as extending the range of commercial Wi-Fi hot spots and consumer electronics (CE) wireless mesh systems.

Given the variety of application scenarios described above, it is apparent that one single solution does not fit all. Each class of applications has a unique set of challenges that require a different set of wireless and networking technologies. As a result, the protocol specification of the IEEE 802.11s standard must be flexible and adaptive so that the solution is functional and applicable to varied networking deployment scenarios.

### Usage Models

The present proposal supports all usage models specified in the 802.11s Usage Models document [17].

The described usage models require from the system the ability to quickly create new deployments under varying propagation conditions (e.g. indoor/outdoor) without adding too much complexity and cost to the network.

Some models have certain characteristics that make them more challenging than others. For instance, the residential model (i.e. digital home/consumer electronics) comprises a relatively small number of nodes in an indoor limited area with none or a single mesh portal, and it usually handles high bandwidth applications (e.g. mainly peer-to-peer). On the other hand, the office and campus (i.e. community/public access network) models have a larger number of nodes, require a wide area coverage system, have one or multiple mesh portals and usually handle medium to high bandwidth applications. Finally, the public safety and military scenarios have a medium number of nodes with a topology with a higher rate of change, they handle relatively low bandwidth applications, and robustness and battery savings are critical.

- Figure 5-1: Usage Models

### WLAN Mesh Networking Requirements

A complete WLAN Mesh specification must provide support for a wide range of applications. The goals of the current specification are:

- Provide efficient transport for multimedia applications and services, such as voice, audio/video multicast, file transfer and web browsing.

- Require minimal configuration, and provide auto-discovery and self healing capabilities – i.e. ease of manageability

- Provide transparent support to 802.11 BSS STA, without compromising the WLAN Mesh data transport

- Allow various practical implementations and deployment configurations, ranging from very simple to highly efficient systems.

- Include mesh-specific QoS, radio and power-efficiency aware features

- Provide robust security compatible with 802.11i

- Interwork seamlessly with non-mesh 802.11 systems, as well as other networks (i..e. 802.3, 802.16, etc.)

### Operation Principles

A WLAN Mesh has an arbitrary topology. The network operates in a “peer-to-peer” fashion which means that each MP may have the routing capabilities built into it and can use its neighbors as the relay point to transmit traffic between mesh nodes or to the external network over the mesh topology.

This document defines the mesh neighbor auto-discovering and configuring negotiation techniques that are needed in order to establish the appropriate operation mode for the network applications. Once the MP neighbor peers are mutually authenticated, the RF resource information is exchanged and the peer-to-peer adjacency is established. The configured routing protocol discovers the logical network topology, establishes and maintains routing and forwarding tables enabling any-to-any communication.

To enable QoS classes over the WLAN Mesh having varying RF resources, distributed RF resource management and control signaling is also defined to coordinate and to prioritize the frame forwarding among peering nodes.

To optimize the RF resource utilization within the WLAN Mesh, especially for the larger scale deployment, support for a more intelligent system operation which leverages RF spatial reuse and RF frequency reuse technology with single or multi-radio is also defined.

### Operation Modes and Radio Configuration Considerations

The radio configuration has a direct impact on the performance and behavior of the network. The choice of physical layer (PHY) determines the quality of communication on a single link, and the choice of media access control (MAC) determines how well the radios can be operated together (e.g. avoiding collisions).

The type of network application is the main factor for choosing the WLAN operation mode for PHY and MAC. Nonetheless, operation modes may also be affected by regulatory, implementation and other constraints.

The operation mode of the WLAN Mesh determines the type of control signaling and data transport mechanism to be used, the type of security mechanism to be enabled across the WLAN Mesh, the type of link quality information that will be available to higher layers, the order of magnitude of the energy consumption of the system due to the data transmission and reception, and the data burst rates at which information can be exchanged in the network. Hence, it is important to select the correct alignment of operational mode and radio configuration to support a certain type of network applications. For instance, VoIP telephony requires coordinated QoS at every level of the infrastructure.

The current specification provides enough flexibility to support all the foreseen network applications under different constraints.

### Topology Management

Wireless link properties are determined by physical location and co-location of other wireless or electromagnetic devices. In the case of wireless mesh the physical topology and collocation of devices can be managed by RF optimization.  Choice of frequencies, power and choice of neighbors can alter the physical topology.  Two nodes may forgo connectivity with a weak signal when other neighbor nodes exist.  Given adhoc connectivity, RF control can optimize the physical topology, for best signals and best number of neighbors.

Mesh Routing systems work on the logical topology that is created by data links created over the MP neighbors. Routing systems typically optimize packets for shortest path routing over a given topology.

The load profile is determined by the actual applications that use the MP. This profile may be quite different from the capacity of the shortest path hops.  Routing topology can be utilized in link state routing to add additional capabilities for alternate paths.  In essence Routing topology can be manipulated to some degree to better adapt the traffic load profile to the network.  Also QoS mechanisms can be used to divert less urgent traffic from RF links with scare resources.

### Routing Design Considerations

Routing may be primitive, only discovering neighbors to intelligent discovering the whole mesh.  Routing is decomposed into the routing hello and the routing algorithm.  The routing hello is the common part that discovers the immediate neighbor's establishing adjacencies.

A distance vector routing protocol can be used to exchange reachability of neighbors beyond the peering nodes. Distance Vector routing has a summarize topology view that sees the neighbor node and the distance or metric from the neighbor to the other nodes in the network. Distance vector protocols are good at discovering reachability of nodes via a single shortest path.  Distance vector protocols are slower and less efficient at figuring out when nodes disappear. Distance vector would work well in small simple topologies or where the network was stable.

Link State protocols can also be used to exchange reachabilty of neighbors beyond the peers as well. Link state protocols deliver the true logical network topology to all neighbors and use common algorithms allow distributed convergence of the forwarding paths. Links state algorithms provide predictable convergent of networks regardless of the network events. Link State algorithms have been enhanced to allow add ional Traffic engineering capabilities which are not available in distance vector approaches.

Another important consideration is the multicast and broadcast support over WLAN Mesh. The fundamental difference for multicasting/broadcasting frame over the WLAN Mesh and over the wired network is that the frame forwarding in WLAN Mesh may arrive and be rebroadcast on the same physical interface. This condition limits some of the typical controls on broadcast/multicast storm mitigation in wired networks. Consequently, channel contention and RF interference will be increased significantly and degrade WLAN Mesh performance.

One way to reduce the multicast and broadcast overhead is to reduce the number of relay nodes (i.e. minimizes the number of frame forwarders) using the graph theory concept of minimum “connected” dominating set (MCDS). However, the tradeoff is between relay efficiency and delivery robustness. The design of the multicast forwarding for WLAN Mesh must consider such performance tradeoff to support various types of the multicast/broadcast solution to optimize the WLAN Mesh performance for a given network configuration.

### Interworking with other Networks and Higher Layer Protcols

The existing IEEE 802.1D specification defines the interworking framework and service access interface across 802 standards that supports 802.11 portal functions.. It is important for the 802.11s specification to comply with such framework in order to interwork with other layer 2 networks (e.g. 802.11s WLAN Mesh, 802.11 WLAN, 802.15 WPAN, 802.16 WMAN, 802.3 LAN, etc.), as well as with higher layer protocols.

Although support for the 802.11 portal and Mesh portal functions is required, the WLAN Mesh can also operate in isolaton. In other words, the actual existence of the Mesh Portal is optional, since it is possible for the WLAN Mesh to operate in an ad-hoc mode fashion without connecting to other WLAN Mesh network or any other external network.

- The following figure shows different WLAN Mesh network and Mesh Portal configurations.

- Figure 5-2: Mesh Networks Interworking

## WLAN Mesh Architecture

### WLAN Mesh Reference Model

One of the design goals of the WLAN Mesh specification is to leverage the existing 802.11 technologies as much as possible, but keeping backward compatibility and interoperability with existing 802.11 BSS (i.e. 802.11 Infrastructure Mode) systems. Hence, not only a new set of system functions is defined for WLAN Mesh, but also a new structure of system components are also introduced to the 802.11 Reference Model. The following figure describes the WLAN Mesh Reference Model.

- Figure 5-3: WLAN Mesh Reference Model

The new key functions for WLAN Mesh are:

- Mesh Control Function (MCF)

- RF Resource Measurement

- Dynamic Multihop Routing

- Security

- Configuration, Control and QoS Management

The design of the WLAN Mesh can be integrated over the existing 802.11e MAC as well as over the upcoming 802.11n MAC with the legacy BSS system. A new function, Distributed Reservation Control Access (DRCA), is designed for coordinating the WLAN Mesh Transmit Opportunity (MTXOP) between MPs. The concept of MTXOP is based on the IEEE 802.11e QoS design that is enhanced for supporting the WLAN Mesh multi-priority QoS transport.

### WLAN Mesh Links Architcture

In WLAN Mesh, a set of system components are introduced and their hierarchial structure is herein defined.

The WLAN Mesh domain is uniquely identified by a Mesh ID. Each WLAN Mesh is comprised of one or more Mesh Points (MP) which are also uniquely identified within the WLAN Mesh domain by Mesh Point IDs (MPID).

MPs communicate with their neighbors over logical channels that are called Mesh Links (ML) which are represented by Mesh Link IDs (MLID). There is one single logical channel between each pair of neighboring MPs. MLs can be mapped to one or more physical channels or radio channels.

- The following figure describes the relationships between these mesh hierarchical identifiers.

- Figure 5-4: Mesh Hierarchical ID

As shown in the figure, a ML is a logical, secured and bi-directional link entity between two MPs, and each ML may relate or map to one or more physical radios. However, for specific applications such as multicast, a unidirectional ML can also be defined between one transmitter and many neighboring receivers.

# Frame Formats

## General Frame Format

The general frame format is based on an extended version of the 802.11-1999 four address format. A Mesh Control field is added to the frame header to facilitate various mesh operations. The general frame format is shown in Figure 6-5.

| Frame Control | Duration/ID | Address 1 | Address 2 | Address 3 | sequence control | Address 4 | QoS Control | Mesh Control | Frame Body | FCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 6-5: General Frame Format

### Mesh Control Field

The format of the Mesh Control field is shown in Figure 6-6.

| Figure 6-6. Mesh Control field format b0-b1 | b2 - b7 | b8 | b9-b10 | b11-b15 | b16 - b31 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Resv | Mesh Frame Type | DP | Resv | Hop Count | Sequence Number | other fileds |

Mesh Frame Type is 6-bit field that defines the mesh data frame type

The discard precedence (DP) bit identifies the discard precedence of the incoming frame. Two discard precedence values are indicated by the DP bit:

| DP bit Value | Usage |
| --- | --- |
| 0 | Low discard precedence |
| 1 | High discard precedence |

Important frames that are crucial for network operation or for application performance are tagged with a low discard precedence values. When network resources are not sufficient due to congestion frames with high DP values will be discarded first before those with low discard precedence values. The discard mechanism is implementation dependent.

Hop Count is a 5-bit field used to eliminate the possibility of infinite loops within the WLAN mesh network.  Upon reception of a frame, each MP will decrement the Hop Count value prior to forwarding the frame onto the next hop.  A frame will stop being forwarded once its Hop Count value equals zero.

Sequence number is 16-bit field used to control broadcast flooding and to enable ordered delivery of messages within the WLAN mesh.

### Address Fields Usage in Frame Header

The 802.11 frames may carry 2, 3, or 4 addresses in the header to serve various purposes.  The “To DS” and “From DS” bits are currently used to indicate the address usage of the 802.11 MAC.  In a mesh network, the concept of “To DS” and “From DS” becomes vague.  The meaning of “To DS” and “From DS” in the mesh frame context is extended to indicate “To Intermediate Node” and “From Intermediate Node” to better reflect the meaning of those bits.  In addition, usage of “BSSID” in the address fields also need to be re-defined.  The new usage of address fields is listed in Table 6-1 below.

| To Intermediate Node | From Intermediate Node | Address 1 | Address 2 | Address 3 | Address 4 | Frame Hops | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | RA=DA | TA=SA | - | - | Single hop | Frame sent directly from original source MP to final destination MP |
| 0 | 1 | RA=DA | TA | SA | - | Last hop | Frame sent from an intermediate MP to final destination MP |
| 1 | 0 | RA | TA=SA | DA | - | First Hop | Frame sent from original source MP to intermediate MP |
| 1 | 1 | RA | TA | DA | SA | Intermediate hop | Frame sent from an intermediate MP to another intermediate MP |

Table 6-1. To/From Intermediate Node bit combinations for mesh

Figure 6-7 illustrates the transmission of a Mesh Management frame from a Source MP to a Destination MP via three intermediate MPs.

Figure 6-7. Example of multi-hop transmission of Mesh Management frame

## Mesh Management Frame

Existing 802.11 management frames always use the 2 address format (the third address used to be the BSSID in data frames of a Infrastructure BSS network). In a WLAN mesh, mesh management frames may need to go through multiple hops (i.e. intermediate MPs) from the Source MP to the Destination MP. Therefore, mesh management frames can use 3 or 4 address frame formats as described before in 6.1.2.  The multi-address management frame format can also help to handle multi-radio MP management information.

### Specific Management Frame Formats

#### Beacon Frame Format

- Modify the Beacon frame format in clause 7.2.3.1 of IEEE 802.11-1999 to add the following:

| Order | Information | Note |
| --- | --- | --- |
| TBD | Mesh Capability info | Used to indicate mesh capability, e.g. MCF mode of operation, mesh AP or mesh non-AP |
| TBD | Mesh IE indicator | One byte bitmap for fast processing to identify if there are additional mesh-specific IEs in the frame |
| TBD | Mesh ID | Used to identify a specific WLAN mesh network |
| TBD | Resource management info | Resource configuration, power-save configuration |
| TBD | QoS management info | Similar to 802.11e with the inclusion of queue sizing info and “color” queing information |
| TBD | Environment info | RF channel profile and mobility profile |
| TBD | Routing info | Routing related info |
| TBD | Authentication (optional) | The Neighbor Discovery will not process a Beacon’s information if the authentication field is present and the management frame does not pass the authentication check |
| TBD | mRSN | The multicast security field contains information for a per Group MAC address specific cipher used in the mesh. The Neighbor Discovery protocol will pass this information to the security process that will obtain the encryption keys for this mesh point |
| TBD | Power saving parameter | Modes of cross-layer operation |

#### Association Request Frame Format

- Modify the Probe Association Request frame format in clause 7.2.3.4 of IEEE 802.11-1999 to add the following:

| Order | Information | Note |
| --- | --- | --- |
| TBD | Mesh Capability info | Used to indicate mesh capability, e.g. MCF mode of operation, mesh AP or mesh non-AP |
| TBD | Mesh IE indicator | One byte bitmap for fast processing to identify if there are additional mesh-specific IEs in the frame |
| TBD | Mesh ID | Used to identify a specific WLAN mesh network |
| TBD | Resource management info | Resource configuration, power-save configuration |
| TBD | QoS management info | Similar to 802.11e with the inclusion of queue sizing info and “color” queing information |
| TBD | Environment info | RF channel profile and mobility profile |
| TBD | Routing info | Routing related info |
| TBD | Authentication (optional) | The Neighbor Discovery will not process a Beacon’s information if the authentication field is present and the management frame does not pass the authentication check |
| TBD | mRSN | The multicast security field contains information for a per Group MAC address specific cipher used in the mesh. The Neighbor Discovery protocol will pass this information to the security process that will obtain the encryption keys for this mesh point |
| TBD | Power saving parameter | Modes of cross-layer operation |

#### Association Response Frame Format

- Modify the Probe Association Response format in clause 7.2.3.5 of IEEE 802.11-1999 to add the following

| Order | Information | Note |
| --- | --- | --- |
| TBD | Mesh Capability info | Used to indicate mesh capability, e.g. MCF mode of operation, mesh AP or mesh non-AP |
| TBD | Mesh IE indicator | One byte bitmap for fast processing to identify if there are additional mesh-specific IEs in the frame |
| TBD | Mesh ID | Used to identify a specific WLAN mesh network |
| Tbd | Authentication (optional) | The Neighbor Discovery will not process a Beacon’s information if the authentication field is present and the management frame does not pass the authentication check |

#### Reassociation Request Frame Format

- Modify the Reassociation frame format in clause 7.2.3.6 of IEEE 802.11-1999 to add the following:

| Order | Information | Note |
| --- | --- | --- |
| TBD | Mesh Capability info | Used to indicate mesh capability, e.g. MCF mode of operation, mesh AP or mesh non-AP |
| TBD | Mesh IE indicator | One byte bitmap for fast processing to identify if there are additional mesh-specific IEs in the frame |
| TBD | Mesh ID | Used to identify a specific WLAN mesh network |
| TBD | Environment info | RF channel profile and mobility profile |
| TBD | Routing info | Routing related info |
| TBD | mRSN | The multicast security field contains information for a per Group MAC address specific cipher used in the mesh. The Neighbor Discovery protocol will pass this information to the security process that will obtain the encryption keys for this mesh point |
| TBD | Authentication (optional) | The Neighbor Discovery will not process a Beacon’s information if the authentication field is present and the management frame does not pass the authentication check |
| TBD | Power saving parameter | Modes of cross-layer operation |

#### Reassociation Response Frame Format

- Modify the Reassociation frame format in clause 7.2.3.7 of IEEE 802.11-1999 to add the following:

| Order | Information | Note |
| --- | --- | --- |
| TBD | Mesh Capability info | Used to indicate mesh capability, e.g. MCF mode of operation, mesh AP or mesh non-AP |
| TBD | Mesh IE indicator | One byte bitmap for fast processing to identify if there are additional mesh-specific IEs in the frame |
| TBD | Mesh ID | Used to identify a specific WLAN mesh network |
| TBD | Authentication (optional) | The Neighbor Discovery will not process a Beacon’s information if the authentication field is present and the management frame does not pass the authentication check |
| TBD | mRSN | The multicast security field contains information for a per Group MAC address specific cipher used in the mesh. The Neighbor Discovery protocol will pass this information to the security process that will obtain the encryption keys for this mesh point |

#### Probe Request Frame Format

- Modify the Probe Request frame format in clause 7.2.3.8 of IEEE 802.11-1999 to add the following:

| Order | Information | Note |
| --- | --- | --- |
| TBD | Mesh ID | Used to identify a specific WLAN mesh network |
| TBD | Requested Mesh IE indicator | One byte bitmap for additional mesh-specific IEs |
| TBD | Authentication (optional) | The Neighbor Discovery will not process a Beacon’s information if the authentication field is present and the management frame does not pass the authentication check |

#### Probe Response Frame Format

- Modify the Probe Response frame format in clause 7.2.3.9 of IEEE 802.11-1999 to add the following:

| Order | Information | Note |
| --- | --- | --- |
| TBD | Mesh Capability info | Used to indicate mesh capability, e.g. MCF mode of operation, mesh AP or mesh non-AP |
| TBD | Mesh IE indicator | One byte bitmap for fast processing to identify if there are additional mesh-specific IEs in the frame |
| TBD | Mesh ID | Used to identify a specific WLAN mesh network |
| TBD | Resource management info | Resource configuration, power-save configuration |
| TBD | QoS management info | Similar to 802.11e with the inclusion of queue sizing info and “color” queing information |
| TBD | Environment info | RF channel profile and mobility profile |
| TBD | Routing info | Routing related info |
| TBD | Authentication (optional) | The Neighbor Discovery will not process a Beacon’s information if the authentication field is present and the management frame does not pass the authentication check |
| TBD | mRSN | The multicast security field contains information for a per Group MAC address specific cipher used in the mesh. The Neighbor Discovery protocol will pass this information to the security process that will obtain the encryption keys for this mesh point |
| TBD | Power saving parameter | Modes of cross-layer operation |

#### Mesh Management Action Frames

The following Action frame formats are defined for the Mesh Management category. Each action frame starts with an Action ID field to identify the content of the frame.

- Table 6-10:	Mesh Management Action Frames

| Category | Actions |
| --- | --- |
| Mesh Management | Mesh Report |
|  | Mesh Channel Switch Notification |
|  | Mesh Measurement Request |
|  | Mesh Measurement Report |
|  | MP TPC Request |
|  | MP TPC Report |
|  | MP Neighbor Request |
|  | MP Neighbor Report |
|  | MP Routing Report |

##### Mesh Report Action Frame Format

| Order | Information | Note |
| --- | --- | --- |
| TBD | Mesh Capability info | Used to indicate mesh capability, e.g. MCF mode of operation, mesh AP or mesh non-AP |
| TBD | Mesh IE indicator | One byte bitmap for fast processing to identify if there are additional mesh-specific IEs in the frame |
| TBD | Mesh ID | Used to identify a specific WLAN mesh network |
| TBD | Resource management info | Resource configuration, power-save configuration |
| TBD | QoS management info | Similar to 802.11e with the inclusion of queue sizing info and “color” queing information |
| TBD | Environment info | RF channel profile and mobility profile |
| TBD | Routing info | Routing related info |
| TBD | Authentication (optional) | The Neighbor Discovery will not process a Beacon’s information if the authentication field is present and the management frame does not pass the authentication check |
| TBD | mRSN | The multicast security field contains information for a per Group MAC address specific cipher used in the mesh. The Neighbor Discovery protocol will pass this information to the security process that will obtain the encryption keys for this mesh point |
| TBD | Power saving parameter | Modes of cross-layer operation |

## Control Frames

Mesh control frames always use 2-address frame format hence they are always hop-to-hop transmitted. Note that MTXOP timing and channel information are included in the Mesh Control field of the MAC header.

| Frame type and subtype in Frame Control Field |  |  |  |
| --- | --- | --- | --- |
| Type value | Type description | Subtype values | Subtype description |
| 01 | Ctrl Frame | TBD | MTXOP Request |
| 01 | Ctrl Frame | TBD | MTXOP Response |
| 01 | Ctrl Frame | TBD | MTXOP Ack |
| 01 | Ctrl Frame | TBD | MTXOP Cancel |

Table 6-19:	Control Frames

## Data Frames

Mesh data frames always uses the 4-address frame format. In addition, if “Tunnel mode” is utilized, the Mesh Control Field contains 2 extra addresses to indicate the addresses of the ingress MP and the egress MP.

| Frame type and subtype in Frame Control Field |  |  |  | Mesh frame type in Mesh Control Field |  |
| --- | --- | --- | --- | --- | --- |
| Type value | Type description | Subtype (16 values) | Subtype description | Mesh type | Mesh type description |
| 11 | Mesh Frame | 1111 | Mesh data | 000000 | Mdata |
| 11 | Mesh Frame | 1111 | Mesh data | 000001 | Mdata + Tunnel |
| 11 | Mesh Frame | 1111 | Mesh data | 000010 | Mdata + MTXOP |
| 11 | Mesh Frame | 1111 | Mesh data | 000011 | Mdata + Tunnel+MTXOP |
| 11 | Mesh Frame | 1111 | Mesh data | 000100 | Mdata + ReMgt |
| 11 | Mesh Frame | 1111 | Mesh data | 000101 | Mdata + Tunnel+ReMgt |
| 11 | Mesh Frame | 1111 | Mesh data | 000110 | Mdata + Tunnel+MTXOP |
| 11 | Mesh Frame | 1111 | Mesh data | 000111 | Mdata + Tunnel+MTXOP+ReMgt |
| 11 | Mesh Frame | 1111 | Mesh data | 001000 | Mdata+ACK |
| 11 | Mesh Frame | 1111 | Mesh data | 001001 | Mdata + Tunnel+ACK |
| 11 | Mesh Frame | 1111 | Mesh data | 001010 | Mdata + MTXOP+ACK |
| 11 | Mesh Frame | 1111 | Mesh data | 001011 | Mdata + Tunnel+MTXOP+ACK |
| 11 | Mesh Frame | 1111 | Mesh data | 001100 | Mdata + ReMgt+ACK |
| 11 | Mesh Frame | 1111 | Mesh data | 001101 | Mdata + Tunnel+ReMgt+ACK |
| 11 | Mesh Frame | 1111 | Mesh data | 001110 | Mdata + Tunnel+MTXOP+ACK |
| 11 | Mesh Frame | 1111 | Mesh data | 001111 | Mdata + Tunnel+MTXOP+ ReMgt+ACK |

Table 6-23:	Data Frames

### Mdata Frame Format

- Figure 6-8: Mdata Frame Format

### Mdata+Tunnel Frame Format

- Figure 6-9: Mdata + Tunnel Frame Format

### Quality of Service Support

This proposal supports QoS features specified in IEEE 802.11e. This includes support for the user priority (UP) as indicated by the TID field of the QoS Control, mapping of UP to access categories, and the support of admission control and its related information elements such as TSPEC, TCLAS and SCHEDULE.

A discard precedence (DP) bit is added in the Mesh Type field. The DP bit identifies the discard precedence of the incoming frame. Two discard precedence values are indicated by the DP bit:

| DP bit Value | Usage |
| --- | --- |
| 0 | Low discard precedence |
| 1 | High discard precedence |

Important frames that are crucial for network operation or for application performance are tagged with a low discard precedence values. When network resources are not sufficient due to congestion frames with high DP values will be discarded first before those with low discard precedence values. The discard mechanism is implementation dependent.

Efficient operation of a mesh networks requires the use of piggybacking of control and management information on data frames. Since this information varies in its importance and priority coordination is needed to ensure that this information are piggybacked on data frames of at least similar transmission and discard priorities.

## Information Elements

[Need to add IEs for Periodic Mode: MTXOP Ownership IE, MTXOP Availability IE, Beacon Period Occupancy IE.]

Each information element (IE) begins with an Element ID and a Length field.  Mesh-specific IEs will be assigned to previously reserved Element ID values as shown in Table [TBD].  Non-mesh devices will ignore these unknown Element IDs and skip over them based on the value represented in the Length field.

- Table [TBD] – Element IDs

| Information Element | Element ID |
| --- | --- |
| Mesh ID | TBD |
| Mesh Capability | TBD |
| Mesh IE Indicator | TBD |
| … | TBD |

### Mesh ID element

The Mesh ID element carries the identity of the IEEE 802.11s WLAN Mesh Network.  It uses the same format as the existing 802.11 SSID.  The Mesh ID element is included by MPs in Beacon, Probe Request, Probe Response, and Association Request frames.

The format of the Mesh ID element is shown in Figure [TBD].

| Octets: 1 | 1 | 0 - 32 |
| --- | --- | --- |
| Element ID | Length | Mesh ID |

Figure TBD: Mesh ID element format

The Mesh ID element is identified by the element ID [TBD].  The length of the Mesh ID may be up to 32 bytes.  The choice of the value of the Mesh ID is entirely up to the user/network administrator.  Note that a Mesh ID element of length zero may be used in Probe Request frames when an MP is attempting to discover all 802.11s mesh networks in its vicinity.

### MP Capability element

The presence of the Mesh Capability element in a MAC frame indicates that the device is a Mesh Point (MP) and supports all mandatory features of this 802.11s specification.  The Mesh Capability element is included by MPs in Beacon, Probe Response, Association Request, and Association Response frames.

The format of the Mesh Capability IE is shown in Figure [TBD].

|  | Element ID | Length | Optional Capabilities Supported |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | TBD |

Figure TBD. Mesh Capability element format

The Mesh Capability element is identified by the element ID [TBD].  The length of the IE is [TBD].  Optional capabilities are represented by a bitmap.

An MP will set the corresponding bits in the bitmap to indicate which optional capabilities are supported.

### Mesh IE Indicator element

The Mesh IE Indicator identifies the category of mesh-specific IEs, if any, that are included within the current frame.  The Mesh IE Indicator is only significant in Beacon and Probe Response frames.  It may also be used in Probe Request frames to solicit mesh-specific information in a Probe Response.

- The format of the Mesh IE Indicator element is shown below in Figure [TBD].

| Bits: 8 | 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | Routing Parameter Set | Security Parameter Set | QoS Parameter Set | Resource Parameter Set | Environment Parameter Set | Reserved | Reserved | Reserved |

Figure [TBD]: Mesh IE Indicator element format

The Mesh IE Indicator element is identified by the element ID [TBD].  The Length is 1-byte and represents a bitmap as follows:

- An MP will set the Routing Parameter Set subfield to 1 to identify the inclusion of the Routing IEs within the frame.

- An MP will set the Security Parameter Set subfield to 1 to identify the inclusion of the mRSN IE and/or Authentication IE within the frame.

- An MP will set the QoS Parameter Set subfield to 1 to identify the inclusion of queue sizing IE and “color” queuing IE within the frame.

- An MP will set the Resource Parameter Set subfield to 1 to identify the inclusion of the Resource Configuration IE, Power-saving Configuration IE, Positioning Capability IE, Direct Portal Connection IE, and/or the Direct AS Connection IE within the frame.

- An MP will set the Environment Parameter Set subfield to 1 to identify the inclusion of the RF Profile IE and/or the Mobility Profile IE within the frame.

### Multicast RSN (mRSN) element

The multicast security field contains information for a per Group MAC address specific cipher used in the mesh. The Neighbor Discovery protocol will pass this information to the security process that will obtain the encryption keys for this mesh point.

### Authentication element

The Neighbor Discovery will not process a Beacon’s information if the authentication field is present and the management frame does not pass the authentication check.

### QoS Parameter Set element

| Order | Information | Note |
| --- | --- | --- |
| TBD | Queue sizing as defined below | Queue sizing information |
| TBD | Color and bandwidth of the node | Color and bandwidth reserved (5.3.6.2) |

      - Queue Sizing element

| Order | Information | Note |
| --- | --- | --- |
| TBD | Number of 802.11e Queues supported | Value of the number of queues |
| TBD | Queue size | Size of Queues in Bits |
| TBD | 802.11E/US Priority mapping flags | 802.11 supports the QoS based on User Priority of 8 queues |
| TBD | User Priorities Mapping flags: used in 802.11E specified in 802.1D User priority | Flag value 01 = User priority mappings based on 802.11E specified in 802.1D (see mapping below) 02 – User priorites are done via a T-Spec into user categories. User Priorites 8-15 + T-Spec are passed in priority list 04 – Color categories are used for User priorities to map access categories (4-8 categories) with table 2 below. 802.11 supports the QoS |
| TBD | # of Access categories 802.1D/802.11e User priorities are mapped to | If 4, the 802 mapping below in table X. If different than 4, a mapping must be specified below. |
| TBD | Maximum number of TSPECs supported for mapping into User categories | Priorities 8-15 are check + a T-SPEC will be passed with priority set in T-SPEC/priority list table. If no T-SPEC, then the priority will be zero |
| TBD | Traffic Buffered on Power save by Qos quality |  |

| Priority | User Priority (UP same as 802.1D) | 802.1D Designation | Access Category (AC) | Designation (informative) |
| --- | --- | --- | --- | --- |
| Lowest | 0 | BK | AC_BK | Background |
|  | 1 | - | AC_BK | Background |
|  | 2 | BE | AC_BE | Best Effort |
|  | 3 | EE | AC_BE | Best Effort |
|  | 4 | CL | AC_VI | Video |
|  | 5 | VI | AC_VI | Video |
|  | 6 | VO | AC_VO | Voice |
| Highest | 7 | NC | AC_VO | Voice |

      - Color Queueing element

| TBD | Color flags | Flag byte for levels of color traffic that Mesh point supports: NL00 YBSG – N – node, L – link or links Yellow(Y) bronze(B), silver(S), gold (G) |
| --- | --- | --- |
| TBD | Bandwidth on link reserved for Gold |  |
| TBD | Bandwidth on link reserved for Silver |  |
| TBD | Bandwidth on link reserved for Bronze |  |
| TBD | Yellow bandwidth reserved (savenger) |  |
| TBD | Total bandwidth reserved for node | Total bandwidth reserved on link or node |

### Resource Configuration element

- Channel configuration

- Operating channels

- Bands supported

- Radio configuration

- Number of radios

- Transmit power per radio

- Utilization and effective throughput per radio

- Antenna configuration

- Scheduling (resource allocation) mode configuration

### Power Saving Configuration element

The Power Saving Configuration element provides informaton about this node’s power saving status, or needs.  (For example, a mesh point with low battery power may be suggested to go-offline for a while if the rest of the mesh can forward traffic without it.)

- Remaining battery power level percentage

- Min. required battery power level percentage

### RF Profile element

The RF Profile element provides the RF profile (interference, signal strength, SNR) on a per channel basis.

### Mobility Profile element

The Mobility Profile element indicates the level of MP mobility (no mobility, low, medium, high).

### Mesh Measurement Capability element

The Mesh Measurement Capability element specifies the optional measurement capabilities supported by the MP.  The format of the Measurement Capability element is shown in Figure [TBD].

| Octets: 1 | 1 | 1 |
| --- | --- | --- |
| Element ID | Length | TBD |

Figure [TBD]: Mesh Measurement Capability element format

Note:  Details to be provided based on status of 802.11k.

### Mesh Channel Switch Announcement element

The Mesh Channel Switch Announcement element is used by an MP to advertise when it is changing to a new common mesh frequency channel.  The format of the Mesh Channel Switch Announcement element is shown in Figure [TBD].

| Octets: 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- |
| Element ID | Length | Reserved | New Channel Number | Channel Switch Count |

Figure TBD: Mesh Channel Switch Announcment element format

The Length field is [TBD].  The New Channel Number field is set to the number of the new common mesh frequency channel to which the MP is moving.  The Channel Switch Count field will be set to the number of target beacon transmission times (TBTTs) until the MP sending the Mesh Channel Switch Announcement element switches to the new channel or it will be set to 0.  A value of 1 indicates that the switch will occur immediately before the next TBTT.  A value of 0 indicates that the switch will occur at any time after the frame containing the element is transmitted.

The Mesh Channel Switch Announcenment element is included in Action frames of category Mesh Management and action value Mesh Channel Switch Announcement, and may also be included in Beacon and Probe Response frames.

### MP Measurement Request element

The MP Measurement Request element contains a request for the receiving MP to undertake the specified measurement action and is based on 802.11k. The information element is defined in Figure TBD.

| Octets: 1 | 1 | 2 | 1 | 1 | 1-6 | variable |
| --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | Measurement Token | Measurement Request Mode | Measurement Type | MLID | Measurement Request |

Figure TBD: MP Measurement Report element format

The Element ID field is set to [TBD]. The Length field is set to number of bytes to follow. The Measurement Token field is used to associate a subsequent report to this request. The Measurement Request Mode field indicates if the requested measurement can occur in parallel with ongoing measurements and whether the request is for  “one-shot”, “event-triggered”, or “periodic” measurement reporting. The Measurement Type field identifies type of measurement requested. The MP Measurement Request Types are:

- Basic Request

- CCA Request

- RPI Histogram Request

- Channel Load Request

- Noise Histogram Request

- Beacon Request

- Frame Request

- Hidden Station Request

- Medium Sensing Time Histogram Request

- Statistics Request

- Location Configuration Indication Request

- Buffer Occupancy Request

- Battery Level

- Preferred Channel Set

The MLID identifies the ML upon which the measurement should be performed. Finally, the Measurement Request field provides detailed information for set-up, execution, and reporting of the requested Measurement Type.

### MP Measurement Report element

The MP Measurement Report element provides a measurement report corresponding to a previous measurement request. The information element is defined in Figure [TBD].

| Octets: 1 | 1 | 2 | 1 | 1 | 1-6 | variable |
| --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | Measurement Token | Measurement Report Mode | Measurement Type | MLID | Measurement Report |

Figure [TBD]: MP Measurement Report element format

The Element ID field is set to “MP Measurement Report.” The Length field is set to number of bytes to follow. The Measurement Token field is used to relate the report to the associated request. The Measurement Report Mode field indicates if the MP was able or unable to perform the requested measurement. The Measurement Type field identifies type of measurement reported. The MP Measurement Report Types are:

- Basic Report

- CCA Report

- RPI Histogram Report

- Channel Load Report

- Noise Histogram Report

- Beacon Report

- Frame Report

- Hidden Station Report

- Medium Sensing Time Histogram Report

- Statistics Report

- Location Configuration Indication Report

- Buffer Occupancy Report

- Battery Level

- Preferred Channel Set

The MLID identifies the ML upon which the measurement was performed. Finally, the Measurement Report field provides detailed information for the reported Measurement Type.

### MP TPC Request element

The MP TPC Request element contains the required information for the receiving MP to undertake the specified MP transmit power and link margin action. The information element is defined in Figure [TBD].

| Octets: 1 | 1 | 1-6 |
| --- | --- | --- |
| Element ID | Length | MLID |

Figure [TBD]: MP TPC Request element format

The Element ID field is set to “MP TPC Request.” The Length field is set to number of bytes to follow. The MLID identifies the ML upon which the action should be taken.

### MP TPC Report element

The MP TPC Report element provides a report according to the associated request. This report can be included in MBeacon or MProbe Response without a corresponding request. The information element is defined in Figure [TBD].

| Octets: 1 | 1 | 1-6 | 1 | 1 |
| --- | --- | --- | --- | --- |
| Element ID | Length | MLID | Current MP Transmit Power | MP Link Margin / Inter-MP Pathloss (optional) |

Figure [TBD]: MP TPC Report element format

The Element ID field is set to “MP TPC Report.” The Length field is set to number of bytes to follow. The MLID identifies the ML upon which the action has been taken. The Current MP Transmit Power identifies the power that was used to transmit this MP TPC Report. Finally, the MP Link Margin / Inter-MP Pathloss provides the link margin / pathloss measured on the associated request, and therefore this field is not included when the MP TPC Report element is sent in a Beacon or Probe Response

### MP Neighbor Request element

The MP Neighbor Request element contains a request for the list of neighbor MPs. The information element is defined in Figure [TBD].

| Octets: 1 | 1 |
| --- | --- |
| Element ID | Length (set to “0”) |

- Figure [TBD]: MP Neighbor Request element format

The Element ID field is set to “MP Neighbor Request.” The Length field is set to zero.

### MP Neighbor Report element

The MP Neighbor Report element provides the list of neighbor MPs. The information element is defined in Figure [TBD].

| Octets: 1 | 1 | variable |
| --- | --- | --- |
| Element ID | Length | MP Neighbor List |

- Figure [TBD]: MP Neighbor element format

The Element ID field is set to “MP Neighbor Report.” The Length field is set to number of bytes to follow. The MP Neighbor List provides a list of all MPs currently associated to this MP.

### MP Channel Report element

The MP Channel Report element contains a list of channels where an MP is likely to find another MP. The information element is defined in Figure [TBD].

| Octets: 1 | 1 | 1 | variable |
| --- | --- | --- | --- |
| Element ID | Length | Regulatory Class – Frequency Band | MP Channel List |

- Figure [TBD]: MP Channel Report element format

The Element ID field is set to “MP Channel Report.” The Length field is set to number of bytes to follow. The Regulatory Class – Frequency Band field contains the identification of the band and regulatory domain. The MP Channel List contains the list of active channels being used by the current WLAN Mesh system.

### MP RCPI Report element

The MP RCPI Report element may be returned in an MProbe Response if requested in an MProbe Request. It contains the MP Received Carrier Power Indication (RCPI) value measured on the received MProbe Request frame. The information element is defined in Figure [TBD].

| Octets: 1 | 1 | 1 |
| --- | --- | --- |
| Element ID | Length (set to “1”) | MP RCPI |

- Figure [TBD]: MP RCPI Report element format

The Element ID field is set to “MP RCPI Report.” The Length field is set to “1.” The MP RCPI field provides the RCPI value as defined in the RCPI measurement at the measuring MP, according to the clause for the PHY type.

### Routing element

The routing function only requires that the Hello protocol be implemented.  The Hello protocol is a Neighbor Greeting protocol. The Routing element has the following information elements that must be recognized for the required Hello protocol.  These fields are described in the subsequent subsections.

| Indicator value | Field | Description of Field |
| --- | --- | --- |
| TBD | Sequence number (required) | Sequence number of the routing IEunction. This sequence number allows backward compatibility for mesh frames that do not have the Mesh Frame (type 11) Mesh Control word. |
| TBD | Routing protocol IE (required) | The routing protocol IE contains the routing protocol, routing protocol version, routing packet type, and the routing metric for this link to the neighbor. Note: The routing metric is the default routing metric and does not preclude the node from sending any other metrics in additional fields. |
| TBD | Routing capability IE (optional) | The routing capability is a bit mask of the additional information fields in the rest of the routing packet. |
| TBD | Forwarding IE (optional) | The flags indicating what type of forwarding this mesh point will do: Unicast, Multicast, and power aware. |
| TBD | Neighbor IE (optional) | This field contains information about the mesh point neighbors of the mesh point. The information carried is the length of the field, the total count of neighbor, and a sequence of neighbor information. Each piece of neighbor information has: the Neighbor MPID, the last sequence number of the hello sent, the default routing metric, and the local link (identified by MLID) attached to this neighbor. |
| TBD | Portal Distance IE (optional) | This field contains information about the distance to reachable portals |
| TBD | Resource Engineering IE (optional) | This field gives a resource aware metrics for this link. |
| TBD | Hello timer IE (optional) | The hello timer information field contains information about the time each hello will be sent (hello interval) and the time at which a neighbor will consider the neighbor to be dead (the router dead interval) |
| TBD | Unicast MAC IE (optional) | This field contains a list of Unicast MAC addresses associated with the mesh point. The stations are not a part of the Mesh Point, but reachable via the mesh point. |
| TBD | Multicast MAC IE (optional) | This field contains a list of Multicast MAC addresses associated with the mesh point . |

#### Routing Protocol Information Element

| Information | Length | Note |
| --- | --- | --- |
| Control protocol | 4 bits | The routing control protocol. This proposal uses two value: 0000 – Hello only 0001 – Hybrid Link State 0002 – On Demand Pro-Active ( Values 3-31 are for expansion of routing protocol values. |
| Version | 4 bits | Revision of the protocol |
| Routing Packet type | 1 byte | The type of routing packet. All protocols share the “hello” packet – packet 0000 as a common packet. Values 1-15 – are determined by the protocol. Please refer to the sections on routing protocols. |
| Packet specific information | Variable | Protocol packet specific information depending on the Protocol and the packet type. All protocols will support the hello protocol. The hello protocol uses protocol 0 and the packet type 0. |

The variable portion of the Routing Protocol element has the following format for the “hello” packet which is defined below.  The Routing protocol ie field definitions for the Hybrid Link state Protocol (H-LS) and the Hybrid On-Demanind Pro-Active (H-ODPA) are contained in these sections.  This specification only requires that the “hello” sub-function be used.

##### Format for the Required Hello Sub-Protocol

| Information | Length | Note |
| --- | --- | --- |
| Control protocol | 4 bits | The routing control protocol. This proposal uses two value: 0000 – link state, 0001 – for on-demand. Values 2-5 are for expansion |
| Version | 4 bits | Revision of the protocol |
| Routing Packet type | 1 byte | The type of routing packet. All protocols share the “hello” packet – packet 0000 as a common packet. Values 1-15 – are determined by the protocol. Please refer to the sections on routing protocols. |
| Routing metric field | 3 bytes | Routing metric that the mesh nodes uses over the routing link. The field is three bytes long: Metrics type flag , Resource aware metric, and topology aware metric. |

- The routing metric field field has the following format.

| Information |  | Length |  | Note |  |
| --- | --- | --- | --- | --- | --- |
| Metrics type | 1 byte |  | Metrics type flag. This metric field indicates the precedence between the resource aware and the topology aware metric. These two metricds are combined to provide the default metrics. The metric that is on the top is the highest priority. The values defined are: 0000 – resource aware metric (top), topology (bottom) 0001 – resource aware metric (bottom), topology (top) |  |  |
| Resource Aware metric |  | 1 byte |  | Metric to indicate resource aware value for this link This does not indicate any specific resource but is a summation of all resources |  |
| Topology aware metric |  | 1 bytge |  | Metric to indicate topology aware metric on this link |  |

#### Routing Capability element

The Routing Capability element contains the following information.

| Information | Length | Note |
| --- | --- | --- |
| Capablities | 2 bytes | The capability values for byte 1 are: XXMU HRNF F – forwarding IE exists in this frame N – Neighbor ie exists in this frame R - Resource aware metrics field exists in this frame H – Hello timers exists in this frame U – Unicast MAC ie field exists in this frame M – Multicast MAC ie field exists in this frame X - is reserved at this point and zero Byte 2 is zero at this point |

#### Forwarding Capability element

| Information | Length | Note |
| --- | --- | --- |
| Forwarding flags | 1 byte | The forwarding flags sare a combination of forwarding bits that indicate what type of forwarding this mesh point supports. 0000 0MFP P – Power saving forwarding is supported. F – Unicast forwarding is supported M – Multicast forwarding is supported During Power saving mode, the node may turn off to save power and return later. |

#### Neighbor Information element

The Neighbor Information element contains the following information

| Information | Length | Note |
| --- | --- | --- |
| Mesh neighbor ID | 6 bytes | MPID of the neighbor |
| Last Sequence # of hello information | 2 bytes | Last sequence number of the routing ie that t sent by this neighbor (of the mesh neighbor). This field provides a “freshness” date |
| Routing metric field | 3 bytes | Routing metric field from the Neighbor’s hello. |
| MLID of the link that this neighbor is connected across | 6 bytes | MLID of the link of the Beacon transmitter that this neighbor is connected across. |

| Information | Length | Note |
| --- | --- | --- |
| Total length | 2 bytes | Total length of the Neighbor ie field. |
| Count of Neighbors | 1 byte | Count of neighbors |
| Neighbor fields | 16 bytes | Sequence of neighbor fields containing the Neighbor information. |

An example will help clarify the Neighbor fields. In figure 8-2 MP2 has a neighbor MP-3 and MP-1.

From MP-2 view point, the link it shares with MP-1 has MLID of 00-00-00-00-00-01. The link MP-2 shares with MP-3 has a MLID of 00-00-00-00-00-02.

MP-2 first brings up a Mesh-point Link (ML) with MP-1, it will send a Beacon with a hello indicating that it has MP-1 as a neighbor. After hearing the Beacon from MP-3, it will send to both MP-1 and MP-3 a Beacon with a Neighbor id containing: MP-1 and MP-3.

The neighbor fields in the first beacon sent by MP-2 prior to hearing MP-3 will be:

- Neighbor MP-1, routing metric, MLID (00-00-00-00-00-01)

- The neighbor fields sent in a beacon after it hears from MP-3 will be:

- Neighbor MP-1, routing metrics, MLID (00-00-00-00-00-01)

- Neighbor MP-3, routing metrics, MLID (00-00-00-00-00-02)

#### Portal Information element

The Portal Information element contains the following information:

| Information | Length | Note |
| --- | --- | --- |
| Type of portals | 1 bytes | Flag bytes for mesh portal ie R000 000AC – where A & C are mesh portals and R = a reserved field for expansion. |
| Authentication Server co-resident (optional) | 1 byte | The authentication server co-resident field indicates the type of authentication servers associated with the portals. The values are: R000 0PUB where B – Basic forwarding server, M – On-Demand P = PKI infrastructure |
| Count of Mesh Portals | 1 bytes | Count of mesh Portal ie following |
| Mesh Portal ie |  | The mesh portal IE provides information about mesh portals. This information includes distance as well as forwarding support. |

##### Mesh Portal IE

The Mesh Portal contains the following information:

| Information | Length | Note |
| --- | --- | --- |
| Indicator field | 1 | Value: Tbd |
| Distance | 3 bytes | Metric value (type, resource-aware, distance) |
| Mesh Portal Mesh Point (optional) | 6 bytes | Mesh Point that has associated Mesh Portal |
| Mesh Portal MAC address | 6 bytes | Mesh portal MAC Address (uniquely identifies) |
| Mesh Portal IPv4 addrerss (optional) | 4 bytes | IPv4 address of Mesh Portal |
| Mesh Portal IPv6 (optional) | 20 bytes | IPv6 address of Mesh Portal |

#### Resource Aware Metrics element

| Information | Length | Note |
| --- | --- | --- |
| Count of Resource aware metric filds | 1 byte | Total count of resources aware metrics. The total length of the resource aware ie = 1 + count*8 |
| Resource Aware field | 8 bytes | Resource aware metrics |

Resource aware field contains the following fields

| Information | Length | Note |
| --- | --- | --- |
| Constraint map | 4 bytes | The constaint map contains either a bit map or a value for the constraints. The high bit of the 4th byte contains a flag that indicates whether the value. If this bit is 1, the constraint value contains a bit map, the metrics are shared between all resource aware constraints. If this bit value is 0, the constraint is a particular value identifying the constraint. |
| Node weight | 2 bytes | The weight of the node for constraint calculations |
| Link weight | 2 bytes | The weight of hte link for constraint calculations. |

#### Hello Timer element

The Hello Timer element indicates the time period for hello information that will be sent in the Beacon message. The neighbor discovery protocol will set mesh link (ML) and mesh link down based on these timers.

| Information | Length | Note |
| --- | --- | --- |
| Hello interval | 4 bytes | Interval that hello is sent at. (milliseconds) |
| Router Dead interval | 4 bytes | Router Dead interval. This is the time period at which the neighbor discovery and router functions dead if no beacon with hello information has been heard on a remote link. |

#### Unicast MAC Address element

The Unicast MAC Address element passes the MAC address that this mesh point will route to. These station addresses are not mesh points in the wireless mesh.

| Information | Length | Note |
| --- | --- | --- |
| Count of Unicast MACs | 1 byte | Count of Unicast MAC station addresses that follow |
| Unicast MAC address | 6 byte | Unicast MAC address |

#### Multicast MAC Address element

The Multicsat MAC Address element passes the MAC address that this mesh point will route to. These station addresses are not mesh points in the wireless mesh.

| Information | Length | Note |
| --- | --- | --- |
| Count of Multicast MAC fields | 1 byte | Count of Unicast MAC station addresses that follow |
| Multicast MAC field | 7 bytes | Multicast MAC (6 bytes), followed by 1 byte of flags. |

The Multicast MAC field contains the following bytes

| Information | Length | Note |
| --- | --- | --- |
| Mutlicast MAC address | 6 bytes | Multicast MAC address |
| Flag for Multicast MAC | 1 byte | Flags: PSXX XXXD The flags are: P – Multicast MAC is requesting a per MAC encryption key, but the key is not valid at this point. S – Multicast MAC has requested and received a MAC encryption key. D – MAC is being deleted from this node. Note that a “zero” in this field indicates that the MAC is being added.) X – is a reserved field. |

The Multicast MAC field contains the following bytes

| Information | Length | Note |
| --- | --- | --- |
| Mutlicast MAC address | 6 bytes | Multicast MAC address |
| Flag for Multicast MAC | 1 byte | Flags: PSXX XXXD The flags are: P – Multicast MAC is requesting a per MAC encryption key, but the key is not valid at this point. S – Multicast MAC has requested and received a MAC encryption key. D – MAC is being deleted from this node. Note that a “zero” in this field indicates that the MAC is being added.) X – is a reserved field. |

#### Network Components Unicast MAC Address element

The Network Components Unicast MAC Address element provides a compressed format for Unicast MAC transmission.

The Unicast MAC addresses pass the MAC address that this mesh point will route to. These station addresses are not mesh points in the wireless mesh.  These network components will be passed in this compressed network components structure.    The second transmission of this group of MAC addresses does not re-transmit the MAC address, but only re-transmits the sequence of component IDs.

| Information | Length | Note |
| --- | --- | --- |
| Count of Components | 1 byte | Count of Component IDs. |
| Component ID | 2 bytes | Component ID indicating group of Unicast MACs (group of MACs, version) The Top bit of the component ID indicates if count and MAC address follows this component. |
| Count of Unicast MACs | 1 byte | Count of Unicast MAC station addresses that follow |
| Unicast MAC address-1 | 6 byte | Unicast MAC address |
| Unicast MAC address-n | 6 bytes | Unicast MAC Address |

| Information | Length | Note |
| --- | --- | --- |
| Component ID | 2 bytes | Component ID indicating group of Unicast MACs (group of MACs, version) |
| Count of Unicast MACs | 1 byte | Count of Unicast MAC station addresses that follow |
| Unicast MAC address-1 | 6 byte | Unicast MAC address |
| Unicast MAC address-n | 6 bytes | Unicast MAC address |

###### Subsequent transmissions of the Network Components Unicast MAC will trans

#### Network Component Multicast MAC

The Multicast MAC addresses pass the MAC address that this mesh point will route to. These station addresses are not mesh points in the wireless mesh.

- Again, the first transmission of the network component MAC addresses utilize the

| Information | Length | Note |
| --- | --- | --- |
| Count of components | 1 byte | Count of components in the IE |
| Component ID | 2 bytes | Component ID indicating Group of Multicast MACs (group, version) The top bit of the ID indicates whether this component has the MAC Addresses associated with it. |
| Count of Multicast MAC fields | 1 byte | Count of Unicast MAC station addresses that follow |
| Multicast MAC field | 7 bytes | Multicast MAC (6 bytes), followed by 1 byte of flags. |

The Multicast MAC field contains the following bytes

| Information | Length | Note |
| --- | --- | --- |
| Mutlicast MAC address | 6 bytes | Multicast MAC address |
| Flag for Multicast MAC | 1 byte | Flags: PSXX XXXD The flags are: P – Multicast MAC is requesting a per MAC encryption key, but the key is not valid at this point. S – Multicast MAC has requested and received a MAC encryption key. D – MAC is being deleted from this node. Note that a “zero” in this field indicates that the MAC is being added.) X – is a reserved field. |

The Unicast MAC addresses pass the MAC address that this mesh point will route to. These station addresses are not mesh points in the wireless mesh.

| Information | Length | Note |
| --- | --- | --- |
| Component ID | 2 bytes | Component ID indicating group of network component id (group of components 1, version x) |
| Count of Unicast MAC network components | 1 byte | Count of unicast network components |
| Unicast MAC network component(s) | 2 bytes | A list of Unicast MAC network component ids. |
| Count of Multicast MAC network components | 1 byte | Count of multicast network components |
| Multicat MAC network component(s) | 2 bytes | A list of Multicast MAC network component ids. |

## Link State Informational Elements [optional]

Hybrid Link state messages are carried in 802.11 Mesh Action frames with a category of routing. The routing action frames carry a routing report. The routing report detail has the following bytes for Hybrid link state.

Contents inside the Routing IE are dependent on the protocol.  This first section shows the required action header.  All routing protocols (hello, hybrid-link state (H-LS), hybrid On-demand Pro-Active (H-ODPA) are required to implement the hello functions.  The hello protocol only implements the “hello” functions.  The hybrid link-state (H-LS) and the Hybrid ON-Demand Pro-Active (H-ODPA) support hello sub-functions to their base protocol.

### Action header

| IE indicator value | Type | Note |
| --- | --- | --- |
| TBD | Category: Routing Report | Routing information action |
| TBD | Action: Topology | Topology information |
| TBD | Routing IE | Identification of the routing information by sequence number. |

### Routing IE contents IE in the Action Frames Link state Hello packet

| IE indicator | Name | Description |
| --- | --- | --- |
| Sequence number | 2 bytes | Sequence number to identify the packet |
| TBD | Routing protocol IE | Routing protocol field for the Hello message contains: Routing-protocol indicator Control protocol (4 bits), Version (4 bits) Routing packet type (0000) – for Hello Routing metric type (type byte, resource metric, topology metric) |
| TBD | Routing capability IE (optional) | The routing capability is a bit mask of the additional information fields in the rest of the routing packet. |
| TBD | Forwarding IE (optional) | The flags indicating what type of forwarding this mesh point will do: Unicast, Multicast, and power aware. |
| TBD | Neighbor Report ie (optional) | This field contains information about the mesh point neighbors of the mesh point. The information carried is the length of the field, the total count of neighbor, and a sequence of neighbor information. Each piece of neighbor information has: the Neighbor MPID, the last sequence number of the hello sent, the default routing metric, and the local link (identified by MLID) attached to this neighbor. |
| TBD | Resource Engineering IE (optional) | This field gives a resource aware metrics for this link. |
| TBD | Portal Distance IE (optional) | This field contains information about the distance to reachable portals |
| TBD | Hello timer IE (optional) | The hello timer information field contains information about the time each hello will be sent (hello interval) and the time at which a neighbor will consider the neighbor to be dead (the router dead interval) |
| TBD | Unicast MAC IE (optional) | This field contains a list of Unicast MAC addresses associated with the mesh point. The stations are not a part of the Mesh Point, but reachable via the mesh point. |
| TBD | Multicast MAC IE (optional) | This field contains a list of Multicast MAC addresses associated with the mesh point . |

#### Routing Protocol IE format for Common Hello packet

| Information | Length | Note |
| --- | --- | --- |
| Control protocol | 4 bits | 0001-Link State Protocol Values 2-5 are for expansion to other |
| Version | 4 bits | Revision of the protocol |
| Routing Packet type | 1 byte | The type of routing packet. The routing packets required for link state are: 0 – Hello packet |
| Routing metric | 3 bytes | 3 types: routing type, resource-aware metric, and topology metric as defined in section 5.2.3. |

### Link State Database Request packet IE fields

The Data Base Request packet has the following Information Entity fields.  The optional fields for this DataBase Request ie are in italics.

| IE indicator | Name | Description |
| --- | --- | --- |
| TBD | Routing protocol IE | Routing protocol field for the Hello message contains: Routing-protocol indicator Control protocol (4 bits), Version (4 bits) Routing packet type (0000) – for Hello Routing metric type (type byte, resource metric, topology metric) |
| TBD | Routing capability IE | The routing capability is a bit mask of the additional information fields in the rest of the routing packet. |
| TBD | Sequence number IE | Field describing sequence numbers. This field includes: starting sequence number, ending sequence number, count of lifetime ie that follow. |
| TBD | Lifetime IE | Lifetime (aging values) for a list of LSAs Life time ie contains: life time value (for aging out of LSAs), a count of LSA sequence numbers, a list of LSA sequence numbers. |
| TBD | QoS IE | Qualtiy of Service information for this database |
| TBD | Resource IE | Resource aware information for this database |
| TBD | Security IE | Security levels for this LSA Database |
| TBD | Power IE | Power Aware IE for this database. |

#### Routing Protocol IE for Hybrid Link State Data Base Request

| Information | Length | Note |
| --- | --- | --- |
| Control protocol | 4 bits | The routing control protocol. This proposal uses two value: 0000 – link state, 0001 – for on-demand. The value will be set to Link state. |
| Version | 4 bits | Revision of the protocol |
| Routing Packet type | 1 byte | The type of routing packet. The routing packets required for link state are: 0 - Hello protocol 1 – DB update requests 2 – Link State Updates |
| Response/request flag | 1 byte | Request/Response flags (XXXF SPTR) Where: R – where a value of (1) = Request, 0 = Reply T – where 1 = Transmit, 0 – Response P – Partial Sequence number response S – Sequence Number list follows F – Reduced flooding will be used in databased updates |
| Last DB Request sequence number | 2 bytes | Sequence number of last DB request |
| Sequence number of this request | 2 bytes | Sequence number of this DB request |
| Database | 2 bytes | Topology Database If zero, the topology database is defined to be default (0) with routing metric type of (resource (high), topology (low)). Non-zero databases, may refer to QoS databases. |
| Requestor (MPID of requestor) | 6 bytes | MPID of the Mesh Point that’s requesting the Link State updates. |

#### Sequence Number IE

Where the sequence number ie field from the Database Request/Response contains:

| Information | Length | Note |
| --- | --- | --- |
| Starting LSA sequence number | 4 bytes | Initial sequence number in data base sequence number |
| Ending LSA sequence number | 4 bytes | Final sequence number in database sequence number |
| Count of lifetime ie | 1 byte | Count of life time ie fields |

#### Lifetime IE

- Where each life-time number ie field contains

| Information | Length | Note |
| --- | --- | --- |
| Lifetime metric | 4 bytes | Life time metric (for aging out of LSAs) |
| Count of LSA sequence numbers | 2 bytes | Count of LSA sequence numbers following |
| List of Sequence numbers | 4 * count | List of sequence numbers with this aging value. |

### Link State Announcement IEs in Action Frame

The Link State Announcement information is passed in the Action frames with the following IEs:

| IE indicator | Name | Description |
| --- | --- | --- |
| TBD | Routing IE | Routing |
| TBD | Routing protocol IE | Routing protocol field for the Lnk State Database update message contains: |
| TBD | LSA Database IE | The Link-State Announcment Database is described by this ie. The data in this ie contains information about the Database ID, routing metric type, the last request (identified by ID number), and the current count of LSAs in the LSA Data Base. |
| TBD | LSA IE | A single Link State Announcement information entity. This entity contains: MPID of the originating Mesh Point that announced the information, LSA sequence number that is associated with this Annoucnement, and 4 bytes of lifetime value |

- The minimal link state announcements for the default topology contain

- Routing protocol ie header (6 bytes)

- LSA Database ie (4 bytes)

- Sequence of LSA ie – (Each LSA  (14 bytes/mesh point + 16 bytes/neighbor link))

This byte count contains sequence number & lifetimes of 4 bytes. A typical full database for all 32 nodes with 2 neighbors per mesh point can be sent in 1472 bytes.

Changes to any mesh point connectivity may only take 46 bytes (with 2 neighbors in the final announcement). Full data base knowledge can be initialized to a node upon initialization. After that point, the changes updates (42 bytes) are minimal.  Resource aware calculations or added functionality (Unicast or Multicast MAC reachability) will increase the Database footprint in Mesh Points the additional functions.

#### Contents of the Routing Protocol ie for the Link State Data Base Announcement Packet

Contents inside the Routing IE for each MGT control frames contain a common header.  Note the length of the protocol ie field differs per packet type.

| Information | Length | Note |
| --- | --- | --- |
| Control protocol | 4 bits | The routing control protocol. This proposal uses two value: 0000 – link state, 0001 – for on-demand. Values 2-5 are for expansion to other |
| Version | 4 bits | Revision of the protocol |
| Routing Packet type | 1 byte | The type of routing packet. The routing packets required for link state are: 0 – Hello sub-protocol 1 – DB update 2 – Link state updates For the LSA, the value is 2. |
| Last DB request update | 2 bytes | Database update information: Last DB request sequence number (2 bytes), |
| Count of LSAs | 2 bytes | Count of LSA ie field that follow |

#### Contents of the LSA Database IE

Contents inside the LSA DB IE field are:

| Information | Length | Note |
| --- | --- | --- |
| Sequence number of Last request | 2 bytes | Seq number of packet (for backward compatibility) |
| Database identifier (optional) | 2 bytes | Topology Database If not present, the topology database is defined to be default (0) with routing metric type of 0000 (resource high, topology low). |
| Routing capaiblity (optional) | 3 bytes | Routing capabilities supported by this DB |

#### Contents of the LSA IE

Contents inside the LSA DB ie field are:

| Information | Length | Note |
| --- | --- | --- |
| Originating MP | 6 bytes | MPID of the originating MP |
| LSA sequence number | 4 bytes | Sequence number of this LSA |
| Lifetime | 4 bytes | Lifetime for this LSA |
| Neighbor link IE | 16 bytes | Connected neighbor information (neighbor, link conected across, routing metric) Defined in section below |
| Neighbor’s Neighbor IE | Variable | Information about Neighbor’s Neighbor learned from network discovery protocol (Defined in section below) Refer to section 5.3 |
| Unicast MAC IE | Variable | Unicast MAC reachable directly this Mesh Point (Wireless stations). The Unicast MAC field is the same as defined for the neighbor discovery protocol. Refer to section 5.3 for the details. |
| Multicast MAC IE | Variable | Mutlicast MAC reachable from fhis Mesh Point (wireless Stations) The Multicast MAC reachable field is the same as defined in the neighbor discvoery protocol. Refer to section 8.3 for the details. |
| Forwarding IE | 2 bytes | Forwarding flags for neighbor. This forwarding ie field is the same as defined in the neighbor Discovery protocol. Refer to section 8.3 for the details |
| Routing capablity IE (optional) | 3 bytes | Routing capabilities supported by this DB This routing capablity field is the same as described by the neighbor discovery protocol. Refer to section 8.3 for the details. Note: a flag in the 8.3 is the “reduced flooding” ie flag to detect the reduced flooding algorithm. |
| Mesh Portal id | Variable | The mesh portal distance information. |
| QoS IE | Variable | QoS metrics for this mesh point neighbor This QoS metrics field is the same as defined by the neighbor discovery protocol. Refer to section 8.3 for the details |
| Resource IE |  | Resource aware metric for this mesh point neighbour. This Resource aware metric field is the same as defined by the neighbor discovery protocol. |
| Power ie |  | Power aware metrics for this mesh point neighbour. The power aware metric field is the same as defined by the neighbor discovery protocol. |
| ACK IE | Variable | List of LSAs ACK by this mesh point. |
| Hello IE | 2 bytes | Last Hello sequence number sent by this mesh point on neighbor discovery protocol |
| Reduced flooding IE | 1 byte | Reducted flooding used. (If zero, not used from this neighbor). |

#### Neighbor Link IE

The neighbor link ie field contains:

| Information | Length | Note |
| --- | --- | --- |
| Mesh Point Neighbor | 6 bytes | MLID of Mesh Point Neighbor (of originating Mesh Point) |
| Mesh Link | 6 bytes | Mesh Link ID for Link Mesh Point neighbor connected across |
| Routing metric | 3 bytes | Routing metric field for link used by neighbor discovery. This field contains a type field, resource-aware metric, topology aware metric. The type field defines how the pathway calculation uses the resource-aware metric and topology aware metric. |

#### Neighbor’s Neighbor IE field

Where the Neighbor’s neighbor ie field contains:

| Information | Length | Note |
| --- | --- | --- |
| Length of neighbor’s neighbor IE | 2 bytes | Total length of the neighbor’s neighbor ie field. This length allows mesh points that do not support this ie to quickly skip the field without parsing interior bytes. |
| Count of neighbors | 1 byte | Count of Neighbor’s of this neighbour |
| Neighbor’s Neighbor IE | Variable | Neighbor informatoin learned from Neighbor discovery protocol . |

Where neighbor’s neighbor ie contains:

| Information | Length | Note |
| --- | --- | --- |
| Neighbor link ie | 10 bytes | Neighbor link information as defined in link above |
| Unicast MAC | Variable | Unicast MAC stations reachable via this mesh point (neighbor’s neighbor) learned from neighbor-discovery |
| Multicast | Variable | Mutlicast MAC stations reachable via this mesh point (neighbor’s neighbor) learned from neighbor discovery |
| Forwarding IE | 2 bytes | Forwarding characteristics of the remote neighbor learned in the neighbor discovery protocol |
| QoS IE |  | QoS metric information learned from the neighbor discovery protocol. |
| Resource IE |  | Resource aware metric learned from the neighbor discovery protocol |
| QoS IE |  | QoS metrics associated with this neighbor learned from the neighbor disccovery protocol. |
| Hello IE | 2 bytes | Last hello sequence # sent from this neighbor |

#### ACK IE

Where the ACK IE field contains:

| Information | Length | Note |
| --- | --- | --- |
| Length of Ack field | 1 byte | Length of ACK field |
| Starting ACK Sequence number | 4 bytes | Lowest sequence number of Range of ACKs received |
| Ending ACK Sequence number | 4 bytes | High sequence number of Range ACKs received |
| List of ACKs | Variable | Sequence numbers (4 bytes per sequence number) for Acknowledgements for LSAs |

## Action Frame for Hybrid On-Demand Pro-Active Routing Protocols

The Hybrid On-Demand Pro-Active Routing Protocol (H-ODV-PA) uses the Action Frame Routing Report to pass routing topology information. It supports the “Hello” sub-protocol function (required by all protocols). The following IEs are used for the Hybrid On-Demand Pro-Active Routing Protocol.

### RREQ Element

| Field | Description/Value |
| --- | --- |
| Element ID | TBD |
| Length | The element length |
| Flags | Routing flags |
| Hop Count | The number of hops from the source to the MP processing this message |
| Metric | The cumulative metric from the source to the MP processing this message |
| RREQ ID | The identification of the RREQ |
| Source Address | The MAC address of the source which originated the Route Request. |
| Source Sequence Number | The current sequence number to be used in the route entry pointing towards the source of the route request. |
| Source IP address | Included only if the flag for source IP address is set |
| Destination Address #1 | The MAC address of the destination for which a route is desired. |
| Destination Sequence Number #1 | The latest sequence number received in the past by the source for any route towards the destination. A value of 0 indicates that the source does not know the sequence number of the destination. |
| Additional destination addresses (if needed) | The additional number of destination address and destination sequence number pairs can be obtained from the element length |
| Additional destination sequence numbers (if needed) | The additional number of destination address and destination sequence number pairs can be obtained from the element length |

### RREP Element

| Field | Description/Value |
| --- | --- |
| Element ID | TBD |
| Length | The element length |
| Flags | Routing flags |
| Hop Count | The number of hops from the source to the destination. For multicast route requests this indicates the number of hops to the multicast tree member sending the RREP. |
| Metric | The cumulative metric from the destination to the MP processing this message |
| Destination Address | The MAC address of the destination for which the route is supplied. |
| Destination Sequence Number | The destination sequence number associated to the route. |
| Destination IP address | Included only if the flag for destination IP address is set |
| Source Address | The MAC address of the MP which originated the RREQ for which the route is supplied. |
| Lifetime | The time in milliseconds for which nodes receiving the RREP consider the route to be valid. |

### RANN Element

| Field | Description/Value |
| --- | --- |
| Element ID | TBD |
| Length | The element length |
| Flags | Routing flags |
| Hop Count | The number of hops from the destination to the MP processing this message |
| Metric | The cumulative metric from the destination to the MP processing this message |
| RANN ID | The identification of the RANN |
| Lifetime | The time in milliseconds for which nodes receiving the RANN consider the route to be valid. |
| Destination Address #1 | The MAC address of the destination for which a route is supplied. |
| Destination Sequence Number #1 | The current sequence number to be used in the route entry pointing towards the destination #1. |
| Destination IP address #1 | Included only if the flag for destination IP address #1 is set |
| Additional destination addresses (if needed) | The additional number of destination address and destination sequence number pairs can be obtained from the element length |
| Additional destination sequence numbers (if needed) | The additional number of destination address and destination sequence number pairs can be obtained from the element length |
| Additional destination IP addresses (if needed) | Included only if the flag for additional destination IP address is set |

### RERR Element

| Field | Description/Value |
| --- | --- |
| Element ID | TBD |
| Length | The element length |
| Unreachable Destination Address #1 | The MAC address of the destination that has become unreachable due to a link break. |
| Unreachable Destination Sequence Number #1 | The sequence number in the route table entry for the destination listed in the previous Unreachable Destination Address field. |
| Additional unreachable destination addresses (if needed) | The additional number of unreachable destination address and destination sequence number pairs can be obtained from the element length |
| Additional unreachable destination sequence numbers (if needed) | The additional number of unreachable destination address and destination sequence number pairs can be obtained from the element length |

## Applicability of Mesh-specific IEs to Frame Type

The following 802.11s IEs support the signaling of MP information for management and control of the WLAN mesh network:

| New IEs for 802.11s | Frame Types |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Management |  |  |  |  |  |  |  |
|  | Beacon | Probe Request | Probe Response | Association Request | Association Response | Reassociation Request | Reassociation Response | Disassociation |
| Mesh ID | M | O | M | M |  |  |  |  |
| Mesh Capability Info | M | ? | M | M |  |  |  |  |
| Mesh IE Indicator | O | O | O | - |  |  |  |  |
| Resource Mgmt Info | O | - | O | - |  |  |  |  |
| Environment Info | O | - | O | - |  |  |  |  |
| Routing Info | O | - | O | - |  |  |  |  |
| Authentication Info | O | - | O | - |  |  |  |  |
| Multicast RSN Info | O | - | O | - |  |  |  |  |
| Power Saving Parameter | O | - | O | - |  |  |  |  |
| TBD… |  |  |  |  |  |  |  |  |

| New IEs for 802.11s | Frame Types (cont’d.) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Management |  |  |  |  |  | Control |  |  | Data |
|  | Action : MM Mesh Report | Action : MM Mesh Channel Switch Notification | Action : MM Mesh Measurement Request/Report | Action : MM MP TPC Request/Report | Action : MM MP Neighbor Request/Report | Action : MM MP Routing Report | MTXOP Request* | MTXOP Response* | MTXOP Ack* | Mesh Data* |
| Mesh ID |  |  |  |  |  |  |  |  |  |  |
| Mesh Capability Info |  |  |  |  |  |  |  |  |  |  |
| Mesh IE Indicator |  |  |  |  |  |  |  |  |  |  |
| Resource Mgmt Info |  |  |  |  |  |  |  |  |  |  |
| Environment Info |  |  |  |  |  |  |  |  |  |  |
| Routing Info |  |  |  |  |  |  |  |  |  |  |
| Authentication Info |  |  |  |  |  |  |  |  |  |  |
| Multicast RSN Info |  |  |  |  |  |  |  |  |  |  |
| Power Saving Parameter |  |  |  |  |  |  |  |  |  |  |
| TBD… |  |  |  |  |  |  |  |  |  |  |

* - New for 802.11s mesh

- M = Mandatory for mesh operation

O = Optional for mesh operation

- = Not Applicable

## Frame Aggregation

Frame Aggregation helps to enhance efficiency of data transmissions, since multiple frames are combined to a larger frame. Frame Aggregation avoids redundant Interframe Spaces (IFS), which are needed for transceiver turnaround. If the 802.11 PHY layer used for the Mesh WLAN does not provide Frame Aggregation function to the 802.11s MAC, the following Frame Aggregation procedure can be performed.

In an MTXOP, the owner combines all data it plans to send during this MTXOP into one large MPDU, which is called a “train” due to its structure: In the MPDU, several data packets are aligned in a row, separated by internal information like CRCs, and send by the owner as one long packet. This is known as frame aggregation, which is proposed to and agreed on in Task Group 802.11n a. Frame aggregation as proposed in 802.11n can be further enhanced. As there may be multiple receivers in a single MTXOP, the long frame is divided into a header and several wagons. The header is send at the basic PHY mode and describes the following structure. The wagons may be addressed to the different receivers and may be sent at different PHY modes.

The owner of a MTXOP can use two different techniques to optimize the usage of the channel time:

- Frame aggregation: The owner may aggregate several MSDUs, fragments of MSDUs, control frames (like ACKs) and information elements into one single entity, separating them by CRCs. The aggregate structure is given in the frame header. Frame aggregation as presented here is an enhanced version of the procedures proposed to Task Group 802.11n.

- Multiple receiver MPDU: The owner may send data to several different receivers during one transmission. It may also change the PHY mode during the transmission. The sequence and the duration of the different PHY modes are indicated in a header which is send in the basic PHY mode. Therefore, every receiver can compute the start time of its reception.

Therefore, the transmitter may send one multi receiver, multi mode frame during the subsequent MTXOPs. This frame is called a “packet train”, as it consists of a header and one or more wagons. Furthermore, each wagon is composed of a wagon header and one or more payloads, separated by CRCs. This structure is presented in Figure [TBD].

- Figure 6-10: The general Structure of a packet train, constituting of a header and several wagons

The intended receivers (as given by the reservation negotiation) listen to the train header and determine if one of the upcoming wagons is addressed to their DEVID. If so, it can compute its start time using the information given in the header.

The content of the train header is defined in section 6.9.1.1, the construction of the wagons including the wagon header in section 6.9.1.2.

#### Train Header

The structure of the train header is shown in Error: Reference source not found:

- Figure 6-11: The structure of the train header

After the transceiver’s DEVID, the number of wagons in the immediately following train is announced (up to mMaxNoOfReceivers), followed by a description of each wagon in the train, which consists of the receiver’s DEVID, the length of each wagon in OFDM Symbols and the chosen PHY mode. The train header ends with a header check sequence.

In many scenarios it is possible that the receiver cannot decode every wagon successfully, especially if it is encoded in a fast PHY mode. Without loss of generality, an OFDM PHY layer is assumed in the following explanations.

##### OFDM PHY layer amendments (Informative)

An MP can always detect the transmission boundaries, and therefore stay synchronized to the transmitter. As the length is given in multiples of (OFDM) Symbols, it can compute the start point of its wagon and single it out in the appropriate moment

The maximum duration of a wagon is restricted by the length indicator of the train header, whose maximum value is 28 – 1. E.g., for the 802.11a OFMD PHY it results in a wagon length of 765 octets (if BPSK½ is used).

#### Wagon Format

Each wagon is consists of a wagon header and the data portion.

##### Wagon Header

The wagon header announces the payload of the wagon, which are either MSDUs, fragments of MSDUs or information elements that are intended to the receiver only. The header states the number of elements and the length in symbols for each payload element.

##### Data Frame Format

Data frames transport the payload from the associated mobile stations through the mesh network, either to another mobile station or to a Mesh Point acting as a gateway or portal to another network. Likewise payload is transported from the gateway or portal to mobile stations. Therefore, the data frame format has to be adapted to the requirements and the standard of the legacy 802.11 format.

Consequently many of the data frame’s field resemble the 802.11 fields: The frame control bits, the final destination/first source address, the sequence control and the frame check sequence. Because of this similar structure, the fields of a MPDU from an associated station, addressed to the mesh network, can be copied directly into the data frame.

This also implies that the sequence numbers and a previous fragmentation are not changed by any Mesh Point, which furthermore reduces the complexity, as a Mesh Point does not have to buffer fragments of a MPDU for a defragmentation before the relaying.

The structure of a data frame can be found in Figure [TBD].

- Figure 6-12: The structure of a data frame

# MAC Sublayer Functional Description

## Mesh Discovery and Association

In order for MPs to form/join a mesh network, they first attempt to collect relevant information from neighbor MPs.  The initial information is acquired via passive scanning for Beacon frames, or via active scanning using Probe Request and Probe Response frames.  The amount of information conveyed by these frames can vary based on the inclusion of optional information elements.

“Nominal” mesh discovery procedures forego the inclusion of these optional elements, e.g., for reduced Beacon overhead or for privacy/security issues.  In this case, additional topology information is exchanged after initial neighbor discovery, association, and authentication.  Additional topology information is subsequently provided on an as-needed basis in a secure manner.

Conversely, if optional elements are included in Beacon and Probe Response frames, an “expeditied” mesh discovery can be accomplished by taking advantage of this additional information. Both approaches are discussed in more detail below.

### Mesh Discovery

Nominal discovery identifies a neighbor MP and determines the following:

- Is the neighbor MP also a Mesh AP (MAP)

- Mesh scheduling (resource allocation) mode

- Power saving mode

Expedited (optional) discovery includes the ability to determine the following:

- Operating channels and radio configuration (communication)

- Routing information

- Neighbor’s neighbors (local topology)

- Routing protocol (supported/active)

- Radio-aware and topology-aware routing metric information

- Security policy

- Battery power level

- Interference profile (environment), signal to noise ratio (SNR), and signal strength measurements

- Loading (congestion) status

- Positioning capability and accuracy

The WLAN Mesh discovery process is compatible with the current 802.11 WLAN station discovery.  When an MP powers up, it perform passive or active scanning on the available channels, looking for beacons or asking for probe responses from neighboring mesh points. The channel dwell time and channel scanning pattern for are vendor specific.

Each mesh point can analyze the channels scanned and build a channel interference profile. The algorithm to choose the channel is vendor specific. A typical choice is to find the minimum interference channel among the neighbour MPs.  Once the channel is selected, the MP broadcasts a beacon periodically on a selected channel where the beacon interval is adjustable.

During the nominal discovery, the mesh beacon is similar to the BSS beacon; it has a short frame body with minimal optional information. During the expedited discovery, the mesh beacon has a longer frame body with additional information, and each operator can choose what additional information is to be included.

The nominal discovery identifies neighboring MPs. It also discovers the supported or preferred scheduling (resource allocation) mode of operation. It also discovers the supported or preferred power saving mode.

For an expedited discovery, the beacon and probe response can carry additional information elements, e.g. hello information element (IE) for routing or robust secure network (RSN) IE for security policy) such that ONE message can accomplish multiple discovery tasks. Expedited discovery can reduce the delay for session setup time and is more energy efficient. It is advantageous to a mobile mesh network for public safety or military usage. It is also advantageous to a mesh network for home electronics usage. However, the increase of frame size may create undesirable overhead if the occurrence rate of these messages is high. Typically, shorter frames should be used in the nominal discovery, but longer frames could be used for expedited discovery. Cross-layer optimization techniques can be used to achieve an optimal balance among QOS, battery power, change of the local topology, and the size of neighbor list.

The mesh beacon and mesh probe response have the same format. Both frames have a Mesh IE Indicator (length of one byte). The bits in the indicator are used to represent the existence of additional IEs. The actual IEs are contained within the mesh IEs set. The mesh probe request can request for extended probe response with additional IEs by setting the desired bits in the indicator.

Figure 7-13: Frame Format for Mesh Beacon/Mesh Probe Response

We also propose to have another management action frame: Mesh Report to supplement the mesh Beacon and mesh Probe Request/Response. The format of Mesh Report is the same as mesh Beacon. The Mesh Report can be sent out at any time, it can be unicast, and it can be secured. When the mesh report is unicast, it does not have to use the lowest PHY rate (as in the case for the beacon frame). The mesh report can be sent after security association has been established. As a result, it can be secured. Two applications for the mesh report are discussed here. The first is when a node needs to send topology information to its neighbors, this node can send a mesh report instead. The routing IEs are contained inside the mesh report. This gives the routing protocol an advantage in expedited discovery of topology changes. The second is the mesh report can be used for RF maintenance (e.g. it can help the time synchronization function).

As previously discussed, a bit in the Mesh IE Indicator represents if additional information is conveyed via the discovery process. Each operator has the choice of how to set the bits. If all bits are set to zero, the mesh Beacon or mesh Probe Response does not carry any additional information. Some of the bits can also be set to take advantage of using one message to discover multiple sets of information. The bits in the indicator represent the existence of the following information:

- Routing parameters set

- Security parameters set

- Quality of Service parameters set

- Resource management parameters set

- Environment parameters set

- Reserved

### Mesh Association

Once a mesh point collects a sufficient number of beacons and probe responses (from different neighbors), it will either associate with another mesh point or performs nothing (e.g. all neighbor MPs have securit concerns or do not support power saving).

The decision for which mesh point to associate with is based on the decision of mesh aware engine. The mesh aware engine considers the following factors:

- MP Admission control

- Battery power saving

- Power saving mode supported

- Battery power level

- Security concerns

- Stability of the link

The following diagrams illustrate the message exchange sequences for mesh point 2 (MP2) to join a mesh network for both nominal and expedited discovery scenarios.

The first illustrates a nominal passive scanning and discovery scenario where a mesh point sends out a mesh beacon on the best channel (e.g. the least interference channel) such that other mesh points can discover its existence.

The first illustrates a nominal passive scanning and discovery scenario where a mesh point sends out a mesh beacon on the best channel (e.g. the least interference channel) such that other mesh points can discover its existence.

Figure 7-14: Mesh Discovery Example- Nominal (Passive) Scenario

The second illustrates a nominal active scanning and discovery scenario where MP2 sends out mesh probe request on the best channel (e.g. the least interference channel) that its neighbour advertised. When MP1 or MP3 receives the mesh probe request, it responds a mesh probe response.

Figure 7-15: Mesh Discovery Example- Nominal (Active) Scenario

The third illustrates an expedited passive scanning and discovery scenario where each MP sends out an extended mesh beacon with a hello included in the Routing Parameter Set. Since the routing hello has been exchanged during the discovery phase, there is no need to exchange hello after the mesh association is completed.

Figure 7-16: Mesh Discovery Example- Expedited (Passive) Scenario

The fourth illustrates an expedited active scanning and discovery scenario where MP2 sends out mesh probe request on the selected channel with the routing bit set in the mesh IE indicator. When MP1 or MP3 receives the mesh probe request, it responds with an extended mesh probe response where the hello is included inside the Routing Parameter Set of  the probe response.

Figure 7-17: Mesh Discovery Example- Expedited (Active) Scenario

The fifth illustrates an expedited passive scanning and discovery scenario where a mesh point sends out a mesh beacon on the selected channel such that other mesh points can discover its existence. After the security association has been established with neighboring peers, the mesh report is exchanged between these MPs. The mesh report contains routing IE and is encrypted.

Figure 7-18: Mesh Discovery Example- Expedited (Passive) Scenario Using Mesh Report

## Mesh Medium Access Control and Coordination Function

In the Extended Distributed Control Access (EDCA) architecture proposed by 802.11e, each priority class of mobile node independently attempt to content the wireless medium after sensing the channel availability. This mechanism relies on the mobile nodes who performance the carrier sensing on the occupancy of the common traffic channel. As the number of nodes and traffic increases on the common channel, higher probability of contention over the wireless medium and more exponential back-off required for such random access of shared medium and the overall system throughput will be degraded. This degradation is especially more serious as the distance between the mobile nodes are increased.

In the case of the Hybridge Coordinated Control Access (HCCA) architecture also proposed by 802.11e MAC, it depends on a single point of coordination function to operate in a shared medium to schedule across the mobile’s transmit opportunity, the design of the pairwise independent dynamic mode scheduling requires only communication between a pair MPs to maintain the distributed schedule among themselves. By taking this approach, the additional complexity of computing schedules at the central coordinator will significantly be reduced in spite of the number of MLs and MPs are increased.

### Mesh Transmission Opportunity

The 802.11e amendment defines a TXOP as an interval of time when a particular station has the right to initiate frame exchange sequences into the wireless medium (WM). A TXOP is then defined by starting time and a maximum duration and it can either be obtained by an individual station contending for the channel or it can be assigned by the hybrid coordinator (HC).

In order to address the WLAN Mesh requirements, we extend the TXOP concept by defining a Mesh Transmission Opportunity (MTXOP). An MTXOP is an interval of time of a specific duration that has been agreed between neighboring MPs for the exchange of information. A MTXOP is defined by

- Timing information

- RF information

- QoS information

- Direction

- Ownership

Similarly to the TXOP, the Timing Information includes the starting time and the duration of the MTXOP.

The RF Information defines the radio and the number of channels used by the MPs to communicate during the MTXOP. It contains a Candidate Channels List for traffic channel the channels in the list of common channels between the 2 MPs that are the “free” channels with good quality from the sending device view point, and the Preferred Number of Radios.

The QoS information provides the QoS required by the MPs to exchange data.

A MTXOP can be either unidirectional or bidirectional. For unidirectional MTXOP, data are exchanged only from the source MP to the destination MP(s). When the MTXOP is bidirectional, the MPs exchange data in both directions.

To avoid collision between the MPs, the first MP to send data is identified during the MTXOP negociation. This MP is the owner of the MTXOP. In case of a unidirectional MTXOP, it is possible for the MP transmitter to have up to mMaxNoOfReceivers receiving Mesh Points (MPs) for the same MTXOP. This allows a transmitter to maximize the usage of an MTXOP. An MTXOP may therefore be multi-receiver, but it can have only one owner.

MTXOPs are assigned through the Distributed Reservation Controlled Access (DRCA) mechanism.

### Distributed Reservation Control Access (DRCA)

The Distributed Reservation Channel Access (DRCA) builds the Mesh Coordination Function (MCF). It works independently from the legacy 802.11 contention based access scheme. It allows MPs to negociate MTXOP with their neighborgs to permit them to exchange data. The MTXOP agreement is a 2 or 3-way handshake message exchange, as shown in Figure 7-19.

- The Source MP initiates the negotiation by sending a proposed MTXOP (MTXOP Req)

- The Destination MP can either agree, reject or propose another MTXOP (MTXOP Rsp)

- Optionally,  the Source MP can terminate the negotiation by sending an acknowledgment (MTXOP Ack), confirming the acceptance or denial of the negotiation.

The DRCA has a similar function as the 802.11 RTS/CTS procedure as it can also signal other nodes a certain occupation of the channel. However, it is more efficient because it permits negotiating several MTXOPs in one single handshake

Figure 7-19: Distributed Reservation Control Access (DRCA)

#### DRCA Procedure

The DRCA procedure follows 5 steps: Identification of Mode of Operation, MTXOP Request, MTXOP Response, MTXOP Acknowledgement and MTXOP Data Transmission. This section discusses about the general purpose of each step, whereas the technical details of each step will be described in subsequent sections.

Step 1: Identification of the WLAN Mesh Mode of Operation

This step is performed during the Mesh Network discovery and Association phase. As discussed in section Error: Reference source not found, when a MP powers up, it scans all channels to find mesh beacons from its neighbors. Based on the Beacon information, the MP learns about the mode of operation used in the mesh network. The mode of operation can be Periodic Mode , Dynamic Mode or Shared Coordination Channel Mode.

Step 2: The MP sends an MTXOP Request message to some of its neighborg MPs.

Once the DRCA Mode has been agreed upon, an MXTOP negotiation can take place. If a device has packets waiting to be sent, it needs to setup a MTXOP with the neighborg MP(s) that which the packet has to be transmitted, as defined by the Routing function. At this step, the sender is the the owner of the MTXOP. Based on the amount of data and the QoS required to transmit them, the MP defines a first MTXOP, which it sends to its neighbor with which it wants to communicate through a MTXOP Req.

The way the MTXOP Req message is sent depends on the mesh network Mode of Operation and will be described in the subsequent sections.

Step 3: When an MP receives the MTXOP Req, it sends back an MTXOP Rsp.

The Destination MP receives and processes the MTXOP Request from the Source MP. If the destination also needs to send data frames to the source, it informs its peer by setting a bi-directional MTXOP and by adjusting the duration accordingly in the MTXOP Rsp. If desired, it can require being the new owner of the MTXOP

The Destination MP sends a MTXOP Rsp message wich contains the following fields:

- Accept MTXOP Req proposal

  - Agreed data rate and duration

  - Agreed number of “free” channels (N) and their channel numbers for traffic channel

    - This channel list considers the view points from both sender and receiver

  - Agreed number of radios (m)

    - N ≥ m

  - MTXOP ownership

- Reject MTXOP Req proposal

  - Clear

    - Channels and their duration

The way the MTXOP Req message is transmitted depends on the mesh network Mode of Operation and will be described in the subsequent sections.

Step 4: When the source receives the MTXOP Rsp, it returns an M-ACK to the destination.

The source MP verifies if the updated MTXOP sent back in the MTXOP Rsp. In that case, it accepts or denies the MTXOP and it can opt to send out an M-ACK containing the following fields:

- Accept proposal

  - Optionally, the MTXOP information (duration, channel, owner…) can be repeated

- Reject proposal

Similarly to a RTS/CTS mechanism, if surrounding devices overhear an M-ACK message with acceptance, they can setup their NAVs to avoid transmitting and  colliding during this MTXOP.

The way the M-ACK message is transmitted depends on the mesh network Mode of Operation and will be described in the subsequent sections.

Step 5: Data transmission phase

Once the MTXOP Starting Time arrives, the data transmission can start over the agreed traffic channel(s). The MTXOP owner is the first MP to transmit data, as agreed during the MTXOP negociation.  Each transmission begins at the starting time of the reserved MTXOP. However, the transmission begin may be deferred, if the MPs chooses to do so. As the solely restriction, any transmission is limited to the duration of the reserved MTXOPs.

In case of a unidirectional MTXOP with multiple receivers, the sender (e.g. the owner) can combine data it plans to send during this MTXOP into a frame aggregation.

##### Periodic Mode DRCA

The periodic scheduling mode uses the standard 802.11 contention and contention free period (CP and CFP) super-frame definition to separate Mesh WLAN and BSS traffic.The CP uses 802.11 medium access functions as DCF, EDCA and HCCA. During the CP, APs serve their associated stations and stations send frames to the APs. The CFP is reserved for the transmission of Mesh WLAN unicast, multicast and broadcast data, control and management frames. During the CFP no Mesh Access Point (MAP)/Access Point (AP) may neither schedule any data transmission to its associated stations nor poll any of them. Hence, the Contention Free Period (CFP) is also called Mesh Contention Free Period (MCFP). MPs use the CFP to exchange packets and to organize the Mesh WLAN. The CFP is subdivided into Beacon Period (BP) and the Mesh Traffic Period. The BP is used to organize the Mesh WLAN, whereas the MTP serves as traffic channel.

Step 1 (Periodic Mode): Identification of the WLAN Mesh Mode of Operation

Once Step 1 of the DRCA is completed, the MTXOP negotiation starts with Step 2.

Step 2 (Periodic Mode): The MP sends an MTXOP Request message to one of its neighborg MP.

Using the Beacon Period Access Protocol (BPAP), any Mesh Point (MP) sends information elements (IE) to its neighbors, which are used to negotiate the ownership of the upcoming MTXOPs in the current and the next MTP. MPs learn for each MTXOP, which MP transmits and which MP receives during an MTXOP.

The occupation of one or more MTXOPs is negotiated between the transmitter (which becomes the owner of the MTXOPs if the negotiation is successful) and the intended receiver. An MTXOP can have up to mMaxNoOfReceivers receiving Mesh Points (MPs). An MP, which intends to transmit data to another MP, must reserve one or more MTXOP during the current or following MTP. The transmitter performs the Conditional Clear Channel Assessment (C-CCA). The C-CCA indicates for the availability for each MTXOPs for data transmission. For those MTXOPs, Then, it starts the MTXOP negotiation process with its intended receivers. In its Beacon, the reservation seeking MP transmits an Ownership IE, which indicates the desired MTXOPs, the DEVID(s) of the intended receiver(s), its role in this reservation negotiation process (“transmitter”) and the status of the ownership (“announced”). The ReservationID shall be set to a randomly chosen value that is currently not used for this set of receivers.

The structure of an OIE is drafted in Figure 7-20; the meaning of the different fields depends on the step of the negotiation.

Figure 7-20: Structure of a MTXOP Ownership

The intended receiver replies to the proposed MTXOP reservation of the transmitter by either accepting or declining.

Via the indication of the negotiation status in the OIE bits, all neighboring Mesh Points (MPs) are informed whether the negotiation process has finished or not. As long as MTXOPs are proposed only, but not successfully reserved via the negotiation process, neighboring MPs may claim the MTXOPs too. MTXOPs are reserved as first come first serve. Successfully proposed and reserved MTXOPs may be reused by neighboring MPs according to the C-CCA .

Any active MP scans all beacons of the BP for the occurrence of its DEVID in an OIE where the role is set to “transmitter”. If such an OIE is received the device checks if the ReservationID is already in use, a new ReservationID indicates an unknown negotiation. The intended receiver evaluates whether it can acknowledge the announced ownership, which is the case in the following situations:

- The medium is free during the announced MTXOPs according to the locally stored information, or

- The new reservation has a higher priority than the reservation(s) occupying the intended slots, or

- Simultaneous transmission is possible with high probability.

- A higher priority reservation is allowed to take over some, but not all slots of an existent lower priority reservation.

- A receiver acknowledges an OIE by including an OIE in its own beacon with the following parameters:

- The indicated MTXOPs and the ReservationID are the same as in the transmitter’s OIE

- The Partner-ID is the transmitter’s DEVID

- The role is set to “receiver”

- The status of the ownership is set to “occupied”

In case of a multiple receiver transmission, which is indicated by several receivers in the transmitter’s OEI, the receiver should include an Availability IE in the OIE to shorten the negotiation process in case the other receivers cannot accept the proposed slots. The Availability IE shall be included until a RIE send by the transmitter with the status “reserved” is received.

Step 3 (Periodic Mode): When a device receives the MTXOP Req, it sends back a MTXOP Rsp.

A receiver may change a proposed ownership by answering with an ownership status of “announced”. If possible it should also propose an alternative reservation by setting the reserved slots in the REI and by including an Availability IE, which can be used by the transmitter to compute a successful reservation.

The transmitter has to keep sending its “announced” occupation until it has received an answer from all intended receivers or the reservation announcement has been mTryReservations Beacon Periods sent unsuccessfully.

After the transmitter has collected acknowledging OIEs from all intended receivers the negotiation process has successfully finished and the transmitter becomes the owner of the MTXOPs. All other Mesh Points in the neighborhood, which have overheard the OIEs, must respect this ownership. The intended receiver(s) have to listen during the reserved MTXOPs for data transmissions.

If an intended receiver of the RIE in the initiating beacon detects the proposed MTXOPs as being occupied and no other MTXOPs can be reserved, or if the device is not willing to accept the reservation for any other reasons, it shall send a RIE in its next beacon with

- The DEVID set to the transceiver’s DEVID

- The ReservationID set to the ReservationID of the initial OIE

- The role set to “receiver”

- The status of the ownership set to “announced”

- The reservation information set to zero

- Such a RIE shall be interpreted as a declining of a reservation.

Step 4 (Periodic Mode): When the source receives the MTXOP Rsp, it returns an M-ACK to the destination.

The MTXOP Acknowledgement is done through the BPAP. After successful negotiation the participating Mesh Points (MPs) include an OIE in their beacon, which indicates the occupied MTXOPs and has the status set to “occupied”. All other devices that receive these beacons honor this negotiated ownership.

In case that the owner or one of the receivers needs to change the occupation, both can restart the negotiation process. To renegotiate, an MP sends an OIE with the new information and the old ReservationID, but with the status set to “announced”. If a receiver initialized the restart of the negotiation, it should include an Availability IE in its beacon.

If a transmitter or a receiver wants to cancel an existing reservation, they send a cancellation OIE which consist of the same ReservationID and the partner’s DEVID, the status “occupied” but the reservation information filled with zeros. All neighboring Mesh Points may delete this reservation from their internal occupancy map after receiving this special OIE.

Step 5 (Periodic Mode): Data transmission phase

The data transmission occurs during the Mesh Traffic Period (MTP)

##### Dynamic Mode DRCA

The dynamic mode scheduling allows for quick negotiation of MTXOP operation. Its operation is based on a time-based scheduling where two neighbors mutually agree on a suitable time to exchange information based on knowledge of each other’s schedule.

Step 1 (Dynamic Mode): Identification of the WLAN Mesh Mode of Operation

Once Step 1 of the DRCA is completed, the MTXOP negotiation starts with Step 2.

Step 2 (Dynamic Mode): The MP sends an MTXOP Request message to one of its neighborg MP.

The MTXOP can be sent either with dedicacetd Control Message MTXOP Req or peggybacked to user data for efficency (see Data Frame format for details).

The MTXOP negotiation between the MP peers can be initiated as soon as the Mesh Discovery and Association are completed and the MP peers are mutually authenticated. When a new MP first associates with the WLAN Mesh, its scheduler will have slots with no neighbors. To begin with discovery of other MPs need not be coordinated in time and may happen by random overlap of radio related performance parameters and time. However once MP neighbors discovery is completed and they exchange messages to reserve slots in each others schedules to allow subsequent scheduled MTXOP negotiations to be coordinated in time. Since the reservations are pairwise, if each node reserves slots in a serialized fashion and ensure non-preemption in its schedule it can satisfy the exclusive pair condition. This implies that the schedule created would have successfully created as many different pairs as possible thereby maximizing medium usage, given that the radio related performance parameters of two links do not overlap in space or time

A message exchange sequence (shown in figureError: Reference source not found) is used to negotiate the free slots and the time required for the MPs to drain their data. The messages contain the parameters above. Negotiation between two MP A and B is as follows. First, A notes its clock value at present as a reference time for the present negotiation and computes the tuple {qAB, FAB, B’AB, QAB, PAB} where the elements of the tuple are defined below. A ensures that all time values in the message is relative to the reference time.

A

B

FAB , FBA:: slots available

qAB, qBA: multi-priority queues

QAB , QBA: change in queues

B’AB , B’BA: previous slots

PAB: parameters available

CAB : collision slots

IAB : susceptible slots

BAB, BBA: slots booked

qAB, qBA: multi-priority queues

QAB , QBA: change in queues

B’AB , B’BA: previous slots

PBA: parameters available

CAB : collision slots

IBA : susceptible slots

IBA = α (IAB , CAB)

Figure 7-21: DRCA Message negotiation

Since the slot will begin a finite amount of time after the exchange to reserve the slot, a function that computes the expected data to be exchanged using the present queue length and history is used at the time of negotiating the slot size. By using suitable policies (e.g. QoS prioritization), by capping the maximum slot that a neighbor is assigned and each busy neighbor is given equal opportunity to book slots according the system policies, MPs can ensure that their schedule is fair. Variations of booking schemes, such as time proportional to the ratio of data expected on one queue to data on all the priority queues, can be adopted according to the particular application to tailor how the neighbors are allocated time. A generalization of the booking function is as follows:

Bpq = {Si : Si is a slot in the future that is booked for data transfer from node p to q} and can be computed by the function pq(),

Therefore

Bpq = pq(qpq, qqp, Fpq, Fqp, B’pq, B’qp, Qpq, Qqp, Ppq) 		(1)

Where

qpq = the multi-priority queue sizes for data from MP p to q

Fpq = { Si : Si is a slot in the future that is available to be booked for data transfer from MP p to q}

B’pq = { Si : Si is a slot in the past that was booked for data transfer from MP p to q}

Qpq = { qipq : qipq is the change in the queue size for data from MP p to q in the slot i}

Ppq = {tuple of radio related performance parameter available for slots for data transfer from MP p to q}

Step 3 (Dynamic Mode): When a device receives the MTXOP Req, it sends back a MTXOP Rsp.

At the other end, MP B receives the message and computes its own tuple {qBA, FBA, B’BA, QBA, PBA} with its own reference time. Then the booking functions for the directions AB and BA are computed by matching the free slots of both A and B, to find common time intervals, i.e. slots (or parts of slots), from FAB and FBA until all the free slots are exhausted or time for all qAB and qBA has been booked. The resulting set of slots, BAB and BBA are said to be the set of booked slots for that negotiation and are conveyed back to node A so that it can book those slots in its schedule as well.

MP B can then send the MTXOP Resp either with dedicacetd Control Message or by peggybacking to user data for efficency (see Data Frame format for details).

Step 4 (Dynamic Mode): When the source receives the MTXOP Rsp, it returns an M-ACK to the destination.

No explicit MACK message is required for this mode. Since BAB and BBA are computed out of FAB and FBA and only one booking occurs and completes at any given time, the bookings can be guaranteed to be serialized and thus it does not conflict with any existing bookings. Note that in order to convey free slots the MPs need to use relative time, because they have free running independent clocks. Relative time is calculated by a common event, referred to as the reference point of the negotiation. The further the reference point of B is from that of A, the more error will occur in the times booked at both ends.

Step 5 (Dynamic Mode): Data transmission phase

Data exchange between the two neighbors takes place as soon as the scheduled MTXOP starts. Data exchange continues until the MTXOP duration expires or there are no more frames destined to the neighbor.

##### SCC Mode DRCA

The shared coordination channel (SCC) mode requires all mesh points (MPs) to tune to the SCC to monitor the resource negotiations where the SCC can be a dedicated time period or a dedicated frequency.  The SCC mode provides a generic framework to accommodate various techniques utilizing a common (control) channel to provide the QOS and mitigating hidden/exposed nodes problem.

Following the same design principle as the conventional 802.11 distributed coordination function (DCF), the SCC uses both physical and virtual carrier sensing to achieve collision avoidance.  By monitoring the resource negotiations in the SCC, all MPs are able to perform virtual carrier sensing by adjusting their network allocation vectors (NAVs).  Virtual carrier sensing combined with physical carrier sensing provide advantages over physical carrier sensing alone in mitigating hidden node problems and allowing MPs to go into dormant mode and save battery power.

The SCC is defined as a signaling channel used to control and to manage traffic on all channels between a source and destination pair. It is a logical channel used for negotiating traffic channels by all MPs in a particular mesh domain.  It can also be used for the exchange of other control and management frames. The use of the SCC scheduling mode may be enabled during MP start-up or via subsequent discovery process.

The SCC with resource allocation has the following characteristics:

- It provides a simple mechanism to mitigate hidden node problems.  To operate in a mesh network with physical carrier sensing alone, each MP requires dynamic sensing threshold adjustment. The SCC approach does not need this technique to operate.

- With virtual carrier sensing, a device can decide to go into sleep state if no traffic is destined for it.

- When the SCC is implemented using a dedicated frequency, an MP can receive management and control frames at any time. It provides a faster react to topology change. It provides faster power control adjustments. Faster scanning of different channels also becomes possible.

Step 1 (SCC Mode): Establishment of Shared Coordination Channel

As discussed in the previous section, when a MP powers up, each MP scans all channels to find mesh beacons from its neighbors. To speed up the scanning process, a MP can also send out a mesh probe request. When its neighboring mesh AP receives a mesh probe request, it will send out a mesh probe response. From the mesh beacon or mesh probe response messages, each MP perceives the channels that are usable from its neighbor view point. Each device also performs scanning on all channels, and has its own view of what channels are usable.

After the channel interference condition in the local environment is understood, a device negotiates a shared coordination channel (SCC) with its neighbors, e.g. via mesh beacons.  As discussed previously, the SCC can be a dedicated time period in a frequency or a dedicated frequency.

In the extended resource management Information Element (IE) of mesh beacon, a new MP can find the information about the configuration for the SCC. The new device can either agree to use this SCC or request for a change of the SCC. If the new device agrees to the use the SCC, it becomes part of the mesh network. If a device requests for the change of the SCC, there are two possibilities. First, another round of negotiation of a new SCC with existing devices can be performed. Second, this device forms its own mesh network and communicates with the existing network through a portal.

At the end of Step 1, all devices use the agreed SCC to perform resource negotiations.  In addition, it is also possible to use the SCC to perform other mesh management and control functions.

The following discusses how the SCC supports all possible radio and channel configurations as well as support both BSS and mesh traffic.

a. Single Radio with One Operating Channel

For a MP with single radio and one operating channel, both SCC and mesh traffic channel share the same frequency. Its operation follows that of the existing basic service set (BSS) distributed coordination function (DCF). The BSS traffic (from the stations) and mesh traffic can also co-exist by contending the channel following DCF operation.

b. Single Radio with Multiple Operating Channels

For a MP with single radio and multiple operating channels, there are two possible operations.  Since there is only one radio, the SCC would be a dedicated time period in a common frequency.  The SCC may be present periodically.

Immediate scheduling

MPs get together to exchange resource negotiation messages at the beginning of the SCC.  Once negotiation between a source and destination pair is completed, these two MP switch their traffic channel to the agreed frequency immediately.  All other MPs would hear this negotiation, and perform their negotiations afterwards.  Once the transaction on the traffic channel is completed, the MP switches back to the frequency where the SCC is located.  When a MP operates in the SCC, it uses the mesh transmission opportunity (MTXOP) to reserve resource; otherwise, it uses the RTS/CTS. The legacy device always uses the RTS/CTS for resource negotiation and they are not able to switch their channel.

Future scheduling

The time is divided into BSS period and Mesh period.  The BSS period stays on the BSS frequency.  The mesh period is further divided into SCC and traffic sub-periods.  The SCC sub-period stays on the mesh common frequency (which can be the same or different from the BSS frequency).  The MPs must listen to all exchanges of resource negotiations within the SCC before switching their traffic channel to a different frequency.  During the traffic sub-period, the traffic channel can switch to any channel on a per MTXOP basis.

c. Multiple Radios with Multiple Channels

For multiple radios with multiple operating channels, there are three techniques:

Technique 1: An extension of one-radio multi-channel configuration and each radio acts on its own.

Technique 2: One radio is for the SCC and all the other radios are for the mesh data.  The BSS traffic can either use the SCC radio or the mesh data radio.  The BSS traffic and the SCC can share the same radio by dividing the time into SCC sub-period and BSS period.  Or the BSS traffic and mesh data can share the same radio by following 802.11 conventional DCF operation.

Technique 3: One radio is for the BSS and all the other radios are for the mesh.  If there is only one additional radio for the mesh, both SCC and mesh data share the same radio by dividing the time into SCC sub-period and mesh data period.  The SCC sub-period is  located in a common frequency and the mesh data can use any one of the available frequencies.  If there is more than one radio for the mesh, one radio is for the SCC and all the other radios are for the mesh data.  Clearly, the SCC scheduling has a better performance when there is more than one operating frequency and the device has more than one radio.

The following discusses the special configuration with 1.5 radios.  The 1.5 radios configuration can be used with one or more than one operating channels.  The extra ½ radio is to receive control and management from the SCC while the other (full-duplex) radio switches the traffic channel to a different frequency.  The ½ radio can also function as a receive-only (simplex) scanner to scan different channels to evaluate their channel condition.

Step 2 (SCC Mode): The MP sends an MTXOP Request message to one of its neighborg MP.

If a device has application packets waiting to be sent, it sends a MTXOP Req message or RTS on the SCC. At this step, each MP continues to scan channels to evaluate the interference condition.  Optionally, each MP can adjust its sensing threshold and transmit power to maximize spatial reuse.

Step 3 (SCC Mode): When a device receives the MTXOP Req, it performs the following procedures:

The receiving device uses the candidate channel list from the MTXOP Req to compare to its own channel list, and finds their intersection. Assume their intersection contains N operating channels. Our procedure allows for bi-directional resource allocation. If the destination also needs to send data frames to the source, the destination will adjust the duration (TXOP) in the MTXOP Rsp accordingly. The receiving device sends a MTXOP Rsp message out. The MTXOP Rsp message contains one or more of the following fields:

- Accept MTXOP Req proposal

  - Agreed duration (TXOP)

  - Agreed data rate

  - Agreed number of “free” channels (N) and their channel numbers for traffic channel.  This channel list considers the view points from both sender and receiver

  - Agreed number of radios

- Reject MTXOP Req proposal

  - Clear

  - Channels and their duration

When a MP receives a RTS, it will send out a CTS.

At this step, each MP continues to scan channels to evaluate the interference condition. Optionally, each MP can adjust its sensing threshold and transmit power to maximize spatial reuse.

Step 4 (SCC Mode): When the source receives the MTXOP Rsp, it returns an M-ACK to the destination.

The M-ACK contains the following fields:

- Accept proposal

  - Channels and their duration

- Reject proposal

  - Channels and their duration

When a MP receives a CTS, the data phase can begin and there is no Step 5.

At this step, each MP continues to scan channels to evaluate the interference condition. Optionally, each MP can adjust its sensing threshold and transmit power to maximize spatial reuse.

Step 5 (SCC Mode): When the destination receives an M-ACK with acceptance, the data phase can begin. The device starts data transmission using agreed traffic channel(s). When the destination receives a M-ACK with rejection, the destination also sends out a M-ACK. Both M-ACKs are used to reset the network allocation vector (NAV) of surrounding devices that overhear the control messages exchanging.

The loading level of the SCC is measured by examining the collision conditions. When the loading level exceeds a threshold, existing MPs have the option of not allowing the association of a new MP into an existing mesh network. These new MPs will need to form their own mesh network if additional time or frequency is available.

#### MTXOP Occupancy Map

All MPs build up an internal MTXOP Occupancy Map (MOM). In this map, a MP marks MTOXPs as occupied or free together with the related MTXOP Ownership information.

- The owner of the MTXOP, which is the MP that will transmit first in the MTXOP.

- The receiver(s) of this MTXOP which expect to receive data from the owner during the MTXOP.

Thus, if in a MP MPa the schedule for any time instant indicates that it is to communicate with a neighbor MPb then there is a corresponding entry for the same time instant in the schedule of MPb that indicates that it is to communicate with MPa.

When a new MP first associates with the WLAN Mesh, its schedul will have only empty slots. The discovery of other MPs does not need to be coordinated in time and it may happen at any time. Once the discovery is completed, neighbor MPs exchange messages to reserve slots in each others’ schedules to allow subsequent scheduled MTXOP negotiations to be coordinated in time.

### Multi-Radio Support

As explained in previous sections, logical Mesh Links (ML) connecting two neighboring MPs can be mapped to multiple physical channels.

Although the treatment of the scheduling problem above assumes that MPs have a single radio dedicated for the ML communication to multiplex to multiple neighbors, the mechanism described above can be easily extended to MPs with more than one radio. The scheduling becomes trivial if the number of radios is greater than the number of neighbors that a MP has to form a link with. If however the number of radios is smaller than the number of links then the mechanism above can be applied as follows:

- Each ML will have its own schedule which is maintained independently of the other schedules in the same MP, except during negotiations. When the set of available slots is being calculated, slots on all available radios are included. The booking function thus can pick slots on any of the neighbors or its own radio’s schedules.

- If more than one radio is available for a given time the function can be clever to select the previous radio to minimize radio set up overhead. Indeed the equation (1) and its arguments will still hold except that the slots will have a superscript to denote the radio number as ski where k is the radio identifier and i a slot identifier. However an important distinction with the single radio case is that MPs with multiple radios need to ensure the serialization of its negotiations and therefore its slot bookings. This can be easily achieved by ensuring that the negotiation with a peer doesn’t start until the time that no other negotiation is ongoing. For instance the negotiation process can mark the slots that it has sent to the other MPs in FAB as under-negotiation and other negotiations will then avoid slots that are marked such. If some of these under-negotiation slots end up being unused, they can be marked as free again.

### MCF Operation

#### Periodic Mode Operation

##### Superframe definition

The periodic scheduling mode uses the standard 802.11 contention and contention free period (CP and CFP) to separate Mesh WLAN and BSS traffic, see chapter 10.1 in Annex A (informative) (chapter 10).The CP uses 802.11 medium access functions as DCF, EDCA and HCCA. During the CP APs serve their associated stations and stations send frames to the APs. The CFP is reserved for the Mesh WLAN uni-, multi-, broadcast data, control and management frames. During the CFP no Mesh Access Point (MAP)/Access Point (AP) may neither schedule any data transmission to its associated stations nor poll any of them. Hence, the Contention Free Period (CFP) is called a Mesh Contention Free Period (MCFP). MPs use the CFP to exchange packets and to organize the Mesh WLAN. Therefore the CFP is subdivided into the Beacon Period (BP) and the Mesh Traffic Period. Whereas the BP is used to organize the Mesh WLAN, the MTP serves as traffic channel. The BP is furthermore used to send sequentially coordinated beacons.

Figure 7-22: The layered structure incorporating legacy 802.11

##### Mesh Contention Free Period (MCFP)

The MCFP is divided into Transmission Opportunities (see 3.89, 9.9, 9.9.1.2 and 9.9.1.4 in [11]) of equal duration. The TXOPs used during the MCFP are called Mesh Traffic Opportunities (MTXOPs). The MTXOPs are used for the Beacon Period (BP) and the Mesh Traffic Period (MTP). Hence, the MCFP is subdivided into the Beacon Period (BP) and the Mesh Traffic Period (MTP). During the Beacon Period (BP) MPs exchange information to negotiate on the usage of Mesh Transmission Opportunities (MTXOPs).

###### Beacon Period (BP)

The Beacon Period (BP) in Mesh WLAN is used for the coordination of neighboring devices. It enables fast Mesh neighborhood discovery and association. The BP forms the initial sequence of each CFP, see chapter Error: Reference source not found in Annex A (informative). With IEEE 802.11s, at Target Beacon Transmission Time (TBTT) all MPs in the Mesh WLAN start a common Beacon Period (BP). During the BP each MP sub-sequentially transmits a Beacon frame to

- Inform associated stations with certain information elements (IE)

- Declare a Contention Free Period (CFP) to its associated and neighboring stations

- Inform associated and neighboring stations of the beginning of the next superframe and the next CFP

- Announce their presence to neighboring MPs

- Coordinate the data, control and management frame exchange among MPs in the MCFP

- Coordinate the Beacon Period (BP) and the beacon transmission herein

- Exchange neighborhood and topology information among MPs

The Beacon Period Access Protocol (BPAP) is enables collision free transmission of Beacon frames during the Beacon Period (BP).

Beacon Period (BP) Timing Structure

The Beacon Period (BP) is divided into Beacon Transmission Slots (BTSs). Each BTS has duration of mBTSDuration. mBTSDuration equals the duration of MTXOP divided by mBTSPerMTXOP. Any transmission of a beacon shall start at the beginning of a BTS. Therefore, the duration of the BP must be a multiple of MTXOP.

Figure 7-23: The detailed structure of a beacon period with three zones

A beacon may occupy several subsequent BTSs. Beacons are ordered according to the amount of BTS their transmission occupies. Beacons, which occupy the same amount of BTSs, are grouped into a dedicated zone. In zone number “i”, only beacons of the length “i” * mBTSDuration are allowed. Zone “i” shall end with the end of a MTXOP. Between each Beacon zone, there are “i” * mFreeBTSsInZone at the end of each zone, which allow new Mesh Points to join the BP by sending a beacon in these BTSs. The owner of the first BTS in the following zone frees its BTS if the number of free BTSs in the previous zone falls below the mFreeBTSsInZone. If the number of free BTSs in the last zone decreases below the minimum, the Beacon Period (BP) grows by the required amount of MTXOPs. A possible Beacon Period (BP) is shown in Figure 7-23. In the configuration chosen here, three BTSs exist per MTXOP. mFreeBTSsInZone equals 2.

If a Mesh Point must send a beacon in a zone, which does not exist, it sends a beacon in the next smaller zone, indicating the creation of the new zone in its BPOIE.

The maximum duration of the Beacon Period (BP) is limited to mMaxBPLength MTXOPs, but may be considerable smaller, and may differ in separated areas of the Mesh WLAN.

Any Mesh Point that wants to transmit or receive data in the upcoming Mesh Traffic Period (MTP) must send at least one beacon in the in an unoccupied BTS and listen to its neighbor’s beacons.

It is possible for any Mesh Point (MP) to send more than one beacon in the BP as long as different information is transmitted in each beacon. It is recommended that a Mesh Point (MP) shall aggregate its beacons to a single one. The MP shall search for a Beacon Zone, where it can transmit the aggregated beacon information in a single beacon frame of appropriate size. It may be possible, that the MP creates a new zone. Each transmission has to end with a guard time of at least mDurationBetweenBeacons before the next BTS starts.

Beacon Contents

The Beacon Information Elements serve the legacy 802.11 stations and coordinate the Mesh WLAN: The beacon structure complies with the structure defined in [10], which is presented in the following figure.

Figure 7-24: The standards 802.11 beacon with a CFP Parameter set and a BPOIE

Every beacon send by a Mesh Point includes a CF-Parameter set. The CF-Parameter set indicates the beginning of a CFP. The beacon period occupancy IE (BPOIE) is supports the Beacon Period Clear Channel Asessment (BP-CCA).

BP Duration field

The BPDuration field is used to indicate the current duration of the BP from a MP’s point of view. The BPDuration represents the amount of MTXOPs that the MP will listen for Beacons before the Mesh Traffic Period (MTP) begins. The BPDuration is calculated as the maximum of

- the last heard traffic in the last BP,

- the last occupied BTS as reported from the received beacons in this BP,

- the last occupied BTS as reported from the received beacons in the last BP

plus the appropriate number of free BTS which can be calculated recognizing the number of zones (by the extension slots of each zone) in the last BP. The BPDuration shall never grow larger than mMaxBPLength.

BP Bitmap

In any beacon send, an MP announces its view of the BTS occupancy of the BP. The information inside the BP Bitmap shall be as recent as possible, e.g. incorporating information of the BP Bitmaps of beacons that have just been received. Each bit double inside the BP Bitmap corresponds to one BTS. These two bits indicate the occupancy of a certain BTS as seen by the Mesh Point (MP). The four possible combinations and their meaning are presented in Error: Reference source not found.

Table 7-2: The Four Possible Beacon Slot States, as indicated in BP Bitmap

| Element value (b1b0) | Beacon Transmission Slot (BTS) interpretation |
| --- | --- |
| 00 | Free BTS The currently transmitting Mesh Point can receive beacons here. |
| 01 | Occupied by sending Mesh Point This slot is occupied by the currently transmitting Mesh Point and this Mesh Point has sent/is sending/will send a beacon in this BTS. |
| 10 | Occupied by neighboring Mesh Point This slot is occupied by a neighboring Mesh Point, and the Mesh Point currently transmitting has successfully received a beacon in this BTS or expects to receive one if it refers to this BTS in the future. |
| 11 | Occupied by neighbor’s neighbor Mesh Point This BTS is occupied by a Mesh Point which is a neighbor’s neighbor but not a direct neighbor or out of the receiving range but still creates noise. The Mesh Point expects that a beacon send to it will not be decoded successfully because of the existent interference. |

The BP Bitmap is build up internally during the BP, incorporating new information while beacons are received. In each transmitted beacon the most recent BP Bitmap is send. If the BP Bitmap has entries set to 10 or 11, the MP indicates that it knows the neighboring MPs. The DEVID of the MP owning a BTS is indicated after the BP Bitmap. According to the order of the BTS owners known to the MP, the MP stores the owner's DEVID in the BTS owner vector. The owner vector consists of mDEVIDBits bits for each entry set to 10 or 11 in the BP Bitmap, and indicates the DEVID of the according Mesh Point.

Beacon Transmission and Reception

After a Mesh Point is powered up, it scans for beacons in mScanBeacons subsequent Beacon Periods (BPs). If the device received no valid Beacons after the scan, it creates a new BP by sending a beacon in the first BTS in the BP.

If an MP receives one or more valid beacons during the scan, it does not create a new BP. Instead it builds up an Internal Occupancy Map (IOM) which is updated with every beacon received. The IOM is a bitmap consisting of mMaxBPLength bits. Each bit corresponds to a BTS in the BP. A bit is set to one if and only if the Mesh Point

- is the owner of this slot, which requires that it has send a beacon in this BTS before, or

- has received a beacon in this BTS, or

- has received a beacon with a 01, a 10 or a 11 in the corresponding entry in the BP Bitmap.

- Otherwise, the bit is set to zero.

Using the occupancy map (and the information about the zones which can be computed from the received beacons), the Mesh Point (MP) sends a beacon in the first BTS marked zero in the zone that corresponds to the length of the beacon to be send. If the appropriate zone does not exist, the Mesh Point creates a new zone by sending a beacon in the last free BTS.

Beacon Collision Detection and Resolving

If a device detects a beacon collision as it shall randomly choose another BTS which is marked zero in its IOM. Let the Mesh Point with the DEVID “x” send a beacon in the BTS “j” in the BP “n”. The MP shall consider the beacon to be transmitted successfully if and only if in all beacons received by neighboring Mesh Points (MPs) in the following BP entry “j” in the BP Bitmap is set to “10” and the corresponding entry in the Owner Vector is “x”, or the entry “j” is set to “00”. If neighboring MPs cannot indicate the successful reception, the MP shall consider itself being involved in a beacon collision. Then, the MP shall draw a uniformly distributed number over the amount of BTSs marked as unoccupied and transmit in a randomly chosen BTS.

BP Leaving

If a Mesh Point can free one of its BTS because the amount of its beacon information has reduced, it shall free its last beacon slot in the BP. It shall furthermore announce its departure by sending a last beacon in the BTS where this BTS is marked as “00” in the BP Bitmap.

BP Contraction

If a Mesh Point is according to its IOM the last beacon holder of a zone and there are BTSs in the same zone which are marked as free, the Mesh Point shall shift its beacon to the free BTSs by the following procedure. If “j”-th BTS is free, the MP shall

- transmit a beacon in the originating BTS with the BP bitmap indicating BTS “01” for BTS “j” and indicating its own DEVID

- transmit a beacon in BTS “j” during the next BP

- transmit a departure beacon as described in section Error: Reference source not found in the originating BTS

It is possible that the extension zone and therefore the beginning of the next zone or the traffic period shifts after a shifting of one or more beacons.

Beacon Period Clear Channel Assessment (BP-CCA)

The Beacon Period Clear Channel Assessment (BP-CCA) generates the Beacon Period Occupancy Information Element (BPOIE) to indicate the occupancy status of each BTS. It is used by the Beacon Period Access Protocol (BPAP). A BPOIE indicates the BP Bitmap (see Error: Reference source not found). For each BTS MPs measure the Received Signal Strength (RSS). If the RSS exceeds the threshold of mBTSNoiseThreshold, MPs store the RSS information regardless whether the Beacon frame can be successfully received or not. In combination with the neighborhood information provided by the Beacon frame IE from neighboring MPs, MPs learn the DEVID of the MP, which holds this BTS for its transmission. Together with the DEVID of the BTS-owner the RSS value per BTS is used set-up an neighborhood map. The neighborhood map stores the information of the measured RSS and the according DEVID. Each MP updates the neighborhood map with the most recently gathered information.

Beacon Period Access Protocol (BPAP)

An AP must transmit a beacon frame at TBTT. The Beacon Period Access Protocol (BPAP) coordinates sub-sequential transmission of Beacon frames of different MAPs and MPs. The beacon frames are enhanced with specific Mesh WLAN Information Elements (IE), which support topology control, neighborhood detection, interference awareness and other functionality in the Mesh WLAN.

MPs transmit beacon frames during the Beacon Period. The BP starts at TBTT and has duration, which depends on the neighborhood. The BP consists of several MTXOPs. These MTXOPs form the Beacon Period (BP). As each MTXOP lasts longer than the transmission of a single beacon frame lasts, the BP is divided into Beacon Transmission Slots (BTSs) of equal duration. The duration of an MTXOP is a multiple of BTS duration.

The occupancy status of each BTS is disseminated in the near neighborhood. This lowers the probability of a collision of two beacons from different Mesh Points (MPs) in the same BTS. The occupancy information is disseminated over three hop distance. To prevent beacon collisions, the beacon period occupancy information element (BPOIE) in every beacon informs neighboring Mesh Points during which BTS a beacon will be sent or received as seen by an MP. Each MP maintains a BTS Occupancy Map (BOM). The MP performs the Beacon Period Clear Channel Assessment (BP-CCA) to decide if a BTS is occupied or available for beacon transmission In the BOM, the MP stores the BPOIE of each BTS.

###### Mesh Traffic Period (MTP)

The Distributed Reservation Channel Access (DRCA) builds the Mesh Coordination Function (MCF). It works independent from legacy 802.11 contention based accessed. MPs use the DRCA for channel access during the CFP, see Error: Reference source not found.

During the BP every Mesh Point (MP) sends a beacon frame. In its beacon frame, it announces the start of a CFP to its associated stations including the start time of the next CFP.

Figure 7-25: Alternation of BSS-traffic and Mesh-traffic

Impact of Non-QoS stations to CFP

A foreshortened CFP has impact on the mesh network. The CFP cannot start as. To guarantee an idle Wireless Medium (WM) at the beginning of the CFP, two approaches may be used

The TBTT is announced to be a short duration before the BP starts. This buffer zone has exactly the duration of a frame of maximum size, which is sent at the basic PHY mode. APs may send traffic in the buffer zone, using the 802.11e EDCA to avoid collisions with other APs.

It is the MAPs responsibility to access the channel via DCF, EDCA or HCCA access the wireless medium before TBTT to block it from stations access.

MSDU Acknowledgements

The ACK frame is used by a receiver to report the reception status of a MSDU or a segment of a MSDU. The ACK may indicates successful or failed reception. To allow for predictable and reliable usage of the Wireless Medium (WM), immediate acknowledgement is not permitted. The receiver of a MSDU must use the delayed ACK scheme. The ACK may be send as either implicit or explicit acknowledgment.

For the explicit acknowledgment the receiver of an MSDU sends ACK or N-ACK frame to the transmitter of the MSDU in an MTXOP different from the MTXOP used for the MSDU transmission.

For the implicit acknowledgment the receiver of an MSDU combines the ACK frame with MSDU, which shall be send to the transmitter, to the transmitter of the MSDU in an MTXOP different from the MTXOP used for the MSDU transmission. The ACK is piggybacked onto MSDU frames.

The ACK frame uses two different schemes:

The cumulative ACK indicates the sequence/fragment number of the next MSDU expected from the receiver. It indicates the successful or failed reception of all MSDUs or fragments of MSDUs that have smaller sequence/fragment number than indicated.

A bitmap ACK is used to indicate the successful and/or failed reception of MSDUs or fragments thereof. The sequence control field specifies the next sequence/fragment number of the next frame which is expected at the receiver. It is followed by a bitmap. The bitmap indicates the status of the MSDUs that have been received, it starts with the last successful MSDU (as indicated by the sequence control field – 1), for each sequence number two octets in the bitmap represent the status of up to 16 fragments of the corresponding MSDU. The last two octets in the bitmap stand for the MSDU with the lowest sequence number (in a modulo sense) that was not received correctly.

In both ACK frame types the buffer size field indicates the amount of free space in the receiving buffer reserved for this particular transmitter, counted in bytes. This size indicator is used as a flow control mechanism to prevent the transmitter from overstraining the receiver. The transmitter may send only the given amount of bytes before it has to wait for the next ACK. Retransmission of the last frame by the transmitter or the receiver may be used to update this indicator. A Mesh Point in an overload condition may signal its problem by lowering the free buffer size below the actual capacity. A Mesh Point may use the free buffer size as told by the next hop to calculate its indicated free buffer size for the previous hop.

Conditional Clear Channel Assessment (C-CCA)

The Clear Channel Assessment (CCA) is a fundamental element of Carrier Sense Multiple Access with Collision Avoidance (CSMA/CA). It defines the wireless medium (WM) as either Busy or Idle. In a single hop WLAN this scheme is sufficient for collision free access.

To allow for increased efficiency, the Conditional Clear Channel Assessment (C-CCA) adds the states

- Clear to receive (CTR)

- Clear to transmit (CTT)

While the Busy and Idle states are unconditional, CTR and CTT are conditional states. The C-CCA may sense an MTXOP of the MTP as CTR if it is likely to successfully receive data during the MTXOP. The CTR state indicates that the MTXOP is unused by other neighboring MPs or neighboring radio transmissions cannot harmfully interfere.

An MTXOP is sensed as CTT if neighboring data reception at neighboring MPs cannot be interfered by a local data transmission.

To predict which MTXOPs can be sensed as Busy, Idle, CTR or CTT, MPs use the MTXOP reservation information they received from their neighboring MPs. Signal strength measurements during the Beacon Period (BP), allow for estimation of neighboring radio transmission levels.

All active Mesh Points that listen to beacons have to process the OIEs to build up an internal Mesh Traffic Period Occupancy Map (MTPOM). In this map, a Mesh Point marks MTOXPs as occupied or free, and if it is occupied it additionally stores

- The owner of the MTXOP, which is the Mesh Point that wants to transmit in the MTXOP.

- The receivers of this MTXOP which expect to receive data from the owner during the MTXOP.

- An MTXOP is marked as occupied if

- A OIE is received in which it is marked as “occupied”

Noise is sensed in this MTXOP during the previous MTPs which is above mNoiseThreshold.

##### Contention Period (CP)

Following the CFP, a CP enables legacy 802.11 stations to transmit data to the AP and receive data from the AP. Since the CP is used to serve the stations associated to an AP, the CP is used for the BSS Traffic Period (BTP).

###### BSS Traffic Period (BTP)

The BSS Traffic Period (BTP) follows each Mesh Traffic Period. During the BTP stations use the Coordination Functions (CF), which are supported by them to access the wireless medium (WM) for data transmission. The AP may further use the HCCA if it is collocated with an HC. The BTP is fully compatible with 802.11, all of its existing and upcoming amendments. The performance of data transmission handled in the BTP is solely defined by the CF used within.

#### Dynamic Mode Operation

##### Beaconing and Synchronization in Dynamic Mode

In order to avoid the problem of clock drift the method proposed only used time intervals that can be bounded as a function of the maximum drift in individual clocks. Scheduling proposed here relies on time intervals relative to each neighbor. Given the worst case variation in the clocks the scheduler is able to bound the variation in each interval with each of its neighbors independently of others. By periodically determining a fresh set of intervals it avoids loosing synchronization. Indeed the method described here doesn’t require any MP to ever change its own notion of time, hence avoiding de-synchronization with other MPs. In fact one can compute the maximum time before two MPs must negotiate another interval. Consider the figure belowError: Reference source not found, where a message is sent from B to A. If e is the error in timing on the message exchange and ms and mus are the deemed to be the minimal slot size and the minimal sized slot that can accomplish useful data exchange then:

mus≥(D+mu)−(e+D+δD)D≤mus−mu−eδ ⋯(1)where:D=maximum time before which two nodes must negotiate another interval ≤Dδ=maximum time that the clocks on two nodes can vary in one unit of time

A

B

A knows the schedule that B had accepted

B starts counting down the interval D

D

D

e

D

mus

ms

Figure 7-26: Clock Drift Re-Synchronization

Time in a MP is a non decreasing counter that can be read at any instant in a variable T. This counter can be viewed as slotted, where a time slot is an atomic time unit for the scheduler (i.e. a singular useful task can be accomplished in a time slot). A MP’s schedule is then a set of contiguous slots SC = {S0 , S1 , S2 , ..., Sk} = {t0 , t1 , t2 ... , }, where t0 is the start of the next slot as well as the end of the present slot; t1 is the end of slot 0 and start of slot 1 and so on and slot Si occurs before Sj implies that the corresponding start time values are such that ti < tj. Furthermore the schedule only has entries for the finite future (i.e., tk is a finite value of the time counter) and the current value of T is never greater than t0.

Time slots are represented as Si = {ti , ni , pi , ci}, where ti is its start time i.e., the value of T at which slot i is to start, ni is the corresponding neighbor, pi a tuple of radio related performance parameters (e.g. radio aware metric & measurements, QoS parameters, routing parameters etc.) that is to be used for the slot and ci a list of MPs that had contended or may contend on this slot. Note that ni , pi , ci = null for the times when there are no neighbor to communicate with the local MP.

Each MP creates a schedule similar to the one above, such that, if in a MP ni the schedule for any time instant indicates that it is to communicate with a neighbor nj then there is a corresponding entry for the same time instant in the schedule of nj that indicates that it is to communicate with ni. This is called the exclusive pair condition. If time was denoted in the schedule in terms of contiguous discrete intervals, called slots, then the exclusive pair condition implies that if in a MP ni, the schedule for a time slot indicates that it is to communicate with a neighbor nj then there is a corresponding entry for the same time slot in the schedule of nj that indicates that it is to communicate with ni.

##### Recovery from Failures of Future MXTOP Negotiation

Scheduled communication with a neighbor may be lost if no booking for a future slot exists in the upcoming schedule. In order to prevent this from happening, minimum time slot to book the future slots is necessary in the absence of any data to be exchanged. Booking small slots that will be purely for the sake of completing another negotiation and thereby keeping the neighbor in the schedule.

Whenever an MP is unable to complete the negotiation phase of a slot the scheduler might then declare a ML down event. A recovery procedure was thus added. The recovery process is parameterizable in the number of attempted recoveries. The mechanism works by remembering the last interval between the previous two successful negotiations. Upon failure to determine the next time the radio is to be aligned each MP will unilaterally book a slot at a predetermined multiple of the last interval between the previous two successful negotiations. Since both ends will have the same value of the interval and the same multiple it’s likely that they will be able to book the same time and be able to continue their negotiations and data exchange in the upcoming slot, if the problem was transient. However if the MPs persistently fail to communicate the recovery will not work and the MPs will stop unilaterally booking slots after a predetermined set of recoveries.

#### SCC Mode Operation

This section discusses how the SCC Mode supports all possible radio and channel configurations as well as the combination of 802.11 BSS (i.e. 802.11 Infrastructure) and WLAN Mesh traffic.

a. Single Radio with One Operating Channel

For a MP with single radio and one operating channel, both SCC and mesh traffic channel share the same frequency. Its operation follows that of the existing basic service set (BSS) distributed coordination function (DCF). The BSS traffic (from the stations) and mesh traffic can also co-exist by contending the channel following DCF operation.

b. Single Radio with Multiple Operating Channels

For a MP with single radio and multiple operating channels, there are two possible operations.  Since there is only one radio, the SCC would be a dedicated time period in a common frequency.  The SCC may be present periodically.

Immediate scheduling

MPs get together to exchange resource negotiation messages at the beginning of the SCC.  Once negotiation between a source and destination pair is completed, these two MP switch their traffic channel to the agreed frequency immediately.  All other MPs would hear this negotiation, and perform their negotiations afterwards.  Once the transaction on the traffic channel is completed, the MP switches back to the frequency where the SCC is located.  When a MP operates in the SCC, it uses the mesh transmission opportunity (MTXOP) to reserve resource; otherwise, it uses the RTS/CTS. The legacy device always uses the RTS/CTS for resource negotiation and they are not able to switch their channel.

Future scheduling

The time is divided into BSS period and Mesh period.  The BSS period stays on the BSS frequency.  The mesh period is further divided into SCC and traffic sub-periods.  The SCC sub-period stays on the mesh common frequency (which can be the same or different from the BSS frequency).  The MPs must listen to all exchanges of resource negotiations within the SCC before switching their traffic channel to a different frequency.  During the traffic sub-period, the traffic channel can switch to any channel on a per MTXOP basis.

c. Multiple Radios with Multiple Channels

For multiple radios with multiple operating channels, there are three techniques:

Technique 1: An extension of one-radio multi-channel configuration and each radio acts on its own.

Technique 2: One radio is for the SCC and all the other radios are for the mesh data.  The BSS traffic can either use the SCC radio or the mesh data radio.  The BSS traffic and the SCC can share the same radio by dividing the time into SCC sub-period and BSS period.  Or the BSS traffic and mesh data can share the same radio by following 802.11 conventional DCF operation.

Technique 3: One radio is for the BSS and all the other radios are for the mesh.  If there is only one additional radio for the mesh, both SCC and mesh data share the same radio by dividing the time into SCC sub-period and mesh data period.  The SCC sub-period is  located in a common frequency and the mesh data can use any one of the available frequencies.  If there is more than one radio for the mesh, one radio is for the SCC and all the other radios are for the mesh data.  Clearly, the SCC scheduling has a better performance when there is more than one operating frequency and the device has more than one radio.

d. Special configurations

As a special case, a 1.5 radios configuration MP can be enviosioned. In this case, a 1.5 radio MP refers to an MP that has one transmitter and two receivers. Normally, we refer to transceivers as a single radio device, having one associated transmitter (Tx) and one receiver (Rx). Thus, we refer to this special asymmetrical one-Tx and two-Rx configuration as a 1.5 radio MP.

The 1.5 radios configuration can be used with one or more than one operating channels.  The extra Rx (½ radio) is used to receive control and management information from the SCC while the other radio (i.e. Rx and Tx pair) switches for the traffic channel to a different frequency.  The ½ radio can also function as a receive-only (simplex) scanner to scan different channels to evaluate their channel condition.

## RF Resource Management

This section describes signaling and protocol support for WLAN mesh network RF resource management. By including such support in this proposal, RF resource management methods may be implemented to address the following objectives:

- Satisfaction of regulatory requirements

- Improvement of overall network throughput

- Maintenance of mesh link/path quality

- Better use of various types of radios and antenna configurations

- STA operation within the WLAN mesh, e.g., wireless distribution system (WDS) capacity currently available for traffic forwarding

- Policy-based RF management

This signaling and protocol support enables the implementation of various methods to satisfy the above-mentioned objectives. For example:

- Distributed Coordinated Scheduling – method to maximize RF frequency reuse by enabling simultaneous co-channel and adjacent channel transmission between MP neighbors.

- Opportunistic Distributed Automatic Channel Selection – method to automatically select among orthogonal channels to avoid co-channel interference to avoid adjacent channel interference in the case of multi-radio configurations, and to distribute energy across the spectrum within a given geographic area.

- Adaptive Physical Carrier Sensing (PCS) and/or Transmit Power Control (TPC) – method to minimize RF co-channel and adjacent channel interference by automatically adjusting the physical carrier sensing threshold and/or transmitter power level to mitigate the hidden node/exposed node problem

### Transmit Power Control

The ability of an MP to control its transmission power is desirable in order to satisfy regulatory requirements as well as to enable vendor-specific methods for radio network optimization.

For regulatory purposes, different countries/bands have different allowances for transmission power. For example:

- Lower U-NII (5.25-5.35GHz, 4 channels) 40mW US, 200mW Europe

- Middle U-NII (5.35-5.45GHz, 4 channels) 200mW US and Europe

- (5.47-5.725GHz, 11 channels) Europe-only, 1000mW

- Upper-U-NII (5.725-5.825GHz, 5 channels) US-only, 800mW

For purposes of radio network optimization, limiting the transmission power can increase system capacity. For example, channel re-use can be increased by limiting the range over which deferral is induced. Limiting the transmission power can also extend the operation lifetime of battery-operated devices.

In both cases, inter-MP signaling supports the proper setting of transmission power. The proposed solution is inspired by 802.11h and consists of:

- Signaling of Allowed Power Setting information

- Exchange of TPC Capability and TPC Setting information

#### Signaling of Allowed Power Setting Information

The Allowed Power Setting information is used to impose limits on power settings that the MP can use. It is required for regulatory purposes. The Allowed Power Setting information should include the following:

- Regulatory domain where the Mesh network currently operates

- Supported channels

- Min/Max Tx power allowed setting

- Silence/quiet periods

  - Used for detection (measurements) and coexistence with radars

The Allowed Power Setting information may be applicable to the entire mesh network, a particular MP or group of MPs, or a particular mesh link or group of mesh links.

The Allowed Power Setting information may be signaled through Mesh Management frames, e.g., Mesh Beacon, Mesh Probe Response, Mesh Association Request.

#### Excahnge of TTC Capability and TPC Setting Information

TPC Capability information will include the Maximum Tx Power and the Minimum Tx Power that the MP is capable of supporting. It may also include the adjustment step size settings that the MP supports.

The TPC Capability information may be signaled through Mesh Management frames, e.g., Mesh Beacon, Mesh Probe Response, Mesh Association Request.

The TPC Settings information will include the current Tx Power operational settings and may include inter-MP pathloss information. The TPC Settings information may be signaled through Mesh Management frames, e.g., Mesh Beacon, Mesh Probe Response, Mesh Association Request, Mesh Action (Mesh Measurement – TPC). It may optionally be included in Mesh Control frames, e.g., MTXOP Request, MTXOP Response, MTXOP ACK.

### Mesh Link Maintenance (informative)

Mesh link maintenance consists of logical functions to monitor, assess, and/or modify local mesh link parameters. It also includes mechanisms for exchanging related information via inter-MP signaling. Such information may be used to enable automatic channel selection, Tx power setting, and physical carrier sense threshold setting.

These functions also provide link status and radio-aware metrics to be used by higher layer mesh routing protocols.

An example is shown in the following figure.

Figure 7-27: Mesh Link Maintenance Example

#### Measurement Processing

Measurement Processing consists of processing local MP measurement information as well as remotely generated measurement information signaled by other MPs.

Local MP measurement information can include RSSI and CCA information provided by the PHY layer, as well as FER calculated by the MAC layer. Local measurement processing is performed in an opportunity-driven fashion on a non-interfering basis. Dedicated measurement periods may be scheduled whenever there is an opportunity as described in Section 7.4.2.2.

Remote MP measurement information includes either solicited or unsolicited information signaled by other MPs. The exchange of remote MP measurement information is addressed in Section 7.4.3

#### Measurement Scheduling

Local measurements may require that a channel be idle during the measurement interval. In such cases, the measurement interval could be scheduled to occur when the channel is not expected to be busy.

#### Link Quality Assessment

An MP should monitor the quality of a mesh link between itself and an associated/authenticated neighbor MP. The method for link quality assessment is implementation-dependant.

This proposal defines signaling and protocols to enable the exchange of related link quality information between MPs. For instance, measurement-related information may be conveyed via Mesh Management frames (e.g., “Action” frames of category Measurement Request/Measurement Report), Mesh Control Frames (e.g., MTXOP Request/MTXOP Response), or Mesh Data frames with piggybacked information.

Based on processing of local and remote MP measurement information, an MP should determine the status of its mesh links, i.e., is it acceptable for the current levels of service is the link quality degrading, or is the link quality unacceptable to the point where should be declared “down”. In the case where the quality is degrading, the MP may take corrective action to improve the link quality as described in Section 7.4.2.4.

#### Link Parameter Modification

In the case where the link quality assessment indicates that the link quality is degrading, the MP may perform a link parameter modification to try and improve the link quality. The method for link parameter modification is implementation-dependant.

This proposal defines signaling and protocols to enable the exchange of related link parameter information between MPs. For instance, related information (e.g., channel, TPC info) may be conveyed via Mesh Management frames (e.g., Mesh Beacon, Mesh Probe Response, Mesh Association Request), Mesh Control Frames (e.g., MTXOP Request/MTXOP Response), or Mesh Data frames with piggybacked information.

### Mesh Measurements

The Mesh Measurement request/report mechanism can be used to improve mesh network operation (throughput efficiency, QoS maintenance, etc.) by enabling the exchange of metrics and statistics between MPs for functions such as:

- Routing / Forwarding

- Resource Management

- Load Balancing

- Flow Control

- Battery Conservation

- Scheduling

Mesh measurement information from one or more MPs may be requested by any MP. Support for specific measurements may be mandatory or optional. Measurement requests may specify “one-shot”, “event-triggered”, or “periodic” measurement reporting. One-hop or multi-hop measurement request/report is possible. It is also possible for an MP to report unsolicited measurement-related information.

Measurement related information is conveyed in information elements similar to 802.11h/k. Measurement-related information may be conveyed via Mesh Management frames (e.g., Mesh Beacon, Mesh Probe Request/Response, Mesh Association Request/Response, and Mesh Action frames of category Measurement Request/Measurement Report), Mesh Control Frames (e.g., MTXOP Request/MTXOP Response), or Mesh Data frames with piggybacked information.

Measurements and messages can be not only MP-specific, but also ML-specific. In order to uniquely specify a ML between it and another mesh point, it is only necessary to specify the MAC address of its associated neighbor MP. Since the ML is a logical concept, the response to a request for a ML measurement can include one or more associated physical measurements (e.g. one for each associated physical channel).

#### Multi-Hop Measurements

In some specific situations, an MP could require to collect measurements over a whole route or path, involving several MPs in the process. Since all the MPs in the route from the source MP to the destination MP would be required to send the measurements back to the source node, the multi-hop measurements request and report mechanism can be used to perform such measurements in a more efficient manner.

In this case, the source MP sends the multi-hop measurement request to the destination node(s), propagating the request through the MPs in the route. All the requested destination nodes in the route report the requested measurements back to the source MP, and intermediate nodes can also take advantage and collect and use the measurements of other MPs on their own routes or paths.

An example of this process is shown in the following figure.

Figure 7-28: Multi-hop Measurements Example

# Mesh Management

## Routing Architecture

### Overview

Different WLAN mesh networks can have different requirements that influence the choice of mesh routing algorithms and path selection metrics.  Mesh networks deployed inside a building may consists of light-weight APs (in terms of memory use, processing power and battery capacity) deployed in a dense group as wall plugs.  Other mesh nodes may be more capable mesh points supporting a campus infrastructure with forwarding based on QoS features.  A third type of network may utilize mobile mesh points supporting voice traffic.  These different types of networks are driven by conflicting requirements.

One requirement may be to keep the routing as minimal and simple as possible.  A a second conflicting requirement is to allow a flexible and extensible framework with multiple routing protocols, many types of link metrics, and different overlay networks. The proposed routing architecture provides the means to support networks that range from simple to extremely complex.

The routing architecture requires every mesh point to support a very simple “hello” protocol that discovers the mesh neighbors and allows seamless transition to one of two routing protocols.  The two seamless routing protocols defined are: 1) a hybrid link state and 2) a hybrid AODV-Pro-Active protocol; but, additional protocols may be defined.  Each of these protocols must support the “hello” neighbor discovery function as a common sub-protocol.

Another set of conflicting requirements impacts the manner in which routing protocols piggy-back information.  Some networks will want transmission of routing information on beacons or Mesh Probe responses to get quicker establishment of forwarding and routing.  Other networks will want to wait until after mesh association to process routing information in the Mesh Reports or Action Mangement frames. This routing architecture allows networks to select when the transmission of routing information will occur.

The critical neighbor information carried in Beacon, Mesh Probe Response, and Mesh Report messages is passed via the Information Elements supported by the Hello protocol.   Routing protocols (hybrid-link state or Hybrid AODV-proactive) include the hello protocol in addition to topology distribution.

The architecuture supports QoS, radio, and power efficiency aware dynamic routing. It can also seamlessly support both single and multiple radios using the mesh logical link architecture.  Routing security is based on extended IEEE802.11i security mechanisms. Optional extensions support efficient, secure multicast and broadcast.

### The Need for Adopting Different Routing Algorithms for Different Networks

Depending on the main services the mesh network provides, the mesh network may require a routing protocol with capabilities to match its need. For example, low latency and fast convergence are critical for VoIP (voice of IP) service, while efficient multicast forwarding can be more important for a network to distribute video images. Similarly, client station mobility can be critical to some deployments but unnecessary for the others.

In general, potential wireless mesh routing protocols can be categorized as follows.

- On-demand Routing: discovers and maintains routes only when they are needed.  The benefit of on-demand routing is that it does not maintain unused routes.   Hence it reduces the impact of stale routes and overhead due to topology changes.  The cost of on-demand routing is a larger router discovery delay that may require larger data buffer to hold data while the path is set up.

- Proactive Routing: each MP maintains routes to all reachable destinations at all times, whether or not there is current need to deliver data to those destinations.  The benefit of pro-active routing is shorter forwarding latency since the forwarding is set-up ahead of time.  The cost of the pro-active routing is more control traffic and processing overhead to keep the routing information current.

Each of these routing algorithms is used today in the wired world or in IP mobility.  But they must be adapted for the wireless mesh environment. These algorithms have not typically been specified or configured to optimize the conditions of mesh networks. Why is that? Compared to bridging/switching/forwarding on a wired network, forwarding/routing from a Mesh Point has the following characteristics for Unicast forwarding.

- Forwarding a data frame out the same interface the packet is received is normal

- Client stations or Mesh Points are Mobile,  therefore Radio links quality and bandwidth may vary and neighbors may change from time to time

- Client stations, or even MPs, may need to detach to save power

There is a trade-offs between fast detection of link change versus less control traffic and processing overhead.    Unicast routing protocols typically include features or knobs to tune the protocols to differing physical networks.  This specification suggests new features to unicast protocols with additional knobs to tune the protocols for the wireless mesh environment.

Wireless Multicast traffic is also different form the wired world or MANET world.  Compared to multicast forwarding on a wired network, WLAN mesh multicast forwarding has the following differences:

- Wireless multicast forwarding cannot restrict flooding out of the interface from which the data arrived

- In a wired LAN, broadcast packets can reach all nodes via hardware. In WLAN mesh, it can be difficult to ensure the recipients of a broadcast or multicast frame.

- Duplication of multicast packets received by a node should be kept to a minimum.

- Reducing the number of relaying MP is important to reduce the total amount of the broadcast traffic.

The tradeoff on multicast is the number of broadcasting nodes and number of copies of a packet a node may receive. In the wireless mesh, efficient flooding with minimal duplication is a key new feature.

Reduced flooding can occur by using the Minimal Connected Dominate Set of nodes (MCDS) that flood traffic.  The concept behind MCDS for flooding is simple, send traffic from a minimal set of nodes and send ACKs from all receiving nodes.  Duplicate ACKs take less resources and time than duplicate messages.

To provide the full range of quality service, routing protocols must be modified to support independent forwarding paths for Unicast and Multicast MAC level traffic.  We term these modifications as “hybrid” routing protocols.

### Highlights of the Routing Protocols

Three WLAN mesh protocols are proposed in the attempt to maximize performance for different mesh networks: one neighbor discovery protocol and two full routing protocols.  The routing protocols must contain the neighbor discovery protocol as a sub-protocol within the routing protocol.  The neighbor discovery protocol is described in section 8.3.  The two routing protocols are:

- Routing Protocol 1: Hybrid Link State Protocols:  Base on Link State routing algorithm (in pro-active category)with overhead reduction during topology changes

- Routing Protocol #2: Follow the Ad-hoc On-demand Distance Vector (AODV) protocol with the addition of pro-active route establishment capability.

These routing protocols be described as “hybrid” because of the addition of the new sub-protocol for neighbor greeting (the hello protocol), additional features to improve performance, and additional knobs.

#### Routing Protocol #1

The Hybrid Link state algorithm based on ad-hoc routing reduced flooding [MANET OSPF] and modified for MAC address based routing, Multicast OSPF [MOSPF] and OSPF resouce (traffic) engineering extension [OSPF TE].  It also supports

- Efficient handling of wireless station changes via indirect forwarding

- Broadcast/Multicast support via modified link state algorithm: algorithms based on research in IP protocols (Link-State Path Vector) and Multicast OSPF (MOPSF)

- Allowing weighting of nodes or links based on QoS, Radio, Resource, and Power aware metrics

- The highlighted features of this protocol are:

- Fast fail-over for routing convergence to minimuse voice service interruption

- Efficient link state routing information flooding (flooding reduction by using MCDS (Minimum Connected Dominating Set) based algorithm to reduce flooding nodes)

- Calculation of multicast forwarding topology (on-demand based).

The hybrid link-state can also provide a weighting of different portals to wired networks.

#### Routing Protocol # 2

The hybrid AODV and pro-active algorithm starts an On-demand path calculation when data packet arrives first time from souce.  This AODV-base algorithm is augmented with:

- Hybrid proactive route establishment that uses AODV with extensions to pro-actively create routes attached to mesh portal, authentication server, or other frequently used destination, and

- A mechanism to allow weighting of nodes or links based on QoS, Radio, Resource, and Power aware metrics

The highlighted features of this protocol are

- Simultaneously support of on-demand and proactive routing

- On-demand  routes from one MP to another MP based on the established IP-layer Ad-hoc On-demand Distance-Vector protocol [AODV] and modified for MAC address-based routing.

Proactive routing can be used to establish and maintain the route to mesh portal (optional) to reduce the effects of stale routes and overhead due to topology changes.

### Summary

A mesh routing architecture is proposed to allow the adoption of various routing algorithms that can use different path selection metrics. Routing IE is carried in Beacon, Probe Response, and Mesh Report messages. The required fields are:

- Sequence IE

- Routing Protocol IE, which carries routing protocol version, packet type and the type of metrics being carried

And the optional fields are:

- Neighbor’s information, hello timers and the link metrics.

- Wireless station associated with MP (unicast or multicast MACs)

- Forwarding capabilities

- Metrics for resource aware calculations

The architecture supports specific routing protocols through the following:

- MP Routing Report message (encapsulated in Mesh Action frame) can carry

- Protocol specific message on topology

- Default metrics, including resource-aware metric and topology metric

The proposal describes two routing protocols, one is based on link state algorithm with WLAN mesh specific modification, and the other is a hybrid distance vector protocol of combining on-demand and proactive protocols. Both of them calculate the forwarding paths based on MAC addresses and support both unicast and multicast forwarding. Although these two protocols should be able to satisfy most scenarios, the list of supported protocols is extensible for future mesh deployments.

## Forwarding Table Generation

This section describes the method for setting up non-group MAC address forwarding table (FDB). The FDB could be created either statically or dynamically. The static method is optional, but providing the flexibility and alternative of updating the MAC address entry in the FDB.

### Static Setup of FDB

The address table can be set up statically. These entries are considered permanent until they are manually removed, and must not be "aged" or overwritten dynamically.  Before being able to forward the MPDUs (MAC Protocol Data Unit) based on the destination MAC address in the FDB, each routable MAP must have an entry in the FDB to other MAPs in the same Mesh administrative domain.

### Dynamic Setup of FDB

The address table can also be set up dynamically. A MAP discovers entries for its address table from two origins

- The non-routable STA management message with the source and destination MAC address field (SA & DA) should the STA be the non-routable end node

- The forwarding table update from a routing protocol.  The routing protocols send Mesh Point topologies and reachabilty from neighbor Mesh APs (sent in the Hello, Hybrid Link State or Hybrid AODV-PA protocol).

If STA management message from associated neighbor is valid and accepted or a routing table update is processed the MAP adds the tuple into the FDB.  This tuple consists of:

{Destination MAC, Next Hop MPID,Outgoing Interface<MLID>}

Forwarding table entries discovered this way are subject to an aging timer. Once the timer for an entry expires, the entry is aged out and removed from the address table. The timer is reset each time a dynamic update for the same Destination address processed.  If Equal Cost load sharing functions are supported, duplicate destination MAC address with different NextHop MPID and outgoing interface may provide multiple pathways to load share.

For the default table, there must be at most one entry for a destination MAC address. If a discovered destination MAC address entry differs from the previously discovered entry, the new entry takes precedence and the default table entry must be overwritten. The exception is that a discovered entry cannot overwrite a statically created entry.

When a STA associates/de-associates or handoffs, the change is flooded immediately to update the FDB

## Hello Discovery protocol

This section describes the mechanisms for processing the Hello Neighbor Discovery Routing protocol.  The hello information may be carried in the Beacon, the Probe Response, the Mesh Report, and the Mesh Action frame. This neighbor discovery may occur as single hop (the neighbor) and multi-hop (the neighbor’s mesh point neighbor or wireless stations attached to this neighbor.)

The hello routing protocol is a required sub-protocol in all routing protocols.   The hello protocol rapidly discovers mesh points up to 2 hops away. This neighbor discovery creates a circle of knowledge around any mesh point that knows rapidly about mesh points and wireless stations attached to this.  The research into mobile nodes or extremely mobile nodes [Fisheye reference] for small mobile networks (32 nodes or less) indicates that a majority of connectivity falls into this range.  Figure X shows how this circle of neighbor information can reach 80% of a 32 node network.

[Fisheye diagram here ]

One thing to note, the Hello Neighbor Discovery process can run on mesh frames of type 11 which supply additional header information to support multi-hop (hop count, sequence number, address 5 and address 6) or it can run with the routing IE functions sent in an existing mesh beacon, probe response, and mesh report, or an in a mesh action frame.

### Neighbor Discovery- "hello neighbour" Processing

A mesh point may send a beacon, a probe response in response to a probe or a mesh report. These management frames contain information fields (IE) that are used to discovery neighbors a single hop away or multiple hops away. The neighbor discovery protocol creates a localized of the mesh network topology within 1 or 2 hops. This localized knowledge can be utilized to create forwarding entries.

For example, in the diagram below Mesh Point 2, sends information about itself (MP2) and its neighbor MP3 (MP3-NF). MP 1 can then process the information learned from the beacon into a neighbor table and a picture of its local topology. This information is processed into forwarding tables (section 8.2) and provides a kick start for any forwarding/routing in the mesh network.

MP2

MP1

MP3-NF

WSTA

Station is part of Mesh

Beacon MP2: Hello

Neighbor: MP3

WLAN station – Mesh Point – Non-Forwarder

- Figure 8-29: Hello message

- This local knowledge of the mesh topology can include both mesh points and wireless stations as Figure

8-2 shows. Wirelless stations appear as reachable MAC addresses associated with a Mesh Point.

MP2

MAP

MP1

Stations access through MAP

WSTA

1

WSTA

2

WSTA

3

Beacon MP2: hello

MAC: WSTA1,

WSTA2, WSTA3

WLAN stations not MPs

- Figure 8-30: Beacon sending Information about Wireless Stations not in the Mesh

#### Fields Used in Beacon

The beacon carries the the information as specified in table x-x in section 6.1. The table below indicates how the Neighbor Discovery protocol will utilize these fields. In addition, the Neighbor Discovery protocol makes use of the neighbor address fields (address 3 and address 4) within the frame header.

| Information | What used for in Neighbor Discovery |
| --- | --- |
| Timestamp | The timestamp will be used in Neighbor Discovery processing of duplicates |
| Beacon interval | The beacon interval is the default “hello” timer interval. If a hello timer IE is set, the hello timer values will take precedence over the beacon interval. |
| Capability information | The capability information may be used in setting up extended forwarding based. |
| Mesh Capability info. | The Mesh Capability (MP or MAP) or MCF information may be used in setting up extended forwarding. |
| Mesh IE indicator | One byte bitmap for fast processing to identify if there are additional IEs |
| Mesh ID | A hello message is specific to a particular neighbor and Mesh. |
| Supported rate | May be used as a constraint for the creation of forwarding tables. |
| radio parameter set | May be used as a constraint for the creation of forwarding tables. |
| RSN Info element | The RSN ie is used to pass the hints need to obtain the pair-wise keys (PMK, PTK), the Global Keys (GTK), and the neighbor keys (NTK). |
| QoS Capability | May be used as the QoS constraint for the creation of forwarding tables. |
| Resource management (optional) | May be used as the Resource constraint for the creation of forwarding tables. |
| Environment (optional) | May be used as Environmental constraint for the creation of forwarding tables |
| Routing (required) | The Neighbor Discovery process requires this field. |
| Authentication (optional) | The Neighbor Discovery will not process a Beacon’s information if the authentication field is present and the management frame does not pass the authentication check |
| mRSN | The multicast security field contains information for a per Group MAC address specific cipher used in the mesh. The Neighbor Discovery protocol will pass this information to the security process that will obtain the encryption keys for this mesh point. |
| Power saving parameter | The Power saving parameter provides informaton about this node’s power saving status, or needs. A neighbor needing power saving may disappear for a time period that is long than the |

#### Fields Used in Probe Response, Mesh Report, and Mesh Association

The Probe Response carries the the information as described in table x-x in section 6.1. The table below indicates how the Neighbor Discovery protocol will utilize these fields. In addition, the Neighbor Discovery protocol makes use of the address fields. These fields include:

- Transmit address: address 1 and Address 2

- Goblal addresses: Address 3 and Address 4

- Tunnel addresses: Address 5 and Address 6

| Information | What used for in Neighbor Discovery |
| --- | --- |
| Capability information | The capability information may be used in setting up extended forwarding based. |
| Mesh Capability info. | The Mesh Capability (MP or MAP) or MCF information Authentication (optional); The Neighbor Discovery will not process a Beacon’s information if the authentication field is present and the management frame does not pass the authentication check. mRSN; The multicast security field contains information for a per Group MAC address specific cipher used in the mesh. The Neighbor Discovery protocol will pass this information to the security process that will obtain the encryption keys for this mesh point.. Power saving parameter; The Power saving parameter provides informaton about this node’s power saving status, or needs. A mesh point with low savings may be suggested to go-offline for a a while if the rest of the mesh can forward traffic without it.. may be used in setting up extended forwarding. |
| Mesh IE indicator | One byte bitmap for fast processing to identify if there are additional IEs |
| Mesh ID | A hello message is specific to a particular neighbor and Mesh. |
| Supported rate | May be used as a constraint for the creation of forwarding tables. |
| Radio parameter set | May be used as a constraint for the creation of forwarding tables. |
| RSN Info element | The RSN IE is used to pass the hints need to obtain the pair-wise keys (PMK, PTK), the Global Keys (GTK), and the neighbor keys (NTK). |
| QoS Capability | May be used as the QoS constraint for the creation of forwarding tables. |
| Resource management (optional) | May be used as the Resource constraint for the creation of forwarding tables. |
| Environment (optional) | May be used as Environmental constraint for the creation of forwarding tables |
| Routing (required for Neighbor Discovery protocol) | The Neighbor Discovery process requires this field. |

#### Hello Processing Inbound

The hello processing for a routing IE received in a beacon, probe response, mesh report, mesh association request:

- Process the beacon headers and sanity checks the routing information. If any thing is illegal, drop the frame.

- (pre-association only) Check for the authentication field in the Beacon. If it exists, authenticate the frame.

- (post association o only) If encrypted, decrypt the frame.

- Upon receiving a good frame, reset the router dead link timer. Process hello information for direct neighbor (up/down and metrics). Flag information with security level (none, authenticated or encrypted.)

- If security association needs to be established or changed, go through the security association process. If security association fails, drop the mesh link that this hello came in on. If the secure association is valid, set the mesh peer adjacency to flag to security.

Start the topology update process to process neighbor and neighbor’s neighbor. Upon completion of this process, the forwarding tables for frames will be updated.

#### Hello Processing – Originating the Information (Outbound)

The hello processing for a beacon received with the routing IE set is the following:

- Upon initializing, discover what links are valid

- Determine the security parameters for each link & configured neighbors.

  - MD authentication

  - AS Priority

  - RSN IE information

  - Pre-Shared Key

  - Pre-configured Multicast MAC address for pre-Multicast MAC key

  - Neighbor local Multicast key request

- Sending the first hello with the beacon information out all links.

  - Set sequence number for hello information (increment each time the routing information is changed).

  - Attempt to minimize the number of hello message sent by utilizing a multicast transmission if possible based on security

  - If MD5 authentication is used, calculate message digest and put in authentication field.

  - If encryption is used, encrypt the packet prior to sending it the appropriate key. (Pair-wise key, Neighbor key, Group key, or Multicast MAC specific key)

- Set hello interval timer

  - Set hello interval timer per link. At the termination of this timer, send another hello message.

- Set router dead interval timer

  - Set router dead interval timer based on all links. If this timer expires, logically take this link down from the neighbor forwarding logic.

    - If the node has received data from a Wireless station attempting to transmit and this is the only link, continue to try to establish.

    - If the node has received no data, engage in the “hello” back-off process.

  - (pre-association only) Calculate the message digest and send it

If a hello is returned, following hello inbound process.

#### Hello Processing- Precedence of Multiple Streams

The routing IE information may be received on the beacon, the mesh probe response, the Mesh Report Frame, the Mesh Association frame, or the Mesh Action frame with a Routing category.

##### Precedence of Information between different Beacons

The routing IE has a sequence number within the packet to determine the order of routing information set. If no new routing information exists, a beacon may be sent without a new routing IE. The routing IE only needs to be sent each routing “hello” interval, but does not need to be changed. The ordering of processing for beacons is as follows:

- Order the beacon in time sense (based on time stamp).

- If the beacon has the same routing IE sequence number as a previously processed routing ie, reset hello interval and router dead timers.

- If  the beacon has a new routing IE sequenced number, process fields order.

##### Precedence of Information between Beacons and Other streams

- Order the packets based on routing IE sequence number.

- If multiple packets within the same routing IE sequence number

  - Order the beacons in ascending time order

  - Order the rest by frame types and receive

  - Process most recent beacon (if there), or one of the frame types.

- Reset hello timer based on most recent frame (beacon or frame type>)

- Process information within the beacon

##### Implications of precedence

On reception, the neighbor discovery information in packets only needs to be handled if it is fresher (routing ie a sequence number that is later than existing information.

On transmission, the beacon needs to be updated with new routing IE information if something changes. For example, if a wireless station is detected and the mesh point is configured to pass this information in the neighbor discovery – then the MAC addresses are placed in the hello information

### Ack Implication with Neighbor IE Field

The passing of the field for hello sequence id in the neighbor IE field can provide additional information about the current state of the topology. If a mesh point receives a neighbor IE in a hello routing IE section of a beacon or probe response or mesh report with:

- Neighbor Mesh Point ID – as the local node mesh point ID and

- A hello sequence number that is less than the current Hello sequence number.

Then the remote mesh node has not processed a previous hello. Additional System Management Entity (SME) logic may be able to reduce processing on some hellos if the remote mesh point is out of date with this mesh point.

In addition, the sequence number does form an “ACK” of the current sequence numbers sent.

MP 1

MP 2

MP 3

MP 4

Helloigure 7-8: ce

Seq #1,

Ack #1

Hello

Seq #3,

Ack #1

Hello

Seq #4,

Ack #1

Hello Seq #1

MP2, Ack 3

MP3, Ack 0

MP4, Ack 0

Hello Seq #1

MP2, Ack 3

MP3, Ack 0

MP4, Ack 0

Hello Seq #1igure 7-8: ce

igure 7-8: ceMP2, Ack 3

MP3, Ack 0

MP4, Ack 0

- Figure xxx: An example of Hello and Ack packets

## Hybrid Link State Routing Protocol

Unicast link state protocols have four functions: determine neighbor up/down, exchange network topology information, and calculate the pathways.   The existing neighbor up/down has been augmented in the hybrid link state to include all sub-protocol functions of the “hello protocol.”

If network topology changes, the link state protocol exchange information in link state updates. Multicast overlays [PIM-SM, PIM-DM, PIM-SSM] over link state protocols then sends information hop-by-hop to establish multicast forwarding state. Alternatively, Multicast enabled link state (MOSPF) exchanges information about multicast reachabilty to each node.

Link state protocols have consistent fast convergence time under a wide variety of topologies. This quick response to changes minimizes impact to real time services such as VOIP.  AODV is not as fast and can impact real-time traffic. Basic link state routing does not calculate multicast pathways.

The proposed Hybrid Link State provides extensions to a basic link state algorithm with:

- Leverage neighbor discovery protocol [section 8.3],

- reduce the repeat transmission in flooding [OSPF MANET],

- calculate Multicast pathways [MOSPF], and

- Streamline the initial update process.

The Hybrid link state takes advantage of  minimizing bytes exchange over the link and to leveraging the existing neighbor discovery protocol.

### Link State Database Update process

This section describes the initial Link state database update procedure and the mechanisms for flooding a database. When a mesh point link is determined to be up (via neighbor discovery), the mesh point starts an initial database update from its neighbors.

Because wireless mesh nodes may come and go, this initial update procedure has been adjusted to allow mesh points to not just blindly send all data, but to query for the specific pieces of topology data needed to be updated upon a re-establishment. As part of the “hybrid” tuning of a link state, the database update procedure has been tuned to allow for minimal exchange of data upon re-establishment of links. This hybrid tuning takes the best from the database update procedure from ISIS/OSPF and tunes the database update procedure to minimize bytes transmitted across wireless links. After the initial database is updated from its mesh peer, the mesh peer participates in flooding changes to all nodes. Again, this hybrid proposes reduced flooding mechanisms that will reduce the number of times a particular LSA is transmitted over a set of links. This hybrid link state takes the best from advances in link state algorithms:

- flooding reductions based on Minimally Connected Dominate sets (MCDS)) being utilized by OSPF with MANET changes, and

- Compression of repeat information in reachabilty (MAC addresses) based on network component algorithms.

#### Initial Database Exchange

The following three diagrams indicate exchange messages that occur to bring MP2 and MP3 up. It is assumed that both MP2 and MP3 have additional mesh points attached to them.

The sequence of logic is described here and indicated in the diagrams below. The first section of the diagram indicates how the MD5 keys can protect the extended mesh beacon containing the neighbor discovery protocol. After the neighbor has been discovered, additional information may be passed in the Probe Response or the Mesh Report. In the description below, steps a) to g) steps are described sequentially for ease of reading. In reality, the requests (a-b, e-f, and c-d, g-h make take place in parallel).

The update of the database starts with the sequence a,b,c,d and in the diagram.

- MP 3 request the DB from MP2

  - Topology database is default (not passed)

  - Requestor is MP3 (implied from packet)

  - Request # is 1

  - Flags are:

    - S = 0 - Sequence number IE not included in packet

    - R = 1 - Request for information on this node

    - P = 0 - not partial request

    - T = 0 - do not transmit

- MP 2 responds to this database request with a list of LSAs

  - Topology database (default) not passed

  - Requestor MPe (implied from packet)

  - Request is #1

  - Flags are:

    - S – 1 Sequence # IE included in packet

    - R – 0 – Response to request of DB

    - P – 0 – Not a partial request

    - T = 0 – Echo of the transmit flag

  - Sequence number IE

    - Start LSA Sequence #1 – start of LSA sequence number

    - End LSA Sequence # - end of sequence number

    - Lifecnt = 1

  - Lifetime IE

    - Time = 100 counts

    - Cnt: 3

    - LSA 1,2,3

- MP2 requests topology database information from MP3

  - Topology id = 0 (default – so not sent in packet)

  - Requestor = MP2 (implied from packet header)

  - Last Request = 0 	– no earlier requests

  - Request = 1 	 - first request

  - Flags

    - S = 0 – Sequence Number (SN) IE not included

    - R = 1 – Response to Request of DB request

    - P = 0 - Not a partial request

    - T = 0 - Do not transmit, just exchange sequence numbers

- MP 3 responds to the DB request with a list of LSAs

  - Topology id = 0 (default – not sent in packet)

  - Requestor: MPID 2

  - Last request: 0 	- no earlier request

  - Request = 1	- first request

  - Flags:

    - S = 1 – Sequence number IE included

    - R = 0 – Reply

    - P = 0 – Not partial request

    - T = 0 – Transmit flag echo

  - SN IE

    - Start LSA sequence # 1 – start of LSA Sequence

    - End LSA sequence # 1 – end of LSA sequence number

    - Life time – count of lifetime blocks

    - Life-time IE

      - Time: 100 counts

      - Cnt 3

      - LSAs: 1

- MP3 has received all the DB sequence numbers form MP2, it now request the database it needs from MP2

  - Topology id = 0 (default – not sent in packet)

  - Requestor: MP3 (not sent in packet, implied)

  - Last Request: 1 	– earlier request to query database

  - Request: 2	- current request number

  - Flags

    - S = 1 – Sequence number IE included

    - R = 1 – Request

    - T = 1 – Transmit

  - SN IE

    - Start LSA sequence # 1

    - End LSA sequence #3

- MP2 responds to the DB request with LSA announcement with LSAs 1-3

- MP2 – now requests the DB form MP3 (in parallel)

  - Topology id (default – not passed)

  - Requestor: MP2 (implied in message)

  - Last request: 1

  - Current request: 2

  - Flags:

    - S = 1 – SN IE attached

    - R = 1 – Request

    - T = 1 – Transmit

Diagram 4 gives a second example of database update that has the following steps:

- MP2, MP3, MP4 come upon MP1

  - MP1 sends DB request to MP2

    - Flags: T(0),R(1),P(0)

    - Receives response (in green) that MP2 originates LSAs 1-4, 6

  - MP1 Sends DB request (0) to MP3

    - Flags: T(0), R(1), P(0)

    - Receives response (in green) that MP3 originates LSA 4

  - MP1 sends dB request (0) to MP4

    - Flags: T(0), R(1), P(0)

    - Receives response in green that MP4 has 1 LSA #7

- MP1

  - Requests LSA 1-3, 5 originated from MP3,

  - Requests LSA 4,6 from MP2

  - Request LSA 7 from MP4

- MP2: sends 1-3,5

- MP3: sends 4-6

- MP4: sends LSA 7

- Topology message sent out from MP1 to MP2, MP3, MP4

  - Acks LSA 1-7

  - Originates with LSAs 8-10

- MP2, MP3, MP4 send topology message

  - Ack 8-10 LSA

One last note, if during this time period, any MP does not receive the LSA it requests, it will re-request the LSA, and the remote site will retransmit. Optionally, on the sending side the MP can resend an LSA if does not receive an acknowledgement.

#### Flooding LSA

The hybrid functions allow optional reduction from simple flooding. The use of this alternate flooding mechanism is signaled:

- In the Neighbor discovery packet in the Routing capabilities or in Reduced flooding flags,

- In the DB update request/response packet in the flags field, and

- In the LSA passed regarding a neighbor.

If all neighbors of a MP signal reduced flooding, then the reduced flooding algorithms are used. Initial exchanges will default to simple flooding.

##### Security issues

The basic link state flooding, like management all frames, are encrypted. This encryption may be via the local multicast encryption (NTK) or the global Multicast encryption (GTK).

##### When does flooding occur

The Database flooding may occur after the following events:

- Neighbor discovery protocol indicates a change in the neighbor information,

- Initial discovery of a mesh peer causes an initial database update form neighbor,

The initial DB requests come first from the MP with the lowest MPID. The node with the higher MPID sends first. If additional topology databases for QoS or Resource aware, or Power aware nodes are configured to be created, these nodes may use a different topology database. This topology database can be identified by a Topology id.

For example, if the power saving function wanted to calculate a topology database and associated functions for all mesh points with over 80% power, then this alternate database could keep track of that database. If the mesh point radios scheduled a power reduction cycle, the forwarding database could be used for low power cycles.

##### Simple Flooding of LSA

Simple flooding

- Flood Link State topology packets out the interface that is not the incoming interface of the LSA

- Increment Sequence number each time new packet occurs

- Send Acks back to node received Topology with Range & list format when ranges won’t due.

- If  the aging flag is set, set timer on all routes created from the simple flooding.

The simple flooding algorithm retransmits packets based on the ACK field from the neighbor. These retransmissions can be scheduled based on a retransmission timer. Normally the transmission of ACK fields should occur at 1/3 of the retransmission timer.

The diagram below indicates simple flooding

- Figure 8-31: Example of Simple Flooding and Sending of LSA

##### Reduced Flooding of LSAs

Reduction in flooding of LSAs can occur by:

- 1st - Flooding all nodes during initial updates

- 2nd – Detect that reduced flooding is available through-out the network

  - Each node then calculates

    - Identify Minimally Connected Dominate Sets (MCDS) nodes

    - If not a node with MDCS, delay sending flooding by delay-MCDS timer and send ACK of received data

    - If a node with MDCS or data delay is inactive, flood data to other nodes

##### Reduced Flooding of LSA Information

Additional reduction of data can be accomplished by compressing updates. An example of compression of update information is the compression of reachable MAC addresses – Unicast or Multicast. If this reduction is used, the following additional IEs can be supported in the Neighbor Discovery and LSA packet.

??? What IEs???

### SPF Algorithm

SPF algorithms for default mesh point topology calculations derive shortest path calculations for:

- Unicast SPF forwarding , and

- Multiacast SPF forwarding.

In addition, constraint based topology calculations can be done any number of metrics passed in the topology information found in the neighbor discovery protocol and the link state announcements.

#### Unicast SPF Algorithm

The Unicast Default SPF calculates the paths based on the least cost metric. The 2-byte metric used is the combination of resource-aware metric and topology metric. The topology metric flag indicates which of the two portions is in the higher byte. The default is that resource-aware metric is in the high byte, and topology metric is in the low byte.

If unicast MAC are associated with a MP, the SPF calculation can be extended to calculate the forwarding path to those Unicast MAC addresses as reachable addresses.

A unicast constraint based SPF calculation has the following parts:

- Use the topology database information that the constration indicates.

- If uploading TE information from the default database:

  - Pre-Select the nodes in the topology base that will be used

  - Remove all noides & links from the available list that do not support TE processing,

  - Remove all nodes that do not meeting the constraints,

- Run the SPF with the remaining nodes using the constraint metric selected, and

- Flood the reduce information using the DB ID

- The following diagram shows the basic link state calculation

- Figure 8-32: Link State Calculation

#### Multicast SPF Algorithms

The Multicast Default SPF calculates the multicast tree paths to:

- All nodes for broadcast flooding, and

- To all nodes announcing a Group Multicast MAC address.

The multicast SPF method per multicast group:

- Remove all nodes that will not forward multicast traffic,

- Remove all end-nodes except nodes reporting the specific multicast grouip address,

- Calculate the shortest path from each node to all other nodes reporting the MAC address,

  - Count each time a path is used on the shortest path.

- Tie break on Equal Cost Multi-paths for multicast distribution:

  - Elect a group leader for the group,

  - Add weight to all other Equal costs (topology metrics) and announce

- The diagram below shows this algorithm

- Figure 8-33: Joining a Multicast Group

Drops the MAC address from Node A’s announcement

F

F

A

B

C

Group Leader

F

Multicast tree after re-calculation

- Figure 8-34: Leaving a Multicast Group

- A leaf mesh point revokes its member status

  - A drops MAC address from announcement

  - B forward topology message with dropped MAC address

  - All needs to re-calculate the multicast

    - Result with trim node

- Figure 8-35: Broken Tree Repair

When a link break occurs, the mesh point associated with the link announces the link down.

- Each Node re-calculates the shortest path to all MPs having Group MAC addresses associated

  - If all destinations can be reached, determine the best usege links

  - Set metrics higher on best usage links

#### Link State and Security Algorithms

The Default encryption required for management control message is:

- PTK keys for the pair-wise mesh-point peer connections, and

- GTK for any multicast or broadcast flooding

Optionally, the NTK supports local multicast from a neighbor (broadcast and multicast MACs).

The MTK supports global multicast MAC Address.

If the MTK is supported, a pre-configured MAC address (ALL-MESH-POINT-LS-Routers) can be used to flood LSA announcements to.

## Hybrid On-Demand and Pro-Active protocol

The hybrid mesh routing protocol described in this section simultaneously supports on-demand routing and proactive routing, which combines the advantages of these two types of routing mechanisms. The on-demand unicast and multicast routing is based upon Ad hoc On-Demand Distance Vector (AODV) Routing protocol [xx] and multicast AODV protocol [xxx] with modifications for support of MAC address-based routing and radio/MAC-aware metrics. Furthermore it also allows proactively establishing and maintaining the route to mesh portals (or other special MPs) in he mesh network, which reduces the route discovery delay and diminishes the routing overhead generated if every MP discover the mesh portal individually. The protocol supports both unicast and multicast routing. It has the following main features

- Simultaneously support the on-demand routing and proactive routing by combining the advantages of these two types of routing mechanisms. The on-demand routing is based upon Ad hoc On-Demand Distance Vector (AODV) Routing protocol. Proactive establishment and maintenance of routes to the mesh portals (or other special MPs) may be performed through route announcement to reduce the route discovery delay and the routing overhead in many-to-one communications.

- Support for not only unicast routing but also efficient multicast routing

- Support for alternative path selection metrics including PER, latency, link rate, available link capacity, available MP battery power, power consumption, and other PHY/MAC/power-aware metrics.

- Support for non-forwarding MPs

- Mesh APs acts as a proxy for its associated stations to generate and respond the routing messages on their behalf

- Route request and route announcement for multiple destinations can be aggregated in one RREQ message or one RANN messages to reduce the routing overhead.

- Support for Quality of Service by specifying the QoS requirements in the RREQ messages.

### On-demand Unicast Path Estrablishment

The on-demand routing mechanism in the hybrid mesh routing protocol uses Route Request and Route Reply messages, which is based on the AODV protocol [AODV]. For on-demand routing, when a source MP wants to send a frame to some destination MP, it checks its routing table for a route. If there is a valid route, it forwards the frame to the next hop specified in the routing table for this destination. If there is no valid route, it initiates a route discovery by broadcasting a Route Request (RREQ) message as shown in Fig. 1a. The RREQ message contains the originator IEEE 802.11 MAC address (not the IP address) with its sequence number and optional layer 3 information as well as the destination MP MAC address with the last known destination sequence number and optional layer 3 information for this destination. It also contains the message ID, the hop count, the routing metric, the flags, the time-to-live (TTL) and the route lifetime.

Once a MP receives a RREQ message, it checks the originator address and message ID to see if it has seen this RREQ message before. If this is the first RREQ message, the MP updates the metric field by adding the metric between the MP from which it received RREQ message and itself and then establishes a reverse route to this originator in its routing table. If the MP is the destination MP or if the MP has an unexpired valid route to the destination and the sequence number for that destination is at least as great as that indicated in the RREQ message, it responds by unicasting a Route Reply (RREP) message back to the originator. Otherwise, the MP propagates the RREQ message with the new metric. If this is not the first RREQ message, it updates the metric field to the originator by adding the link metric between the MP from which it received RREQ message and itself. The MP updates the reverse route if the new metric is better than the metric recorded in its routing table. Otherwise it discards the RREQ message. The MP replies with an RREP message to the originator if it meets the requirements as described above. Otherwise it propagates the RREQ message with the new reverse route metric. The reverse path/route is used for sending the RREP message back to the originator of the RREQ message to establish the forward path and also for bi-directional communications between the source MP and the destination MP.

The RREP message is sent back to the originator of the RREQ message by unicast to establish the forward path as shown in Fig. 1B. It contains the originator MAC address, the destination MAC address, destination sequence number and optional layer 3 information of this destination, the hop count, the routing metric, the flags, the time-to-live and the route lifetime. If the destination MP responds, it uses the maximum of its current sequence number and the destination sequence number in the RREQ message. The initial value of the metric is zero. The destination also sets the lifetime of the route. If the intermediate MP responds, it uses its record of the destination's sequence number and the metric, as well as the calculated route lifetime based on the route table entry.

The RREP message is unicast through the reverse route established during the route request broadcast. When an intermediate MP receives the RREP message, it updates the metric by adding the link metric between the MP from which it received the RREP message and itself. It sets up a forward path in its routing table and forwards the RREP message to the originator of the RREQ message. If a MP receives more than one RREP message, it forwards the first RREP message. It updates the routing table and forwards the new RREP message only if the new RREP message contains a greater destination sequence number or the same destination number with a better metric. Otherwise it discards the new RREP message. The originator can commence the data frame transmission as soon as the first RREP message is received and can update its routing information later if a better route is discovered.

A MP may have multiple IEEE 802.11 wireless interfaces. The MP has a unique MP identifier, i.e. a MP IEEE 802.11 MAC address, and each interface also has its own IEEE 802.11 MAC address. The MP MAC address is used in the RREQ and RREP messages as well as the other routing control messages described below. When a multi-interface MP multicasts the RREQ message, it may multicast the RREQ message on all its interfaces. When a multi-interface MP responds to a RREQ message by unicasting a RREP message, it sends the RREP message to the interface on which it received the RREQ message

The routing table includes an entry for a destination MP. Each entry contains the destination MAC address and its optional layer 3 information (supported layer 3 protocol and address, e.g. the IP address of destination MP), the destination sequence number, the next hop MAC address and the interface to reach the next hop, a list of the upstream MPs and interfaces using this route, the state and routing flags (e.g. valid, invalid), the metric to the destination, and the route lifetime. The route lifetime is updated each time it is used. The route becomes invalid if it is not used within the route lifetime. The invalid route will be deleted after its deletion timer expires. The originator can start data transmission as soon as the first RREP message is received and can later update its routing information if a better route is discovered. When an intermediate MP receives a data frame, it checks its routing table based on the destination MAC address. If there is a valid entry for this destination, it forwards the frame to the next hop specified in this routing entry. This process continues until the destination MP is reached.

Source

Destination

Reverse Route

RREQ flooding

- Flooding of the route request (RREQ) message and reverse path establishment.

Source

Destination

Forward Route

RREP unicast

- Figure 8-36: On Demand Unicast Route Establishment

### Proactive Path Establishment

In the on-demand routing, only currently used routes are maintained. This reduces the routing overhead. However it results in extra delay because the source MP must establish the route before it can send the data. The source MP also needs to buffer the data during the route discovery period, especially a mesh AP is required to buffer the data frames originated by its associated stations. This may result in a large buffer requirement and delay. To reduce the route discovery delay, buffer requirement, proactive routing can be used. Furthermore, in the case of many MPs communicating with a special MP, e.g. a mesh portal, significant routing control overhead may be required if each of these MPs individually discovers the route to this special MP. For example, many MPs in a mesh network access the Internet or other networks through one or more portal MP that connects the mesh network to the Internet or other networks. It is desirable that the portal MP proactively announces the routes to it in the mesh network. This proposal integrates the on-demand route discovery and proactive route announcement.

A MP can be explicitly configured by the network administrator or implicitly determines according to certain policies, to perform proactive routing in the mesh network. For example, one policy is that all the mesh portals shall perform the proactive route announcement. Referring to Fig. 9-10A, the mesh portal advertises itself by periodically broadcasting an unsolicited Route Announcement (RANN) message so that other MPs in the mesh network can learn the route to the RANN message originator. That is, the mesh point that originates the RANN message floods the mesh network with unsolicited RANN messages in order to proactively establish routes to itself. When a multi-interface MP broadcasts the RANN message, it may broadcast the RANN message on all its interfaces. The RANN message contains the IEEE 802.11 MAC address of the originator MP with its destination sequence number and optional layer 3 information. It also contains the hop count, the route metric, the flags, the time-to-live and the route lifetime.

When an intermediate mesh point receives a RANN message, it updates the metric field to the originator of the RANN message by adding the link metric between the MP from which it received the RANN message and itself. If the MP does not have a valid route to this destination (i.e. the RANN message originator) in its routing table, it creates the route to this destination MP in its routing table. The MP broadcasts the RANN message with the new metric to its neighbors on one or more of its interfaces. When it has a valid route to this destination, the MP updates its routing table and broadcasts the RANN message with the new metric to its neighbors only if the RANN message contains a greater destination sequence number or the same destination sequence number with a better metric. Otherwise it discards the RANN message. In this way, the routes to the RANN message originator are established in the mesh network.

If bi-directional route flag is set in the RANN message, the MP should send a gratuitous RREP in unicast to the mesh portal. The gratuitous RREP is routed to the mesh portal through the forward path, which establishes a reverse path to the RREP originator.

When a source MP wants to send data frames to a destination MP, it may already have a forward path to this destination MP obtained from the destination MP's route announcement. In this instance, it can transmit the frame immediately. However it is possible that there is no reverse route from destination MP to source MP. If bi-directional communications are needed, source MP can send a gratuitous RREP message in unicast to destination MP through intermediate MPs along the forward path established by the destination MP's RANN message. The RREP message establishes a reverse route to source MP.

Figure 9-10A. A mesh portal initiates flooding of the route announcement (RANN) messages to proactively establish the routes to it in the mesh network.

Reverse Route to the Source

Gratuitous RREP unicast

Figure 9-10B. A mesh point unicasts a gratuitous route reply (RREP) to the originator of the RANN (the destination) for establishing a reverse route to it.

- Figure 8-37: Proactive Routing

### Route Maintenance

If a link breaks, the Route Error (RERR) message is sent to the affected source MPs of the active paths. The upstream MP of the broken link, i.e. the MP near the source, initiates the RERR message. Before sending the RERR message, it also marks the damaged routes invalid, sets the metric of the damaged routes to infinite, and increments the destination sequence numbers of the unreachable destinations due to this link failure in the routing table. The RERR message contains a list of all the unreachable destinations due to this link failure and their incremented sequence numbers. It broadcasts the RERR message to its one or more upstream neighbors. For a multi-interface MP, it sends the RERR message on the interfaces with the routes using this failed link. When a neighbor receives a RERR message from its downstream MP, it checks whether it has a route to use this downstream neighbor to the listed destinations. If so, it marks these routes as invalid and sets the metric of these routes to infinite. It then propagates the RERR message to its upstream MPs. When a source MP receives the RERR message, it re-initiates the route discovery. If a MP receives a data frame with a destination MAC address that does not have an active/valid route, the MP creates a RERR message for the destination MP and multicasts the RERR message to its upstream neighbors. Local connectivity management is accomplished by the MPs periodically sending beacons (HELLO messages) to their neighbors as describe in section 8.2.

New data

Setup

route

Forward RREQ

Not forward

RANN

Has a

forward route?

Send RREQ

Non-forwarding

node?

Forward RERR

Invalidate Route

Send gratuitous

RREP

Transmit data

N

Y

N

Non-forwarding

node?

N

Y

Reverse route

for bi-directional

communications?

Y

Discard

RREQ

Destination?

Y

Destination or

valid route to

destination?

N

Proactive?

Start

/Idle

Y

Send RANN

periodically

Setup reverse

route

Receive

RREP?

Receive

RANN

Forward

RANN

Receive RREQ

Non-forwarding

node?

N

Send RREP

Detect link failure

Or receive RERR

Not forward RERR

Setup

forward

route

Destination or

Non-forwarding

node?

Forward RREP

Setup route

Receive RREP

Not forward RREP

N

Source?

Y

N

Y

Rediscover a route

Y

N

Y

N

Y

N

N

Y

Retry limit?

Y

N

Error

Figure 8-38: the method whereby a MP establishes the routes on-demand and proactively, and processes the routing control messages.

### Support of Quality of Service via Contraint-Based Routing

It is necessary to support the quality of service (QoS) in WLAN mesh networks, e.g. for multimedia applications. The QoS requirements and/or power requirements, for example, maximum delay, and/or minimum bandwidth, and/or minimum battery power requirements for a data flow can be carried in the optional fields of an extended RREQ message. To respond or forward a RREQ message with QoS extensions, a MP must be able to satisfy the QoS constraints. Otherwise, it discards this QoS RREQ message. After the QoS route is established, if any MP along the path detects that it no longer satisfies the requested QoS parameters, it sends a RERR message to the originator. The RERR message may also carry the currently measured QoS parameters for example available bandwidth and delay parameter for this link. The originator may decide to continue use this route with lower QoS or discover another route. For example, the RERR message indicates that the current bandwidth available on a link is equal to a value less than that requested by the originator before. The originator may reduce its source rate to meet the current available bandwidth or discover a new route with the originally required bandwidth.

Mesh AP

with

Proxy

Station

WLAN Mesh Network

Infrastructure

Based WLAN

- Figure 8-39: Mesh AP with Multiple Assocaited Stations

### Support for Non-Forwarding MP

Some MPs want to join the wireless local area mesh network only as the source MP or destination MP, i.e. not forwarding the traffic originated from other MPs due to battery, processing power and/or memory limitation. A MP can be configured as a non-forwarding MP by the administrator or is determined to be a non-forwarding MP based on certain policies. For example, one such policy is that if the energy in the battery of the MP is less than a threshold, the MP will become a non-forwarding MP. A non-forwarding mesh MP sends the RREQ message when it wants to transmit frames. It replies to the route request message only if it is the destination MP in the received RREQ message. The non-forwarding MP does not reply to the route discover message (RREQ) if it is not the destination MP in the RREQ message. It receives the RANN message to learn to the route to the originator of the RANN message. It sends the RANN message so the route destined for it can be proactively established. However, the non-forwarding mesh MP does not forward any routing control messages, including RREQ, RREP, and RANN messages, to its neighbors. By so doing, there is no route using it as an intermediate MP.

### Support for IEEE 802.11 Stations

Multiple stations may associate with an access point (AP) in a WLAN. As shown in Fig. 4, the mesh AP joins the mesh network. However the IEEE 802.11 stations are not part of the mesh network but are associated with the mesh AP. The mesh AP acts as a proxy of these stations and routing is transparent to the non-mesh stations. The mesh AP discovers the route to the destination when it forwards data frames originated by an associated station. It also responds to a RREQ message for its associated stations by unicasting a RREP message to the RREQ message originator. The mesh AP announces the route to its associated stations by broadcasting the RANN message. Multiple destination addresses with their individual sequence numbers, optional layer 3 information can be requested using a RREQ message and can be announced using a single RANN message.

When a STA moves from one MAP to another, the new MAP rediscovers the route to the destination by sending a RREQ. The old MAP would respond the RREQ with RREP since it has a route to the destination.The new MAP would use this route to send the data frames to the destination and the old MAP would use this route to forward the data frames destined for the moved STA to the new MAP. A better route may be discovered between the new MAP and the destination later, the new MAP would be switched to the better route if this happens.

### Interaction between ARP and Route Establishment

Before sending an ARP request, a source MP may optionally send a RANN so that the route to it is established. Route announcement information may be optionally piggybacked with ARP request broadcast. Before sending an ARP reply, a target MP may optionally send a gratuitous RREP so that the route to the target MP is established.

### Multicast Routing

The same protocol supports both multicast and unicast routing. A separate multicast routing table is maintained by a MP. Multicast on-demand route discovery is based on Route Request and Route Reply messages, which is similar to the multicast AODV. A MP can dynamically join or leave a multicast group at any time. Each multicast group has a multicast group leader. The group leader maintains the multicast group sequence number. Upon detecting the failure of the multicast group leader a new group leader is created so that there is no central point of failure.

When a MP wants to join a multicast group, the MP broadcasts a RREQ message to all mesh MPs. The RREQ message includes the originator's MAC address, its current sequence number, its optional layer 3 information (e.g. IP address), the destination MAC address (i.e. multicast group address to be joined), the group's last known sequence number, the message ID, hop count, metric, time-to-live parameter and a join flag. Any member of the multicast tree may respond to the RREQ message but only members of the multicast tree may respond. A non-member MP (not a member of the multicast tree) does not respond to the RREQ message, but creates a reverse route entry to the originator in its unicast routing table per unicast route establishment rules. The non-member MP then forwards the RREQ message to its neighbors. The originator waits for the term of the discovery period to receive a reply or replies. If there is not reply then the originator retransmits/re-broadcasts the RREQ message with the message ID incremented by 1. The originator continues in this manner until it receives a reply or the retry limit has been exceeded. If no reply is received after the maximum number of retries, then the originator may become the multicast group leader for a new multicast group. Figs. 5A - 5D show how a new MP "N" joins a multicast group.

When a MP receives a request to join a multicast group from a MP (not in the group), it updates the metric field and adds an inactive entry for the originator in its multicast routing table. Each entry in the multicast routing table has a flag indicating whether a link is active or inactive. No frames will be forwarded/transmitted over an inactive link for the multicast group.

A multicast tree includes MPs that are members of the multicast group and forwarding MPs for the multicast group. A forwarding MP is a MP that is a member of the multicast tree but not a member of the multicast group. A forwarding MP acts as a "pass-through" for the datframes that it receives. It does not use the frames that it receives. Any member of the multicast tree may reply to a RREQ message to join the multicast group, if its recorded sequence number is at least as great as that carried in the RREQ message. The multicast group leader always responds to a RREQ message to join the multicast group. The MP responding to a RREQ message to join the multicast group sets up a unicast reverse route to the originator in its routing table per the unicast route establishment rules. The responding MP also sets up a route for the originator in the multicast routing table. The multicast route is flagged as inactive. The responding MP then unicasts a RREP message back to the originator of the RREQ message. As MPs along the reverse path receive the RREP message, they update the metric and set up a forward route in their multicast routing table. The route set up in the multicast routing table is flagged as inactive. The responding MP then forwards the RREP message to the next hop.

- Figure 8-40: A node joins the multicast group

Each MP in a multicast tree has a multicast routing table. A multicast routing table has entries for each multicast group. A multicast routing table entry includes the multicast group MAC address, the next hop MAC address, the interface to the next hop, the multicast group sequence number, the hop count and metric to the multicast group leader, flags (active/inactive and routing flags) and a route lifetime parameter. The route lifetime parameter is updated each time that a route is used. The route becomes invalid if it is not used within the specified route lifetime.

The originator of a RREQ message to join a multicast group may receive multiple RREP messages from different members of the multicast tree. Each reply represents a potential route to the multicast group. The originator tracks the replies received and waits for the expiration of the route discovery period. The originator then selects a route having the greatest multicast group sequence number and the best metric to the multicast group. The originator activates the selected route at the expiration of the route discover period by unicasting a multicast activation (MACT) message with the active/inactive flag set to active to its next hop. The originator sets the active/inactive flag to active for the selected route in its multicast routing table. As each hop along the path receives the MACT message, it activates the route in its multicast routing table and forwards it to its next hop if it is not the originator of the RREP message. This continues until the originator of the RREP message receives the MACT message.

- Figure 8-41: A node leaves Multicast group

If a MP that is a member of the multicast group wishes to exit/depart from the group, then it unicasts a MACT message to its next hop with the prune flag set to prune and deletes the entry to the multicast group in its multicast routing table. Once the next MP along the path receives the MACT message with the prune flag set to prune, it deletes the routing information for the MP that transmitted the MACT message to it. If the MP receiving the MACT message with the prune flag set to prune is not a member of the multicast group and with the deletion of the MP desiring to relinquish its membership in the multicast group becomes a leaf MP, then it prunes itself from the multicast tree. The leaf MP prunes itself from the multicast tree by unicasting a MACT message with the prune flag set to prune to its next hop. If the MP receiving the MACT message with the prune flag set to prune is a member of the multicast group or is not a leaf MP then it does not prune itself.

Figs. 6A - 6B show how a leaf member MP "A" in the multicast group leaves a multicast group. MP "A" unicasts a MACT message with the prune flag set to prune in order to relinquish its membership in the multicast group. Fig. 8B shows the multicast tree after pruning. After MP "A" relinquished its membership from the multicast group and tree, MP "B" which was a forwarding MP was left as a leaf MP so it pruned itself from the multicast tree.

A MP on the multicast tree must receive a transmission from each of its neighbors every Hello_interval. A transmission includes a multicast data frame, a RREQ message, a Hello_message, a beacon message or a group hello (GRPH) message. A GRPH message is sent periodically by the group leader through the mesh network. If a MP on the multiast tree fails to receive any transmissions from a neighbor on the multicast tree for Hello_Life then the link to this neighbor is broken. When a link is broken, the MP downstream of the break (i.e., the MP farther from the multicast group leader) attempts to repair it. Actually, it is an attempt to bypass the broken link and generate an alternate path back into the multicast tree. The downstream MP that is responsible for repairing the broken link discovers an alternate route by sending a RREQ message to join the multicast group. The RREQ message includes an extension field indicating the sending MP's metric from the group leader. MPs responding to the RREQ message must be members of the multicast tree with a fresh enough sequence number (a sequence number that is at least as great as the multicast group sequence number carried in the RREQ message) and the metric to the multicast group leader must be better than the metric indicated in the RREQ message. At the expiration of the route discovery period, the MP that originated the RREQ message in an attempt to bypass the broken link, selects a route and unicasts a MACT message to its next hop to activate the newly discovered route. If it is not possible to repair the multicast tree by rejoining the tree through any branch then the downstream MP responsible for bypassing the broken link becomes the new multicast group leader for a new multicast tree. Figs. 7A - 7E illustrate reparing a broken multicast tree link. Fig. 7A illustrates a multicast tree with a broken link. In this case the link between MP "A" and MP "B" is broken. Fig. 7B depicts the downstream MP (MP "A") attempting to bypass the broken link by sending out a RREQ message requesting to join the multicast group. Fig. 7C shows the downstream MP (MP "A") receiving RREP messages from qualified multicast tree members. Fig. 7D shows the downstream MP (MP "A") activating the new links with a MACT message with the activation flag set. Fig. 7E depicts the repaired multicast tree with the broken link bypassed.

- Figure 8-42: Repair of broken tree link

## Mesh Interworking

As mentioned earlier on, the existing IEEE 802.1D specification defines the interworking framework and service access interface across 802 standards that supports 802.11 portal functions. It is important for the 802.11s specification to comply with such framework in order to interwork with other layer 2 networks (e.g. 802.11s WLAN Mesh, 802.11 WLAN, 802.15 WPAN, 802.16 WMAN, 802.3 LAN, etc.), as well as with higher layer protocols. Hence, in this section we describe the way the WLAN Mesh interworks with other networks.

### Background

In “section 6.2 - Compatibility” of the Five Criteria document specified for 802.11 TGs [x], the following standard requirements for WLAN Mesh protocol specification are stated:

- IEEE 802 defines a family of standards. All standards shall be in conformance with the IEEE 80.21 Architecture, Management and Interworking documents as follows: 802 Overview and Architecture, 802.1D, 802.1Q and parts of 802.1F. If any variances in conformance emerge, they shall be thoroughly disclosed and reviewed with 802. Each standard in the IEEE 802 family of standards shall include a definition of managed objects, which are compatible with systems management standards.

- ESS Mesh specifies one possible Wireless Distribution System (WDS) that behaves in every respect as an IEEE 802.11 Infrastructure Mode network. As such, it is entirely compatible with the IEEE 802.11 architecture and, by inference, compatible with the IEEE 802 architecture, including IEEE 802.1D, IEEE 802.1Q and IEEE 802.1F.

IEEE Std 802.1D-2004, section 1.1, as highlighted in the following, specifies the bridging function that is designed to interconnect various IEEE 802 networks.

“IEEE 802 Local Area Networks (or LANs; see 3.4) of all types can be connected together using MAC Bridges. The Bridge Local Area Network created allows the Interconnection of stations as if they were attached to separate LANs each with its own independent MAC. A MAC Bridge operates below the MAC Service Boundary, and is transparent to protocols operating above this boundary, in the Logical Link Control (LLC) sublayer or Network Layer (ISO/IEC 7498-1: 1994).”

IEEE 802 Local Area Networks (LANs) of all types can be connected together with Media Access Control (MAC) Bridges, as specified in IEEE Std 802.1D, 2004 Edition. Whereas, IEEE Std 802.1Q defines the operation of Virtual LAN (VLAN) Bridges that permit the definition, operation and administration of VLAN topologies including the transport prioritization tagging within a Bridge LAN infrastructure to support quality of services.

To relay frame between different MAC methods, IEEE Std 802.1D-2004 specifies the MAC Internal Sublayer Service (ISS) which is provided by individual LANs to the MAC Independent Function that is part of the MAC frame relay entity of the 802.1D Bridge.

IEEE Std. 802.1D-2004, section 6.4 describes the ISS function provided within the MAC Bridge.

“The Internal Sublayer Service (ISS) provided by a MAC entity to the MAC Relay Entity within a Bridge is that provided by the individual MAC for the LAN Port. This observes the appropriate MAC procedures and protocol for the LAN to which it attaches. No control frames, i.e. frames that do not convey MAC user data, are forwarded on any LAN other than that on which they originated.

The Internal Sublayer Service is derived from the MAC Service defined by ISO/IEC 15802-1 by augmenting that specification with elements necessary to the performance of the relay function…. Two parameters are added to the list of parameters associated with the MA_UNITDAT.request and MA_UNITDATA.Indication primitives defined by ISO/IEC 15802-1. These are frame_type and frame_check_sequence. The definition of the Internal Sublayer Service does NOT add any new service primitives to those defined by the LAN MAC Service Definition. “

The following figure describes the IEEE Std. 802.1D Transparent Bridging architecture (see Figure 7-3: Bridge Architecture in IEEE Std. 802.1D specification).

- Figure 8-43: IEEE Std. 802.1D Transparnet Bridging Architecture

The ISS Service Mapping is defined in IEEE Std 802.1D-2004, section 6.5.4.

IEEE Std. 802.1Q-2003 specifies the Enhanced Internal Sublayer Service, which is extended from ISS and is provided by the Media Access Independent Functions to support frame relay (see IEEE Std. 802.1Q-2003 section 3.7) operation in the VLAN Bridge.

The following figure describes the IEEE Std. 802.1Q VLAN Bridging architecture.

- Figure 8-44: IEEE Std. 802.1Q VLAN Bridging Architecture

The ISS Service Mapping is defined in IEEE Std 802.1Q-2003, section 6.4.

### WLAN Mesh Interworking Frame Translation

For the WLAN Mesh interworking support, four network interworking scenarios must be considered:

- Interworking of Non-Mesh 802.11 network with 802.11 WLAN Mesh

- WLAN Mesh interworking with other WLAN Mesh

- WLAN Mesh interworking with other 802 network (i.e. non 802.11)

- WLAN Mesh interworking with the higher-layers

In general, when relaying frames between different MAC methods, there are a number of frame translations that may be required, depending upon the source and destination MAC methods concerned, the tagging state of the received and transmitted frames, and the data carried within the frames.

- If the source and destination MAC methods differ, then the overall format of the received frame must be translated into the frame format required for the MAC onto which the frame will be transmitted. The details of the frame translation at this level is defined by the definition of the ISS (IEEE Std 802.1D-2004, section 6.4)., the support of the ISS by specific MAC procedures (IEEE Std. 802.1D-2004, section 6.5) and the standards that define the specific MAC procedures concerned.

- If the received frame is being relayed in tagged format between differing MAC methods, or if the tagging state is to change, then the tag header may require translation, insertion or removal as defined in IEEE Std 802.1Q-2003 specification.

- If Ethernet Type-encoded data is being carried in the frame, then the format of that data may require translation as defined in IEEE Std 802.1H, IETF RFC1042 and IETF RFC 1390. These translations are essentially concerned with providing a standardized means of representing Ethernet Type-encoded frames on LANs that have no inherent ability within their MAC procedures to represent Ethernet Type values (i.e. LAN MACs where the “native” link layer protocol is LLC), and ensuring that the frame translation in the reverse direction results in the correct format on different 802 LAN. The mechanisms specified in these standards are based around the use of SNAP encoding to carry Ethernet Type values in native LLC-encoded environments, and the use of translation tables to control the reverse translation if this should be necessary. Without following the RFC 1042, there is no way that additional information can be carried in the frame in order to identify the embedded format, in order for such a Bridge to successfully translate embedded address information, it needs to be able to recognize the higher layer protocol carried in the frame and act accordingly.

For interworking scenarios 1-3, the translation mechanisms a) and b) decribed above are to be used. For interworking scenario 4, translation mechanism c) described above is to be used.

The following figure depicts the general WLAN Mesh interworking architecture.

- Figure 8-45: WLAN Mesh Interworking Architecture

## Consistent Routing Topology via Hello LSA update

In a typical Mesh network, it's imperative to have the consistent view of the reachabilty of all STAs and MPs from any single node of the Mesh network. As illustrated in the following diagram. Node 2 in the diagram receives the Hello neighbour information inside the Hello Neighbour IE from the adjacent Node 1 and Node 2 become aware of the MAC addresses of MAC11,MAC12 and MAC13 being reachable via its interface If 2.1. In turn Node 2 will broadcast out its own Hello updates to Node 3 notifying the reachabilty of the above learned MAC addresses plus its own STA MAC 21. Based on the same Hello hop by hop updates mechanism, each node is able to update its forwarding table with the consistent view of the topology

- Figure 46 Hello Protocol

## Traffic Engineering Routing

When classified traffic access the WLAN Mesh network, they impose different QoS requirements on the network. In IEEE 802.11E which specifies the framework of how to map the user prioritized (UP) traffic (IEEE 802.1P&Q based) to the different Access Category (AC), then mapped to the priotized transmitting queues, it’s necessary for Mesh APs to share the same QoS pre-conditioning requirement as the entry MP. In the following diagram, MAP1 is the entry point where three types of traffic, Voice , Video and Data, are flowing into. In this particular case, Voice traffic is priotized with priority 6 with the P tagging , the video is priority 4 and the data is priority 2. By default, in MAP1, the mapping of the IEEE 802.1P priority metric to IEEE 802.11E AC is specified in the standard, the Voice which is delay /jitter stringent application is mapped to AC_VI and then mapped onto the High priority queue. Considering the TSPEC parameters , including Peak rate, mean rate, MSDU size etc, the MAP1 will calculate the optimum size of the buffer in Kbits to accommodate the Voice application, in this particular case, 20% percent of the total buffer size is allocated to the Voice application., the same process applies to Video and data traffic till 30% of the buffer size is allocated to Video and another 30% is allocated to user data, (The system has to preserve 2% for management data)

In order to guarantee the end-to-end QoS for the applications, the MAP2 and MAP3, which are supposed to be on the transmitting path, will have to share the same Queue condition information based on each individual capability. Via the QoS IE conveyed inside the Beacon , Probe and Association messages, MAP 2 and MAP3 get the knowledge of the traffic types and queue sizes for different application. The SPF on each MAP node will utilize the above information to come up with the constrainted Traffic Engineering Route.

The coloring scheme, Gold, Silver and Bronze, identifies the Queue sizes which make the SPF in each node understand the transmitting capability , therefore, it improves the end-to-end QoS .

The queue buffer size varies dynamically from time to time according to the traffic type and load, the changes will be automatically updated through the periodic beacon, management messages

The traffic classification process could be manually accomplished by specifying the ACL (Access Control List) which instead follows the default mapping table specified in IEEE 802.11e. MAP1 in the diagram could be customized to decrease the buffer size for the data traffic to compensate for the time sensitive Voice application

- Figure 47 Traffic Classification

## Power Saving

Conservation of power is crucial for battery-powered mesh points used in home electronics, public safety, and military.  We have identified various techniques that can be used for power savings.

Cross-layer design is used to achieve maximal saving on power without affecting network connectivity.  Power savings can

be achieved at different phases of the mesh protocol. An example of the power saving opportunity is shown in the following figure.

- Figure 48 Example of Power Saving

MAPs , node 1, 2,3 and 4 are aligned in a chain connecting both Network N1 and N2. Both Node 2 and node 4 have a WLAN association with MAP node 5 and node 6 is connected to node 5 . The sample routing table on MAP node 2 is shown in the following table

| Destination | Next-hop If | Link Type | State | Power Weight f |
| --- | --- | --- | --- | --- |
| Node1 | 2.1 | D | Active |  |
| Node 3 | 2.2 | D | Active |  |
| Node 5 | 2.3 | D | In-active |  |
| Node 4 | 2.2 | I | Active |  |
|  | 2.3 | I | In-Active |  |

The routing table lists for each destined node, the next hop interface information, the link type, and whether directly connected or indirectly connected. In the table, The State denotes at the time the power saving function taking place, whether the link state should be Active or In-active. Power weight f is the function to calculate the power consumption weight associated with each wireless link. The calculation is based on the algorithm of if nodes have information about the position and activity of all other nodes. The optimal power saving algorithm that will minimize the total energy per packet can be obtained by applying Dijkstra’s single source shortest weighted path algorithm.

In the above example, when the network with all the nodes get association and links reach a stable state, after invoking the power saving function in each node and flooding the power consumption parameters throughout the network via the periodic routing updates, the node 5 (orange) color will be put into the sleepy mode to save its power consumption, because the majority of traffic will not go through node 5 under normal conditions. In consequence, the node 6 will be either put into sleepy mode or being actively attempting to get association from another nearby node.

### Discovery and Association

During discovery, we can use extended management frames (beacon or probe response) to reach connected state quicker and to reduce the number of required transmissions to save power. We can also discover the battery power situation using extended management frames to avoid associating with a mesh point with battery power constraint. The frequency of extended beacon transmission can also be adjusted based on battery power level.

### Resource Allocation and Medium Access

The priority of medium access can be a function of the remaining battery power, similar to 802.11e where the application QoS is used to determine the priority on the medium access. We can also use aggregated frames to keep the MP either in transmit or receive state. Power can be saved if we can minimize the transitions between transmit and receive states. We can also choose a subset of mesh points to go into sleep mode without affecting topology connectivity by coordinating with the routing layer. The mechanism to achieve this to use a combination of physical and virtual carrier sensing; specially, the SCC combined with virtual carrier sensing allows a device to go into sleep if the device determines that there is no traffic destined to itself. We also can adopt power-aware scheduling (resource allocation) to conserve power by considering data rate (802.11 a/b/g/n mode) and transmit power.

### Routing

We propose to use the remaining battery power as part of the routing metric. We need to avoid using the node with battery power constraint for forwarding. We also need to use the routing path with least transmit power requirement.

## Multicast/Broadcast Reduction Experimental Results

It is important to enable efficient multicast and broadcast data delivery in WLAN mesh networks. One approach is that the source MP marks the frame header with a unique sequence number for each multicast or broadcast frame. Intermediate MPs can detect and filter the duplicated multicast or broadcast frames. This approach requires each intermediate node maintains a record which frames have been forwarded. Another approach to reduce multicast and broadcast duplicates in classical flooding is to use multi-point relays. Source-specific Multi-point Relay (S-MPR) allows only “locally elected” MPRs to relay frames that are received from upstream selector nodes. It requires knowledge of previous hop identification to determine the next forwarding decision. An alternative is to use non source-specific Multi-point Relay (NS-MPR). The NS-MRP combines all S-MPRs into a common relay node set (i.e. specific hierarchy). It requires only the identity of neighbor MPRs, which is simpler, but does not scale as good as S-MPR.

Classical Flodding

S-MPR MCDS = 1

S-MPR MCDS = 2

S-MPR MCDS = 3

S-MPR MCDS = 4

S-MPR MCDS = 5

Total Multicast Overhead Traffic

Rate (kb/s)

200

400

600

800

1000

1200

1400

1600

1800

200

400

600

800

1000

1200

1400

Time (s)

- Figure 8-49: Multicast/Broadcast reduction Results

Figure 8-21 shows the performance of different multicast & broadcast data delivery approaches. The testing is based on 2 Mbps 802.11b link with 10 Mesh Points. The multicast source traffic changes from 10-800 kbps incrementally every 10 sec. With the classical flooding without using MPRs, traffic transmission rate is saturating around 1.3 Mbps. The improved relay set efficiency keeps the ad-hoc network from saturating until higher multicast source rate. Furthermore, the slope measuring total overhead traffic increases as the number of nodes in the relay set increases

The decreasing slope is indicative of the network topology’s increase in density allowing a smaller ratio of relay nodes to be used.

# Mesh Security

This proposal utilizes 802.11i-based security mechanism and optional additional security mechanisms to support:

- Scalable security for data and control traffic,

- On-demand very secure multicast group for high-security applications, and

- Authentication of non-secure pre-association traffic: Mesh Discovery, Mesh Probe Response and Mesh Report information control frames.

The 802.11 Data Frames that are secured are: data frames and control frames.  Non-secure pre-security assocation control traffic includes: Mesh Discovery, Mesh probe Response and Mesh Report control frames. Authenticating of pre-association traffic allows operational flexibility in deploying multiple networks with multiple vendor environments.

This proposal uses the 802.11i concepts and mechanisms for wireless station to mesh access point.  This proposal extends the use of 802.11i concepts and mechanisms to mesh discovery and mesh association.

This proposal supports distributed and centralized models for AS functions. 802.11i mechanisms are used to distribute the required 802.11i keys and optional keys. The 802.11i required keys are: pair-wise keys for neighbors (PMK, PTK) and Group keys for all nodes (GTK). Group temporary keys (GTK) are generated from a group master key

Section 8.1 provides an overview of the security architecture highlighting the extensions made to the 802.11i functionality. Encryption of data by data path type is describedin section 8.2. The mechanisms and algorithms for Mesh Discovery and Mesh Association are covered in section 8.3.  Section 8.4 covers knobs to tune the security for different networks with differing levels of attacks including replay attacks.

## Security Architecture

The security architecture is built on existing 802.11i concepts and mechanisms extended to allow:

- Multi-hop encryption of Unicast and multicast frames (data and control) using a virtual link/tunnel.

- Mesh Points to encrypt traffic inter-mesh point traffic  based on pair-wise keys (PMK/PTK), two type of group keys (GTK/NTK), and keys specific to on-demand multicast (MMK/MTK),  and

- Extensions to protect pre-associated data with Authentication.

A multi-hop encryption of unicast and multicast frames are tunneled using a 6 part WDS address (final source, final destination, link source, link destination, tunnel ingress, tunnel egress.)  The inner most packet is encrypted over the virtual link that is the tunnel at the tunnel ingress.  At the tunnel egress, the data packet is de-encrypted.   This tunnel acts like a virtual link.  The multi-hop link that forms the tunnel can be encrypted or non-encrypted.

Since 802.11i only deals with the interaction between a wireless AP and stations and not inter-AP traffic, the 802.11i must be extended to handle mesh points.  Inter-Mesh point traffic may be encrypted using pair-wise keys or a group key.  As defined in 802.11i, the group master key is per AP.  The group keys between mesh points may share a Group Master Key (GMK) between all mesh points or the GMK may be specific to an AP.  In either case, each AP will provide a GTK for all links.  The PMK/PTK and GMK/GTK are the first two encryption keys.

This specification denotes a shared master key among all nodes as a GMK with specific GTKs per AP.  If the Group master key is specific to the node, this proposal denotes this as the Neighbor Master key (NMK).   Individual links translate this key to a NTK.   Both these keys support local multicast.  Local multicast is defined to include the local broadcast functions.  This  Neighbor Master Key and Neighbor Transient Key (NMK/NTK) are the third set of security encryption keys.  The NMK/NTK keys are optional and may/maynot be supported.

A fourth of encryption key  is specific to a Group MAC address with a master key (MMK) specified for all nodes for this group MAC address.  For each link, a Mutlicast Transient key is created to encrypt the data across each link.

Very secure multicast data transmission of frames may require on-demand securing of a multicast group. A military application that uses Triple play (Video, Voice and Data) sent to a multicast group requires a higher level of security on each frame sent to the MAC group addresss. An on-demand encryption key for the multicast group provides an encryption key that is only understood by the members of the group for a short duration. This fourth of encryption key is specific to a Group MAC address with a master key (MMK) specified for all nodes supports such the on-demand highly secure MAC address based encryption.  Again, for each link, a Mutlicast Transient key is created to encrypt the data across each link.  This on-demand multicast groups is optional, and requires routing protocol support.

Upon detecting the existence of a new entrant into this on-demand key set, the coordinated keys are queried from the authentication server associated with a Group leader.  This group leader may signal a re-keying of these keys at any point.  The routing protocols will signal the existence of the MAC address as “Secured or un-secured”.  An un-secured mac can only be used to signal to the members of the group and especially the group leader about additional requests to secure this multicast group.  After the MAC address has been secured, the routing infrastructure will calculate a multicast forwading path.  At any point, the routing infrastructure may announce that the MAC exists but has gone from secured to un-secured.

Very secure multicast transmissions will be critical to some deployments and unused by others. This proposal has carefully tailored the algorithms to limit or remove any burden of this support from basic security.

The security architecture can run with a centralized AS model or a distributed AS model as denoted in Figure 8-1 below. Section 8.x describes how the mesh points SME establishes a RNSA in one of three ways: Centralized 802.1X authentication, Distributed 802.1X authentication model, and pre-shared authentication model.

Figure 9-50: Secuirty Architecture

Section 8.x  describes the in detail the negotiation of the authenticator versus supplicant status for two MP that both believe they have been authenticated to the mesh.

## Support of Hop-by-Hop and Multi-Hop Encryption

This proposal supports securing data via encryption for:

- Hop-by-hop links – via pair-wise encryption between neighbors

- Multi-hop

- Tunnel links – via pair-wise encryption between ends of a tunnel

- Broadcast Data – via group keys for all members of a wireless mesh

- Locally Multicast Data to neighbors

    - Via group keys as the default security,

    - (Optionally) via a neighbor key (NTKn)

- Data sent to multicast groups

    - Via group keys as the default security, or

    - (Optionally) for very tight security, via an encryption key per multicast group (MMK, MTK).

The management frames can secured in the same way as the data frames across the same paths. Management frames with routing have additional information: sequence numbers and originating sources to allow detection of repeated information in neighbor “hello” or mesh topology information.

### Use of Pair-Wise Keys (PMK, PTK)

Pair-wise keys can support different types of data paths: wireless stations, hop-by-hop, or tunnels.

An example of these different data paths is below in figure 9-1. For example, Wireless Station 2 will encrypt the data prior to sending it to MAP/MP4. MAP/MP4 will de-encrypt the data and re-encrypt it to send it MAP/MP 5. A tunnel is created from MAP/MP 3 to MAP/Mesh Point 4. All traffic going trom WLAN Station 2 (WLAN STA 2) or WLAN Station 3 (WLAN Station 3) to WLAN Station 4 (WLAN STA 4) will be routered from MAP/MP4 to MAP/MP2 to MAP3 via Tunnel 1. Via the Tunnel MP/MAP 3 is a neighbor to MAP/MP 4. These two neighbors authenticate via the tunnel and exchange pair-wise keys. As the packets enter the tunnel, the packets are encrypted with the tunnel security key. The packets are forwarded via the tunnel header to MAP/MP 3 at which point the packets are decrypted.

Figure 9-51: Secure Architecture with Tunnels

### Use of Group Key (GTK)

A single Group key per mesh supports encryption of Broadcast data and all multicast frames (local and global groups) by default. Group keys creation follows normal 802.11i group key creation. The Group key can encrypted data frames or control frames.

A few examples will illustrate the use of the Group key. In our example in figure 9-1, MAP/MP-1 can broadcast a data frame to all nodes in the mesh. MAP/MP-1 encrypts the frame with the group key and transmits it to all of its neighbors. Each of the neighbors that have been associated with the mesh has the group key and decrypts the packet. If the packet needs to be forwarded on, it can be forwarded to other nodes.

If only default multicast support is enabled, the transmission of a data frame sent to a MAC group address performs in the same way, the multicast frame is encrypted at the source and sent to the group MAC address. Local control frames sent between MP/MAP 1 to MP/MAP 3 and MP/MAP 4 will also be encryped with the group key, and de-encrypted upon reception. Note, an MAP should use a different GTK for sending multicast traffic to its associated wireless STA’s. This is because a wireless STA should not be able to see the mesh traffic being sent by any MP.

### Use of Neighbor Keys (NTK)

Neighbor keys are used for locally multicast frames (data or control). During mesh discovery, the Mesh Point gets the neighbor keys (NTK) is sent from the Aspirant-MP to the Member-MP to indicate its neighbor key for local multlicast. The Member-MP sends its Neighbor key (NTK-Member) to the Aspriant-MP.

In figure 9-2, MAP/MP4 could send local frames to Local Multicast traffic (MAP1, MAP/MP2, MP/MAP5) and wireless stations (WLAN STA 2 and WLAN STA 3) via a single neighbor key. An example a control frame sent as local multicast traffic is the “hello” packet from the neighbor discovery function. Note, an MAP should use a different GTK for sending multicast traffic to its associated wireless STA’s.

The Neighbor keys provide two benefits:

- single encryption for all local multicast instead of multiple encryptions

- less chance for replay attacks if GTK not used always

The receiver knows to use NTK for the Link addresses (RA and TA), with the following logic for precedence in keys:

- Is WEP encryption bit in Frame Control field set and processing Link addresses (RA and TA)?

  - If WEP bit is not set:

    - If the frame is a tunnel frame, go to Step 2.

    - No decryption needed. Pass clear text packet to be processed (forwarded or processed with local machine.)

  - Check the RA for Group bit in MAC specific address

    - If WEP and Unicast MAC – use the PTK encryption key and decrypt.

    - If WEP and Group MAC (including broadcast MAC), go to step c.

  - Check for a matching MAC address to RA in the MTK key table. (The MTK key table contains MAC address/MTK keys).

    - If MAC address matches an MAC address/MTK key table entry, use the MTK to decrypt the packet.

    - If no match on MAC address in MAC address/MTK table – go to step d.

  - Check the link transmitter address (TA) against the neighbor MAC addresses in the NTK table. (The NTK table has neighbor address (unicast) and associated keys).

    - If the transmitter address (TA) matches a neighbor with keys in the NTK key, decrypt the packet.

    - If the transmitter address (TA) does not match a neighbor with keys in the NTK key, go to step e

  - Default to using the GTK key to decrypt the packet.

- Process tunnel frame with WEP encryption bit in Frame Control field cleared:

  - Check if the local node is the Egress MP of this tunnel

    - If this local node is not the Egress MP of this tunnel frame, pass this frame to be processed (forwarded or processed with local machine.)

    - Otherwise, go to step b.

  - Check the the Tunnel WEP flag (pre-configured or following Appendix 1)

    - If the tunnel WEP flag is not set, the tunnel traffic is in the clear – pass the packet to be processed (forwarded or processed by local machine.)

    - If the tunnel WEP flag is set, go to step c.

  - If the tunnel WEP flag (pre-configured on virtual interface or in extended CTL header) is set, check if DA address is a group MAC address

    - If DA is not a group MAC address, use the PTK for the tunnel’s source (Ingress MP address) to decrypt

    - If DA is a group MAC address, go to step d

  - Search for DA address in the MTK key table

    - If the key exists, decrypt the packet using the MTKmac key that matches

    - If the key does not exist, go to step e

  - Search for Ingress MP address in the NTK key table

    - If the Ingress MP address matches a neighbor address in the NTK key table, decrypt the packet.

    - If not, go to step f

  - Use the GTK key to decrypt the packet

### Multicast Group Specific keys (MMK/MTK)

Optional Multicast Group keys are created with one key per multicast group (Master Multicast Group key) (MMK).)  Each node uses this group Multicast key to calculate a local temporary key (MTKg).

These group keys are neogitated during the 802.11i and 802.1X negotiations if the Group MAC addresses are pre-configured into the node. An extension to the RSNie information denoted as the M-RSNie information provides the hints for the 802.1X queries (section 9.1.2.x). Once the initial negotiation phase is completed, each associated MP has the pre-configured GTKs established.

If the Group MAC addresses are on-demand, the MP must re-negotiate a key association for the multicast Group once Group MAC addresses are detected. If multiple group MAC addresses are detected, the MP may obtain several on-demand keys at once.

Group MAC addresses local to an MP can be detect via the MP’s forwarding logic or from a layer 2 protocol (GMRP). Remote Group addresses are learned from local neighbors (single or multi-hop neighbors) via the neighbor discovery “hello” information or from the routing topology information.

Both the “hello” information (section 8.3.2) and the topology information may pass a Routing IE with a sub-field of a Multicast MAC IE and a security field of M-RSN for hints about the security.  This Multicast MAC IE contains a count of MAC addresses, and a tuple of (MAC address, flag byte) per MAC address. The flag byte contains bits to indicate:

- Per MAC address Group encryption key required,

- Per MAC address Group encryption aquired,

- Per MAC address Group Leader used for security.

Upon detecting a remote MAC Group MAC address, the MP can choose to obtain per group encryption key at that point or wait until a local event occurs that will cause it to join the multicast tree. A few local events that cause multicast forwarding to a group address set-up are:

  - Frame received destined to that Group MAC address,

  - Frame received sent from that Group MAC address, or

  - Policy configured on node to pre-associated a set of group MAC addresses if heard from remote sites.

A multicast group will have a multicast group leader for security. The group leader can be pre-configured for a range. The group leader forms a guaranteed place to concentrate on-demand securing to for the group.

The link access protocol is based 802.11i RSNA security and supports centralized and distributed 802.1X based authentication and key management. An optional field in the RSNA information supports the secure multicast key (M-RSN ie field).

## Authentication of Pre-Association Management Frames prevents Forgery

Prior to the security association, none of the control frames passed across links are encrypted. All Management frames need to be protected against forgery. Post security association, the management frames are protected via encryption.

Forgery can be impacts the data security and the scalability of the node. Repeated sending of bogus management frames can impact the resources used by each mesh point. The mesh point requires time to process and discard bogus management frames. In order to reduce overhead for processing pre-associated frames from a network with the same mesh id (probably a pre-configured mesh id), an optional authentication key can be used for packets that are received. Forgery can be intention or accidental.

Accidental forgery can occur in some operational deployments with multiple WLAN mesh networks will be deployed with multiple vendor equipment will be utilized. These networks may be deployed with the same pre-configured wireless mesh ID. A network manager can configure MD5 authentication keys into the box to protect the pre-association day from forgery.

Intentional or malicious forgery can be specific attacks on mesh points to deny service (Denial of Service (DOS) attacks) or to break into the pre-associated data. DOS attacks can take take precious resources away from the Mesh Point.

Mesh management frames are in general in clear text except for Disassociation Request, Mesh Report, and Action, which are protected by M-PTK. Management frames send in clear text the pre-association phase are: Probe Request/Probe Response, Association Request/Association Response, and Mesh Beacon frames. Of these frames, the Probe Request only contains requests for information. The MD5 authentication protects the integrity of the information passed back in management frames for:

- Mesh Beacon,

- Probe Request/Probe Response,

- Association Request/Association Response,

Each of these management frames contains the authentication ie field that contains the authentication value.

The Mesh Probe Request will not be authenticated. Protection against intentional or malicious forgery can utilize other mechanism to DOS attacks without adding the burden of an authentication field. For example, repeat Mesh Frame Requests can be detected and tossed. If the Mesh Probe Requests a non-authenticated Mesh probe response when an authentication is required, the Mesh Probe Request can be ignored.

A HMAC MD5 encryption key is proposed as the initial cryptographic technique to be deployed by this proposal. A pre-shared key is calculated over the Frame Body, and Frame header.

### RSNA Esablishment

Prior to association, each MP exchanges: RSN ie information, AS connectivity information, and optionally (for MTK/MMK) M-RSN ie information. Any MP that assumes the authenticator role may also host the AS. As a result, mesh points SME may establishes an RSNA in one of the three ways: distributed, centralized or pre-configured. The 802.1X mechanisms pass keys for PTK, GTK, NTK, and pre-configured MAC address based MMK/MTK. Additional the Multicast group keys may nominate a “Group Leader” for security that forms a secure hub for authentication. Section xxxx covers the RSNA establishment mechanisms in the face of this authentication.

As described in section 7.x on mesh discovery these message exchanges can occur in several different message scenarios: nominal passive, nominal active, special passive and special active. This section does not repeat those scenarios. This section describes the mechanisms for RSNA establishment for distributed, centralized and pre-configured. Section 7.1 also indicates that if both MPs indicate that they are authenticators, they utilize the beacon to negotiate the Authentication. Section 9.1.4 describes this negotiation process.

#### Distributed 802.1X authentication model

When using the distributed IEEE 802.1X AKM in a WLAN Mesh, a MP’s SME establishes an RSNA by:

- Identifying a peer MP as being RSNA capable from the peer’s Beacon or Probe Response frames.

- Elect a authenticator based on the information in the peer’s Beacon or Probe Response frames

  - Authenticator with highest AS priority and lowest IP address is elected as authenticator for pair

  - If no AS priority exists, it is determined to be zero.

- Invoking an Open systems authentication

  - The cipher suites are negotiated between peers to be mutually agreeable during the association process.

- The authenticator MP use 802.1X to authenticate with the AS associated with the other MP’s (via various passive or active scenarios). If non-mutual authentication occurs, the supplicant may also authenticate to the other MP’s AS if it exists. If two authentications occur, each MP is done separately and independently.

- Each MP’s SME establishes temporal keys (PTK, GTK, NTK, MTK) by excuting the key management algorithm using the protocol defined in 802.11ma 8.5.

- Both MPs use the agreed-upon temporal key portion of the PTK and pairwise cipher suite from one the exchanges to protect the link for unicast traffic. Broadcast traffic is protected by the GTK. Multicast traffic is either protected by GTK and optionally NTK or MTK keys.

- In the case of a Mesh AP, the MESH AP (MAP) may take on the authenticator role for legacy stations for PTK, GTK, NTK or MTK.

#### Centralized 802.1 Authentication Model

When using a centralized IEEE 802.1X AKM in a WLAN mesh, a MP’s SME establishes an RSNA via the following steps:

- The MP is identified as being RSNA capable and as connected to an AS.

  - The RSN ie field gives the hints for the PTK, GTK, and NTK

  - The M-RSN ie field gives the hints for the MMK/MTK for known MAC groups

- Elect an authenticator for the connection based on information in the peer’s Beacon or Probe Response frames

  - Authenticator with highest AS priority and lowest IP address is elected as authenticator for pair.

  - If no AS priority exists, it is determined to be priority zero.

- The Mesh Point’s SME invokes Open System Authentication.

  - The MP negotiates cipher suites during the association process, as described in 802.11ma sections 8.4.2 and 8.4.3.

- The authenticator MP uses 802.1X to authenticate with the AS associated with the other MP’s (via various passive or active scenarios) as decribed in 802.11 ma section 8.4.6 and 8.4.7. The supplicant’s MP may also authenticate to the the AS associated with the other MP, if it exists.

  - Both 802.11X authentications may occur independently.

  - The location of the AS as collocated with an MP or available across a secure connection is an implementation detail and beyond this specficiation. This proposal will support both.

- Each MP’s SME establishes temporal keys (PTK, GTK, NTK, MTK) by excuting the key management algorithm using the protocol defined in 802.11ma 8.5.

- Both MPs use the agreed-upon temporal key portion of the PTK and pairwise cipher suite from one the exchanges to protect the link for unicast traffic. Broadcast traffic is protected by the GTK. Multicast traffic is either protected by the GTK, NTK or MTK.

- In the case of a Mesh AP, the MESH AP (MAP) may take on the authenticator role for legacy stations for PTK, GTK, NTK or MTK.

#### Pre-configured Authentication Mode

When using pre-configured keys, the following steps are taken to create RSNA association:

- The MP identifies the peer as RSNA capable from the peer’s Beacon and Probe Response frames.

- Elect an authenticator for the connection based on information in the peer’s Beacon or Probe Response frames

  - Authenticator with highest AS priority and lowest IP address is elected as authenticator for pair.

  - If no AS priority exists, it is determined to be priority zero.

- The Mesh Point’s SME invokes Open System Authentication.

- Both MPs use PSK (pre-shared keys) to establish Temporal keys (PTK, GTK, NTKneighbor, MTKmac)

#### Group Reconfogured Authentication Mode

- The MP is identified as being RSNA capable and as connected to an AS.

  - The RSN ie field gives the hints for the PTK, GTK, and NTK

  - The M-RSN ie field gives the hints for the MMK/MTK for known MAC groups

- Elect an authenticator for the connection based on information in the peer’s Beacon or Probe Response frames

  - If both MPs indicate authenticator process, chose the authenticator with the Group Leader flag.

  - If both MPs have the same status for the group leader flag (on/off), chose the Authenticator with highest AS priority and lowest IP address is elected as authenticator for pair.

    - If no AS priority exists, it is determined to be priority zero.

- The Mesh Point’s SME invokes Open System Authentication.

  - Depending MP negotiates cipher suites during the association process.

- The authenticator MP uses 802.1X to authenticate with the AS associated with the other MP’s (via various passive or active scenarios) as decribed in 802.11 ma section 8.4.6 and 8.4.7. The supplicant’s MP may also authenticate to the the AS associated with the other MP, if it exists.

  - Both 802.11X authentications may occur independently.

  - The location of the AS as collocated with an MP or available across a secure connection is an implementation detail and beyond this specficiation. This proposal will support both.

  - SME for either MP may choose to authenticate MMK/MTK information to an AS associated with the MP that is associated with the Group leader.

- Each MP’s SME establishes temporal keys (PTK, GTK, NTK, MTK) by excuting the key management algorithm using the protocol defined in 802.11ma 8.5.

- Both MPs use the agreed-upon temporal key portion of the PTK and pairwise cipher suite from one the exchanges to protect the link for unicast traffic. Broadcast traffic is protected by the GTK. Multicast traffic is either protected by the GTK, NTK or MTK.

- In the case of a Mesh AP, the MESH AP (MAP) may take on the authenticator role for legacy stations for PTK, GTK, NTK or MTK.

## Security for Different Types of Data Paths

Most of the mesh frames are protected by encryption keys that are generated during discovery process. The following sections describe how each mesh frame type is protected.

### Mesh Management Frames Security

Mesh management frames are in general in clear text except for Disassociation Request, Mesh Report, and Action, which are protected by M-PTK. In the case where Mesh Report and Action frames are multicast to from a MP to all of its neighbors, they will be protected by M-GTK or M-NTK if it is available.

Management frames such as Probe Request/Probe Response, Association Request/Association Response, and Mesh Beacon frames are in clear-text so that discovery process can be complete successfully.  However, these frames  can be protected by the existing M-PTK during the re-key process. Optionally, the data carrying frames (Mesh Beacon, Mesh Probe Response, Mesh Report and Mesh Association) can be MD5-authenticated to ensure the integrity of these frames during discovery process. Each of these management frames carries an authentication ie field that contains the result of the cryptographic message digest.

The Mesh Probe request carries a a mesh ie indicator field indicating whether the Probe Response requests this optional MD5-authentication of the Probe Response.

### Mesh Control Frames

Control frames are always protected by either M-PTK if they are unicast or M-GTK if they are broadcast. By default, all control frames multicasted are encrypted by the M-GTK. The exception to this default is the availability of a M-NTK for local multicast to neighbors or a MTK for the specific Group MAC address.

If the M-NTK is available and control frames are multicast to an MP’s neighbors, they will be protected by the M-NTK originating from the MP.

If a routing protocol utilizes a pre-known global multicast group MAC Address (such as All-MESH-MP-H-LSPV-Routers), then the MAC address may be pre-configured to be secured via the MMK/MTK for that multicast group.

### Mesh Data Frames

Mesh data frames are always protected. For unicast frames, M-PTK is used for encryption/decryption from one MP to the next one. For tunneled unicast frames, prior to sending the packet into a tunnel a encryption/decryption is performed at the ingress/egress MP’s if there is an encryption key for the tunnel. The tunnel is treated as a virtual link so 1 encryption is sufficient.

Multicast data frames are protected by either GTK or NTK or MTK if it is available.

Multicast MAC Addresses may be forwarded over direct mesh links or tunnel links. The encryption of the data is determined at the point the data is forwarded over a link. Encryption via GTK, NTK or MTK key depends on the multicast forwarding.

As an example of this encryption mechanism, the multicast forwarding in the wireless mesh in Figure 9-3 connects Multicast senders/receivers to a Multicast group address. The senders/receivers are A, B, C, and N Mesh Point Forwarding at MP1, MP2, and MP3. Mesh point N creates a tunnel between the MP1 and Mesh Point N to send multicast MAC farmes. For clarity, mesh point N links are denoted “D” for direct links and “T” for tunneled links. Multicast Data sent from Node N to the multicast group utilizes the tunnel link “T”. This Tunnel link goes through MP6. MP6 has a unicast mid-tunnel forwarding entry for all packets that the tunnel carries.

(need to add back in the diagram 9-3).

Data sent into the tunnel at Mesh Point N uses PTK to encrypt forward unicast frames and either GTK, NTK or MTK to encrypt forward multicast frames. The tunnel frame consists of:

- RA and TA (source) – hop by hop destination across the nodes that form the tunnel

- DA and SA – final destination and source for packet.

- Tunnel Egress MP address and Tunnel Ingress MP Address – two ends of the virtual interface for the tunnel.

As data enters a tunnel, the WEP key gets set to zero because no encryption of the packet is no valid on the hops through the tunnel.

However, the packet is encrypted and tunnel WEP flag (pre-configured on each end of the tunnel or set in the frame). When the packet reaches the remote end of the tunnel (Egress MP address), it will decrypt the frame according to the tunnel WEP flag value. If the frame has a destination address (Egress MP address) with a Unicast MAC Address, the Tunnel uses the PTK for the virtual tunnel link. If the frame is being sent ot a Multiast group (group MAC address in Egress MP address), the tunnel ingress uses one of the multicast encryption keys (GTK, NTK, or MTK) to encrypt the packet depending on the keys set-up for the link.

On the remote side, the frame is decrypted – a tunnel WEP flag (either pre-configured or set in frame). If unicast encryption is being used, it should be checked that Egress MP address is a unicast MAC address.

A PTK encryption key for the tunnel is used. If Egress MP address is a multicast address, the tunnel WEP flag is used.

[Sue – editor check this section to match diagram 9-3 eventually.]

## Mesh Association and Keys

The following keys are used in a mesh for encrypting/decrypting mesh frames:

- M-PTK (Mesh Pairwise Transient Key) – used for protecting mesh frames exchanged between two mesh neighbors

- M-GTK (Mesh Group Transient Key) – used for protecting broadcast mesh frames and multicast mesh frames if no M-MTK is created, and used for protecting local neighbor multicast of mesh control frames if no M-NTK is created

- M-NTK (Mesh Neighbor Transient Key) – used for protecting multicast mesh frames to all neighbor of a mesh node of mesh, such as routing topology

- M-MTK (Mesh Mutlicast Transient Key) – used for protecting of mesh frames to all MP supporting multicast group

The generation of M-PTK, M-GTK and M-NTK keys is done in a 4-way handshake similarly to the 802.11i after 802.1x authentication is complete. M-MTK is created during the creation of a multicast group.

Note, M-NTK and M-MTK are optional key.

### RSNA keys Exchange PTK, GTK

The establishment of required keys between a supplicant and a authenticator is shown PTK and GTK for a particular node follows the basic sequence shown in diagram 9-x below as a diagram between a supplicant MP and a Authenticators MP.

MP1

MP2

Supplicant

Authenticator

Key(PMK) is Known

Generate SNonce

Key(PMK) is Known

Generate ANonce

EAPOL-KEY (ANonce Unicast)

Derive PTK

EAPOL-KEY (SNonce Unicast, MIC)

Derive PTK

If needed GTK

EAPOL-KEY (Install PTK, Unicast ,MIC, Encrypted GTK)

Install PTK &GTK

Install PTK

ACK (or Install)

### Keys exchanged for PTK, GTK, and an NTK per peer.

In the case when both mesh neighbors support M-NTK, the handshake procedure is as follows between an MP1 Supplicant and an MP2 Autheneticator.

MP1

MP2

Supplicant

Authenticator

Key(PMK) is Known

Generate SNonce

Key(PMK) is Known

Generate ANonce

EAPOL-KEY (ANonce Unicast)

Derive PTK

EAPOL-KEY (SNonce Unicast, NTK-Aspirant, MIC)

Derive PTK

If needed GTK

EAPOL-KEY (NTK-Member, Encrypted GTK, MIC)

Install PTK &GTK

Install PTK

Install

The Encrypted NTK-Aspirant is the neighbor key from the MP1 supplicant that will be used when MP1 sends local Multicast data. The Encrypted NTK-Member is the neighbor MP2 will use when sending local Multicast data.

### Keys exchanged with PTK, GTK, NTK per Mesh point, and MMK/MTK (pre-configured MAC addresses)

In this diagram, MP1 and MP2 exchange only 1 pre-configured key Multicast group key. Multiple pre-configured keys are possible, and will follow the same flow. However, for ease of reading of the diagram the each MP only sends one MAC key.

MP1

MP2

Supplicant

Authenticator

Key(PMK) is Known

Key (MMKm) is known

Generate SNonce

Key(PMK) is Known

Key(MMKn) is Known

Generate ANonce

EAPOL-KEY (ANonce Unicast)

Derive PTK

EAPOL-KEY (SNonce Unicast, Encrypt NTK-mp1,

Encrypt MTK-m (aspirant))

Derive PTK, MTKm

If needed GTK,

NTK-mp2

EAPOL-KEY (Encrypted NTK-mp2, Encrypted GTK, Encrypted MTK-n (authenticator), Install PTK, MIC)

Install PTK, NTK-mp1, NTK-mp2, GTK, MTK-n, MTK-m

Install PTK, NTK-mp1, NTK-mp2, GTK, MTK-n, MTK-m

Install

### MTK Set-up Group

- Multicast Group Data

- Encrypted/De-Encrypted at A,B,C only

- Only A,B,C need to obtain the MTK

- PF between A,B,C only forward encrypted multicast Data

- Multicast group is detected on MP & secure flag is set on multicast group

- MAC address is included in Hello frame & distributed in routing protocol with flag that indicates the group is secure/pending

- Mesh members are detected without multicast data flowing  (nodes A, B and C in example)

- Mesh association begins from each node

- Multicast group leader initially secures the multicast group

- Aspirant-MP sends MP association to the Group Leader

- With RSN-ie & M-RSN-ie hints

- EAPOL occurs

- M1, M2, M3, M4 steps occur to install (MMK) and MTK for this group

- Mesh Group members are signaled

- Mesh Group members are signaled that MP member has arrived via the routing message in the Group MAC message

## Key Update and Re-Key

Keys used in mesh are updated based on:

- a pre-configuration time interval,

- change of MP membership

- A change of multicast group membership, or

- A SME event (such as a IDS suspicion of intrusion.)

### Re-keying based on a timer or change in MP membership

For example, a pair of neighbors is re-associated after a certain period of time to re-established M-PMK (after a 802.1x re-authentication), and M-PTK (after a 4-way handshake).

M-GTK is updated based on a pre-configured interval. The responsibility of initiating an update of M-GTK is that of a pre-elected MP-Secuiriy Designated Router. The MP-Security Designated Router is determined by the following process:

- Elect the MP with the associated AS with the highest priority

- If a tie occurs between AS values, use the MP with the lowest Mesh Point ID

- AS prority of zero is not used, but reserved for MP’s with no AS association.

- Elect a Back-up MP-Security Designated Router as runner up based on the same priority scheme.

- If the main MP fails, switch over to the Back-up.

- If a new MP arrives with a better AS value, switch to that MP.

- During a grace period of the switch over process between MP Security DRs (XXX time), obey the both BDR requrest for re-keying

- If no MPs have an Associated AS, pick the lowest Mesh Point with AS priority of zero (no AS).

A group key update message is sent from the MP-security DR to the rest of the MP in the same mesh. This message is protected by the existing M-PTK between each pair of the mesh points. If M-NTK is available, it can be used in protecting the new M-GTK when a MP multicasts the group key update message to its neighbor.

M-NTK can be updated the same time during mesh re-association. It can also be updated by a MP to all of its neighbors in a simple 2-way (update/ack) handshake. This handshake is protected by the existing M-NTK.

M-MTKm is updated by the Group leader for the MP-MTKmac address. The group leader is selected by:

- First MP with Group MAC MTK with Security Group leader flag,

- If two MPs are announced with the Group leader flag, a leader election election occurs using the: Highest AS priority (0 = none, 1-n). If a tie occurs, the lowest mesh point ID win breaks the tie.

- Using this election algorithm, a Group leader is elected.

- If the Group leader fails, the Back-up Group leader takes over being the secure point.

- If a better group leader arises, the new group leader will be elected.

The switching a MTK Security Group leader involves allowing a certain time for the routing system to distribute the information on the Group leader. The Former group leader will respond to security requests for a period of time.

#### Re-keying times

| Key | Established | Initial delay for association | Impact of short term failures | When it will be dropped | When key drop timer started | Timer for dropping |
| --- | --- | --- | --- | --- | --- | --- |
| PTK | After association | yes | Keep | Link Inactive, Down | Local Link Down | Local Link Down |
| GTK | After association | Yes | Keep | Link Inactive Down | All Local links down | Local links Down timer On last link |
| NTK | After association | Yes | Keep | Link Inactive down | All Local Lnks down | Local Links down timer on last link |
| MTK | Association required when member joins the multicast group Re-association required when Multicast member leaves & drop timer expires | Delay occurs after initial set-up | Keep Keep | Pre-Configured: Not dropped Not pre-configured: MAC address disappears for Secure MAC down key | Pre-configured: Only when key cache gets to big Not pre-configured: MAC disappears for “Secure MAC down” timer | pre-configured: no drop. Not pre-configured: MAC disappears for “secure MAC down timer” |

### Re-keying times during failures

| Key | Node connection established re-keying times | Initial delay for association | Impact of short term failures | When it will be dropping | When key dropping Timer started | Timer for dropping |
| --- | --- | --- | --- | --- | --- | --- |
| PTK | With beacon as part of hello | yes | Keep | Link Inactive, Down | Local Link Down | Local Link Down |
| GTK | With Beacon as part of hello | Yes | Keep | Link Inactive Down | All Local links down | Local link Timer on last link |
| NTK | With Beacon as part of hello | Yes | Keep | Link Inactive Down | All Local links down | Local link Timer on last link |
| MTK – on-demand | Association required when member joins the multicast group Re-association required when Multicast member leaves & drop timer expires | Delay occurs after initial | Keep | Pre-configured: not dropped | Pre-configured: when cache exceeded | Secure MAC down |
| MTK pre-configured | Association required when member joins the multicast group Re-association required when Multicast member leaves & drop timer expires | No delay as is set up prior to first establishment | Keep | Pre-configured: not dropped | Pre-configured: when cache exceeded | Secure MAC down |

### Change in multicast membership

An MP can learn about new multicast address based on Layer 2 data packet snooping or beacon information or routing protocol information.  Upon detection a Group MAC address that policy indicates should be encrypted, the MP can request a re-association to obtain the MMK key.

Optional an MP may gather MAC address until its next re-keying interval.

### SME event

A MP SME can determine that re-keying is critical based on implementation specific information.  A few types of events that may cause this are reconfiguration or evaluation of statistics.

## Optional MD5 Authentication Methodology

To ensure the integritity of mesh management frames during mesh association process, an optional MD5 authentication can apply to mesh beacon frame, mesh probe request/response frames, and mesh association request/response frames. A comman pre-shared key needs to be configured for all MP’s for this purpose.

The following diagram shows an example of mesh neighbor discovery with MD5-Authenticated management frames.

# Annex A (informative)

## IEEE 802.11 Contention Free Period

As the non Mesh stations use the listen-before-talk scheme, CSMA/CA, a backoff is included with each frame transmission. Multiple hops include multiple backoffs, increase the overhead and decrease the efficiency dramatically, therefore. Since the legacy 802.11 devices perform an aggressive channel access, the prioritization of the Mesh traffic is impossible without a dedicated logical channel for the Mesh WLAN. The CFP provides such a logical channel for the Mesh WLAN.

The Coordination Function (CF) used during the CFP is called Point Coordination Function (PCF) in contrast to the Distributed Coordination Function, which is used during the CP. “The DCF and the PCF shall coexist in a manner that permits both to operate concurrently within the same BSS.” [10].

All 802.11 stations respect the CFP as a period during which no transmission may be initiated by any other station than the Point Coordinator (PC), see 9.3 in [10]. Usually the PC is collocated with the AP. Having learned from a beacon frame about a CFP, all 802.11 stations refrain from channel access. During a Contention Free Period (CFP), only those stations which are informed to receive frames, because the AP has data buffered for them, must stay in awake state, see 11.2.1.5 and 11.2.1.7 in [10]. All other stations may switch to sleep mode. It is the PC only, which initiates frame transmissions to the stations. Furthermore, the PC may poll those stations, which are CFPollable by either sending explicit poll frames or piggyback a poll frame to a data frame, to allow stations to start a transmission. Since no station will access the wireless medium during the CFP without being polled by the PC, the CFP is ideal for coordination and frame exchange of an AP based Mesh WLAN.

Since with 802.11, a superframe may consist of a Contention (CP) and a Contention Free Period (CFP), the CFP is used for a collision free coordination of the Mesh WLAN. The CP uses 802.11 medium access functions as DCF, EDCA and HCCA. During the CP APs serve their associated stations and stations send frames to the APs. The CP is fully compatible with all legacy and 802.11e stations.

uses a modified 802.11 MAC protocol. It avoids frame collisions in total. The CFP. The CP and the CFP together form the Superframe known from standard 802.11. A superframe has a fixed length of mSuperframeSize. The fraction of this superframe that is used for the MTP must be in between mMTPMinTime and mMTPMaxTime. The duration of the CFP is not restricted furthermore.

Figure 10-52: As the basic configuration for simple Mesh WLAN Access Points, a single radio is sufficient to provide the AP services to the stations and to be able to participate in the Mesh WLAN.

The periodic scheduling mode is capable to efficiently work with limited resources. It enables Mesh WLANs using single or multiple channels, starting with a single radio, see Error: Reference source not found. Its flexible design allows seamless integration of more resources (channels and radios) as they are available.

Non-QoS stations (see 3.71 in [11]) may foreshorten a CFP. 802.11 stations do not calculate the duration of any of their transmissions in advance. Since 802.11 stations may access the channel shortly before the beginning of a CFP, their transmissions may delay the beginning of the BP. In 802.11 it is foreseen that an AP must sense the wireless medium at TBTT to make sure it is idle. If the wireless medium is detected as busy, the AP refrains from channel access and defers the beacon transmission until the channel becomes idle. Since the end of the CFP is fixed as it was announced with previous beacons, a delayed beacon transmission incurs a foreshortened CFP.

## Beacon generation and application in 802.11

According to standard 802.11, each Superframe starts with a Beacon frame transmitted at Target Beacon Transmission Time (TBTT). In IBSS mode each station is responsible for the generation of a beacon frame. Hence, a distributed algorithm is used. In an infrastructure wireless network, it is the AP’s responsibility to generate a beacon frame. The Beacon frame consists of several information elements. Some of them are mandatory others are optional. As a common element, the Beacon frame is used for synchronization. In an IBSS each station adjusts its clock to the fastest one. In an infrastructure network, stations synchronize to the AP’s timing information. The timing information is especially important for WLANs, which need a synchronized timing as the Frequency Hopping Spread Spectrum (FHSS) PHY defined in 802.11 for example.

Furthermore the Beacon frame is used to organize the Contention Free Period (CFP). It announces the beginning of a CFP, its duration and after which of the following beacons the next CFP will start. At none, each or an arbitrary repetition interval a CFP may exist in a superframe. As defined in standard 802.11, all stations, which receive the beacon frame, respect all CFPs announced herein, regardless if a neighboring AP or the APs of their own BSS declared the CFP, see 11.1.2.3 and 9.3.2.2 in [10]. Furthermore, any station stores the start timet of any next CFP. Stations will refrain from channel access, even if no beacon was received at the beginning of the next superframe/CFP. Thus the start time of the next CFP is defined too.

## Concurrent transmission on the same frequency channel

The DRCA allows an efficient multihop communication in the Mesh WLAN. The usage of negotiated ownerships of equal duration MTXOPs result in a predictable medium access, as all neighboring Mesh Points are able to learn which Mesh Point plays which part during a MTXOP.

This enhanced knowledge allows the DRCA to allow a greater spatial reuse, which directly is followed by a capacity increase of the mesh network.

A simple example for the possibilities of spatial reuse can be found in Figure 10-53.

Figure 10-53: A simple scenario where spatial reuse is possible

Mesh Points “1” to “4” have their own BSS and probably several associated mobile stations. The mobile stations in the BSS of Mesh Point “1” generate traffic which is addressed to Mesh Point “4” (which is for example a gateway or portal to the internet), and Mesh Point “4” replies to the traffic.

As Mesh Point “1” and “4” are mutually out of reception range, they cannot communicate directly with each other. They must use two three hop routes via Mesh Point “2” and “3”, which is depicted as (1a-c) and (2a-c).

If Mesh Point “3” is able to guess that simultaneous usage of link (1a) and (2c) is possible because the interference created by Mesh Point “1” at Mesh Point “3” during the transmission is low, it may negotiate with Mesh Point “4” the number of used MTXOP to be the same as they are used for the link (1a). The latter information is directly available to Mesh Point “3” via the negotiation procedure between Mesh Point “1” and Mesh Point “2”.

Similarly, links (1c) and (2a) can be used simultaneously, which results in a traffic/time diagram as given in Figure 10-54.

Figure 10-54: An optimal alignment of the transmissions during time for scenario in Figure 10-53.

The described scenario is an example for an optimal behavior of the Mesh Points which can be seen from an external observer, but it is not obvious how the Mesh Points can reach this behavior. The possible internal mechanisms of the Mesh Points are explained in the next section.

## Learning Mesh Points

Before Mesh Points can take advantage of simultaneous, concurrent transmissions, they must learn a model of the current environment, called the world model. This world model shall be as simple as possible, abstracting from reality as much as possible. Also, it shall be as detailed as needed to give good estimations of the options of a specified transmission. The world model is updated continuously by the sensors of a Mesh Point, which are the receiving entity of the physical layer together with the information about the MTXOP ownerships, received beacons, information elements and heard transmissions.

From time to time, a request for a new MTXOP ownership or a change of an existent one arises in the Mesh Point, for example because a new traffic stream is started by an associated Mesh Point or a MTXOP ownership negotiation request is received by a neighboring Mesh Point. This request is processed using the world model to find free MTXOPs that suit the current status regarding the intended role (transmitter or receiver) and the priority of the traffic.

With this information, the MTXOP negotiation process selects a suitable set of MTXOPs and starts the negotiation process (or answers the request respectively), probably preferring MTXOPs that lead to a simultaneous transmission.

The abstracted structure of a station which is able to adapt to the current interference can be seen in Figure 10-55.

Figure 10-55: The general structure if an interference-aware Mesh Points

## DEVID addressing

The expected number of Mesh Points in a typical scenario, for example a campus environment, is typically less than or equal to 32. Therefore, a six octet field (like it is used in 802.11) for addressing traffic between Mesh Points is unnecessarily large. As long as it can be ensured that every Mesh Point in the mesh network has its own unique identifier, a shorter identifier may be used. The source and the destination of a data packet in the network are still addressed using the common address. However, any intermediate Mesh Point uses a mDevIdBits bit Device Id (DEVID) as the transmitter and the receiver address during the MTP. Hence a function to map MAC to DEVIDs is needed.

## Beacon dissemination

Figure 10-56: While MP1 sends a beacon, the beacon period access protocol has to avoid interference by MP4

Figure 10-56 presents an exemplary scenario. Here, the situation on the wireless medium is shown during a BTS, which is occupied by Mesh Point "1". It is important that all Mesh Points (MPs) and non-802.11s-stations in the transmission range of a Mesh Point (MP) correctly receive its beacon frame. A beacon collision may occur at a receiving station if a Mesh Point (like MP 4) transmits a beacon simultaneously during the same Beacon Transmission Slot (BTS).

In the scenario in Figure 10-56, Mesh Point "1" sets the beacon information to BTS type 1. Mesh Point "2" sets it to type 2, and so on. Mesh Point "2" propagates this information in its beacons. As station "4" receives the interference information about this slot from Mesh Point "3", it knows that it shall not send in this BTS, because Mesh Point "3" might not be able to receive the beacon successfully, or the beacon may collide with another beacon.

## Alternative Beacon Timing Structure

1.	The BP Structure can be simplified if zones of different beacon frame durations are prohibited. MPs may use several independent BTSs to transmit all necessary IEs.

2.	As an alternative proposal for the BP, we consider an arbitrary alignment of the beacons during the BP. If a Mesh Point stops sending beacons, a gap of one or several continuous slots is created, which should be filled by existing beacons either by sliding (if the beacon is situated directly after the gap) or by jumping. To be efficient, large jumps shall be preferred over several consecutive small jumps. This can be done by a last-come-first-served strategy during the processing of the intentions to change to this beacon slot: The Mesh Point that sends the last beacon in the BP in which it announces the new ownership of the free slots will get the slots.

Associated 802.11 legacy stations are able to receive the legacy 802.11 beacon IEs and ignore any other, e.g. new IE. From the legacy beacon IEs, 802.11 STAs identify the start and the duration of a CFP during which they will refrain from channel access.

The beacon transports information elements (IE), which are used to coordinate the beacon period access protocol (BPAP) and the Mesh Traffic Period (MTP).

It prohibits associated stations from channel access during the current and the next CFP.

Figure 10-57: The four steps of a BP contraction

Figure 10-57 shows an example of beacon shifting: First the BTS “3” is sensed as unoccupied by the Mesh Point with DEVID “8”. Furthermore MP “8” knows that it is the Mesh Point (MP), which sends the last beacon during the BP. Therefore it tries to occupy BTS “3”. Therefore MP “8” reserves the BTS in Figure 10-57b. In the next BP, it is sends its beacon in BTS “3” and announce its departure for BTS “10”. Finally, in Figure 10-57d the shift is completed successfully.

## Measuring the Learning Performance

As the scope of this chapter is limited to the ability of simultaneous transmissions, this will be the only quality measure; other criteria that involve the optimal selection of MTXOPs under fairness conditions or QoS requirements like throughput and delay are not discussed. Therefore, the algorithm which chooses and negotiates the MTXOPs is handled as a black box which gets a set of MTXOPs that could be suitable for a specified transmission to/from a Mesh Point, optionally combined with a rating of each MTXOP. As a result, the performance of the learning algorithm can be measured by the number of “good” MTXOPs it proposes to this black box, compared to the number of “bad” MTXOPs.

To define the terms “good” and “bad” MTXOP more precise, the Figure 10-58 presents an example.

| (a) | (b) |
| --- | --- |

Figure 10-58: RSS measurement while (a) Tx transmits or (b) Rx receives.

Both subfigures show an example environment with 11 Mesh Points, two of them are marked as the transmitting and the receiving Mesh Point respectively. In the left figure, the transmission power of the transmitting Mesh Point is drawn in red color. Its strength is proportional to the distance to the Mesh Point. The right figure shows the transmission power of all stations in the environment as it is seen from the Rx Mesh Point. In both cases, a green line indicates the traffic from the Tx to the Rx Mesh Point.

The decision if a MTXOP is “good” must be made regarding the desired role of the Mesh Point: If a Mesh Point wants to transmit, a MTXOP is “good” if it does not disturb a simultaneous transmission by its interference. With the power that is indicated in Figure 10-58a, the transmitting Mesh Point would certainly infer with any transmission that is received at the Mesh Points “1a-1c”. The impact on a reception in Mesh Points “2a-2d” would be much lower; a transmission from Mesh Point “2b” to “2a” should be no problem; whereas Mesh Points “3a-3b” would not sense anything from the transmission. Additionally, the effect of the transmission depends not only on the distance to the other Mesh Point, but also on the position of the simultaneous transmission’s sender: It is less interfering if the distance from the sender to the transmitter is very small.

The second case, indicated in Figure 10-58b, would be if the role of the Mesh Point wants to receive. A MTXOP is now called “good” if in the same time a simultaneous transmission creates only low interference at the receiver. This is for example the case if Mesh Points “2a-2b” or “3a-3b” are sending.

In the drafted environment some simplifications are made, as the shape of the signal strength may be more complicated than a circle around the sending Mesh Point. Furthermore, the shape may not be constant during time. Moving obstacles or different channel conditions can change the effects of a transmission.

### The World Model

The task of the world model inside the learning Mesh Point is to represent the environment in the simplest way that allows a good prediction if a given MTXOP is “good” or not. The detailed implementation of the world model, which also includes how the outputs of the sensors are used to update its state, is of course independent of the protocol specifications, and can be optimized to fulfill different aims; for example a trade off between the needed complexity, the used computational effort and the accuracy of the predictions must be made.

The world model is limited by the potential and the accuracy of the given sensors. An optimal model in the case discussed here would know the position of all Mesh Points in the network, as well as the link characteristics between them and the placement of any obstacles. This situation is of course out of reach, as some of the knowledge can only be obtained by much overhead traffic (for the mutual link characteristics) or is unachievable at all (like the obstacles).

The following world model is therefore only a proposal that relies on the described Mesh MAC protocol and some of the information that can be obtained as a side product of it.

It is derived from the fact that in wireless networks the success probability of a transmission is mainly determined by the ratio of the useful signal strength at the receiver versus the strength of the interfering signals. The two possible reasons for interference are the background noise and simultaneous transmissions. Therefore, this ratio, the Carrier over Interference (CoI), is measured as

.

C is the carrier's signal strength, N the current noise and the sum stands for the interference which is produced by other transmissions. Usually, if a simultaneous transmission is existent; therefore, the noise can be neglected in the non trivial cases.

It is important to notice that two different CoI ratios have to be taken into account before a new, simultaneous transmission is started:

1. The receiver CoI

This CoI reflects the success probability that the receiver of a simultaneous transmission is able to decode the signal in spite of the primary transmission.

2. The interference CoI

By introducing a new simultaneous transmission, the transmitter creates a new source of interference for the primary transmission. Therefore, both Mesh Points of the new link have to avoid that this new interference is severe at the original receiver.

In this proposal, the current status of the world is represented by the signal strength graph, which is a complete graph G = (V, E) together with a weight function w: E -> N that connects an integer to every edge of the graph. Any Mesh Point that is recognized by a sensor (like the Rx entity or the beacon protocol) is represented as a node in the graph. The weight of an edge between two nodes (X, Y) is an estimation of the signal strength that is measured at node Y if node X is sending data. As the links between nodes are by assumption bidirectional, w (X, Y) = w (Y, X) and the graph can be undirected.

A simple example is given in Figure 10-59: The complete graph is given for the five Mesh Points Tx, Rx, 1, 2 and 3, and the signal strength is abstracted as an weight of the connecting edge.

Figure 10-59: The signal strength graph for scenario with stations Tx Rx and 1 to 3

Having well trained this world model in every Mesh Point, it approximates the current state of the environment. Then, Mesh Points possessing this graph can compute an estimation of the interference CoI during a simultaneous transmission from Tx to Rx. Furthermore, the model can support the computation of the receiver CoI at Rx.

The interference CoI is estimated by dividing the weight of the link that represents the simultaneous transmission by the interference that is produced by Tx (given by w (Tx, [receiver of the simultaneous transmission]). The higher the quotient of those two weights, the lower is the chance that Tx interferes with the transmission.

Similarly the receiver computes the value of CoI as the quotient of w (Tx, Rx) and the interference of the simultaneous transmission, which is represented by w (Rx, Sender of the simultaneous transmission). A high indicator would here also express a high chance of a successful reception.

Of course the method can be extended to multiple simultaneous transmissions or to multiple receiver transmissions.

An algorithm can compute the CoI for every possible simultaneous transmission and then rate all MTXOPs given the information about the current ownerships using the ownership protocol as a sensor. Using this graph, the outcome is a list of “good” MTXOPs, which are likely to provide high success of reception and a low interference ratio to other transmissions in parallel. Furthermore, a threshold may be given which determines whether the computed CoI ratios high enough. Alternatively, the decision can be made based upon a (learnable) soft threshold function like the sigmoid function ( ).

The computed indicators for the transmission Tx to Rx in the given example graph can be seen in Table 10-3, all impossible pairs of transmissions (like Tx -> Rx and Rx -> 2 in the same time) are omitted.

Table 10-3: Interference CoI and Receiver CoI if a simultaneous transmission from Transmitter to Receiver would happen

| Transmissions in MTXOP | Receiver CoI [dB] | Interference CoI [dB] |
| --- | --- | --- |
| None | 0 | maximum |
| 1 -> 2 | 4 | -2 |
| 2 -> 1 | 10 | -14 |
| 1 -> 3 | 4 | -1 |
| 3 -> 1 | 13 | -16 |
| 2 -> 3 | 10 | 5 |
| 3 -> 2 | 13 | 1 |

This table clearly shows that the transmission Tx -> Rx cannot be scheduled simultaneously to most of the other possible transmissions, perhaps only parallel to the transmission 2->3. A different case can be seen if the graph of the introduction example (Figure 10-53) is examined, which is given in Figure 10-60.

Figure 10-60: The signal strength graph for the scenario given in Figure 10-53

If the transmission from Mesh Point 4 to Mesh Point 3 is scheduled in a MTXOP, the interference indicator for 1 -> 2 is 10*Log(60 / 15) = 6dB, and the reception indicator for 1 -> 2 is also 6dB, which may be rated as a “possible” MTXOP if a slow PHY mode is used.

Before the possible methods of learning the graph and the weights are presented, it has to be noticed that the abstraction which is done in the world model incorporates easily all kinds of transmission technologies like directed antennas or MIMO devices: If they improve the receiver CoI ratio and/or lower the interference CoI, their performance is directly incorporated into the model.

Similar, the effects of obstacles like walls indirectly influence the graph and are therefore also incorporated.

The continuous learning of the graph can be divided into two separate tasks: First, the graph's structure (V, E) has to be learned, which is the identification of the network's participants. Second, the weights in the graph are learned. Those two tasks are carried out continuously and with an adaptable speed, allowing the model to become a good approximation of the environment and reacting towards changes. The learning is made difficult by the insufficient and unreliable output of the three used sensors, as they are not made to fulfill the given task. A filtering of the sensor's output is therefore one of the most important subtasks of the learning process.

A last demand to the learning process is that it should recognize situations where its knowledge is insufficient to result good estimations for the two CoI values. In detail, it should be prevented that the interference CoI is overestimated and thus an existing transmission is disturbed.

### Learning the Network’s Participants

Recognizing other Mesh Points in the network can be done easily using the beacon period access protocol and by receiving other Mesh Point’s traffic headers. From the beacon protocol, a Mesh Point can identify the beacon’s sender, the sender’s neighbors and the neighbor’s neighbors, because each of them is announced in the owner vector of the BPOIE.

In the traffic during the MTP, each traffic train has an initial header which gives the structure of the following wagons, including the recipient of each wagon. Using this information, a Mesh Point can detect other Mesh Points by listening to the headers even if in the MTXOP it is not a receiver.

Each occurrence of a Mesh Points’s DEVID (either in the BP or during the MTP) can be seen as a “ping” indicating the Mesh Point being “alive”. It is recommended that the Mesh Points are included to the graph the first time a “ping” was heard from them, they should be deleted from the graph with a probability that increases with the time no “ping” has been heard.

### Learning the Signal Strength

For every new Mesh Point that is recognized, the weights to the other Mesh Points have to be estimated, which is done is several ways. Each sensor gives some hints how the weight should be set. The sensor’s outputs are noisy and have to be filtered or weighted before they can be taken into account.

If the current graph consists of N Mesh Points, (N+1) * N/2 weights have to be estimated. Of those links, (N – 1) are directly connected to the learning Mesh Point. Therefore, they can be learned faster and with more confidence. It is noteworthy that in the interference and in the receiver CoI, three out of four needed weights are direct links of either the transmitter or the receiver; only one weight in the interference CoI is in a one hop distance of one of them, as this weight describes the signal strength of the primary transmission measured at the primary receiver. To avoid overestimating the interference CoI, the lower bound of this weight is crucial.

Learning (N – 1) direct links can be done by using the timing information in the beacon access period protocol together with some side information by the PHY layer. Using the BP, a Mesh Point knows the point in time when a neighboring Mesh Point is transmitting its beacon. Furthermore, because of the strict rules in the BP, it knows that no other near Mesh Point is transmitting during this time.

For each beacon slot, the PHY layer can measure the integrated signal strength, and then report this strength to the MAC layer, which combines this information with the BP access protocol to determine an estimation of the signal strength of a particular neighbor.

The weight on the link can now be computed using this estimation. The easiest solution would simply take the most current estimation, neglecting older values. Another, more intelligent solution would be a low pass filtering of the estimates to obtain a running exponential weighted average. If the newest measurement, obtained in the beacon period number t, is denoted as , the running estimation is computed as

withas a parameter weighting the importance of new measurements versus the old knowledge. This solution would of course solve the problem of short noisy measurements, although it increases the computational complexity.

Finally, a third possibility is the usage of a one dimensional Kalman filter to obtain an incremental estimation using the measurements. A Kalman filter assumes an additive white Gaussian noise with an unknown variance as an error on the PHY measurements; it can compute the current expected “real” signal strength together with the variance that it assumes together with this estimation. An advantage of the Kalman filter is that it weights the influence of new measures proportional to the current degree of believe of the estimation. Therefore, it can be seen as an enhancement of the exponential weighted average: In the latter case, all measurements are weighted with the same . In contrast, the Kalman filter adapts thie coefficients to the current variance.

The increased computational complexity in comparison to the exponential weighted average is an obvious downside of the Kalman filter.

Using one of the described mechanisms, the learning Mesh Point is able to learn the weight of all direct links whereas all other links remain unknown. As it was explained above, an estimation of the lower bound of the weight of the other links suffices for a good interference CoI computation; therefore, two different methods with different complexity can be used.

The first method is explained with the help of Figure 10-61. In this very simple scenario, Mesh Point “2” wants to initialize a transmission which is simultaneous to the transmission (1) from Mesh Point “3” to Mesh Point “4”. Therefore, it has to compute the interference CoI, which needs a lower bound of the signal strength that is detected at Mesh Point “4” if Mesh Point “3” is transmitting.

Figure 10-61: Mesh Point "2" wants to learn the signal strength of route (1)

Here, the medium access protocol during the MTP can be used as a simple sensor to get information about this signal strength. In the train header for each receiver the used PHY mode is indicated. As the train header is send in the basic PHY mode, chances are high that Mesh Point “2” can understand this header and therefore it knows the used PHY mode. As fragile PHY modes can only be used successfully if the signal strength at the receiver is above a minimum threshold, Mesh Point “2” can conclude the minimum signal strength, which suffices for the CoI. Table 10-4 shows the minimum signal strength in dBm for the different 802.11a PHY modes.

Table 10-4: The minimum signal strength for the successful reception, depending on the PHY - mode

| PHY mode | Minimum C (dBm) |
| --- | --- |
| BPSK ½ | -82 |
| BPSK ¾ | -81 |
| QPSK ½ | -79 |
| QPSK ¾ | -77 |
| 16QAM ½ | -74 |
| 16QAM ¾ | -70 |
| 64QAM 2/3 | -66 |
| 64QAM ¾ | -65 |

The other possible method results in more overhead because it uses special IEs to disseminate the information about the received signal strengths over the network. This signal strength IE consists only of three fields: The Mesh Point where the signal is received, the transmitting Mesh Point and finally an 8b value expressing a lower bound on the signal strength.

The lower bound can be obtained by the estimation of direct links at it was discussed earlier, especially if a Kalman filter was used: Together with the variance, a confidence interval can be computed for the estimation, and the lower limit of this interval can be disseminated.

The frequency of sending of SSIEs should be very low; additionally, it is possible to adapt it to the behavior of the link, e.g. information about a steady, only slightly changing link is disseminated fewer than information about a fluctuating link. Furthermore, information about a link should not be send at all if the current knowledge is not very profound.

A the data in a received SSIE can be handled with more trust than data from the sensors about direct links, as it was already filtered and only the lower limit was sent. Therefore, a low pass filtering of the data with a high alpha should be sufficient. A station may decide whether to resent a received SSIE or to drop it. The probability of dropping the SSIE should be anti-proportional to the maximum direct link strength to the mentioned stations in the SSIE, as the information becomes irrelevant for Mesh Points that are even farer away.

The time flow during one Mesh Traffic Period (MTP) from the viewpoint of a Mesh Point (MP) is drafted in Figure 10-62.

Figure 10-62: Structure of the CFP with Mesh Traffic Period and Beacon Period

At the beginning of the Mesh Traffic Period (MTP), each Mesh Point (MP) sends a beacon, in which it states the ownership of one or more of the following MTXOPs, which have already been negotiated. After the BP, the owners may transmit in the appropriate MTXOPs.

Each transmission during the MTP shall start at the beginning of a reserved MTXOP. However, the transmission begin may be deferred, if the MPs chooses to do so. It is recommended either to start the transmission at the beginning of the first owned MTXOP or to align it so that it ends simultaneously with the last owned MTXOP, which could be helpful for simultaneous transmissions in the same MTXOP, as it may shorten the duration of the interference.

Because of the ownerships of MTXOPs, the receiver is not allowed to send an ACK frame immediately after the reception of the data frame. In a multi reception MTXOP this would be impossible. Furthermore, immediate acknowledgments would imply a change of the transceiver/receiver roles which is unpredictable for the neighboring Mesh Points.

Additionally, congestion control like schemes can be used by the transmitter to estimate the optimal number of packets he can send before waiting for the next ACK.

Note: In the case of a multi-hop route, the buffer size indicator might be useful to avoid bottlenecks.

In this way, the information about congestion travels from the congested node back to the source.

Additionally, an Availability IE might be included before or during the negotiation to improve the speed of MTXOPs selection, which are free for all participants.

e can be determined for any message sequence that an implementation may use.