IEEE P802.11
Wireless LANs

| Siemens Partial Proposal for WLAN Mesh Networking |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-06-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Bahr | Siemens AG, Corporate Technology | CT IC 2, Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-49926 | bahr@siemens.com |
| Norbert Vicari | Siemens AG, Corporate Technology | CT IC 2, Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-44703 | norbert.vicari@siemens.com |
| Christian Schwingenschlögl | Siemens AG, Corporate Technology | CT IC 2, Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-44168 | chris.schwingenschloegl@siemens.com |
| Matthias Kutschenreuter | Siemens AG, Corporate Technology | CT IC 2, Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-51744 | matthias.kutschenreuter@siemens.com |
| Michael Finkenzeller | Siemens AG, Corporate Technology | CT IC 2, Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-42725 | michael.finkenzeller@siemens.com |

Abstract

This document is the “Siemens Partial Proposal for WLAN Mesh Networking” in response to the Call for Proposals of the IEEE 802.11s task group. We consider a mainly unmanaged WLAN mesh network. All mesh points have to authenticate in order to become part of the WLAN mesh network. Mesh points can be both access point devices and client devices. Our proposed WLAN mesh networks with wireless multi-hop communication can be deployed in different usage scenarios: residential and home networks, office networks, campus, community, and public access networks, as well as public safety networks.

The proposal is centered around but is not exclusively about multi-hop path selection. A layer 2 adaptation of the AODV routing protocol is proposed. An important aspect is extensibility. Furthermore we provide contributions to security in WLAN mesh networks and to interworking.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Table of Contents

Table of Contents	1

Table of Contents	2

List of Figures	3

List of Tables	3

Abbreviations	4

1	Introduction	5

2	Glossary of IEEE 802.11s Terms	6

3	WLAN Mesh Path Selection	8

3.1	General	8

3.2	Routing Metrics	8

3.2.1	Routing Metric Identifier	8

3.2.2	Path Metric	8

3.3	Destination Sequence Numbers	9

3.4	Reactive Mesh Path Selection based on AODV	9

3.4.1	Route Request Information Element (RREQ)	10

3.4.2	Route Reply Information Element (RREP)	11

3.4.3	Route Reply Acknowledgement Information Element (RREP-ACK)	12

3.4.4	Route Error Information Element (RERR)	13

3.4.5	Routing Actions for Sending a Data Frame at the Source Mesh Point	14

3.4.6	Processing of Route Request Messages	16

3.4.7	Processing of Route Reply Messages	18

3.4.8	Generation and Processing of Route Reply Acknowledgement Messages	19

3.4.9	Generation and Processing of Route Error Messages	19

3.4.10	General Purpose Forwarding Table	21

3.5	Proactive Routing Towards Mesh Portals	21

3.5.1	Mesh Portal Initiated Routing Towards Mesh Portals	21

3.5.2	Mesh Point Initiated Routing Towards Mesh Portals	23

3.6	Multiple Interfaces per Mesh Node	23

3.7	Integration of Legacy IEEE 802.11 Stations (STA)	23

3.8	Integration of Vendor Specific WLAN Mesh Routing Protocols	24

4	Frame Formats	24

4.1	Mesh Data Frame Format	24

4.1.1	Frame Control Field	25

4.1.2	Sequence Control Field	25

4.1.3	Address Fields	25

4.1.4	Time-To-Live Field (TTL)	26

4.1.5	Mesh Data Frame Flags Field	26

4.1.6	Mesh Data Frame Option Field(s)	26

4.2	Control Frame Format and Management Frame Format	27

4.3	Beacon	27

4.3.1	Mesh Identity Information Element	28

4.3.2	Active Mesh Routing Protocol Information Element	28

4.3.3	Active Mesh Routing Metrics Information Element	29

4.3.4	Mesh Security Information Element	29

5	Data Frame Forwarding	29

6	Broadcast Data Service	30

6.1	Link-local broadcast	31

6.2	Mesh-wide Broadcast	31

7	Fragmentation and Defragmentation of Frames	32

8	Mesh Portal Functionality	32

8.1	Portal Behavior	33

8.1.1	Address Learning	33

8.1.2	Data Frame Forwarding from Mesh Point to Mesh Portal and Vice Versa	33

8.1.3	Unicast: From Mesh Point to Destination outside the Mesh	33

8.1.4	Unicast: From Node outside the Mesh to Destination inside the Mesh	33

8.1.5	Broadcast Transmission	33

9	Quality of Service	33

10	Security	33

10.1	Basic Authentication of Mesh Nodes	34

10.1.1	Central Model	34

10.1.2	Distributed Model	34

10.2	Efficient IEEE 802.11i in a Multi-hop Mesh Environment	34

10.2.1	Central Model	35

10.2.2	Distributed Model	35

11	Extensibility	35

12	Attribute Definitions	36

13	References	38

## List of Figures

Figure 1: Set diagram of station, mesh point, and mesh AP	6

Figure 2: Routing metric identifier format	8

Figure 3: Route request information element (RREQ) format	10

Figure 4: Flags of route request information element	11

Figure 5: Route reply information element (RREP) format	11

Figure 6: Flags of route reply information element	12

Figure 7: Route reply acknowledgement information element (RREP-ACK) format	12

Figure 8: Flags of route reply acknowledgement information element	13

Figure 9: Route error information element (RERR) format	13

Figure 10: Flags of route error information element	14

Figure 11: Mesh data frame format	24

Figure 12: Flags for mesh data frame flags field	26

Figure 13: Mesh data frame option field format	26

Figure 14: Broadcast identifier option format	27

Figure 15: Mesh identity information element format	28

Figure 16: Active mesh routing protocol information element format	28

Figure 17: Routing protocol identifier format	28

Figure 18: Active mesh routing metrics information element format	29

Figure 19: Mesh security information element format	29

Figure 20: Flags of mesh security information element	29

Figure 21: Usage of same PMK between different mesh points over multiple hops	35

## List of Tables

Table 1: Routing metric identifiers	8

Table 2: Fields of route request information element	10

Table 3: Fields of route reply information element	11

Table 4: Fields of route reply acknowledgement information element	13

Table 5: Fields of route error information element	13

Table 6: Values of RREQ message	14

Table 7: Values for routing table entry to originator	16

Table 8: Values of RREP message	16

Table 9: Different values of gratuitous RREP message	17

Table 10: Different values of RREP message by destination	18

Table 11: Values for routing table entry to destination	18

Table 12: Values of RREQ-ACK message	19

Table 13: Values of RERR message	20

Table 14: Values of RREP message by mesh portal	22

Table 15: Values for routing table entry to mesh portal	22

Table 16: Different values for RREQ message for mesh portal	23

Table 17: Valid type and subtype combinations (IEEE 802.11s extension)	25

Table 18: Mesh data frame options	27

Table 19: Routing protocol identifiers	28

Table 20: IEEE 802.11s attribute description	36

## Abbreviations

|  | AAA | authentication authorization and accounting |
| --- | --- | --- |
|  | AIFS | arbitration interframe space |
|  | AODV | Ad hoc On-demand Distance Vector routing protocol |
|  | AP | access point |
|  | BSS | basic service set |
|  | BSSID | basic service set identifier |
|  | CTS | clear to send |
|  | CWmax | maximum size contention window |
|  | CWmin | minimum size contention window |
|  | DA | destination address |
|  | DS | distribution system |
|  | DSS | distribution system services |
|  | EAP | Extensible Authentication Protocol |
|  | ESS | extended service set |
|  | FCS | frame check sequence |
|  | GMK | group master key |
|  | GTK | groupwise master key |
|  | IBSS | independent basic service set |
|  | ID | identifier |
|  | IE | information element |
|  | IEEE | Institute of Electrical and Electronics Engineers |
|  | IETF | Internet Engineering Task Force |
|  | IP | Internet protocol |
|  | IPSec | IP security |
|  | LAN | local area network |
|  | MAC | media access control |
|  | MCF | mesh coordination function |
|  | MP | mesh point |
|  | MSDU | MAC service data unit |
|  | OUI | organizational unique identifier |
|  | PHY | physical layer |
|  | PMK | pairwise master key |
|  | PSK | pre-shared key |
|  | PTK | pairwise transient key |
|  | QoS | Quality of Service |
|  | RA | receiver address |
|  | RERR | route error (message) |
|  | RFC | Request for Comments |
|  | RREP | route reply (message) |
|  | RREP | route reply acknowledgement (message) |
|  | RREQ | route request (message) |
|  | RTS | request to send |
|  | SA | source address |
|  | SN | sequence number |
|  | SSID | service set identity |
|  | STA | station |
|  | TA | transmitter address |
|  | TTL | time to live |
|  | TU | time unit (1024 µs) |
|  | WDS | wireless distribution system |
|  | WLAN | wireless local area network |
|  | WM | wireless media |

# Introduction

The wireless communication link in an IEEE 802.11 Wireless LAN is between a client device and an IEEE 802.11 access point. The client device is an IEEE 802.11 station (STA) that has to associate with the access point (AP) in order to communicate in the WLAN. The access points are connected with wires to other access points or to a wired LAN or the Internet.

A WLAN mesh network as to be defined within IEEE 802.11 task group “s” is based on wireless multi-hop communications. This provides increased flexibility and robustness. A node in a WLAN mesh network is called a mesh point. All mesh points in a WLAN mesh network are peers and can communicate with each other over multiple wireless hops. Further information on the anticipated architecture and properties of IEEE 802.11s WLAN mesh networks can be found in the corresponding documents of the task group, for instance, [4][5][7][8][9][10].

Mesh points can be both access point devices and client devices. Many client devices, such as laptops, are now so powerful that they also can provide access point services. A mesh point that also provides access point services is called a mesh access point. So, IEEE 802.11 stations can associate to these client devices with access point functionality.

According to the ideas of the IEEE 802.11s task group, the WLAN mesh network is located in the wireless distribution system. This is illustrated in more detail in [10]. This allows compatibility with traditional IEEE 802.11 stations. They can associate in the usual way with mesh access point. They do not have to be aware of the WLAN mesh network.

This document is the Siemens Partial Proposal for WLAN Mesh Networking in response to the Call for Proposals of the IEEE 802.11s task group [6]. We consider a mainly unmanaged WLAN mesh network, where it is not necessary to have a professional IT department for managing the WLAN network. All mesh points have to authenticate in order to become part of the WLAN mesh network. All mesh points are under a single “administrative entity”. This “administrative entity” might be a single person, who actually owns a WLAN mesh network.

Our proposed WLAN mesh network with wireless multi-hop communication can be deployed in different usage scenarios. It is flexible enough to support the following usage scenarios:

- residential networks / home networks

- office networks

- campus network / community networks / public access network

- public safety networks / spontaneous networking.

The proposal is centered around, but is not exclusively about, multi-hop path selection. We propose a layer 2 adaptation of the Ad hoc On-demand Distance Vector routing protocol [2] for path selection in the WLAN mesh network. An important aspect in our proposal is extensibility. Furthermore, we provide contributions to security in WLAN mesh networks and to interworking.

# Glossary of IEEE 802.11s Terms

The following definitions and descriptions of IEEE 802.11s terms are taken directly from the terms and definitions document of the IEEE 802.11 task group “s” [10]. Refer to this document for further information.

WLAN Mesh – A WLAN Mesh (previously known as ESS Mesh) is an IEEE 802.11-based WDS which is part of a DS, consisting of a set of two or more Mesh Points interconnected via IEEE 802.11 links and communicating via the WLAN Mesh Services. A WLAN Mesh may support zero or more entry points (Mesh Portals), automatic topology learning and dynamic path selection (including across multiple hops).

WLAN Mesh Services – The set of services provided by the WLAN Mesh that support the control, management, and operation of the WLAN Mesh, including the transport of MSDUs between Mesh Points within the WLAN Mesh. WLAN Mesh Services supplement DSS (Distribution System Services).

Mesh Point - Any IEEE 802.11 entity that contains an IEEE 802.11–conformant Medium Access Control (MAC) and Physical Layer (PHY) interface to the Wireless Medium (WM), that is within a WLAN Mesh, and that supports WLAN Mesh Services (see Figure 1).

Mesh AP - Any Mesh Point that is also an Access Point (see Figure 1).

802.11 Stations

Mesh Points

Access Points

Mesh Access Points

Figure 1: Set diagram of station, mesh point, and mesh AP

Mesh Portal - A point at which MSDUs exit and enter a WLAN Mesh to and from other parts of a DS or to and from a non-802.11 network. A Mesh Portal can be collocated with an IEEE 802.11 portal.

Mesh Link - A bidirectional IEEE 802.11 link between two Mesh Points.

Mesh Path - A concatenated set of connected Mesh Links from a source Mesh Point to a destination Mesh Point.

Mesh Path Selection – The process of selecting Mesh Paths.

Path Metric – Criterion used for Mesh Path Selection.

Mesh Topology – A graph consisting of the full set of Mesh Points and Mesh Links in a WLAN Mesh.

Mesh Neighbor - Any Mesh Point that is directly connected to another Mesh Point with a Mesh Link.

Mesh Unicast - Frame forwarding mechanism for transporting MSDUs to an individual Mesh Point within a WLAN Mesh.

Mesh Multicast - Frame forwarding mechanism for transporting MSDUs to a group of Mesh Points within a WLAN Mesh.

Mesh Broadcast - Frame forwarding mechanism for transporting MSDUs to all Mesh Points within a WLAN Mesh.

Mesh Management Message – Message defined for managing and operating the mesh. The message is sent between Mesh Points, e.g. for path determination, neighbor discovery, topology discovery, etc. This definition of message is intended to be generic and does not specify the form of conveyor.

Mesh Control Message - Message defined for controlling access to the WM (Wireless Media) used for communication among Mesh Points.

Mesh Unicast Acknowledgment – An acknowledgement sent back from destination Mesh Point to source Mesh Point of a mesh path for a unicast message.

Authenticated Mesh Point – A Mesh Point that has been authenticated as a valid participant in the WLAN Mesh. The authentication is with respect to a common policy determined by a single administrative entity.

Mesh Identifier – A unique identifier for a WLAN Mesh.

Mesh Discovery - A method to discover one or more WLAN Meshes.

Mesh neighborhood – A set of Mesh neighbors of a mesh point.

Mesh Performance Metric – A criterion used to evaluate the performance of the Mesh.

Mesh Link Metric – A criterion used to characterize the performance/quality/eligibility of a mesh link as a member of a mesh path. A mesh link metric may be used in a computation of a path metric.

Connected Mesh – The status of the WLAN Mesh in which all Mesh Points that are participating members of a WLAN Mesh are reachable.

Disconnected Mesh – The status of the WLAN Mesh in which a subset of Mesh Points that are participating members within the WLAN Mesh are not reachable. It is also called a partitioned Mesh.

Mesh Association - The service used to establish the Mesh Point membership within a WLAN Mesh. Mesh association is independent from STA association to an AP.

Mesh Member – An associated Mesh Point.

Mesh Member Set – The set of associated Mesh Points within a WLAN Mesh.

Mesh Service Area - The conceptual area within which members of a WLAN Mesh may communicate.

Mesh Coordination Function (MCF) - A logical function used to coordinate access of the Mesh Points (MPs) on the WM. The MCF is used for communication among MPs.

# WLAN Mesh Path Selection

## General

The WLAN mesh routing protocol is used for path selection. It is a reactive protocol, that is, it computes a path to the destination mesh point only on demand. The routing protocol is based on the Ad hoc On-demand Distance Vector routing protocol (AODV) as described in RFC 3561 [2]. However, the address format has been changed and some mechanisms have been adapted in order to use it for layer 2 routing in a WLAN mesh network.

Because the general principles of AODV are the same here and in RFC 3561 [2], the RFC 3561 on Ad-hoc On-demand Distance Vector routing [2] can be used for further information if necessary.

## Routing Metrics

Different routing metrics might be used in different usage scenarios. Therefore, a flexible mechanism is used that allows to use different routing metrics. This includes also vendor specific routing metrics.

Hop count is the traditional and basic routing metric. It is included as backup metric. However, it is recommend to use a more advanced radio-aware routing metric.

### Routing Metric Identifier

Each routing metric has a unique identifier (4 octets) as defined in Figure 2 below:

Figure 2: Routing metric identifier format

The values of the routing metric identifier are defined as shown in the following Table 1:

Table 1: Routing metric identifiers

| OUI | Routing Metric Sub Identifier | Meaning |
| --- | --- | --- |
| 00-0F-AC | 0 | not a routing metric or no routing metric given |
| 00-0F-AC | 1-255 | reserved |
| vendor OUI | 0-255 | vendor specific routing metric, it is recommended not to use 0 for a routing metric |

### Path Metric

During the routing process, a path metric is computed. This path metric is used to determine the least cost path or the suitability of a path.

The operations for computing the value of the path metric depend on the used routing metric. Some routing metrics are additive, some look for the link with the minimum/maximum value of the routing metric. The routing metric identifier defines the size of the value of the routing metric and the operations necessary to compute the value of the path metric during the routing process.

## Destination Sequence Numbers

Each mesh node has a generator for its destination sequence numbers. The destination sequence numbers are associated to routing control information. Because they are strictly monotonically increasing, they provide a chronological order of the routing control information. Only the newest information is considered, outdated information is discarded. This mechanism eliminates all problems caused by stale routing information usually associated with traditional distance vector routing protocols.

The destination sequence number is a 32-bit integer. Sequence number wrap around is handled by incrementing the destination sequence number as unsigned but comparing sequence numbers as signed (cf. [2]).

## Reactive Mesh Path Selection based on AODV

The routing protocol is based on RFC 3561 [2]. The address format has been changed, so that MAC addresses are used. Some mechanisms have been adapted in order to use it for layer 2 routing and in order to combine them with other MAC mechanisms.

The reactive mesh path selection becomes active only if a path to a destination node is needed. When a path to a destination is needed and there is no corresponding entry in the forwarding table, a route request message is flooded within the WLAN mesh. The destination node replies with a route reply message. The route request message sets up reverse paths towards the source node, the route reply message sets up the forward path from the source node to the destination node.

Destination sequence numbers establish a chronological order of the routing information. This ensures that older information cannot overwrite newer routing information. Loop freedom is achieved with this mechanism.

Routing messages are sent as management frames of subtype Action frame. The management frame body contains the actual routing message as an information element. Management frames have only 3 addresses in the MAC header (cf. 7.2.3 of [1]). Moreover, they are intended for single-hop communication, where the source address and the destination address are the same as the transmitter address and the receiver address. We follow the naming of [1] in the description of the routing management messages: source address (SA) and destination address (DA) mean the addresses of the transmitter and the receiver of the frame respectively.

The routing messages have to be protected (i.e. encrypted) in order to avoid unauthorized reception and generation of routing messages. This is absolutely necessary for a secure WLAN mesh network. Management frames, however, are not protected in the current IEEE 802.11 standard. IEEE 802.11 task group “w” (Protected Management Frames) is working on this problem. In this proposal, we assume that there are protected management frames available. For the case that this is not true, we see two alternatives:

Protected Information Element: A special information element is defined, that contains the protected (encrypted) original information element in its information field. The element ID indicates that the information field has to be decrypted.

Routing messages as frame body of protected mesh data frames: The MAC header of the mesh data frame indicates that the frame body contains an encrypted routing message. The mesh data frame is handled accordingly as a routing management message.

### Route Request Information Element (RREQ)

The route request information element contains the following fields as shown in Figure 3.

Figure 3: Route request information element (RREQ) format

There can be none or more than one pair of routing metric identifier – path metric value fields.

The meaning of the fields of the route request information element is as follows as listed in Table 2.

Table 2: Fields of route request information element

| Field | Description |
| --- | --- |
| element ID | Unique ID of the information element according to section 7.3.2 of the IEEE 802.11™ standard [1]. The actual value has to be assigned by IEEE. |
| length | Length of the information element in bytes according to section 7.3.2 of the IEEE 802.11™ standard [1]. |
| TTL | Time to live field with the same meaning as in IP |
| RREQ ID | route request ID: Unique identifier of the route request when taken in conjunction with MAC address of originator. RREQ ID is a sequence number (monotonically increasing). |
| flags | This octet contains flags that control the processing and propagation of the route request. Figure 4 shows the flags that are defined. |
| hop count | Number of hops from the mesh point which originated this route request (originator) to the mesh point handling this route request message |
| originator MAC address | The MAC address of the node which originated this route request. |
| originator sequence number | The current (destination) sequence number to be used in the route entry pointing towards the originator of the route request. This is the sequence number of the originator at the time it issued the route request. |
| destination MAC address | The MAC address of the destination node for which a route is desired. |
| destination sequence number | The latest sequence number received in the past by the originator for any route towards the destination (might not be known). |
| routing metric identifier | routing metric identifier according to the definitions in section 4.2.1 |
| path metric value | Cumulative value of the metric along the path found so far. This field is variable in length. The length and the operations for cumulating the metric are defined through the routing metric identifier. |
| … | further routing metrics (routing metric identifier and path metric value): Together with the 2 fields above, there can be n routing metrics (0  n) |
| no further routing metric | 00-0F-AC-00 (no routing metric) |

The following Figure 4 lists all the flags that are defined for controlling the processing and propagation of the route request.

Figure 4: Flags of route request information element

The flags have the following meaning:

| gratuitous route reply | If set, a gratuitous route reply is unicast to the destination node (see section 4.4.6) |
| --- | --- |
| destination only | If set, only the destination node is allowed to respond with a route reply to this route request. Intermediate nodes are not allowed to generate a route reply for this route request. |
| unknown destination sequence number | If set, the originator did not know a sequence number for the destination node |
| mesh portal | only routes to mesh portals requested |
| reserved | set to 0 and ignored on reception |

### Route Reply Information Element (RREP)

The route reply information element contains the following fields as shown in Figure 5.

Figure 5: Route reply information element (RREP) format

There can be none or more than one pair of routing metric identifier – path metric value fields.

The meaning of the fields of the route reply information element is as follows:

Table 3: Fields of route reply information element

| Field | Description |
| --- | --- |
| element ID | Unique ID of the information element according to section 7.3.2 of the IEEE 802.11™ standard [1]. The actual value has to be assigned by IEEE. |
| length | Length of the information element in bytes according to section 7.3.2 of the IEEE 802.11™ standard [1]. |
| TTL | Time to live field with the same meaning as in IP |
| flags | This octet contains flags that control the processing and propagation of the route reply. Figure 6 shows the flags that are defined. |
| hop count | Number of hops from the destination mesh point towards the originator as collected during the propagation of the route reply. |
| originator MAC address | The MAC address of the node which originated the route request to which this route reply is a response. |
| destination MAC address | MAC address of the destination node for which a route is supplied. |
| destination sequence number | The destination sequence number associated to the supplied route. |
| lifetime | The time in milliseconds for which nodes receiving the route reply consider the route to be valid. |
| routing metric identifier | routing metric identifier according to the definitions in section 4.2.1 |
| path metric value | Cumulative value of the metric along the path determined during the route request. This field is variable in length. The length is defined through the routing metric identifier. |
| … | further routing metrics (routing metric identifier and path metric value): Together with the 2 fields above, there can be n routing metrics (0  n) |
| no further routing metric | 00-0F-AC-00 (no routing metric) |

The following Figure 6 lists all the flags that are defined for controlling the processing and propagation of the route reply.

Figure 6: Flags of route reply information element

The flags have the following meaning:

| acknowledgement | If set, an acknowledgement to the route reply is required (see section 4.4.8) |
| --- | --- |
| mesh portal | If set, indicates that destination is mesh portal |
| reserved | set to 0 and ignored on reception |

### Route Reply Acknowledgement Information Element (RREP-ACK)

The route reply acknowledgement information element contains the following fields as shown in Figure 7.

Figure 7: Route reply acknowledgement information element (RREP-ACK) format

The meaning of the fields of the route error information element is as follows as listed in Table 4.

Table 4: Fields of route reply acknowledgement information element

| Field | Description |
| --- | --- |
| element ID | Unique ID of the information element according to section 7.3.2 of the IEEE 802.11™ standard [1]. The actual value has to be assigned by IEEE. |
| length | Length of the information element in bytes according to section 7.3.2 of the IEEE 802.11™ standard [1]. |
| flags | This octet contains flags that provide further information about the route reply acknowledgement message. Currently, there are no flags defined. |

The following Figure 8 lists all the flags that are defined for controlling the processing and propagation of the route reply acknowledgement. Currently, there are no flags defined.

Figure 8: Flags of route reply acknowledgement information element

The flags have the following meaning:

| reserved | set to 0 and ignored on reception |
| --- | --- |

### Route Error Information Element (RERR)

The route error information element contains the following fields as shown in Figure 9.

Figure 9: Route error information element (RERR) format

The meaning of the fields of the route error information element is as follows as listed in Table 5.

Table 5: Fields of route error information element

| Field | Description |
| --- | --- |
| element ID | Unique ID of the information element according to section 7.3.2 of the IEEE 802.11™ standard [1]. The actual value has to be assigned by IEEE. |
| length | Length of the information element in bytes according to section 7.3.2 of the IEEE 802.11™ standard [1]. |
| flags | This octet contains flags that control the processing and propagation of the route error message. Figure 10 shows the flags that are defined. |
| destination count | The number of unreachable destinations contained in this message. It is at least 1. |
| unreachable destination MAC address | MAC address of the first unreachable destination node. |
| unreachable destination sequence number | The destination sequence number of the first unreachable destination as given in the route table of the mesh point which initiated the route error message. |
| additional unreachable destinations | Additional pairs of MAC address and destination sequence number of additional unreachable destinations. destination count unreachable destinations have to be specified in the route error message. |

The following Figure 9 lists all the flags that are defined for controlling the processing and propagation of the route error message.

Figure 10: Flags of route error information element

The flags have the following meaning:

| no delete | If set, a node has performed a local repair of a link, and upstream nodes should not delete the route |
| --- | --- |
| reserved | set to 0 and ignored on reception |

### Routing Actions for Sending a Data Frame at the Source Mesh Point

When a mesh point wants to send data frames to some other, possibly several hops away, mesh point, it checks first whether there is an entry in the routing table for this destination mesh point.

If there is an entry in the routing table for the destination mesh point, the lifetime of the route is updated to current time + dot11sactiveroutetimeout and the data frame is sent to the next hop on the path to the destination.

If there is no entry in the routing table, the source mesh point initiates a route discovery. Data frames for the destination in question have to be buffered. The RREQ ID counter and the sequence number of the source mesh point will be incremented before composing the route request message.

The fields of the route request message contain the following values as described in Table 6. See 4.4.1 for a description of the format of a RREQ message.

Table 6: Values of RREQ message

| Field | Value |
| --- | --- |
| Frame Control | Action Management Frame, to/from DS both 1 |
| DA | broadcast address (all one’s) |
| SA | MAC address of source mesh point |
| BSSID | not defined; may use all zero’s or a procedure similar to the one for IBSS in 7.1.3.3.3 of [1] |
| element ID | route request IE ID as assigned by IEEE |
| length | 27+((4 + length of path metric value field[1]) + … + (4 + length of path metric value field[n])) + 4 (n is the number of used routing metrics, it might be 0. In this case, the length is 31.) |
| TTL | dot11smeshdiameter |
| RREQ ID | taken from route request counter, which is incremented right before taking this RREQ ID |
| flags | set as needed |
| hop count | 0 |
| originator MAC address | MAC address of source mesh point |
| originator sequence number | sequence number of the source mesh point, incremented prior to insertion |
| destination MAC address | MAC address of the destination mesh point for which a route is desired |
| destination sequence number | The latest sequence number received in the past by the originator for any route towards the destination. If there is no such destination sequence number available, the unknown destination sequence number flag is set and this field is filled with all zeros. |
| routing metric identifier | routing metric identifier according to the definitions in section 4.2.1. If no routing metric is used, it is 00-0F-AC-00 and no path metric value is following. |
| path metric value | initial value according to specified routing metric |
| … | further considered routing metrics |
| no routing metric identifier | 00-0F-AC-00 |

If the route request is for a path that will be used in both directions, the destination mesh point needs a route to the source mesh point as well. In order to force the creation of such a reverse route it is recommended to

- set the destination only flag, so that the reverse route is going all the way to the destination. Intermediate nodes are not allowed to send route replies. The destination will be aware of the route setup and has a route to the source mesh point.

or

- set the gratuitous route reply flag, so that a route is setup between the destination mesh point and the intermediate mesh point which answers the route request with a route reply. The intermediate mesh point sends a (gratuitous) route reply to the destination mesh node in order to inform the destination about the route setup and to ensure a route between destination and source.

The source mesh point waits dot11spathdiscoverytime for a corresponding route reply message. If the source mesh node receives a route reply message, the path to the destination is set up and the buffered and subsequent packets can be forwarded on this path to the destination (see also section 4.4.7 on processing route replies). If the acknowledgement flag has been set in the route reply message, the source mesh point generates a route reply acknowledgement message and sends it to the transmitter of the route reply message. See sections 4.4.3 and 4.4.8 for further details on route reply acknowledgement messages.

If no route reply message was received within time, the source mesh point may try again to discover a route by broadcasting another route request, up to the maximum of dot11srreqretries times. Each new attempt must increment and update the RREQ ID and the sequence number of the source mesh point. Route discovery retries have to deploy an exponential backoff, that is, the source mesh point waits after the first route request dot11spathdiscoverytime, after the second route request (i.e. the first retry) 2*dot11spathdiscoverytime, and so on. After each retry the waiting time for a route reply message is doubled. The mesh source point should not generate more than dot11srreqratelimit route request messages per second.

If still no route reply has been received, the buffered data frames for the destination are dropped and the higher layers are informed that the destination could not be found.

### Processing of Route Request Messages

When an intermediate mesh point receives a route request message it does the following actions:

- It creates or updates a routing table entry to the sender (=transmitter) of the route request message. The destination and next hop for this entry are taken from the SA field of the action management frame containing the route request information element. The path metric is based on local knowledge.

- It checks whether it already has received this route request message by comparing the originator MAC address and RREQ ID with the corresponding fields of already received route request messages. If so and the considered path metric has not a better value, the route request message is silently discarded and no further action is done.

- The TTL value, the hop count value, and the path metric values in the route request message are updated (TTL := TTL-1; hopcount := hopcount+1; path metric values according to routing metric specification).

- The route to the originator / source mesh point is created or updated (if the sequence number test is okay). Table 7 shows the values or their source of some fields of the routing table entry.

Table 7: Values for routing table entry to originator

| routing table entry | taken from |
| --- | --- |
| destination sequence number | originator sequence number of RREQ |
| next hop | source address(SA/addr2) of action management frame with RREQ (= transmitter address) |
| hop count | hop count of RREQ |
| path metric values | updated path metric fields from route request message |
| lifetime | max(existing lifetime, minimal lifetime) with minimal lifetime := current time + 2*dot11smeshtraversaltime + 2*hopcount*dot11snodetraversaltime) |

- If the mesh point has a valid route to the requested destination and the destination only flag is not set, the mesh point generates a route reply message.

The fields of the route reply message contain the following values as listed in Table 8. See 4.4.2 for a description of the format of a RREP message.

Table 8: Values of RREP message

| Field | Value |
| --- | --- |
| Frame Control | Action Management Frame, to/from DS both 1 |
| DA | next hop towards originator / source, taken from routing table entry or from source address (SA/addr2) of route request message |
| SA | MAC address of mesh point who generates the route request |
| BSSID | not defined; may use all zero’s or a procedure similar to the one for IBSS in 7.1.3.3.3 of [1] |
| element ID | route reply IE ID as assigned by IEEE |
| length | 23+((4 + length of path metric value field[1]) + … + (4 + length of path metric value field[n])) + 4 (n is the number of used routing metrics, it might be 0. In this case, the length is 27.) |
| TTL | 64 |
| flags | set as needed |
| hop count | hop count from routing table entry to destination |
| originator MAC address | originator MAC address from route request message |
| destination MAC address | destination MAC address from route request message |
| destination sequence number | destination sequence number associated to route to the destination in the routing table |
| lifetime | expiration time of route to destination from routing table – current time |
| routing metric identifier | routing metric identifier from route request message |
| path metric value | corresponding cumulative value of the metric taken from routing table entry to destination |
| … | further considered routing metrics from route request message |
| no routing metric identifier | 00-0F-AC-00 |

The next hop towards the originator / source mesh point is added to the precursor list of the route to the destination.

The next hop towards the destination is added to the precursor list of the route to the originator / source mesh point.

- If a route reply message has been generated and the gratuitous route reply flag has been set in the RREQ, the node generates a gratuitous route reply message and sends it to the destination. The gratuitous route reply message has the format of a normal route reply message. It differs from the route reply message of (5.), which is described in Table 8, in the following fields as shown in Table 9.

Table 9: Different values of gratuitous RREP message

| Field | Value |
| --- | --- |
| DA | next hop towards destination, taken from routing table entry |
| hop count | hop count from routing table entry of originator / source or from route request message |
| originator MAC address | destination MAC address from route request message |
| destination MAC address | originator MAC address from route request message |
| destination sequence number | destination sequence number of the originator / source taken from the route request message |
| lifetime | expiration time of route to originator / source from routing table – current time |

N.B.: The gratuitous route reply is a route reply to a fictitious RREQ for a route from the destination to the source.

- If no route reply message has been generated and TTL>0 of the RREQ, the updated RREQ is broadcasted. However, the mesh point waits a uniformly distributed random time between 0 and dot11smaxbroadcastjitter before actually broadcasting the route request message (cf. 7.2).

When a mesh point receives a route request message and it is the destination of the route which is to be discovered, it does the same actions as described above with some minor differences. Actions 1 to 4 are done. If the RREQ passed all tests and has to be processed, the destination mesh point increments its sequence number when the destination sequence number of the RREQ is equal to the sequence number of the destination. Finally, it generates a route reply message. However, some fields have different values compared to the RREP generated by an intermediate mesh point. The differences to the RREP by an intermediate mesh point (cf. Table 8) are given below in Table 10.

Table 10: Different values of RREP message by destination

| Field | Value |
| --- | --- |
| hop count | 0 |
| destination MAC address | MAC address from destination mesh point |
| destination sequence number | sequence number of destination mesh point, might have been incremented prior to generation of route reply message |
| lifetime | dot11smyroutetimeout |
| routing metric identifier | routing metric identifier from route request message |
| path metric value | initial value according to specified routing metric |
| … | further considered routing metrics from route request message |
| no routing metric identifier | 00-0F-AC-00 |

The destination only flag and the gratuitous route reply flag have no meaning at the destination.

### Processing of Route Reply Messages

When an intermediate mesh point receives a route reply message it does the following actions:

- It creates or updates a routing table entry to the sender of the route reply message. The destination and next hop for this entry are taken from the source address field (SA/addr2) of the action management frame containing the route reply information element. This route does not have a valid sequence number. The path metric is based on local knowledge.

- The TTL value, the hop count value, and the path metric values in the route reply message are updated (TTL := TTL-1; hopcount := hopcount+1; path metric values according to routing metric specification).

- It creates or updates the forward route to the destination mesh point. If a route to the destination mesh point already exists, the route reply is only used if it is newer or better information. The latter means, that a route reply with the same destination sequence number but a better path metric value is considered. Table 11 lists the values for the route table entry of the forward route to the destination.

Table 11: Values for routing table entry to destination

| routing table entry | value |
| --- | --- |
| destination MAC address | destination MAC address of route reply message |
| destination sequence number | destination sequence number of RREP message |
| SN valid flag | true / valid |
| active route flag | true / active |
| next hop | source address (SA/addr2) of action management frame (= transmitter address) with RREP IE |
| hop count | updated hop count of RREP |
| path metric values | updated path metric values from route reply message |
| lifetime | current time + lifetime of route reply message |

- If TTL >0 in the updated RREP, the mesh point sends the updated route reply message to the next hop towards the originator of the route discovery.

- It updates the precursor list of the route to the destination with the MAC address of the node to which the updated route reply message has been sent in (4.). The same is done with the precursor list of the route entry to the next hop towards the destination.

- The reverse route, that is the route towards the originator / source mesh point of the RREQ and on which the RREP message is forwarded, has its lifetime updated to the maximum of the existing lifetime and the sum of current time and dot11sactiveroutetimeout. The lifetime of the route entry to the next hop towards the originator is updated the same way.

- If the acknowledgement flag has been set in the received route reply message, a route reply acknowledgement message is sent to the node which sent that RREP (see section 4.4.8)

If the mesh point which received the route reply message is the originator of this route discovery, it has to perform actions 1 through 3 and 7 only. A new route reply message is not generated.

### Generation and Processing of Route Reply Acknowledgement Messages

If a mesh point receives a route reply message with the acknowledgement flag set, it has to generate a route reply acknowledgement message (RREP-ACK) and has to send this message to the sender of the route reply. The fields of the route reply acknowledgement message have the following values as shown in Table 12. See 4.4.3 for a description of the format of a RREP-ACK message.

Table 12: Values of RREQ-ACK message

| Field | Value |
| --- | --- |
| Frame Control | Action Management Frame, to/from DS both 1 |
| DA | source address (SA/addr2) of route reply message to be acknowledged |
| SA | MAC address of mesh point who generates the route reply acknowledgement |
| BSSID | not defined; may use all zero’s or a procedure similar to the one for IBSS in 7.1.3.3.3 of [1] |
| element ID | route reply acknowledgement IE ID as assigned by IEEE |
| length | 1 |
| flags | 00 |

Actions on not receiving a route reply acknowledgement (and also on receiving a RREP-ACK) are outside of this proposal.

### Generation and Processing of Route Error Messages

If an upstream mesh point detects a link break to the next hop of an active route it has to repair the active route. The link break is detected when the short retry counter for the frame to be sent is equal to dot11ShortRetryLimit or when the long retry counter for the frame to be sent is equal to dot11LongRetryLimit (cf. section 9.2.5.3 of [1]).

The mesh point may try to do a local route repair, that is, to discover a backup route to the destination mesh point. If this is not successful or it does not consider a local route repair, it has to notify all mesh points that are affected by this link break. Route error messages are used to achieve this.

The mesh point compiles a list of all unreachable destinations from its routing table. These are the unreachable neighboring mesh point and all destinations that use this unreachable mesh point as next hop. The route error message is generated from this information with the following values as shown in Table 13. See 4.4.4 for a description of the format of a RERR message.

Table 13: Values of RERR message

| Field | Value |
| --- | --- |
| Frame Control | Action Management Frame, to/from DS both 1 |
| DA | broadcast address (all one’s) |
| SA | MAC address of mesh point sending the route error message |
| BSSID | not defined; may use all zero’s or a procedure similar to the one for IBSS in 7.1.3.3.3 of [1] |
| element ID | route error IE ID as assigned by IEEE |
| length | 2 + destination count * 10 |
| flags | set as needed |
| destination count | number of unreachable destinations |
| unreachable destination MAC address[1] | MAC address of the first unreachable destination |
| unreachable destination sequence number[1] | destination sequence number of the first unreachable destination |
| … | further unreachable destinations and the corresponding destination sequence numbers; There have to be destination count pairs of unreachable destination MAC address – unreachable destination sequence number. |

Just before transmitting the route error message, the destination sequence numbers of all unreachable destinations in the routing table are incremented, if they exist and are valid. The routing table entries are set to invalid. The lifetime field is set to current time + dot11sdeleteperiod. Now, the lifetime fields indicate the deletion times of these routing table entries.

When a mesh point receives a route error message, it checks whether it has routes that use the mesh point that sent the RERR message (source address (SA/addr2)) as next hop and those destinations are amongst the unreachable destinations listed in the RERR message. If so, the mesh point compiles a new RERR message containing only these unreachable destinations. Just before transmitting the route error message,

- the destination sequence numbers of all these unreachable destinations in the routing table are set to the corresponding value from the received RERR message, if they exist and are valid.

- the routing table entries are set to invalid. The lifetime field is set to current time + dot11sdeleteperiod.

- the mesh point waits a uniformly distributed random time between 0 and dot11smaxbroadcastjitter (cf. 7.2).

If there are no destinations of the mesh point amongst the unreachable destinations from the received RERR message, nothing is done and no RERR message is sent.

If the upstream route should not be deleted since the node which detected the link break is performing a local route repair and the RERR message is only for informing the source mesh points about the occurred link break and subsequent suboptimal local route repair, the no delete flag has to be set. Mesh points that receive a RERR message with the no delete flag set, do all the steps as described above except the deletion of the route.

### General Purpose Forwarding Table

Each mesh point has to contain a forwarding table. The actual structure of the forwarding table is rather implementation specific. Nevertheless, it should be general purpose so that other routing protocols than the basic IEEE 802.11s one can use the forwarding table flexibly.

The general purpose forwarding table should include the following fields:

- destination MAC address

- destination sequence number

- valid destination sequence number flag

- other state and routing flags (e.g., valid/active route, invalid/inactive route, repairable, being repaired)

- next hop

- network interface (network interface to be used when forwarding to the next hop)

- hop count (number of hops needed to reach destination)

- list of path metric(s) (accumulated values of the routing metric(s) of the complete path to the destination)

- list of precursors

- lifetime (expiration or deletion time of the route)

- additional general format columns for extensibility

Any frame, no matter if data frame or routing control message, that can be related to a route entry, resets the lifetime of the corresponding route entry to current time + dot11sactiveroutetimeout.

## Proactive Routing Towards Mesh Portals

A large proportion of the traffic in a WLAN mesh network is probably destined for nodes outside. This traffic has to transit a mesh portal. Therefore, it is advantageous to have proactive routes towards mesh portals.

The proactive routing towards mesh portals comes in two flavors – mesh portal initiated and mesh point initiated.

The proactive routing towards mesh portals is a mechanism of which use is optional. Its implementation, however, is mandatory so that all mesh points can process the messages.

### Mesh Portal Initiated Routing Towards Mesh Portals

A mesh portal periodically broadcasts its existence. Mesh points that receive such a mesh portal announcement, rebroadcast it and include their distance/path metric to the announced mesh portal. Eventually, all nodes know a path to the mesh portal with shortest distance/best path metric. The next hop towards the mesh portal is the mesh point from which the best mesh portal announcement has been received.

The mesh portal includes a route reply information element (cf. 4.4.2) into its periodic beacon. It increments its destination sequence number before compiling the route reply information element. The route reply information element contains the following values as listed in Table 14. See 4.4.2 for a description of the format of a RREP message.

Table 14: Values of RREP message by mesh portal

| Field | Value |
| --- | --- |
| element ID | route reply IE ID as assigned by IEEE |
| length | 23+((4 + length of path metric value field[1]) + … + (4 + length of path metric value field[n])) + 4 (n is the number of used routing metrics, it might be 0. In this case, the length is 27.) |
| TTL | 64 |
| flags | 0 (The acknowledgement flag must not be set.) |
| hop count | 0 |
| originator MAC address | broadcast address (all one’s) |
| destination MAC address | MAC address of mesh portal |
| destination sequence number | destination sequence number of mesh portal |
| lifetime | dot11smymeshportalroutetimeout |
| routing metric identifier | first routing metric identifier |
| path metric value | corresponding “zero” value of the metric |
| … | further considered routing metrics |
| no routing metric identifier | 00-0F-AC-00 |

When a mesh point receives a beacon with such a mesh portal announcement information element, it will process it with the following steps:

- The mesh point creates or updates a routing table entry to the sender of the beacon. The destination and next hop for this entry are taken from the source address field (SA/addr2) of the beacon management frame containing the route reply information element. This route does not have a valid sequence number. The path metric is based on local knowledge.

- The TTL value, the hop count value, and the path metric values in the route request message are updated (TTL := TTL-1; hopcount := hopcount+1; path metric values according to routing metric specification).

- The mesh point creates or updates the forward route to the mesh portal. If a route to the mesh portal already exists, the route reply information element is only used if it contains newer or better information. The latter means, that a route reply information element with the same destination sequence number but a better path metric value is considered. The values of the fields for the route table entry to the mesh portal are given in Table 15.

Table 15: Values for routing table entry to mesh portal

| routing table entry | value |
| --- | --- |
| destination MAC address | MAC address of mesh portal which can be found in destination MAC address of RREP IE in beacon |
| destination sequence number | destination sequence number of mesh portal which can be found in destination sequence number of RREP IE in beacon |
| SN valid flag | true / valid |
| active route flag | true / active |
| next hop | source address (SA/addr2) of beacon management frame (= transmitter address) with RREP IE |
| hop count | updated hop count of RREP IE |
| path metric values | updated path metric values from route reply information element |
| lifetime | current time + lifetime of route reply information element |

The route may be marked as path to a mesh portal in the routing table.

- If TTL >0 in the updated RREP IE of the beacon, the mesh point includes the updated route reply information element into its own beacon management frame.

### Mesh Point Initiated Routing Towards Mesh Portals

Mesh Points ask actively for paths to the mesh portals. They send a route request message to which only mesh portals will answer with a route reply message. Since the number of mesh portals and their MAC addresses are not known, the destination MAC address of the route reply message is the broadcast address. Furthermore, it is recommended to set the destination only flag. The mesh point is responsible for maintaining the paths to the mesh portals.

The route request message for the mesh point initiated routing towards mesh portals is the same as described in Table 6 of section 4.4.5 except for the values shown below in Table 16.

Table 16: Different values for RREQ message for mesh portal

| Field | Value |
| --- | --- |
| flags | destination flag set (recommended), mesh portal flag set |
| destination MAC address | broadcast address (all one’s) |
| destination sequence number | 00 00 00 00 |

The mesh portals answer with a route reply message, composed in the same way as described in 4.4.6. Moreover, the mesh portal flag is set.

## Multiple Interfaces per Mesh Node

The different MAC addresses of the multiple interfaces of a mesh point are treated as synonymous MAC addresses of the mesh point in the sense of being an identifier of the mesh point. A mesh point with n (n>1) interfaces appears to other nodes like n mesh points with a common networking intelligence that connects these n (logical) mesh points.

If a mesh point has to send a broadcast it broadcasts the frame on all n network interfaces.

A unicast route uses exactly one of the n interfaces for transmitting the frame to the next hop on the path. Therefore, the corresponding network interface is part of the entries in the forwarding table.

A separate interface at a mesh access point that is exclusively used for communication with associated legacy IEEE 802.11 stations cannot be used as a MAC address in the WLAN mesh network.

## Integration of Legacy IEEE 802.11 Stations (STA)

Legacy IEEE 802.11 stations may have an association with a mesh access point (mesh AP). There is a single wireless hop between the legacy stations and the mesh AP. The mesh AP knows about all legacy stations associated with it, but the stations are not aware of the mesh.

In order to enable legacy stations to communicate with other devices via the WLAN mesh network, the associated mesh AP performs the mesh services instead of the stations. Packets to an associated legacy station are received by the mesh AP and forwarded to the station via the BSS. The translation between the different data frame formats is described in section 6.

## Integration of Vendor Specific WLAN Mesh Routing Protocols

Vendor specific WLAN mesh routing protocols can be integrated by defining new routing metrics and/or new routing management messages.

The general format of routing metrics contains the 3 byte long organizational unique identifier (OUI) and has room for 255 vendor specific routing metrics.

New vendor specific routing management messages can be defined within the vendor specific management frame. A mesh point with a vendor specific mesh routing protocol may ignore the routing management messages defined within 802.11s. In order to do so, it must announce that it is not using the basic 802.11s mesh routing protocol by saying which other mesh routing protocol it uses. This is done with the active mesh routing protocol information element in beacons (cf. 5.3.2).

The general purpose forwarding table contains some additional general format columns for extensibility. Moreover, the forwarding table is implementation specific, so it should be easy to adapt the forwarding table if necessary.

# Frame Formats

## Mesh Data Frame Format

The mesh data frame is based on the data frame format as defined by the IEEE 802.11e amendment [3]. The IEEE 802.11e data frame is described in section 7.1.2 of the amendment [3].

Figure 11: Mesh data frame format

The mesh data frame has three additional fields: a time-to-live field and a flag field, which are fixed, and option field(s), that are variable.

### Frame Control Field

The frame control field follows the traditional IEEE 802.11 rules. They are described in section 7.1.3.1 of [1]. The following subsections explain some values that are important for the mesh data frame.

#### Type and Subtype Frame Control Subfields

A new data frame subtype is defined for mesh data frames. The following IEEE 802.11 frame subtype has to be added to table 1 in section 7.1.3.1.2 of [1] as shown in Table 17.

Table 17: Valid type and subtype combinations (IEEE 802.11s extension)

| Type value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 10 | Data | 1101 | Mesh Data |

#### To DS and From DS Subfields

Both the To DS subfield and the From DS subfield are set to 1.

#### More Fragments Subfield

The More Fragments subfield is set in a similar way as in IEEE 802.11[1]. See section 8 for details on fragmentation and defragmentation.

### Sequence Control Field

The sequence control field of the MAC header is used in a similar way as in IEEE 802.11[1]. See section 8 for details on fragmentation and defragmentation.

### Address Fields

The WLAN mesh network uses the 4 address data frame format. The four addresses have the same meaning as in table 4 of section 7.2.2 of the IEEE 802.11 standard [1]. The meaning is as follows:

| address 1 | Receiver Address (RA) |
| --- | --- |
| address 2 | Transmission Address (TA) |
| address 3 | Destination Address (DA) |
| address 4 | Source Address (SA) |

The receiver address is the MAC address of the mesh point which received this data frame from the mesh point with the transmission address. There is only a single wireless hop between transmitter and receiver.

The transmission address is the MAC address of the mesh point which transmitted this actual wireless data frame. There is only a single wireless hop between transmitter and receiver.

The destination address is the MAC address of the destination of the multi-hop transmission. The destination is the final recipient of the mesh data frame. The destination address is often not the MAC address to which the data frame is sent directly over the air. The destination hands the data frame up to a processing unit which is on a “higher layer” than the data frame forwarding.

The source address is the MAC address of the source of the multi-hop transmission. The source is the originator. The source address is often not the MAC address from which the data frame has been received directly over the air. The source received the data from higher layers or a processing unit which is on a “higher layer” than the data frame forwarding.

### Time-To-Live Field (TTL)

The meaning of this field is the same as of the time-to-live field in the Internet protocol. The TTL field is used to discard data frames that traveled too long in the network. This might happen because of temporary routing loops. The TTL field is decremented after each transmission on the multi-hop path. The source mesh point, which generates mesh data frames, sets the TTL field to its initial value (usually 255). Intermediate mesh points decrement the TTL value by 1 before forwarding the data frame. If the TTL value reaches zero (0), the data frame is discarded.

### Mesh Data Frame Flags Field

The mesh data frame flags field is one octet long. It contains flags that control the processing of the mesh data frame. The following lists all the flags that are defined.

Figure 12: Flags for mesh data frame flags field

#### Mesh Data Frame Options Flag

If this flag is set, the mesh data frame header contains one or more mesh data frame options. They are defined in 5.1.6.

#### Reserved

These bits are reserved for future definition of flags. They are set to 0 and ignored on reception.

### Mesh Data Frame Option Field(s)

This field allows a flexible extension of the MAC header of the mesh data frame. A mesh data frame option identifier (4 octets) defines the included option. It consists of an organizational unique identifier and a mesh data frame option subidentifier. The length field contains the length of the option information in octets. See Figure 13 for the structure of mesh data frame options.

Mesh data frame options can only be included if the mesh data frame options flag is set in the mesh data frame flags field (cf. 5.1.5). When the mesh data frame options flag is set, the mesh data frame options follow directly the mesh data frame flags field. The non option indicates the end of the sequence of options.

A mesh data frame option has the following structure as shown in Figure 13.

Figure 13: Mesh data frame option field format

The following options are defined as listed in Table 18 below.

Table 18: Mesh data frame options

| OUI | Mesh Data Frame Option Sub Identifier | Meaning |
| --- | --- | --- |
| 00-0F-AC | 0 | no mesh data frame option is given (non option) |
| 00-0F-AC | 1 | broadcast identifier |
| 00-0F-AC | 2-255 | reserved |
| vendor OUI | 0-255 | vendor specific mesh data frame options, it is recommended not to use 0 for a mesh data frame option |

#### Non Option

This mesh data frame option has no information. It consists solely of the mesh data frame option identifier and the length field with a value of 0. It indicates that there is no (further) mesh data frame option present. The non option is the last option and indicates that the frame body starts.

#### Broadcast Identifier Option

This mesh data frame option is used in mesh data frames that belong to a mesh-wide broadcast. See section 6 for a description how this broadcast identifier is used.

The option information has the following format as shown in Figure 14.

Figure 14: Broadcast identifier option format

#### Vendor Specific Option

The vendor specific option can contain any option information defined by the vendor with the given OUI.

## Control Frame Format and Management Frame Format

The format of the management frames is not extended. All control and management frames of the WLAN mesh services are defined as information elements and appended to the corresponding control or management frames or as information elements in the action management frame.

It is absolutely necessary that management frames which contain routing control messages are protected. It should not be possible for unauthorized nodes to interpret these management frames nor to inject management frames with routing control messages into the WLAN mesh network.

## Beacon

Every mesh point periodically broadcasts beacon management frames. The beacon is broadcast every dot11sbeaconinterval plus a random value for jittering the beacons. The random value is uniformly distributed in the interval [max(-0.2*dot11sbeaconinterval , 2000 TU), min(+0.2*dot11sbeaconinterval, 2000 TU)]. The beacon interval field contains dot11sbeaconinterval measured in time units.

The beacon contains additional information elements in a WLAN mesh network. These are mesh identity, active mesh routing protocol, active mesh routing metrics, and mesh security. They are defined in the following subsections.

### Mesh Identity Information Element

The mesh identifier information element is similarly defined as the SSID information element. It can contain up to 32 bytes in its information field. The format of the mesh identity information element is shown in Figure 15.

Figure 15: Mesh identity information element format

### Active Mesh Routing Protocol Information Element

The active mesh routing protocol information element announces the currently active routing protocol of the WLAN mesh network. There can be only exactly one active routing protocol. The format of the active mesh routing protocol information element is shown in Figure 16.

Figure 16: Active mesh routing protocol information element format

The routing protocol identifier has the following structure as shown in Figure 17.

Figure 17: Routing protocol identifier format

The following routing protocol identifiers are defined as shown in the following Table 19.

Table 19: Routing protocol identifiers

| OUI | Routing protocol Sub Identifier | Meaning |
| --- | --- | --- |
| 00-0F-AC | 0 | IEEE 802.11s layer 2 AODV-based routing protocol |
| 00-0F-AC | 1-255 | reserved |
| vendor OUI | 0-255 | vendor specific routing protocols |

### Active Mesh Routing Metrics Information Element

The active mesh routing metrics information element announces the currently active routing metrics of the WLAN mesh network. More than one routing metric can be active. Moreover, no routing metric might be given in the information element. In this case, the routing metric is hop count, which is incorporated into the IEEE 802.11s routing control messages. The format of the active mesh routing metrics information element is shown in Figure 18.

Figure 18: Active mesh routing metrics information element format

The number of active routing metrics n can be zero, one, or greater than one. The routing metric identifiers are defined in section 4.2.1. The non routing metric identifier indicates the end of the sequence of routing metric identifiers. The length of the information field is (n+1)*4 bytes.

### Mesh Security Information Element

This information element specifies whether a mesh point has already been authenticated and how. The format of the mesh security information element is shown in Figure 19.

Figure 19: Mesh security information element format

The following Figure 20 lists all the flags that are defined for information on mesh point authentication.

Figure 20: Flags of mesh security information element

The flags have the following meaning:

| AAA authenticated | If set, the mesh node has been authenticated with a AAA server and provides connectivity to this AAA server for its neighbors. See section 11.1.1. |
| --- | --- |
| Distributed authentication | If set, distributed authentication is used. The neighboring nodes have to perform the IBSS 4-way handshake to participate in the mesh network. See section 11.1.2. |
| reserved | set to 0 and ignored on reception |

# Data Frame Forwarding

An intermediate mesh point which receives a mesh data frame which has to be forwarded to another mesh point from some other mesh point does the following:

- decrement TTL field by 1.

- check if mesh data frame for itself, if yes hand it up

- if TTL = 0 discard mesh data frame

- update lifetimes to current time + dot11sactiveroutetimeout of routes towards

- transmitter of this mesh data frame

- destination of this mesh data frame

- next hop on path of this mesh data frame

- set addresses for next wireless transmission in the WLAN mesh network

- address1 (RA) := next hop MAC address from route to DA

- address2 (TA) := MAC address of this mesh point

- address3 (DA) : no change

- address4 (SA) : no change

- transmit mesh data frame to next hop (RA) on path to destination (DA)

A mesh access point which receives a mesh data frame which has to be forwarded to an associated legacy IEEE 802.11 station does the following:

- decrement TTL field by 1.

- check if mesh data frame for itself, if yes hand it up

- if TTL = 0 discard mesh data frame

- update lifetimes to current time + dot11sactiveroutetimeout of routes towards

- transmitter of this mesh data frame

- create new data frame for BSS

- set addresses for next wireless transmission in BSS of mesh AP

- address1 (RA) := destination MAC address of address3 (DA) of mesh data frame

- address2 (TA/BSSID) := MAC address of this mesh access point

- address3 (SA) := source MAC address from address4 of mesh data frame

- transmit data frame to associated legacy IEEE 802.11 station (DA).

A mesh access point which receives a data frame from an associated legacy IEEE 802.11 station to be sent into the WLAN mesh network does the following:

- check if data frame for itself, if yes hand it up

- create new mesh data frame for WLAN mesh network

- set TTL to 254

- update lifetimes to current time + dot11sactiveroutetimeout of routes towards

- destination of this mesh data frame

- next hop on path of this mesh data frame

- set addresses for next wireless transmission in the WLAN mesh network

- address1 (RA) := next hop MAC address from route to DA

- address2 (TA) := MAC address of this mesh point

- address3 (DA) := destination MAC address from address3 (DA) of data frame

- address4 (SA) := transmitter/source MAC address from address2 (TA/SA) of data frame

- transmit mesh data frame to next hop on path to destination (DA)

# Broadcast Data Service

Higher network layers and 802.3 LANs consider a WLAN mesh network as a single broadcast domain. Due to the wireless multi-hop communication within the WLAN mesh network, the broadcast data service has to be extended in order to take this into account.

There are two different kinds of broadcasts in a WLAN mesh network:

- link local broadcast

- mesh wide broadcast

## Link-local broadcast

The link local broadcast is the broadcast as known from the IEEE 802.11 base standard [1]. A frame which is broadcast is transmitted via the wireless interface and all nodes that can receive the frame process it.

Some mesh routing messages are sent with link-local broadcast, because they are management frames and cannot be sent with a mesh-wide broadcast.

If mesh data frames have to be sent with link-local broadcast, the TTL is set to one (1) or/and a broadcast identifier is not given as a mesh data frame option.

## Mesh-wide Broadcast

A mesh-wide broadcast (sometimes also called network-wide broadcast) has to be received by all mesh points belonging to the WLAN mesh, even if they are not in radio range of the originator of the broadcast. The broadcast frame is flooded in the WLAN mesh network in order to achieve this. Recipients of the broadcast rebroadcast the frame on all their interfaces.

The originator of the mesh-wide broadcast does the following actions:

- It sets the TTL field in the mesh data frame to the number of hops the broadcast should travel. The default value is 255 which comprises the complete network. A significantly smaller TTL value can be used to control and to limit the dissemination of the broadcast data frame. A TTL=1 means that the broadcast is actually link-local.

- It increments its broadcast message counter. The new value of the broadcast message counter is the broadcast identifier, which is put into the broadcast identifier option (cf. 5.1.6.2).

- The broadcast identifier option is appended to the MAC header of the broadcast mesh data frame.

- The address fields in the MAC header contain the following MAC addresses:

address1: RA := broadcast address (all one’s)

address2: TA := MAC address of originator of mesh-wide broadcast (this mesh point)

address3: DA := broadcast address (all one’s)

address4: SA := MAC address of originator of mesh-wide broadcast (this mesh point)

- It sends the broadcast data frame.

- It puts the tuple {SA = MAC address of originator of mesh-wide broadcast/this mesh point; broadcast identifier; dot11smeshbroadcastlisttimeout} into its list of received broadcasts.

Mesh points which receive this broadcast data frame recognize the mesh-wide broadcast on the broadcast address in the address3 (DA) field of the MAC header and on the existing broadcast identifier mesh data frame option. The receiver of a mesh-wide broadcast data frame does the following actions:

- It compares the source address (address4/SA) and the broadcast identifier with the entries in the list of received broadcasts. If it already received the broadcast data frame previously, the current, duplicate broadcast data frame is silently discarded and the timeout of the entry in the list of received broadcasts reset.

- If the comparison in (1.) has been negative, this is a new broadcast data frame. The content of the broadcast data packet is given to the higher layers.

- The TTL is decremented by 1. If TTL=0, the broadcast data frame is not rebroadcast.

- The address2/TA is updated with the MAC address of this node.

- The mesh point waits a uniformly distributed random time between 0 and dot11smaxbroadcastjitter. All neighbors of the last transmitter of the broadcast data frame might have to rebroadcast the frame. They will interfere with each other when they do this at (roughly) the same time. The jittering of the rebroadcasts distributes them more sequentially.

- It (re)broadcasts the updated broadcast data frame.

- It puts the tuple {SA = MAC address of originator of mesh-wide broadcast; broadcast identifier; dot11smeshbroadcastlisttimeout} into its list of received broadcasts.

A mesh-wide broadcast following the rules as described above is only possible with mesh data frames. Some routing control messages rely on a mesh-wide broadcast as well, for instance, route request messages. They are, however, sent as management frames and cannot use this mesh-wide broadcast mechanism. Instead, the rebroadcasting is initiated in the processing of the routing control message (see 4.4.6 for RREQ). Nevertheless, they have to do the jittering before rebroadcasting.

# Fragmentation and Defragmentation of Frames

In principle, the fragmentation and defragmentation of frames follows the same rules as described in 9.1.4, 9.4, and 9.5 of the IEEE 802.11 WLAN standard [1]. However, there are some important differences due to the multi-hop transmission in a WLAN mesh network:

- Only the source mesh point sets the sequence control field in the MAC header. Intermediate mesh points do not change the values in the sequence control field set by the source. The destination mesh node will find in the sequence control field the sequence number and the fragment number as assigned by the source mesh point.

- The fragments are not sent as a fragment burst but rather as independent frames. Intermediate nodes do not defragment the MSDU.

The sequence control field as described in section 7.1.3.4 of [1] and the more fragment bit of the frame control field (section 7.1.3.1.5 of [1]) are used for controlling the defragmentation.

# Mesh Portal Functionality

The mesh portal works as L2 bridge between the 802.11s MAC (incl. L2 routing) and the 802 MAC. The solution should conform to 802.1D on the 802 MAC side.

The portal has to transport packets for the following constellations:

- from mesh point to mesh point in the same mesh

- from mesh point to address outside the mesh

- from address outside to mesh point inside the mesh

Depending on the source and destination of the packets the portal changes the packet format accordingly. In the case of management packets (e.g. L2 mesh route request) the portal may take an action.

The solution should be capable of connecting several mesh networks.

## Portal Behavior

### Address Learning

The portal learns the addresses and the ports of nodes when receiving/forwarding packets. The portal discerns by this means between addresses that are in the mesh, outside the mesh and optionally are reached by crossing the mesh.

### Data Frame Forwarding from Mesh Point to Mesh Portal and Vice Versa

Data frames destined for a destination outside the mesh are transferred from and to a mesh portal via tunneling by means of encapsulation, cf. also section 11.2. The node at the end of the tunnel decapsulates the data frame and forwards it to the bridging component or higher protocol layers, respectively.

The IEEE 802.11s routing protocol provides information on the locality of nodes, i.e. the reachability of destinations inside or outside the mesh. If a destination is located outside the mesh, the mesh portals and the routing protocol should provide information on the mesh portal through which the destination is reachable.

### Unicast: From Mesh Point to Destination outside the Mesh

For known destinations the packet is sent according to 9.1.2.

If the destination is not in the forwarding table, a route request is issued. If a mesh portal already knows the destination, it issues a route reply indicating that the destination is reachable by the mesh portal.

When the route request times out, the mesh point sends the first data frame by means of encapsulation to all potential mesh portals. The mesh portals relay the data frame on all segments. When a reply from the destination is received, the mesh portal learns the address, the forwarding table is updated by a route reply and the packets are sent through the mesh portal.

### Unicast: From Node outside the Mesh to Destination inside the Mesh

If the destination is not known in the forwarding table of the mesh portal, the mesh portal issues a route request. The remaining procedure is identical to the description in 9.1.3.

### Broadcast Transmission

The mesh portal translates between the broadcast mechanism in the mesh and outside the mesh. It might be necessary to tunnel the non-mesh broadcast data frame within the WLAN mesh network.

# Quality of Service

Traffic prioritization is performed according to 802.11e, i.e. the parameters AIFS, CWmin, and CWmax are set for each access category.

# Security

IEEE 802.11i [11] describes robust mechanisms for WLAN security that have undergone extensive reviews. It is out of question that the reuse of these well understood mechanisms is desirable for IEEE 802.11s security. However, certain adoptions have to be made as IEEE 802.11i has been designed without multi-hop communication and mesh networks in mind.

The scope of this proposal is the protection of nodes within the administrative domain of the mesh network from outside nodes. Nodes within the same administrative domain are considered to be trusted, i.e. it is assumed that they behave cooperatively regarding signaling messages within the mesh. For this purpose, pairwise keys and group keys have to be established. The former are used for unicast communication of mesh points with its direct neighbors while the latter are used for broadcast communication.

The key parts of this proposal are the following:

- Authentication of mesh nodes joining a mesh

  - Central model using AAA (e.g. Radius RFC 2865, 2866) and IEEE 802.1x

  - Distributed model

- Efficient operation of IEEE 802.11i in a multi-hop mesh environment

## Basic Authentication of Mesh Nodes

### Central Model

In the central model, the availability of an AAA server is assumed. This server can either be within the mesh network or be reachable via LAN. The server authenticates each mesh point via 802.1x/EAP. To achieve this, each mesh point (all participants except stations (STA)) has to realize both roles: supplicant and authenticator.

If a mesh point is connected to the AAA server (this is the case for direct, single-hop neighbors initially) it can be authenticated directly and receives a PMK (pairwise master key) as well as a GMK (group master key). As the main purpose of this proposal is the protection of the WLAN mesh against outside nodes, the same PMK and GMK can be used for all mesh points. Different session keys (PTKs, Pairwise Transient Keys) that are used for the actual encryption of unicast communication will then be deduced from the PMK using standard 802.11i protocol mechanisms. The same is done with GTKs (Groupwise Transient Keys).

If a mesh point is already authenticated, it includes this information into its beacon packets (see section 5.3.4). Any new mesh point without direct connection to the AAA server (but in range of a mesh point that has already been authenticated) can associate to its neighboring, authenticated mesh point and use IEEE 802.1x / EAP to authenticate with the AAA server.

### Distributed Model

If no AAA server is available, the mesh points have to be provided with a pre-shared key (PSK). This key can be used as PMK and then to deduce PTKs for secure unicast communication using standard IEEE 802.11i protocol mechanisms. To achieve this, each mesh point has to support both roles, supplicant and authenticator. The PTKs can then be deduced from the PMK using the IEEE 802.11i IBSS mechanisms (4-way handshake). Also GTKs can be deduced using a GMK that has to be provided initially.

## Efficient IEEE 802.11i in a Multi-hop Mesh Environment

End-to-end security is out of scope of this proposal. It can be realized using well established protocols like e.g. IPSec. However, from a performance perspective, it makes sense to introduce part of this functionality as a possible replacement of the strictly link-based mechanisms that have been described above.

If link-based encryption is used, each message has to be completely decrypted and encrypted by every intermediate mesh node that forwards the traffic on its way from source mesh point to destination mesh point. This is especially critical for delay sensitive traffic.

Figure 21: Usage of same PMK between different mesh points over multiple hops

To avoid this unnecessary overhead (that does not increase security in any way), we propose to establish a PMK for data transmissions between any two source and destination nodes in the mesh network. Figure 21 illustrates this idea: “Header” describes the MAC Header and “Data” the frame body of a normal data frame.

For inbound and outbound traffic via the mesh portal, the mesh portal is the endpoint of the secure tunnel. This means, the mesh portal acts as source node for inbound traffic and as destination node for outbound traffic regarding key establishment.

### Central Model

If the central model with its AAA server is used, it is possible to have different PMKs for every source-destination relation within the mesh. Thus, the traffic is protected from potential eavesdroppers within the mesh.

Signaling traffic that is sent via broadcast packets can be encrypted as described in sections 11.1.1 and 11.1.2 using the linkwise GTKs. However, data traffic can be encrypted using the PTK derived from the PMK that have been established between source and destination node. To achieve this, parts of the MAC header, that are not directly relevant for radio transmission, have to be encrypted with the linkwise PTK so the intermediate nodes can make its forwarding decisions. However, the much bigger data part of the MAC frames is encrypted within the source node with the PTK established between source and destination node. Thus, no security-related action (encryption, decryption, integrity check) is necessary in the intermediate nodes. The data part of the packets is decrypted within the destination node.

### Distributed Model

While there is no protection against internal eavesdroppers in this model, the performance benefits when using a source-destination PMK for data traffic are the same as described in the central model. The PTKs between source and destination node can be deduced via multi-hop as described in section 11.1.2 using a multi-hop encapsulated IBSS 4-way handshake.

Further operation is equal to 11.2.1: link-specific PTKs are used to protect mesh header information while the data part of the packets is protected using the PTK that has been established between source and destination node. Again, no security-related action is necessary in the intermediate nodes.

# Extensibility

This proposal for the IEEE 802.11s amendment provides flexibility for adaptation of the WLAN mesh network to different usage scenarios and extensibility for incorporation of specialized or vendor-specific mechanisms in the WLAN mesh network. Especially, it is possible to use different routing metrics and to extend the IEEE 802.11s routing protocol or to integrate a different mesh routing protocol. The following gives an overview of the means that are provided to achieve the extensibility.

Routing metrics: The concept of routing metric identifiers as described in section 4.2.1 allows to define several different, vendor-specific routing metrics for usage with the IEEE 802.11s mesh routing protocol or with a vendor-specific mesh routing protocol. It is also possible to vary the number of considered routing metrics, since the sequence of routing metric identifiers is terminated with a special routing metric identifier (no routing metric).

Routing Protocols: The definition of new routing control messages is necessary for integrating other routing protocols. This can be done by defining new routing control messages within the vendor-specific information element that will be included in the action management frame.

General purpose columns in forwarding table: The forwarding table as defined in section 4.4.10 already accommodates for several possible general routing mechanisms. Additional columns in a general format are defined for increased extensibility. However, the forwarding table is implementation specific, so there is already a large flexibility available.

Routing identifiers in beacon management frames: The periodically sent beacons of mesh points include information elements that identify the currently used routing protocol and routing metrics (see section 5.3). New mesh points which want to associate with this WLAN mesh network can check whether they have the required capabilities based on this information.

Options field in mesh data frame: The flexible definition of the options field in the mesh data frame allows an easy extension of the mesh data frame for vendor-specific routing protocols if necessary (cf. section 5.1).

# Attribute Definitions

The following Table 20 lists all the attributes that are used in this proposal for IEEE 802.11s. Table 20 contains also some attributes from IEEE 802.11 [1] that are referenced. They are not changed.

Table 20: IEEE 802.11s attribute description

| Attribute Name | Default Value | Description |
| --- | --- | --- |
| IEEE 802.11s Attributes |  |  |
| dot11spathdiscoverytime | > 2 * dot11smeshtraversaltime | anticipated maximum time for a route discovery |
| dot11srreqretries | 2 | number of retries for route discovery for the same destination after the first route discovery timed out |
| dot11srreqratelimit | 10 | maximum number of route request messages a mesh point can send per second |
| dot11smeshdiameter | 32 | maximum number of hops within mesh network between any source mesh point and destination mesh point |
| dot11snodetraversaltime | 40 ms | maximum time for traversing a node (including wireless transmission) |
| dot11smeshtraversaltime | >(2 * (dot11snodetraversaltime + dot11smaxbroadcastjitter) * dot11smeshdiameter) | anticipated maximum time for traversing the WLAN mesh network on any path |
| dot11sactiveroutetimeout | 10,000 ms | lifetime of a route in the routing table |
| dot11sdeleteperiod | 10,000 ms | time before an invalidated route will be deleted from routing table |
| dot11smyroutetimeout | dot11spathdiscoverytime | lifetime set on paths during route discovery |
| dot11smymeshportalroutetimeout | dot11spathdiscoverytime | time out for a route to a mesh portal during discovery of routes to mesh portal |
| dot11sbeaconinterval | 3000 ms | interval for periodic beaconing; the actual interval is jittered (5.3) |
| dot11smaxbroadcastjitter | 200 ms | upper limit for the uniformly distributed random value a mesh point has to wait before actually rebroadcasting a mesh wide broadcast packet (7.2); should be at least 100 ms. |
| dot11smeshbroadcastlisttimeout | > 2 * dot11smeshtraversaltime | time out for an entry in the list of received broadcasts; has to be large enough so that there is no frame of this mesh wide broadcast in the mesh anymore |
| IEEE 802.11 Attributes [1] |  |  |
| dot11ShortRetryLimit | 7 | number of times a single frame (dot11RTSThreshold) may be retransmitted |
| dot11LongRetryLimit | 4 | number of times a single frame (>dot11RTSThreshold) may be retransmitted |
| dot11RTSThreshold | 3000 | distinguishes between short frames () and long frames (>); RTS/CTS is only done for long frames |

# References

[1]	IEEE Wireless LAN Edition – A compilation based on IEEE Std 802.11™-1999 (R2003) and its amendments; IEEE Standards Information Network, IEEE Press

[2]	Charles Perkins, Elizabeth Belding-Royer, Samir Das: Ad hoc On-Demand Distance Vector (AODV) Routing; IETF RFC 3561 (Experimental), July 2003

[3]	IEEE P802.11e/D13.0: Amendment : Medium Access Control (MAC) Quality of Service Enhancements; January 2005

[4]	11-04/54r2 “PAR for IEEE 802.11 ESS Mesh”

[5]	11-04/56r1 “Five Criteria for IEEE 802.11 ESS Mesh”

[6]	11-04/1430r12 „Call for Proposals for IEEE 802.11s“

[7]	11-04/1174r13 „Functional Requirements and Scope for IEEE 802.11s“

[8]	11-04/1175r10 „Comparison Categories and Informative Checklists for IEEE 802.11s“

[9]	11-04/662r16 „Usage Models for IEEE 802.11s“

[10]	11-04/1477r4 „Terms and Definitions for 802.11s“

[11]	IEEE Std 802.11i™-2004: 802.11i™ - Amendment 6: Medium Access Control (MAC) Security Enhancements