IEEE P802.11
Wireless LANs

| [A MAC Partial Proposal for IEEE 802.11s] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-07-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Rui Zhao | ComNets, RWTH Aachen University | M 225, Kopernikusstr. 16, 52074 Aachen, Germany | +49 241 80 27925 | rui@comnets.rwth-aachen.de |
| Bernhard Walke | ComNets, RWTH Aachen University | W 225, Kopernikusstr. 16, 52074 Aachen, Germany | +49 241 88 90320 | walke@comnets.rwth-aachen.de |
| Michael Einhaus | ComNets, RWTH Aachen University | M 324, Kopernikusstr. 16, 52074 Aachen, Germany | +49 241 80 23923 | ein@comnets.rwth-aachen.de |

Abstract

This document presents a MAC amendment for IEEE 802.11s by taking advantage of the multi-radio platform. A Wireless Distribution System (WDS) can be created like following: Mesh APs form a multi-hop network in a fully distributed manner on one frequency channel, while an 802.11 legacy or 802.11e station is associated with a nearby mesh AP of a Basic service Set (BSS) network and communicates with the mesh AP on another frequency channel. All features proposed here are fully compliant with the 802.11s PAR.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Additional Supporting Material

This section contains requirements for additional documentation that must be submitted with a proposal.  This is a template that must be filled in and included with a proposal submission.

| Number | Name | Definition | Coverage (Yes/No) |  | Notes | References |
| --- | --- | --- | --- | --- | --- | --- |
| AD1 | Reference submissions | A list of IEEE 802 submissions related to the proposal, both documents and presentations. |  | Yes |  | 11-05-0595-01-000s-mac-partial-proposal-Mesh-DCF.doc 11-05-0594-01-000s-mac-partial-proposal-Mesh-DCF.ppt |
| AD2 | Simulation and/or experimental methodology | Any proposal submission that includes simulation results must include a description of the simulation methodology used for mesh simulations. The simulation methodology documentation should provide enough information to, in principle, reproduce the simulation (e.g., including node positions, traffic and propagation model (including PHY assumptions), packet sizes, etc.). |  | Yes |  | 11-05-0594-01-000s-mac-partial-proposal-Mesh-DCF.ppt |

Coverage of Minimum Functional Requirements

This section contains a template for disclosure of coverage of minimum functional requirements with a proposal.   This template must be filled in and included with a proposal submission.

| Number | Category | Name | Coverage (Complete /Partial/ None) | Notes | References |
| --- | --- | --- | --- | --- | --- |
| FR1 | TOPO_RT_FWD | Mesh Topology Discovery | Partial |  | Section 8.3 |
| FR2 | TOPO_RT_FWD | Mesh Routing Protocol | Partial |  | Section 8.4 |
| FR3 | TOPO_RT_FWD | Extensible Mesh Routing Architecture | Partial |  | Section 8.4 |
| FR4 | TOPO_RT_FWD | Mesh Broadcast Data Delivery | Complete |  | Section 7.2.9 |
| FR5 | TOPO_RT_FWD | Mesh Unicast Data Delivery | Complete |  | Section 7.2.4 |
| FR6 | TOPO_RT_FWD | Support for Single and Multiple Radios | Complete |  | Section 4.2 |
| FR7 | TOPO_RT_FWD | Mesh Network Size | Partial |  | Section 8.5 |
| FR8 | SECURITY | Mesh Security | Partial |  | Section 8.2 |
| FR9 | MEAS | Radio-Aware Routing Metrics | Partial |  | Section 8.4 |
| FR10 | SERV_CMP | Backwards compatibility with legacy BSS and STA | Complete |  | Section 4.2 |
| FR11 | SERV_CMP | Use of WDS 4-Addr Frame or Extension | Complete |  | Section 4.2 |
| FR12 | DISC_ASSOC | Discovery and Association with a WLAN Mesh | Complete |  | Section 7.1.2 |
| FR13 | MMAC | Amendment to MAC with no PHY changes required | Complete |  | Section 8.1 |
| FR14 | INTRWRK | Compatibility with higher-layer protocols | Complete |  | Section 5 |

Table of Contents

1.	References	8

2.	Definitions	9

3.	Abbreviations and Acronyms	11

4.	Introduction	13

4.1.	Purpose	13

4.2.	General description	13

5.	Overview of MAC Services	16

5.1.	Protocol stack	16

5.2.	Overview of the MDCF	16

5.3.	Relay function	17

6.	Frame Formats in the MDCF	18

6.1.	General	18

6.2.	MACP Protocol Data Units (MPDUs)	18

6.2.1.	General frame format	18

6.2.2.1.	Frame control field	19

6.2.2.2.	QoS control field	19

6.2.2.	Management frames	20

6.2.2.1.	Beacon frame	20

6.2.3.	Control frames	20

6.2.3.1.	Link setup request (LinkSetupReq) frame	21

6.2.3.2.	Traffic channel reservation request (TCHReq) frame	21

6.2.4.	Data frames	21

6.3.	RLCP Protocol Data Units (RPDUs)	21

6.3.1.	General frame format	21

6.3.1.1.	Frame control field	22

6.3.2.	Link control frames	23

6.3.2.1.	Link setup request (LinkSetupReq) frame	23

6.3.2.2.	Link release request (LinkRelReq) frame	23

6.3.3.	Association control frames	24

6.3.4.	Radio resource control frames	24

6.3.4.1.	Traffic channel utilization request (TCHUtiReq) frame	24

6.3.4.2.	Traffic channel utilization respond (TCHUtiRes) frame	24

6.3.4.3.	Traffic channel reservation request (TCHReq) frame	24

6.3.5.	AM Data frames	25

6.3.5.1.	Request frame for adjusting ARQ parameters (ARQParReq)	25

6.3.5.2.	Respond frame for adjusting ARQ parameters (ARQParRes)	25

6.3.5.3.	ARQ polling frame	25

6.3.5.4.	ARQ data frame	25

6.3.5.5.	ARQ status report (ARQStatusRep) frame	25

6.3.6.	UM data frames	26

7.	MDCF description	27

7.1.	Architecture of MDCF	27

7.1.1.	Media Access Control Protocol (MACP)	27

7.1.2.	Radio Link Control Protocol (RLCP)	28

7.2.	MACP	29

7.2.1.	TDMA frame and energy signals	29

7.2.2.	Prioritized access	30

7.2.2.1.	Contention process	30

7.2.2.2.	PP Contention levels (PPCLs)	31

7.2.2.3.	Fairness access	32

7.2.3.	Link setup and TCH reservation	32

7.2.4.	Transmission and On-demand-TDD	33

7.2.5.	Calming down hidden stations	34

7.2.6.	Eliminating Exposed Station problem	36

7.2.7.	Packet multiplexing	36

7.2.8.	Multi-hop operation	37

7.2.9.	Broadcast and multicast	37

7.2.10.	TCH control	37

7.2.10.1.	Hang-on release	37

7.2.10.2.	Valid transmission time (VTT)	38

7.2.10.3.	Forced release of TCH	38

7.2.10.4.	Adaptation of TCH number for a link	38

7.2.10.5.	MACP Protocol Data Unit (MPDU) trains	39

7.2.11.	Synchronization	40

7.2.11.1.	General	40

7.2.11.2.	Maintaining synchronization	41

7.2.11.3.	The MTSF finite state machine	41

7.2.11.4.	MTSF timers	43

7.2.11.5.	On reception of a valid beacon	44

7.2.11.6.	Multi-hop support	46

7.3.	RLCP	46

7.3.1.	Service modes	46

7.3.1.1.	UM	46

7.3.1.2.	AM	47

7.3.2.	Error control and flow control (informative)	47

7.3.3.	RLCP transmission processes	48

7.3.3.1.	Virtual Link setup	48

7.3.3.2.	Data delivery	49

7.3.3.3.	TCH reservation request	50

7.3.4.	Allocation of service entities	50

7.3.5.	Adaptation of ARQ parameters during transmission	50

7.3.6.	Radio Resource Control	50

7.3.6.1.	Call Admission Control (CAC)	50

7.3.6.2.	Adjusting of traffic channels for use	52

8.	Compatible issues	53

8.1.	Physical layer (PHY)	53

8.2.	Security	53

8.3.	Mesh topology discovery	53

8.4.	Routing	53

8.5.	Mesh network size	53

9.	Acknowledgements	54

List of Figures

Figure 1: An example of WDS	13

Figure 2: The Data Link Layer model in a Mesh AP	14

Figure 3: Mapping of IEEE 802 Reference Model to scope of the proposal.	16

Figure 4: Relationships of various PDUs.	18

Figure 5: MPDU frame format.	19

Figure 6: Frame control field in the MPDU	19

Figure 7: Beacon frame format	20

Figure 8: Control frame format	20

Figure 9: Format of Data Frame Type 1	21

Figure 10: Format of Data Frame Type 2	21

Figure 11: Format of Data Frame Type 3	21

Figure 12: RPDU frame format	22

Figure 13: Frame control field in the RPDU	22

Figure 14: Format of request frame for ARQ parameter changing.	25

Figure 15: Format of ARQ data frame.	25

Figure 16: Format of ARQ status report frame.	26

Figure 17: Format of UM Data frame.	26

Figure 18: The architecture of MDCF	27

Figure 19: TDMA frame and energy signals	29

Figure 20: ACH structure.	30

Figure 21: An example of contending for an access. STA S1, S2 and S3 are in the transmission range of one another.	31

Figure 22: Process for link setup and TCH reservation.	32

Figure 23: An example of calming down hidden station nearby transmission pairs by transmitting BESes.	33

Figure 24: An example of transmission process and on-demand-TDD.	33

Figure 25: Calming down hidden station in MDCF mesh networks.	35

Figure 26: An example of the packet multiplexing between two mesh APs.	36

Figure 27: Multi-hop forwarding might take place in a same TDMA frame	37

Figure 28: Release of a TCH after the expiration of the set hang-on time. The hang-on time in the example is 2 TDMA frames.	38

Figure 29: An example of adaptation of TCH number for a link. The hang-on time is 1 TDMA frame and the maximum allowed number of TCHs for the link is 3.	39

Figure 30: An example of transmitting as MPDU trains.	39

Figure 31: Finite state machine of the MTSF.	42

Figure 32: Time skew in TDMA operation	44

Figure 33: Data delivery between two UM entities over the radio interface	47

Figure 34: Data delivery between two AM entities over the radio interface	47

Figure 35: RLCP Transmission processes	49

List of Tables

Table 1: Type and Subtype combinations	19

Table 2: QoS Control fields	20

Table 3: Beacon frame body	20

Table 4: Type and Subtype combinations	22

Table 5: Frame body of the connection request frame	23

Table 6: Frame body of the disconnection request frame	23

Table 7: Frame body of connection request frame	24

Table 8: Frame body of the traffic channel reservation frame	24

Table 9: Contention levels in the MDCF.	31

Table 10: The states used in the MTSF state machine.	42

Table 11: Timers in the MTSF.	43

- References

- IEEE 802 11-04/54r2, PAR for IEEE 802.11s ESS Mesh.

- IEEE 802 11-04/56r1, Five Criteria for IEEE 802.11s ESS Mesh.

- IEEE Std. 802.11, Wireless LAN Media Access Control (MAC) and Physical Layer (PHY) specification, 1999.

- IEEE 802.11e/D13.0, Medium Access Control (MAC) Quality of Service (QoS) Enhancements, Jan. 2005

- IEEE 802 11-04/662r11, TGs Usage Models.

- S. Mangold, S. Choi, G. Hiertz, O. Klein and B. Walke, "Analysis of IEEE 802.11 for QoS Support in Wireless LANs," In IEEE Wireless Communications, Vol. 10, pp. 2-12, Dec. 2003

- D. Chen, S. Garg,, M. Kappes, and K. S. Trivedi, "Supporting VoIP traffic in IEEE 802.11 WLAN with enhanced medium access control (MAC) for quality of service", www.research.avayalabs.com/techreport/ALR-2002-025-paper.pdf.

- F. A. Tobagi and L. Kleinrock, “Packet Switching in Radio Channels: Part II—The Hidden Terminal Problem in Carrier Sense Multiple-access and the Busy-tone Solution,” IEEE Trans. Commun., Vol. COM-23, pp. 1417–1433, Dec. 1975.

- IEEE 802.11a: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications: High-speed Physical Layer in the 5 GHZ Band, 1999.

- RFC 2598 - An Expedited Forwarding PHB, June 1999.

- B. Williams and T. Camp, “Comparison of broadcasting techniques for mobile ad hoc networks.” Proc. ACM Symposium on Mobile Ad Hoc Networking & Computing (MOBIHOC 2002), pp.194–205, 2002.

- Definitions

Acknowledged mode (AM): The service mode which provided by the RLCP for reliable unicast transmissions.

Access channel (ACH): Time slots used to contend for the channel access by using AESes.

Access energy signal (AES): The energy signals used to implement the prioritized access mechanism in the MDCF.

Association: The mobile station used to establish a station membership in a BSS network.

Busy energy signal (BES): The energy signals used to notify the occupancy of traffic channels.

Call admission control (CAC): An algorithm to ensure the admittance of a new flow into a resource constrained network does not violate parameterized service commitments made by the network to admitted flows.

Disassociation: The mobile station removing an existing association.

Echo channel (ECH): Time slots used to transmit BESes.

Energy signal: A short period of busy signal transmitted over the wireless medium by a transmitter. Energy signals are classed into two types according to the time slots in which they are used: AESes in the ACH while BESes in ECHs.

Fragmentation: In the MDCF, the process of partitioning of a LLC PDU into a sequence of smaller RPDUs prior to transmission. The process of recombining a set of fragment RPDUs into a LLC PDU is known as defragmentation.

Frame: The format of aggregated bits from a protocol entity that are transmitted together in time.

Hidden station: A station whose transmissions cannot be detected using carrier sense by a second station but whose transmissions interfere with transmissions from the second station to a third station,

MACP protocol data unit (MPDU): The units of data exchanged between two peer MACP entities in MDCF networks, using the services of the physical layer.

Mesh distributed coordination function (MDCF): A class of distributed function used in each mesh AP to form a multi-hop network based on the TDMA/TDD technology.

Link: A physical path consisting of exactly one traversal of the wireless medium that is used to transfer RPDUs between two stations.

Virtual Link identifier (V-Link ID): Each service entity in the RLCP entity is assigned with a virtual link identification number. A V-Link ID should be unique between a transmission pair. In a mesh AP, RPDUs from service entities with different V-Link IDs but a same destination shall be multiplexed on the reserved TCHs between the mesh AP and its transmission partner.

Protocol Data Unit (PDU): The unit of data exchanged between peer entities.

Packet: The format of aggregated bits that are transmitted together in time across the physical medium,

Piggyback: The overloading of a frame of type data with an acknowledgement of previously received RPDUs to the station to which the frame is directed.

Quality of Service (QoS): A collective measure of the level of service delivered between mobile stations. Quality of Service is characterized by basic performance criteria, including availability, drop rate, throughput, packet delay and jitter and connection setup time.

RLCP protocol data unit (RPDU): The units of data exchanged between two peer RLCP entities in MDCF networks, using the services of MACP.

Time division duplexing (TDD): A transmission method that uses only one time slot both for transmitting and receiving.

Time division multiple access (TDMA): Dividing a radio frequency into time slots and then allocating slots to multiple links. In this way, a single frequency can support multiple, simultaneous data channels.

Traffic channel (TCH): Time slots used to transmit data packets.

Traffic identifier (TID): Any of identifiers usable by higher-layer entities to distinguish MPDUs to support QoS within the MACP in the MDCF.

Unacknowledged mode (UM): The service mode provided by the RLCP for connectionless unicast, multicast and broadcast transmissions.

Wireless medium (WM): The medium used to implement the transfer of PDUs between peer PHY entities of an WLAN.

- Abbreviations and Acronyms

ACH 	Access Channel

AES	Access-E-Signal

AGC	Automatic Gain Control

AM 	Acknowledged Mode

AP 	Access Point

ARQ	Automatic Request

BES	Busy-E-Signal

BSS	Basic Service Set

BSSID	Basic Service Set identification

CAC 	Call Admission Control

CRC 	Cyclic Redundancy Code

DA	Destination Address

DCF 	Distributed Coordination Function

DiffServ 	Differentiated Services

DL	Data Link

DVB 	Double Value Busy-E-Signal

ECH 	Echo Channel

EDCA	Enhanced Distributed Channel Access

EF 	Expedited Forwarding

ESS	Extended Service Set

HCF	Hybrid Coordination Function

FA 	Fair Access

FCS	Frame Check Sequence

FTP 	File Transmission Protocol

V-Link ID	Virtual Link identification

LLC	Logic Link Control

LSB	Least Significant Bit

MAC 	Media Access Control

MACP	Media Access Control Protocol

MDCF	Mesh Distributed Coordination Function

MPDU	MACP Protocol Data Unit

MSB	most Significant Bit

MTSF	MDCF time synchronization Function

nQSTA	non-QoS Station

PDU	Protocol Data Unit

PHB 	Per Hop Behaviors

PHY  	Physical layer

PLR	Packet Loss Rate

PPM	parts per million

QoS	Quality of Service

QTS	QoS-related Traffic Specification

QSTA	QoS station

RA	Receiver Address

RLCP 	Radio Link Control Protocol

RPDUs	RLC PDUs

SA	Source Address

SR-ARQ	Selective Repeat Automatic Request

STA	Station

SVB 	Single Value Busy-E-Signal

TA	Transmitter Address

TC	Traffic Category

TCH 	Traffic Channel

TCP		Transmission Control Protocol

TDD 		Time Division Duplex

TDMA 		Time Division Multiple Access

UM 	Unacknowledged Mode

VoIP	Voice over IP

VTT 	Valid Transmission Time

WDS	Wireless Distribution System

WM	Wireless Medium

WLAN 		Wireless LAN

- Introduction

  - Purpose

This document describes a Media Access Control (MAC) proposal for Extended Service Set (ESS) Mesh Networking with the scope of the Project Authorization Request (PAR) [1] and Five Criteria [2] specified for IEEE 802.11 Task Group “s”.

  - General description

An IEEE 802.11 ESS mesh network is a collection of Access points (AP) interconnected with wireless links [1]. Based on a self-formed ESS mesh network, a Wireless Distributed System (WDS) can be created, covering a wide range of area. It is required that the MAC protocol, used to form an ESS mesh network, is able to support multi-hop operation efficiently. But the IEEE 802.11 legacy MAC protocol [3] and its Quality of Service (QoS) amendment [4] support multi-hop operation poorly. Meanwhile, driven by the market, a WDS is expected to be able to deliver various types of real-time traffic [5] in addition to best-effort traffic.  Transport of real-time traffic flows requires delivering data under QoS requirements. The QoS metrics especially delay and jitter metrics impose a great challenge for multi-hop operation. A high achievable network throughput not necessarily goes in hand with a low packet delay. Moreover, as formed to exchange various flows between APs, an ESS mesh network would be quite often in a heavily loaded situation. From results reported in [6][7], the 802.11e Enhanced Distributed Channel Access (EDCA) and HCF Controlled Channel Access (HCCA) can hardly handle this situation. In short, an ESS mesh network is supposed to be able to support QoS transport in multi-hop operation under a highly loaded situation. Therefore minor amendments on the current IEEE 802.11 MAC mechanisms might hardly fulfill the task requirement.

A WDS normally covers a wide range of area, providing the wireless service for a large number of mobile stations. One frequency channel mostly cannot offer enough bandwidth to sustain a normal traffic exchange in a WDS. We therefore deem that a WDS should be built up on a multi-radio platform. In this document, we present a MAC amendment with the combination of newly proposed Mesh Distributed Coordination Function (MDCF) and current IEEE 802.11 MAC functions like Distributed Coordination Function (DCF) and EDCA etc. by taking advantage of the multi-radio platform.

Figure 1: An example of WDS

Figure 1 shows an example of a WDS. The MDCF is used in between mesh APs working on one or several frequency channels dependent on the expected traffic volume, while the EDCF/HCCA/DCF/PCF is used between a mobile station and a mesh AP in a Basic Service Set (BSS) network on another frequency channel. The network, formed among mesh APs by using MDCF, supports QoS delivery in multi-hop operation, while a BSS network, formed by using EDCF/HCCA/DCF/PCF, allows only single hop delivery. A WDS is created by connecting BSS networks with the multi-hop AP network.

A mesh AP should work on two or even more frequencies with at least two transceivers. Figure 2 describes the Data Link (DL) Layer model of a mesh AP. The DL layer comprises Logical Link Control (LLC) and MAC sublayers. A MAC entity consists of three functions. The Hybrid Coordination Function (HCF) including EDCA and HCCA is used for providing the BSS service with the QoS capability to associated mobile stations. The MDCF is used to form a multi-hop network between mesh APs. The Relay function performs the packet relay in a mesh AP. The frame conversion is a basic service of it. No need to modify IEEE 802.11 Physical layer (PHY) is required.

Figure 2: The Data Link Layer model in a Mesh AP

Like the DCF and EDCA, the MDCF is also a distributed control protocol. A MDCF network is formed in a fully ad-hoc fashion. In difference to current IEEE 802.11 MAC mechanisms, the MDCF is based on Time Division Multiple Access (TDMA) technology. Transmission takes place in periodic time slots. The operation of such a network requires that stations are synchronized.

The major features of the MDCF can be simply summarized as follows:

- A distributed synchronization mechanism is proposed for achieving synchronization among mesh APs for TDMA operation.

- Inhibit hidden stations in multi-hop environments. Eliminate exposed station problem.

- An efficient multi-hop operation is implemented on a single frequency channel without using an extra device.  In case of a heavily loaded ESS mesh network, more frequency channels can be put in use purely for increasing the bandwidth.

- A two-stage prioritized access mechanism is used to implement a differentiated access as well as guarantee a fair share of bandwidth between end-to-end connections. The differentiated levels can be easily extended a lot at the expense of a small overhead.

- The hop-to-hop forwarding of a multi-hop transmission might take place simultaneously in different traffic slots, achieving a very low end-to-end packet delay.

- Traffic slots established between adjacent stations are used to multiplex any packets transmitted on the route. A higher Qos packet is transmitted firstly. For highly loaded links, several traffic slots can be long time reserved for use, which dramatically reduces overall contention, and therefore significantly increases the network throughput and reduces end-to-end packet delays.

- Several time slots can be combined together for a specific traffic service according to it QoS requirement. A distributed Call Admission Control (CAC) mechanism helps to guarantee the QoS of established connections. Radio resource utilization, which information is necessary for a CAC, can be derived by stations directly from observing time slots.

- Channel management mechanisms help to achieve high channel utilization and a fair share of bandwidth when serving for a larger number of connections in parallel.

The remaining parts of the document are organized as follows: Section 5 introduces the overview of the MAC services. The Frame formats used in the MDCF are defined in Section 6. Section 7 describes the MDCF in details. , Section 8 presents a description of various compatible issues including PHY, security, mesh topology discovery, routing and mesh network size, respectively.

- Overview of MAC Services

  - Protocol stack

Figure 3 shows the protocol stack in the IEEE 802 reference model and the logical divisions of the proposed mesh MAC protocol, used in a mesh AP. The mesh MAC amendment comprises three function blocks. The Hybrid Coordination Function (HCF) including EDCA and HCCA is used in a BSS network, providing communication services with QoS capability between a mesh AP and it associated QoS station (QSTA) or non-QoS station (nQSTA) on a frequency channel. The Mesh Distributed Function (MDCF) is used by mesh APs to form a multi-hop network on another frequency channel. The Relay function performs the packet relay in a mesh AP like frames conversion between the HCF and MDCF. The MDCF contains two protocols: Radio Link Control Protocol (RLCP) and Media Access Control Protocol (MACP).

Figure 3: Mapping of IEEE 802 Reference Model to scope of the proposal.

  - Overview of the MDCF

A MDCF network is formed in a fully distributed manner on a single frequency channel. Any STAs can communicate with any other STAs as long as they are in the transmission range of one another. It may allow multi-hops to route messages from any STA to any other STA on the network. The MDCF is based on the TDMA\TDD technology. The operation of a network requires that STAs are synchronized. Transmissions take place in periodic time slots called traffic slots. Energy-Signals, in-band busy tones, are used to implement a two-stage prioritized access mechanism and to inhibit hidden stations nearby a transmitter. A MACF entity consists of a RLCP and a MACP entity, respectively.

The functions of the RLCP include:

- Association control

- Link management

- Error control

- Radio resource control

- Flows control

- Segmentation and reassembly operation

The functions of the MACP include:

- Prioritized wireless medium access

- Synchronization for TDMA operation

- Transport of MACP protocol data units (MPDUs) over TDMA channels

- Multiplexing packets onto reserved traffic channels under Differentiated Services (DiffServ)

- Priority handling of packet data flows

- TDMA channels on Time Division Duplex (TDD) operation

- Claming down hidden stations

- Error checking

  - Relay function

The major functions of the relay module include:

- Frame formats conversion between the frames used in the HCF and the frames used in MDCF.

- Frame filtering based on layer 2 fields.

- Frame Formats in the MDCF

This clause specifies the frame formats used in the MDCF.

  - General

Figure 4 describes the relationships of various protocol data units (PDUs). The RLCP fragments a received LLC PDU into several shorter RLCP protocol data units (RPDUs). A protocol header is added to each RPDU. The length of a RPDU is dependent on the PHY data rate and the duration of a traffic slot. A MACP protocol data unit (MPDU) is assembled in the MACP by adding a MACP header directly on a RPDU. Then a PHY preamble is added to a MPDU before the MPDU is transmitted over the radio interface at a specified time slot. The preamble enables a MPDU to be decoded at the receiver side. The amount of PHY overheads include Tx power ramp on, the preamble used for Automatic Gain Control (AGC), synchronization and channel estimation, Tx power ramp off and a guard time. Those overheads are PHY scheme and application dependent. For instance, assuming the IEEE 802.11a PHY and WLAN application, the amount of PHY overheads is 9 µs.

Figure 4: Relationships of various PDUs.

  - MACP Protocol Data Units (MPDUs)

Each frame consists of the following components:

- A MACP header, comprising frame control, address, QoS control fields;

- A frame body, containing information specific to the frame type;

- A frame check sequence (FCS), containing an IEEE 32-bit cyclic redundancy code (CRC).

### General frame format

- Figure 5 shows the MPDU frame format. The MPDU frame format comprises a set of fields that occur in a fixed order in all frames. The fields Address 1, Address 2, Address 3 and Address 4 only appear in certain frame types. The length of a frame body varies from 0 to 1024 dependent on the frame type as well as the PHY rate. The FCS has a fixed length of 4 octets.

The definitions of addresses are same as the definitions in [3].  There are five kinds of addresses: Receiver Address (RA), Transmitter Address (TA), Destination Address (DA), Source Address (SA) and Basic Service Set identification (BSSID). All of those addresses are 6 octets in length.

- Octets:     1                6	                    6                    6                    6                1              0 – 1024               4

| - Frame - Control | - Address 1 | - Address 2 | - Address 3 | - Address 4 | - QoS Control | - Frame Body | - FCS |
| --- | --- | --- | --- | --- | --- | --- | --- |

MACP Header

Figure 5: MPDU frame format.

#### Frame control field

The Frame control field consists of the following fields: Type, Subtype and Reserved field. Figure 6 depicts the format. The Type filed is 2 bits in length and the Subtype field is 4 bits in length. The Type and Subtype fields together identify the function of the frame. There are three frame types: control, data and management. Table 1 defines the combinations of type and subtype.

Octets: B0               B1B2           B5                      B7

| Type | Subtype | Reserved |
| --- | --- | --- |

Figure 6: Frame control field in the MPDU

| Type value B0 B1 | Type description | Subtype value (binary) | Subtype description |
| --- | --- | --- | --- |
| 00 | Management | 0000 | Beacon |
| 00 | Management | others | Reserved |
| 01 | Control | 0000 | Link setup request |
| 01 | Control | 0001 | Traffic channel reservation request |
| 01 | Control | others | Reserved |
| 10 | Data | 0000 | Data frame Type 1: frame without address |
| 10 | Data | 0001 | Data frame Type 2: frame with 2 addresses (RA, TA) |
| 10 | Data | 0010 | Data frame Type 3: frame with 4 addresses (DA, SA, RA, TA) |
| 10 | Data | others | Reserved |
| 11 | Reserved | 0000-1111 | Reserved |

Table 1: Type and Subtype combinations

There are three subtypes of the Data frames. The first subtype frame contains no address. The second subtype contains the RA and TA. And the third one contains the RA, TA, DA and SA.

#### QoS control field

The QoS Control field is 8 bits in length. It describes the Traffic Category (TC) that the frame belongs to. 16 traffic levels are distinguished in the format of traffic identifier (TID). Table 2 shows the details. The third field describes the status of pending data packets in the transmission buffer. There are 16 levels of number of pending data packets. The way to organize the levels is implementation dependent.

| Applicable | Bits 0-3 | Bits 4-7 |
| --- | --- | --- |
| All frames | TID | Status of Pending Data Packets |

Table 2: QoS Control fields

### Management frames

#### Beacon frame

Figure 7 depicts the format of Beacon frame. The SA is the address of the station transmitting the frame. The value of BSSID field is the MAC address in use by the STA in the mesh AP of the BSS.

Octets:    1              6             6              1                0 – 1024          4

| Frame Control | SA | BSSID | QoS Control | Frame Body | FCS |
| --- | --- | --- | --- | --- | --- |

MACP Header

Figure 7: Beacon frame format

The information carried by the Beacon frame is shown in Table 3. The Transmitting time slot field uses 1 octet to indicate the time slot used for transmitting the beacon. The most significant bit (MSB) is used to indicate the type of time slot. The value 1 represents the ACH slot, while the value 0 means the TCH slot. If the MSB is 1, then remaining 7 bits are all filled with 0, otherwise the bits are used to indicate the sequence of traffic channel.

The traffic channel utilization is expressed in the second field with a length of 48 bits. The value of each bit represents the status of a traffic channel subsequently. The least significant bit (LSB) corresponds the traffic channel one. The value 1 indicates a corresponding traffic channel is in use, the value 0 means that the traffic channel is free.

| Order | Field | Length in Octet | Description |
| --- | --- | --- | --- |
| 1 | Transmitting time slot | 1 | Description of the time slot used to transmit the Beacon frame |
| 2 | Traffic channel utilization | 4 | Description of free and busy traffic channels observed by the source |

Table 3: Beacon frame body

### Control frames

The Control frame format is shown in Figure 8.  The RA is the MAC address of the intended immediate recipient. The TA is the address of the STA transmitting the frame. The DA is the address of the final recipient of forwarded application PDUs. The SA is the address of the STA generating application PDUs.

Octets:          1             6               6            6            6            1             0 – 1024         4

| Frame Control | RA | TA | DA | SA | QoS Control | Frame Body | FCS |
| --- | --- | --- | --- | --- | --- | --- | --- |

MACP Header

Figure 8: Control frame format

#### Link setup request (LinkSetupReq) frame

The frame body of the link setup request frame is the link setup request frame used in the RLCP. See 6.3.2.1 for detailed information.

#### Traffic channel reservation request (TCHReq) frame

The frame body of the traffic channel reservation frame is the traffic channel reservation request frame used in the RLCP. See 6.3.4.3 for detailed description.

### Data frames

Date frames are used to carry various RPDUs. There are three types of Data Frames: Data Frame Type 1, Data Frame Type 2 and Data Frame Type 3. Figure 9, Figure 10 and Figure 11 show their formats, respectively. The Data Frame Type 1 is most frequently used one, containing no address. The Data Frame Type 2 indicates the intended immediate recipient RA and the transmitting station TA in the MACP Header. The Data Frame Type 3 indicates RA, TA, the final recipient station DA and the source station SA in the MACP header.

Octets:          1              1             0 – 1024           4

| Frame Control | QoS Control | Frame Body | FCS |
| --- | --- | --- | --- |

MACP Header

Figure 9: Format of Data Frame Type 1

Octets:          1             6               6           1             0 – 1024         4

| Frame Control | RA | TA | QoS Control | Frame Body | FCS |
| --- | --- | --- | --- | --- | --- |

MACP Header

Figure 10: Format of Data Frame Type 2

Octets:          1             6               6            6            6            1             0 – 1024         4

| Frame Control | RA | TA | DA | SA | QoS Control | Frame Body | FCS |
| --- | --- | --- | --- | --- | --- | --- | --- |

MACP Header

Figure 11: Format of Data Frame Type 3

  - RLCP Protocol Data Units (RPDUs)

Each frame consists of the following components:

- A RLCP header, comprising frame control;

- A frame body, containing information specific to the frame type;

### General frame format

Figure 12 depicts the RPDU frame format. The RPDU Header is 1 octet in length, comprising the frame control filed. The length of frame body is dependent on the frame type, which information is contained in the frame control field.

Octets:	        1                         0 - 1023

| Frame Control | Frame Body |
| --- | --- |

RPDU Header

Figure 12: RPDU frame format

#### Frame control field

The Frame control field consists of the following fields: Type, Subtype and Reserved field. Figure 13 depicts the format. The Type filed is 2 bits in length and Subtype field is 4 bits in length. The Type and Subtype fields together identify the function of a frame. There are five frame types: Link Control, Association Control, Radio Resource Control, AM data and UM Data. Table 4 defines the combinations of type and subtype. Data frames are categorized as two types: acknowledged mode (AM) and unacknowledged mode (UM). All link Control frames are for one-hop links between a RA and a TA.

Octets: B0               B2B3           B6                      B7

| Type | Subtype | Reserved |
| --- | --- | --- |

Figure 13: Frame control field in the RPDU

| Type value B0 B1 | Type description | Subtype value (binary) | Subtype description |
| --- | --- | --- | --- |
| 000 | Link Control | 0000 | Link setup request |
| 000 | Link Control | 0001 | Link Release request |
| 000 | Link Control | Others | Reserved |
| 001 | Association Control | 0000-1111 | Reserved for no-AP MDCF stations |
| 010 | Radio Resource Control | 0000 | Channel utilization request |
| 010 | Radio Resource Control | 0001 | Channel utilization response |
| 010 | Radio Resource Control | 0010 | Traffic channel reservation request for an established link under QoS requirement. |
| 010 | Radio Resource Control | Others | Reserved |
| 011 | AM Data | 0000 | Request for ARQ parameter changing |
| 011 | AM Data | 0001 | Respond for ARQ parameter changing |
| 011 | AM Data | 0010 | ARQ polling frame |
| 011 | AM Data | 0011 | ARQ data frame |
| 011 | AM Data | 0100 | ARQ status report frame |
| 011 | AM Data | Others | Reserved |
| 100 | UM Data | 0000 | UM data frame |
| 100 | UM Data | Others | Reserved |
| other | Reserved | 0000-1111 | Reserved |

Table 4: Type and Subtype combinations

### Link control frames

#### Link setup request (LinkSetupReq) frame

Table 5 defines the frame body of the link setup request frame. The one-hop virtual link Identification (V-Link ID) field is 1 octet in length, specifying an integer number for an intended one-hop virtual link. A V-Link ID should be unique between a transmission pair. The traffic channel utilization is shown in Channel Utilization field. See 6.2.1.1 for a detailed description.

| Order | Field | Length in Octet | Description |
| --- | --- | --- | --- |
| 1 | Virtual Link ID | 1 | Specify the virtual link identification (V-Link ID). A V-Link ID should be unique between a transmission pair. |
| 2 | Traffic channel utilization | 4 | Description of free and busy traffic channels observed by the source |
| 3 | QoS description | 4 | Description of traffic type, it characteristic, the maximum allowed traffic channel number for use and the number of traffic channels presently needed. |
| 4 | Link parameter setting | 4 | Related link parameters setting in the RLCP |
| 5 | Traffic channels proposed for use | 4 | Description of a list of proposed channel for transmission |
| 6 | Proposal of PHY mode for use | 1 | Description of the proposed PHY mode for transmission |
| 7 | Remaining hops to DA | 1 | Description of the remaining hops needed to reach the final destination |

Table 5: Frame body of the connection request frame

The QoS Description field describes the traffic type and the estimated traffic characteristic like mean traffic load and the ratio of the highest load to mean load (burstiness). The traffic types include Voice over IP (VoIP), video, background and best-effort. It also specifies the maximum number of traffic channels needed for the transmission, and requested number of traffic channels right now. The link parameter setting field describes the service mode that should be selected for the one-hop virtual link in the RLCP. There are two modes available: Acknowledged mode (AM) and unacknowledged mode (UM). In case of AM, this field further specifies the key-parameter for Automatic Request (ARQ) protocol including the receiving window size, polling period and status reporting period etc. The traffic channel proposed for use filed proposes a list of traffic channels for transmission. It is 4 octets in length. The value of each bit represents a traffic channel subsequently. The LSB corresponds the traffic channel one. The value 1 indicates the corresponding traffic channel is proposed for use, the remaining bits are set as 0.

#### Link release request (LinkRelReq) frame

Table 6 shows the frame body of the link release request frame.

| Order | Field | Length in Octet | Description |
| --- | --- | --- | --- |
| 1 | V-Link ID | 1 | Specify the V-Link ID. A V Link ID should be unique between a transmission pair. |
| 2 | Traffic channel utilization | 4 | Description of free and busy channels observed by the source |

Table 6: Frame body of the disconnection request frame

### Association control frames

It is indicated in [1] that the mesh function will be implemented among mesh APs. There is no need for mesh APs to associate with each other. This field is reserved for future use when non-AP MDCF STAs shall be supported.

### Radio resource control frames

#### Traffic channel utilization request (TCHUtiReq) frame

No frame body is needed.

#### Traffic channel utilization respond (TCHUtiRes) frame

The frame body of traffic channel utilization request frame is shown in Table 7. The traffic channel utilization field is 4 octets in length, describing free and busy traffic channels observed by the transmitter. The definition of field structure is same as the definition of traffic channel utilization field described in 6.2.1.1. The field provides information including traffic channels being used by the transmitter, each transmission pair of a specific traffic channel and estimated release time of each traffic channel. This field is informative. The detailed description is left out in order to save space.

| Order | Field | Length in Octet | Description |
| --- | --- | --- | --- |
| 1 | Traffic channel utilization | 4 | Description of free and busy traffic channels observed by the transmitter |
| 2 | Traffic channels in use | 30 | Description of traffic channels being used by the transmitter, each transmission pair of a specific traffic channel and estimated release time of each traffic channel. |

Table 7: Frame body of connection request frame

#### Traffic channel reservation request (TCHReq) frame

Table 8 shows the frame body of the traffic channel reservation frame. It is a short version of the link setup request frame. See 6.3.2.1 for a detailed description.

| Order | Field | Length in Octet | Description |
| --- | --- | --- | --- |
| 1 | V-Link ID | 1 | Specify the V-Link ID. The combination of the V-Link ID and RA should be unique in a station. |
| 2 | Traffic channel utilization | 4 | Description of free and busy traffic channels observed by the source |
| 3 | QoS description | 4 | Description of traffic type, it characteristic, the maximum allowed traffic channel number for use and the number of traffic channels presently needed. |
| 4 | Traffic channel proposed for use | 4 | Description of a list of proposed traffic channel for transmission |
| 5 | Proposal of PHY mode for use | 1 | Description of the proposed PHY mode for transmission |

Table 8: Frame body of the traffic channel reservation frame

### AM Data frames

      - Request frame for adjusting ARQ parameters (ARQParReq)

Figure 14 depicts the format of request frame for adjusting ARQ parameters. The TxWindow and RxWindow fields specify the transmission window size at the transmitter side and receiving window size at the receiving side, respectively. The Poll_Period field describes the polling period of the transmitter and the Status_Period field specifies the status report period for the receiver. Both fields are 2 octets in length. An actual period is equal to the integer value time the TDMA frame length. The MaxDAT field specifies the maximum allowed number of times until a RPDU is correctly received or otherwise should be dropped.

Octets:	   2                   2                       2                            2                       1

| TxWindow | RxWindow | Poll_Period | Status_Period | MaxDAT |
| --- | --- | --- | --- | --- |

Figure 14: Format of request frame for ARQ parameter changing.

      - Respond frame for adjusting ARQ parameters (ARQParRes)

The format of respond frame for adjusting ARQ parameters is same as the format of request frame for adjusting ARQ parameters.

      - ARQ polling frame

No frame body is needed. The sender requests a status report from the receiver by sending the frame.

      - ARQ data frame

The format of ARQ data frame is shown in Figure 15. Both the Current Fragmented Sequence Number (CFSN) and Last Fragmented Sequence Number (LFSN) fields are 4 bits in length, indicating the current and last sequence numbers of fragmented RPDUs from a LLC PDU, respectively. The first fragmented Sequence number is 0. The Seq. field is a 14 bits field, indicating the sequence number of a LLC PDU. The Poll field (P) is a 1 bit field. This field is used to request a status report of the receiver. The Bit 1 means the sender requests a status report, while 0 means that a status report is not requested. The Status bit (S) is a 1 bit field. When the S bit is set, it means that a status report is piggybacked with the data frame. The Len_Data filed is an optional field, with a length of 16 bits. It indicates the length of the following data field. The Len_Status and Status fields are also optional fields. When S bit is not set, those two fields do not appear in an ARQ data frame. The Len_Status field is 8 bits in length, indicating the length of the following piggybacked status packet. The Status field is used to carry an ARQ status report packet. See 6.3.5.5 for a detailed information.

| CFSN | LFSN | Seq. | P | S | Len_Data | Data | Len_Status | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Bits:     4            4           14       1     1            16              0 - 8152             8                80 - 248

Figure 15: Format of ARQ data frame.

      - ARQ status report (ARQStatusRep) frame

Figure 16 depicts the format of ARQ status report frame. The RxWindow field shows the current receiving window size at the receiver side. The Status_Period field is a 2 octets field, indicating the status report period setting at the receiver. The Len_Bitmap field is 2 octets in length, indicating the length of the Bitmap field. The Starting Fragmented Sequence Number (SFSN) is 1 octet in length, describing the first fragmented sequence number appeared in the bitmap. The Starting Sequence Number field is a 2 octet field, specifying the first sequence number of a LLC PDU used for fragmentation in the bitmap. The Bitmap filed comprises a variable number of octets specified in the Len_Bitmap field.  The bitmap shows the status of received RPDUs from (SFSN, SS), the starting point corresponding to the LSB of the bitmap. The value 1 in a bit means that the corresponding RPDU is correctly received, while the value 0 means that the RPDU is not correctly received.

Octets:             2                      2                       2                1            2             1-22

| RxWindow | Status_Period | Len_Bitmap | SFSN | SSN | Bitmap |
| --- | --- | --- | --- | --- | --- |

Figure 16: Format of ARQ status report frame.

### UM data frames

Figure 17 depicts the formats of UM Data frames. The Current Fragmented Sequence Number (CFSN) and Last Fragmented Sequence Number (LFSN) fields are 3 bits in length, indicating the current and last sequence numbers of fragmented RPDUs from a LLC PDU, respectively. The first fragmented sequence number is 0. The Seq. field is a 10 bits field, indicating the sequence number of a LLC PDU. The Len_Data filed is 10 bits in length, indicating the length of the following data field.

| CFSN | LFSN | Seq. | Len_Data | Data |
| --- | --- | --- | --- | --- |

Bits:     3            3           10           16             0-8160

Figure 17: Format of UM Data frame.

- MDCF description

This clause presents the functional description of the Mesh Distributed Coordination Function (MDCF). The architecture of MDCF is introduced in 7.1. The MDCF comprises two parts: Media Access Control Protocol (MACP) and Radio Link Control Protocol (RLCP). The complete functional descriptions of the MACP and RLCP are provided in 7.2 and 7.3, respectively. Unless otherwise stated in this chapter, mesh APs are assumed to operate on a single frequency channel, and networks specially mean networks formed by those mesh APs.

  - Architecture of MDCF

Figure 18 depicts the architecture of the MDCF. The RLCP and MACP are two components of the MDCF. Transport of RPDUs from a RLCP entity to its peer entity relies on the services provided by its underlying MACP entity through the Service Access Point (SAP) between them. A RLCP entity provides the transmission service to the LLC entity and it also interacts with IEEE 802.11 HCF/DCF/PCF entities in a same mesh AP via the Relay function. The MACP implements the transmission service over the Wireless Medium (WM) by interacting with the PHY layer.

Figure 18: The architecture of MDCF

### Media Access Control Protocol (MACP)

The MACP is used by a mesh AP to acquire the right to use the WM in a fully distributed manner. It is based on Time Division Multiple Access\Time Division Duplex (TDMA\TDD) technology. A mesh AP is allowed to perform the media access procedure after it has achieved the frame level synchronization with its network neighbors in its initiation stage. A TDMA frame consists of three phases: media access phase, data transmission phase and notification of traffic slots occupation phase. Time slots in those phases are called Access Channel (ACH), Traffic Channel (TCH) and Echo Channel (ECH), respectively. Each ECH is associated to an ECH.

When a mesh AP has data to transmit, it at first checks whether there are sufficient free available TCHs for transmission. If so, it contends for channel access in the ACH following the two-stage prioritized contention procedure. If it wins, it transmits a link setup request packet to the destination mesh AP in the transmission phase of ACH. A link setup request packet contains the traffic description, the free TCHs observed by the initiator. On reception of the link set request packet, the destination mesh AP would decide to accept the request or not. It evaluates whether the common free TCHs available for both sides are able to satisfy the QoS requirement of requested link and whether the establishment of the link would not corrupt the QoS of established links. In case of accepting, the destination mesh AP will transmit energy signals on the ECHs corresponding to the granted TCHs in a same TDMA frame in which it received the link setup request in the transmission phase of the ACH.  On reception of energy signals on ECHs, the initiator knows the link request has been accepted and the TCHs have been reserved. Later on, it transmits data packets on the reserved TCH(s).  And the receiver transmits energy signals to notify its neighbors that specific TCH(s) are in use.

The mesh APs which lost a contention in the ACH shall contend again with an increased contention number used in the second stage of the ACH. A mesh AP, hearing either data packets on a TCH or energy signal on the corresponding ECH of the TCH, knows that the TCH has been reserved by other STAs, and it cannot use the TCH at present. The access contention and data packets transmission take place in different time slots. Therefore data packet transmissions and an ACH contention will not interfere with each other.  Selection of a TCH for data packet transmission needs an agreement between the transmitter and receiver. Each side has its own knowledge of available TCHs. Transmission on a newly selected TCH will therefore not cause interference to the existing transmissions, and also will not be interfered by the existing transmissions. The transmission range of energy signals used in the ACH is tuned to be larger than that of data packets on TCHs and energy signals on ECHs. A larger ACH contention range protects the potential hidden stations from simultaneously setting up two links, which might interfere with each other when later transmitting on a same TCH. .However a relatively short transmission range of data packets and energy signals assures a reasonable spatial reuse distance. A detailed description of the related issues is given in 7.2.5.

Multi-hop operation is possible when intermediate mesh APs would like to forward data packets which are not intended to them. The reserved TCH(s) can be used by the transmission pair to multiplex any data packet on the route. A higher QoS level packet would be handled first. Multiplexing operation increases the channel utilization and also reduces the number of overall contention in a network. A transmission pair would consider to release some or all the reserved TCH(s), or to contend again to gain more TCH(s) as changing of their traffic load and overall TCH(s) usage in their area.

### Radio Link Control Protocol (RLCP)

A RLCP entity runs on the top of the MACP and interacts with the LLC layer and the Relay function module. It fragments a LLC PDU into several smaller RPDUs suitable for transmission over the lossy WM before passing to the MACP. At the receiver side, the RLCP entity reassembles the received RPDUs into LLC PDUs and delivers them to the LLC layer. A RLCP entity may exchange data packets with the IEEE 802.11 HCF/DCF/PCF entity resided also in a mesh AP via the Relay function module, which mainly performs the frame format conversion between RPDUs and DCF MAC PDUs.

The RLCP also takes care of the link management issue. There are two types of service modes managed by the RLCP. An acknowledged mode (AM) entity offers error-free delivery service to the LLC layer. The Selective Repeat Automatic Request (SR_ARQ) mechanism, providing error control and flow control service, is the key component of AM. An unacknowledged mode (UM) entity transports a RPDU to its peer entity without the guarantee that the RPDU would be correctly delivered. A RLCP entity allocates link buffers according to the traffic type. For some traffic, it may allocate an ARQ entity only for a virtual link. And it may also allow several traffic flows to share an ARQ entity if their destinations and packet delay requirements are same. For UM traffic, the RLCP allocate a service buffer per destination. In a multi-hop environment, a RLCP entity needs to maintain several ARQ entities in parallel, which may lead to consume quite a lot buffer. If necessary, the RLCP entity would negotiate with some of its partners to change key parameters like transmission\receiving windows size, status reporting period etc. to control its buffer consumption. A RLCP entity may also change those ARQ parameters of an ARQ entity during transmission for achieving better traffic performance.  For instance, in order to lower the drop rate of a real-time traffic due to break the delay requirement, the RLCP entity may poll more frequently and ask the receiver to report more frequently.

The RLCP also manages the radio resource control. After receiving a link setup request indication from the MACP, the RLCP entity shall consider to accept the request only after it thinks that the current available TCH(s) can satisfy the QoS requirement of the connection and setting up the connection would not corrupt the QoS of established virtual links. Multi-hop connection requests shall be evaluated at the first forwarder by accounting for that a multi-hop connection consumes a multiple amount of radio resource than a single-hop connection does. A distributed Connection Admission Control (CAC) mechanism is used to decide whether to accept or reject a link setup request. A RLCP entity may contend to request more TCH(s) for a link if there is a sufficient amount of free TCH(s). It may ask its MACP entity to release TCH(s) currently used by a low QoS level link, and attempt to reserve the TCH(s) for a higher QoS level link by contending in the ACH.  A RLCP entity shall sometimes send channel utilization request packets to inquire the channel usage of the concerned mesh APs nearby in order to better control the radio resource.

Another function of the RLCP is the association control. Since the MDCF is currently used to form a network between mesh APs, there is no need for mesh APs to associate with each other. A mobile station needs to associate with a nearby mesh AP in a DCF/PCF/HCF BSS by using the procedure described in [3][4]. The RLCP reserves this function for future use when also non-AP MDCF mobile stations are supported.

  - MACP

### TDMA frame and energy signals

Unless otherwise stated, all the following time related parameters are example values assuming the IEEE 802.11a PHY.

The MDCF is based on TDMA/TDD technology. The operation of a network requires that the involved STAs are synchronized. A solution for synchronization is given in 7.2.11. The TDMA frame and the waveform of energy signals are shown in Figure 19. Energy signals, in-band busy tones [8], play important roles in the MDCF. An energy signal occupies a short time slice, for instance 6 μs. Energy signals are classed into two types according to the time slot in which they are used.

Figure 19: TDMA frame and energy signals

Each TDMA frame contains a number of time slots. Time slots are logically grouped into 4 types. The first type is the Access Channel (ACH), in which Access-E-Signals (AESes) are used to implement a two-stage prioritized access mechanism. The second type is called Traffic Channel (TCH), each slot carrying one data packet per TDMA frame. The third one is the Echo Channel (ECH). In a TDMA frame, the number of ECH slots is exactly same as that of TCH slots. Each ECH slot is paired with one TCH slot. An ECH slot is used by a receiving station to signal the occupancy of the corresponding TCH by transmitting a Busy-E-Signal (BES). The last slot in a TDMA frame is the guard time slot, with a period of less than an energy signal. It is used to separate two continuous TDMA frames for the purpose of synchronization.

Busy-E-Signals are used in the ECH, while Access-E-Signals are used in the ACH. BESes are categorized as Single Value Busy-E-Signals (SVBs) and Double Value Busy-E-Signals (DVBs) according to the signal length as shown in Figure 19. An Access-E-Signal has the exact waveform of a DVB. A SVB is used by a receiving STAs purely for informing its nearby stations of the occupancy of a specific TCH slot. In case that a receiving STAs has data packets for its transmitting STAs, it will transmit a DVB instead of a SVB to request the reverse transmission opportunity, i.e. the TCH in TDD mode of operation, in addition to its basic function as a BES.

The parameters like the number of TCHs, waveform of an energy signal, number of energy signals and length of a MAC frame may be different with different PHY schemes and applications. During operation, those parameters are never changed.

### Prioritized access

#### Contention process

An ACH slot consists of three phases: Prioritization Phase (PP), Contention Phase (CP) and Transmission Phase (TP), as shown in Figure 20. A number of binary AESes are used in the first two phases to implement a prioritized access mechanism. The PP is the prioritized contention phase, preferable to higher QoS traffic. The setting of CP is to guarantee with a high probability that there is only one winner under a heavy contention. It is assumed that the numbers of binary AESs in the PP and CP of an ACH are m and n, respectively. The number m is associated with the amount of contention priorities, n with the STA density. As long as a STA needs to reserve TCHs, or set up a link with the destination STA, or broadcast beacons or other packets in case that the STA has no reserved TCH for use, it would contend in the ACH for obtaining a chance to transmit in the TP of an ACH. The contention levels in the PP and CP are called PP contention levels (PPCLs) and CP contention levels (CPCLs), respectively.

Figure 20: ACH structure.

The contention is performed as follows:

- A station shall select a PPCL number according to the type of traffic and whether for multi-hop forwarding.  See 7.2.2.2 for a detailed description of PPCLs. The amount of contention levels is up to 2m. The higher the number, the higher the access priority.

- The STA checks the number bit by bit, when the bit is 1 it sends an AES, for 0 it listens. The most significant digit is transmitted first.

- During a listening period, once hearing an energy signal, the STA knows that it has lost the contention in the current TDMA frame. It must cancel its pending energy signals and contend again in the future.

- If surviving in PP, the STA shall use the same listening and sending scheme again to contend in CP by a number randomly generated from [0, 2n-1].

- If the station wins in the previous phases, it is allowed to transmit in the TP of the ACH.

- In case of losing, the STA shall contend again in the next TDMA frame.

Figure 21: An example of contending for an access. STA S1, S2 and S3 are in the transmission range of one another.

Figure 21 illustrates a contention process. Station S1, S2 and S3 are in the transmission range of each other. They contend for channel access at the same time. S1 and S2 want to reserve TCH(s) for delivering Voice over IP (VoIP) packets to their partners, while S3 wants to transmit video streams. Assume that the QoS priorities of the VoIP and video stream are 5 (101) and 3 (011) respectively. Both S1 and S2 win in the PP by means of listening and sending AESes. After that, each of them randomly generates a number and uses the number to compete again in the CP. As shown in Figure 21, the generated numbers of S1 and S2 for the CP are 441 (110111001) and 283 (100011011) respectively. S2 quits the CP contention immediately since it hears the second AES from S1 in the CP. Finally, S1 sends out a request packet in the TP

#### PP Contention levels (PPCLs)

Table 9 defines the PPCLs used in the PP.

| Access Category | Traffic Type | Multi-hop Forwarding | Contention Level |
| --- | --- | --- | --- |
| AC_Beacon | Control | - | 7 |
| AC_VO_MULTI | Voice | yes | 6 |
| AC_VO | Voice | no | 5 |
| AC_VI_MULTI | Video | yes | 4 |
| AC_VI | Video | no | 3 |
| AC_BE_MULTI | Background | yes | 2 |
| AC_BE | Background | no | 1 |
| AC_BK | Best Effort | - | 0 |

Table 9: Contention levels in the MDCF.

Beacons used for synchronization should be handled in the top priority. Four traffic services are currently under consideration: voice, video, background and best-effort. According to the different packet delay requirements, those traffic flows are assigned with different access priorities. The higher the packet delay requirement, the higher the contention level. In the multi-hop environment, a real-time traffic flow which needs to be forwarded by intermediate stations shall be handled with a higher priority over a same level single hop flow in order to achieve a better end-to-end delay performance.

#### Fairness access

The CP of the ACH is used to assure with a high probability that there is only one winner in every ACH contention. The contention level is the CP can be set in a reasonable range without introducing too much overhead. Assuming an AES has a duration of 6 μs and the number of AESes in the CP is n. by increasing an overhead of 6*n μs, the contention is up to 2n. A fairness access mechanism is implemented by taking advantage of the structure.

Whenever a STA contends for setting up a link or reserving TCHs, it is always for a specific virtual link (See 7.3.3) with a dedicated Virtual link identifier (V-link ID). In a STA, there may be several virtual links wishing to contend for the access at a certain period. As long as a STA loses a contention in the ACH when competing for a virtual link, the contention loss counter in association with the virtual link is incremented by one. When the STA contends for the access in the ACH, the contention number used in the CP is generated according to the contention loss counter of a virtual link. The higher the loss counter, the bigger contention number would be.  In a STA, a virtual link which is intended to serve for a higher level Access Category (AC) traffic, as shown in Table 9, will be handles first. For same level AC traffic virtual links, the one with the higher loss counter is handled first.

### Link setup and TCH reservation

When a STA wishes to transmit data packets, it first checks the local TCH status. A STA thinks that a TCH is free available if:

- No carrier is sensed in the TCH.

- No BES is sensed in the ECH corresponding to the TCH.

Otherwise the TCH is mark as busy by the station. In case the amount of available TCH(s) observed meets the traffic need, it would contend for an access in the ACH. If it wins the contention, it broadcasts a request packet for link setup/TCH(s) reservation containing the receiver address, QoS-related traffic specification (QTS) and a list of proposed TCH slots in the TP of the ACH. After receiving the request packet, the destination station makes the decision in the RLCP whether to accept the request by evaluating the received QTS and the free TCH slots available at its location. In case of acceptance, the receiver transmits SVB(s) in ECH(s) corresponding to the accepted TCH(s). Both the originator and nearby stations of the receiver obtain valuable information from the SVB(s). For the originator, it knows that the TCH(s) have been reserved. For the nearby stations, they know that the respective TCH(s) are in use and they cannot use them right now. Therefore potential hidden stations nearby the transmission pair are calmed down.

shows the process. MPME_xx in the Figure is used to represent a specific MACP management entities (MPME) primitive, used to exchange information between the RLCP and MACP. See 7.3.3 for more information. Section 7.3.3 also describes the difference between the link setup and TCH(s) reservation.

Figure 22: Process for link setup and TCH reservation.

An example of calming down hidden station nearby a transmission pair is shown in Figure 23. The notation TCH n/ECH n means nth TCH slot/nth ECH slot in a TDMA frame in the following context. A transmission is ongoing between station S1 and S8. S1 uses the TCH 3 to transmit data packets and the receiving station S8 replies with BESs on the ECH 3 to inform its nearby stations that the TCH 3 is in use. S4 and S5 are potential hidden stations to S1. When they have data to transmit, they shall select TCH(s) other than TCH 3 for transmission, since both of them hear the BES on ECH 3 and infer that the TCH 3 is currently in use. In this example, the TCH 4 is chosen by them.

Figure 23: An example of calming down hidden station nearby transmission pairs by transmitting BESes.

It should be noted that transmitting BESes on ECHs can only calm down hidden stations in the vicinity of a transmission pair. Hidden stations located far away may interfere with an on-going transmission in a multi-hop network. The integrated Hidden Station solution in the multi-hop environment is presented in 7.2.5.

### Transmission and On-demand-TDD

Once TCH(s) have been reserved for a transmission pair, the sender uses one or some of them to send out its data packets. No matter whether the receiver correctly receives the packets or not, it replies with the SVB(s) in the related ECH(s) to signal the occupancy of the respective TCH(s) in its environment. In case the receiver has some data to send back, it transmits a DVB instead of SVB on the corresponding ECH. If the sender senses the DVB, from the next frame on, it stops the transmission in the respective TCH(s) and takes the charge of transmitting energy signals in the ECH(s). And the receiver shall send out packets via the reserved TCH(s). This mechanism is called On-Demand-TDD. Figure 24 shows transmission and on-demand-TDD process.

Figure 24: An example of transmission process and on-demand-TDD.

It should be noted that a Type 1 Data frame only contains 9 octets overhead (6 octets from the MACP and 3 octets from the RLCP, see 6.2.4 and 6.3.5.4 for details). Whether to transmit a DVB by the receiver to request the reverse transmission opportunity is made according to the numbers of pending packets in the sender and receiver. If necessary, a station in the transmission shall initiate the TCH reservation procedure to obtain more TCH(s) for use in order to meet the QoS requirement.

### Calming down hidden stations

Working on a single frequency, the MDCF is able to calm down hidden stations. This part describes the solution and presents an in-depth analysis. The terms transmission range and carrier sensing range which are used in this part are defined as follows:

Transmission range – when a station is in the transmission range of a transmitter, it can decode the data packets from the transmitter.

Carrier sensing range – Stations in the carrier sensing range can sense the transmission from the transmitter, but they may not decode the data packets. The carrier sensing range and transmission range depend on the transmission power level and the PHY mode. The carrier sensing range is larger that the transmission range. In this context, it is assumed that the carrier sensing range is 2 times larger that the transmission range. When STA A is in the carrier sense range of STA B, transmissions from B may interfere with simultaneous transmissions destined to A.

The path loss of two STAs can be modeled as:

Pr=PTdγ

(1)

Where Pr is the received signal strength while PT is the transmission power, d is the distance between two STAs, and γ Є [2,4] is the attenuation factor.

Transmission policies are summarized as follows:

- When contending in the ACH, a STA transmits AESes using 2 times transmission power than for data packets on TCHs. A request packet is transmitted in the TP of ACH with a same power level as that used for transmitting data packets on TCHs. As indicated in Eq. 1, given a carrier sense threshold, the use of 2 times larger transmission power enlarges the carrier sense range 1.2 to 1.4 time assuming γ Є [2, 4].

- Data packets and BESes are transmitted with a same level transmission power on TCHs/ECHs to assure a reasonable spatial reuse distance.

Figure 25: Calming down hidden station in MDCF mesh networks.

Figure 25 shows a typical mesh network. Suppose STA E wants to initiate a transmission with STA F. It first senses the wireless medium to check the TCH status. Transmission from B, C, D, F, G, H can be sensed by it, while transmission from C, D, E, G, H, I can be sensed by F. In case that E finds enough free TCHs meeting its transmission requirement, it shall contend in the ACH to send a link setup/TCH reservation request packet to F. Since an AES contains no data information, a STA only needs to sense AESes during the PP and CP period. With using 2 time transmission power than for Data packets and BESes, its carrier influence range is extended to cover station A and I.  Therefore when STA E contends for the channel access in the ACH, there is only one winner (with a high probability) in an area covering STA A, B, C, D, E, F, G, H, I. In that area, it is not possible for two potential hidden stations to win contention simultaneously. Otherwise, they might select a same TCH slot for transmission, which cause to interfere with each other when transmitting. On reception of a link setup/TCH reservation request from STA E, F would select free TCHs which are not currently used by B, C, D, E, F, G, H, I. When STA I, which might be a potential hidden station to F, wants to initiate a transmission with J or H, since it can sense BESes or Data packets (on-demand-TDD) transmitted by F, it would select TCHs other than that are being used between station E and F. The similar choice will be made when station J wants to initiate a transmission to I. STA B and A are in the same situation as I and J, respectively. Since a STA located far away like J will use high enough transmitting power, which might cause interference to the transmission between E and F, to transmit AESes. However the contention in the ACH and transmission of Data packets take place in different time slots. Existing transmissions on the TCH will be not interfered by transmission on the ACH. When STA A and J want to transmit with the STAs which are not in the carrier sense range of E and F, they can reuse the TCHs currently used by E and F. The spatial reuse distance is the sum of the transmission range and carrier sensing range of the Data packet.

### Eliminating Exposed Station problem

In the example shown in Figure 25, assume a transmission is on-going between station E and F. When E transmits on TCHs, stations D, C and B are potential exposed stations of the transmission pair. While when F transmits on TCHs (On-demand-TDD), stations G, H and I are potential exposed stations of the transmission pair. However as long as there are free available TCHs in the vicinity of those stations, they can reserve some of TCHs for use. Therefore parallel transmissions between exposed station pairs are possible as long as TCHs are not fully reserved in a neighborhood.

### Packet multiplexing

A TCH established between adjacent STAa is used to multiplex any packets transmitting on the route. The sequence of transmission of packets competing for a TCH is according to their QoS priorities. Expedited Forwarding (EF) Per Hop Behaviors (PHB) specified in the Differentiated Services (DiffServ) protocol [10] is deployed in every station to multiplex packets onto any TCH that it is using.

Figure 26 illustrates the scheme. There is an ESS mesh network. Four mesh AP form a network on a frequency channel. BSS networks are formed between a mesh AP and its nearby mobile stations. Assume that three TCHs are reserved for transmission between the AP 2 and AP 1. Packets of different peer-to-peer connections are placed into several priority queues in the MACP entity of each station. The scheduler in the MACP issues the grants separately for each queue. All the queued packets are multiplexed into the reserved TCHs. The higher priority packets are preferred over the lower priority ones.

Figure 26: An example of the packet multiplexing between two mesh APs.

Multiplexing operation increases the traffic channel utilization. For highly loaded links, several TCHs can be long time reserved for use, leading to dramatically reduce overall contentions and therefore significantly increase the network throughput and reduce the end-to-end packet delays.

### Multi-hop operation

A multi-hop connection consists of multiple one-hop connections in tandem that each is independently controlled. As shown in Figure 27, owing to the TDMA structure, the hop-to-hop forwarding of a multi-hop transmission might take place simultaneously in different TCHs of a TDMA frame, obtaining very low end-to-end packets delays.

Figure 27: Multi-hop forwarding might take place in a same TDMA frame

### Broadcast and multicast

Broadcast and multicast MPDUs can be Type 2 or Type 3 Data MPDUs, containing 2 or 4 addresses as introduced in 6.2.4.  Transport of broadcast or multicast MPDUs is implemented by following approaches:

- In case a STA has reserved TCH(s) for use with another station, it will use one or some of the reserved TCH(s) to transmit broadcast/multicast MPDUs. And its transmission partner would transmit BESes on the corresponding ECH(s) as in the unicast situation.

- In case a STA does not have any reserved TCH(s) for use, it will select one STA from its one-hop neighbors list maintained in its cache, and try to send a TCH reservation request to the selected STA by contending in the ACH. As long as one or several TCH(s) have been reserved by the originator, it will transmit broadcast/multicast MPDUs on reserved TCH(s), and its selected partner STA will send BESes on ECH(s) corresponding to the reserved TCH(s).

- A STA may transmit single broadcast/multicast MPDU in the TP of the ACH after winning an ACH contention if the size of the MPDU fits into the TP.

Broadcast/multicast transmissions will not cause interference to, or get interference by other unicast transmissions in the multi-hop environment.

Delivery of broadcast/multicast MPDUs in an entire mesh network needs the support of the specific higher level protocol. The article [11] presents a summary of various algorithms used to implement an efficient broadcast/multicast delivery in multi-hop networks.

### TCH control

#### Hang-on release

A TCH is freed when meeting the following conditions:

- There is no packet in the TCH transmit buffer.

- The hang-on time specified for the TCH is expired.

When there is no packet for transmission in a TCH, the involved STAa would not release the TCH immediately. Instead, they may hold the TCH for a certain period (hang-on time), to wait for the arrival of new packets from the up-layer or from nearby STAs to be forwarded on the TCH. During the hang-on period, the newly arrived packets in the TCH transmit buffer are also transmitted via the TCH, and this cause a reset of the hang-on time at both sides. During a hang-on period, the receiving STA of the TCH shall transmit SVBs on the corresponding ECH. After the expiration of the hang-on time, a reserved TCH would be freed (No packet in TCH and No SVB in ECH). Figure 28 gives an example.

Figure 28: Release of a TCH after the expiration of the set hang-on time. The hang-on time in the example is 2 TDMA frames.

A TCH in hang-on status avoids high priority flows that must contend for reserving TCH(s). A longer hang-on time might also lead to the waste of capacity. Therefore a trade-off must be made. In the MDCF, a TCH in hang-on status is immediately assigned to any urgent request to establish a link, if no free TCHs are available.

The hang-on time for a TCH is set according to the contention level of flows. The higher the QoS level, the longer the duration.

#### Valid transmission time (VTT)

Several long-lived flows like File Transmission Protocol (FTP) traffic and video stream might cause a network to be in a saturated state. When this happens, stations cannot obtain a transmission chance even if they might have higher priority traffic like VoIP to transmit.

To prevent an excessive use of a TCH, each TCH is associated with a VTT value according to the traffic type. Generally speaking, a higher QoS traffic is assigned a long VTT value. Whether a STA releases a reserved TCH after the expiration of VTT or not depends on current TCH status. In case that there are not so many free TCHs available, the station will release the TCH immediately. Otherwise it keeps the TCH for use till the end of transmission or releases as long as there are not many free TCHs available. In case of releasing the TCH during transmission, the STA shall compete for a TCH again if necessary.

#### Forced release of TCH

The MACP may release a TCH on reception of the related primitive from the RLCP. See 7.3.6.2.1 for details.

#### Adaptation of TCH number for a link

- A STA shall contend for a link to get one more TCH if the current TCH(s) cannot satisfy the traffic needs. The allowed maximum number of TCH(s) for a link is associated with the type of traffic.

- A TCH shall be released for a number of reasons, see 7.2.10.1, 7.2.10.2, 7.2.10.3.

Figure 29: An example of adaptation of TCH number for a link. The hang-on time is 1 TDMA frame and the maximum allowed number of TCHs for the link is 3.

Figure 29 illustrates how the MDCF protocol handle a highly bursty real-time service like video streams in an efficient way, satisfying the transmission needs while not wasting the time slots.

#### MACP Protocol Data Unit (MPDU) trains

To meet the QoS requirement of a link, several TCHs can be concatenated together for transmission. When more than two adjacent TCHs are used together, some intermediate control periods can be removed as shown in Figure 30. However a long PHY PDU results in a high packet error rate when being transmitted over the wireless medium.  Therefore a long data period must be filled with separated short packets, each of which contains its own Cyclic Redundancy Check (CRC) information. The more adjacent TCHs are combined, the higher transmission efficiency will be.

Figure 30: An example of transmitting as MPDU trains.

### Synchronization

#### General

Operation of a MDCF mesh network requires that the frame level synchronization is achieved between all mesh APs in the network. This function is provided by an MDCF timing synchronization function (MTSF). The MTSF is implemented in a distributed fashion that shall be performed by all mesh APs. Each mesh AP may transmit beacons according to the algorithm described in the following sections. A mesh AP does not need to maintain a local time for synchronization. Instead, it only needs to record the starting time instant of the last TDMA frame.

Each mesh AP shall attempt to transmit beacons if it does not receive a valid beacon (see the definition of valid beacons in 7.2.11.2.2) in the specified interval. A beacon should be transmitted using the most robust PHY mode in a multi-PHY modes environment or a higher transmission power than other MPDUs. On reception of a valid beacon, a mesh AP will adjust the starting point of next TDMA frame which has been scheduled from the recorded starting time instant of last TDMA frame. A beacon contains no time related information but the information of the time slot on which it was transmitted. A receiving mesh AP can estimate the starting time instant of the TDMA frame of the sending mesh AP from the time slot information contained in the beacon. And it will then try to start the next TDMA frame simultaneously with the sending mesh AP.

The MTSF uses a compensation mechanism to combat the clock skews owing to different draft rates of oscillators at different mesh APs. Based on that, a high precision synchronization is achieved. A mechanism is used to help a mesh AP to check whether it is located at an overlapped area in a multi-hop network. If so, it would transmit beacons more often than nearby mesh APs. Otherwise, it shall reduce the frequency to transmit beacons. With the adaptive scheme, synchronization for TDMA operation in the multi-hop environment can be achieved.

Concerning the initiation of a MDCF network, mesh APs are classed into 2 types. Mesh APs which have right to initiate a MDCF network are called Type 1 mesh AP. Type 2 mesh APs are not allowed to initiate a mesh network. The initiator of a network in this context means the mesh AP which is allowed to send first beacon packet. In a single-hop MDCF network, every mesh APs can be a Type 1 mesh AP. However in a multi-hop MDCF network, only one mesh AP can be the Type 1 mesh AP and others should be Type 2 mesh AP for sake of the implementation of time synchronization for TDMA operation in the multi-hop environment.

The following definitions will be used in the rest of the part:

PTDMA	– the time period of a TDMA frame

BTS 	– the minimal integer number used for calculating beacon generation timer period

BTW 	– the window size used for calculating beacon generation timer period

STLin 	– the starting instant of the nth TDMA frame at the mesh AP i

STRin    – the starting instant of the nth TDMA frame of the beacon sending mesh AP, which instant is a derived value at the mesh AP i from the newly received beacon without considering the propagation delay and the clock skew caused by different drift rates of crystal oscillators at the beacon sender and receiver

ΔSTRLin– the time difference between STRin and STLin (i.e. ΔSTRLin = STRin - STLin) at the nth TDMA frame in the mesh AP i

Tin	– the local time when receiving the last bit of a beacon frame at the nth TDMA frame in the mesh AP i

Dpro       – the amount of process delays on the way from a beacon sending to a beacon analysis, including the computation times at the transmitting mesh AP and the receiving mesh AP, and initialization times of antenna and transceivers

ΔXij      – the clock skew caused by different drift rates of crystal oscillator between the sender i and the receiver j in a beacon interval

tij	– propagation delay between the mesh AP i and the mesh AP j

di       – a variable used to represent the compensated drift rate of the mesh AP i; di in units of ppm (parts per million)

Tadj	– the time instant till when a mesh AP is allowed to adjust di since it is synchronized

Tslow 	– the time instant from when a mesh AP is only allowed to slowly adjust di

T SYN	– the time instant when a mesh AP gains synchronization.

PACH    – the duration of an ACH including Prioritization Phase (PP), Contention Phase (CP) and Transmission Phase (TP)

PPP	– the duration of the PP in an ACH

PCP	– the duration of the CP in an ACH

PTCH	– the duration of a TCH

SB	– the length of the Beacon frame in bits

PHYB	– PHY data rate used to transmit beacons

iTCH   – the numbered TCH slot that a mesh AP received a beacon; the one next to ACH is number as 1, then  the number increases one by one till the one close to ECH.

#### Maintaining synchronization

Each mesh AP shall maintain a MTSF timer. Mesh APs expect to receive beacons at a nominal rate. The interval between beacon generations is defined by the parameter MeshBeaconPeriod within a mesh AP.

        - Beacon generation

Beacon generation in a mesh network is distributed. All members of the mesh network participate the beacon generation. Each mesh AP shall maintain its own MTSF timer that is used for MeshBeaconPeriod timing. The MeshBeaconPeriod shall be a multiple of the duration of a TDMA frame. The number TDMA frames comprising the MeshBeaconPeriod shall be randomly selected from a defined interval [BTS*PTDMA, (BTS+BTW)*PTDMA]. At the expiration of its own MTSF timer a mesh AP shall:

- Select a TDMA channel for the beacon transmission. If at least one TCH is in used by the mesh AP, one of them shall be used for the beacon transmission. Otherwise the beacon shall be sent on the ACH.

- Wait for the start of the selected channel, either ACH or TCH. In the case that the beacon has to be transmitted on the ACH, the mesh AP shall contend for access in the ACH with the highest priority.

- Cancel the pending beacon transmission, if a beacon arrives before the channel selected for beacon transmission starts.

- Send a beacon if the contention for the ACH is successful, or if the TCH selected for the beacon transmission starts.

- If a mesh AP receives a beacon before the expiration of its MTSF timer, it shall delete the pending beacon and initiate a new MTSF timer with a randomly selected MeshBeaconPeriod.

        - Beacon reception

Mesh APs shall use the information in any received beacon to check whether the TDMA frame structure of the MDCF is synchronized. Two types of a received beacon are distinguished:

- A valid beacon contains TDMA frame structure information that coincides with the local TDMA frame structure. That means that the beacon has been received in exactly the channel, either in the ACH or in a TCH, which is announced in the beacon. Therefore, a mesh AP that receives a valid beacon is assumed to be synchronized with the beacon transmitter.

- The second type of beacon is an invalid beacon. In an invalid beacon the contained transmission channel description does not coincide with the local TDMA frame structure. When a mesh AP receives an invalid beacon, it shall assume to be interfered.

#### The MTSF finite state machine

The MTSF finite state machine consists of four states shown in Table 10:

| state | Description | TDMA operation |
| --- | --- | --- |
| CLOSED | The mesh AP is inactive | Not allowed |
| SCAN | The mesh AP is scanning for a beacon | Not allowed |
| SYNC | The mesh AP is synchronized to all neighboring mesh APs | Allowed |
| INT | The mesh AP is surrounded by at least two unsynchronized mesh APs | Not allowed |

Table 10: The states used in the MTSF state machine.

The description of MTSF timers is presented in 7.2.11.4.

The behavior of a mesh AP concerning the MTSF is determined by the finite state machine shown in Figure 31.

Figure 31: Finite state machine of the MTSF.

A detailed description of the finite state machine is as follows:

- After being switched on or waked up, a mesh AP changes its state from the CLOSED to SCAN, and it shall initiate a T1 timer if it is a Type 1 mesh AP. In that state, the mesh AP is not synchronized to any other mesh AP and scans the WM for beacons. The duration of a T1 MTSF timer is determined by the parameter MeshBeaconPeriodSCAN. The parameter MeshBeaconPeriodSCAN shall be lager then MeshBeaconPeriodSYNC to cover a scanning duration long enough to receive any possible MeshBeaconPeriod of nearby mesh APs. Considering the possible loss of sent beacons, the MeshBeaconPeriodSCAN shall be a multiple of the mean MeshBeaconPeriod.

- In case a STA receives a beacon in the SCAN state, it synchronizes to the TDMA frame structure of the beacon sender. This synchronization is accomplished due to the beacon information that describes the beacon position in the sending mesh AP’s TDMA frame structure. The pending beacon transmission is deleted, the mesh AP’s state is changed to SYNC, and a T2 timer with a randomly selected MeshBeaconPeriodSYNC is initiated. In case a Type 1 mesh AP senses signals which it cannot decode or it can decode but the decoded packet is not a Beacon frame, it reset its T1 timer. As long as the T1 timer expires, a Type 1 mesh AP will broadcast a beacon, switch its state to the SYNC state and initiate a T2 timer.

- In the SYNC state, mesh APs are allowed to send and receive packets via TDMA operation. In case of the expiration of the T2 timer, a mesh AP shall broadcast a beacon either in the TP of the ACH or on a TCH if the mesh AP happens to reserve the TCH for use. After that, the mesh AP initiates a new T2 timer for future use. When a mesh AP receives a valid beacon, it would adjust the starting time of next TDMA frame by analyzing the arrival time of the beacon.

- In the SYNC state, the mesh APs which often receive more than 2 beacons in a T2 timer interval shall be given an enhanced right to transmit beacons frequently. The reason and approaches to enhance the beacon transmission right are presented at 7.2.11.6.

- In the SYNC state, in case a mesh AP receives invalid beacons defined at 7.2.11.2.2 b), it shall switch its state to the INT state, cancel T2 timer and initiate a T3 timer. The duration of a T3 is determined by MeshIntPeriod, a multiple of the mean MeshBeaconPeriodPeriodSYNC.

- In the INT state, a mesh AP is not allowed to send beacons and any other MPDUs. However it shall scan the WM for beacons. If a mesh AP senses unknown signal or receives beacons with different TMDA frame structure, it will reset T3 timer and stay in the state. Otherwise, when the T3 timer expires, it shall switch its state to the SCAN state.

#### MTSF timers

There are three types of timers defined in the MTSF. The T1, T2, T3 timers are used in the SCAN, SYNC and INT states, respectively. Table 11 presents a summary.

| MTSF timer | Used state | Interval |
| --- | --- | --- |
| T1 | SCAN | MeshBeaconPeriodSYNC, Randomly from [BTS*PTDMA, (BTS+BTW)*PTDMA] |
| T2 | SYNC | MeshBeaconPeriodSCAN, Multiple time of (BTS+BTW/2)*PTDMA |
| T3 | INT | MeshIntPeriod, Multiple time of (BTS+BTW/2)*PTDMA |

Table 11: Timers in the MTSF.

Beacon generation is in a distributed manner. In the SYNC state, every mesh AP shall attempt to broadcast beacons. Immediately after a mesh AP sent or received a valid beacon, it would set a new T2 timer for a new beacon generation. The period used for setting the T2 timer should be multiple time of PTDMA. A period is the multiply of a number randomly generated from the [BTS, BTS + BTW] and PTDMA. The value BTS is bigger than the value BTW. The mean beacon interval is (BTS+ BTW/2)*PTDMA.

The T1 timer is used in the SCAN state, during which period a mesh AP waits for the arrival of a beacon. The period of the T1 timer is equal to 10 * (BTS + BTW) * PTDMA.

The T3 timer is used in the INT state, during which period a mesh AP checks whether it is still located in an interfered area on a frequency channel where it sensed interference. The period of the T3 timer is equal to 5 * (BTS + BTW) * PTDMA.

#### On reception of a valid beacon

After receiving a valid beacon, a mesh AP shall adjust the starting time of the next TDMA frame based on the information gained from the received beacon.

Assume that a sending mesh AP and a receiving mesh AP are absolutely synchronized at the starting instant of a TDMA frame. After a certain period, the sending mesh AP successfully sends a Beacon frame, and the receiving mesh AP receives this valid beacon. There are a number of reasons contributing the time skew between two mesh APs at the instant that the receiving mesh AP finishes its Beacon frame reception.

- The clock skew owing to different drift rates of oscillators at two mesh APs since the absolutely synchronized instant.

- The propagation delay of the beacon from the sender to the receiver.

- The access delay which time period starts from the instant that a beacon is generated till it is transmitted by the sender over the radio.

- Various process delays including the computational time at the sender and receiver, as well as initialization times at intermediate devices like antenna and transmitter etc.

In summary, the time skews caused by the first two reasons are non-deterministic. The skew values may be quite different from time to time and also from STA to STA. However only the time skews caused by the last two are deterministic. In the MDCF TDMA system, a beacon can be generated after the mesh AP has gained the access right to use the WM, therefore the access delay is quite deterministic. It is equal to 0. In a given environment, the amount of process delays is almost a constant value for all mesh APs independent of time. In the MTSM, it is assumed that the sum of the access day and various process delays on the way from sending a beacon till receiving the beacon is a fix value. The remaining work is to estimate the propagation delay and clock skew due to different oscillator drift rates.

Figure 32 illustrates the time skew in a mesh AP when receiving a beacon.

Figure 32: Time skew in TDMA operation

The drift rates of crystal oscillator found in most consumer electronics range from 1 ppm (parts per million) to 100 ppm. The variable di is used in every mesh AP to compensate the oscillator drift effect when calculating the start of the next TDMA frame. The initial value of the variable is 0 and the variable is adjusted whenever receiving a valid beacon within a specified period from T SYN to T adj.

Assume at the nth TDMA frame the mesh AP i receives a beacon from the mesh AP j. The STRin and STLin (see the definitions in 7.2.11.1) can be calculated as follows (directly from definitions):

STLjn=STLjn−1+PTDMA+PTDMA×di                                          When no beacon arrives (n>1)   (1)

STRin=Tin−PPP−PCP−DPro−SBPHYB                           Received beacon in ACH             (2)

STRin=Tin−PACH−(iTCH)×PTCH−DPro−SBPHYB        Received beacon in TCH              (3)

In case it is the first received beacon to the mesh AP i, the mesh AP i shall start its first TDMA frame, and starting time can be calculated using Eq. 4:

STLi1=STRi0+PTDMA                                                                                                                        (4)

The STRin is a derived value without deducting the propagation delay and the clock skew caused by different drift rates of crystal oscillators. Then the value ΔSTRLin (see the definition in 7.2.11.1, ΔSTRLin = STRin - STLin) is equal to the propagation delay plus the clock skew between the sending mesh AP and the receiving mesh AP. Obviously, there exists a relation:

tij−ΔXij<ΔSTRLin<tij+ΔXij                                                                                                  (5)

The value ΔSTRLin is very important for TDMA operation. If ΔSTRLin is larger than the guard time in a TDMA frame, TDMA operation may be out of order. The Eq. 5 gives the lowest boundary to select the guard time in a specified environment. Upon receiving a beacon, a mesh AP shall compute its next TDMA frame starting time by Eq. 6:

STLin+1=STLin+ΔSTRLi2+PTDMA+PTDMA×di                                                                               (6)

The relation shown Eq. 5 can be used to design an oscillator drift compensation mechanism. As shown in Eq. 1 and Eq. 6, when computing the starting instant of a new TDMA frame, a mesh AP would use the estimated di to compensate the oscillator drift rate. After a certain period, the sum of the physical oscillator drift rate plus di in each mesh AP in a network tends to get close. Consequently, higher synchronization accuracy would be achieved as indicated in Eq. 5 (ΔXij --->0).

The adjustment of variable di is divided into two phases: fast compensation and slow compensation phase. After the second phase, di will not be changed anymore. The following are principles to adjust the variable di based on relation shown in Eq. 5.

When T SYN < Tin  < Tslow (Fast compensation phase):

If ΔSTRLin >  max(tij), 	            then di= di +10 ppm

Else if ΔSTRLin >  2*max(tij)/3,  then di= di +5 ppm

Else if ΔSTRLin < - 2*max(tij)/3, then di= di -5 ppm

Else if ΔSTRLin < - *max(tij),      then di= di -10 ppm

When Tslow < Tin  < Tadj (Slow compensation phase):

If ΔSTRLin >  max(tij), 	            then di= di +2 ppm

Else if ΔSTRLin >  2*max(tij)/3,  then di= di +1 ppm

Else if ΔSTRLin < - 2*max(tij)/3, then di= di -1 ppm

Else if ΔSTRLin < - max(tij),        then di= di -2 ppm

#### Multi-hop support

In the SYNC state, the mesh APs which receive a valid beacon shall reset their T2 timer. For beacon transmission on the ACH, the two-stage prioritized access mechanism guarantees that a beacon, which has the highest contention level, can be broadcasted even in a heavily contention environment. As introduced in 7.2.11.2.1, a STA shall cancel the pending beacon transmission as long as it receives a valid beacon. Therefore in an area where mesh APs are in the transmit range with each other, there should be exactly one beacon in a T2 timer interval. If a mesh AP receives more than 2 beacons in a T2 timer interval, the mesh AP realizes that it is in an overlap area of more than two transmission neighborhoods. When this mesh AP transmits a beacon, mesh APs, which are located in different transmitting neighborhood but nearby it, can receive the beacon. And if it is allowed to transmit beacons frequently, mesh APs which stand far away can adjust their time step on same messages, and be therefore kept synchronized.

There are two ways to help a mesh AP to transmit beacons more frequently than others.

The first approach is to divide the BTW into n smaller windows. Then there are ten levels for setting beacon generation timer: [BTS, BTS + BTW/n], [BTS + BTW/n-1, BTS + BTW/n-2], … [BTS + BTW/2, BTS + BTW]. In case a mesh AP has received multiple beacons in a period of (BTS+ BTW/2)*PTDM more than a specified times, it shall upgrade the window level by 1. In case the amount of continuously transmitted beacons exceeds a predefined number, it shall downgrade the window level by 1.

The second approach is to take advantage of the two-stage structure in ACH. The windows used to generate contention number for the CP of ACH can be divided into m levels like the first approach. In case a mesh AP should adjust its beacon sending level, it shall upgrade or downgrade the window level by 1.

The combined use of two approaches offers a high flexibility.

  - RLCP

### Service modes

There are two kinds of service modes: Unacknowledged Mode (UM) for connectionless point-to-point, multicast and broadcast applications; Acknowledged Mode (AM) for reliable point-to-point applications.  The Automatic Request (ARQ) protocol is a key component of the AM, offering error control and flow control for established one-hop connections.

#### UM

Figure 33 illustrates the data delivery process between two UM entities over the radio. The transmission entity segments the received LLC PDUs into smaller size RPDUs, and puts them into the transmission buffer after adding the RPDU header. On receiving RPDUs, the receiving UM entity removes the RPDU headers and reassemblies them into LLC RPUs and pass the LLC PDUs to the LLC layer.

Figure 33: Data delivery between two UM entities over the radio interface

#### AM

Figure 34 illustrates the data delivery process between two AM entities over the radio. The transmission AM entity segments the upcoming LLC PDUs into smaller size RPDUs, and put them into the transmission buffer after adding the RPDU header. On receiving RPDUs, the receiving AM entity removes the RPDU headers and reassemblies them into LLC RPUs and passes the LLC PDUs to the LLC layer. The transmission control module at the transmission entity transmits the data flow to the receiving control module at the receiving entity on the forward link, and the receiving control module sends receiving status reports to the transmission control modules on the backwards link. The error control and flow control are implemented by negotiating between two control modules.

Figure 34: Data delivery between two AM entities over the radio interface

### Error control and flow control (informative)

The Selective Repeat Automatic Request (SR_ARQ) mechanism is used to implement the error control and flow control for the AM link service. The transmission ARQ entity maintains a transmission window and the receiving window maintain a receiving window. On receiving the first Data RPDUs, the receiving entity would initiate the periodic status report mechanism, sending the receiving status report to the transmission entity. The status report can be transmitted by the status report RPDUs or piggybacked with the Data RPDUs in case that the receiving side has Data RPDUs to transmit to the transmitting side by the same entity pair.  The transmission entity shall also transmit a Data RPDU with a set poll bit or a polling RPDU to ask the transmission entity to report the receiving status immediately. The transmission entity initiates a poll in case that:

- scheduled polling timer expire

- the transmission window size is approaching the upper limitation

- last Data RPDU in the transmission buffer.

An arrival of a status report will cause the reset of the polling timer at the transmission entity.

The use of bigger transmission and receiving window sizes leads to a higher throughput, but also a high buffer consumption. The shorter status report and polling periods may not result in a higher throughput, but can improve the packet delay performance. Those key parameters are set differently for different traffic services in order to meet the specific buffer control and traffic performance requirement. Those parameters may be adjusted during transmission according to the need to enhance the delay performance or the traffic channel congestion situation.

### RLCP transmission processes

Transmission in this part specially means the transmission between the two peer service entities in the RLCP. On the reception of the LLC PDUs, the transmission RLCP entity shall check whether the corresponding service entity has been allocated for transmission in terms of the type of traffic service and destination. If no proper service entity has been found, the RLCP shall allocate an UM or AM entity for specific the traffic type and destination. Then it requests the peer RLCP entity to allocate an according service entity. As long as the receiving service entity is allocated, the virtual link is setup. If some TCHs have already been reserved for transmission, then Data packets can be transmitted to the receiving service entity. Otherwise the RLCP will attempt to request to reserve some TCHs for transmission.

Figure 35 describes the RLCP Transmission processes. The MACP management entities (MPME) primitives are used to exchange information between the RLCP and MACP inside a mesh AP. The following part uses MPME_xx to represent a specific primitive.

#### Virtual Link setup

After allocating a service entity, the transmission RLCP entity send the primitive MPME_LinkSetup.request (Source_ID, Destination_ID, V-Link_ID, LinkSetupReq RPDU) to the MACP entity. The V-link_ID should be unique between a transmission pair. On reception of the primitive, the MACP entity generates a LinkSetupReq MPDU carrying the received LinkSetupReq RPDU, and tries to transmit the MPDU by contending in the ACH. In case of the correct reception of the MPDU, the receiving MACP entity will notify its RLCP entity with the primitive MPME_LinkSetup.Indication (Source_ID, Destination_ID, V-Link_ID, LinkSetupReq RPDU). The CAC module in the RLCP entity makes a decision whether to accept the link setup request by evaluating:

- whether the common free TCHs of both sides can satisfy the QoS requirement

- whether the establishment of link will corrupt the QoS of existing links.

In case of accepting a link setup request, the RLCP entity shall allocate a service in its buffer with parameters suggested by the sender. Then it informs the MACP entity with the primitive MPME_LinkSetup.respond (Source_ID, Destination_ID, V-Link_ID, granted TCH seq.). The MACP will transmit SVBs on the ECHs corresponding to the TCHs informed in the primitive.

After receiving SVBs, the transmitting MACP entity will generate the primitive MPME_LinkSetup.confirm (Source_ID, Destination_ID, V-Link_ID, granted TCH Seq.) and pass it to the RLCP entity. The information of granted TCHs is obtained at the transmitter side by observing the SVBs. On reception of the primitive, the RLCP knows that it is able to transmit the Data RPDUs.

Figure 35: RLCP Transmission processes

#### Data delivery

After a virtual link for specific traffic and destination has been set up, the transmission RLCP can start to transmit the RPDU Data frames. It sends the primitive MPME_Data.request (Source_ID, Destination_ID, V-Link_ID, Data RPDU) to its MACP entity. The MACP entity generates the Type 1 Date MPDU by adding an overhead of 2 octets to the Data RPDU, and transmits on the reserved TCHs. On reception of the Data MPDU, the receiving MACP will pass the newly generated primitive MPME_Data.indication (Source_ID, Destination_ID, V-Link_ID, Data RPDU) from the received Data MPDU to the RLCP entity. The receiving MAC shall also transmit SVB or DVB (on-demand-TDD, see 7.2.4) on the specific ECHs.

#### TCH reservation request

As stated in 7.2.10, the reserved TCHs might be released even though the service entities in the RLCP still exist and new Data RPDU may arrive. In case that the RLCP finds no reserved TCHs for transmitting the newly arrived Data RPDUs, it shall initiate the TCH request process. On reception of a TCH reservation request, a STA shall not use the principles to evaluate whether to accept or deny the request as described in 7.3.3.1. Instead, it shall grant the request as long as it finds free TCHs agreed with the requester. See 7.3.6.1 for more details.

### Allocation of service entities

Each service entity is assigned a Virtual link identifier (V-link ID), which is unique between a transmission pair. A service entity should be allocated according to traffic type and RA destination. Data RPDUs with the same traffic type and RA destination can share a service entity. A multi-hop traffic flow with the strict packet delay requirement may also be allocated an individual service entity at the RLCP of an intermediate mesh AP. And the parameters set for the service entity are suitable for a quick delivery.

For single-hop Voice over IP (VoIP) traffic flows, the service entity can be a UM entity if the channel situation is good enough. But multi-hop VoIP flows should be allocated with AM entities in order to obtain the required end-to-end packet drop rate. For traffic flows like video and TCP traffic which have higher drop rate requirements, the service mode should be AM.

### Adaptation of ARQ parameters during transmission

As stated in 7.3.2, the ARQ parameters like transmission/receiving window size and the status report period/polling period may need to be changed during transmission. The side in a transmission pair, which wants to initiate the adjusting ARQ parameters process, tries to transmit an ARQParReq RPDU to its partner. On reception of the ARQParRep RPDU, the RLCP entity will change the parameters following the parameters specified in the ARQParRep RPDU, and then reply with an ARQParRes RPDU to the initiator. After receiving the ARQParRes RPDU, the initiator changes its own parameters.

### Radio Resource Control

#### Call Admission Control (CAC)

On reception of a link setup request (LinkSetupReq) or a TCH request (TCHReq) RPDU, the receiving RCLP entity shall use the CAC mechanism to decide whether to accept or deny the request. The information used to make a decision includes:

- From the request side:

- traffic channel utilization;

- proposal of PHY mode for use;

- proposal of traffic channels for use;

- traffic description: peak rate, mean rate, delay and jitter requirement and drop rate requirement;

- the number of hops till the final destination.

- From the requested side:

- traffic channel utilization;

- estimated traffic load from maintaining links;

On reception of a link setup request intended for multi-hop forwarding, the receiving RLCP entity shall take the remaining number of hops till the final destination into consideration. A multi-hop forwarding consumes a multiple traffic channel resource compared to a single hop transmission.

The CAC handles the link setup request and TCH request differently. For a link setup request, the CAC make a decision by taking the existing links into account. For a TCH request, the CAC just evaluates whether there are free available traffic channels. The following shows an example of the CAC mechanism.

Following definitions will be used later:

TCHS   – traffic channel utilization observed by the sender (the field of traffic channel utilization is 4 octets in length, see 6.3.2.1 for details)

TCHR	– traffic channel utilization observed by the receiver

TCHA	– common free available traffic channels of the sender and receiver

Num (TCHA) – the number of common free available traffic channels of the sender and receiver

NUM (TCH) – the number of traffic channels in a TDMA frame

RLP	– the peak traffic load of request traffic

RLM	– the mean traffic load of request traffic

B (RL)  – the burstiness of request traffic (ratio of RLP to RLM)

Num (RLP) –  the number of traffic channels needed to handle the peak rate case of the requested traffic

Num (RLM) – the number of traffic channels needed to handle the mean rate case of the requested traffic

Num (RQTCH) – the request TCH number contained in a LinkSetupReq or TCHReq RPDU

Num (G) – the granted number of traffic channels for the link

Num (Sum (LM)) – the number of traffic channels needed to handle the sum of traffic loads when each of the existing real-time link operates at its mean rate

Num (Sum (LH)) – the number of traffic channels needed to handle the sum of traffic loads when each of the existing real-time link operates at its peak rate

min (a, b) – return the minimal value of a and b

The following relation can be written directly from the definition:

TCHA = TCHS  (and) TCHR

For background and best effort traffic:

(Applicable for both the link setup request and TCH request)

If       Num (TCHA) > 0

Then 	accept the request

and Num (G)= min (Num (TCHA), Num (RQTCH))

Else   reject the request

For real-time traffic:

(Applicable for the link setup request)

If        Num (RLM)+ Num (Sum (LM)) > NUM (TCH)

Then  	reject the request

Else if Num (RLP) + Num (Sum (LM)) > NUM (TCH)

and  B (RL) > 2

Then 	reject the request

Else if  Num (TCHA) < Num (RQTCH)

Then      reject the request

Else if   accept the request

and  Num (G)= Num (RQTCH)

For real-time traffic:

(Applicable for the TCH request)

If       Num (TCHA) > 0

Then 	accept the request

and Num (G)= min (Num (TCHA), Num (RQTCH))

#### Adjusting of traffic channels for use

        - Forced release of TCHs

In case that the current free TCHs cannot satisfy the QoS requirements of real-time traffic flows for a destination, the RLCP will check whether there are reserved TCHs for background and best effort traffic flows destined to other STAs. If so, it notifies the MACP to release some of TCHs. The transmitting mesh AP, whose reserved TCHs for back-group traffic have been released, can request TCHs later when free traffic channels are available.

        - Request more TCHs for use

A  TCHs may be released due to the reasons listed in 7.2.10. When arrival of new MPDUs, in case that there is no TCH available for the link or the reserved TCHs cannot satisfy the specific Qos requirement of the link, the transmitting mesh AP would request more TCHs following the procedure described in 7.3.3.

- Compatible issues

  - Physical layer (PHY)

The proposed MAC amendment protocol can run on the IEEE 802.11a/b/g PHYs. The implementation of MDCF needs driver modification. The time-related parameters like the TDMA frame length, waveform of energy signals, the number of traffic channels (TCHs) are PHY scheme dependent.

  - Security

The IEEE 802.11i security mechanisms or extensions of the mechanisms can be used in the proposed MAC protocol for the purpose of implementing security in a WDS.

  - Mesh topology discovery

A MACF network is formed in a fully distributed manner. The mesh topology knowledge can be built up in the routing discovery stage of running a routing protocol at each mesh AP. The mesh topology knowledge includes status and quality of mesh links between Mesh APs. A routing protocol should also take care of the topology knowledge refreshment during operation.

  - Routing

The proposed MAC protocol supports the accepted routing protocols in the scope of Mobile Ad Hoc Networks (manet) working group in the Internet Engineering Task Force (IETF), and extensible mesh routing schemes. Future routing schemes may be proposed by closely interacting with MAC protocol, especially the MDCF for achieving a higher performance.

  - Mesh network size

The MDCF provides an efficient prioritized access mechanism (see 7.2.2) and a good solution for calming down hidden stations in the multi-hop environment (see 7.2.5). The MTSF supports synchronization for multi-hop operation. The mesh network size then is dependent on the routing scheme. As indicated in Figure 25, the number of mesh APs in a MDCF network may be over than 10×10.

- Acknowledgements

This work is supported by the IST project IST-2003-507581 WINNER, which is partly funded by the European Union.