IEEE P802.11
Wireless LANs

| Simulation Results for Proactive Mesh Networks |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bing Zhang | National Institute of Information and Communications Technology | 3-5 Hikaridai, Seika-cho, Soraku-gun, Kyoto, Japan | +81-774-98-6820 | zhang@nict.go.jp |
| Oyunchimeg Shagdar | ATR Adaptive Communication Research Laboratories | 2-2-2 Hikaridai, Seika-cho, Soraku-gun, Kyoto, Japan | +81-774-95-1532 | oyunaa@atr.jp |
| Suhua Tang | ATR Adaptive Communication Research Laboratories | 2-2-2 Hikaridai, Seika-cho, Soraku-gun, Kyoto, Japan | +81-774-95-1544 | shtang@atr.jp |
| Youiti Kado | Oki Electric Industry Co., Ltd. | 2-5-7 Honmachi, Chuo-ku, Osaka, Japan | +81-6-6260-0700 | kado567@oki.com |
| Masanori Nozaki | Oki Electric Industry Co., Ltd. | 2-5-7 Honmachi, Chuo-ku, Osaka, Japan | +81-6-6260-0700 | nozaki765@oki.com |

Abstract

This document presents primary simulation results to confirm the effectiveness of our proposal including QoS priority control scheme, frame aggregation scheme and distributed channel assignment scheme. The document first specifies the simulation parameters and scenario, and then simulation results are investigated for the three proposed schemes against the original schemes.  Through the simulations over WLAN mesh network with MAPs associating with STAs, we confirm the utility of the implemented three schemes. Our simulation results are very promising, and we are currently in the process of implementing the load balancing scheme over WLAN mesh network.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Table of Contents

1	Performance Evaluation of QoS Priority Control Schemes	3

1.1  Simulation Setup	3

1.2	Average Delay of Individual Flows	4

1.3	Distribution of End-to-End Delay for Each Schemes	6

1.4	Analysis of Simulation Results	8

2	Performance Evaluation of Frame Aggregation	8

2.1.	Simulation Model	8

2.2.	Simulation Results	10

2.3.	Analysis of Simulation Results	11

3.	Simulation Results of Channel Assignment	11

3.1.	Simulation Topology and Condition	11

3.2.	Simulation Results	12

4	Summary	16

List of Figures

Figure 1 - Simulation model	3

Figure 2 - Average delay of individual flows (K=4).	5

Figure 3 - Average delay of individual flows (K=ALL)	5

Figure 4 - Normal distibution of end-to-end delay for each scheme (K=4)	7

Figure 5 - Normal distribution of end-to-end delay for each scheme (K=ALL)	7

Figure 6 – Network topology.	9

Figure 7 – Wireless LAN radio receiver throughput.	10

Figure 8 – Comparison of before/after frame aggregation	10

Figure 9 – Dropped frame and Retransmission attempts	11

Figure 10 - GCA with two interfaces and four channels	13

Figure 11 - GCA with three interfaces and four channels	14

Figure 12 - Number of sessions limited by average delivery ratio.	15

Figure 13 - Number of sessions limited by average delay.	15

Figure 14 - Average delay jitter under different sessions.	16

List of Tables

Table 1 - Simulation parameters.	4

Table 2 - Performance comparison (K=4)	6

Table 3 - Performance comparison (K=ALL)	6

Table 4 – Simulation parameters	9

Table 5 - Simulation condition	12

Table 6 - Simulation condition	14

# Performance Evaluation of QoS Priority Control Schemes

Among the three proposed schemes related with QoS priority control, we investigated the formance of congestion condition based priority control scheme (see [1]) by using an ns-2 network simulator [2].

## 1.1  Simulation Setup

In the simulations we target a mesh network model that consists of twenty-four MPs as shown in Figure 1.

0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

Figure 1 - Simulation model

Since vice is one of the real time applications that characterized with the strict QoS requirements, we set up VoIP sessions in the network and investigated their performance for the proposed schemes against the original scheme where no priority control is performed. Table 1 shows the parameters used for the simulations. Each VoIP session is set up for 60 seconds. The codec type is G. 711, where packet size is 160 [bytes] and RTP packet transmission interval is 20 [ms]. In the proposed scheme, each relay MP compares the congestion conditions of the paths corresponding to the first K packets in its buffer. If the number of packets inserted in the buffer is less than K, the MP compares all the packets in the buffer. In the simulations, we considered two cases where a) the number of packets to be compared is limited to a fixed value 4 (K=4) and b) all the packets in its buffer are compared (we denote this case as K=ALL). The congestion condition of a path is characterized by the sum of the queue lengths of each MP on the path. In everyτtime interval, each MP measures its queue length and broadcasts the information throughout the network by using mesh measurement frames. This mechanism enables each MP to know the queue length of any other MP in the network. In the simulations, τ is set to 5 seconds. At a relay MP, a packet’s priority is calculated in two ways a) the sum of the queue lengths of each transmitter MP on the end-to-end path or b) the sum of the queue lengths of each transmitter MP on the path from the relay MP to the destination MP. We call the former way as Sender-to-Destination and the latter as Transmitter-to-Destination. It is obvious that larger the sum of queue lengths is severe the congestion of the path. Therefore, the packet that is correspondent to the the most congested path is sent with the highest prioirity. Subsection 1.2 and subsection 1.3 show simulation results and 1.4 summarizes this section.

Table 1 - Simulation parameters.

| Simulation time | 60 [sec] |
| --- | --- |
| Distance between neighboring MPs | 30 [m] |
| Transmission power | 0.03 [W] |
| MAC layer transmission rate | 54 [Mbps] |
| Modulation scheme | 64-QAM |
| Transmission range | 44.7 [m] |
| Sensing range | 77.4 [m] |
| Buffer size | 50 [pkts] |
| Routing protocol | OLSR |
| Codec | G.711 |
| Packet size | 160 [bytes] |
| RTP packet transmission interval | 20 [ms] |

## Average Delay of Individual Flows

We evaluated performance of the proposed schemes when thirty one VoIP sessions (since VoIP is bidirectional, there are 62 flows exist) are set up in the network. Simulation topology consists of twenty two 1-hop flows, twenty 2-hop flows, and twenty 5-hop flows. It is easy to guess that compared to 1-hop and 2-hop flows, 5-hop flows tend to get longer end-to-end delay, resulting to the scenario where 5-hop flows are not being able to satisfy their delay requirements whereas 1-hop and 2-hop flows reach at their destination with time to spare. End-to-end delay requirements are characterized by threshold values that define the maximum value of the allowed end-to-end delay. We considered two classes (Class A and Class B) for delay requirements. The delay threshold of Class A is 0.1 seconds, and 0.15 seconds for Class B. The Figure 2 and Figure 3 show average end-to-end delay of individual flows when K=4 and K=ALL, respectively. The horizontal axis is labeled with flow number of each flow. As Figure 2 and Figure 3 show, flows 0 - 21 are 1-hop, flows 22 - 41 are 2-hop, and flows 42 - 61 are 5-hop. The figures show that the proposed schemes decrease the delay of long-hop flows (5-hop flows), at the cost of increasing the delays of the short-hop flows (1-hop and 2-hop flows). Both of Transmitter-to-Destination and Sender-to-Destination schemes increase the number of flows that satisfy the delay requirements compared to the original scheme (where no prioiry control is performed). This can be seen from Table 2 and Table 3. Transmitter-to-Destination scheme increased the number of flows that met the delay requirement of Class A by 4 and 10, when K=4 and K=ALL, respectively. On the other hand, Sender-to-Receiver scheme increased the number of flows that met the delay requirement of Class A by 8 and 11, when K=4 and K=ALL, respectively.

Figure 2 - Average delay of individual flows (K=4).

Figure 3 - Average delay of individual flows (K=ALL)

Table 2 - Performance comparison (K=4)

| Scheme | Number of flows met delay requarement |  |
| --- | --- | --- |
|  | Class A | Class B |
| Original (No priority control) | 46 | 62 |
| Proposed (Transmitter-to-Destination) | 50 | 62 |
| Proposed (Sender-to-Destination) | 54 | 62 |

Table 3 - Performance comparison (K=ALL)

| Scheme | Number of flows met delay requarement |  |
| --- | --- | --- |
|  | Class A | Class B |
| Original (No priority control) | 46 | 62 |
| Proposed (Transmitter-to-Destination) | 56 | 62 |
| Proposed (Sender-to-Destination) | 57 | 61 |

## Distribution of End-to-End Delay for Each Schemes

The main purpose of the proposed schemes is to increase number of real time traffics that meet their delay requiments. To do so, the proposed schemes decrease end-to-end delay of the flows that traverse through congested paths, at the cost of the increasing the delay of the flows that traverse through non-congested paths. In order to see the effectiveness of the proposed schemes, we compare distributions of end-to-end delay for each scheme. Figure 4 and Figure 5 show normal distribution of end-to-end delay for each scheme, when K=4 and K=ALL respectively. From the figures, we can see that the curves of the normal distribution for the proposed schemes are sharper than the ones of the original scheme, proving the efficiency of the proposed schemes.

Figure 4 - Normal distibution of end-to-end delay for each scheme (K=4)

Figure 5 - Normal distribution of end-to-end delay for each scheme (K=ALL)

## Analysis of Simulation Results

Among the three proposed schemes, the congestion condition based priority control scheme is chosen and evaluated. The congestion condition based priority control is performed in two ways as follows: 1) considering the congestion condition of the end-to-end path and 2) considering congestion condition of the path from the relay MP to the destination. In the simulations, the performances of each scheme are evaluated against the original scheme where no priority control is performed. In the simulations sixty two VoIP flows (thirty one VoIP sessions) are set up on the 24-MP mesh network topology. We compared end-to-end delay of individual flows for each scheme. The results showed that the proposed schemes can increase the number of flows that satisfy their delay requirement by up to 11 in this 31-VoIP session scenario. This is because the proposed schemes decrease the end-to-end delay of the flows that traverse through congested path, at the cost of decreasing the end-to-end delay of the flows that traverse through non-congested path. Finally, in order to see the effectiveness of the proposed schemes, we compared normal distribution of end-to-end delay for each scheme.

# Performance Evaluation of Frame Aggregation

## Simulation Model

To confirm the effectiveness of the frame aggregation on the WLAN mesh network described as our proposal [1], a simulation evaluation was performed by using OPNET of version 11.0. Figure 6 shows the topology of WLAN mesh network

Similarly to Figure 1, the network is composed of 24 MAPs, and every MAP is able to have 3 neighbor MAPs or less. MAP01 is a gateway MAP and the legacy STAs are associated with MAP21, MAP24, and MAP04, respectively. STAs generate traffics of downstream and upstream like VoIP toward MAP01. Simulation scenario is as follows:

- 5 VoIP flows from MAP04, MAP21 and MAP24, respectively, toward MAP01 (15 fllows in total) are transmitted (simulation time is set from 100s to 300s.).

- 3 STAs are newly associated with MAP06, and 3 flows toward MAP01are also added (simulation time is set from 250s to 300s)

- When congestion is detected on MAP01, the frame aggregation based on Pull model is performed between MAP01 and its neighbour MAP.

- When MAP01, MAP06, MAP21 and MAP24 receive the aggregation frame as a backward notification  frame of congestion, the frame aggregation based on Push model is executed in each MAP (simulation time is set from 300s to 400s).

01

02

03

04

05

06

07

08

09

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

Figure 6 – Network topology.

Moreover, the simulation parameter is shown in Table 4.

Table 4 – Simulation parameters

| Parameter |  | Description |
| --- | --- | --- |
| General Parameters | WLAN Type | IEEE802.11a |
|  | Data Speed | 54Mbps |
|  | Node Distance | 50 m |
|  | Simulation Duration | 400 s |
| Traffic Model | CBR (G.711 Codec) | 64Kbps |
|  | Frame Body Size | 160 Byte + 58B (Over head) |
|  | Frame Interval | 20 ms |
| PHY Parameter | Transmit Power | 80 mW |
|  | Reception Power Threshold | -62 dBm |
| Routing Parameter | Routing Protocol | OLSR |
|  | Hello Interval | 1 s |
|  | TC Interval | 2 s |
|  | Neighbour Hold Time | 6 s |
|  | Topology Hold Time | 15 s |

## Simulation Results

Simulation results are shown in Figure 7, where Y axis shows the throughputs (bit/sec) obtained by each MAP, whereas X shows the simulation time (minute). The traffic from MAP01 to MAP12 is plotted in this figure.

Figure 7 – Wireless LAN radio receiver throughput.

Figure 8 – Comparison of before/after frame aggregation

Next, the throughputs of all MAPs in the WALN mesh network obtained before and after the frame aggregation are depicted in Figure 8. Finally, the numbers of frame loss from MAP01 to MAP06 and frame retransmission in MAC layer are shown in Figure 9.

Figure 9 – Dropped frame and Retransmission attempts

## Analysis of Simulation Results

It is clear from Figure 7 that the throughput of every MAP is improved by performing the frame aggregation, also including the MAP such as MAP12 that doesn't relay the frame directly. Figure 8 compares the throughputs obtained with the frame aggregation and without frame aggregation. It is clear from the figure that the average throughput of every MAP is improved to about 43%. Moreover, since the frame aggregation also results in the reduction of frame loss rate and frame retransmission number, the throughput of every MAP can be improved based on Figure 9. From the above simulation resuls, it can be said that the frame aggregation by the BEANS method is effective to improve the throughput for VoIP application with the small size of frame over the WLAN mesh network.

# Simulation Results of Channel Assignment

## Simulation Topology and Condition

The topology used in CA simulation is the same with the topology shown in Figure 1. The mesh network consists of several loops in the hexagon structure with APs on the vertex of the hexagons. Each Mesh AP is 40m away from its neighbors. In the simulation, VoIP traffic is used to verify the system capacity improvement. Mesh AP pairs are randomly selected to run VoIP sessions. Each VoIP session contains two uni-directional flows. The detailed simulation conditions are listed in Table 5.

Table 5 - Simulation condition

|  | Parameters | Value |
| --- | --- | --- |
| MAC/PHY layer | Transmitting power | 0.03W |
|  | Carrier frequence | 5.2GHz |
|  | Carrier sense threshold | 10dB |
|  | Communication range | 44.7m |
|  | Interference range | 77.4m |
|  | Propagation model | Two-ray |
|  | Bandwidth | 20MHz |
|  | Basic rate | 6 Mbps |
|  | Data rate | 54 Mbps |
|  | Contention window size | 15~1023 |
|  | Slot time | 9 us |
|  | RTS_threshold | 2304Byte (no RTS/CTS) |
| OLSR routing protocol parameters | Hello interval | 2 s |
|  | TC interval | 5 s |

## Simulation Results

Figure 10 and Figure 11 show the association between links and channels after the GCA procedure is finished. In Figure 10, each Mesh AP has two interfaces and four channels are usable. Because one interface is devoted to monitoring the common channel, each Mesh AP has only one free data interface. In the figure, the same channel tends to be used by adjacent links due to the limitation of free interfaces. In the whole network, channel 0, 1, 2 and 3 are shared by 10, 1, 8 and 10 links respectively. In Figure 11, each Mesh AP has three interfaces. Except the one for common channel, the other two are freely adjustable, providing more flexibility in the CA procedure. The channels are assigned in a distributed way with a more meaningful result compared with the two-interface case. In the whole network, channel 0, 1, 2 and 3 are shared by 6, 6, 8 and 9 links respectively. All the channels are nearly evenly used.

Figure 12 and Figure 13 show the system capacity in terms of the maximum supportable sessions under the restriction of packet delivery ratio and delay respectively. It is obvious that by CA the system capacity is greatly improved. At the same time the delay jitter is effectively reduced, as is shown in Figure 14. The system capacities are also listed in Table 16. GCA with 2 interfaces almost has twice capacity compared that acquired without CA and GCA with 3 interfaces nearly has three times improvement. It is expected that higher improvement can be achieved as the number of available channels increases.

0

1

4

5

2

3

6

7

16

12

17

13

8

9

18

14

19

15

10

11

20

21

22

23

Common channel 0

Data channel 1

Data channel 2

Data channel 3

(Usage: 10 links)

(Usage: 1 links)

(Usage: 8 links)

(Usage: 10 links)

20

21

22

23

16

17

18

19

12

13

14

15

4

5

6

7

8

9

10

11

0

1

2

Figure 10 - GCA with two interfaces and four channels

Common channel 0

Data channel 1

Data channel 2

Data channel 3

(Usage: 6 links)

(Usage: 6 links)

(Usage: 8 links)

(Usage: 9 ks)

0

1

4

5

2

3

6

7

16

12

17

13

8

9

18

14

19

15

10

11

20

21

22

23

20

21

22

23

16

17

18

19

12

13

14

15

4

5

6

7

8

9

10

11

0

1

2

Figure 11 - GCA with three interfaces and four channels

Table 6 - Simulation condition

| CA scheme | Class A (sessions) | Class B (sessions) |
| --- | --- | --- |
| Single channel (No CA) | 30~34 | 35~40 |
| GCA (2interfaces, 4 channels) | 60 | 75 |
| GCA (3interfaces, 4 channels) | 99 | 106 |

Figure 12 - Number of sessions limited by average delivery ratio.

Figure 13 - Number of sessions limited by average delay.

Figure 14 - Average delay jitter under different sessions.

# Summary

In this document, we investigated the effectiveness of our proposal over the WLAN mesh network. Based on simulations using an ns-2 network simulator, we implemented QoS priority control scheme and distributed channel assignment scheme. Based on simulations using an OPNET network simulator, we implemented the frame aggregation scheme. Through the simulations with VoIP application, we found that both frame aggregation scheme and channel assignment scheme significantly improved the throughputs over the WLAN mesh network, and QoS priority control scheme decreased the end-to-end delay of VoIP flows. Through simulations over the WLAN mesh network, we confirmed the utility of the implemented schemes. Our simulation results are very promising, and we are currently in the process of performing simulation experiments on load balancing, and doing the further improvement and refinement.

References

- IEEE 802 11-05/588r0, 802.11 TGs Proactive Mesh Network (ProM) Proposal.

- USB/LBNL/VINT Networks Simulator ns (version2), http://www.isi.edu/nsnam/ns..