Instructions:  Fill out the table below, answering “yes” or “no” for each question.  If a shaded box is checked, an explanation is required.  Answer in the space below the question; if more room is needed, attach a separate sheet.

ED/SCD Drawing No. or ASIC Part No.____________________________________________________

ASIC Name___________________________		Project Name_____________________________

Lead Designer _________________________		Date____________________________________

| ASIC SPECIFICATION |  | Yes | No |
| --- | --- | --- | --- |
| 1. Is the ASIC Hardware Requirements Document (AHWRD) through signoff? |  |  |  |
| 2. Are the interfaces for this design specified in the AHWRD up-to-date? |  |  |  |
| 3. Do simulations fully verify the performance requirements documented |  |  |  |
| in the AHWRD? If no, briefly describe requirements that are not verified and reasons for these omissions. |  |  |  |
| 4. Is the Envelope Drawing or Source Control Drawing (ED/SCD) |  |  |  |
| through signoff? |  |  |  |
| 5. Is the ED/SCD up to date? |  |  |  |
| 6. Are supplier test requirements clearly defined in the ED/SCD? |  |  |  |
| 7. Are all pins, pads, and voltage/logic polarities clearly defined in the |  |  |  |
| ED/SCD? |  |  |  |

| ASIC SPECIFICATION (continued) |  |  |  | Yes |  | No |
| --- | --- | --- | --- | --- | --- | --- |
| 8. Are any non-standard parametric tests required? If yes, have these tests |  |  |  |  |  |  |
| been examined and approved by the supplier? |  |  |  |  |  |  |
| 9. Are all PDR action items completed? |  |  |  |  |  |  |
| 10. Are the schematics legible when copied? |  |  |  |  |  |  |
| 11. Are simulation netlists and LVS netlists generated from the same |  |  |  |  |  |  |
| schematics or source code? |  |  |  |  |  |  |
| 12. Are the absolute maximum ratings for this ASIC consistent with |  |  |  |  |  |  |
| foundry process specifications? |  |  |  |  |  |  |
| 13. Is the chip size compatible with the package and the anticipated |  |  |  |  |  |  |
| yield of the target process? |  |  |  |  |  |  |
| 14. Are all I/O signal pads ESD protected? If not, explain reasons for |  |  |  |  |  |  |
| omitting ESD protection devices. |  |  |  |  |  |  |
| 15. Have a mask copyright symbol, pin 1 marker, and die identification label |  |  |  |  |  |  |
| (part number, etc.) been included in the layout? |  |  |  |  |  |  |
| 16. Has a package been selected and approved by the supplier? |  |  |  |  |  |  |
| 17. Has a bonding diagram been prepared? |  |  |  |  |  |  |

| ASIC SPECIFICATION (continued) |  |  |  | Yes |  | No |
| --- | --- | --- | --- | --- | --- | --- |
| 18. Have any foundry-specific design checklist(s) been completed? (attach |  |  |  |  |  |  |
| copy) |  |  |  |  |  |  |
| 19. Has an independent audit of the design been conducted? |  |  |  |  |  |  |

| DIGITAL ASIC DESIGN |  |  |  | Yes | No |
| --- | --- | --- | --- | --- | --- |
| 20. Does this ASIC contain circuitry that was designed and analyzed using |  |  |  |  |  |
| discrete-state simulation and analysis tools (i.e. - “digital” simulators)? |  |  |  |  |  |
| (If answer is no, skip questions 21 - 66) |  |  |  |  |  |
| 21. Are there an adequate number of power/ground pins (40ma of output drive |  |  |  |  |  |
| drive per power/ground pair)? |  |  |  |  |  |
| 22. Are internal tri-state busses used? If so, explain: How is contention |  |  |  |  |  |
| avoided? |  |  |  |  |  |
| 23. Are all counters in the ASIC synchronous? |  |  |  |  |  |
| 24. Are all internal registers initialized at reset? |  |  |  |  |  |
| 25. Are cross-coupled gates without external set or reset used? |  |  |  |  |  |
| 26. On set/reset latches, has the chip reset been gated into the reset input? |  |  |  |  |  |
| (If no, explain how such latches can be set to a known state) |  |  |  |  |  |

| DIGITAL ASIC DESIGN (continued) |  |  |  | Yes |  | No |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 27. Have all asynchronous inputs been synchronized prior to use in the chip? |  |  |  |  |  |  |  |
| 28. Has logic been minimized to avoid fabrication of unnecessary or |  |  |  |  |  |  |  |
| redundant circuitry? |  |  |  |  |  |  |  |
| 29. Have design techniques been used to improve controllability and |  |  |  |  |  |  |  |
| observability? |  |  |  |  |  |  |  |
| 30. Does built-in-test capability support the requirements of the AHWRD? |  |  |  |  |  |  |  |
| 31. Has cell loading been analyzed to avoid excessive delays, as well as |  |  |  |  |  |  |  |
| excessive rise and fall times? |  |  |  |  |  |  |  |
| 32. Have critical high speed nets been analyzed for parasitic layout resistance |  |  |  |  |  |  |  |
| and capacitance? Have these parasitics been back-annotated and simulated to verify critical performance characteristics? |  |  |  |  |  |  |  |
| 33. Is there a clock signal on chip or a clock input? |  |  |  |  |  |  |  |
| (If answer is “no”, skip questions 34 - 42) |  |  |  |  |  |  |  |
| 34. What is the maximum clock frequency? |  |  |  |  | Hz |  |  |
| 35. Is all logic clocked synchronously (from the same clock)? |  |  |  |  |  |  |  |
| 36. Are different levels or edges of the clock used in the same design? |  |  |  |  |  |  |  |
| If so, explain: Is the reduced setup time taken into account? |  |  |  |  |  |  |  |

| DIGITAL ASIC DESIGN (continued) |  |  |  | Yes |  | No |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 37. Has the clock’s duty cycle specification been used in simulations? |  |  |  |  |  |  |  |
| 38. Are gated clocks used? If so, explain why. Have these clocks been |  |  |  |  |  |  |  |
| analyzed for glitches at nominal, best and worst case conditions? |  |  |  |  |  |  |  |
| 39. Have setup and hold times been verified to be adequate for every |  |  |  |  |  |  |  |
| clocked register or cell? |  |  |  |  |  |  |  |
| 40. Are there any self-generated clocks or pulses? |  |  |  |  |  |  |  |
| 41. Has skew and loading been analyzed for clock and reset lines? |  |  |  |  |  |  |  |
| If so, describe the techniques used. If not, explain why. |  |  |  |  |  |  |  |
| 42. Have best, nominal, and worst case timing been verified on the chip |  |  |  |  |  |  |  |
| (including effects of temperature, voltage and process)? |  |  |  |  |  |  |  |
| 43. Have the simulations been run at best, nominal, and worst cases |  |  |  |  |  |  |  |
| (including effects of temperature, voltage and process) and do the outputs match? |  |  |  |  |  |  |  |
| 44. Have system simulation vectors been compared against chip simulations? |  |  |  |  |  |  |  |
| 45. Has the effect of board-level (chip-to-chip) timing been considered |  |  |  |  |  |  |  |
| in chip simulations? |  |  |  |  |  |  |  |
| 46. Do the chip’s test vectors meet the AHWRD’s requirement for |  |  |  |  |  |  |  |
| fault coverage? |  |  |  |  |  |  |  |

| DIGITAL ASIC DESIGN (continued) |  | Yes | No |
| --- | --- | --- | --- |
| 47. Do the test vectors produce 100% toggle coverage? |  |  |  |
| 48. Have any internal nodes been stimulated (forced) in the test vectors? |  |  |  |
| 49. Do the test vectors initialize the chip to a known state? |  |  |  |
| 50. Have all the states in state machines been accounted for and simulated? |  |  |  |
| 51. Have all paths been analyzed for potential race conditions? |  |  |  |
| 52. Were external loads meeting system and test requirements used in |  |  |  |
| simulations and timing analyses? |  |  |  |
| 53. Have the format, timing, size, and speed of the test vectors been |  |  |  |
| approved by the supplier? |  |  |  |
| 54. Are any inputs in the test vectors asynchronous? |  |  |  |
| 55. Are all outputs in the test vectors stable at the strobe point for best and |  |  |  |
| worst case simulations? |  |  |  |
| 56. Have test vectors been developed for AC parametric testing? |  |  |  |
| 57. Has the design / fabrication process been proven with a test chip |  |  |  |
| designed with the same version of the design tool(s)? |  |  |  |

| DIGITAL ASIC DESIGN (continued) |  | Yes | No |
| --- | --- | --- | --- |
| 58. Are any non-standard or custom pads or cells used? (If answer is no, skip |  |  |  |
| question 59). |  |  |  |
| 59. Have any non-standard or custom pads and cells been analyzed by |  |  |  |
| SPICE (or equivalent), using worst and best case environment, process, and I/O loading parameters? |  |  |  |
| 60. Have non-standard simulation models been used? If so, explain why and |  |  |  |
| describe the methodology used to develop these models. |  |  |  |
| 61. Have all outstanding cell or software problems with the tool been |  |  |  |
| accounted for in this design? |  |  |  |
| 62. Have all power and ground metal runs been analyzed for maximum current |  |  |  |
| density, and are these values consistent with worst case reliability guidelines for the target process? |  |  |  |
| 63. Are padring and core VDD (Vcc) and VSS (Gnd) separated? |  |  |  |
| 64. Are sufficient vias used in VDD (Vcc) and VSS (Gnd) routing to handle |  |  |  |
| the expected current densities? |  |  |  |
| 65. Are clock input pad(s) surrounded by VDD and VSS pads? |  |  |  |
| 66. Have estimates of static and dynamic power dissipation been |  |  |  |
| made? Are the estimates compatible with requirements of the AHWRD? |  |  |  |

| ANALOG ASIC DESIGN |  | Yes | No |
| --- | --- | --- | --- |
| 67. Does this ASIC contain circuitry that was designed and analyzed |  |  |  |
| using continuous-state simulation and analysis tools (i.e. - “analog” simulators such as SPICE, SABER, etc.)? |  |  |  |
| (If answer is no, skip questions 68 - 97) |  |  |  |
| 68. Are all device models used to simulate ASIC performance supplied by the |  |  |  |
| foundry? If no, describe the methodology used to develop these models. |  |  |  |
| 69. Are all resistors that require matched or ratioed values constructed of the |  |  |  |
| same type of diffusion, implant or thin film? |  |  |  |
| 70. Do any bipolar transistors approach saturation in anticipated circuit |  |  |  |
| operating conditions. If yes, have these instances been examined for slow recovery & potential latchup conditions? (Note: Most spice models are unreliable for simulating saturation recovery characteristics.) |  |  |  |
| 71. Are any devices operated at or near maximum power or current ratings? |  |  |  |
| If yes, have these devices received detailed reliability and thermal analysis. |  |  |  |
| 72. Has the parasitic junction capacitance of MOSCAP bottom plates been |  |  |  |
| considered in design and modeled in simulation? |  |  |  |
| 73. Has the design been reviewed for possible breakdown effects? |  |  |  |
| (Particular attention should be given to emitter-base junctions.) |  |  |  |
| 74. Have all power and ground metal runs been analyzed for maximum current |  |  |  |
| density, and are these values consistent with worst case reliability guidelines for the target process? |  |  |  |

| ANALOG ASIC DESIGN (continued) |  |  |  | Yes |  | No |
| --- | --- | --- | --- | --- | --- | --- |
| 75. Have critical high speed nets been analyzed for parasitic layout resistance |  |  |  |  |  |  |
| and capacitance? Have these parasitics been back-annotated and simulated to verify critical performance characteristics? |  |  |  |  |  |  |
| 76. Have low value diffused or implant resistors (10 squares or less) been |  |  |  |  |  |  |
| designed incorporating the effects of contact resistance? |  |  |  |  |  |  |
| 77. Has power supply sequencing been analyzed for possible latchup effects? |  |  |  |  |  |  |
| 78. Have all cells / functional blocks of the design been simulated across |  |  |  |  |  |  |
| variations of process and temperature (e.g. - low, nom, high) and are cell performance requirements met for all simulated conditions? If not, describe any exceptions and justify these simulation results. |  |  |  |  |  |  |
| 79. Have interfaces between cells / functional blocks been analyzed for |  |  |  |  |  |  |
| functionality over worst case process / temperature conditions? |  |  |  |  |  |  |
| 80. Have effects of source and termination impedances been modeled in |  |  |  |  |  |  |
| the simulation of all I / O signals? |  |  |  |  |  |  |
| 81. Are all significant parasitics accounted for (package, wire bonds, and pads) |  |  |  |  |  |  |
| and off-chip components in final simulation? |  |  |  |  |  |  |
| 82. Does the layout include a sufficient number and distribution of substrate |  |  |  |  |  |  |
| contacts? |  |  |  |  |  |  |

| ANALOG ASIC DESIGN (continued) |  |  |  | Yes |  | No |
| --- | --- | --- | --- | --- | --- | --- |
| 83. Are substrate contacts placed adjacent to any lateral PNP transistors, or any |  |  |  |  |  |  |
| NPN transistors that enter saturation or are used as base-collector junction diodes (e.g. - ESD protection devices)? |  |  |  |  |  |  |
| 84. Are substrate contacts placed adjacent to bulk-constrained MOS devices? |  |  |  |  |  |  |
| 85. If well - (+) diodes were used, has vertical current in the associated parasitic |  |  |  |  |  |  |
| vertical BJT been considered? Are substrate contacts placed adjacent to any such devices? |  |  |  |  |  |  |
| 86. Have all epi tubs containing implant resistors been provided with |  |  |  |  |  |  |
| contacts, and are all such contacts connected to appropriate voltage sources that reverse-bias the parasitic implant-epi junctions? (Note: These connections should be modeled in simulation, and ideally should be verified by LVS analysis.) |  |  |  |  |  |  |
| 87. Are matched MOS devices laid out so that current flows in the same |  |  |  |  |  |  |
| direction? |  |  |  |  |  |  |
| 88. If a bias voltage bus is used, have the effects of voltage variation across the |  |  |  |  |  |  |
| chip (due to IR drops, FET threshold variations, etc.) been considered? |  |  |  |  |  |  |
| 89. Have epi bias and IR voltage effects on lightly doped diffused or implant |  |  |  |  |  |  |
| resistors been considered in design and modeled in simulation? (Note: Epi bias voltage changes the resistivity of the diffusion/implant, and excessive IR voltage drop causes saturation (current limiting) in the affected resistor.) |  |  |  |  |  |  |

| ANALOG ASIC DESIGN (continued) |  |  |  | Yes |  | No |
| --- | --- | --- | --- | --- | --- | --- |
| 90. Has the stability of all amplifiers / gain stages been verified with both |  |  |  |  |  |  |
| frequency domain (AC) and time domain (transient) analysis techniques? Did these stability analyses include effects of amplifier source and load impedances? |  |  |  |  |  |  |
| 91. Have beta-helpers or other bias structures incorporating feedback been |  |  |  |  |  |  |
| checked for stability? |  |  |  |  |  |  |
| 92. Can bias for critical circuits be externally overridden? |  |  |  |  |  |  |
| 93. Can critical circuits be isolated for troubleshooting? (Acceptable methods |  |  |  |  |  |  |
| include laser cutting of metal traces and dedicated test cells on separate die) |  |  |  |  |  |  |

| RF/MMIC ASIC DESIGN (A subset of analog design) |  |  |  | Yes |  | No |
| --- | --- | --- | --- | --- | --- | --- |
| 94. Does this analog ASIC contain very high frequency (RF/MMIC) circuitry |  |  |  |  |  |  |
| that required specialized MMIC design and analysis techniques? |  |  |  |  |  |  |
| (If answer is no, skip questions 95 - 97) |  |  |  |  |  |  |
| 95. Has simulation accounted for variations in component parameters that may |  |  |  |  |  |  |
| not be represented in vendor-supplied spice models, such as interconnect sheet resistance, MIM capacitance, FET DC and RF characteristics, etc.? |  |  |  |  |  |  |
| 96. Can any wideband AC coupled inputs and outputs be accessed through |  |  |  |  |  |  |
| DC coupled test or probe pads? |  |  |  |  |  |  |

| RF/MMIC ASIC DESIGN (continued) |  |  |  | Yes |  | No |
| --- | --- | --- | --- | --- | --- | --- |
| 97. Has the layout been manually checked for following: |  |  |  |  |  |  |
| a. Metal interconnect widths at bond pads. |  |  |  |  |  |  |
| b. Maximum separation between adjacent metal traces on the same layer. |  |  |  |  |  |  |
| c. Widths of interconnects not constrained by RF parasitics are twice |  |  |  |  |  |  |
| as wide as needed to handle maximum current. |  |  |  |  |  |  |
| d. Any unnecessary metalization avoided. |  |  |  |  |  |  |
| e. Maximum die length to width ratio less than 3:1 (L>W). |  |  |  |  |  |  |
| f. Backside vias are away from edges and center of chip, and are not |  |  |  |  |  |  |
| located in line across chip. |  |  |  |  |  |  |
| g. Backside vias have been minimized and shared where possible. |  |  |  |  |  |  |
| h. Capacitors and other fragile components are placed away from |  |  |  |  |  |  |
| edges and at least 25um away from bond pads. |  |  |  |  |  |  |
| i. Effect of line width precision on inductors and resistors has been |  |  |  |  |  |  |
| evaluated. |  |  |  |  |  |  |
| j. Resistors are correct length and width (>3:1 ratio) and meet |  |  |  |  |  |  |
| minimal width requirements. |  |  |  |  |  |  |
| k. Current density per line width for resistors and metal traces is correct |  |  |  |  |  |  |
| for the target process. |  |  |  |  |  |  |
| l. Die labeling, alignment marks and identification requirements have been |  |  |  |  |  |  |
| followed? |  |  |  |  |  |  |
| m. RF inputs and outputs laid out properly for probing. |  |  |  |  |  |  |

| RF/MMIC ASIC DESIGN - Manual Layout Checks (continued) |  |  |  | Yes |  | No |
| --- | --- | --- | --- | --- | --- | --- |
| 97.(continued) Has the layout been manually checked for the following? |  |  |  |  |  |  |
| n. Enclosed geometries on metal layers avoided. |  |  |  |  |  |  |
| o. RF lines are separated by at least ground plane distance. |  |  |  |  |  |  |
| p. RF and DC I/O pads have separate and dedicated adjacent ground pads? |  |  |  |  |  |  |
| q. Capacitor top plate metal has been minimized. |  |  |  |  |  |  |
| r. Have PCM requirements been accounted for in the reticle layout. |  |  |  |  |  |  |
| s. Reticle layout requirements been observed. |  |  |  |  |  |  |
| t. Probe and bond pad usage has been checked. |  |  |  |  |  |  |

| IN-HOUSE (BOEING) ASIC TEST PREPARATION |  |  |  | Yes |  | No |
| --- | --- | --- | --- | --- | --- | --- |
| 98. Will testing or characterization of the ASIC be performed at Boeing? |  |  |  |  |  |  |
| (If no, skip questions 99 - 101). |  |  |  |  |  |  |
| 99. Is all required test equipment calibrated and available? |  |  |  |  |  |  |
| 100. Has test socket availability been confirmed? |  |  |  |  |  |  |
| 101. Have any test board components been ordered? |  |  |  |  |  |  |

| ASIC DATABASE VERIFICATION AND RELEASE |  | Yes | No |
| --- | --- | --- | --- |
| 102. Have DRC, ERC, and LVS checks been run on the design? |  |  |  |
| 103. Have the DRC, ERC, and LVS decks been checked for compatibility with |  |  |  |
| the foundry? |  |  |  |
| 104. Have the DRC, ERC, and LVS decks been modified in any way? |  |  |  |
| 105. Are there any discrepancies in the DRC, ERC, and LVS runs? |  |  |  |
| 106. Does the supplier have a current copy of the ED/SCD? |  |  |  |
| 107. Have any supplier-required design release forms been completed? |  |  |  |
| (attach copy(s)) |  |  |  |
| 108. Has the Statement of work (SOW) been accepted by the supplier? |  |  |  |
| 109. Is the contract with the supplier finalized? |  |  |  |

_________________________	_________	________________________	_________

Lead Design Engineer	 Date 	 Project Manager	 Date