Status Report

Nuttorn Jangkrajarng

February 28, 2003

Topic:   Parasitic-Aware Analog/RF Layout Automation via Template-Based

Design Reuse.

Last two weeks progress:

- Handled via insertion problem.  Extra is the choosing either one via per metal-pair, array of vias based on the original metal-pair size, or array of vias based on original via numbers.

- Expanded via removal/insertion into contacts.

- Updated constraint generation (connectivity part).  So it won’t disconnect the rectangle tiles, in case tiles are not purely in corner-stitched data structure.

- Revised the transistor resizing so contact removal/replacement works in SOI technology process case.

- VCO layout

  - Scanning the full-layout VCO problem is working.  The resizing and solving problem (only via-contact removal, transistors resizing, shortest path, individual rectangle minimization, via-contact insertion, and output generation) are performed.  Note that the symmetry and inductor part is not taken care yet.

  - DRC rule check of the VCO yields around 100,000 errors.

  - Located and resolved the DRC errors.  Currently only two more types of errors are left.

Plan for the next two weeks:

- VCO layout.

  - Fix the last two DRC errors.

  - Try symmetry axis computation.

- Update on a parasitic netlist extraction with Sam.

- Journal:  re-edit, introduction and examples.

- Discuss about the VCO and another analog example showing MIM.  Re-design the transistor sizes so the circuit has different performances.