Status Report

Nuttorn Jangkrajarng

January 17, 2003

Project: Parasitic-Aware Analog/RF Layout Automation via Template-Based

Design Reuse

Progress

========

- Planned the detailed timetable for project.

- Finished VIA insertion function (after VIA removal as part of run-time

reduction).

- Verified and fixed the SOI design-rule problems (with Roy) as

a. Fixed the unnessary constraints.

b. Manually added the missing design rule from the cadence tech dump.

(After investigating a few routes, none seems to be useful.  So  manually

edition is the recommended way - both us and Larry - to do because we

won't work with a lot of design rules in the future).

c. Updated the new kind of design rule constraints - as of extension of

0 and -1.  Also updated the original sweep-line function to include

identifying these kinds of rules.

d. Introduced the minimum width and length design rule.

- Tried loading the VCO design and running sweep line function.  At

least, the sweep-line constraints are generated (meaning, not hang

anymore).

- Researched on the wire-length-minimization.  There are still problems on

the hacked-version code I did.

Plan

====

- Redo the wire-length-minimization part.

- Check (in somewhat detail) if the constraints generated from VCO/SOI is

correct or not, after wrap up the SOI design rule problems.

- Edit the ISCAS paper.