Status Report

Roy Hartono

April 27, 2003

Summary:

Last two weeks progress:

- Created and modified test case layouts: cascode op-amp, 2-stage op-amp, ADC, and VCO

- Help editing the ICCAD paper

- One-tile resistor resizing is working after debugging and fixing the code for port connection.

- Integrated the IPRAIL code with Nuttorn’s version. However, the VCO resizing is not working after the CVS integration.

- Try to add the resistor matching for ADC case using the symmetry information to match the sizes and set the spacing of the resistors. However, as for now, we have not a solid algorithm to handle matching. What is done so far is having the parser and the data structure for the symmetry information. What needs to be done is to use this information to set the constraints between resistors. We probably need to put this aside because of more important matters.

Current work:

- Resistor matching

- Inductor regeneration for inductor.

Plan for the next two weeks:

- Get the inductor regeneration that can replace multi-turn inductor to a one-turn inductor. What we have now is code to resize multi-turn to multi-turn inductor and one-turn to one-turn inductor. For the time being, I plan to get multi-turn to one-turn inductor, which will be used for the VCO case.

- Either going back to resistor matching, or continuing on the inductor generation problem, since we recently figure out that there are still many more port connectivity cases to handle.