Status Report

Roy Hartono

January 17, 2003

Items accomplished:

- Added codes that support different inductor port connection in the inductor detection and inductor regeneration/placement.

- Added “counter-clockwise” square spiral inductor generation. Previously only has “clockwise” inductor generation.

- Fix the old EDAM code that identifies the contact connection between active to metal. The bug causes a single net to be fragmented into hundreds of smaller nets.

- Update the technology file parser and data structure to accommodate the SOI technology file. However, there are still some issues with the design rule, and some of them have to be added manually. This is because the dumped technology file from Cadence does not contain all the DRC rules. The actual DRC rules are found in divaDRC.rul, which is manually written and (according to Larry McMurchie) is not suitable to be parsed.

Next goal:

- To add textual reading of transistor size

- To have the code for capacitor and resistor resizing

Note:

Inductor modeling is being put aside for now. We discussed with Prof. Shi earlier, and he suggested not worrying about this for now.